| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0182-v1 |
| --- | --- |

| Title: | Non-EE2: Bi-predictive local illumination compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Ning Yan, Hong-Jheng Jhu, Wei Chen, C.-W. Kuo, Changyue Ma, Xianglin Wang | Email: | xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In ECM-6.0, local illumination compensation (LIC) is one inter tool to address the illumination variation between one current block and its prediction block. The tool is only enabled to uni-predicted blocks. This contribution proposes to extend the existing LIC design to bi-prediction. Specifically, when applying the LIC to bi-predicted blocks, two sets of LIC parameters (i.e., scale and offset) are calculated and applied to L0 and L1 prediction blocks separately, which are then combined to form the final bi-prediction. One iterative approach is used to calculate the LIC parameters in L0 and L1 by minimizing the difference between the template samples of the current block and their prediction samples.

The proposed method is tested based on ECM-6.0 software with tool-on configuration (BASE_ENCODER = 1, BASE_NORMATIVE = 1 and TOOLS = 0). It reportedly shows that the proposed bi-predictive LIC scheme can provide 0.33% and 0.29% BD-rate savings for RA and LDB configurations, respectively, over the existing uni-predictive LIC.

# Introduction

In ECM-6.0, local illumination compensation (LIC) is one inter coding technique that aims at addressing the illumination variations between one block and its prediction block. The LIC is based on one linear model where a scale α and an offset β are derived from the template samples neighboring to the current block and their corresponding prediction samples. The derived LIC parameters are then applied to adjust the prediction samples of the block as

P'x,y=α∙Px,y+β

Currently, the LIC is only applicable to uni-predictive inter CUs which contains no less than 32 luma samples.

# Proposal

It is proposed to extend the existing LIC design to bi-predicted CUs. Specifically, in the proposed scheme, two different linear models are derived to compensate the illumination changes that exist between the current block and its two prediction blocks. Then, the final bi-prediction of the current block is calculated as the combination of two uni-prediction blocks after the LIC adjustment, i.e.,

P'x,y=1-ω∙p0'x,y+ω∙p1'x,y

and

p0'x,y=α0∙P0x,y+β0

p1'x,y=α1∙P1x,y+β1

where α0 and β0, and α1 and β1 indicate the scales and the offsets in L0 and L1, respectively; ω indicates the weight (as indicated by the CU-level BCW index) that is applied when combining the two uni-prediction blocks. Same to the current LIC design, one control flag is signaled for AMVP bi-predicted CUs to indicate the enabling/disabling of the LIC while the flag is inherited from one neighboring block for merge inter CUs. Additionally, the LIC is disabled when any of combined inter and intra (CIIP), decoder-side motion vector refinement and bi-directional optical flow (BDOF) is applied.

To reuse the existing linear model derivation of the uni-predictive LIC, one iterative approach is applied in the contribution to alternately derive the L0 and L1 linear models. Specifically, given the two MVs of the current block, it assumes T0 and T1 are the two predictions of the current block’s template T. The method firstly derives the L0 linear model (α0 and β0) that result in the minimum difference between T0 and T; Then, based on the resulting L0 parameters, the L1 linear model (α1 and β1) can be calculated that minimizes the template T1’s difference with 2T-T0', where T0' is the LIC compensated template prediction in L0. Finally, the L0 linear model is re-calculated which utilizes 2T-T1' as the reference, where T1' is the LIC compensated template prediction in L1.

# Experimental results

The proposed scheme is implemented on top of ECM-6.0 software and tested under the tool-on condition, i.e., BASE_ENCODER = 1, BASE_NORMATIVE = 1 and TOOLS = 0.

In the below, the anchor is the ECM-6.0 LIC tool-on results, i.e., BASE_ENCODER = 1, BASE_NORMATIVE = 1, TOOLS = 0, INTER_LIC = 1. The test is the anchor plus the proposed bi-predictive LIC.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 ToolON LIC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -0.39% | -0.43% | 112% | 101% |
| Class A2 | -0.24% | -0.26% | -0.21% | 116% | 98% |
| Class B | -0.35% | -0.24% | -0.15% | 116% | 100% |
| Class C | -0.26% | -0.10% | -0.20% | 121% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.24% | -0.23% | 116% | 100% |
| Class D | -0.19% | -0.07% | 0.09% | 117% | 102% |
| Class F | -0.45% | -0.44% | -0.42% | 116% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 ToolON LIC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.30% | -0.12% | -0.39% | 114% | 100% |
| Class C | -0.33% | -0.63% | -0.49% | 114% | 100% |
| Class E | -0.22% | -0.06% | 0.48% | 116% | 102% |
| Overall | -0.29% | -0.27% | -0.20% | 115% | 100% |
| Class D | -0.02% | -0.40% | -0.10% | 116% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# Conclusion

This contribution proposes to extend the existing design of uni-predictive LIC to bi-prediction. Simulation results show that the proposed bi-predictive LIC scheme can provide 0.33% and 0.29% BD-rate savings for RA and LDB configurations, respectively, on top of the ECM-6.0 tool-on configuration. It is recommended to study the proposed scheme in the next round of EE.

# Reference

- ECM-6.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).