| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0183-v1 |
| --- | --- |

| Title: | Algorithm Description for Neural Compression Software |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Kai Zhang, Li Zhang Hongtao Wang, Samuel Eadie, Muhammed Coban, Marta Karczewicz Liqiang Wang, Xiaozhong Xu, Shan Liu Franck Galpin Jacob Ström | Email: | yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com hongtaow@qti.qualcomm.com seadie@qti.qualcomm.com mcoban@qti.qualcomm.com martak@qti.qualcomm.com liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com franck.galpin@interdigital.com jacob.strom@ericsson.com |
| Source: | Bytedance, Qualcomm, Tencent, InterDigital, Ericsson. |  |  |

_____________________________

Contents

1	Algorithm description of Neural Compression Software	1

1.1 Neural network-based loop filter set 0	2

1.1.1	Pre-processing and post-processing of chroma	2

1.1.2	Neural network	2

1.1.3	Residue scaling	2

1.1.4	Combination with deblocking filter	2

1.1.5	Inference details	3

1.2 Neural network-based loop filter set 1	3

1.2.1	Neural network for luma component	3

1.2.2	Neural network for chroma component	4

1.2.3	Adaptive inference granularity	4

1.2.4	Parameter selection	4

1.2.5	Residue scaling	5

1.2.6	Combination with deblocking filter	5

1.2.7	Inference details	5

1.3 Small ad-hoc deep learning (SADL) library	6

2	Patent rights declaration(s)	6

Algorithm description of Neural Compression Software

The technical detail of each method is described in the following sub-sections.

Neural network-based loop filter set 0

Pre-processing and post-processing of chroma

In filter set 0, a single filter model is designed to process three components. Since the resolutions of luma and chroma are different, pre-processing and post-processing steps are introduced to up-sample and down-sampling chroma components respectively as shown in Figure. 1. In the resampling process, the nearest-neighbor interpolation method is used.

Figure. 1 the pre-processing and post-processing units

Neural network

The network structure of the CNN filter is shown in Figure. 2. Along with the reconstructed image (rec_yuv), additional side information is also fed into the network, such as the prediction image (pred_yuv), slice QP, base QP and slice type. In the ResBlock, the number of channels firstly goes up before the activation layer, and then goes down after the activation layer. Specifically, K and M are set to 64 and 160 respectively, and the number of Resblock is set to 32.

Figure. 2. Architecture of the CNN in filter set 0.

Residue scaling

Refer to Section 1.2.5

Combination with deblocking filter

There are 3 fixed weights to blend the results of the CNN filter and the conventional filter. These fixed weights are 1, 0.75 and 0.5.

Inference details

SADL (see Section 1.3) is used for performing the inference of the CNN filters. Both floating point-based and fixed point-based implementations are supported. In the fixed-point implementation, both weights and feature maps are represented with int16 precision using a static quantization method. The network information in the inference stage is provided in Table 1.

Table 1. Network Information of filter set 0 in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.9M |
|  | Total Parameter Number | 1.9M |
|  | Parameter Precision (Bits) | float: 32 int: 16 |
|  | Memory Parameter (MB) | float: 7.6MB int: 3.8MB |
|  | Multiply Accumulate (kMAC/pixel) | 485/pixel (assuming frame-level input) 615/pixel (assuming block-level input) |
| Optional |  |  |
|  | Total Conv. Layers | 101 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144×144 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Neural network-based loop filter set 1

Neural network for luma component

The backbone of the CNN filter is shown in Figure. 3. The calculation process in the attention module can be written as:

F_out = F_in × f (Rec, Pred, BS, QP) + F_in

where F_in and F_out denote the input and the output of the attention module, respectively. Rec, Pred, BS, and QP stand for the reconstruction, the prediction, the boundary strengths, and the sequence-level input quantization parameter respectively. f comprises 2 convolutional layers, where an activation function is applied after the first convolutional layer. The objective of f is to generate a spatial attention map from external information, which then recalibrates the feature maps F_in.

The input of the network comprises reconstruction, prediction, boundary strengths, and QP (split is additionally taken as input in intra slice models). The numbers of feature maps and attention residual blocks are set as 96 and 8 respectively.

Figure. 3. (a) Architecture of the CNN in filter set 1. M denotes the number of feature maps. N stands for the number of samples in one dimension. (b) Construction of Attention Residual Block in (a).

Neural network for chroma component

Luma information is taken as additional input for the in-loop filtering of chroma. Considering the resolution of luma is higher than chroma in YUV 4:2:0 format, features are first extracted separately from luma and chroma. Then luma features are down-sampled and concatenated with chroma features. Regarding network backbone, chroma components use the same one as luma.

Adaptive inference granularity

The granularity of the filter determination and the parameter selection is dependent on resolution and QP. Given a higher resolution and a larger QP, the determination and selection will be performed in a larger region.

Parameter selection

Each slice or block could determine whether to apply the CNN-based filter or not. When the CNN-based filter is determined to be applied to a slice/block, which conditional parameter from a candidate list including three candidates derived from QP could be further decided. Denote the sequence level QP as q, the candidate list includes conditional parameters {Param_1, Param_2, Param_3}. For low temporal layers, Param_1 = q, Param_2 = q-5, Param_3 = q-10. For high temporal layers, Param_1 = q, Param_2 = q-5, Param_3 = q+5. In other words, the third candidate is different across different temporal layers.

The selection process is based on the rate-distortion cost at the encoder side. Indication of on/off control as well as the conditional parameter index, if needed, are signalled in the bitstream. Figure. 4 shows the diagram of parameter selection at encoder and decoder sides. All blocks in the current frame need to be processed with three conditional parameters first. Then five costs, i.e. Cost_0, ..., Cost_5, are calculated and compared against each other to achieve optimum rate-distortion performance. In Cost_0, CNN-based filter is prohibited for all blocks. In Cost_i, {i = 1, 2, 3}, the parameter Param_i is used for all blocks. In Cost_4, different blocks may prefer different parameters, and the information regarding whether to use CNN-based filter or which parameter to be used is signaled for each block. At decoder side, whether to use CNN-based filter or which parameter to be used for a block is based on the Param_Id parsed from the bit-stream as shown in Figure. 4 (b).

Note that for all-intra configuration, parameter selection is disabled while filter on/off control is still preserved. A shared conditional parameter is used for the two chroma components to ease the burden in worst case at decoder side. In addition, the max number of conditional parameter candidates could be specified at encoder side.

Figure. 4. (a) Parameter selection at encoder side. (b) Parameter selection at decoder side.

Residue scaling

When a NN filter is being applied to reconstructed pictures, a scaling factor is derived and signaled for each color component in the slice header. The derivation is based on least square method. The difference between the input samples and the NN filtered samples (residues) are scaled by the scaling factors before being added to input samples.

Combination with deblocking filter

To enable a combination with deblocking, the input samples used in the residual scaling is the output of deblocking filtering. The residual scaling process is shown below, where RNN and RDB refer to the outputs of NN filtering and deblocking filtering respectively.

RRefine = (RNN-RDB)×w + RDB=w×RNN + (1-w)×RDB

Inference details

SADL (see Section 1.3) is used for performing the inference of the CNN filters. Both floating point-based and fixed point-based implementations are supported. In the fixed-point implementation, both weights and feature maps are represented with int16 precision using a static quantization method. The network information in the inference stage is provided in Table 2.

Table 2. Network Information of filter set 1 in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 1.56M/model, 4 models in total for all tests |
|  | Parameter Precision (Bits) | float: 32 int: 16 |
|  | Memory Parameter (MB) | float: 6.3MB/model, 4 models int: 3.1MB/model, 4 models |
|  | Multiply Accumulate (kMAC/pixel) | 539K/pixel (assuming frame-level input) 682K/pixel (assuming block-level input) |
| Optional |  |  |
|  | Total Conv. Layers | 25+16 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144×144, 272×272 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Small ad-hoc deep learning (SADL) library

SADL (Small Ad-hoc Deep-Learning Library) is a header only small library for inference of neural networks. SADL provides both floating-point-based and integer-based inference capabilities. The inference of neural networks in NCS is based on the SADL.

The table below summarizes the framework characteristics.

Table 3. Characteristics of SADL

| Language | Pure C++, header only. |
| --- | --- |
| Footprint | 5000 LOC, library ~200kB, no dependency |
| Optimization | Some SIMD at hot spots (best effort) |
| Compatibility | ONNX file |
| Layer Supports | constants, add, maxPool, matMul, reshape, ReLU, conv2D, mul, concat, max, leakyReLU, shape, expand, transpose |
| Type support | float, int32, int16, int8 |
| Quantization | Support adaptive quantizer per layer |
| License | BSD 3-Clause |

Patent rights declaration(s)

Bytedance, Qualcomm, Tencent, InterDigital, and Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).