| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0195 |
| --- | --- |

| Title: | Crosscheck of JVET-AB0184 (EE2-5.1: Extended Fixed-Filter-Output based Taps for ALF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu | Tel: Email: | xuluhang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-AB0184 EE2-5.1: Extended Fixed-Filter-Output based Taps for ALF. It is reported that the partial crosscheck results match exactly with the results provided by the proponent.

Introduction

The contribution JVET-AB0184 proposes to an extension for ALF is shown in Figure-1, where spatial taps (i.e., tap #0 ~ #19) and reconstruction-before-DBF based taps (i.e., tap #26, #27, #30) are kept the same as the ECM-6.0, and more fixed-filter-output based taps (i.e., tap #20 ~ #25) are introduced into luma online-trained filters. Please note that the fixed-filter-output based taps have already been used in ECM-6.0. The coefficient signaling mechanism is the same as the current design in ECM and there is no filter shape switching.

Figure-1 The Newly Introduced Filter Shape for ALF.

Simulation Results

The crosscheck has been performed in All Intra, Random Access and Low Delay B configuration. So far, only partial results are available (attached to this file) and they are matched with the results provided by the proponent.

Table 1. Crosscheck reults (AI)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.00% | 0.00% | 106% | 103% |
| Class A2 | -0.11% | 0.00% | 0.00% | 104% | 103% |
| Class B | -0.07% | 0.02% | 0.02% | 103% | 102% |
| Class C | -0.06% | 0.01% | 0.01% | 103% | 101% |
| Class E | -0.14% | 0.04% | 0.04% | 104% | 102% |
| Overall | -0.09% | 0.02% | 0.02% | 104% | 102% |
| Class D | -0.04% | 0.05% | 0.05% | 103% | 102% |
| Class F | -0.07% | 0.03% | 0.03% | 103% | 103% |

Table 2. Crosscheck reults (RA)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.15% | -0.03% | -0.02% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.22% | -0.15% | 0.08% | 103% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 3. Crosscheck reults (LDB)

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.27% | -0.56% | 0.12% | 103% | 101% |
| Class E | -0.09% | 0.20% | 0.11% | 107% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.31% | -0.25% | 0.39% | 105% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |