| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0054-v2 |
| --- | --- |

| Title: | EE2-1.12: Gradient and location based convolutional cross-component model (GL-CCCM) for intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ramin G. Youvalari Pekka Astola Jani Lainema | Email: | ramin.ghaznavi-youvalari@nokia.com, pekka.astola@nokia.com, jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports results for EE2 Test 1.12a and Test 1.12b which were proposed in JVET-AB0119. Test 1.12a proposes a 7-tap gradient and location based convolutional cross-component model (GL-CCCM) and Test 1.12b proposes a 5-tap gradient based CCCM.  The proposed methods map luma values into chroma values when the prediction mode is activated by a PU level flag. The filter input in Test 1.12a consists of one spatial luma sample, two gradient values, two location information, a nonlinear term, and a bias term. The filter inputs for Test 1.12b are the same as Test 1.12a but without the location information. Filter coefficients are derived for each chroma block separately using regression based MSE minimization (i.e., the same solver as CCCM) on reference samples in the PU’s neighborhood. The impacts on coding efficiency and runtimes over ECM-7.0 are reportedly {for Y, U, V, EncT, DecT}:

Test 1.12a:

AI { -0.04%, -0.56%, -0.59%, 100%, 100%}, RA { -0.00%, -0.40%, -0.35%, 100%, 99%}

Test 1.12b:

AI { 0.02%, -0.17%, -0.27%, 99%, 99%}, RA { 0.03%, -0.07%, -0.15%, 99%, 100%}

Moreover, the gains are observed to be mostly additive when it is combined with EE2-1.13 and EE2-1.14 tests in JVET-AC0147. The impacts on coding efficiency and runtimes of combined tests over ECM-7.0 are reportedly {for Y, U, V, EncT, DecT}:

Combined Test 1.12a and Test 1.13:

AI { -0.08%, -0.68%, -0.71%, 104%, 101%}, RA { -0.05%, -0.58%, -0.57%, 106%, 101%}

Combined Test 1.12a and Test 1.14:

AI { -0.07%, -0.51%, -0.60%, 104%, 99%}, RA { -0.00%, -0.34%, -0.48%, 105%, 99%}

The runtimes in combined tests are not reliable.

# Studied approach

## Introduction

Convolutional cross-component model (CCCM) [2] predicts chroma samples from reconstructed luma samples using a 7-tap filter. Similarly to CCLM, there is an option of using a single model or multi-model variant of CCCM. The multi-model variant uses two models, one model derived for samples above the average luma reference value and another model for the rest of the samples (following the spirit of the CCLM design). Multi-model CCCM mode can be selected for PUs which have at least 128 reference samples available.

The CCCM filter consist of a 5-tap plus sign shape spatial component, a nonlinear term and a bias term. The input to the spatial 5-tap component of the filter consists of a center (C) luma sample which is collocated with the chroma sample to be predicted and its above/north (N), below/south (S), left/west (W) and right/east (E) neighbors as illustrated below.

Figure 1: Spatial part of the convolutional filter

The nonlinear term P is represented as power of two of the center luma sample C and scaled to the sample value range of the content:

P = ( C*C + midVal ) >> bitDepth

That is, for 10-bit content it is calculated as:

P = ( C*C + 512 ) >> 10

The bias term B represents a scalar offset between the input and output (similarly to the offset term in CCLM) and is set to middle chroma value (512 for 10-bit content).

Output of the filter is calculated as a convolution between the filter coefficients ci and the input values and clipped to the range of valid chroma samples:

predChromaVal = c0C + c1N + c2S + c3E + c4W + c5P + c6B

## Proposed Gradient and Location based convolutional cross-component model (GL-CCCM)

The proposed GL-CCCM method [1] uses gradient and location information instead of the 4 spatial neighbor samples in the CCCM filter. The GL-CCCM filter for the prediction is:

predChromaVal = c0C + c1Gy + c2Gx + c3Y + c4X + c5P + c6B

Where Gy and Gx are the vertical and horizontal gradients, respectively, and are calculated as:

Gy = (2N + NW + NE) – (2S + SW + SE)

Gx = (2W + NW + SW) – (2E + NE + SE)

Moreover, the Y and X parameters are the vertical and horizontal locations of the center luma sample.

The rest of the parameters are the same as CCCM tool. The reference area for the parameter calculation is the same as CCCM method.

Figure 2: Spatial samples used for GL-CCCM

Two tests have been studied in this EE:

Test 1.12a: gradient and location based CCCM, as proposed in [1], where a 7-tap filter with following information are used as inputs to the filter: C, Gy, Gx, Y, X, P, B

Test 1.12b: gradient based CCCM, where a 5-tap filter with following information are used as inputs to the filter: C, Gy, Gx, P, B

## Bitstream signalling

Usage of the mode is signalled with a CABAC coded PU level flag. One new CABAC context was included to support this. When it comes to signalling, GL-CCCM is considered a sub-mode of CCCM. That is, the GL-CCCM flag is only signalled if original CCCM flag is true.

## Encoder operation

Similar to the CCCM tool in ECM-7.0, GL-CCCM tool has 6 modes for calculating the parameters:

- Single-model GL-CCCM from above and left templates

- Single-model GL-CCCM from above template

- Single-model GL-CCCM from left template

- Multi-model GL-CCCM from above and left templates

- Multi-model GL-CCCM from above template

- Multi-model GL-CCCM from left template

The encoder performs SATD search for the 6 GL-CCCM modes along with the existing CCCM modes to find the best candidates for full RD tests. Moreover, the proposed method does not increase the maximum number of full RD modes in ECM-7.0 for CCCM tool.

# Experimental results

Tables below summarize the coding efficiency and runtime impact of the proposed method in ECM 7.0 using the JVET common test conditions [3].

Results for EE2-1.12a:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06 % | -0.72 % | -0.99 % | 100 % | 102 % |
| Class A2 | -0.03 % | -0.81 % | -0.78 % | 101 % | 102 % |
| Class B | -0.03 % | -0.67 % | -0.78 % | 100 % | 98 % |
| Class C | -0.03 % | -0.28 % | -0.41 % | 101 % | 97 % |
| Class E | -0.05 % | -0.34 % | 0.10 % | 100 % | 102 % |
| Overall | -0.04 % | -0.56 % | -0.59 % | 100 % | 100 % |
| Class D | -0.01 % | -0.48 % | -0.30 % | 101 % | 101 % |
| Class F | -0.05 % | -0.41 % | -0.53 % | 101 % | 102 % |
| Class TGM | 0.01 % | -0.07 % | -0.05 % | 101 % | 98 % |

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01 % | -0.37 % | -0.43 % | 100 % | 99 % |
| Class A2 | -0.01 % | -0.28 % | -0.25 % | 99 % | 99 % |
| Class B | 0.00 % | -0.53 % | -0.56 % | 100 % | 98 % |
| Class C | 0.00 % | -0.33 % | -0.11 % | 100 % | 99 % |
| Class E |  |  |  |  |  |
| Overall | 0.00 % | -0.40 % | -0.35 % | 100 % | 99 % |
| Class D | 0.01 % | -0.10 % | -0.25 % | 100 % | 99 % |
| Class F | -0.11 % | -0.59 % | -0.40 % | 100 % | 98 % |
| Class TGM | 0.05 % | -0.09 % | -0.01 % | 100 % | 99 % |

Results for EE2-1.12b:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04 % | -0.29 % | -0.42 % | 98 % | 96 % |
| Class A2 | 0.04 % | -0.24 % | -0.18 % | 99 % | 98 % |
| Class B | 0.02 % | -0.18 % | -0.43 % | 98 % | 99 % |
| Class C | 0.01 % | -0.12 % | -0.15 % | 100 % | 99 % |
| Class E | -0.01 % | -0.01 % | -0.10 % | 98 % | 101 % |
| Overall | 0.02 % | -0.17 % | -0.27 % | 99 % | 99 % |
| Class D | 0.02 % | -0.26 % | -0.14 % | 101 % | 100 % |
| Class F | 0.00 % | -0.13 % | -0.40 % | 98 % | 101 % |
| Class TGM | -0.02 % | -0.12 % | -0.14 % | 100 % | 101 % |

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06 % | -0.35 % | -0.17 % | 99 % | 101 % |
| Class A2 | 0.03 % | 0.11 % | -0.07 % | 99 % | 100 % |
| Class B | 0.01 % | -0.04 % | -0.36 % | 99 % | 100 % |
| Class C | 0.02 % | -0.05 % | 0.06 % | 99 % | 101 % |
| Class E |  |  |  |  |  |
| Overall | 0.03 % | -0.07 % | -0.15 % | 99 % | 100 % |
| Class D | -0.01 % | 0.08 % | 0.17 % | 100 % | 101 % |
| Class F | -0.06 % | -0.35 % | -0.11 % | 98 % | 99 % |
| Class TGM | 0.04 % | -0.07 % | 0.04 % | 99 % | 105 % |

Combination tests with EE2-1.13 and EE2-1.14

The impact of the proposed GL-CCCM method (Test 1.12a), in AI and RA configurations, is studied when it is combined with EE2-1.13 and EE2-1.14 in JVET-AC0147 [4].

Results in AI configuration:

| vs. ECM 7.0 | Test 1.12a | Test 1.13 | Test 1.14 | 1.12a + 1.13 | 1.12a + 1.14 |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.06 % | 0.01 % | 0.02 % | -0.05 % | -0.05 % |
| Class A2 | -0.03 % | -0.23 % | -0.23 % | -0.28 % | -0.28 % |
| Class B | -0.03 % | 0.01 % | 0.02 % | -0.03 % | -0.01 % |
| Class C | -0.03 % | -0.02 % | 0.00 % | -0.05 % | -0.04 % |
| Class E | -0.05 % | 0.04 % | 0.02 % | -0.02 % | 0.00 % |
| Average Y | -0.04 % | -0.03 % | -0.03 % | -0.08 % | -0.07 % |
| Average UV | -0.57 % | -0.21 % | -0.02 % | -0.69 % | -0.55 % |
| Class D | -0.01 % | 0.00 % | 0.01 % | -0.01 % | -0.01 % |
| Class F | -0.05 % | -0.22 % | -0.23 % | -0.28 % | -0.27 % |
| Class TGM | 0.01 % | -1.27 % | -1.16 % | -1.26 % | -1.16 % |

Results in RA configuration:

| vs. ECM 7.0 | Test 1.12a | Test 1.13 | Test 1.14 | 1.12a + 1.13 | 1.12a + 1.14 |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.01 % | 0.05 % | 0.11 % | 0.03 % | 0.11 % |
| Class A2 | -0.01 % | -0.23 % | -0.20 % | -0.26 % | -0.19 % |
| Class B | 0.00 % | 0.01 % | 0.06 % | 0.01 % | 0.03 % |
| Class C | 0.00 % | 0.00 % | 0.03 % | -0.01 % | 0.01 % |
| Class E |  |  |  |  |  |
| Average Y | 0.00 % | -0.03 % | 0.01 % | -0.05 % | 0.00 % |
| Average UV | -0.37 % | -0.25 % | -0.15 % | -0.58 % | -0.41 % |
| Class D | 0.01 % | 0.00 % | 0.00 % | -0.02 % | -0.02 % |
| Class F | -0.11 % | -0.26 % | -0.23 % | -0.29 % | -0.25 % |
| Class TGM | 0.05 % | -0.33 % | -0.29 % | -0.35 % | -0.32 % |

# Conclusion

This contribution proposes the gradient and location based CCCM method as extensions to the CCCM tool. The results show that the gain of CCCM can be further improved using the GL-CCCM method in Test 1.12a. It is suggested to adopt the proposed method in ECM-8.0 software.

# References

- R. G. Youvalari, P. Astola, J. Lainema, “Non-EE2: Gradient and location based convolutional cross-component model (GL-CCCM) for intra prediction,” JVET-AB0119, Oct. 2022.

- P. Astola, J. Lainema, R. G. Youvalari, A. Aminlou, and K. Panusopone, “EE2-1.1a: Convolutional cross-component intra prediction model,” JVET-AA0057, Jul. 2022.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

- H.-J. Jhu, C.-W. Kuo, X. Xiu, W. Chen, N. Yan, C. Ma, X. Wang, V. Seregin, Y.-J. Chang, B. Ray, M. Karczewicz,” EE2-1.13 and 1.14: CCCM using non-downsampled luma samples,” JVET-AC0147, Jan. 2023.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).