| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0059-v1 |
| --- | --- |

| Title: | AHG12: TMP using Reconstruction-Reordered for screen content coding (RR-TMP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jung-Kyung Lee, Damian Ruiz Coll, Vikas Warudkar | Tel: Email: | jlee@ofinno.com, duizcoll@ofinno.com, vwarudkar@ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution proposes Reconstruction-Reordered Template Matching Prediction (RR-TMP) for screen content video coding. When RR-TMP is applied, the prediction signal is generated by matching the flipped L-shaped template causal neighbor of the current block with another reference block for intraTMP in a predefined search area. RR-TMP finds the best-matched flipped template position, flips the corresponding reference block according to the flip type, and uses it as a prediction block of the current block. Simulation results based on ECM-7.0 are reported below:

AI: Class F -0.14%, 0.08%, -0.12%, 101%, 101%; Class TGM -0.28%, -0.25%, -0.24%, 101%, 104%

RA: Class F -0.06%, -0.27%, -0.14%, 100%, 100%; Class TGM -0.16%, -0.27%, -0.14%, 100%, 100%

# Introduction

Since symmetry is often observed in screen content, a Reconstruction-Reordered IBC (RR-IBC) mode was adopted in ECM-6.0 [1] to improve the coding efficiency of IBC further. In RR-IBC, a motion shift is added to a block vector candidate obtained from a neighbouring block according to a symmetric rule in case the neighbouring block is coded with RR-IBC.

In IntraTMP, the prediction signal is generated at the decoder side by matching the L-shaped template of the current block with another block in a predefined search area. Using IntraTMP BV of neighbouring blocks was proposed in JVET-AB0061 [2], which employs IntraTMP BV of the neighbour blocks for the IBC candidate list.

# Proposal

In this contribution, Reconstruction-Reordered TMP is proposed for screen content video coding. When RR-TMP is applied, both the encoder and decoder search for the most similar template to the flipped current template in a predefined search range, which is a reconstructed part of the current frame. The corresponding block is flipped according to flip type, horizontal or vertical flip, and used as a prediction block.

In contrast with the regular TMP, the search regions for RR-TMP are constrained. For the horizontal flip type, only a portion of the current CTU (R1) and left region (R4) are explored. Regarding the vertical flip mode, only a portion of the current CTU (R1) and the above region (R3) are explored.

The dimensions of search regions, search range width (SRW), and search range height (SRH), are set proportional to the block dimension, which is 5 times of block dimension (CbHeight, CbWidth) as shown in Figure 1.

Figure 1. RR-TMP search region for (a) horizontal flip, and (b) vertical flip.

# Experimental Results

The proposed method is tested on top of ECM-7.0 software [3] under JVET ECM common test conditions [4] in AI and RA configurations. The following results were obtained:

Table I. Experimental result of the proposed method under AI configuration

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.14% | 0.08% | -0.12% | 101% | 101% |
| Class TGM | -0.28% | -0.25% | -0.24% | 101% | 104% |
| Overall | -0.21% | -0.09% | -0.18% | 101% | 103% |

Table II. Experimental result of the proposed method under RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.06% | -0.27% | -0.14% | 101% | 100% |
| Class TGM | -0.16% | -0.27% | -0.14% | 100% | 100% |
| Overall | -0.11% | -0.27% | -0.14% | 100% | 100% |

- References

- Zhipin Deng, Kai Zhang, Li Zhang, “EE2-3.2: Reconstruction-Reordered IBC for screen content coding”, JVET-AA0070, July. 2022.

- Woong Lim, Donghyun Kim, Jongho Kim, Sung-Chang Lim, “EE2-3.2: Using block vector derived from IntraTMP for IBC”, JVET-AB0061, Oct. 2022.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-7.0

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-Y2017, 25th meeting, by teleconference, Jan. 2022.

# Patent rights declaration(s)

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).