| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0078-v1 |
| --- | --- |

| Title: | EE1-2.1: Updates on RPR encoder and post-filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Sunmi Yoo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10680 Treena St, San Diego, CA 92131, USA | Tel: Email: | junghak.nam@lge.com sunmi.yoo@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE1-2.1 from JVET-AB0102, which proposed adaptive selection of picture resolution with four scale factors {×2.0, ×1.5, ×1.25, ×1.0} and also reported performance of long-tap filters for reduced resolution. In this EE, three tests are further investigated. Test 2.1.1 only supports two scale factors {×2.0, ×1.0} comparing with NN-based up-sampling techniques in super-resolution category. Test 2.1.2 uses all scale factors {×2.0, ×1.5, ×1.25, ×1.0} to achieve bit-saving with RPR functionality in VVC. Test 2.1.3 applies long-tap post filter (8-tap for luma, 6-tap for chroma) for small pictures, which are up-scaled to display full-resolution, on top of test 2.1.2. The experimental results are reportedly shown as follows,

For EE1-2.1.1,

  - RA: -0.70% (Y), 0.54% (U), 0.56% (V), 97% (EncT), 90% (DecT)

  - AI: -0.35% (Y), 0.76% (U), 0.70% (V), 101% (EncT), 93% (DecT)

For EE1-2.1.2,

- RA: -1.23% (Y), 3.21% (U), 2.66% (V), 93% (EncT), 81% (DecT)

- AI: -0.80% (Y), 4.23% (U), 3.19% (V), 100% (EncT), 88% (DecT)

For EE1-2.1.3,

- RA: -1.23% (Y), 2.91% (U), 2.36% (V), 93% (EncT), 81% (DecT)

- AI: -0.81% (Y), 3.90% (U), 2.95% (V), 99% (EncT), 87% (DecT)

Introduction

RPR functionality in VVC had used as an anchor for the proposals in EE1 super-resolution category. At the JVET-Y0068 [1] and JVET-Z0065[2], RPR encoder utilizing adaptive resolution selection with multiple scale factors {×2.0, ×1.5, ×1.0} had proposed. In the last meeting, JVET-AB0102 proposed adaptive selection of picture resolution with four scale factors {×2.0, ×1.5, ×1.25, ×1.0} to further improve coding gain. The contribution provides EE tests for JVET-AB0102.

Proposed method

The proposed adaptive resolution is selected at GOP level, and applied to all pictures in the GOP. To determine a scale factor, a PSNR value and initial QP for the first picture of each GOP are utilized. The first picture is downscaled to quarter resolution with ×2.0 scale factor using down-sampling filters in VTM software and then resampled to the source resolution. The PSNR is calculated between original and down-up scaled pictures. One of scale factors, {×2.0, ×1.5, ×1.25, ×1.0}, is selected based on a threshold which is adaptively varied depending on the initial QP. RPR scale factor (RPR_SF) of i-th GOP is derived as follows,

RPR_SFGOPi=2.0,DnUpPSNRi>(BasePSNR)-QPi-BaseQP*0.5                                       1.5,DnUpPSNRi>BasePSNR-OffsetPSNR-QPi-BaseQP*0.5    1.25,DnUpPSNRi>BasePSNR-OffsetPSNR*2-QPi-BaseQP*0.51.0, Oherwise

where DnUpPSNRi represents down-up scaled PSNR for the first picture of i-th GOP. BasePSNR and BaseQP indicate the predefined criteria for PSNR and QP, respectively. OffsetPSNR is a gap of PSNR to determine a scale factor among ×2.0, ×1.5, ×1.25 and ×1.0. QPi is the initial QP for the first picture of i-th GOP. The threshold is adjusted in multiples of 0.5 for the difference between the initial QP and the BaseQP. In this contribution, BaseQP and OffsetPSNR are set to 37 and 3. BasePSNR for RA and AI are set to 47 and 44, respectively. When one of ×2.0, ×1.5, or ×1.25 scale factors is encoded, the QP value is adjusted with offset −6, −4, or −2, respectively.

A picture having reduced resolution requires resampling (or up-sampling) at original picture size for performance evaluation or display in manner of post-filter. In NNVC-3.0, 8-tap and 4-tap up-sampling filters are used for luma and chroma components, respectively, which are identical to ones in VVC specification. Currently, both VTM and ECM use long-tap filters for picture upsampling. This contribution proposes to use the long-tap resampling filters available in ECM, which are 12-tap and 6-tap filters for luma and chroma components, respectively.

Experimental results

The proposed method is implemented on top of NNVC-3.0 anchor [3] and the tests are conducted by NNVC CTC [4]. Three experiments are planned for EE1-2.1 as follows,

EE1-2.1.1: Only using ×2.0 scale factor

EE1-2.1.2: Adaptive resolution selection using ×1.25, ×1.5 and ×2.0 scale factors

EE1-2.1.3: EE1-2.1.2 + long-tap post filter

For EE1-2.1.1, encoding parameters set as follows,

| ScalingRatioHor ScalingRatioVer FractionNumFrames UpscaledOutput | : 2.0 : 2.0 : 1 : 2 |
| --- | --- |

To support multiple scale factors for both EE1-2.1.2 and EE1-2.1.3, encoding parameters such as ScalingRatioHor2, ScalingRatioVer2, ScalingRatioHor3, and ScalingRatioVer3 are added and set as follows,

| ScalingRatioHor ScalingRatioVer ScalingRatioHor2 ScalingRatioVer2 ScalingRatioHor3 ScalingRatioVer3 FractionNumFrames UpscaledOutput | : 2.0 : 2.0 : 1.5 : 1.5 : 1.25 : 1.25 : 1 : 2 |
| --- | --- |

Table 1 and Table 2 show the performance of the EE1-2.1.1 under RA and AI configurations, respectively.

Table 1. Performance of the EE1-2.1.1 in RA

|  | BD-rate Over VTM-11.0_nnvc-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -3.14% | 1.70% | 1.87% | 88% | 69% |
| Class A2 | -0.26% | 0.59% | 0.50% | 97% | 93% |
| Class B | -0.07% | 0.25% | 0.27% | 101% | 96% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.70% | 0.54% | 0.56% | 97% | 90% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 99% |

Table 2. Performance of the EE1-2.1.1 in AI

|  | BD-rate Over VTM-11.0_nnvc-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.98% | 1.04% | 1.02% | 104% | 79% |
| Class A2 | -1.15% | 2.38% | 1.83% | 95% | 89% |
| Class B | 0.03% | 0.67% | 0.82% | 102% | 96% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 98% |
| Overall | -0.35% | 0.76% | 0.70% | 101% | 93% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 99% |

Table 3 and Table 4 show the performance of the EE1-2.1.2 under RA and AI configurations, respectively.

Table 3. Performance of the EE1-2.1.2 in RA

|  | BD-rate Over VTM-11.0_nnvc-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -4.16% | 2.31% | 2.40% | 79% | 56% |
| Class A2 | -1.86% | 10.86% | 9.16% | 87% | 75% |
| Class B | -0.08% | 1.72% | 1.05% | 99% | 91% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -1.23% | 3.21% | 2.66% | 93% | 81% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 100% |

Table 4. Performance of the EE1-2.1.2 in AI

|  | BD-rate Over VTM-11.0_nnvc-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -1.91% | 3.79% | 2.07% | 101% | 66% |
| Class A2 | -3.45% | 17.99% | 13.17% | 93% | 77% |
| Class B | 0.29% | 2.02% | 2.17% | 101% | 90% |
| Class C | 0.02% | 0.06% | 0.08% | 101% | 99% |
| Class E | 0.02% | 0.15% | 0.20% | 102% | 96% |
| Overall | -0.80% | 4.23% | 3.19% | 100% | 88% |
| Class D | 0.02% | 0.06% | 0.05% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 103% | 102% |

Table 5 and Table 6 show the performance of the EE1-2.1.3 under RA and AI configurations, respectively.

Table 5. Performance of the EE1-2.1.3 in RA

|  | BD-rate Over VTM-11.0_nnvc-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -4.17% | 2.11% | 2.02% | 79% | 55% |
| Class A2 | -1.84% | 9.81% | 8.19% | 87% | 74% |
| Class B | -0.08% | 1.59% | 0.94% | 99% | 91% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -1.23% | 2.91% | 2.36% | 93% | 81% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 100% |

Table 6. Performance of the EE1-2.1.3 in AI

|  | BD-rate Over VTM-11.0_nnvc-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -1.92% | 3.66% | 1.92% | 101% | 67% |
| Class A2 | -3.41% | 16.41% | 12.06% | 92% | 77% |
| Class B | 0.27% | 1.85% | 2.03% | 100% | 88% |
| Class C | 0.01% | 0.06% | 0.08% | 100% | 99% |
| Class E | 0.01% | 0.16% | 0.21% | 101% | 97% |
| Overall | -0.81% | 3.90% | 2.95% | 99% | 87% |
| Class D | 0.02% | 0.06% | 0.05% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 98% |

Conclusion

This contribution reports performance evaluation of an RPR encoder with four scale factors {×2.0, ×1.5, ×1.25, ×1.0}. The proposed method determines a scale factor depending on the PSNR value and initial QP for the first picture of each GOP. This contribution also proposes to use long-tap filter for post-filter in RPR scenarios. RPR functionality in VVC is used to compare coding benefits for NN-based up-sampling proposals in EE1 super resolution category. In addition, bitrate matching within 10% is required for fair comparison. The proposed adaptive resolution selection method could be satisfied to bitrate constraints and used to base software. Therefore, it is recommended to include the next version of NNVC software.

References

[1] J. Nam, S. Yoo, J. Lim, S. Kim, “EE1-2.1-related: RPR encoder with multiple scale factors”, JVET-Y0068, Meeting by Teleconference, January 2022.

[2] J. Nam, S. Yoo, J. Lim, S. Kim, “EE1-2.1: RPR encoder with multiple scale factors”, JVET-Z0065, Meeting by Teleconference, April 2022.

[3] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tags/NNVC-3.0

[4] E. Alshina, R-L. Liao, S. Liu, A. Segall,	“JVET common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-AB2016, Mainz DE, October 2022.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).