| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0137 |
| --- | --- |

| Title: | On Affine AMVR setting for VTM CTC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li | Email: | xlxiangli@google.com |
| Source: | Google, Inc. |  |  |

_____________________________

# Abstract

In the current VTM CTC, affine AMVR is enabled for RA but is disabled for LDB and LDP. It is reported that affine AMVR actually shows good tradeoff for LDP, i.e., 0.41% luma gain with 105% encoding time. It is proposed to turn on affine AMVR for LDP in VTM CTC.

# Introduction

Affine AMVR was proposed in JVET-M0247. It showed 0.32% luma gain with 108% encoding time for RA and 0.23% luma gain with 110% encoding time for LDB on top of VTM-3.0 [1]. No results were provided for LDP in the M meeting. As the tradeoff between coding efficiency and encoding time was not good for LDB, affine AMVR is only enabled for RA in VTM CTC [2].

# Test results

Affine AMVR is tested on top of VTM-19 commit ca4ab7e7 for LDB and LDP. The results are as follows.

|  |  |  |  | Low delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over vtm-19-20221223_ca4ab7e7 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.27% |  | -0.25% |  | -0.31% | 110% |  | 98% |  |
| Class C | -0.24% |  | -0.15% |  | -0.30% | 109% |  | 96% |  |
| Class E | -0.27% |  | -0.15% |  | 0.17% | 115% |  | 96% |  |
| Overall | -0.26% |  | -0.19% |  | -0.19% | 111% |  | 97% |  |
| Class D | -0.49% |  | -0.92% |  | -0.54% | 107% |  | 95% |  |
| Class F | -0.76% |  | -0.74% |  | -0.47% | 109% |  | 92% |  |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |
|  |  |  |  | Over vtm-19-20221223_ca4ab7e7 |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.42% |  | -0.82% |  | -0.48% | 105% |  | 98% |  |
| Class C | -0.49% |  | -0.69% |  | -0.51% | 105% |  | 95% |  |
| Class E | -0.30% |  | 0.68% |  | 0.06% | 106% |  | 93% |  |
| Overall | -0.41% |  | -0.40% |  | -0.35% | 105% |  | 96% |  |
| Class D | -1.04% |  | -1.39% |  | -1.60% | 104% |  | 95% |  |
| Class F | -0.45% |  | -0.52% |  | -0.72% | 103% |  | 96% |  |

For LDB, similar performance to what reported in JVET-M0247 is observed. While for LDP, affine AMVR shows 0.41% luma gain with 105% encoding time, which looks a good tradeoff. Therefore, it is proposed to enable affine AMVR for LDP in VTM CTC.

# Conclusion

Affine AMVR shows good tradoeff for LDP. It is proposed to enable affine AMVR for LDP in VTM CTC.

# Reference

[1] H. Liu, K. Zhang, L. Zhang, J. Xu, J. Luo, Y. He, X. Xiu, “CE2-related: Joint Test of AMVR for Affine Inter Mode (Test 2.1.1 and Test 2.1.2)”, JVET-M0247, Marrakech, MA, Jan. 2019.

[2] F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring, “VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video”, JVET-AB1100, Mainz, DE, Oct. 2022.

# Patent rights declaration(s)

Google, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.