| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0185-v2 |
| --- | --- |

| Title: | EE2-2.7: Enhanced temporal motion information derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA Lien-Fei Chen, Roman Chernyak, Xin Zhao, Xiaozhong Xu, Shan Liu 2747 Park Boulevard Palo Alto, CA 94306, U.S.A. | Tel: Email: | zhaolei.0324@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com lienfeichen@global.tencent.com |
| Source: | Bytedance Inc. Tencent |  |  |

_____________________________

Abstract

This proposal reports the performance of EE2-2.7a and EE2-2.7b. It is reported that on top of ECM-7.0, simulation results of EE2-2.7a and EE2-2.7b are as below:

Test 2.7a:

RA: -0.10 % (Y), -0.20 % (U), -0.24 % (V), 101% (EncT), 101% (DecT)

LB: -0.14 % (Y), -0.38 % (U), -0.22 % (V), 102% (EncT), 103% (DecT)

Test 2.7b:

RA: -0.11 % (Y), -0.22 % (U), -0.20 % (V), 101% (EncT), 102% (DecT)

LB: -0.11 % (Y), -0.42 % (U), -0.20 % (V), 103% (EncT), 104% (DecT)

- Introduction

In ECM, the temporal MVP for AMVP mode is derived by fetching the motion information from the center or the bottom-right of the collocated block. And a similar strategy is also applied to SbTMVP mode, where the motion information from the left neighboring position is used as a motion shift, which is then employed to obtain TMVPs at sub-CU level. It is asserted that such a design may not ensure the trajectory consistency between the pre-defined positions and the current CU.

- Proposed methods

In EE2-2.7, two aspects are proposed to further improve TMVP. Firstly, two collocated frames are utilized to provide temporal motion information. Secondly, the motion shift to locate TMVP is adaptively determined from multiple locations according to template costs.

Two reference frames with the least POC distance relative to the to-be-coded frame are selected to be the collocated frames. Two motion shift candidate lists are constructed respectively for the two collocated frames. The motion shifts with the minimum template matching cost are used to derive SbTMVP or TMVP candidates. At most 4 SbTMVP candidates are included in the sub-block-based merge list. The SbTMVP candidate with the least template matching cost derived from the first collocated frame is placed in the first entry without reordering, while other SbTMVP candidates are sorted together with AFFINE candidates.

The difference between EE2-2.7a and EE2-2.7b lies on the subblock template generation of SbTMVP. In EE2-2.7a, the prediction direction of each subblock template is determined based on the center subblock. As illustrated in Figure 1(a), if the center subblock is uni-predicted, then all the subblock templates are uni-predicted, and vice versa. If the motion vector of corresponding adjacent subblock at the determined reference list is not available for a subblock template, zero MV is used for that subblock template. In EE2-2.7b, the prediction direction and the motion vector of each subblock template is determined based on the corresponding adjacent subblock. As shown in Figure 1(b), when a subblock in template area has bi-prediction MV, the bi-prediction is applied to obtain the corresponding subblock reference template; When a subblock has uni-prediction MV, uni-prediction is applied to get the subblock reference template.

- EE2-2.7a                                            (b) EE2-2.7b

Figure 1: Example of subblock templates generation of SbTMVP in Test 2.7a and Test 2.7b

- Experimental results

Two experiments are tested on top of ECM-7.0 under the common test conditions [1], which are listed as follows:

Test 2.7a: Enhanced temporal motion information derivation

Test 2.7b: Enhanced temporal motion information derivation with SbTMVP template matching using adjacent subblock MVs

Table 1: Coding performance of Test 2.7a.

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.28% | -0.21% | 101% | 100% |  |  |  |  |  |
| Class A2 | -0.08% | -0.14% | -0.20% | 101% | 101% |  |  |  |  |  |
| Class B | -0.08% | -0.09% | -0.30% | 101% | 102% | -0.12% | -0.46% | -0.64% | 103% | 103% |
| Class C | -0.16% | -0.34% | -0.22% | 102% | 103% | -0.26% | -0.40% | -0.28% | 103% | 105% |
| Class E |  |  |  |  |  | -0.02% | -0.20% | 0.57% | 101% | 101% |
| Overall | -0.10% | -0.20% | -0.24% | 101% | 101% | -0.14% | -0.38% | -0.22% | 102% | 103% |
| Class D | -0.13% | -0.24% | -0.20% | 104% | 103% | -0.02% | -0.51% | -0.79% | 104% | 103% |
| Class F | -0.06% | -0.02% | 0.08% | 102% | 99% | -0.10% | 0.22% | 0.17% | 103% | 102% |

Table 2: Coding performance of Test 2.7b.

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.35% | -0.22% | 101% | 101% |  |  |  |  |  |
| Class A2 | -0.08% | -0.20% | -0.17% | 101% | 100% |  |  |  |  |  |
| Class B | -0.08% | -0.11% | -0.23% | 102% | 103% | -0.14% | -0.35% | -0.44% | 103% | 103% |
| Class C | -0.18% | -0.30% | -0.17% | 102% | 104% | -0.17% | -0.31% | -0.04% | 104% | 106% |
| Class E |  |  |  |  |  | 0.04% | -0.65% | -0.02% | 101% | 104% |
| Overall | -0.11% | -0.22% | -0.20% | 101% | 102% | -0.11% | -0.42% | -0.20% | 103% | 104% |
| Class D | -0.12% | -0.29% | -0.21% | 102% | 103% | -0.01% | -0.73% | 0.10% | 102% | 106% |
| Class F | -0.06% | -0.01% | 0.05% | 101% | 102% | -0.11% | -0.17% | -0.23% | 101% | 103% |

- Conclusions

This proposal reports the performance of EE2-2.7a and EE2-2.7b. Simulation results indicate promising coding gain in RA/LDB configurations with acceptable computational complexity. It is recommended to adopt this contribution in ECM.

- References

[1] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).