| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 |  |  | Document: JVET-AC0199-v1 |  |
| --- | --- | --- | --- | --- |
| Title: | AHG13: On Film Grain Synthesis Subjective Evaluation |  |  |  |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Informative Report |  |  |  |
| Author(s) or Contact(s): | Xuewei Meng, Wenhao Zhang, Scott Labrozzi | Email: |  | xuewei.meng@disneystreaming.com wenhao.zhang@disneystreaming.com Scott.Labrozzi@disneystreaming.com |
| Source: | Disney Streaming |  |  |  |

_____________________________

# Abstract

The subjective evaluation of film grain synthesis is recommended in the last JVET meeting. This document provides the film grain synthesis evaluation results of both the frequency-based method and the auto-regressive based method. The main observations are as follows,

- In the VTM-19.0, the film grain synthesis (FGS) denoiser doesn’t change the input of the encoder.

  - A typical (FGS) framework includes denoising, noise modeling and grain synthesis. In the VTM-19.0, the effect of bitrate saving can’t be evaluated.

- The level of synthesized film grain changes regularly in the temporal domain.

  - The film grain intensity ramps up every fourth frame. The main reason may be the MCTF and QP adjustment.

- For videos with relative heavy film grain,

  - The synthesized grain is coarse-grained, and there are obvious repetitive patterns. Parameters are recommended to control the granularity of synthesized film grain.

  - The FGS is turned off in some frames with heavy grain. This may due to the denoiser or noise modeling method. More test videos are recommended to cover different content and film grain level.

- For videos with low film grain, the synthesized grain looks fine.

- The frequency based FGS method and auto-regressive (AR) based method show different results under different circumstances.

  - How to select the proper method for a film grain test video is important.

# 1. 	Introduction

In the entertainment industry, film grain is widely present in TV and motion pictures. Although digital cameras do not produce film grain, film grain is often added to the captured videos for creative intent. When encoding these TV or motion picture videos, preserving film grain under limited bandwidth is a challenging task. To address this issue, film grain synthesis methods are proposed in video coding. A typical film grain framework can be described as shown in Fig. 1.

Figure 1. A typical framework of Film Grain Synthesis.

In the current VTM-19.0, the frequency-based film grain synthesis (FGS) method is included. And the current FGS in VTM-19.0 can be described as shown in Fig. 2, in which the de-graining module in FGS has no impact on the encoding process.

Figure 2. The framework of Film Grain Synthesis in the VTM-19.0.

In this proposal, we evaluate the subjective results of film grain methods, including the frequency-based method, and the auto-regressive(AR) based method based on the FGS framework in VTM-19.0. In the evaluation the denoiser and the encoding process are the same.

# 2. 	Test Conditions

## 2.1 Test Platform

Anchor: VTM-19.0

Film grain synthesis methods:

- Frequency-domain film grain synthesis method in VTM-19.0

- Auto-regressive model

## 2.2 Test Sequences and Configuration

Test configuration: Random Access (RA) with GOP32

Test QP: 22, 27, 32, 37

Table 1. Test sequences.

|  | Sequences | Resolution | Bitdepth | Frame rate | Chroma Format | Frame Number |
| --- | --- | --- | --- | --- | --- | --- |
| JVET | oldTownCross | 1920x1080 | 8 | 50 | YUV420 | 500 |
|  | DinnerScene | 4096x2160 | 10 | 60 | YUV420 | 600 |
| Disney | PrinceOfPersia | 1920x1080 | 10 | 24 | YUV420 | 240 |
|  | StarwarESB | 3840x2160 | 10 | 24 | YUV420 | 240 |
|  | TheMandalorian | 3840x2160 | 10 | 24 | YUV420 | 240 |
|  | TheHandmaid | 3840x2160 | 10 | 24 | YUV420 | 240 |

The examples of sequence “oldTownCross” and “DinnerScene” are shown in Fig. 3 and Fig. 4. The video content of sequences from Disney will be available when Disney Legal and Copyright approves.

Figure 3. A frame of test sequence “oldTownCross”.

Figure 4. A frame of test sequence “DinnerScene”.

## 2.3 Frequency-domain film grain synthesis configuration

In this proposal, we evaluate the frequency-domain film grain synthesis implemented in the current VTM-19.0. And the parameters are set as shown in Table 2.

Table 2. Frequency-domain film grain synthesis configuration.

| Parameters | Settings |
| --- | --- |
| SEIFGCEnabled | 1 |
| SEIFGCAnalysisEnabled | 1 |
| SEIFGCCancelFlag | 0 |
| SEIFGCPersistenceFlag | 1 |
| SEIFGCPerPictureSEI | 0 |
| SEIFGCModelID | 0 |
| SEIFGCSepColourDescPresentFlag | 0 |
| SEIFGCBlendingModeID | 0 |
| SEIFGCCompModelPresentComp0 | 1 |
| SEIFGCCompModelPresentComp1 | 0 |
| SEIFGCCompModelPresentComp2 | 0 |

## 2.4 Auto-regressive film grain synthesis configuration

The auto-regressive film grain synthesis is also evaluated in the VTM-19.0. And the parameters are set as shown in Table 3.

Table 3. Auto-regressive film grain synthesis configuration.

| Parameters | Settings |
| --- | --- |
| Block_size in flat block detection | 32x32 |
| Pattern size | 64x64 |

# 3. 	Evaluation Results

oldTownCross

Observation1: The synthesized grain of frequency-based FGS is coarse-grained, and there are obvious repetitive patterns. The synthesized grain of AR-based FGS is fine-grained and the grain level is lower than the original video, as shown in Fig. 5.

(a) Original video   (b) Frequency-based FGS   (c) AR-based FGS   (d) VTM-19.0 without FGS

Figure 5. Subjective results of oldTownCross. (QP = 22, frameIdx = 160)

Observation2: The film grain level of the original video is stable. But the synthesized grain level is not stable, frameIdx=20, 24, 28 is higher, other frames is lower. VTM-19.0 without FGS shows the similar results (frameIdx=20, 24, 28 have more grain noise), as shown in the slides (Page 17 to 24).

DinnerScene

Observation1:The generated grain in light area is coarse. In the dark area, there is almost no grain generated. And the grain in the source video is more colorful, as shown in Fig. 6.

- Original video                                                       (b) Frequency-based FGS

(c) VTM-19.0 without FGS

Figure 6. Subjective results of DinnerScene. (QP = 22, frameIdx = 239)

PrinceOfPersia (Picture and thumbnails will be available when Disney Legal and Copyright approves.)

Observation1: The film grain level is very high in the source video. But film grain synthesis is turned off in some frames.

Observation2: The film grain level of the original video is stable. But the synthesized grain level is not stable, frameIdx=44, 48 is higher, other frames is lower. VTM-19.0 without FGS shows the similar results (frameIdx=44,48 have more grain noise).

TheMandalorian (Picture and thumbnails will be available when Disney Legal and Copyright approves.)

Observation1: The grain of frequency-based FGS is heavy and coarse-grained. And the grain of AR FGS is low and fine-grained. The synthesized film grain can hide encoding artifact.

TheHandmaids (Picture and thumbnails will be available when Disney Legal and Copyright approves.)

Observation1: The level of film grain in the source video is low. The synthesized film grain is lower than the original video, but looks fine.

# 4. 	Observations and Conclusions

- In the VTM-19.0, the film grain synthesis (FGS) denoiser doesn’t change the input of the encoder.

  - A typical (FGS) framework includes denoising, noise modeling and grain synthesis. In the VTM-19.0, the effect of bitrate saving can’t be evaluated.

- The level of synthesized film grain changes regularly in the temporal domain.

  - The film grain intensity ramps up every fourth frame. The main reason may be the MCTF and QP adjustment.

- For videos with relative heavy film grain,

  - The synthesized grain is coarse-grained, and there are obvious repetitive patterns. Parameters are recommended to control the granularity of synthesized film grain.

  - The FGS is turned off in some frames with heavy grain. This may due to the denoiser or noise modeling method. More test videos are recommended to cover different content and film grain level.

- For videos with low film grain, the synthesized grain looks fine.

- The frequency based FGS method and auto-regressive (AR) based method show different results under different circumstances.

  - How to select the proper method for a film grain test video is important.

# 5. 	Patent rights declaration(s)

Disney Streaming, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.