| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0007-v2 |
| --- | --- |

| Title: | AHG report: ECM tool assessment (AHG7) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xiang Li (chair) Lien-Fei Chen Zhipin Deng Jonathan Gan Edouard François Hong-Jheng Jhu Ru-Ling Liao Hongtao Wang (vice chairs) | Tel: Email: | xlxiangli@google.com lienfeichen@global.tencent.com zhipin.deng@bytedance.com v-jonathan.gan@oppo.com Edouard.Francois@interdigital.com jhuhong-jheng@kwai.com ruling.lrl@alibaba-inc.com hongtaow@qti.qualcomm.com |
| Source: | AhG7 on ECM tool assessment |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AHG7 on ECM tool assessment that has taken place between the 29th and 30th JVET meetings.

# Introduction

The mandates given to the AHG are:

- Discuss JVET-AC0354, and prepare an updated document as input to the next meeting on the methodology of tool assessment, tool grouping and a list of tools in each group to be tested.

- Investigate missing tool controls in ECM, and suggest appropriate modifications to software in coordination with AHG6.

- Prepare configuration files and generate bitstreams and results of tool-on/tool-off testing.

- Prepare reporting of tool assessment results.

# AhG meeting summary

The AhG meeting was held on Feb. 17, 2023. The meeting report was uploaded as JVET-AD0041. The agreed items during the meeting are summarized as below.

- ECM tool grouping

  - Group 1: Inter template matching tools

  - Group 2: Coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process

  - Group 3: Intra and IBC template matching (with search) related tools

  - Group 4: Tools that require more processing on the neighboring reconstructed samples than VVC

  - Group 5: Remaining tools

- Tool grouping list

| Group | Tools |
| --- | --- |
| 1 | TM, TM OBMC, ARMC-TM, TM GPM, TM-Merge, TM-AMVP, TM-MMBD, block level reference picture list reordering, TM based BCW index derivation, MVD sign prediction |
| 2 | LIC, CIIP with TIMD and TM merge, coefficient sign prediction |
| 3 | TM IBC, IBC BVD prediction, IBC BVP clustering, intra TMP, TIMD, TMRL, SGPM, Direct BV for chroma prediction |
| 4 | DIMD, CCCM |
| 5 | Remaining tools |

- The missing cfg options of tools in group 1-4 are as below. The cfg options of tools in group5 are to be further identified. Note that the missing control for TM IBC and TMRL affected testing for Group 3 off and Group 1-4 off. The controls of TM based BCW, CIIP with TM merge and IBC BVP clustering have been added to ECM 8.1 and included in the following tests.

  - TM based BCW index derivation, CIIP with TM merge, TM IBC, IBC BVP clustering, TMRL

- Testing configurations

  - ECM anchor: ECM-8.1

  - VTM anchor: VTM11-ecm8.1

  - Configurations: AI, RA, LDB (with TGM class), do not test LDB for group 3 and 4

  - Cfg options are used to disable tools

  - Test each group on their own, i.e., anchor vs anchor + disable tool group x (1-5)

# Issues during testing

During the testing, several issues were identified.

ECM-8 decoding mismatch was observed for SCC sequences during tool off tests. In addition, a tool control issue and several compiling issues were reported. These issues which blocked the tool off tests were resolved in a short time. We thank the effort from the proponents and AHG6 to resolve these issues.

Minor issues on test setting and bitrate/PSNR calculating were observed and resolved during the crosschecking.

There are still open software issues related to tool off tests, such as issue #27, #36, #42, #43 (https://vcgit.hhi.fraunhofer.de/groups/ecm/-/issues). These issues didn’t block AHG7 tests. But it would be good to have them fixed.

# Results of group off tests

## Test anchor and crosschecking

As ECM-8.0 has software issues when some tools are disabled, ECM-8.1 is used as the first anchor for the group off tests. The second anchor is VTM-11ecm8.1. As the second anchor data is directly copied from AHG6 report, the runtime info is removed to avoid confusion.

The testers and crosscheckers are summarized in the table below. All the tests below have been crosschecked.

| Tests | Tester | Crosschecker |
| --- | --- | --- |
| Group 1 off | Jonathan Gan (v-jonathan.gan@oppo.com) | Charles Salmon-Legagneur (charles.salmon-legagneur@interdigital.com) |
| Group 2 off | Jonathan Gan (v-jonathan.gan@oppo.com) | Ru-Ling Liao (ruling.lrl@alibaba-inc.com) |
| Group 3 off | Xiang Li (xlxiangli@google.com) | Zhipin Deng (zhipin.deng@bytedance.com) |
| Group 4 off | Xiang Li (xlxiangli@google.com) | Hong-Jheng Jhu (jhuhong-jheng@kwai.com) |
| Group 1-4 off | Xiang Li (xlxiangli@google.com) | Lien-Fei Chen (lienfei.chen@global.tencent.com), Hongtao Wang (hongtaow@qti.qualcomm.com) |

## Group 1 off

Group 1 includes inter template matching tools.

The command line setting of this test is: --EnableTMTools=0

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% | -9.04% | -18.79% | -25.18% | #DIV/0! | #DIV/0! |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% | -13.53% | -22.78% | -25.89% | #DIV/0! | #DIV/0! |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% | -8.31% | -21.80% | -20.07% | #DIV/0! | #DIV/0! |
| Class C | -0.01% | -0.01% | -0.01% | 100% | 100% | -9.23% | -14.01% | -14.35% | #DIV/0! | #DIV/0! |
| Class E | -0.01% | -0.01% | -0.01% | 100% | 100% | -10.50% | -18.87% | -17.71% | #DIV/0! | #DIV/0! |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 100% | -9.87% | -19.24% | -20.23% | #DIV/0! | #DIV/0! |
| Class D | -0.03% | -0.02% | -0.02% | 100% | 100% | -7.53% | -11.50% | -11.55% | #DIV/0! | #DIV/0! |
| Class F | 0.32% | 0.24% | 0.74% | 100% | 97% | -21.97% | -30.46% | -29.87% | #DIV/0! | #DIV/0! |
| Class TGM | 1.48% | 1.45% | 1.48% | 100% | 88% | -34.68% | -42.11% | -41.45% | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 3.63% | 4.06% | 4.42% | 89% | 84% | -16.99% | -19.85% | -26.72% | #DIV/0! | #DIV/0! |
| Class A2 | 3.93% | 4.25% | 4.61% | 86% | 78% | -19.33% | -26.63% | -29.84% | #DIV/0! | #DIV/0! |
| Class B | 3.40% | 3.66% | 3.62% | 86% | 78% | -15.47% | -26.52% | -24.41% | #DIV/0! | #DIV/0! |
| Class C | 3.94% | 4.12% | 4.39% | 81% | 70% | -16.77% | -20.48% | -20.12% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 3.70% | 3.98% | 4.18% | 85% | 77% | -16.89% | -23.60% | -24.81% | #DIV/0! | #DIV/0! |
| Class D | 3.23% | 3.79% | 3.80% | 80% | 65% | -18.15% | -22.11% | -22.24% | #DIV/0! | #DIV/0! |
| Class F | 2.77% | 2.74% | 2.67% | 90% | 82% | -22.80% | -30.73% | -30.86% | #DIV/0! | #DIV/0! |
| Class TGM | 4.47% | 4.45% | 4.39% | 91% | 82% | -30.30% | -35.90% | -36.18% | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 4.18% | 4.84% | 5.12% | 77% | 69% | -12.85% | -27.67% | -25.87% | #DIV/0! | #DIV/0! |
| Class C | 4.43% | 4.86% | 5.46% | 75% | 60% | -14.03% | -22.60% | -22.14% | #DIV/0! | #DIV/0! |
| Class E | 4.28% | 5.75% | 5.37% | 78% | 78% | -11.04% | -20.42% | -19.67% | #DIV/0! | #DIV/0! |
| Overall (Ref) | 4.29% | 5.07% | 5.30% | 77% | 68% | -12.79% | -24.17% | -23.08% | #DIV/0! | #DIV/0! |
| Class D | 3.73% | 3.63% | 4.80% | 73% | 58% | -16.80% | -24.23% | -23.72% | #DIV/0! | #DIV/0! |
| Class F | 3.63% | 3.04% | 2.93% | 87% | 75% | -19.17% | -28.87% | -27.57% | #DIV/0! | #DIV/0! |
| Class TGM | 5.98% | 6.05% | 5.85% | 88% | 78% | -28.11% | -34.34% | -34.26% | #DIV/0! | #DIV/0! |

## Group 2 off

Group 2 includes coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process.

The command line setting of this test is: --LIC=0 --CIIPTM=0 --CIIPTIMD=0 --NumSignPred=0 --IBCLIC=0

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.45% | 0.71% | 0.73% | 87% | 91% | -8.62% | -18.26% | -24.61% | #DIV/0! | #DIV/0! |
| Class A2 | 0.50% | 0.59% | 0.70% | 84% | 91% | -13.09% | -22.32% | -25.37% | #DIV/0! | #DIV/0! |
| Class B | 0.45% | 0.58% | 0.63% | 83% | 89% | -7.89% | -21.35% | -19.56% | #DIV/0! | #DIV/0! |
| Class C | 0.37% | 0.44% | 0.50% | 80% | 87% | -8.89% | -13.62% | -13.92% | #DIV/0! | #DIV/0! |
| Class E | 0.66% | 0.82% | 0.91% | 83% | 90% | -9.90% | -18.24% | -16.96% | #DIV/0! | #DIV/0! |
| Overall | 0.47% | 0.61% | 0.68% | 83% | 89% | -9.44% | -18.76% | -19.68% | #DIV/0! | #DIV/0! |
| Class D | 0.30% | 0.33% | 0.48% | 78% | 85% | -7.23% | -11.19% | -11.11% | #DIV/0! | #DIV/0! |
| Class F | 2.47% | 2.44% | 2.86% | 88% | 92% | -20.39% | -29.10% | -28.55% | #DIV/0! | #DIV/0! |
| Class TGM | 0.49% | 0.56% | 0.56% | 88% | 98% | -35.30% | -42.63% | -41.99% | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 2.14% | 1.29% | 1.77% | 89% | 98% | -18.18% | -21.79% | -28.57% | #DIV/0! | #DIV/0! |
| Class A2 | 1.60% | 1.06% | 1.56% | 89% | 97% | -21.14% | -28.86% | -31.85% | #DIV/0! | #DIV/0! |
| Class B | 1.50% | 1.05% | 1.23% | 88% | 96% | -17.03% | -28.35% | -26.17% | #DIV/0! | #DIV/0! |
| Class C | 1.14% | 0.70% | 0.92% | 86% | 95% | -19.05% | -23.14% | -22.82% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.55% | 1.01% | 1.32% | 88% | 96% | -18.62% | -25.75% | -26.89% | #DIV/0! | #DIV/0! |
| Class D | 0.71% | 0.63% | 0.52% | 85% | 94% | -20.18% | -24.57% | -24.82% | #DIV/0! | #DIV/0! |
| Class F | 2.91% | 2.39% | 2.72% | 91% | 95% | -22.81% | -31.17% | -31.01% | #DIV/0! | #DIV/0! |
| Class TGM | 1.52% | 1.39% | 1.50% | 90% | 99% | -32.21% | -37.78% | -37.91% | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 1.57% | 0.68% | 1.11% | 86% | 95% | -15.04% | -30.43% | -28.52% | #DIV/0! | #DIV/0! |
| Class C | 1.08% | 1.16% | 1.27% | 85% | 95% | -16.82% | -25.36% | -25.26% | #DIV/0! | #DIV/0! |
| Class E | 1.10% | 2.15% | 0.47% | 87% | 95% | -13.73% | -23.05% | -23.41% | #DIV/0! | #DIV/0! |
| Overall (Ref) | 1.29% | 1.21% | 1.00% | 86% | 95% | -15.31% | -26.90% | -26.16% | #DIV/0! | #DIV/0! |
| Class D | 0.59% | 0.17% | 1.21% | 85% | 96% | -19.33% | -26.86% | -26.34% | #DIV/0! | #DIV/0! |
| Class F | 2.64% | 1.10% | 1.12% | 90% | 97% | -20.03% | -30.26% | -28.94% | #DIV/0! | #DIV/0! |
| Class TGM | 1.68% | 1.70% | 1.86% | 92% | 99% | -31.01% | -37.15% | -36.76% | #DIV/0! | #DIV/0! |

## Group 3 off

Group 3 includes intra and IBC template matching (with search) related tools. Note that the control of TM IBC and TMRL are still missing. The tests were performed with these two tools on.

The command line setting of this test is: --BvdPred=0 --IntraDBV=0 --BvpCluster=0 --IntraTMP=0 --TIMD=0 --SGPM=0

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.45% | 0.29% | 0.35% | 83% | 93% | -8.62% | -18.55% | -24.91% | #DIV/0! | #DIV/0! |
| Class A2 | 1.03% | 1.10% | 1.19% | 84% | 88% | -12.59% | -21.88% | -24.97% | #DIV/0! | #DIV/0! |
| Class B | 1.15% | 1.04% | 1.23% | 83% | 86% | -7.24% | -20.96% | -19.06% | #DIV/0! | #DIV/0! |
| Class C | 0.95% | 0.78% | 0.80% | 84% | 83% | -8.36% | -13.33% | -13.64% | #DIV/0! | #DIV/0! |
| Class E | 1.89% | 1.88% | 1.88% | 84% | 84% | -8.80% | -17.33% | -16.14% | #DIV/0! | #DIV/0! |
| Overall | 1.09% | 1.01% | 1.09% | 84% | 86% | -8.87% | -18.41% | -19.33% | #DIV/0! | #DIV/0! |
| Class D | 0.72% | 0.38% | 0.53% | 85% | 82% | -6.84% | -11.13% | -11.06% | #DIV/0! | #DIV/0! |
| Class F | 1.79% | 1.69% | 2.05% | 92% | 92% | -20.84% | -29.45% | -28.95% | #DIV/0! | #DIV/0! |
| Class TGM | 4.10% | 4.52% | 4.91% | 93% | 98% | -33.05% | -40.44% | -39.56% | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.26% | 0.13% | 0.39% | 96% | 99% | -19.67% | -22.70% | -29.57% | #DIV/0! | #DIV/0! |
| Class A2 | 0.54% | 0.22% | 0.71% | 96% | 99% | -21.96% | -29.46% | -32.43% | #DIV/0! | #DIV/0! |
| Class B | 0.61% | 0.40% | 0.55% | 95% | 99% | -17.76% | -28.83% | -26.67% | #DIV/0! | #DIV/0! |
| Class C | 0.40% | 0.30% | 0.63% | 96% | 98% | -19.63% | -23.42% | -23.01% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 0.47% | 0.28% | 0.57% | 96% | 99% | -19.48% | -26.29% | -27.43% | #DIV/0! | #DIV/0! |
| Class D | 0.38% | 0.24% | 0.37% | 96% | 98% | -20.45% | -24.87% | -24.96% | #DIV/0! | #DIV/0! |
| Class F | 1.01% | 1.14% | 0.96% | 97% | 99% | -24.22% | -31.97% | -32.16% | #DIV/0! | #DIV/0! |
| Class TGM | 2.04% | 2.28% | 2.40% | 96% | 100% | -31.92% | -37.31% | -37.43% | #DIV/0! | #DIV/0! |

## Group 4 off

Group 4 includes tools that require more processing on the neighboring reconstructed samples than VVC.

The command line setting of this test is: --DIMD=0 --CCCM=0

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 1.53% | 4.43% | 5.96% | 88% | 96% | -7.64% | -15.40% | -20.83% | #DIV/0! | #DIV/0! |
| Class A2 | 2.41% | 4.37% | 4.80% | 87% | 98% | -11.55% | -19.47% | -22.51% | #DIV/0! | #DIV/0! |
| Class B | 1.00% | 4.75% | 3.80% | 88% | 97% | -7.39% | -18.37% | -17.18% | #DIV/0! | #DIV/0! |
| Class C | 0.87% | 2.21% | 1.97% | 87% | 98% | -8.44% | -12.20% | -12.72% | #DIV/0! | #DIV/0! |
| Class E | 1.11% | 4.73% | 2.84% | 88% | 97% | -9.50% | -15.26% | -15.52% | #DIV/0! | #DIV/0! |
| Overall | 1.31% | 4.07% | 3.76% | 87% | 97% | -8.71% | -16.17% | -17.41% | #DIV/0! | #DIV/0! |
| Class D | 0.79% | 1.79% | 1.75% | 87% | 98% | -6.77% | -9.91% | -9.99% | #DIV/0! | #DIV/0! |
| Class F | 1.15% | 2.95% | 2.87% | 93% | 96% | -21.31% | -28.65% | -28.46% | #DIV/0! | #DIV/0! |
| Class TGM | 1.74% | 2.69% | 2.84% | 92% | 98% | -34.53% | -41.48% | -40.76% | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 1.36% | 2.96% | 4.83% | 93% | 99% | -18.78% | -20.62% | -26.72% | #DIV/0! | #DIV/0! |
| Class A2 | 1.13% | 2.55% | 3.37% | 94% | 99% | -21.50% | -27.88% | -30.73% | #DIV/0! | #DIV/0! |
| Class B | 0.58% | 4.28% | 3.64% | 93% | 99% | -17.79% | -26.18% | -24.55% | #DIV/0! | #DIV/0! |
| Class C | 0.34% | 1.41% | 1.39% | 95% | 100% | -19.68% | -22.56% | -22.43% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 0.78% | 2.91% | 3.22% | 94% | 99% | -19.23% | -24.45% | -25.65% | #DIV/0! | #DIV/0! |
| Class D | 0.35% | 2.04% | 1.32% | 96% | 100% | -20.47% | -23.43% | -24.25% | #DIV/0! | #DIV/0! |
| Class F | 0.61% | 1.96% | 1.60% | 97% | 100% | -24.50% | -31.39% | -31.66% | #DIV/0! | #DIV/0! |
| Class TGM | 0.53% | 1.09% | 1.02% | 94% | 100% | -32.93% | -37.99% | -38.25% | #DIV/0! | #DIV/0! |

## Group 5 off

Group 5 includes all coding tools which are not in group 1-4. Unfortunately, many tools in this group cannot be switched off with cfg options. Considering these limitations, the tools off test of group 5 was not performed.

## Group 1-4 off

This test was not discussed during the AHG interim meeting. As no progress was achieved for group 5 off test, this test was suggested.

The command line setting of this test is: --EnableTMTools=0 --LIC=0 --CIIPTM=0 --CIIPTIMD=0 --NumSignPred=0 --IBCLIC=0 --BvdPred=0 --IntraDBV=0 --BvpCluster=0 --IntraTMP=0 --TIMD=0 --SGPM=0 --DIMD=0 --CCCM=0

Note that the control of TM IBC and TMRL are still missing. The tests were performed with these two tools on.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 2.72% | 5.93% | 7.43% | 61% | 81% | -6.55% | -14.21% | -19.69% | #DIV/0! | #DIV/0! |
| Class A2 | 4.20% | 6.34% | 6.84% | 58% | 75% | -9.95% | -17.91% | -20.96% | #DIV/0! | #DIV/0! |
| Class B | 2.83% | 6.57% | 5.78% | 58% | 70% | -5.71% | -16.93% | -15.57% | #DIV/0! | #DIV/0! |
| Class C | 2.50% | 3.63% | 3.52% | 56% | 64% | -6.97% | -10.97% | -11.38% | #DIV/0! | #DIV/0! |
| Class E | 4.13% | 7.65% | 5.95% | 59% | 70% | -6.79% | -12.87% | -12.95% | #DIV/0! | #DIV/0! |
| Overall | 3.18% | 5.95% | 5.76% | 58% | 71% | -7.02% | -14.64% | -15.79% | #DIV/0! | #DIV/0! |
| Class D | 2.15% | 3.11% | 3.11% | 56% | 60% | -5.52% | -8.74% | -8.79% | #DIV/0! | #DIV/0! |
| Class F | 5.85% | 7.74% | 7.60% | 76% | 75% | -17.79% | -25.61% | -25.40% | #DIV/0! | #DIV/0! |
| Class TGM | 9.12% | 10.07% | 10.93% | 76% | 81% | -29.90% | -37.36% | -36.20% | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 7.85% | 9.43% | 12.35% | 69% | 81% | -13.60% | -15.79% | -21.40% | #DIV/0! | #DIV/0! |
| Class A2 | 7.55% | 9.08% | 10.44% | 68% | 76% | -16.51% | -23.31% | -26.02% | #DIV/0! | #DIV/0! |
| Class B | 6.31% | 9.83% | 9.65% | 66% | 74% | -13.07% | -22.23% | -20.12% | #DIV/0! | #DIV/0! |
| Class C | 6.11% | 7.38% | 7.61% | 62% | 65% | -15.02% | -17.98% | -17.62% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 6.81% | 8.95% | 9.80% | 66% | 73% | -14.39% | -20.03% | -20.89% | #DIV/0! | #DIV/0! |
| Class D | 4.89% | 6.34% | 6.41% | 62% | 61% | -16.80% | -20.21% | -20.31% | #DIV/0! | #DIV/0! |
| Class F | 7.67% | 8.82% | 8.71% | 76% | 78% | -19.25% | -26.86% | -26.97% | #DIV/0! | #DIV/0! |
| Class TGM | 9.51% | 10.19% | 10.44% | 77% | 82% | -26.93% | -32.44% | -32.52% | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 6.25% | 7.49% | 8.35% | 60% | 63% | -11.10% | -25.86% | -23.62% | #DIV/0! | #DIV/0! |
| Class C | 6.11% | 6.82% | 7.45% | 58% | 53% | -12.65% | -21.18% | -20.70% | #DIV/0! | #DIV/0! |
| Class E | 5.76% | 10.68% | 8.79% | 63% | 73% | -9.76% | -16.83% | -17.10% | #DIV/0! | #DIV/0! |
| Overall (Ref) | 6.08% | 8.06% | 8.16% | 60% | 62% | -11.29% | -22.04% | -21.02% | #DIV/0! | #DIV/0! |
| Class D | 4.71% | 5.31% | 7.21% | 58% | 52% | -15.99% | -22.98% | -21.93% | #DIV/0! | #DIV/0! |
| Class F | 7.48% | 8.07% | 7.73% | 73% | 70% | -16.24% | -25.50% | -24.25% | #DIV/0! | #DIV/0! |
| Class TGM | 10.15% | 10.71% | 11.08% | 79% | 75% | -25.27% | -31.58% | -31.10% | #DIV/0! | #DIV/0! |

## Result summary

The average results compared to ECM-8.1are summarized in the table below. Note that SCC results are not included.

| AI | PRSNY | PSNRU | PSNRV | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Off1 | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Off2 | 0.47% | 0.61% | 0.68% | 83% | 89% |
| Off3 | 1.09% | 1.01% | 1.09% | 84% | 86% |
| Off4 | 1.31% | 4.07% | 3.76% | 87% | 97% |
| Test1 (off1-4) | 3.18% | 5.95% | 5.76% | 58% | 71% |
| RA | PRSNY | PSNRU | PSNRV | EncT | DecT |
| Off1 | 3.70% | 3.98% | 4.18% | 85% | 77% |
| Off2 | 1.55% | 1.01% | 1.32% | 88% | 96% |
| Off3 | 0.47% | 0.28% | 0.57% | 96% | 99% |
| Off4 | 0.78% | 2.91% | 3.22% | 94% | 99% |
| Test1 (off1-4) | 6.81% | 8.95% | 9.80% | 66% | 73% |
| LDB | PRSNY | PSNRU | PSNRV | EncT | DecT |
| Off1 | 4.29% | 5.07% | 5.30% | 77% | 68% |
| Off2 | 1.29% | 1.21% | 1.00% | 86% | 95% |
| Test1 (off1-4) | 6.08% | 8.06% | 8.16% | 60% | 62% |

## Open Questions

The following questions were raised during AHG teleconference and testing.

- How to further improve the methodology of tool grouping and assessment?

- Should cfg controls for tools in group 5 be added? If not, how to assess tools in group 5?

- Should the visual quality of group off tests be evaluated?

# Input contributions

| JVET-AD0041 | m62485 | 2023-02-27 18:21:13 | 2023-02-27 22:19:31 | 2023-02-27 22:19:31 | AHG7: Report on AHG meeting on ECM tool assessment | X. Li |
| --- | --- | --- | --- | --- | --- | --- |
| JVET-AD0082 | m62708 | 2023-04-14 05:52:30 | 2023-04-14 19:59:34 | 2023-04-14 19:59:34 | [AHG7] A configuration option for TMRL mode | L. Xu, J. Gan, Y. Yu, H. Yu, D. Wang(OPPO) |
| JVET-AD0131 | m62775 | 2023-04-14 12:37:51 | 2023-04-14 21:48:52 | 2023-04-14 21:48:52 | AhG 7: Coding efficiency/runtime tradeoff metrics | P. Onno, G. Laroche (Canon) |
| JVET-AD0229 | m62914 | 2023-04-15 00:15:43 | 2023-04-15 00:19:30 | 2023-04-17 07:34:52 | AHG4/AHG12/AHG7: Mixed-Content Sequences for ECM CTC or Tool Assessment | X. Li (Google), Z. Deng, K. Zhang, L. Zhang (Bytedance) |
| JVET-AD0246 | m62973 | 2023-04-17 02:10:57 | 2023-04-17 03:19:18 | 2023-04-17 03:19:18 | [AHG7] ECM tool assessment with HDR dataset | Z. Fan, Y. Yausig, T. Chujoh, T. Ikai (Sharp) |

# Recommendations

- Continue and improve tool assessment

- Given the existing and potential software issues related to tool assessment, it is recommended to add a new mandate “Coordinate with AHG6 on resolving tool assessment related ECM software issues”

- Discuss the open questions

- Review all the input contributions