| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0023-v1 |
| --- | --- |

| Title: | EE2: Summary Report on Enhanced Compression beyond VVC capability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Roman Chernyak Karam Naser Jacob Ström Fan Wang Martin Winken Xiaoyu Xiu Kai Zhang | Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com chernyak@tencent.com Karam.Naser@InterDigital.com jacob.strom@ericsson.com wangfan6@oppo.com martin.winken@hhi.fraunhofer.de xiaoyuxiu@kwai.com zhangkai.video@bytedance.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This document provides a summary report of Exploration Experiment on Enhanced Compression beyond VVC capability. The tests are categorized as intra prediction, inter prediction, screen content coding, transform, and in-loop filtering.

The software basis for this EE is ECM-8.0, released at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-8.0. ECM-8.0 is used as an anchor in the tests.

Software for EE tests is released in the corresponding branches at https://vcgit.hhi.fraunhofer.de/ecm/jvet-ac-ee2/ECM/-/branches.

Test results can be found in input JVET contributions, cross-check results are uploaded to https://vcgit.hhi.fraunhofer.de/ecm/jvet-ac-ee2/simulation-results if cross-check reports are not submitted as they are optional for EE tests.

# List of tests

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Intra prediction |  |  |  |
| 1.1 | Extends the CCCM template selection to six | InterDigital P. Bordes JVET-AD0147 | Kwai H.-J. Jhu |
| 1.2 | CCCM using multiple downsampling filters | Qualcomm Y.-J. Chang JVET-AD0202 | Nokia J. Lainema |
| 1.3a | MM-CCLM/MM-CCCM filtering with neighbouring samples | Bytedance K. Zhang JVET-AD0120 | vivo Z. Lv |
| 1.3b | Cross-component prediction usage is determined by template cost | Bytedance K. Zhang JVET-AD0120 | vivo Z. Lv |
| 1.3c | Cross-component prediction signalling modification | Bytedance K. Zhang JVET-AD0120 | vivo Z. Lv |
| 1.3d | Test 1.3a + Test 1.3b + Test 1.3c | Bytedance K. Zhang JVET-AD0120 | vivo Z. Lv |
| 1.4a | Non-adjacent cross-component prediction | Bytedance K. Zhang | withdrawn |
| 1.4b | History-based cross-component prediction | Bytedance K. Zhang | withdrawn |
| 1.4c | Test 1.4a + Test 1.4b | Bytedance K. Zhang | withdrawn |
| 1.5 | Inherit cross-component prediction model from neighbours | MediaTek C.-M. Tsai | withdrawn |
| 1.6a | Cross-component prediction merge list includes spatial adjacent, non-adjacent, and default candidates | MediaTek C.-M. Tsai Bytedance K. Zhang JVET-AD0188 | Kwai C.-W. Kuo |
| 1.6b | Cross-component prediction merge list includes spatial adjacent, history-based, and default candidates | MediaTek C.-M. Tsai Bytedance K. Zhang JVET-AD0188 | Qualcomm Y.-J. Chang |
| 1.6c | Cross-component prediction merge list includes spatial adjacent, non-adjacent, history-based, and default candidates | MediaTek C.-M. Tsai Bytedance K. Zhang JVET-AD0188 | Alibaba J. Chen |
| 1.7 | TIMD using selectable template regions | Sharp Y. Yasugi JVET-AD0103 | Qualcomm H. Wang |
| 1.8a | HLS tool control with encoder optimization for IBC for camera captured content | Qualcomm B. Ray JVET-AD0208 | KDDI Y. Kidani |
| 1.8b | Test 1.8a + fractional pel BV | Qualcomm B. Ray | withdrawn |
| 1.8c | Test 1.8a + fractional pel IBC | Qualcomm C.-C. Chen Kwai W. Chen JVET-AD0208 | KDDI Y. Kidani |
| 1.9a | Fractional pel IBC | Kwai W. Chen | withdrawn |
| 1.9b | Test 1.9a (fractional pel IBC) + Test 1.8a | Kwai W. Chen Qualcomm B. Ray | withdrawn |
| 1.10a | Multi-candidate IntraTMP without search procedure change | OPPO F. Wang JVET-AD0073 | Qualcomm P.-H. Lin JVET-AD0327 |
| 1.10b | Multi-candidate IntraTMP with search procedure change | OPPO F. Wang JVET-AD0073 | Qualcomm P.-H. Lin JVET-AD0327 |
| 1.11a | Intra template-matching prediction fusion | OPPO L. Zhang JVET-AD0072 | Dolby F. Pu JVET-AD0309 |
| 1.11b | Intra template-matching prediction fusion with no change to the IntraTMP search procedure | OPPO L. Zhang JVET-AD0072 | Dolby F. Pu JVET-AD0309 |
| 1.11c | Intra template-matching prediction fusion with different number of candidates | OPPO L. Zhang JVET-AD0072 | Dolby F. Pu JVET-AD0309 |
| 1.12 | IntraTMP with sub-pel precision | Alibaba X. Li JVET-AD0125 | Dolby F. Pu |
| 1.13 | Fusion of intra template matching | Ittiam J. R. Arumugam Dolby F. Pu JVET-AD0158 | OPPO F. Wang Alibaba X. Li |
| 1.14a | Extended the adjacent search area of IntraTMP | Xidian Univ. Y. Ma H. Du JVET-AD0115 | Alibaba X. Li |
| 1.14b | Enlarged the search range for small blocks | OPPO F. Wang JVET-AD0074 |  |
| 1.14c | Test 1.14a + Test 1.14b | Xidian Univ. Y. Ma H. Du OPPO F. Wang JVET-AD0075 |  |
| 1.15a | Intra template matching based on linear filter model | Xidian Univ. J. Huo W. Qiao JVET-AD0112 | Alibaba X. Li |
| 1.15b | Filtered template matching based intra prediction | Nokia R. Youvalari JVET-AD0099 | Alibaba X. Li |
| 1.15c | Test 1.15a+Test 1.15b | Xidian Univ. J. Huo W. Qiao Nokia R. Youvalari | withdrawn |
| 1.16 | IntraTMP fusion with multiple reference blocks | Xidian Univ. J. Huo H. Du JVET-AD0116 | Alibaba X. Li |
| 1.17 | IntraTMP fusion with intra prediction | Bytedance Y. Wang JVET-AD0092 | Ericsson R. Yu JVET-AD0305 |
| 1.18 | CIIP extension with IntraTMP | InterDigital K. Naser JVET-AD0177 | OPPO F. Wang JVET-AD0259 |
| 1.19a | IntraTMP left and above template modes | Qualcomm P.-H. Lin JVET-AD0194 | OPPO F. Wang JVET-AD0263 |
| 1.19b | IntraTMP L-shape fusion mode | Qualcomm P.-H. Lin JVET-AD0194 | OPPO F. Wang JVET-AD0263 |
| 1.19c | Test 1.19a + Test 1.19b | Qualcomm P.-H. Lin JVET-AD0194 | OPPO F. Wang JVET-AD0263 |
| 1.20a | Test 1.10 + Test 1.11 | OPPO F. Wang JVET-AD0076 | Qualcomm P.-H. Lin JVET-AD0328 |
| 1.20b | Test 1.10 + Test 1.11 + Test 1.14 | OPPO F. Wang JVET-AD0076 | Qualcomm P.-H. Lin JVET-AD0328 |
| 1.20c | Test 1.14 + Test 1.19 | Qualcomm P.-H. Lin Xidian Univ. J. Huo OPPO F. Wang JVET-AD0198 | Kwai H.-J. Jhu JVET-AD0279 |
| 1.20d | Test 1.10 + Test 1.12 + Test 1.14 + Test 1.15 + Test 1.19 | Qualcomm P.-H. Lin Xidian Univ. J. Huo OPPO F. Wang Alibaba X. Li | withdrawn |
| 1.20e | Test 1.13 + Test 1.10 | Ittiam J. R. Arumugam OPPO F. Wang JVET-AD0191 | Alibaba X. Li |
| 1.20f | Test 1.13 + Test 1.12 | Dolby F. Pu Alibaba X. Li JVET-AD0192 | OPPO F. Wang |
| 1.20g | Test 1.13 + Test 1.17 | Dolby F. Pu Bytedance Y. Wang | withdrawn |
| 1.20h | Test 1.14 + Test 1.15 + Test 1.16 | Xidian Univ. Y. Ma H. Zhang JVET-AD0118 | Alibaba X. Li |
| 1.20i | Test 1.10 + Test 1.11 + Test 1.12 + Test 1.15 + Test 1.16 + Test 1.19 + Test 1.14a | OPPO F. Wang Xidian Univ. Y. Ma H. Zhang Qualcomm P.-H. Lin Alibaba X. Li JVET-AD0086 | Google X. Li JVET-AD0254 |
| 1.20j | Test 1.10 + Test 1.11 + Test 1.12 + Test 1.15 + Test 1.16 + Test 1.19 + Test 1.14c | OPPO F. Wang Xidian Univ. Y. Ma H. Zhang Qualcomm P.-H. Lin Alibaba X. Li JVET-AD0086 | Google X. Li JVET-AD0254 |
| 1.20k | Test 1.12 + Test 1.14c | Alibaba X. Li Xidian Univ. Y. Ma OPPO F. Wang JVET-AD0126 | Dolby F. Pu |
| 1.20m | Test 1.17 + Test 1.18 | Bytedance Y. Wang InterDigital K. Naser JVET-AD0119 | OPPO F. Wang JVET-AD0260 |
| 1.20n | Test 1.20f + Test 1.14c | Dolby F. Pu Alibaba X. Li JVET-AD0247 | Nokia R. Youvalari |
| 1.21a | Checking more neighbouring positions with intra prediction modes replacement by partitioning angles for SGPM/GPM | OPPO L.Xu JVET-AD0071 |  |
| 1.21b | Angular modes extension from 65 to 129 for TMRL | OPPO L. Xu JVET-AD0071 |  |
| 1.21c | Test 1.21a + Test 1.21b | OPPO L. Xu JVET-AD0071 |  |
| 1.22 | Template-based intra MPM list construction | vivo C. Zhou JVET-AD0173 | Alibaba J. Chen |
| 1.23 | Modification to MPM list derivation | Qualcomm H. Wang JVET-AD0190 | Sharp Y. Yasugi |
| 1.24a | Test 1.21 + Test 1.22 | OPPO L. Xu vivo C. Zhou JVET-AD0085 | Bytedance Z. Deng |
| 1.24b | Test 1.21 + Test 1.23 | OPPO L. Xu Qualcomm H. Wang JVET-AD0085 |  |
| 1.24c | Test 1.22 + Test 1.23 | vivo C. Zhou Qualcomm H. Wang JVET-AD0085 |  |
| 1.24d | Test 1.21 + Test 1.22 + Test 1.23 | OPPO L. Xu vivo C. Zhou Qualcomm H. Wang JVET-AD0085 | Sharp Y. Yasugi |
| 1.25 | Filtered template matching based intra prediction | Nokia R. Youvalari | duplicate of 1.15b |
| 2 Inter prediction |  |  |  |
| 2.1a | Step 1 of CPMV refinement with simplification | Xiaomi M. Blestel Qualcomm Y. Zhang JVET-AD0182 | InterDigital F. Le Léannec |
| 2.1b | Step 2 of CPMV refinement with simplification | Xiaomi M. Blestel Qualcomm Y. Zhang JVET-AD0182 | InterDigital F. Le Léannec |
| 2.1c | Adaptive DMVR for affine merge + affine DMVR for affine MMVD | Qualcomm Y. Zhang JVET-AD0182 | Kwai W. Chen |
| 2.1d | Affine parameter refinement | Alibaba J. Chen Xiaomi M. Blestel JVET-AD0182 | MediaTek C.-M. Tsai |
| 2.1e | Affine parameter refinement with simplification | Alibaba J. Chen Xiaomi M. Blestel JVET-AD0182 | MediaTek C.-M. Tsai |
| 2.1f | Test 2.1a + Test 2.1b | Xiaomi M. Blestel Qualcomm Y. Zhang | withdrawn |
| 2.1g | Test 2.1a + Test 2.1c | Qualcomm Y. Zhang Xiaomi M. Blestel | withdrawn |
| 2.1h | Test 2.1b+ Test 2.1c | Qualcomm Y. Zhang Xiaomi M. Blestel | withdrawn |
| 2.1i | Test 2.1c+ Test 2.1f | Qualcomm Y. Zhang Xiaomi M. Blestel | withdrawn |
| 2.1j | Test2.1d + Test 2.1a | Alibaba J. Chen Xiaomi M. Blestel Qualcomm Y. Zhang | withdrawn |
| 2.1k | Test 2.1d + Test 2.1f | Alibaba J. Chen Xiaomi M. Blestel Qualcomm Y. Zhang JVET-AD0182 | InterDigital F. Le Léannec |
| 2.1l | Test 2.1d + Test 2.1c | Alibaba J. Chen Qualcomm Y. Zhang Xiaomi M. Blestel JVET-AD0182 | ByteDance L. Zhao |
| 2.1m | Test 2.1d + Test 2.1i | Alibaba J. Chen Qualcomm Y. Zhang Xiaomi M. Blestel | withdrawn |
| 2.2 | AMVP with SbTMVP mode | Alibaba R.-L. Liao JVET-AD0152 | Ofinno V. Rufitskiy |
| 2.3a | SbTMVP with MMVD | Tencent L.-F. Chen JVET-AD0209 | Kwai X. Xiu |
| 2.3b | SbTMVP with MMVD by using TM-based reordering | Tencent L.-F. Chen JVET-AD0209 | Kwai X. Xiu |
| 2.4 | Template matching-based subblock motion refinement | Bytedance L. Zhao JVET-AD0165 | Alibaba R.-L. Liao |
| 2.5a | Test 2.2 + Test 2.4 | Alibaba R.-L. Liao Bytedance L. Zhao JVET-AD0153 | Canon G. Laroche |
| 2.5b | Test 2.2 + Test 2.3 | Alibaba R.-L. Liao Tencent L.-F. Chen JVET-AD0210 | Google X. Li |
| 2.6 | High precision MV refinement for BDOF | Bytedance M. Salehifar JVET-AD0195 |  |
| 2.7a | LIC for bi-prediction | Kwai X. Xiu JVET-AD0213 | Tencent L.-F. Chen |
| 2.7b | Test2.7a + OBMC with LIC | Kwai X.Xiu JVET-AD0213 | Tencent L-F. Chen |
| 2.8a | OBMC flag inheritance in merge mode | Qualcomm K. Cui | withdrawn |
| 2.8b | OBMC flag is signalled only for affine block without MHP | Qualcomm K. Cui | withdrawn |
| 2.9a | OBMC boundary level decision based on prediction samples, keeping the existing CU level OBMC on/off flag | Google X. Li | withdrawn |
| 2.9b | OBMC boundary level decision based on prediction samples, without the existing CU level OBMC on/off flag | Google X. Li | withdrawn |
| 2.9c | OBMC boundary level decision based on reference samples, keeping the existing CU level OBMC on/off flag | Google X. Li | withdrawn |
| 2.9d | OBMC boundary level decision based on reference samples, without the existing CU level OBMC on/off flag | Google X. Li | withdrawn |
| 2.10a | OBMC on/off decision based on prediction samples and neighbouring information | Bytedance Z. Deng | withdrawn |
| 2.10b | Using reference samples rather than prediction samples for OBMC on/off decision | Bytedance Z. Deng | withdrawn |
| 2.11a | OBMC on/off decision at CU level and boundary level based on prediction samples and neighbouring information, with existing CU level OBMC on/off flag | Google X. Li Bytedance Z. Deng | withdrawn |
| 2.11b | OBMC on/off decision at CU level and boundary level based on prediction samples and neighbouring information, without existing CU level OBMC on/off flag | Google X. Li Bytedance Z. Deng | withdrawn |
| 2.11c | OBMC on/off decision at CU level and boundary level based on reference samples, with existing CU level OBMC on/off flag | Google X. Li Bytedance Z. Deng | withdrawn |
| 2.11d | OBMC on/off decision at CU level and boundary level based on reference samples, without existing CU level OBMC on/off flag | Google X. Li Bytedance Z. Deng | withdrawn |
| 2.11e | Adaptive OBMC control | Qualcomm K. Cui Google X. Li Bytedance Z. Deng JVET-AD0193 | Kwai X. Xiu OPPO J. Gan |
| 2.12a | Prediction of MVD magnitude suffix bins (up to 6 bins for blocks with width and height larger than 4, and up to 2 bins otherwise) | Ofinno A. Filippov JVET-AD0140 | Xiaomi M. Blestel |
| 2.12b | Prediction of MVD magnitude suffix bins (up to 6 bins for blocks with width and height larger than 4, and up to 4 bins otherwise) | Ofinno A. Filippov | withdrawn |
| 2.12c | Prediction of MVD magnitude suffix bins (up to 6 bins per block) | Ofinno V. Rufitskiy | withdrawn |
| 2.12d | Prediction of MVD magnitude suffix bins (up to 8 bins for blocks with width and height larger than 4, and up to 4 bins otherwise) | Ofinno V. Rufitskiy | withdrawn |
| 2.12e | Prediction of MVD magnitude suffix bins (up to 8 bins for blocks with width and height larger than 4, and up to 6 bins otherwise) | Ofinno V. Rufitskiy | withdrawn |
| 2.12f | Tool-off test for MVD sign prediction as implemented in ECM | Ofinno V. Rufitskiy JVET-AD0140 | Xiaomi M. Blestel |
| 2.12g | Test 2.2 + Test 2.12 | Ofinno V. Rufitskiy Alibaba R.-L. Liao JVET-AD0256 | Xiaomi M. Blestel |
| 2.12h | Test 2.7 + Test 2.12 | Ofinno A. Filippov Kwai X. Xiu JVET-AD0257 | Xiaomi M. Blestel |
| 3 Screen content coding |  |  |  |
| 3.1 | IntraTMP using reconstruction-reordered for screen content coding | Ofinno D. Ruiz Coll JVET-AD0171 |  |
| 3.2a | IBC MBVD list derivation with max BVD offset 128-pel | Qualcomm Z. Zhang JVET-AD0199 | Ofinno D. Ruiz Coll |
| 3.2b | IBC MBVD list derivation with max BVD offset 256-pel | Qualcomm Z. Zhang JVET-AD0199 | Ofinno D. Ruiz Coll |
| 3.3 | Temporal block vector prediction | Bytedance N. Zhang JVET-AD0095 | Qualcomm Z. Zhang |
| 3.4a | Copy-padding for IBC | Bytedance N. Zhang JVET-AD0096 | Hanbat National University H. Han JVET-AD0139 Qualcomm Z. Zhang |
| 3.4b | Test 3.4a + Test 3.3 | Bytedance N. Zhang JVET-AD0096 | Qualcomm Z. Zhang |
| 3.5 | Template matching for IBC BVD suffix derivation | Qualcomm P. Nikitin | withdrawn |
| 3.6 | IBC-CIIP with adaptive weights | Kwai C. Ma | withdrawn |
| 3.7a | Test 3.2 + Test 3.4b | Bytedance N. Zhang Qualcomm Z. Zhang JVET-AD0097 | MediaTek C.-M. Tsai |
| 3.7b | Test 3.2b + Test 3.4b | Bytedance N. Zhang Qualcomm Z. Zhang JVET-AD0097 | MediaTek C.-M. Tsai |
| 4 In-loop filtering |  |  |  |
| 4.1 | Additional fixed and signalled filters for ALF | Bytedance W. Yin JVET-AD0221 | Qualcomm N. Hu |
| 4.2 | ALF classifier based on residual data | Qualcomm I. Jumakulyyev JVET-AD0219 | BytedanceW. Yin |
| 4.3 | ALF filter shape with more taps applied to fixed filter results | Qualcomm I. Jumakulyyev JVET-AD0219 | BytedanceW. Yin |
| 4.4a | Test 4.2 + Test 4.3 | Qualcomm I. Jumakulyyev JVET-AD0219 | BytedanceW. Yin |
| 4.4b | Test 4.1 + Test 4.2 + Test 4.3 | Qualcomm I. Jumakulyyev Bytedance W. Yin JVET-AD0222 | Ericsson K. Andersson |
| 4.4c | Test 4.1 + Test 4.2 | Qualcomm I. Jumakulyyev Bytedance W. Yin JVET-AD0222 | Ericsson K. Andersson |
| 5 GDR |  |  |  |
| 5.1 | Reference picture padding for GDR | InterDigital T. Poirier JVET-AD0123 | Nokia S. Hong |

# Description of tests

## Intra prediction

### Test 1.1 Extends the CCCM template selection to six (JVET-AD0147)

In this test, the number of CCCM templates is extended (in addition to top and left, only left, and only top) to 6 as shown in the below figure in the second row. When one of the 3 extended templates is selected, the model is derived selecting the samples of the template with the reconstructed luma value within the range of the reconstructed current CU luma values. Also, the CCCM parameters are applied with 32-bits precision. If CCCM mode is selected, similarly to GL-CCCM, a flag is coded to indicate whether regular or extended template is used.

### Test 1.2 CCCM using multiple downsampling filters (JVET-AD0202)

It the test, multiple downsampling filters are applied to a group of reconstructed luma samples in a CCCM. The linear combination of these downsampled reconstructed samples is multiplied by derived filter coefficients to form the final chroma predictor. The horizontal or vertical location of the center luma sample are also considered in the tested model. The cross-component models shown below are tested as additional CCCM modes with a mode index signalled in the bitstream:

- Model 1: predChroma = c0 * H(C) + c1 *  G1(C) + c2 *  G2(C) + c3 *  G3(C) + c4 *  P(H(C)) + c5 *  P(G1(C)) + c6 * P(G2(C)) + c7 * X + c8 * Y + c9 * B

- Model 2: predChroma = c0 * H(C) + c1 *  H(W) + c2 *  H(E) + c3 *  G1(C) + c4 *  G1(W) + c5 *  G1(E) + c6 *  P(H(C)) + c7 *  P(H(W)) + c8 *  P(H(E)) + c9 * X + c10 * B

- Model 3: predChroma = c0 * H(C) + c1 *  H(NE) + c2 *  H(SW) + c3 *  G3(C) + c4 *  G3(NE) + c5 *  G3(SW) + c6 *  P(H(C)) + c7 *  P(H(NE)) + c8 *  P(H(SW)) + c9 * Y + c10 * B

where H(·), G1(·), G2(·), G3(·) are various downsampling filters as indicated in Figure 1, C denotes the current chroma sample position, and N, S, W, E, NE, SW are the positions around C as indicated in Figure 2, ci are filter coefficients, P and B are nonlinear term and bias term, and X and Y are the horizontal and vertical locations of the center luma sample with respect to the top-left coordinates of the block.

Figure 1. Various downsampling filters used in the tested cross-component models

Figure 2. The positions of chroma samples

### Test 1.3: Local-boosting cross-component prediction (JVET-AD0120)

In this method, the local information including neighbouring and prediction samples are considered for cross-component prediction (CCCM and CCLM). The aspects are included.

Aspect #1: Prediction samples of MM-CCLM/MM-CCCM can be filtered with neighbouring samples with a 3×3 low-pass filter. At a top/left boundary, the filtering window involves neighbouring reconstructed samples. For inner samples, the filtering window only involves prediction samples, which are padded. A flag is signalled to indicate whether filtering is applied or not for a block coded with MM-CCLM/MM-CCCM.

Aspect #2: Template costs are calculated to implicitly determine the usage of the cross-component prediction. The template cost is derived by applying the candidate cross-component mode on the template and calculating the SAD between the prediction samples and reconstruction samples in the template. The mode with a lower template cost is selected as the final method for the current block.

Aspect #3: The binarization of the mode signalling is modified. Instead of signaling CCLM-L/CCLM-T/MM-CCLM-L/MM-CCLM-T (or CCCM-L/CCCM-T/MM-CCCM-L/MM-CCCM-T) with a truncated unary code, two syntax elements are coded: multi_mode_flag indicates whether multiple or single model is applied, and ccp_dir_flag indicates whether left or above direction is applied.

All aspects are tested individually and in a combination as follows.

Test 1.3a: Aspect #1 (MM-CCLM/MM-CCCM filtering with neighbouring samples).

Test 1.3b: Aspect #2 (cross-component prediction usage is determined by template cost).

Test 1.3c: Aspect #3 (cross-component prediction signalling modification).

Test 1.3e: Aspects 1 and 2.

Test 1.3d: Aspects 1, 2, and 3.

### Test 1.6: Non-local cross-component prediction and cross-component merge mode (JVET-AD0188)

In the test, a cross-component prediction merge candidate list is constructed from the spatial adjacent, spatial non-adjacent, or history-based candidates. After including these candidates, default models are further included to fill the remaining empty positions in the merge list. Pruning is applied to remove redundant candidates. After constructing the list, the models in the list are reordered depending on the SAD costs, which are obtained using the neighbouring template of the current block.

Spatial adjacent and non-adjacent candidates

The positions and inclusion order of the spatial adjacent and non-adjacent candidates are the same as those defined in ECM for regular inter merge prediction candidates.

History-based candidates

A history-based table is maintained to include the recently used cross-component models, and the table is reset at the beginning of each CTU row. If the current list is not full after including spatial adjacent and non-adjacent candidates, the models from the history-based table are added into the list.

Default candidates

CCLM candidates with default scaling parameters are considered, only when the list is not full after including the spatial adjacent, spatial non-adjacent, or history-based candidates. If the current list has no candidates with the single model CCLM mode, the default scaling parameters are {0, 1/8, -1/8, 2/8, -2/8, 3/8, -3/8, 4/8, -4/8, 5/8, -5/8, 6/8}. Otherwise, the default scaling parameters are {0, the scaling parameter of the first CCLM candidate + {1/8, -1/8, 2/8, -2/8, 3/8, -3/8, 4/8, -4/8, 5/8, -5/8, 6/8}}. The offset parameter is derived according to the default scaling parameter, average neighbouring reconstructed luma sample value, and average neighbouring reconstructed Cb/Cr sample value.

A flag is signalled to indicate whether the cross-component prediction merge mode is applied or not. If it is applied, an index is signalled to indicate which candidate model is used by the current block. The mode is not allowed when the current CU is coded by intra subpartitions (ISP) with single tree, or the current chroma coding block size is less than or equal to 16.

The following tests are performed:

Test 1.6a: the merge list includes spatial adjacent, non-adjacent, and default candidates.
Test 1.6b: the merge list includes spatial adjacent, history-based, and default candidates.
Test 1.6c: the merge list includes spatial adjacent, non-adjacent, history-based, and default candidates.

In Test 1.6a and Test 1.6b, the maximum allowed list size is 6. In Test 1.6c, the size is changed to 12 to accommodate more potential candidates. Besides, the maximum allowed history-based table size is 6 in Test 1.6b and Test 1.6c.

### Test 1.7: TIMD using selectable template regions (JVET-AD0103)

In ECM, the templates for TIMD mode are implicitly selected from the left, top, or top and left templates depending on the coordinates for the current block. If x=0, only top template is used, if y=0 only left template is used, otherwise left and top templates are applied.

In the test, a syntax element is signalled to explicitly indicate one of the three types of template regions, even for CUs that are not located at the top or left boundary of the frame.

### Test 1.8 IBC adaptation for camera captured content (JVET-AD0208)

In the tests, there are two main aspects: to provide encoder optimization for IBC and extending IBC to use fractional pel BVs.

Aspect #1. This aspect involves high level tool control of various IBC coding tools and encoder optimization, which is applied to camera captured content (non-SCC) classes:

- IBC merge modes are disabled by using an SPS level flag to indicate that. So, only IBC AMVP mode is activated.

- RR-IBC, TM-IBC, and IBC-CIIP are disabled. For RR-IBC and TM-IBC, corresponding SPS flags are implemented (as ECM-8.0 does not have SPS control for these tools).

- IBC is applied to intra slices only which is indicated by HLS.

- At encoder, IBC block vector search is optimized, and the RDO process may be skipped for an IBC AMVP mode if its SAD cost is much worse than the lowest SAD cost of all intra modes.

- IBC AMVP modes may not be evaluated when the best intra mode has less than 3 nonzero coefficients.

- In ECM-8.0 encoder, some partitioning depth in an inter slice is skipped depending on the POC distance between the current picture and its nearest reference picture. When IBC is enabled from SPS level, ECM-8.0 always sets this POC distance equal to 0, while in this test, for inter slices, the true POC distance is used instead of setting to 0.

Aspect #2. Fractional pel BVs for IBC as follows:

- The option of block vector resolutions is extended to include quarter-pel resolution in additional to full-pel and 4-pel. Like inter AMVR syntax, the first bin is signalled to indicate whether BV is in quarter-pel resolution, and the second bin is signalled to switch between full-pel and 4-pel resolutions.

- The interpolation filters applied to the luma (8-tap) and chroma (6-tap existed inter interpolation) components of an IBC block. For template-based IBC tools, a 2-tap bilinear interpolation filter is applied to generate template prediction blocks.

- Reference sample padding is performed when some of them are located outside IBC reference area. When needed, it performs in horizontal direction first and then vertical direction.

Two tests are performed:

Test 1.8a: HLS tool control with encoder optimization for IBC for camera captured content (aspect #1)

Test 1.8c: Test 1.8a with fractional pel BVs (aspect #2, applied for all classes)

### Test 1.10: Multi-candidate IntraTMP (JVET-AD0073)

In the test, a candidate list is constructed, and the candidate BVs are ranked in ascending order of their template matching costs. An index is signalled in the bit-stream to indicate which candidate BV is used for current block.

To build a BV candidate list of size N, in the sparse search, the sub-sampling factor is set to 3 instead of 2 used in ECM-8.0 and 2N BVs with the smallest template matching costs are maintained. Then, in the refinement search, each 3x3 block around each of the 2N BVs is checked and N BVs with the smallest cost are selected to form the candidate list.

N equal to 15 and 19 is tested.

Test 1.10a: Multi-candidate IntraTMP without search procedure change.

Test 1.10b: Multi-candidate IntraTMP with search procedure change.

### Test 1.11: Intra template matching prediction fusion (JVET-AD0072)

The tested fusion method includes the following aspects:

- Generate multiple matched blocks during the IntraTMP search.

During the subsampled IntraTMP search process with the step size 3 instead of the current 2, a candidate list is initially generated for 30 matched blocks with the smallest template SAD. For each candidate, a full 3x3 pixel refinement search around each of the 30 matched blocks is performed and the best 3 candidate matched blocks measured by template SAD are selected.

- Select candidate matched blocks for fusion.

For each of the best 3 candidate matched blocks, a threshold equal to 2 * SAD1 is used to select candidates for fusion, SAD1 is the smallest template SAD of the three candidate matched blocks. Candidates SAD <= Threshold are used for fusion.

- Calculate fusion weight for each of the selected matched blocks.

Once blocks to be fused are decided, they are fused with the weights calculated by their SAD as follows:

sum= i=1nSADi

wi= sum-SADin-1*sum

To reduce the implementation cost, the division operations are replaced by an integer look-up table.

- The final fused predictor is determined by

pfusion=i=1nwipi

where pi is the ith matched block, and n is the number of blocks selected for fusion. In a special case, when only one matched block remains after step b), the final predictor is calculated as

pfusion=w1pTMP+w2pplanar

where pTMP is the single matched block and pplanar is the planar intra predictor, where the weights are set as w1=7/8 and w2=1/8.

A CU level flag is added to signal whether an IntraTMP CU is predicted by the proposed fusion method or by the original method.

Test 1.11a: Intra template-matching prediction fusion with 3 candidates

Test 1.11b: Intra template-matching prediction fusion with no change to search procedure

Test 1.11c: Intra template-matching prediction fusion with 5 candidates

### Test 1.12: IntraTMP with sub-pel precision (JVET-AD0125)

In the test, three sub-pel precisions, including half-pel, quarter-pel and three quarter-pel, with eight directions around the integer-pel position are tested as shown in Figure 3. Furthermore, the eight directions are sorted by template cost with half-pel precision for each CU, and only the first four directions can be used for each sub-pel precision.

Figure 3. The adjacent half-pel positions in 8 directions.

If Intra TMP mode is selected for the current block, a precision index is signalled to indicate which of the integer-pel and three sub-pel precisions is used. If one of the three sub-pel precisions is used, a direction index is signalled to indicate which of the four directions is used.

Four-tap DCT-IF interpolation filters in ECM are used for sub-pel interpolation in Intra TMP.

### Test 1.13: Fusion of intra template matching (JVET-AD0158)

In the first search pass, the best matching block with the lowest TM cost is selected, specified its template as Tbest. Then in the second search pass, the template is modified as T'=2*T-Tbest. A separate flag is signalled to indicate whether the second search with the template update is used at decoder.

Test 1.13a: Adaptive IntraTMP fusion with up to two prediction blocks.

Test 1.13b: Test 1.13a + search process from Test 1.10

### Test 1.14a: Extended the adjacent search area of IntraTMP (JVET-AD0115)

In this test, R3 and R4 regions are extended shown in Figure 4 as vertical upper side and horizontal left side of the current block that are also reconstructed and can be used for prediction.

Figure 4. IntraTMP search regions in ECM-8.0 (top) and extended regions (bottom)

### Test 1.14b: Enlarged the search range for small blocks (JVET-AD0074)

In ECM, IntraTMP search range is based on the current block size:

SearchRangeWidth = a * BlkWidth

SearchRangeHight = a * BlkHeight,

where a is equal to 5.

In the test, IntraTMP search range for small blocks is enlarged as follows:

SearchRangeWidth = max(a * BlkWidth, minSearchRange)

SearchRangeHight = max(a * BlkHeight, minSearchRange),

where minSearchRange is set as 64.

### Test 1.14c: Combination of Test 1.14a and Test 1.4b (JVET-AD0075)

### Test 1.15a: Intra template matching based on linear filter model (JVET-AD0112)

In this method, a linear model is derived between the reference and current templates, the derived model is applied to the IntraTMP predictor. Similar to CCCM, a model is represented by 6-tap filter consisting of a 5-tap spatial and a bias terms.

predLumaVal = c0C + c1N + c2S + c3E + c4W + c5 * (1<<(bitdepth – 1) )

The template size is 4, and the method is applied to the best BV candidate. A flag is signalled to indicate the mode usage.

### Test 1.15b: Filtered template matching based intra prediction (JVET-AD0099)

Similar to 1.15a, the same 6-tap filter model applied to IntraTMP is tested. The template size used for training is 4 lines above and to the left of the current block depending on their availability.

The filtered method uses the current IntraTMP’s SAD based search in ECM-8.0 for creating a list of four candidate reference blocks to be filtered. Entry to the candidate list is based on SAD cost of the unfiltered template.

The filter parameters are calculated for each candidate in the list and the best performing one in terms of template cost after filtering is selected and used as the final candidate. The final prediction of the block is then generated by applying the derived filter for the best candidate.

### Test 1.16: IntraTMP fusion with multiple reference blocks (JVET-AD0116)

IntraTMP fusion is tested, N best BVs are derived during the subsampled search with step size of 2, then those N best predictors are fused with the weights as follows.

predSamples=n=0N-1wn*refBlockn+wN*(1≪bitdepth-1)

where predSamples represent the final predicted block, refBlockn is one of the N candidate reference blocks. The weights wn are derived from the candidate matching templates and current template using the decomposition method from CCCM.

A flag is signalled to indicate the mode usage.

### Test 1.17: IntraTMP fusion with intra prediction (JVET-AD0092)

In IntraTMP fusion with intra prediction, the prediction block is the weighted sum of the two prediction signals generated by IntraTMP and intra prediction derived by TIMD. A CU flag is signalled to indicate whether the tested method is used.

### Test 1.18: CIIP extension with IntraTMP (JVET-AD0177)

The tested method is similar to inter CIIP, where the inter part is replaced by IntraTMP prediction, while the intra prediction and weights calculation is kept unchanged. A flag is signalled to indicate the mode usage.

Test 1.18b: CIIP extension with IntraTMP

Test 1.18a: Test 1.18b + Test 1.14c (search range modification)

### Test 1.19: IntraTMP with multiple modes (JVET-AD0194)

In Test-1.19a, 3 template types are defined. In addition to L-shape (left and above templates), using only left and only above templates as shown in the below figure is tested. For each template type, the best 4 candidates are derived in the template matching process and selected by HAD cost. To signal the selected candidate (template type and candidate index), a flag is signalled to indicate which template type is used followed by the candidate index.

In Test-1.19b, a fusion method is used to combine multiple candidates, the fusion weights are derived by template matching costs or using the derivation method of MSE minimization existed in CCCM. A flag is signalled to indicate which weight derivation method is used. The candidates to be fused are the best 2 L-shape candidates or the best 5 L-shape candidates. Another flag is signalled to indicate the number of candidates to be fused (2 or 5).

Test 1.19c is the combination of tests 1.19a and 1.19b.

### Tests 1.20: Combination of IntraTMP related tests

In the tests, there are the following elements:

- Extended IntraTMP search range

- Enlarged search range for small blocks

- Modified IntraTMP search process

- Multiple candidates for IntraTMP

- Fusion candidates for IntraTMP (TM cost and MSE based weights derivation)

- Filtered IntraTMP mode

- Fractional pel IntraTMP

- Fusion of intra template matching

- Combination of IntraTMP and intra prediction

A combination Test 1.20j includes all aspects except the last two.

Test 1.20a (JVET-AD0076): Test 1.10b (19 candidates) + Test 1.11c (without fusion with intra)

Test 1.20b (JVET-AD0076): Test 1.10b (19 candidates) + Test 1.11c (without fusion with intra) + Test 1.14c

Test 1.20c (JVET-AD0198): Test 1.14c + Test 1.19c

Test 1.20e (JVET-AD0191): Test 1.13b + Test 1.10b (15 candidates)

Test 1.20f (JVET-AD0192): Test 1.13b + Test 1.12

Test 1.20h (JVET-AD0118): Test 1.14a + Test 1.15a + Test 1.16

Test 1.20i (JVET-AD0086): Test 1.10b(19 candidates) + Test 1.11c + Test 1.12 + Test 1.15a + Test 1.16 + Test 1.19c + Test 1.14a

Test 1.20j (JVET-AD0086): Test 1.10b(19 candidates) + Test 1.11c + Test 1.12 + Test 1.15a + Test 1.16 + Test 1.19c + Test 1.14c

Test 1.20k (JVET-AD0126): Test 1.12 + Test 1.14c

Test 1.20m (JVET-AD0119): Test 1.17 + Test 1.18b

Test 1.20m* (JVET-AD0119): Test 1.17 + Test 1.18a (includes search range modification of Test 1.14c)

Test 1.20n (JVET-AD0247): Test 1.20f + Test 1.14c

### Test 1.21: Modifications on template-based multiple reference line intra prediction (JVET-AD0071)

Two aspects are tested modifying TMRL prediction.

In Test 1.21a, the construction of intra candidate list for MPM and TMRL is modified as follows:

- Non-adjacent positions are added as candidates in constructing the intra candidate list.

- If the neighbouring or non-adjacent blocks are coded with SGPM or GPM modes, the intra modes of the blocks are replaced by the partitioning angles.

In Test 1.21b, the precision of angular prediction is extended from 65 to 129 for TMRL.

Test 1.21c is a combination of Test 1.21a and Test 1.21b.

### Test 1.22: Template-based intra MPM list construction (JVET-AD0173)

In ECM, the regular MPM list consists of primary MPMs, secondary MPMs, and DIMD modes. There is no ordering of the modes based on template cost.

In the test, the regular MPM list is constructed by the following three steps:

- The first entry in the general MPM list is always planar mode.

- The next entries are composed of the intra modes of neighboring blocks, and DIMD modes, which are sorted in ascending order of SAD cost. Up to 5 modes with the smallest SAD cost are added. The SAD cost is computed between the prediction and the reconstruction samples of the template.

- The sorted directional modes with added offset are added into the general MPM list, and then the default modes, until the general MPM list with 22 entries is constructed.

For the signaling part, MPM list is equally divided into four groups and the group index is parsed first. Then, a mode index is further parsed to indicate which mode in the selected group is used.

### Test 1.23: Modification to MPM list derivation (JVET-AD0190)

In ECM, there are two methods to construct regular MPM and TMRL MPM lists. In the tests, one MPM list derivation method is introduced for both cases as follows:

- For the spatial neighbors of the current block, if a neighbor block is coded by MIP/IntraTMP/Planar mode, the first non-planar mode from the spatial MPM modes of the neighbour block is selected as the derived mode and the derived mode is tagged as ‘reserved’ for the partial MPM reordering performed in ECM-8.0 when generating MPM list for the current block.

- Secondary MPM modes are derived by adding offset to all primary angular modes, this process is the same as in ECM-8.0 for TMRL modes, where the offset is added to DIMD modes first.

- If MPM list is incomplete, the same default modes are added as in ECM-8.0.

### Tests 1.24: Combinations of MPM related tests (JVET-AD0085)

Test 1.24a: Test 1.21c + Test 1.22

Test 1.24b: Test 1.21c + Test 1.23

Test 1.24c: Test 1.22 + Test 1.23

Test 1.24d: Test 1.21c + Test 1.22 + Test 1.23

## Inter prediction

### Tests 2.1: Affine DMVR (JVET-AD0182)

Several aspects related to affine DMVR are tested.

1. Affine DMVR extension.

The refinement of the translational of the affine motion existed in DMVR is further extended to affine MMVD and adaptive BM merge modes. For affine MMVD mode, the MMVD offset is added to the affine DMVR refined affine merge base candidate if the base candidate meets the affine DMVR refinement condition. For adaptive BM merge mode, an affine merge list that only contains affine merge candidates that meet the affine DMVR conditions are constructed and then refined.

2. CPMV search.

For each control-point, perform bilateral matching for a block that centered by the control-points coordinates to derive the refined CPMVs. Loop over combinations of initial and refined CPMVs to derive the best set of CPMVs that minimize the bilateral matching cost of the current block.

3. CPMV refinement.

For CPMV refinement, an iterative search process is applied to further refine the CPMVs to minimize the bilateral matching cost of the current block. In each iteration, only one CPMV is refined while the others are fixed.

4. Affine parameters refinement.

In this refinement process, wherein each of CPMVs is fixed as base MV in turn, and an offset is added to the non-translation parameter of affine model by minimizing the bilateral matching cost, and then the other two CPMVs are calculated according to the base MV and refined non-translation parameters. To reduce the complexity, different number of search rounds and search directions are set for different stages with cost based early termination.

5. Simplifications.

Early termination based on the best PU distortion is applied. If initial distortion before CPMV refinement is lower than a threshold, then the iterative CPMVs refinement is bypassed.

The affine CPMV refinement is disabled if one of the following conditions is true:

- If block is square and is large than 128x128

- If block is non-square and width or height is greater than 64

The following tests are performed.

In Test 2.1a, CPMV search with simplification is tested.

In Test 2.1b, CPMV refinement with simplification is tested.

In Test 2.1c, affine DMVR extension is tested. Affine DMVR extension includes applying affine DMVR and affine regression to affine MMVD mode and adaptive BM merge mode.

In Test 2.1d, affine parameters refinement is tested.

In Test 2.1e, Test 2.1d with simplification is tested.

In Test 2.1k, a combination of Test 2.1a, Test 2.1b, and Test 2.1d is tested.

In Test 2.1l, a combination of Test 2.1e and Test 2.1c is tested.

### Test 2.2: AMVP with SbTMVP mode (JVET-AD0152)

In the test, SbTMVP is applied to AMVP mode. When the mode is applied, a CU is split into 4x4 subblocks, and each subblock derives its own motion from a corresponding subblock in the collocated picture. The corresponding subblock is obtained using MVP with a signalled MVD. The MVP is derived in the same way to that of MVP of normal AMVP mode. The number of MVDs is determined according to the following rules:

- If the POC distance between current picture and its collocated picture is larger than 2, the number of MVD is set to 16 which comprises 4 motion directions and 4 motion magnitudes (Figure 5);

- Otherwise, if the current picture is a low delay picture, the number of MVD is set to 32 which comprises 4 motion directions and 8 motion magnitudes (Figure 6);

- Otherwise, the number of MVD is set to 64 which comprises 8 motion directions and 8 motion magnitudes (Figure 7).

Figure 5. 16 candidates for picture whose POC distance between current picture and collocated picture is larger than 2

Figure 6. 32 candidates for low delay picture

Figure 7. 64 candidates for non-low delay picture whose POC distance between current picture and collocated picture is smaller than or equal to 2

The adaptive motion vector resolution (AMVR), local illumination compensation (LIC) and multi-hypothesis prediction (MHP) are not applied to the proposed mode. The OBMC is always applied to the proposed mode without signalling.

When a CU is coded using AMVP mode, a flag is signalled to indicate whether the CU is predicted with SbTMVP. If the SbTMVP is used, another flag is signalled to indicate whether MVD exists, and an MVD index is signalled to indicate the motion direction and motion magnitude if MVD exists. The inter prediction direction and reference picture index are directly set to the collocated picture instead of being signalled.

### Test 2.3: SbTMVP with MMVD (JVET-AD0209)

In the test, MVD offsets are applied to SbTMVP candidate in subblock MMVD merge list, an index is signalled to indicate the MVD offset, and this offset is used to derive the offset value of the displacement vector (DV) for each SbTMVP-MMVD candidate. The final DV for each SbTMVP-MMVD candidate is calculated from the DV of SbTMVP merge candidate with the selected offset.

The spiral scanning order is used to scan all MMVD candidates and the available candidate, which has the valid center motion vector, will be added into the candidate list.

Picture level mode flag is used to indicate whether the SbTMVP-MMVD is enabled, it is determined based on the temporal ID and the mode is only enabled at the highest two temporal layers for RA.

In Test 2.3a, MMVD offset is the integer pel step size of {4, 8, 12, 16, 20}, the number of directions is kept as 8 which is used in affine MMVD as well. Like the affine MMVD, the total number of the SbTMVP-MMVD candidates is less than or equal to 16. The spiral scanning order is applied and up to 16 available candidates are added into the candidate list.

In Test 2.3b, all 40 MMVD candidates will be scanned in spiral order and available candidates are added to the candidate list. Then, the subblock-based template-matching is used to reorder all SbTMVP-MMVD candidates by using template-matching cost in ascending order and up to 16 candidates with the smallest template-matching cost will be kept. The constraint of the size of collocated position is extended from one CTU row to three CTU rows. This extension is applied for SbTMVP-MMVD only, and there is no size extension for TMVP and SbTMVP.

### Test 2.4: Template matching-based subblock motion refinement (JVET-AD0165)

Two aspects are included in the test.

Firstly, after the candidate list is constructed for affine merge mode, uni-directional affine candidates will perform CPMV refinement based on TM, where the same MV offset is applied to all the CPMVs of an affine candidate, and the TM cost of the affine candidate is calculated accordingly. The optimal CPMV offset with the minimum TM cost is used to refine the corresponding affine candidate if CRefined<T×COrig, where CRefined  denotes the TM cost after CPMVs are refined by the optimal offset, COrig is the TM cost calculated by the original CPMVs, and T is a constant threshold.

Secondly, the initial motion shift to specify SbTMVP is further refined with TM.

### Test 2.5: Combination of SbTMVP related tests

Test 2.5a (JVET-AD0153): Test 2.2 + Test 2.4 (enabled for larger than 720p at encoder).

Test 2.5b (JVET-AD0210): Test 2.2 + Test 2.3a

Test 2.5c (JVET-AD0210): Test 2.2 + Test 2.3b

### Test 2.6: High precision MV refinement for BDOF (JVET-AD0195)

In the test, high-precision equations to derive the BDOF MV refinement parameters are used:

Gx.Gx * vx  + Gx.Gy * vy  = dI . Gx   s1 * vx + s2 * vy = s3

Gx.Gy * vx  + Gy.Gy * vy  = dI . Gy   s2 * vx + s5 * vy = s6

where Gx/Gy are the summation of the 2 horizontal/vertical gradients derived for each reference block.

Summations () are weighted sums, where weights depend on the position in the summation window, higher weights are given for center samples.

The subblock size of BDOF DMVR is adaptively selected depending on the width×height. For blocks smaller than 256, subblock size of 4×4, and otherwise 8×8 is used.

### Test 2.7: LIC for bi-prediction (JVET-AD0213)

In the first aspect, LIC mode is extended to bi-predictive CUs, where two different linear models are applied to the two prediction blocks which are then combined to generate the bi-prediction samples of the current CU, i.e.,

P'x,y=1-ω∙p0'x,y+ω∙p1'x,y

and

p0'x,y=α0∙P0x,y+β0

p1'x,y=α1∙P1x,y+β1

where α0 and β0, and α1 and β1 indicate the scales and the offsets in L0 and L1, respectively; ω indicates the weight (as indicated by the CU-level BCW index) for the weighted combination of L0 and L1 predictions.

The method firstly derives the L0 parameters by minimizing difference between L0 template prediction T0 and the template T and the samples in T are updated by subtracting the corresponding samples in T0.  Then, the L1 parameters are calculated that minimizes the difference between L1 template prediction T1 and the updated template. Finally, the L0 parameter is refined again in the same way.

Following the current LIC design, one flag is signalled for AMVP bi-predicted CUs for the indication of the LIC mode while the flag is inherited for merge related inter CUs. Additionally, the LIC is disabled for DMVR and BDOF.

In the second aspect, the OBMC is enabled for the inter blocks that are coded with the LIC mode. And, to reduce the complexity, the OBMC is only applied to the top and left CU boundaries while being always disabled for the boundaries of the internal sub-blocks of one LIC CU. Additionally, when one neighboring block is coded with the LIC, its LIC parameters are applied to generate the corresponding prediction samples for the OBMC of one current block.

Test 2.7a: LIC for bi-prediction

Test 2.7b: Test 2.7a + OBMC with LIC

### Test 2.11e: Adaptive OBMC control (JVET-AD0193)

In the test, in addition to the existed SPS flag, OBMC is adaptively controlled on a block level as follows:

- OBMC flag is inherited from a neighboring affine block for affine merge mode.

- OBMC is not applied to a block if there is a neighbor block coded with IBC, palette, or BDPCM modes.

- When applying OBMC to a block, block boundary check whether OBMC is applied to the boundary is further made based on the reference samples of the current block. If any absolute difference between the prediction sample and non-interpolated (integer pel) reference sample is greater than a threshold, the OBMC is not applied to that boundary.

In the test, addition results are provided for the mixed context sequences from JVET-AC1015 “Common test conditions for SCM screen content coding” denoted as class M.

### Test 2.12: Prediction of MVD magnitude suffix bins (JVET-AD0140)

In ECM, MVD sign prediction is utilized. In the test, the most important difference between the MVD sign prediction and the tested method is in removing the restriction of predicting just MVD signs whereas the number of bins to be predicted is kept the same.

Most significant bins of MVD remainder suffixes are predicted using template matching and the correctness of prediction hypotheses are indicated by corresponding MVD suffix bins that are coded in the bitstream using regular CABAC mode. The proposed method is applied for MVDs of translational motion, including SMVD and affine modes. Candidates for template matching used to predict bin values are shown in Figure 8. The less significant bins of magnitude suffixes of horizontal and vertical MVD components may be coded in by-pass mode.

Figure 8. Prediction of MVD sign and magnitude suffix bins

Test 2.12a: Prediction of MVD magnitude suffix bins

Test 2.12f: ECM-8.0 with MVD sign prediction disabled

Test 2.12a*: Test 2.12 with MVD sign prediction disabled

Test 2.12h (JVET-AD0256): Test 2.12a + Test 2.7a

Test 2.12g (JVET-AD0257): Test 2.12a + Test 2.2

## Screen content coding

### Test 3.1: IntraTMP using reconstruction-reordered for screen content coding (JVET-AD0171)

In ECM, there is a reconstruction-reordered IBC mode. In the test, reconstruction-reordered IntraTMP mode is tested. When this mode is applied, the encoder and decoder search for the most similar template to the flipped current template in a predefined area of the search region. Only the current CTU (R1) and a portion of the left region (R4) are explored for the horizontal flip type. Otherwise, only the current CTU (R1) and a part of the above region (R3) are explored in the vertical flip mode.

The dimensions of search regions in R3 and R4 regions are five times of block dimension (CbHeight, CbWidth) as it is determined for the regular IntraTMP, as shown in Figure 9.

The flip type is selected among the regular TMP (non-flip), horizontally flipped, and vertical flipped TMP modes according to the lowest TM cost. Consequently, no additional signaling is required. The corresponding block is flipped according to flip type, horizontal or vertical flip, and used as a prediction block.

Figure 9. RR-IntraTMP search region for (a) horizontal and (b) vertical flip.

For horizontal flipping, in case the neighboring block is coded with a horizontal flip of RR-TMP, the horizontal component of BVcur is calculated by adding a motion shift to the horizontal component of BVnbr (denoted as BVhnbr), and the vertical component remains the same:

- BVhcur=2xnbr-xcur+BVhnbr

- BVvcur=BVvnbr

Similarly, in case the neighboring block is coded with a vertical flip, the vertical component of BVcur is calculated by adding a motion shift to the vertical component of BVnbr (denoted as BVvnbr), and the horizontal component remains invariant:

- BVhcur=BVhnbr

- BVvcur=2ynbr-ycur+BVvnbr

Where (xnbr,ynbr) and (xcur,ycur) represent the coordinates of the center sample of the neighboring block and the current block, respectively, BVnbr and BVcur denotes the BV of the neighboring block and the current block, respectively.

### Test 3.2: IBC MBVD list derivation (JVET-AD0199)

In the test, MBVD offsets are searched starting from fixed offsets defined by the initial step size along the MBVD direction. At every search step, the best 8 candidates are identified based on TM SAD cost and the search process repeats with the MVBD offsets checked around the identified candidates with the reduced step size by half. The search process stops when the step size reaches 1-pel resulting in the best 8 MBVD offset candidates.

When a MBVD candidate has a left or above template outside of the BV search area, the MBVD candidate is set to invalid candidate. When two BVP candidates have the same BV component in horizontal or vertical direction, the BV difference in other direction has to be larger than a threshold T.

The amount of MBVD candidates and MBVD index signaling is kept the same as ECM-8.0.

Test 3.2a: IBC MBVD list derivation with max BVD offset 128-pel.

The max BVD offset is 128-pel, initial search step size is 4-pel, BVP difference threshold T = 6-pel.

Test 3.2b: IBC MBVD list derivation with max BVD offset 256-pel.

The max BVD offset is equal to 256-pel, initial search step size is 4-pel, BVP difference threshold T = 32-pel.

### Test 3.3: Temporal block vector prediction (JVET-AD0095)

In ECM, IBC merge/AMVP candidates are derived from spatial, HMVP, pairwise average, and some predefined BV candidates. However, temporal BV candidates are not utilized.

In this test, a temporal BVP is introduced to mimic TMVP. Temporal BV candidates are added to IBC merge/AMVP candidate lists.

Temporal BV candidates are derived with the full pruning applied from the same temporal positions as TMVP, in the list they are added before the HMVP candidates.

### Test 3.4: Copy-padding for IBC (JVET-AD0096)

In this test, copy-padding can be applied to non-reconstructed area. With copy-padding, an unreconstructed sample in the overlapped with the current block region is padded by copying its prediction samples, as shown in Figure 10, and is represented as follows.

P’(x,y) = P(x+BVx, y+BVy)

wherein P’(x,y) is a padded sample at position (x, y), P(x+BVx, y+BVy) is a prediction sample, and (BVx, BVy) is the BV of the current block.

Copy-padding is performed only if the horizontal BV component is smaller than or equal to 0 and the vertical BV component is smaller than or equal to 0.

Figure 10. Unreconstructed samples (shaded) in the reference block are padded by copying their prediction samples

Test 3.4a: Copy-padding for IBC.

Test 3.4b: Test 3.4a + Test 3.3

### Test 3.7: Combination of Test 3.2 and Test 3.4b (JVET-AD0097)

The following tests are performed.

Test 3.7a: Test 3.2a + Test 3.4b

Test 3.7b: Test 3.2b + Test 3.4b

## In-loop filtering

### Test 4.1: Additional fixed-filter for ALF (JVET-AD0221)

In the test, one additional fixed filter with a diamond 7x7 shape and one signalled 7x7 cross filter shape are introduced to ALF shown in Figure 11 with turquoise colour, there is no classification for the newly added fixed filter.

The reconstructed samples before DBF are used as an input to the additional fixed filter to produce the filter output, then this filter output is used as an input to the introduced 7x7 cross signalled filter.

The coefficient signaling mechanism is the same as the current design in ECM. The proposed method is always enabled without any filter shape switching.

Figure 11. 7x7 cross signalled filter for ALF

### Test 4.2: ALF classifier based on residual data (JVET-AD0219)

In the test, a third classifier based on luma residual sample values is introduced. For each 2x2 luma block, the sum of absolute values of the residual samples in a neighbouring 8x8 window is calculated, and the class index is derived as:

classIdx = sum >> (sample bit depth – 4).

The value of classIdx is in the range of 0 to 24, same as in ECM-8.0. The classifier usage is signalled for each luma filter set in APS.

### Test 4.3: ALF filter shape with more taps applied to fixed filter results (JVET-AD0219)

In the test, the signalled ALF filter shape is modified, compared to the filter shape in ECM-8.0, the overall number of the filter coefficients for a signalled luma filter is increased by 2 as shown in Figure 12, the length of the filter that is applied to the neighboring reconstructed samples (taps #0-#9) is reduced and the length of the filter applied to the samples after the fixed-filter (taps #10-#29) is increased.

Figure 12. Modified filter shape of the signalled ALF filter.

### Test 4.4: Combination of ALF related tests

Test 4.4a (JVET-AD0219): Test 4.2 + Test 4.3

Test 4.4b (JVET-AD0222): Test 4.1 + Test 4.2 + Test 4.3

Test 4.4c (JVET-AD0222): Test 4.1 + Test 4.2

## GDR

### Test 5.1: Reference picture padding for GDR (JVET-AD0123)

In ECM, CUs in the “clean area” use the reference pictures where all reconstructed samples in the “dirty area” are set to 2bitdepth-1.

In this test, the “dirty area” is padded using samples from the “clean area” of the reference as shown in Figure 13.

Figure 13. Padding of the "dirty area"

The test was conducted using LB configuration following GDR test conditions described in EE2 (JVET-AC2024).

# Test results

See the accompanied Excel table.