| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0045-v1 |
| --- | --- |

| Title: | AHG10: Encoder MV selections and DMVR revisited |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, R. Yu | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

In JVET-W0061 it was pointed out that DMVR can cause visual artifacts at subblock boundaries when it fails to find the correct motion and an encoder only fix was proposed. The solution at that time was deemed to give too much objective overhead. In this contribution additional criterions to disable DMVR on the encoder side are proposed to reduce the overhead while removing visual artifacts. Firstly, the use of DMVR is always restricted for temporal layers lower than the highest temporal layer. When frame rate is equal or lower than 30Hz the use of DMVR is also restricted for the highest temporal layer. In higher temporal layers and when frame rate is higher than 30Hz it is generally better coherence between reference pictures which gives less problem to find the correct motion. If artifacts happen in the highest temporal layer they can not propagate to other pictures. Secondly, it is avoided to disable DMVR when average absolute subblock boundary differences are small when possible DMVR artifacts are less prominent.

The BDR effect on random access for VTM-20.0:

SDR CTC RA Y/Cb/Cr: 0.04%/0.06%/0.05%

HDR CTC RA DeltaE/PSNRL wY/wCb/wCr/: 0,07%/0,09% 0,08%/0,09%/0,13%

Similar encoding and decoding time as anchor.

# Introduction

Decoder side motion vector refinement (DMVR) can bring nice gains by enhancing the merge candidate bi-predictive motions on subblocks on the decoder side by searching in a small region around the merge candidate motion. In some cases when the spatial activity is low in at least one reference subblock it can be difficult to find a reliable match. In such cases artifacts from DMVR can appear as dislocated subblocks. This have been seen on several CTC and non-CTC test sequences.

In JVET-W0061 a solution was proposed but it was not accepted since it had too large objective overhead (BDR on CTC +0.19%). This contribution proposes additional enhancement of the original proposal to reduce objective overhead to make it more attractive and is asserted to fix subjective issues with DMVR.

# Proposal

In addition to checking subblock motion vector difference and subblock spatial activity it is also proposed to check subblock boundary activity and also to limit the checks to only be performed on temporal layers lower than the highest temporal layer when video frame rate is higher than 30Hz. The check on subblock spatial activity is also modified to use predicted samples instead of source samples and to only check samples inside the current subblock which makes the spatial activity more specific and relevant to detect areas that may be difficult for DMVR to find a good motion for. The subblock boundary activity is computed as the average absolute sample difference at the left respectively above subblock boundary. The method is only enabled for blocks with size equal or greater than 64x64 and for QPs greater than 33. For such blocks DMVR is disabled when the spatial activity is less than a QP dependent spatial activity threshold, the boundary activity is greater than a boundary activity threshold and the subblock motion vector difference for at least one component is equal or greater than 28 (in 1/16 sample accuracy).

# Results

The objective performance is compared against CTC [2] based on VTM-20.0 [3] and also for the case of enabling GOP based RPR in VTM-20.0.

## Comparison for CTC

|  | Proposal over VTM-20.0 Random access (SDR) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,06% | 0,09% | 0,07% | 101% | 101% |
| Class A2 | 0,09% | 0,11% | 0,12% | 100% | 100% |
| Class B | 0,02% | 0,05% | 0,04% | 100% | 100% |
| Class C | 0,00% | 0,02% | 0,01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0,04% | 0,06% | 0,05% | 100% | 100% |
| Class D | 0,00% | 0,01% | 0,01% | 100% | 101% |
| Class F | 0,00% | -0,01% | 0,00% | 101% | 99% |

|  | Random Access (HDR) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Proposal over VTM-20.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0,07% | 0,09% | 0,08% | 0,09% | 0,13% | 0,07% | 0,09% | 0,12% | 100% | 100% |
| Class H2 |  |  |  |  |  | 0,05% | 0,07% | 0,07% | 101% | 101% |
| Overall | 0,07% | 0,09% | 0,08% | 0,09% | 0,13% | 0,06% | 0,08% | 0,10% | 101% | 100% |

Impact of turning off DMVR (DMVR=0) for QP greater than 33 is shown below. The encoding and decoding time in this case is not reliable.

|  | Over VTM-20.0 Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,22% | 0,21% | 0,21% | 100% | 98% |
| Class A2 | 0,42% | 0,43% | 0,45% | 100% | 100% |
| Class B | 0,20% | 0,24% | 0,23% | 100% | 98% |
| Class C | 0,19% | 0,22% | 0,21% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0,25% | 0,27% | 0,27% | 100% | 99% |
| Class D | 0,21% | 0,21% | 0,25% | 105% | 103% |
| Class F | 0,08% | 0,10% | 0,10% | 100% | 101% |

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-20.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0,32% | 0,24% | 0,22% | 0,32% | 0,37% | 0,22% | 0,31% | 0,36% | 98% | 98% |
| Class H2 |  |  |  |  |  | 0,19% | 0,23% | 0,23% | 98% | 100% |
| Overall | 0,32% | 0,24% | 0,22% | 0,32% | 0,37% | 0,21% | 0,28% | 0,31% | 98% | 99% |

## Comparison for GOP based RPR at higher QPs

We also compare performance on a high QP range (QP 37, 40, 43, 46 and 49) when also GOP based RPR encoder control (enabled by config parameter GOPBasedRPR = 1) is used since GOP based RPR generally gives better performance for higher QPs. The test content is six 4K SDR sequences and one 4K HLG sequence, all 10s long.

GOP based RPR vs VTM-20.0: QP 37, 40, 43, 46 and 49

| Sequence | Y | U | V |
| --- | --- | --- | --- |
| NeptuneFountain3_3840x2160_60fp | -4,93% | -49,29% | -39,14% |
| MountainBay2_3840x2160_30fps_42 | -1,93% | -28,02% | -40,10% |
| Campfire_3840x2160_30fps_bt709_ | -9,26% | 1,07% | -5,96% |
| DaylightRoad2_3840x2160_60fps_1 | -5,92% | -14,52% | -9,01% |
| ParkRunning3_3840x2160_50fps_10 | -6,99% | 49,42% | 24,89% |
| DrivingPOV3_3840x2160_60fps_10b | -5,63% | -14,89% | -4,77% |
| H3_AMS01_3840x2160_10bit_420_HL | 0,00% | 0,00% | 0,00% |
| Average | -4,95% | -8,03% | -10,59% |

DMVR off (GOP based RPR anchor): QP 37, 40, 43, 46 and 49

| Sequence | Y | U | V |
| --- | --- | --- | --- |
| NeptuneFountain3_3840x2160_60fp | 0,86% | 2,63% | 1,05% |
| MountainBay2_3840x2160_30fps_42 | 2,59% | 3,88% | 2,86% |
| Campfire_3840x2160_30fps_bt709_ | 0,17% | 0,18% | 0,42% |
| DaylightRoad2_3840x2160_60fps_1 | 4,27% | 4,29% | 4,06% |
| ParkRunning3_3840x2160_50fps_10 | 1,78% | 2,67% | 2,68% |
| DrivingPOV3_3840x2160_60fps_10b | 3,71% | 4,29% | 4,32% |
| H3_AMS01_3840x2160_10bit_420_HL | 2,84% | 2,70% | 2,57% |
| Average | 2,32% | 2,95% | 2,56% |

DMVR encoder control proposal (GOP based RPR anchor): QP 37, 40, 43, 46 and 49

| Sequence | Y | U | V |
| --- | --- | --- | --- |
| NeptuneFountain3_3840x2160_60fp | 0,08% | 0,12% | -0,35% |
| MountainBay2_3840x2160_30fps_42 | 0,44% | 0,78% | 0,92% |
| Campfire_3840x2160_30fps_bt709_ | 0,01% | 0,11% | 0,14% |
| DaylightRoad2_3840x2160_60fps_1 | 1,12% | 1,46% | 1,41% |
| ParkRunning3_3840x2160_50fps_10 | 0,13% | 0,26% | 0,18% |
| DrivingPOV3_3840x2160_60fps_10b | 0,69% | 1,05% | 1,13% |
| H3_AMS01_3840x2160_10bit_420_HL | 1,16% | 1,49% | 1,52% |
| Average | 0,52% | 0,75% | 0,71% |

# Conclusions

It has been shown that the BDR penalty for restricting the use of DMVR is relatively small compared to the gain of DMVR for CTC. For higher QPs when its beneficial to use GOP based encoder control the loss is 0.5% on average where the loss of turning off DMVR is 2.3%. It is proposed to at least include the fix as an optional feature for the VVC reference software but possibly also having it enabled by default for GOP based RPR to make sure best subjective quality is obtained when the reference software is used.

# References

[1] R. Yu, K. Andersson, “AHG10: Encoder MV selections and DMVR”, JVET-W0061, Online, 7-16 July, 2021.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Meeting by Teleconference, January 2021

[3] VTM-20.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-20.0

[4] A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-T2011, Meeting by Teleconference, October 2020.

[5] S. Iwamura, S. Nemoto, A. Ichigaya, “4K HLG test sequences for HDR verification test”, JVET-S0218, Meeting by Teleconference, June 2020.

# Patent rights declaration

Ericsson AB may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).