| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0070-v1 |
| --- | --- |

| Title: | EE1-1.9: Improvement over multi-frame model for NNVC filter set1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the results of EE1-1.9, which was described in JVET-AC0355. In JVET-AC0355, a slice-level model switching method was proposed to further enhance the performance of filter set#1. Specifically, it is proposed to switch the original luma model and the multi-frame model for B slices at slice level. One slice-level flag is signalled to indicate which model is used for the current slice. On top of NNVC-4.0 filter set#1, the performance of this EE can be summarized as below:

Compared with NNVC-4.0 filter set#1 with NnlfSet1Multiframe=1,

RA: {0.00%, 0.00%, 0.00%} EncT 100%, DecT 100%;

LDB: {-1.44%, -0.08%, -0.26%} EncT 103%, DecT 103%.

Compared with NNVC-4.0 EE anchor,

RA: {-10.23%	, -20.88%	, -21.43%} EncT 162%, DecT 33669%;

LDB: {-10.00%, -19.77%, -20.95%} EncT 187%, DecT 25533%.

Introduction

In NNVC-4.0 filter set#1[1], a multi-frame model[2] is applied to those B slices belonging to temporal layers with tid > 2. Fig.1 shows the architecture of this multiple frame-based CNN model.

Fig.1 Architecture of multiple frame-based CNN model

Apart from the reconstructed blocks, the prediction blocks, and QP information, this multi-frame model takes the collocated blocks from pictures in the reference picture list 0 and 1 as additional inputs.

Proposed

In JVET-AC0355[3], two aspects were proposed to further improve the coding efficiency for filter set#1.

In the first aspect, it is proposed that the original inter-intra luma model and the multiple frame-based model may be switched at the slice level for temporal layer slices with tid <= 2. For higher temporal layer slices, which is the same as nnvc-4.0 filter set#1, the multiple frame-based model remains used in B slices by default. A slice-level flag is signalled for lower temporal layer B slices to indicate which model is used. If the flag is true, all CTUs in the slice use the multiple frame-based model. Otherwise, all CTUs in the slice use the default inter-intra luma model.

In the second aspect, if the picture POC from RPL0 is equal to the picture POC from RPL1 and there are at least two reference pictures in RPL1, then the second reference picture is taken as Col_1.

Inference

The inference information is identical to NNVC-4.0 filter set#1.

Table. 1 Inference Information

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 4.66M |
|  | Parameter Precision (Bits) | 16 (I) |
|  | Memory Parameter (MB) | 9.3MB |
|  | Multiply Accumulate (kMAC/pixel) | 539K/pixel (assuming frame-level input) 682K/pixel (assuming block-level input) |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

No training is conducted in this contribution.

Experimental results

The proposed method is implemented on top of NNVC-4.0 filter set#1 and all of the tests are conducted under the common test condition[4]. When the multi-frame model method is enabled, it means that the configuration of NnlfOption is set to 2 and NnlfSet1Multiframe is set to 1.

Compared with NnlfOption=2 and Nnlfset1Multiframe=1

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.48% | -0.06% | -0.06% | 102% | 98% |
| Class C | -1.35% | 0.07% | -0.16% | 102% | 100% |
| Class E | -1.51% | -0.32% | -0.73% | 104% | 115% |
| Overall | -1.44% | -0.08% | -0.26% | 103% | 103% |
| Class D | -1.19% | 0.28% | -1.03% | 102% | 100% |
| Class F | -0.91% | 0.39% | -0.13% | 104% | 100% |

Compared with NNVC-4.0 EE anchor (NnlfOption=0)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.39% | -16.40% | -20.54% | 164% | 34420% |
| Class A2 | -10.69% | -20.18% | -16.05% | 156% | 33950% |
| Class B | -9.72% | -22.15% | -22.91% | 173% | 34706% |
| Class C | -11.16% | -23.19% | -24.27% | 152% | 31685% |
| Class E |  |  |  |  |  |
| Overall | -10.23% | -20.88% | -21.43% | 162% | 33669% |
| Class D | -13.22% | -24.18% | -27.26% | 155% | 34690% |
| Class F | -5.60% | -14.26% | -12.45% | 222% | 14717% |

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -9.02% | -19.77% | -20.60% | 172% | 27407% |
| Class C | -10.79% | -21.69% | -21.92% | 152% | 25242% |
| Class E | -10.56% | -17.20% | -20.23% | 283% | 23041% |
| Overall | -10.00% | -19.77% | -20.95% | 187% | 25533% |
| Class D | -12.25% | -24.73% | -27.95% | 147% | 22799% |
| Class F | -6.45% | -13.66% | -10.69% | 229% | 13150% |

Conclusion

This contribution reports the results of EE1-1.9, which proposed a slice-level model switching method for filter set#1. For LDB, this method can provide -1.44% coding gain with a minor running time increase. Considering the good trade-off performance and the discussions in the last meeting, it is suggested to adopt this method into NNVC common software as part of JVET-AC-EE1-1.7.

Reference

[1] Y. Li, K. Zhang, L. Zhang. EE1-1.6: Deep In-Loop Filter With Fixed Point Implementation. JVET-AA0111, 27th Meeting, by teleconference, 13-22 July 2022.

[2] Y. Li, K. Zhang, L. Zhang, D. Liu, J. Ström, M. Damghanian, P. Wennersten, K. Andersson. EE1-related: Combination of EE1-1.5 and EE1-1.7, JVET-AC0310, by teleconference, 11-20 Jan 2023.

[3] Z. Xie, Y. Yu, H. Yu, D. Wang. EE1-related: On combination of EE1-1.5 and EE1-1.7. JVET-ACC0355, 29th Meeting, by teleconference, 11-20 Jan 2023.

[4] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AC2016, 29th Meeting, by teleconference, 11-20 Jan 2023.

Appendix

The proponent of JVET-AC0310 recently proposed a correction to the QP input for the inter-intra luma model in filter set#1. Hence the proposed correction has also been integrated into this EE and tested.

6.1 shows the results of the proposed method with QP input correction.

3.1 with QP-input correction

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.67% | 0.16% | 0.26% | 103% | 102% |
| Class C | -0.68% | 0.84% | 0.82% | 102% | 100% |
| Class E | -1.17% | -0.64% | 0.23% | 105% | 111% |
| Overall | -0.80% | 0.19% | 0.44% | 103% | 103% |
| Class D | -0.28% | -0.05% | 0.28% | 102% | 100% |
| Class F | -0.42% | 0.45% | 1.22% | 104% | 102% |

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).