| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0087-v1 |
| --- | --- |

| Title: | AHG9: On cascading of post-processing filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Li Zhang1, Jizheng Xu1, Chaoyi Lin2, Yue Li1, Junru Li3, Kai Zhang1 18910 University Center Lane San Diego, CA 92122, USA 2Bafangcheng 9th building F14 Yuhang District, 311100, Hangzhou City, Zhejiang Province, China 3Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com lizhang.idm@bytedance.com xujizheng@bytedance.com linchaoyi.cy@bytedance.com yue.li@bytedance.com lijunru@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

Currently, activation of multiple NNPFs for a picture is allowed. However, when there are multiple NNPFs activated for a picture, except for the special case of two NNPFs doing spatial resolution upsampling followed by picture rate upsampling, for all other cases, it is unclear how the multiple activated NNPFs are applied, neither in terms of preferred processing order nor in terms of which pictures being the input pictures to the NNPFs other than the first.

This seems inconsistent from a basic design point of view. Further adding to the inconsistency is the ongoing work of the SEI processing order SEI message, which indicates preferred processing order of different types of post-processing filters and has been tentatively planned for the fourth edition of VVC.

This contribution proposes to address the inconsistency issue by either of the following two options:

- Option 1: Clearly allow cascading of post-processing filters (NNPFs or non-NN post-processing filters) signalled by SEI messages and clearly specify preferred processing order when cascading.

- Option 2: Clearly disallow cascading of post-processing filters (NNPFs or non-NN post-processing filters) signalled by SEI messages.

Option 1 includes some sub-options and is summarized as follows:

- Option 1.1: Update the SEI processing SEI message to cover NNPFs, and include the SEI processing order SEI message into VVC v3.

- Option 1.2: Extend the SEI processing SEI message to include NNPFs but still plan the SEI processing order SEI message for VVC v4.

- Option 1.3: Address the signalling of preferred processing order of NNPFs in VSEI v3, leaving the SEI processing SEI message for different types of non-neural-network post-processing filters.

  - Option 1.3.1: Use the value of nnpfc_id to indicate the preferred processing order of NNPFs.

  - Option 1.3.2: Use a new SEI message to indicate the preferred processing order of NNPFs, e.g., like the proposal in JVET-AC0299.

Option 2 includes all of the following actions:

- Explicitly disallow the activation of more than one post-processing filter (NNPFs or non-NN post-processing filters) for any particular picture.

- Remove process for the only currently allowed special-case cascading of NNPFs doing spatial resolution upsampling followed by picture rate upsampling.

- Cancel the development of the SEI processing SEI message.

# Proposal option 1: clearly allow cascading and clearly specify preferred processing order when cascading

## Summary of proposal option 1

There are three sub-options herein:

- Option 1.1: Update the SEI processing SEI message to cover NNPFs, and include the SEI processing order SEI message into VVC v3.

- Option 1.2: Extend the SEI processing SEI message to include NNPFs but still plan the SEI processing order SEI message for VVC v4.

- Option 1.3: Address the signalling of preferred processing order of NNPFs in VSEI v3, leaving the SEI processing SEI message for different types of non-neural-network post-processing filters.

  - Option 1.3.1: Use the value of nnpfc_id to indicate the preferred processing order of NNPFs.

  - Option 1.3.2: Use a new SEI message to indicate the preferred processing order of NNPFs, e.g., like the proposal in JVET-AC0299.

## Text changes for updating the SEI processing SEI message to cover NNPFs

In the text changes herein and in subsections below, additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts.

## D.11 SEI processing order SEI message

D.11.1 SEI processing order SEI message syntax

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_sei_num_payload_types_minus1 | ue(v) |
| for( i = 0, b = 0; b < payloadSize; i++, b += 4 ) { |  |
| for( i = 0; i <= po_sei_num_payload_types_minus1; i++ ) { |  |
| po_sei_payload_type[ i ] | u(16) |
| if( po_sei_payload_type[ i ] = = 4 ) { |  |
| po_num_t35_byte[ i ] | b(8) |
| b++ |  |
| for( j = 0; j < po_num_t35_bytes[ i ]; j++ ) |  |
| po_t35_byte[ i ][ j ] | b(8) |
| b += po_num_t35_byte[ i ] |  |
| } else if( po_sei_payload_type[ i ] = = 210 ) { /* 210: the NNPFC SEI message */ |  |
| po_sei_num_nnpfs_minus1[ i ] | ue(v) |
| for( j = 0; j <= po_sei_num_nnpfs_minus1[ i ]; j++ ) |  |
| po_sei_nnpf_id[ i ][ j ] | ue(v) |
| } |  |
| numPostProcessingFilters[ i ] = ( po_sei_payload_type[ i ] = = 210 ) ? po_sei_num_nnpfs_minus1[ i ] + 1 : 1 |  |
| for( j = 0; j < numPostProcessingFilters[ i ]; j++ ) |  |
| po_sei_processing_order[ i ][ j ] | u(16) |
| } |  |
| } |  |

D.11.2 SEI processing order SEI message semantics

The SEI processing order SEI message carries information indicating the preferred processing order, as determined by the encoder (i.e., the content producer), for different types of SEI messages that may be present in a CVS.

When an SEI processing order SEI message is present in any access unit of a CVS, an SEI processing order SEI message shall be present in the first access unit of the CVS. The SEI processing order SEI message persists in decoding order from the current access unit until the end of the CVS. When there are multiple SEI processing order SEI messages present in a CVS, they shall have the same content.

It is a requirement of bitstream conformance that, within an SEI processing order SEI message, there shall be at least two pairs instances of the syntax elements po_sei_payload_type[ i ] and po_sei_processing_order[ i ][ j ], and there shall be at least two values of po_sei_processing_order[ i ][ j ] that are not equal.

po_sei_num_payload_types_minus1 plus 1 specifies the number of the po_sei_payload_type[ i ] syntax elements in the SEI processing order SEI message. The value of po_sei_num_entries_minus1 shall be in the range of 0 to 255, inclusive.

po_sei_payload_type[ i ] specifies the payloadType value of the i-th SEI message type for which preferred processing order information is provided in the SEI processing order SEI message. For any two different non-negative integer values of m and n, the values of po_sei_payload_type[ m ] and po_sei_payload_type[ n ] shall not be identical unless they are both equal to 4.

po_num_t35_byte[ i ], when present, specifies the number of bytes associated with the i-th user data registered by Recommendation ITU-T T.35 SEI message for which preferred processing order information is provided in the SEI processing order SEI message. When not present, the value of po_num_t35_byte[ i ] is inferred to be equal to 0. po_num_t35_byte[ i ] equal to 0 indicates that there is no preferred order of processing between user data registered by Recommendation ITU-T T.35 SEI messages.

po_t35_byte[ i ][ j ], when present, specifies the j-th byte value of the i-th user data registered by Recommendation ITU-T T.35 SEI message.

po_sei_num_nnpfs_minus1[ i ] plus 1 specifies the number of NNPFs for which information is provided in the SEI processing order SEI message. The value of po_sei_num_nnpfs_minus1[ i ] shall be in the range of 0 to 255, inclusive.

po_sei_nnpf_id[ i ][ j ] indicates the value of the nnpfc_id of the j-th NNPF for which information is provided in the SEI processing order SEI message. The values of po_sei_nnpf_id[ i ][ m ] and po_sei_nnpf_id[ i ][ n ] shall not be identical when m is not equal to n.

po_sei_processing_order[ i ][ j ] indicates the preferred order of processing of the post-processing filter signalled by the SEI message with the i-th SEI message type and, when po_sei_payload_type[ i ] is equal to 4, with bytes po_t35_byte[ n ] for n ranging from 0 to po_num_t35_byte[ i ] − 1, inclusive, or when po_sei_payload_type[ i ] is equal to 210, with nnpfc_id equal to po_sei_nnpf_id[ i ][ j ], for which preferred processing order information is provided in the SEI processing order SEI message.

For any two different pairs of non-negative integer values of m and n {a, b} and {c, d} that are greater than or equal to 0, po_sei_processing_order[ a ][ b ][ m ] less than po_sei_processing_order[ c ][ d ][ n ] indicates that 1) any SEI message type with payloadType equal to po_sei_payload_type[ a ][ m ] and, when present po_sei_payload_type[ a ] is equal to 4, with bytes po_t35_byte[ a ][ m ][ p ] for p ranging from 0 to po_num_t35_byte[ a ][ m ] − 1, inclusive, or when po_sei_payload_type[ a ] is equal to 210, with nnpfc_id equal to po_sei_nnpf_id[ a ][ b ], should be processed before any SEI message type with payloadType equal to po_sei_payload_type[ c ][ n ], and, when present po_sei_payload_type[ c ] is equal to 4, with bytes po_t35_byte[ c ][ n ][ q ] for q ranging from 0 to po_num_t35_byte[ c ][ m ] − 1, inclusive, or when po_sei_payload_type[ c ] is equal to 210, with nnpfc_id equal to po_sei_nnpf_id[ c ][ d ], and 2) po_sei_processing_order[ a ][ b ][ m ] equal to po_sei_processing_order[ c ][ d ][ n ] indicates that there is no preferred order of processing between the SEI message types.

When there are multiple user data registered by Recommendation ITU-T T.35 SEI messages with the same content in a CVS, they shall have the same SEI processing order value.

## Text changes for using the value of nnpfc_id to indicate the preferred processing order of NNPFs

...

nnpfc_id contains an identifying number that may be used to identify an NNPF and indicates the preferred order of processing of the NNPF. The value of nnpfc_id shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfc_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering an NNPFC SEI message with nnpfc_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message.

A value of nnpfc_id in a first NNPFC SEI message less than the value of nnpfc_id in a second NNPFC SEI message indicates that the NNPF specified by the first NNPFC SEI message, if activated by an NNPFA SEI message, should be processed before the NNPF specified by the second NNPFC SEI message, if activated by an NNPFA SEI message.

...

# Proposal option 2: clearly disallow cascading

## Summary of proposal option 2

This option includes all of the following actions:

- Explicitly disallow the activation of more than one post-processing filter (NNPFs or non-NN post-processing filters) for any particular picture.

- Remove process for the only currently allowed special-case cascading of NNPFs doing spatial resolution upsampling followed by picture rate upsampling.

- Cancel the development of the SEI processing SEI message.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).