| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0089-v1 |
| --- | --- |

| Title: | AHG9: NNPF activation parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Yue Li1, Jizheng Xu1, Chaoyi Lin2, Junru Li3, Li Zhang1, Kai Zhang1 18910 University Center Lane San Diego, CA 92122, USA 2Bafangcheng 9th building F14 Yuhang District, 311100, Hangzhou City, Zhejiang Province, China 3Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com yue.li@bytedance.com xujizheng@bytedance.com linchaoyi.cy@bytedance.com lijunru@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

It is asserted that the NNPFC SEI message is supposed to signal the property of a neural network used for certain post-filtering purpose. However, in the current design, some syntax elements in the NNPFC SEI message are not part of the property of the NNPF, but rather depending on other factors such as the property of the video bitstream containing the NNPFC SEI message, the location where the NNPF is activated, etc.

One example of such NNPFC syntax elements is nnpfc_input_pic_output_flag[ i ], for which the value can change depending on for which picture the NNPF involving picture rate upsampling and possibly other types of upsampling is activated. For example, assuming that the NNPF does both picture rate upsampling and spatial resolution upsampling, there are two input pictures, and one picture is interpolated between the two input pictures, and the bitstream contains 3 pictures with POC values 0, 2, and 4. In this case, the NNPF can be activated for picture 2 (i.e., the picture with POC equal to 2) with pictures 0 and 2 as input, and the NNPF outputs pictures 0, 1, and 2, i.e., the nnpfc_input_pic_output_flag[ i ] would be equal to 1 for both input pictures 1 and 2 herein. Next, the NNPF can be activated for picture 4 with pictures 2 and 4 as input, and the NNPF outputs pictures 3 and 4, i.e., the nnpfc_input_pic_output_flag[ i ] would be equal to 1 for the input picture 4 but would be equal to 0 for the input picture 2. If this time the nnpfc_input_pic_output_flag[ i ] is also equal to 1 for both the input pictures, then the NNPF would output picture 2 twice, which can be problematic if not handled properly.

As can be seen, this flag (nnpfc_input_pic_output_flag[ i ]) is not a property of the NNPF, but rather it should be a parameter that is passed to the NNPF when the NNPF is activated.

One consequence of having this flag as a syntax element in the NNPFC SEI message syntax is that, as in the above example, multiple NNFPC SEI messages containing the same ISO/IEC 15938-17 bitstream are needed, while usually the size of an ISO/IEC 15938-17 bitstream contained in an NNPFC SEI message is large.

Syntax elements currently in the NNPFC SEI message that appear better be activation parameters, i.e., parameters passed to the NNPF during activation, include the following:

- nnpfc_input_pic_output_flag[ i ]

- nnpfc_out_order_idc

- nnpfc_overlap

- nnpfc_padding_type

- nnpfc_luma_padding_val

- nnpfc_cb_padding_val

- nnpfc_cr_padding_val

It is therefore proposed to specify a mechanism that enables passing of certain parameters to an NNPF when the NNPF is activated, including the above-listed syntax elements.

Furthermore, it is suggested to have a discussion on whether nnpfc_component_last_flag is needed at all, and if needed, whether it should be an activation parameter, or a parameter signalled in the NNPFC SEI message.

# An analysis of which NNPFC syntax elements should be signalled as NNPF activation parameters

| Syntax element | Part of network topology? | Part of the trained network parameters? | Can the network work with different values? | Is value change meaningful (beneficial?) Only valid when the previous answer is Y. | Better in NNPFC or as activation parameter through NNPFA? |
| --- | --- | --- | --- | --- | --- |
| nnpfc_purpose | Y | Y |  |  | C |
| nnpfc_id | Y | Y |  |  | C |
| nnpfc_mode_idc | Y | Y |  |  | C |
| nnpfc_reserved_zero_bit_a1 |  |  |  |  | C |
| nnpfc_tag_uri | Y | Y |  |  | C |
| nnpfc_uri | Y | Y |  |  | C |
| nnpfc_property_present_flag | Y | Y |  |  | C |
| nnpfc_base_flag | N | N | N |  | C |
| nnpfc_num_input_pics_minus1 | Y | Y | N |  | C |
| nnpfc_out_sub_c_flag | Y | Y | N |  | C |
| nnpfc_out_colour_format_idc | Y | Y | N |  | C |
| nnpfc_pic_width_in_luma_samples | Y | Y | N |  | C |
| nnpfc_pic_height_in_luma_samples | Y | Y | N |  | C |
| nnpfc_interpolated_pics | Y | Y | N |  | C |
| nnpfc_input_pic_output_flag | N | N | Y | Y | A |
| nnpfc_component_last_flag | Y |  |  |  | C |
| nnpfc_inp_format_idc | Y | Y | N |  | C |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | Y | Y | N |  | C |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | Y | Y | N |  | C |
| nnpfc_inp_order_idc | Y | Y | N |  | C |
| nnpfc_auxiliary_inp_idc | Y | Y | N |  | C |
| nnpfc_separate_colour_description_present_flag | N | Y | N |  | C |
| nnpfc_colour_primaries | N | Y | N |  | C |
| nnpfc_transfer_characteristics | N | Y | N |  | C |
| nnpfc_matrix_coeffs | N | Y | N |  | C |
| nnpfc_out_format_idc | Y | Y | N |  | C |
| nnpfc_out_tensor_luma_bitdepth_minus8 | Y | Y | N |  | C |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | Y | Y | N |  | C |
| nnpfc_out_order_idc | N | N | Y | Y | A |
| nnpfc_overlap | N | N | Y | Y | A |
| nnpfc_constant_patch_size_flag | Y | Y | N |  | C |
| nnpfc_patch_width_minus1 | Y | Y | N |  | C |
| nnpfc_patch_height_minus1 | Y | Y | N |  | C |
| nnpfc_extended_patch_width_cd_delta_minus1 | Y | Y | N | N | C |
| nnpfc_extended_patch_height_cd_delta_minus1 | Y | Y | N | N | C |
| nnpfc_padding_info_present_flag2 |  |  |  |  | C |
| nnpfc_padding_type | N | N | Y | Y | A |
| nnpfc_luma_padding_val | N | N | Y | Y | A |
| nnpfc_cb_padding_val | N | N | Y | Y | A |
| nnpfc_cr_padding_val | N | N | Y | Y | A |
| nnpfc_complexity_info_present_flag2 |  |  |  |  | C |
| nnpfc_parameter_type_idc2 |  |  |  |  | C |
| nnpfc_log2_parameter_bit_length_minus32 |  |  |  |  | C |
| nnpfc_num_parameters_idc2 |  |  |  |  | C |
| nnpfc_num_kmac_operations_idc2 |  |  |  |  | C |
| nnpfc_total_kilobyte_size2 |  |  |  |  | C |
| nnpfc_reserved_zero_bit_b1 |  |  |  |  | C |
| nnpfc_payload_byte2 |  |  |  |  | C |

1Byte alignment, necessary but not about the network.

On nnpfc_component_last_flag, the authors had different opinions. Some thought that it is clearly not a property of the network, but just an implementation difference on an array hierarchy, which determines the location of certain data elements in the memory or in the hard disk, but has nothing to do with the post processing itself, thus it should be an activation parameter. Some other thought that transposing an multi-dimensional array can be a tedious task for some implementations.

We'd suggest a discussion on whether nnpfc_component_last_flag is actually needed at all.

# Proposed text changes

Added or modified parts are highlighted in turquoise and deleted parts are highlighted in yellow strikethrough in red fonts.

### 8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| ... |  |
| if( ( nnpfc_purpose & 0x08 ) != 0 ) |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_output_flag[ i ] | u(1) |
| } |  |
| ... |  |
| nnpfc_out_format_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_order_idc | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| if( nnpfc_constant_patch_size_flag ) { |  |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| } else { |  |
| nnpfc_extended_patch_width_cd_delta_minus1 | ue(v) |
| nnpfc_extended_patch_height_cd_delta_minus1 | ue(v) |
| } |  |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ) { |  |
| nnpfc_luma_padding_val | ue(v) |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| } |  |
| ... |  |
| } |  |
| ... |  |
| } |  |

### 8.28.2 Neural-network post-filter characteristics SEI message semantics

...

nnpfc_interpolated_pics[ i ] specifies the number of interpolated pictures generated by the NNPF between the i-th and the ( i + 1 )-th picture used as input for the NNPF. The value of nnpfc_interpolated_pics[ i ] shall be in the range of 0 to 63, inclusive. The value of nnpfc_interpolated_pics[ i ] shall be greater than 0 for at least one i in the range of 0 to nnpfc_num_input_pics_minus1 − 1, inclusive.

For each i from 0 to nnpfc_num_input_pics_minus1, inclusive, the variable nnpfcInputPicOutputFlag[ i ] is set equal to 0.

When an NNPF is activated by an NNPFA SEI message with nnpfa_target_id equal to the nnpfc_id of the NNPF, the list nnpfcInputPicOutputFlag is updated as follows, where nnpfa_num_activation_param_types, nnpfa_activation_param_type[ j ], and nnpfa_activation_param_val[ j ][ i ] are syntax elements in the NNPFA SEI message:

for( j = 0; j < nnpfa_num_activation_param_types; j++ )
	if( nnpfa_activation_param_type[ j ]  = =  0 )		(X)
		for( i = 0; i < nnpfc_num_input_pics_minus1; i++ )
			nnpfcInputPicOutputFlag[ i ] = nnpfa_activation_param_val[ j ][ i ]

nnpfc_input_pic_output_flag nnpfcInputPicOutputFlag[ i ] equal to 1 indicates that for the i-th input picture the NNPF generates a corresponding output picture. nnpfcInputPicOutputFlag[ i ] equal to 0 indicates that for the i-th input picture the NNPF does not generate a corresponding output picture.

The variables numInputPics, specifying the number of pictures used as input for the NNPF, and numOutputPics, specifying the total number of pictures resulting from the NNPF, are derived as follows:

numInputPics = nnpfc_num_input_pics_minus1 + 1
if( ( nnpfc_purpose & 0x08 )  !=  0 ) {
	for( i = 0, numOutputPics = 0; i < numInputPics; i++ )
		if( nnpfcInputPicOutputFlag[ i ] )
			numOutputPics++
	for( i = 0; i  <=  numInputPics − 2; i++ )		(76)
		numOutputPics  +=  nnpfc_interpolated_pics[ i ]
} else
	numOutputPics = 1

...

nnpfc_auxiliary_inp_idc greater than 0 indicates that auxiliary input data is present in the input tensor of the NNPF. nnpfc_auxiliary_inp_idc equal to 0 indicates that auxiliary input data is not present in the input tensor. nnpfc_auxiliary_inp_idc equal to 1 specifies that auxiliary input data is derived as specified in Formula 84.

...

The process DeriveInputTensors( ), for deriving the input tensor inputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numInputPics; i++ ) {
	if( nnpfc_inp_order_idc  = =  0 )
		for( yP = −nnpfcOverlap; yP < inpPatchHeight + nnpfcOverlap; yP++)
			for( xP = −nnpfcOverlap; xP < inpPatchWidth + nnpfcOverlap; xP++ ) {
				inpVal = InpY( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				yPovlp = yP + nnpfcOverlap
				xPovlp = xP + nnpfcOverlap
				if( !nnpfc_component_last_flag )
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpVal
				else
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpVal
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = strengthControlScaledVal
			}
	else if( nnpfc_inp_order_idc  = =  1 )		(84)
		for( yP = −nnpfcOverlap; yP < inpPatchHeight + nnpfcOverlap; yP++)
			for( xP = −nnpfcOverlap; xP < inpPatchWidth + nnpfcOverlap; xP++ ) {
				inpCbVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ] ) )
				inpCrVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ] ) )
				yPovlp = yP + nnpfcOverlap
				xPovlp = xP + nnpfcOverlap
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = strengthControlScaledVal
			}
	else if( nnpfc_inp_order_idc  = =  2 )
		for( yP = −nnpfcOverlap; yP < inpPatchHeight + nnpfcOverlap; yP++)
			for( xP = −nnpfcOverlap; xP < inpPatchWidth + nnpfcOverlap; xP++ ) {
				yY = cTop + yP
				xY = cLeft + xP
				yC = yY / SubHeightC
				xC = xY / SubWidthC
				inpYVal = InpY( InpSampleVal( yY, xY, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ] ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ] ) )
				yPovlp = yP + nnpfcOverlap
				xPovlp = xP + nnpfcOverlap
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpYVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpYVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = strengthControlScaledVal
			}
	else if( nnpfc_inp_order_idc  = =  3 )
		for( yP = −nnpfcOverlap; yP < inpPatchHeight + nnpfcOverlap; yP++)
			for( xP = −nnpfcOverlap; xP < inpPatchWidth + nnpfcOverlap; xP++ ) {
				yTL = cTop + yP * 2
				xTL = cLeft + xP * 2
				yBR = yTL + 1
				xBR = xTL + 1
				yC = cTop / 2 + yP
				xC = cLeft / 2 + xP
				inpTLVal = InpY( InpSampleVal( yTL, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpTRVal = InpY( InpSampleVal( yTL, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpBLVal = InpY( InpSampleVal( yBR, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpBRVal = InpY( InpSampleVal( yBR, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCbPic[ i ] ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCrPic[ i ] ) )
				yPovlp = yP + nnpfcOverlap
				xPovlp = xP + nnpfcOverlap
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpTLVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpTRVal
					inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpBLVal
					inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = inpBRVal
					inputTensor[ 0 ][ i ][ 4 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 5 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpTLVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpTRVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpBLVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = inpBRVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 4 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 5 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 6 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 6 ] = strengthControlScaledVal
			}
}

...

nnpfc_matrix_coeffs has the same semantics as specified in subclause 7.3 for the vui_matrix_coeffs syntax element, except as follows:

–	nnpfc_matrix_coeffs specifies the matrix coefficients of the picture resulting from applying the NNPF specified in the SEI message, rather than the matrix coefficients used for the CLVS.

–	When nnpfc_matrix_coeffs is not present in the NNPFC SEI message, the value of nnpfc_matrix_coeffs is inferred to be equal to vui_matrix_coeffs.

–	The values allowed for nnpfc_matrix_coeffs are not constrained by the chroma format of the decoded video pictures that is indicated by the value of ChromaFormatIdc for the semantics of the VUI parameters.

–	When nnpfc_matrix_coeffs is equal to 0, nnpfcOutputOrderIdc shall not be equal to 1 or 3.

...

nnpfc_out_tensor_chroma_bitdepth_minus8 plus 8 specifies the bit depth of chroma sample values in the output integer tensor. The value of nnpfc_out_tensor_chroma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

When nnpfc_purpose & 0x10 is not equal to 0, the value of nnpfc_out_format_idc shall be equal to 1 and at least one of the following conditions shall be true:

–	nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is greater than BitDepthY.

–	nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is greater than BitDepthC.

The variable nnpfcOutputOrderIdc is set equal to 2.

When an NNPF is activated by an NNPFA SEI message with nnpfa_target_id equal to the nnpfc_id of the NNPF, the variable nnpfcOutputOrderIdc is updated as follows, where nnpfa_num_activation_param_types, nnpfa_activation_param_type[ i ], and nnpfa_activation_param_val[ i ][ 0 ] are syntax elements in the NNPFA SEI message:

for( i = 0; i < nnpfa_num_activation_param_types; i++ ) 
	if( nnpfa_activation_param_type[ i ]  = =  1 )		(X)
		nnpfcOutputOrderIdc = nnpfa_activation_param_val[ i ][ 0 ]

nnpfc_out_order_idc nnpfcOutputOrderIdc indicates the output order of samples resulting from the NNPF.

The value of nnpfcOutputOrderIdc shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 255, inclusive, for nnpfcOutputOrderIdc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfcOutputOrderIdc in the range of 4 to 255, inclusive. Values of nnpfcOutputOrderIdc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When nnpfc_purpose & 0x02 is not equal to 0, nnpfcOutputOrderIdc shall not be equal to 3.

Table 22 contains an informative description of nnpfcOutputOrderIdc values.

Table 22 – Description of nnpfcOutputOrderIdc values

| nnpfc_out_ order_idc | Description |
| --- | --- |
| 0 | Only the luma matrix is present in the output tensor, thus the number of channels is 1. |
| 1 | Only the chroma matrices are present in the output tensor, thus the number of channels is 2. |
| 2 | The luma and chroma matrices are present in the output tensor, thus the number of channels is 3. |
| 3 | Four luma matrices and two chroma matrices are present in the output tensor, thus the number of channels is 6. This nnpfcOutputOrderIdc can only be used when the output chroma format is 4:2:0. |
| 4..255 | Reserved |

The process StoreOutputTensors( ), for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensor outputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numOutputPics; i++ ) {
	if( nnpfcOutputOrderIdc  = =  0 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				...
			}
	else if( nnpfcOutputOrderIdc  = =  1 )		(85)
		for( yP = 0; yP < outPatchCHeight; yP++)
			for( xP = 0; xP < outPatchCWidth; xP++ ) {
				xSrc = cLeft * horCScaling + xP
				ySrc = cTop * verCScaling + yP
				...
			}
	else if( nnpfcOutputOrderIdc  = =  2 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				yC = yY / outSubHeightC 
				xC = xY / outSubWidthC 
				yPc = ( yP / outSubHeightC ) * outSubHeightC
				xPc = ( xP / outSubWidthC ) * outSubWidthC
				...
			}
	else if( nnpfcOutputOrderIdc  = =  3 )
		for( yP = 0; yP < outPatchHeight; yP++ )
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				ySrc = cTop / 2 * outPatchHeight / inpPatchHeight + yP
				xSrc = cLeft / 2 * outPatchWidth / inpPatchWidth + xP
				...
			}
}

The variable nnpfcOverlap is set equal to 0.

When an NNPF is activated by an NNPFA SEI message with nnpfa_target_id equal to the nnpfc_id of the NNPF, the variable nnpfcOverlap is updated as follows, where nnpfa_num_activation_param_types, nnpfa_activation_param_type[ i ], and nnpfa_activation_param_val[ i ][ 0 ] are syntax elements in the NNPFA SEI message:

for( i = 0; i < nnpfa_num_activation_param_types; i++ ) 
	if( nnpfa_activation_param_type[ i ]  = =  2 )		(X)
		nnpfcOverlap = nnpfa_activation_param_val[ i ][ 0 ]

nnpfc_overlap nnpfcOverlap indicates the overlapping horizontal and vertical sample counts of adjacent input tensors of the NNPF. The value of nnpfcOverlap shall be in the range of 0 to 16 383, inclusive.

nnpfc_constant_patch_size_flag equal to 1 indicates that the NNPF accepts exactly the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input. nnpfc_constant_patch_size_flag equal to 0 indicates that the NNPF accepts as input any patch size with width inpPatchWidth and height inpPatchHeight such that the width of an extended patch (i.e., a patch plus the overlapping area), which is equal to inpPatchWidth + 2 * nnpfcOverlap, is a positive integer multiple of nnpfc_extended_patch_width_cd_delta_minus1 + 1 + 2 * nnpfcOverlap, and the height of the extended patch, which is equal to inpPatchHeight + 2 * nnpfcOverlap, is a positive integer multiple of nnpfc_extended_patch_height_cd_delta_minus1 + 1 + 2 * nnpfcOverlap.

nnpfc_patch_width_minus1 plus 1, when nnpfc_constant_patch_size_flag equal to 1, indicates the horizontal sample counts of the patch size required for the input to the NNPF. The value of nnpfc_patch_width_minus1 shall be in the range of 0 to Min( 32 766, CroppedWidth − 1 ), inclusive.

nnpfc_patch_height_minus1 plus 1, when nnpfc_constant_patch_size_flag equal to 1, indicates the vertical sample counts of the patch size required for the input to the NNPF. The value of nnpfc_patch_height_minus1 shall be in the range of 0 to Min( 32 766, CroppedHeight − 1 ), inclusive.

nnpfc_extended_patch_width_cd_delta_minus1 plus 1 plus 2 * nnpfcOverlap, when nnpfc_constant_patch_size_flag equal to 0, indicates a common divisor of all allowed values of the width of an extended patch required for the input to the NNPF. The value of nnpfc_extended_patch_width_cd_delta_minus1 shall be in the range of 0 to Min( 32 766, CroppedWidth − 1 ), inclusive.

nnpfc_extended_patch_height_cd_delta_minus1 plus 1 plus 2 * nnpfcOverlap, when nnpfc_constant_patch_size_flag equal to 0, indicates a common divisor of all allowed values of the height of an extended patch required for the input to the NNPF. The value of nnpfc_extended_patch_height_cd_delta_minus1 shall be in the range of 0 to Min( 32 766, CroppedHeight − 1 ), inclusive.

Let the variables inpPatchWidth and inpPatchHeight be the patch size width and the patch size height, respectively.

If nnpfc_constant_patch_size_flag is equal to 0, the following applies:

–	The values of inpPatchWidth and inpPatchHeight are either provided by external means not specified in this document or set by the post-processor itself.

–	The value of inpPatchWidth + 2 * nnpfcOverlap shall be a positive integer multiple ofnnpfc_extended_patch_width_cd_delta_minus1 + 1 + 2 * nnpfcOverlap and inpPatchWidth shall be less than or equal to CroppedWidth. The value of inpPatchHeight + 2 * nnpfcOverlap shall be a positive integer multiple ofnnpfc_extended_patch_height_cd_delta_minus1 + 1 + 2 * nnpfcOverlap and inpPatchHeight shall be less than or equal to CroppedHeight.

Otherwise (nnpfc_constant_patch_size_flag is equal to 1), the value of inpPatchWidth is set equal to nnpfc_patch_width_minus1 + 1 and the value of inpPatchHeight is set equal to nnpfc_patch_height_minus1 + 1.

The variables outPatchWidth, outPatchHeight, horCScaling, verCScaling, outPatchCWidth, and outPatchCHeight are derived as follows:

outPatchWidth = ( nnpfc_pic_width_in_luma_samples * inpPatchWidth ) / CroppedWidth	(86)

outPatchHeight = ( nnpfc_pic_height_in_luma_samples * inpPatchHeight ) / CroppedHeight	(87)

horCScaling = SubWidthC / outSubWidthC		(88)

verCScaling = SubHeightC / outSubHeightC		(89)

outPatchCWidth = outPatchWidth * horCScaling		(90)

outPatchCHeight = outPatchHeight * verCScaling		(91)

It is a requirement of bitstream conformance that outPatchWidth * CroppedWidth shall be equal to nnpfc_pic_width_in_luma_samples * inpPatchWidth and outPatchHeight * CroppedHeight shall be equal to nnpfc_pic_height_in_luma_samples * inpPatchHeight.

The variable nnpfcPaddingType is set equal to 1.

When an NNPF is activated by an NNPFA SEI message with nnpfa_target_id equal to the nnpfc_id of the NNPF, the variable nnpfcPaddingType is updated, and the variables nnpfcLumaPaddingVal, nnpfcCbPaddingVal, and nnpfcCrPaddingVal are derived as follows, where nnpfa_num_activation_param_types, nnpfa_activation_param_type[ i ], and nnpfa_activation_param_val[ i ][ j ] are syntax elements in the NNPFA SEI message:

for( i = 0; i < nnpfa_num_activation_param_types; i++ ) {
	if( nnpfa_activation_param_type[ i ]  = =  3 ) {
		nnpfcPaddingType = nnpfa_activation_param_val[ i ][ 0 ]
		if( nnpfa_activation_param_val[ i ][ 0 ]  = =  4 ) {			nnpfcLumaPaddingVal = nnpfa_activation_param_val[ i ][ 1 ]	(X)
			nnpfcCbPaddingVal = nnpfa_activation_param_val[ i ][ 2 ]			nnpfcCrPaddingVal = nnpfa_activation_param_val[ i ][ 3 ]
		}
}

nnpfc_padding_type nnpfcPaddingType indicates the process of padding when referencing sample locations outside the boundaries of the cropped decoded output picture as described in Table 23. The value of nnpfc_padding_type shall be in the range of 0 to 15, inclusive. The value of nnpfcPaddingType shall be in the range of 0 to 4, inclusive, in bitstreams conforming to this edition of this document. Values of 5 to 15, inclusive, for nnpfcPaddingType are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfcPaddingType in the range of 5 to 15, inclusive. Values of nnpfcPaddingType greater than 15 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

Table 23 – Informative description of nnpfcPaddingType values

| nnpfcPaddingType | Description |
| --- | --- |
| 0 | zero padding |
| 1 | replication padding |
| 2 | reflection padding |
| 3 | wrap-around padding |
| 4 | fixed padding |
| 5..15 | Reserved |

nnpfc_luma_padding_val nnpfcLumaPaddingVal indicates the luma value to be used for padding when nnpfcPaddingType is equal to 4. The value of nnpfcLumaPaddingVal shall be in the range of 0 to 2 inpTensorBitDepthY − 1, inclusive.

nnpfc_cb_padding_val nnpfcCbPaddingVal indicates the Cb value to be used for padding when nnpfcPaddingType is equal to 4. The value of nnpfcCbPaddingVal shall be in the range of 0 to 2 inpTensorBitDepthC − 1, inclusive.

nnpfc_cr_padding_val nnpfcCrPaddingVal indicates the Cr value to be used for padding when nnpfcPaddingType pe is equal to 4. The value of nnpfcCrPaddingVal shall be in the range of 0 to 2 inpTensorBitDepthC − 1, inclusive.

The function InpSampleVal( y, x, picHeight, picWidth, croppedPic ) with inputs being a vertical sample location y, a horizontal sample location x, a picture height picHeight, a picture width picWidth, and sample array croppedPic returns the value of sampleVal derived as follows:

NOTE 6 – For the inputs to the function InpSampleVal( ), the vertical location is listed before the horizontal location for compatibility with input tensor conventions of some inference engines.

if( nnpfcPaddingType  = =  0 )
	if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x  >=  picWidth )
		sampleVal = 0
	else
		sampleVal = croppedPic[ x ][ y ]		(92)
else if( nnpfcPaddingType  = =  1 )
	sampleVal = croppedPic[ Clip3( 0, picWidth − 1, x ) ][ Clip3( 0, picHeight − 1, y ) ]
else if( nnpfcPaddingType  = =  2 ) 
	sampleVal = croppedPic[ Reflect( picWidth − 1, x ) ][ Reflect( picHeight − 1, y ) ]
else if( nnpfcPaddingType  = =  3 ) 
	if( y >= 0  &&  y < picHeight )
		sampleVal = croppedPic[ Wrap( picWidth − 1, x ) ][ y ] 
else if( nnpfcPaddingType  = =  4 ) 
	if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x  >=  picWidth )
		sampleVal[ 0 ]  =nnpfcLumaPaddingVal			sampleVal[ 1 ]  = nnpfcCbPaddingVal
		sampleVal[ 2 ] = nnpfcCrPaddingVal
	else
		sampleVal = croppedPic[ x ][ y ]

The following example process may be used, with the NNPF PostProcessingFilter( ), to generate, in a patch-wise manner, the filtered and/or interpolated picture(s), which contain Y, Cb, and Cr sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic, respectively, as indicated by nnpfcOutputOrderIdc:

if( nnpfc_inp_order_idc  = =  0  | |  nnpfc_inp_order_idc  = =  2 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight )
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}
else if( nnpfc_inp_order_idc  = =  1 )
	for( cTop = 0; cTop < CroppedHeight / SubHeightC; cTop  +=  inpPatchHeight )
		for( cLeft = 0; cLeft < CroppedWidth / SubWidthC; cLeft  +=  inpPatchWidth ) {	(93)
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}
else if( nnpfc_inp_order_idc  = =  3 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight * 2 )
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth * 2 ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}

The order of the pictures in the stored output tensor is in output order, and the output order generated by applying the NNPF in output order is interpreted to be in output order (and not conflicting with the output order of the input pictures).

...

### 8.29.1 Neural-network post-filter activation SEI message syntax

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_persistence_flag | u(1) |
| nnpfa_num_activation_param_types | ue(v) |
| for( i = 0; i < nnpfa_num_activation_param_types; i++ ) { |  |
| nnpfa_activation_param_type[ i ] | ue(v) |
| if( nnpfa_activation_param_type[ i ] = = 0 ) { |  |
| nnpfa_num_activation_params[ i ] | ue(v) |
| for( j = 0; j < nnpfa_num_activation_params[ i ]; j++ ) |  |
| nnpfa_activation_param_val[ i ][ j ] | ue(v) |
| } else |  |
| nnpfa_activation_param_val[ i ][ 0 ] | ue(v) |
| if( nnpfa_activation_param_type[ i ] = = 3 && nnpfa_activation_param_val[ i ][ 0 ] = = 4 ) |  |
| for( j = 0; j < 3; j++ ) |  |
| nnpfa_activation_param_val[ i ][ j + 1 ] | ue(v) |
| } |  |
| } |  |
| } |  |

### 8.29.2 Neural-network post-filter activation SEI message semantics

...

nnpfa_persistence_flag specifies the persistence of the target NNPF for the current layer.

nnpfa_persistence_flag equal to 0 specifies that the target NNPF may be used for post-processing filtering for the current picture only.

nnpfa_persistence_flag equal to 1 specifies that the target NNPF may be used for post-processing filtering for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with a NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 1 is output that follows the current picture in output order.

NOTE 2 – The target NNPF is not applied for this subsequent picture in the current layer associated with a NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 1.

Let the nnpfcTargetPictures be the set of pictures to which the last NNPFC SEI message with nnpfc_id equal to nnpfa_target_id that precedes the current NNPFA SEI message in decoding order pertains. Let nnpfaTargetPictures be the set of pictures for which the target NNPF is activated by the current NNPFA SEI message. It is a requirement of bitstream conformance that any picture included in nnpfaTargetPictures shall also be included in nnpfcTargetPictures.

nnpfa_num_activation_param_types specifies the number of types of activation parameters present in the SEI message. The value of nnpfa_num_activation_param_types shall be in the range of 0 to 4, inclusive, in bitstreams conforming to this edition of this document. Values of 5 to 256, inclusive, for nnpfa_num_activation_param_types are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFA SEI messages with nnpfa_num_activation_param_types in the range of 5 to 256, inclusive.

nnpfa_activation_param_type[ i ] specifies the activation parameter type of the i-th activation parameter present in the SEI message. The value of nnpfa_activation_param_type[ i ] shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 255, inclusive, for nnpfa_activation_param_type[ i ] are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFA SEI messages with nnpfa_activation_param_type[ i ] in the range of 4 to 255, inclusive.

For any two different values of i and j in the range of 0 to nnpfa_num_activation_param_types, inclusive, nnpfa_activation_param_type[ i ] shall not be equal to nnpfa_activation_param_type[ j ].

nnpfa_num_activation_params[ i ] specifies the number of activation parameters of type indicated by nnpfa_activation_param_type[ i ] in the SEI message. When present, the value of nnpfa_num_activation_params[ i ] shall be equal to nnpfc_num_input_pics_minus1.

nnpfa_activation_param_val[ i ][ j ] specify the value of the j-th activation parameter of type indicated by nnpfa_activation_param_type[ i ] in the SEI message. Table X provides an informative definition of the activation parameter type and name.

Table X – Definition of NNPF activation parameter types and names

| Activation parameter type ID | Activation parameter name |
| --- | --- |
| 0 | NNPF input picture output control |
| 1 | NNPF picture component output control |
| 2 | NNPF patch overlap control |
| 3 | NNPF padding parameters |

When nnpfa_activation_param_type[ i ] is equal to 0, the value of nnpfa_activation_param_val[ i ][ j ] shall be equal to 0 or 1.

When nnpfa_activation_param_type[ i ] is equal to 1, the following applies: The value of nnpfa_activation_param_val[ i ][ 0 ] shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 255, inclusive, for nnpfa_activation_param_val[ i ][ 0 ] are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFA SEI messages with nnpfa_activation_param_val[ i ][ 0 ] in the range of 4 to 255, inclusive. Values of nnpfa_activation_param_val[ i ][ 0 ] greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When nnpfa_activation_param_type[ i ] is equal to 2, the value of nnpfa_activation_param_val[ i ][ 0 ] shall be in the range of 0 to 16 383, inclusive.

When nnpfa_activation_param_type[ i ] is equal to 3, the following applies:

–	The value of nnpfa_activation_param_val[ i ][ 0 ] shall be in the range of 0 to 4, inclusive, in bitstreams conforming to this edition of this document. Values of 5 to 15, inclusive, for nnpfa_activation_param_val[ i ][ 0 ] are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFA SEI messages with nnpfa_activation_param_val[ i ][ 0 ] in the range of 5 to 15, inclusive. Values of nnpfa_activation_param_val[ i ][ 0 ] greater than 15 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

–	When present, the value of nnpfa_activation_param_val[ i ][ 1 ] shall be in the range of 0 to 2inpTensorBitDepthY − 1, inclusive.

–	When present, the value of nnpfa_activation_param_val[ i ][ 2 ] or nnpfa_activation_param_val[ i ][ 3 ] shall be in the range of 0 to 2inpTensorBitDepthC − 1, inclusive.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).