| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0181-v1 |
| --- | --- |

| Title: | AHG8: Update on TVD object tracking dataset |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zizheng Liu, Liqiang Wang, Xiaozhong Xu, Shan Liu | Email: | {zizhengliu, liqiangwang, xiaozhongxu, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution provides some updates on the TVD object tracking dataset.

.

Introduction

The Tencent Video Datasets (TVD) [1][2] has been adopted as the test dataset in the AhG8 common test conditions (CTC) [3], for the evaluation of object tracking task. In this contribution, the videos in TVD are updated to lossless version for better evaluation of the compression efficiency of a video coding solution.

Update information

The TVD video dataset contains three videos, i.e., TVD-01, TVD-02 and TVD-03, with different content and the corresponding tracking ground truth. In previous version, the videos are compressed in lossy mode. In this contribution, we provide the lossless raw sequence for each video.

The following are the detail update information of each video:

TVD-01: updated with lossless compression.

TVD-02: updated to 10-bit yuv420p format, with lossless compression.

TVD-03: updated to 10-bit yuv420p format, with lossless compression.

It should be noted that the content of the videos is not changed, only the quality and the color format are updated. The tracking ground truth of each video is kept without any modifications consequently.

Table 1. Information of TVD datasets

| Sequence | Format | Resolution, fps | Length (frame) | Content |
| --- | --- | --- | --- | --- |
| TVD-01 | yuv420p, 8bit | 1920x1080, 50 | 3000 |  |
| TVD-02 | yuv420p, 10bit | 1920x1080, 50 | 636 |  |
| TVD-03 | yuv420p, 10bit | 1920x1080, 50 | 2334 |  |

Format conversion:

The original videos are compressed with lossless coding into MP4 format to save space. The original mp4 should be converted to yuv format using ffmpeg with the following command before encoding:

ffmpeg -i {input.mp4}{output.yuv}

Table 2 shows the MD5 checksum of each converted yuv file.

Table 2. File size and MD5 checksum of converted yuv

| Sequence | File size (Byte) | MD5 checksum |
| --- | --- | --- |
| TVD-01.yuv | 9,331,200,000 | 1dddac6c82e5c8e59f06d283458e2db7 |
| TVD-02.yuv | 3,956,428,800 | aad63df298fa6401c16a36ede61e9798 |
| TVD-03.yuv | 14,519,347,200 | 9aa26e98ac34e7da9712c3ed4677da4b |

Performance

The compression performance evaluation of the VTM on the updated TVD datasets are conducted following the CTC [3]. The experimental results can be found in Table 3-4. It should be noted that the sequences in the Tables are clipped from the original three video sequences. For more information about experimental settings, please refer to the CTC [3].

Table 4. results of VCM-RS (VTM InnerCodec)

| Sequence | Quality | RA |  | LD |  | AI |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Rate (Kbps) | MOTA | Rate (Kbps) | MOTA | Rate (Kbps) | MOTA |
| TVD-01_1 | QP1 | 4069.35 | 54.50 | 2548.83 | 52.32 | 102947.43 | 53.41 |
|  | QP2 | 2592.79 | 52.95 | 1477.11 | 50.96 | 73127.83 | 51.81 |
|  | QP3 | 1622.13 | 52.00 | 812.45 | 46.56 | 49810.38 | 48.87 |
|  | QP4 | 1014.84 | 46.53 | 444.96 | 40.96 | 32705.81 | 43.25 |
|  | QP5 | 627.72 | 40.74 | 235.31 | 34.30 | 20585.73 | 35.55 |
|  | QP6 | 378.40 | 31.56 | 131.01 | 25.06 | 12295.93 | 27.29 |
| TVD-01_2 | QP1 | 3574.30 | 56.35 | 2215.20 | 55.52 | 91899.45 | 56.26 |
|  | QP2 | 2220.35 | 56.06 | 1234.71 | 55.76 | 64440.48 | 55.49 |
|  | QP3 | 1197.16 | 53.84 | 555.61 | 53.36 | 37551.31 | 54.14 |
|  | QP4 | 735.39 | 52.94 | 288.33 | 48.39 | 24010.75 | 49.63 |
|  | QP5 | 447.88 | 49.43 | 156.02 | 40.73 | 14628.50 | 40.69 |
|  | QP6 | 263.73 | 39.80 | 88.41 | 27.54 | 8478.30 | 31.88 |
| TVD-01_3 | QP1 | 3002.89 | 49.13 | 1951.04 | 47.37 | 70067.43 | 49.36 |
|  | QP2 | 1884.14 | 49.31 | 1112.93 | 47.52 | 47761.80 | 48.17 |
|  | QP3 | 1009.67 | 44.93 | 526.76 | 41.61 | 27086.97 | 42.00 |
|  | QP4 | 536.48 | 38.03 | 256.27 | 33.60 | 14163.46 | 34.20 |
|  | QP5 | 276.64 | 31.97 | 135.17 | 27.09 | 6849.36 | 27.61 |
|  | QP6 | 141.04 | 25.80 | 74.29 | 17.63 | 3161.87 | 20.04 |
| TVD-02_1 | QP1 | 679.29 | 54.22 | 561.85 | 44.64 | 13225.01 | 45.82 |
|  | QP2 | 306.66 | 49.14 | 243.35 | 41.14 | 6198.92 | 46.64 |
|  | QP3 | 163.28 | 45.41 | 132.51 | 44.69 | 3222.66 | 44.82 |
|  | QP4 | 88.01 | 41.24 | 73.25 | 45.05 | 1681.02 | 41.78 |
|  | QP5 | 44.26 | 40.60 | 36.98 | 34.01 | 788.57 | 35.79 |
|  | QP6 | 26.17 | 32.79 | 21.76 | 31.52 | 409.79 | 32.02 |
| TVD-03_1 | QP1 | 1253.55 | 71.62 | 805.71 | 69.98 | 30809.77 | 69.34 |
|  | QP2 | 600.66 | 66.99 | 360.70 | 64.82 | 14666.60 | 64.64 |
|  | QP3 | 374.96 | 65.62 | 222.94 | 62.29 | 8920.01 | 60.78 |
|  | QP4 | 195.60 | 61.26 | 118.36 | 57.38 | 4313.02 | 55.28 |
|  | QP5 | 98.18 | 53.70 | 63.61 | 44.47 | 1958.69 | 40.70 |
|  | QP6 | 49.50 | 31.61 | 34.65 | 20.19 | 895.60 | 12.53 |
| TVD-03_2 | QP1 | 3757.16 | 73.13 | 3263.33 | 72.57 | 67946.28 | 72.59 |
|  | QP2 | 2027.79 | 72.00 | 1627.71 | 71.13 | 41253.57 | 71.06 |
|  | QP3 | 1088.29 | 68.29 | 820.25 | 65.50 | 23934.34 | 65.76 |
|  | QP4 | 570.50 | 63.24 | 396.93 | 63.77 | 12791.51 | 62.31 |
|  | QP5 | 344.43 | 59.06 | 226.70 | 61.62 | 7774.65 | 58.22 |
|  | QP6 | 117.20 | 52.54 | 72.76 | 41.59 | 2430.64 | 30.40 |

Conclusions

This contribution provides the lossless version update for TVD videos. It is suggested to adopt the updated version as the test sequences.

Reference

[1] X. Xu, S. Liu and Z. Li, "A Video Dataset for Learning-based Visual Data Compression and Analysis," in 2021 International Conference on Visual Communications and Image Processing (VCIP), Dec. 2021.

[2] W. Gao, X. Xu, M. Qin, S. Liu, "An Open Dataset for Video Coding for Machines Standardization", in 2022 IEEE International Conference on Image Processing (ICIP), Sept. 2022.

[3] S. Liu, C. Hollmann, “Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content”, JVET-AC2031, Jan. 2023, Teleconference.

Patent rights declarations

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).