| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0186-v1 |
| --- | --- |

| Title: | AHG8: VVC tool analysis for Machines Vision Tasks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zizheng Liu, Liqiang Wang, Xiaozhong Xu, Shan Liu | Email: | {zizhengliu, liqiangwang, xiaozhongxu, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution analyses of the new tools in VVC in terms of the impact on the coding efficiency for machine tasks and the encoding/decoding complexity. Based on the experimental results, it is shown that the VTM can be configured with 70% complexity off while maintaining the detection accuracy on the object detection task.

.

Introduction

VVC, also known as H.266 [1], was finalized in July 2020. VVC also adopts the classical block-based hybrid coding schemes with intra/inter prediction, transform and entropy coding. By employing some well-designed new coding tools, VVC can reduce the bitrate by around 50% at the same subjective quality, compared to the previous generation of coding standards. In this document, we will analyze the impact of some new coding tools for the machine vision tasks.

Tools included in test

Due to the large number of coding tools adopted in VVC, we will only select a small set of them to further investigate the impact on machine vision tasks. According to JVET-T0013 in which the coding tools were analyzed separately, we manually select some tools which incur relatively large encoding and/or decoding overhead.

We briefly list the tools by categories as following:

- Intra prediction:

Table 1. Major Intra Coding Tools

| Tool name | Description |
| --- | --- |
| MRLP | multiple reference line prediction |
| ISP | intra sub-partitioning |
| MIP | matrix based intra prediction |
| DualTree | Chroma separate tree |

- Inter prediction

Table 2. Major Inter Coding Tools

| Tool Name | Description |
| --- | --- |
| AFFINE | Affine motion model |
| SbTMC | Subblock-based temporal merging candidates |
| AMVR | Adaptive motion vector resolution |
| GPM | Geometric partition mode |
| CIIP | Combined intra/inter prediction |
| MMVD | Merge with MVD |
| BDOF | Bi-directional optical flow |
| DMVR | Decoder motion vector refinement |
| PROF | Prediction refinement using optical flow |

- Transform and quantization

Table 3. Major Transform Coding Tools

| Tool Name | Description |
| --- | --- |
| MTS | Multiple transform set |
| LFNST | Low frequency non-separable transform |
| DQ | Dependent quantization |

- In-loop filtering

Table 4. Major in loop filtering coding tools

| Tools Name | Description |
| --- | --- |
| LMCS | Luma mapping and chroma scaling |
| ALF | Adaptive loop filtering |
| SAO | Sample adaptive offset |

- Combinations Tests

We also conducted the combination tests of the selected tools above. The combination tests are listed in Table 5. When evaluating the impact of tool combination, the selected tools are turned off simultaneously.

Table 5. Combinations tests

| Tools Name | Description |
| --- | --- |
| Combination1 | ISP+MIP |
| Combination2 | Affine+AMVR+MMVD+PROF |
| Combination3 | MTS+LFNST |
| Combination4 | ALF+LMCS |
| Combination5 | All Off |

Experimental results

Experiment settings

Following the CTC [2], the experiments are conducted on VTM 12.0. We evaluate the performance of selected coding tools on SFU-HW (Object detection) and TVD-video (Object tracking) datasets. The RA/LD/AI configurations are tested.

It should be noted that the coding tools listed in Sec. 2 are all enabled in the anchor. When evaluating the specific coding tools on a machine task, we disabled the selected tools and then compare the compression results to the anchor for the evaluation of the tools.

Single tool off results

Table 6. Summary table

|  |  | BD-rate (* for estimated by Pareto) |  |  |  |  |  | Complexity (EncR/DecR) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Category | Tool | SFU-RA | SFU-LD | SFU-AI | TVD-RA | TVD-LD | TVD-AI | RA | LD | AI |
| Intra | MIP | -2.45%* | 0 | 1.57%* | 7.71% | 0 | -0.36 | 95%100% | 100%/100% | 88%/100% |
|  | MRL | -2.54%* | -2.91% | 0.83%* | 10%* | 8.3%* | -3.00% | 99%99% | 99%/99% | 98%100% |
|  | ISP | -3.05%* | -4.20% | -1.92%* | 6.66% | ### | 0.68% | 97%98% | 99%98% | 85%96% |
|  | DualTree | -4.48%* | -0.30% | 0.94% | 7.03%* | -3.67% | -1.03% | 103%99% | 100%98% | 150%108% |
| Inter | Affine | -1.13%* | 3.00%* | 0 | 0.89%* | -6.74%* | 0 | 81%98% | 66%94% | 100%100% |
|  | SbtMVP | 0.28% | 2.39%* | 0 | -0.64% | 3.20%* | 0 | 101%98% | 100%91% | 99%99% |
|  | AMVR | 0.94%* | 2.72% | 0 | 4.19% | 6.08%* | 0 | 88%100% | 90%100% | 99%100% |
|  | GEO | 0.57%* | 2.27%* | 0 | 4.56%* | -1.70%* | 0 | 97%100% | 96%101% | 99%100% |
|  | CIIP | 0.25%* | -2.94%* | 0 | 2.60%* | -6.6%* | 0 | 97%99% | 97%99% | 99%99% |
|  | MMVD | -3.27%* | 3.25%* | 0 | 0.87% | -4.14%* | 0 | 88%100% | 95%99% | 96%99% |
|  | BIO | 0.28%* | 0 | 0 | 0.91% | 0 | 0 | 95%96% | 100%100% | 100%100% |
|  | DMVR | 1.63% | 0 | 0 | 1.82% | 0 | 0 | 99%96% | 99%99% | 100%100% |
|  | PROF | -2.26%* | -3.02%* | 0 | 1.94%* | 3.25%* | 0 | 99%99% | 98%99% | 100%100% |
| Tansform /quant | MTS | -2.55%* | -2.20%* | 0.98%* | 3.68%* | 0.11%* | 0.71% | 95%99% | 98%98% | 87%99% |
|  | LFNST | -2.60%* | 0 | 1.0%* | 4.95% | 0 | -1.50% | 97%99% | 100%100% | 110%99% |
|  | DQ | -3.36%* | 1.57%* | 0.97%* | 3.07% | -6.83%* | -1.41% | 100%100% | 100%100% | 96%103% |
| Filter | ALF | -11.45%* | -9.50%* | 0.89%* | -1.37% | 7.02% | 1.41%* | 89%95% | 88%91% | 89%89% |
|  | SAO | -1.77%* | 1.02%* | -0.18%* | 2.68%* | -0.46%* | -0.17% | 100%99% | 99%99% | 99%98% |
|  | LMCS | 0.1%* | -1.17%* | -0.99%* | 2.51%* | 11.5% | -0.42% | 94%96% | 95%94% | 94%95% |

Table 6 shows the summary results of single tool off tests. We compared the complexity and BD-rate luma data with those provided in JVET-T0013 and got approximately matched results. The complexity results shown here are reliable.

Tool combination results

Table 7. Summary table of tool combination tests

|  |  | BD-rate (* for estimated by Pareto) |  |  |  |  |  | Complexity (EncR/DecR) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Category | Tools | SFU-RA | SFU-LD | SFU-AI | TVD-RA | TVD-LD | TVD-AI | RA | LD | AI |
| Combinations | MTS+LFNST | 2.38%* | -2.20%* | 2.71% | 5.62% | 0.11%* | 1.91% | 90%98% | 98%96% | 64%94% |
|  | ALF+LMCS | -7.53%* | -8.71% | 0.46%* | 2.20%* | 13.77%* | 1.69%* | 83%90% | 84%84% | 84%83% |
|  | ISP+MIP | -5.37%* | -4.2%* | 0.51% | 10.08%* | -3.03%* | 1.65% | 94%99% | 99%98% | 73%96% |
|  | AFFINE+AMVR +MMVD+PROF | 2.66%* | 5.76%* | -0.02%* | 3.50% | 6.97% | 0.01% | 59%100% | 52%94% | 97%99% |
|  | All off | -3.54%* | -7.97%* | 6.94%* | 14.47% | 12.77%* | 11.64% | 32%89% | 34%81% | 30%80% |

Observations

From the results above, we have the following observations:

- On SFU-HW (Object detection) dataset, -11.4% coding gain can be achieved by disabling ALF.

- On SFU-HW (Object detection) dataset, -7.53% coding gain can be achieved while the encoding complexity is 83% compared to anchor.

- When the selected tools (ISP/MIP/Affine/AMVR/MMVD/PROF/MTS/LFNST/ALF/LMCS) are all disabled, up to 70% encoding complexity can be saved, while still -3.54% coding gain is remained.

Conclusions

This contribution analyses the impact of coding tools in VVC on machine vision tasks. Based on the experimental results, it is suggested that VVC codec can be configured with ISP/MIP/Affine/AMVR/MMVD/PROF/MTS/LFNST/ALF/LMCS disabled to save about 70% encoding complexity compared to the current anchor with negligible performance impact.

Reference

[1] W.-J. Chien, J. Boyce, et al., “JVET AHG report: Tool reporting procedure (AHG13)”, JVET-T0013, Oct. 2020, Teleconference.

[2] S. Liu, C. Hollmann, “Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content”, JVET-AC2031, Jan. 2023, Teleconference.

Patent rights declarations

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).