| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0237-v1 |
| --- | --- |

| Title: | EE1-related: Residue input for filter set #1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Kai Zhang, Li Zhang | Tel: Email: | yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to introduce the residue as additional input information for filter set #1 in NNVC. In particular, the residue between the prediction and the reconstruction is fed into the NN filter model in EE1-1.6.2.

On top of NNNC-4.0 with NN tools enabled (abbreviated as NNVC-4.0-NnIntra-NnFilterSet1), the proposed method shows BD-rate and complexity changes reported as below:

RA: {-1.78%, -3.40%, -0.87%}, EncT 101%, DecT 106%

LDB: {-2.93%, -4.87%, -1.53%}, EncT 102%, DecT 115%

AI: {-1.23%, -0.25%, 0.94%}, EncT 102%, DecT 102%

On top of EE1-1.6.2 (NN-based intra + NN-based filter set #1 in EE1-1.6.2), the proposed method shows BD-rate and complexity changes reported as below:

RA: {-0.13%, -0.01%, -0.01%}, EncT 102%, DecT 101%

LDB: {-0.22%, 0.19%, -0.03%}, EncT 102%, DecT 102%

AI: {-0.20%, 0.68%, 0.37%}, EncT 100%, DecT 102%

Introduction

EE1-1.6.2 [1] presents a NN filter design based on a new type of residual block with both wide activation and large receptive field as shown in Fig. 1.

Proposed method

Network architecture

This contribution proposes to use residue as the additional input on top of EE1-1.6.2. The residue is obtained by subtracting the prediction from the reconstruction.

Fig. 1. gives the architecture of the proposed CNN filter, which is the same as the NN filter in EE1-1.6.2 except for the input channels. Specifically, Cin denotes the number of input channels. The proposed model takes reconstruction, prediction, slice type, bs, residue, and qp as input, therefore Cin is equal to 6.

Note that other designs of the proposed method such as chroma model, parameter selection, residual scaling, combination with deblocking, etc. remain the same as the NN-based filter set #1 in EE1-1.6.2.

Fig. 1. Architecture of the proposed deep in-loop filter.

Inference

SADL is used for performing the inference of the proposed CNN filters in VTM. Both weights and feature maps are represented with int16 precision using a static quantization method. The network information in the inference stage is provided in Table 1 as suggested by [2].

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | see Section 3.2 |
|  | Parameter Precision (Bits) | 16 (I) |
|  | Memory Parameter (MB) | see Section 3.2 |
|  | Multiply Accumulate (kMAC/pixel) | see Section 3.2 |
| Optional |  |  |
|  | Total Conv. Layers | 85 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144×144, 272×272 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

PyTorch is used as the training platform. DIV2K [3] and BVI-DVC [4] are adopted as the training datasets. The network information in the training stage is provided in Table 2 as suggested by [2].

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: A100-SXM-80GB |
|  | Framework: | PyTorch v1.6 |
|  | Number of GPUs per Task | 4 |
|  | Epoch: | 60 |
|  | Batch size: | 64 |
|  | Training time: | 140h |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {17, 22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Experimental results

The proposed method is integrated into NNVC-4.0 (filter set #1) [5] and tested according to the common test conditions defined in [2].

Performance comparison

On top of NNVC-4.0-NnIntra-NnFilterSet1 and EE1-1.6.2, the BD-rate and complexity changes of the proposed method are summarized in Tables below.

Compared with NNVC-4.0-NnIntra-NnFilterSet1, the proposed method brings on average {-1.78%, -3.40%, -0.87%}, {-2.93%, -4.87%, -1.53%}, and {-1.23%, -0.25%, 0.94%} BD-rate changes for {Y, Cb, Cr} under RA, LB, and AI.

Compared with EE1-1.6.2, the proposed method brings on average {-0.13%, -0.01%, -0.01%}, {-0.22%, 0.19%, -0.03%}, and {-0.20%, 0.68%, 0.37%} BD-rate changes for {Y, Cb, Cr} under RA, LB, and AI.

Anchored on NNVC-4.0-NnIntra-NnFilterSet1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0-NnIntra-NnFilterSet1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -1.88% | -4.77% | -1.16% | 101% | 102% |
| Class A2 | -2.14% | -2.50% | -3.12% | 102% | 105% |
| Class B | -1.79% | -4.76% | -0.74% | 102% | 108% |
| Class C | -1.41% | -1.37% | 0.88% | 100% | 108% |
| Class E |  |  |  |  |  |
| Overall | -1.78% | -3.40% | -0.87% | 101% | 106% |
| Class D | -0.75% | -1.33% | 3.89% | 103% | 114% |
| Class F | -0.52% | -2.17% | -2.74% | 102% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | BD-rate Over NNVC-4.0-NnIntra-NnFilterSet1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.88% | -4.40% | 0.74% | 102% | 109% |
| Class C | -2.59% | -4.31% | -0.65% | 100% | 108% |
| Class E | -3.47% | -6.39% | -6.49% | 105% | 140% |
| Overall | -2.93% | -4.87% | -1.53% | 102% | 115% |
| Class D | -2.29% | -1.57% | 4.26% | 100% | 110% |
| Class F | -1.96% | -2.99% | -3.53% | 104% | 96% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-4.0-NnIntra-NnFilterSet1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -1.03% | -1.51% | -0.50% | 103% | 101% |
| Class A2 | -1.01% | -0.08% | -0.21% | 99% | 103% |
| Class B | -1.04% | 0.46% | 2.43% | 100% | 101% |
| Class C | -1.54% | 0.34% | 1.82% | 104% | 103% |
| Class E | -1.58% | -1.14% | -0.16% | 104% | 104% |
| Overall | -1.23% | -0.25% | 0.94% | 102% | 102% |
| Class D | -1.26% | -0.52% | 2.99% | 98% | 100% |
| Class F | 0.42% | 2.43% | 0.72% | 100% | 87% |

Anchored on EE1-1.6.2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over EE1-1.6.2 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.11% | -0.05% | -0.11% | 101% | 101% |
| Class A2 | -0.10% | 0.12% | 0.11% | 102% | 101% |
| Class B | -0.12% | -0.03% | 0.08% | 102% | 102% |
| Class C | -0.16% | -0.05% | -0.12% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.01% | -0.01% | 102% | 101% |
| Class D | -0.41% | -0.21% | -0.02% | 100% | 102% |
| Class F | -0.08% | 0.05% | 0.00% | 101% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | BD-rate Over EE1-1.6.2 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.18% | 0.32% | 0.27% | 102% | 101% |
| Class C | -0.02% | 0.26% | -0.50% | 101% | 100% |
| Class E | -0.53% | -0.14% | 0.10% | 102% | 105% |
| Overall | -0.22% | 0.19% | -0.03% | 102% | 102% |
| Class D | 0.00% | 0.18% | 1.14% | 98% | 100% |
| Class F | -0.12% | -0.64% | -0.63% | 100% | 98% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over EE1-1.6.2 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.17% | 1.16% | 0.38% | 102% | 100% |
| Class A2 | -0.20% | 0.13% | 0.50% | 102% | 103% |
| Class B | -0.20% | 1.40% | 0.57% | 97% | 103% |
| Class C | -0.18% | 0.33% | 0.31% | 100% | 102% |
| Class E | -0.24% | 0.00% | 0.00% | 101% | 100% |
| Overall | -0.20% | 0.68% | 0.37% | 100% | 102% |
| Class D | -0.10% | 0.06% | 0.10% | 99% | 98% |
| Class F | -0.30% | -0.01% | -0.01% | 103% | 105% |

Comparison of kMAC/pixel and memory

|  | kMAC/pixel (frame basis) | kMAC/pixel (block basis) | Total Parameter Number | Total parameter memory (int16) |
| --- | --- | --- | --- | --- |
| NNVC-4.0 filter set #1 | 532 | 673 | 3.10M | 6.20M |
| EE1-1.6.2 | 446.1 | 564.6 | 2.055M | 4.110M |
| Proposed | 446.6 | 565.2 | 2.055M | 4.110M |

Conclusions

This contribution presents a NN filtering method taking residue as additional input. The proposed filtering method shows a favorable trade-off in terms of the coding performance and complexity. It is suggested to study the proposed method in EE1.

References

- Y. Li, K. Zhang, L. Zhang, EE1-1.6: In-Loop Filter with wide activation and large receptive field, JVET-AD0106, Antalya, TR, April 2023.

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AC2016, by teleconference, January 2023.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- Ma, Di, Fan Zhang, and David R. Bull. "BVI-DVC: A Training Database for Deep Video Compression." arXiv preprint arXiv:2003.13552 (2020).

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/NNVC-4.0

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).