| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0238-v2 |
| --- | --- |

| Title: | On Film Grain Synthesis Subjective Evaluation and Further Work |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xuewei Meng Wenhao Zhang Andrey Norkin Joel Sole Alberto Duenas | Email: | xuewei.meng@disneystreaming.com wenhao.zhang@disneystreaming.com anorkin@netflix.com jsole@netflix.com alberto_duenas@discovery.com |
| Source: | Disney Streaming, Netflix, Warner Bros. Discovery |  |  |

_____________________________

# Abstract

At the last meeting, it was decided to recommend for publication a technical report on Film grain synthesis technology for video applications and its draft text was presented on JVET-AC015[1].

In addition, the subjective evaluation of film grain synthesis was recommended in the last JVET meeting, and the current plan is captured on JVET-AC2022[2].

The authors of this contribution believe that those two activities are of great value and encourage JVET to continue with the current plan.

As part of this work, in the subjective test preparation for film grain synthesis and the technical report on Film grain synthesis technology for video applications, two use cases are being considered and supported:

- Film grain synthesis can be used to recreate a grain that is visually similar to the original, which could otherwise be lost due to compression, thus preserving the artistic intent.

- Film grain/noise can also be used, to some extent, to mask coding defects.

This contribution would like to encourage the group to prioritize the use case that tries to preserve the artistic intent, this is considered of great potential, and it is also a harder goal to achieve.

In JVET-AC0199 [3], it was claimed by some of the authors of this contribution that after subjective evaluation of the current shared tools that the current algorithm may not be optimized, which leads to significant artifacts. It was noted that the MCTF-based denoising may also not be optimum and an alternative filtering process could be developed. It was also mentioned that for heavy grain sequences, the synthesized grain is rather coarse, and there are obvious repetitive patterns. Parameters are recommended to control the granularity of synthesized film grain, and the frequency-based FGS method and auto-regressive (AR) based method shows different results under different circumstances. Better implementations of the denoiser and film grain modeling are recommended to show the effectiveness of the current FGS SEI message and film grain synthesis method.

This contribution would like to encourage the group to further develop and make available tools and methodologies that facilitate the usage of the FGS techniques under development in JVET.

# Introduction

In the entertainment industry, film grain is widely present in TV and motion pictures. Although digital cameras do not produce film grain, film grain is often added to the captured videos for creative intent. When encoding these TV or motion picture videos, preserving film grain under limited bandwidth is a challenging task. To address this issue, film grain synthesis methods are proposed in video coding. A typical film grain framework can be described as shown in Fig. 1.

Figure 1. A typical framework of Film Grain Synthesis.

In the current VTM-19.0, the frequency-based film grain synthesis (FGS) method is included. And the current FGS in VTM-19.0 can be described as shown in Fig. 2, in which the de-graining module in FGS has no impact on the encoding process.

Figure 2. The framework of Film Grain Synthesis in the VTM-19.0.

The following could be some of the steps that JVET could be some of the additional steps that could be considered in this activity.

- Continue development of subjective and objective testing methodologies to evaluate these technologies.

- Continue development and optimization of the current software tools to be able to utilize the available technologies for Film Grain Synthesis.

- Full integration of the available software on https://vcgit.hhi.fraunhofer.de/jvet-ahg-fgt/vfgs into the VTM.

## Table 1. Minimum Requirements of an end-to-end film grain testing toolchain.

|  | Current VTM | Requirements |
| --- | --- | --- |
| Input | YUV content file with embedded grain | - Either a YUV content file with embedded grain - Or a YUV clean content file + a YUV grain file |
| Denoiser | MCTF-based denoiser | - An integrated denoiser, with feasibility to turn on and off, and/or change the denoising strengths - Or interfaces for 3rd party denoiser |
| Film grain modeling | Frequency-based film grain modeling | Support both the Frequency-based modeling and AR-based modeling. Capability to switch between the two types of modeling at least at sequence level. Frame level switching may be a stretching request. |
| Film grain synthesis | Frequency-based film grain synthesis | Support both Frequency-based synthesis and AR-based synthesis. Interfaces to test different synthesis settings, such as different pattern sizes, different pattern shapes, and so on. |

With special focus on the preservation of the artistic intent it is recommended to continue with:

- Careful design of the subjective tests that verifies the techniques preserve the artistic intent.

  - Relevance of the question being asked. Results might differ if we ask for 1) quality, 2) fidelity to the original, 3) preservation of the artistic intent, etc.

The answer might vary depending on to whom the question is being asked: 1) naive viewers, 2) video coding experts, 3) the artist/director.

# References

[1] D. Grois, Y. He, W. Husak, P. de Lagrange, A. Norkin, M. Radosavljević, A. Tourapis, W. Wan. Proposed text: Film grain synthesis technology for video applications (CD draft). JVET-AC0151, 29th JVET meeting, teleconference, 11-20 January 2023.

[2] P. de Lagrange, W. Husak, M. Radosavljević, M. Wien. Draft plan for subjective quality testing of FGC SEI message. JVET-AC2022, 29th JVET meeting, teleconference, 11-20 January 2023.

[3] X. Meng, W. Zhang, S. Labrozzi. AHG13: On Film Grain Synthesis Subjective Evaluation. JVET-AC0199, 29th JVET meeting, teleconference, 11-20 January 2023.

# Patent rights declaration(s)

Disney Streaming, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.

Netflix may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Warner Bros. Discovery, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.