| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0248-v1 |
| --- | --- |

| Title: | Cross-check of JVET-AD0226 (EE1-1.5: Ablation Study and Minor Improvements of RTNN) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Maria Santamaria Nokia Technologies, Tampere, Finland | Email: | maria.santamaria_gomez@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document presents the inference cross-check results for JVET-AD0226, which studies the NN loop filter in JVET-AC0156. The cross-check is reported as a success given that there are minor deviations between the proponent’s results and the cross-checker’s. Overall, the deviations are up to 0.02% and, they may be due the usage of floating-point inference and different hardware.

# Introduction

JVET-AD0226 [1] is an ablation study on the loop filter presented in JVET-AC0156 [2]. The inference is done with LibTorch 1.9.0. The source code and models are available at [3].

Subtests EE1-1.5.1 and EE1-5.3 were cross-checked. The tests ran on CPU only on top of NNVC-3.0. AI and RA configurations were evaluated [4]. Notice that the encoding and decoding times are not reliable.

## Subtest EE1-1.5.1

The target of this test is to study the benefits of the transformer and attention blocks of the loop filter. For that purpose, the baseline is the architecture with both modules, while the modules are “turn off” separately.

### Architecture with attention and transformer

The following tables present the cross-checker’s results on top of NNVC 3.0.

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -8,54 % | -15,81 % | -16,45 % | -8,18 % | -20,57 % | -18,80 % | 282 % | 124678 % | 1 % |
| Class A2 | -10,16 % | -22,94 % | -25,45 % | -9,04 % | -23,37 % | -25,15 % | 270 % | 111058 % | 1 % |
| Class B | -9,39 % | -27,63 % | -23,76 % | -8,24 % | -27,31 % | -24,36 % | 273 % | 112998 % | 1 % |
| Class C | -9,65 % | -20,58 % | -21,53 % | -8,70 % | -19,22 % | -20,29 % | 238 % | 101617 % | 1 % |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -9,44 % | -22,45 % | -22,04 % | -8,51 % | -23,01 % | -22,32 % | 265 % | 111639 % | 1 % |
| Class D | -10,83 % | -22,98 % | -23,19 % | -7,28 % | -21,83 % | -19,64 % | 229 % | 89387 % | 1 % |
| Class F | -4,52 % | -12,64 % | -12,35 % | -4,89 % | -14,83 % | -14,92 % | 327 % | 61732 % | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -5,65 % | -20,81 % | -22,90 % | -5,41 % | -24,33 % | -24,65 % | 395 % | 87849 % | 0 % |
| Class A2 | -7,55 % | -25,15 % | -27,86 % | -8,32 % | -27,17 % | -27,26 % | 306 % | 68930 % | 0 % |
| Class B | -8,29 % | -24,52 % | -22,48 % | -8,20 % | -26,66 % | -24,95 % | 283 % | 63630 % | 0 % |
| Class C | -8,86 % | -19,30 % | -22,82 % | -8,63 % | -22,31 % | -25,05 % | 241 % | 43383 % | 1 % |
| Class E | -11,90 % | -24,62 % | -25,39 % | -11,61 % | -26,14 % | -28,73 % | 298 % | 70422 % | 1 % |
| Overall | -8,45 % | -22,86 % | -24,01 % | -8,42 % | -25,30 % | -25,94 % | 295 % | #DIV/0! | 0 % |
| Class D | -8,26 % | -20,16 % | -22,28 % | -7,34 % | -23,42 % | -24,76 % | 231 % | 39747 % | 1 % |
| Class F | -5,82 % | -16,06 % | -15,57 % | -5,90 % | -18,24 % | -18,89 % | 222 % | 58137 % | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

The following tables present the cross-checker’s results over the proponent’s .

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | 0,00 % | -0,13 % | -0,13 % | -0,02 % | -0,08 % | -0,18 % |  |  | 0 % |
| Class A2 | 0,00 % | -0,03 % | -0,03 % | -0,01 % | 0,00 % | 0,01 % |  |  | 0 % |
| Class B | 0,02 % | 0,00 % | -0,13 % | 0,02 % | 0,04 % | -0,07 % |  |  | 0 % |
| Class C | 0,00 % | -0,03 % | -0,07 % | -0,03 % | -0,08 % | -0,13 % |  |  | 0 % |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0,01 % | -0,04 % | -0,10 % | -0,01 % | -0,02 % | -0,09 % |  |  | 0 % |
| Class D | -0,01 % | 0,01 % | 0,06 % | 0,02 % | 0,04 % | 0,06 % |  |  | 0 % |
| Class F | -0,02 % | -0,02 % | 0,01 % | -0,05 % | -0,02 % | 0,04 % |  |  | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | 0,00 % | 0,00 % | -0,01 % | 0,00 % | 0,00 % | -0,01 % |  |  | 0 % |
| Class A2 | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class B | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class C | 0,00 % | 0,01 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class E | 0,00 % | -0,03 % | -0,01 % | 0,00 % | -0,02 % | 0,00 % |  |  | 0 % |
| Overall | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class D | 0,00 % | 0,02 % | 0,02 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class F | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

### Architecture without attention

The following tables present the cross-checker’s results on top of NNVC 3.0.

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -8,91 % | -15,04 % | -16,81 % | -8,68 % | -19,45 % | -19,73 % | 273 % | 117713 % | 1 % |
| Class A2 | -10,35 % | -22,13 % | -24,03 % | -9,09 % | -22,78 % | -23,71 % | 260 % | 106386 % | 1 % |
| Class B | -9,40 % | -26,47 % | -22,57 % | -8,22 % | -25,95 % | -23,73 % | 267 % | 108388 % | 1 % |
| Class C | -9,61 % | -19,84 % | -20,83 % | -8,64 % | -18,37 % | -19,11 % | 235 % | 95089 % | 1 % |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -9,55 % | -21,55 % | -21,24 % | -8,60 % | -21,99 % | -21,69 % | 258 % | 106016 % | 1 % |
| Class D | -10,64 % | -22,17 % | -22,30 % | -6,96 % | -21,14 % | -19,47 % | 226 % | 83447 % | 1 % |
| Class F | -4,53 % | -11,86 % | -12,03 % | -4,90 % | -13,74 % | -13,70 % | 318 % | 59860 % | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -6,64 % | -19,63 % | -22,57 % | -6,67 % | -23,03 % | -24,42 % | 381 % | 81266 % | 0 % |
| Class A2 | -8,45 % | -23,26 % | -25,72 % | -8,84 % | -25,30 % | -25,13 % | 297 % | 65081 % | 0 % |
| Class B | -8,22 % | -22,87 % | -20,89 % | -8,12 % | -24,77 % | -23,47 % | 279 % | 60419 % | 0 % |
| Class C | -8,84 % | -18,59 % | -21,34 % | -8,58 % | -21,76 % | -23,46 % | 235 % | 40555 % | 1 % |
| Class E | -11,86 % | -24,35 % | -23,79 % | -11,64 % | -25,57 % | -26,77 % | 289 % | 65804 % | 1 % |
| Overall | -8,74 % | -21,69 % | -22,56 % | -8,69 % | -24,03 % | -24,45 % | 288 % | #DIV/0! | 0 % |
| Class D | -8,25 % | -19,59 % | -21,50 % | -7,34 % | -22,98 % | -23,88 % | 229 % | 37244 % | 1 % |
| Class F | -5,83 % | -15,30 % | -14,92 % | -6,00 % | -17,39 % | -17,27 % | 218 % | 54513 % | 0 % |
| Class H |  |  |  |  |  |  |  |  | #DIV/0! |

The following tables present the cross-checker’s results over the proponent’s .

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | 0,01 % | -0,06 % | -0,05 % | 0,01 % | -0,05 % | -0,03 % |  |  | 0 % |
| Class A2 | 0,00 % | 0,00 % | 0,10 % | -0,01 % | -0,02 % | 0,10 % |  |  | 0 % |
| Class B | 0,00 % | -0,01 % | -0,04 % | -0,02 % | -0,02 % | -0,04 % |  |  | 0 % |
| Class C | 0,01 % | 0,00 % | 0,00 % | 0,00 % | 0,06 % | 0,01 % |  |  | 0 % |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0,00 % | -0,02 % | 0,00 % | -0,01 % | -0,01 % | 0,01 % |  |  | 0 % |
| Class D | 0,01 % | 0,04 % | 0,01 % | 0,01 % | 0,07 % | -0,09 % |  |  | 0 % |
| Class F | 0,00 % | 0,02 % | 0,02 % | 0,00 % | 0,02 % | 0,06 % |  |  | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class A2 | 0,00 % | -0,01 % | -0,02 % | 0,00 % | -0,01 % | -0,02 % |  |  | 0 % |
| Class B | 0,00 % | 0,01 % | 0,01 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class C | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class E | 0,00 % | 0,03 % | 0,01 % | 0,00 % | 0,02 % | 0,00 % |  |  | 0 % |
| Overall | 0,00 % | 0,01 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class D | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class F | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

### Architecture without transformer

The following tables present the cross-checker’s results on top of NNVC 3.0.

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -7,97 % | -10,75 % | -13,26 % | -7,90 % | -13,11 % | -14,81 % | 226 % | 56114 % | 1 % |
| Class A2 | -8,74 % | -15,13 % | -11,47 % | -7,95 % | -14,99 % | -10,25 % | 218 % | 51715 % | 1 % |
| Class B | -7,67 % | -15,75 % | -16,50 % | -7,20 % | -15,88 % | -17,29 % | 225 % | 54028 % | 1 % |
| Class C | -8,11 % | -17,10 % | -17,64 % | -7,80 % | -15,27 % | -15,67 % | 205 % | 47222 % | 1 % |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -8,06 % | -14,99 % | -15,15 % | -7,65 % | -14,99 % | -14,96 % | 218 % | 52061 % | 1 % |
| Class D | -9,41 % | -17,52 % | -18,44 % | -6,70 % | -16,35 % | -15,48 % | 198 % | 42035 % | 1 % |
| Class F | -3,69 % | -9,99 % | -10,21 % | -4,02 % | -11,07 % | -11,79 % | 247 % | 36573 % | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -6,52 % | -12,72 % | -14,55 % | -6,27 % | -15,14 % | -16,48 % | 291 % | 39782 % | 0 % |
| Class A2 | -6,89 % | -14,01 % | -11,71 % | -7,30 % | -14,51 % | -10,69 % | 251 % | 31872 % | 0 % |
| Class B | -6,70 % | -10,90 % | -14,73 % | -6,74 % | -14,08 % | -17,24 % | 240 % | 29595 % | 0 % |
| Class C | -7,11 % | -13,70 % | -16,24 % | -7,03 % | -16,87 % | -18,28 % | 217 % | 20243 % | 1 % |
| Class E | -9,89 % | -16,42 % | -18,03 % | -9,87 % | -16,41 % | -19,96 % | 243 % | 32697 % | 0 % |
| Overall | -7,32 % | -13,26 % | -15,08 % | -7,34 % | -15,34 % | -16,71 % | 245 % | #DIV/0! | 0 % |
| Class D | -7,14 % | -13,30 % | -16,43 % | -6,44 % | -16,61 % | -18,56 % | 209 % | 18599 % | 1 % |
| Class F | -4,72 % | -11,02 % | -10,64 % | -4,83 % | -12,94 % | -12,90 % | 199 % | 26992 % | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

The following tables present the cross-checker’s results over the proponent’s .

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -0,01 % | -0,12 % | -0,06 % | -0,02 % | -0,07 % | -0,07 % |  |  | 0 % |
| Class A2 | -0,05 % | -0,15 % | -0,14 % | -0,08 % | -0,11 % | -0,12 % |  |  | 0 % |
| Class B | -0,01 % | 0,06 % | -0,04 % | -0,01 % | 0,07 % | -0,13 % |  |  | 0 % |
| Class C | -0,01 % | -0,07 % | -0,02 % | 0,00 % | -0,11 % | -0,03 % |  |  | 0 % |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0,02 % | -0,05 % | -0,06 % | -0,02 % | -0,04 % | -0,09 % |  |  | 0 % |
| Class D | 0,00 % | -0,04 % | -0,04 % | -0,01 % | -0,03 % | 0,02 % |  |  | 0 % |
| Class F | -0,01 % | 0,05 % | -0,02 % | 0,08 % | 0,10 % | -0,01 % |  |  | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | 0,00 % | 0,00 % | 0,00 % | 0,00 % | -0,01 % | 0,00 % |  |  | 0 % |
| Class A2 | 0,00 % | -0,01 % | -0,02 % | 0,00 % | -0,01 % | -0,02 % |  |  | 0 % |
| Class B | 0,00 % | 0,00 % | 0,03 % | 0,00 % | -0,01 % | 0,01 % |  |  | 0 % |
| Class C | 0,00 % | 0,03 % | 0,02 % | 0,00 % | 0,02 % | 0,01 % |  |  | 0 % |
| Class E | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Overall | 0,00 % | 0,00 % | 0,01 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class D | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class F | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

## Subtest EE1-1.5.2

This subtest evaluates modifications done to the RTNN loop filter architecture.

The proponent reported results for AI configuration only and the cross-check is partial as classes A1 and A2 were not ready by the time this report was submitted.

The following table present the cross-checker’s results on top of NNVC 3.0.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -8,36 % | -24,64 % | -22,37 % | -8,26 % | -26,85 % | -24,84 % | 280 % | 61896 % | 0 % |
| Class C | -8,92 % | -19,62 % | -22,68 % | -8,69 % | -22,80 % | -24,92 % | 236 % | 41900 % | 1 % |
| Class E | -11,75 % | -25,39 % | -25,29 % | -11,60 % | -26,64 % | -28,59 % | 293 % | 67902 % | 1 % |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | -8,30 % | -20,67 % | -23,03 % | -7,43 % | -23,66 % | -25,43 % | 228 % | 38474 % | 1 % |
| Class F | -5,75 % | -15,89 % | -14,82 % | -6,00 % | -18,14 % | -18,19 % | 220 % | 55597 % | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

The following tables present the cross-checker’s results over the proponent’s .

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0,00 % | -0,01 % | -0,01 % | 0,00 % | -0,01 % | 0,00 % |  |  | 0 % |
| Class C | 0,00 % | 0,00 % | -0,01 % | 0,00 % | 0,00 % | -0,01 % |  |  | 0 % |
| Class E | 0,00 % | 0,01 % | 0,00 % | 0,00 % | 0,01 % | 0,00 % |  |  | 0 % |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class F | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

## Subtest EE1-1.5.3

In JVET-2023 [5] this test is about trying to implement the architecture in SADL. However, in JVET-AD2026 [1] this test corresponds to evaluating the correctness of JVET-AC0156 [2]. The latter is cross-checked.

The following tables present the cross-checker’s results on top of NNVC 3.0.

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -8,52 % | -14,87 % | -16,70 % | -8,29 % | -20,00 % | -18,92 % | 278 % | 126255 % | 1 % |
| Class A2 | -10,05 % | -23,33 % | -25,86 % | -8,96 % | -23,81 % | -25,40 % | 264 % | 110196 % | 1 % |
| Class B | -9,16 % | -27,85 % | -24,12 % | -8,19 % | -27,73 % | -24,65 % | 273 % | 114112 % | 1 % |
| Class C | -9,26 % | -20,73 % | -21,67 % | -8,52 % | -19,60 % | -20,37 % | 240 % | 100913 % | 1 % |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -9,24 % | -22,45 % | -22,33 % | -8,45 % | -23,23 % | -22,51 % | 263 % | 111904 % | 1 % |
| Class D | -10,44 % | -23,03 % | -23,55 % | -7,11 % | -21,84 % | -19,89 % | 229 % | 89056 % | 1 % |
| Class F | -4,09 % | -12,70 % | -11,78 % | -4,47 % | -14,43 % | -14,21 % | 326 % | 60492 % | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -5,65 % | -20,20 % | -22,99 % | -5,37 % | -23,94 % | -24,62 % | 389 % | 87974 % | 0 % |
| Class A2 | -8,02 % | -25,34 % | -28,49 % | -8,27 % | -27,38 % | -27,88 % | 300 % | 68514 % | 0 % |
| Class B | -8,27 % | -24,77 % | -22,54 % | -8,11 % | -26,94 % | -24,93 % | 283 % | 64192 % | 0 % |
| Class C | -8,79 % | -19,65 % | -22,70 % | -8,53 % | -22,80 % | -24,92 % | 240 % | 43362 % | 1 % |
| Class E | -11,78 % | -24,85 % | -25,09 % | -11,55 % | -26,32 % | -28,53 % | 296 % | 71293 % | 1 % |
| Overall | -8,49 % | -22,98 % | -24,07 % | -8,35 % | -25,49 % | -25,97 % | 293 % | #DIV/0! | 0 % |
| Class D | -8,23 % | -20,61 % | -22,94 % | -7,29 % | -23,61 % | -25,34 % | 230 % | 39871 % | 1 % |
| Class F | -5,10 % | -15,88 % | -14,80 % | -5,42 % | -18,11 % | -18,17 % | 221 % | 56998 % | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

The following tables present the cross-checker’s results over the proponent’s .

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | 0,01 % | 0,06 % | 0,04 % | 0,01 % | 0,12 % | 0,03 % |  |  | 0 % |
| Class A2 | -0,03 % | -0,06 % | -0,18 % | -0,03 % | -0,03 % | -0,15 % |  |  | 0 % |
| Class B | 0,00 % | 0,02 % | -0,05 % | 0,00 % | 0,07 % | -0,04 % |  |  | 0 % |
| Class C | -0,03 % | 0,01 % | 0,02 % | -0,02 % | -0,04 % | -0,04 % |  |  | 0 % |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0,01 % | 0,01 % | -0,04 % | -0,01 % | 0,03 % | -0,05 % |  |  | 0 % |
| Class D | -0,01 % | 0,08 % | 0,01 % | -0,02 % | 0,08 % | -0,02 % |  |  | 0 % |
| Class F | -0,01 % | 0,08 % | -0,03 % | -0,01 % | 0,10 % | -0,01 % |  |  | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class A2 | 0,00 % | 0,01 % | 0,00 % | 0,00 % | 0,01 % | 0,00 % |  |  | 0 % |
| Class B | 0,00 % | -0,01 % | -0,01 % | 0,00 % | -0,01 % | -0,01 % |  |  | 0 % |
| Class C | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class E | 0,00 % | 0,03 % | 0,02 % | 0,00 % | 0,01 % | 0,01 % |  |  | 0 % |
| Overall | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class D | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class F | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % | 0,00 % |  |  | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

# Conclusion

The inference cross-check was successful. There were minor differences between the proponent’s and cross-checker’s results. Those may be due to the usage of floating-point arithmetic and different hardware.

# References

- H. Zhang, C. Jung, Y. Liu, M. Li. EE1-1.5: Ablation Study and Minor Improvements on RTNN. JVET-AD0226. April 2023.

- H. Zhang, C. Jung, Y. Liu, M. Li. [EE1-related] RTNN: An In-loop Filter Based on Resblock and Transformer. JVET-AC0156. January 2023.

- https://vcgit.hhi.fraunhofer.de/jvet-ac-ee1/VVCSoftware_VTM/-/tree/EE1-1.5/VTM-3.0%20Software

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AC2031. October 2022.

- E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang, Z. Xie. Exploration experiment on neural network-based video coding (EE1). JVET-AC2023. January 2023.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).