| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0249-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0174: Non-EE2: CIIP with more combination candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu | Tel: Email: | xuluhang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-AD0174 Non-EE2: CIIP with more combination candidates. It is reported that (a). the implementation and the algorithm descriptions in JVET-AD0174 are matched. (b). the partial crosscheck results match exactly with the results provided by the proponent.

Introduction

In JVET-AD0174, a new inter-intra combination method is proposed to improve CIIP. Firstly, an intra prediction modes list is obtained using the MPM construction method, followed by modes sorting process according to the SAD cost, where the SAD cost is computed between the prediction and the reconstruction samples of the template.

Then the mapping method which bounds a pair of merge candidate and the intra prediction mode in the list obtained in the first step, is applied to get the inter prediction samples and the corresponding intra prediction samples. The final prediction samples for each pair are generated by weighting the paired intra and inter prediction samples. In this contribution, the linear model-based weighted method is proposed as follows:

PCIIP= α0*Pinter+α1*Pintra+α2*midValue

where PCIIP represents the final prediction samples, Pinter represents the inter prediction sample. Pintra represents the intra prediction samples. For 10-bit content, the midValue is set equal to 512. The model parameters α0, α1 and α2 are derived using the adjacent samples based on the LDL decomposition method same as CCCM.

Simulation results

The crosscheck has been performed in Random Access (RA) configuration. So far, only partial results are available (attached to this file) and they are matched with the results provided by the proponent.

Table 1. RA cross-check results of JVET-AD0174

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.09% | 0.01% | -0.02% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.06% | 0.22% | 0.04% | 102% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |