| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0268-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0128 (modified CTC proposal for low-delay configurations) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | M. Blestel | Tel: Email: | medericb@xiaomi.com |
| Source: | Xiaomi |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results of the contribution JVET-AD0128 proposed by InterDigital.

The contribution JVET-AD0128 proposes to modify EE2 common test conditions, to significantly reduce the worst-case encoding runtime for LDB/LDP, together with slightly improved overall BD-rate results.

ECM-8.0 encoding time is becoming problematic for investigating new coding tools. In particular, the worst-case encoding time for low-delay B HD sequences makes it very long to wait for complete results.

Results from JVET-AD0128 were cross-checked and the simulation results perfectly match the reported result reported in JVET-AD0128.

# Introduction

In the current Ahg12 CTC, the worst cases correspond to the LDB QP 22 encoding of class B and class F “Arena of Valor” sequences.

Depending on the processing performance and configuration used, around 15 days of computation can be needed to generate full results (case of BQTerrace QP 22).

Therefore, the JVET-AD0128 contribution proposes to reduce the encoding worst case, in a similar way to what was adopted for random class A QP 22 cases ([1]).

The encoding parameters used in the proposed modified CTCs are:

- MaxBTLumaISlice=128

- MaxBTChromaISlice=128

- MaxTTLumaISlice=128

- MaxTTChromaISlice=128

- MaxMTTHierarchyDepthISliceL=4

- MaxMTTHierarchyDepthISliceC=4

In addition, for class B and class F HD sequence, following parameter is used:

- MaxMTTHierarchyDepthByTidOverrideByQP: 22 332222

# Cross-check results

Cross-check of the simulation results have been conducted with modified configuration files to match the proposal.

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | -0,64% | -0,74% | -2,39% | 106% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0,10% | -0,93% | 0,04% | 95% | 96% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# References

- JVET-AA0098, “Ahg 12: encoder configuration proposal to reduce worst case encoding time”, F. Le Léannec, P. Andrivon, M. Radosavljević, Xiaomi, July 2022

- JVET-Y2017, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, 	M. Karczewicz, Y. Ye, January 2022

# Patent rights declaration(s)

Xiaomi does not have any current or pending patent rights relating to the technology described in this contribution.