| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0304 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0095 and JVET-AD0096 (EE2-3.3: Temporal block vector prediction and EE2-3.4: Copy-Padding for IBC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Z. Zhang | Tel: Email: | zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of crosschecking JVET-AD0095 and JVET-AD0096 proposed by Bytedance Inc. JVET-AD0095 proposes temporal block vector prediction to further improve the efficiency of BV prediction. JVET-AD0096 proposes Copy-Padding for IBC for an overlapped area to improve the efficiency of IBC. The methods are tested as standalone test of EE2-3.3 and EE2-3.4a. The combination of the two methods is tested as test of EE2-3.4b.

The update of the code had been reviewed and confirmed consistent with the proposal description. The crosscheck reports the performance of AI, RA and LDB configurations over ECM-8.0. The simulation results match the reported results in JVET-AD0095 and JVET-AD0096.

# Verification

Bytedance had provided the software and results of their proposed methods in JVET-AD0095 and JVET-AD0096. The code was updated according to the description in the proposals.

The BD-rate results match the results reported. The simulation results are as follows:

## Crosscheck results of Temporal block vector prediction (Test 3.3)

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% | -0.04% | -0.06% | -0.01% | 100% | 100% |
| Class TGM | 0.00% | 0.00% | 0.00% | 100% | 100% | -0.41% | -0.42% | -0.50% | 99% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% | -0.23% | -0.24% | -0.26% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.22% | 0.08% | 0.46% | 100% | 101% |
| Class TGM | -0.42% | -0.39% | -0.46% | 99% | 99% |
| Overall | -0.32% | -0.16% | 0.00% | 100% | 100% |

## Crosscheck results of Copy-Padding for IBC (Test 3.4a)

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | 0.00% | -0.09% | 0.09% | 101% | 101% | -0.08% | -0.58% | -0.45% | 100% | 101% |
| Class TGM | -0.22% | -0.21% | -0.16% | 101% | 100% | -0.21% | -0.10% | -0.17% | 99% | 99% |
| Overall | -0.11% | -0.15% | -0.04% | 101% | 100% | -0.15% | -0.34% | -0.31% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.17% | -0.01% | 0.58% | 99% | 99% |
| Class TGM | -0.45% | -0.23% | -0.44% | 99% | 96% |
| Overall | -0.14% | -0.12% | 0.07% | 99% | 98% |

## Crosscheck results of Combination (Test 3.4b)

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | 0.00% | -0.09% | 0.09% | 100% | 102% | -0.15% | -0.59% | -0.53% | 100% | 101% |
| Class TGM | -0.22% | -0.21% | -0.16% | 100% | 99% | -0.60% | -0.41% | -0.51% | 99% | 99% |
| Overall | -0.11% | -0.15% | -0.04% | 100% | 100% | -0.38% | -0.50% | -0.52% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.16% | -0.56% | 0.92% | 100% | 99% |
| Class TGM | -0.90% | -0.67% | -0.95% | 99% | 98% |
| Overall | -0.37% | -0.61% | -0.01% | 99% | 99% |

# References

[1] N. Zhang, K. Zhang, L. Zhang, “EE2-3.3: Temporal block vector prediction”, JVET-AD0095, Apr. 2023.

[2] N. Zhang, K. Zhang, L. Zhang, “EE2-3.4: Copy-Padding for IBC”, JVET-AD0096, Apr. 2023.

[3] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-8.0

[4] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Qualcomm does not have any current or pending patent rights relating to the technology described in this contribution.