| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0338-v2 |
| --- | --- |

| Title: | Non-EE2: On MHP weight derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Y. Zhang H. Huang V. Seregin M. Karczewicz (Qualcomm) | Tel: Email: | yzh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of template matching (TM) based MHP weight derivation. On top of ECM-8.0, the reported BDR Y/Cb/Cr are -0.03% / -0.02% / -0.04% in RA and -0.11% / 0.08% / 0.02% in LDB.

# Introduction

In ECM [1], multiple hypothesis prediction (MHP) is included. This inter prediction method allows weighted superposition of more than two motion-compensated prediction signals. The resulting overall prediction signal is obtained by sample-wise weighted superposition. With the uni/bi prediction signal puni/bi (referred to as base prediction signal) and the additional inter prediction signal/hypothesis h3, the resulting prediction signal p3 is obtained as follows:

p3=(1-α)puni/bi+αh3

The weighting factor α is specified by the syntax element add_hyp_weight_idx, according to the following mapping:

| add_hyp_weight_idx | α |
| --- | --- |
| 0 | 1/4 |
| 1 | -1/8 |

As can be seen from the above table, two different weight indices are used, and the corresponding weight index are signalled.

More than one additional prediction signals can be used. The resulting overall prediction signal is accumulated iteratively with each additional prediction signal.

pn+1=1-αn+1pn+αn+1hn+1

Currently in ECM, the maximum number of allowed MHP predictor is set to 2.

In JVET-AB0079, it is proposed to derive BCW index for merge coded CUs according to template matching (TM) cost and extend bi-predicted weights for merge mode. The proposed method was adopted in ECM.

# Proposal

In this proposal, the template matching based weight derivation in BCW is also applied for weight derivation in MHP. The template of the merge candidate is combined with the reference template using a MHP weight to generate the predictor’s template. Then the SAD based template matching cost between the template of the current block and the template of the predictor is computed. The costs for all the weights are derived and the weight that corresponds to the minimum TM cost is selected. Similar to that of JVET-AB0079, the number of weights in MHP is extended.

# Experimental results

This section reports the simulation results of the proposed method, according to common test conditions [2]. The proposed method brings -0.03% and -0.11% gain for RA and LDB respectively.

Table 1: simulation result

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.02% | 0.01% |  |  |
| Class A2 | -0.02% | -0.04% | 0.07% |  |  |
| Class B | -0.02% | 0.08% | -0.15% |  |  |
| Class C | -0.05% | -0.14% | -0.04% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.02% | -0.04% |  |  |
| Class D | -0.05% | 0.14% | -0.10% | 102% | 101% |
| Class F | -0.15% | -0.14% | -0.14% |  |  |
| Class TGM | -0.19% | -0.13% | -0.12% |  |  |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.13% | -0.09% |  |  |
| Class C | -0.12% | 0.01% | -0.08% |  |  |
| Class E | -0.20% | 0.55% | 0.36% | 101% | 100% |
| Overall | -0.11% | 0.08% | 0.02% |  |  |
| Class D | -0.01% | 0.08% | 0.37% | 103% | 100% |
| Class F | -0.24% | -0.22% | -0.54% |  |  |
| Class TGM | 0.04% | 0.04% | 0.03% |  |  |

# Conclusion

This contribution reports the results of template matching based MHP weight derivation. Gain can be observed for both RA and LDB configurations across all the classes. It is proposed to further study the proposed method in the next round of EE2.

# References

- https://vcgit.hhi.fraunhofer.de/ecm/ECM

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).