| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0342-v1 |
| --- | --- |

| Title: | EE2-related: Combination of JVET-AD0084, JVET-AD0117 and JVET-AD0214 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu, Lai Zhang, Fan Wang Jonathan Gan, Haoping Yu, Luhang Xu Zhihuang Xie, Dong Wang (OPPO) Yanzhuo Ma, Hongli Zhang, Hongqing Du, Wenhan Qiao, Junyan Huo, Fuzheng Yang (Xidian University) Xiaoyu Xiu, Ning Yan, Changyue Ma Hong-Jheng Jhu, C.-W. Kuo, Wei Chen, Xianglin Wang (Kwai, Inc.) | Email: | yue.yu@oppo.com yzma@mail.xidian.edu.cn xiaoyuxiu@kwai.com |
| Source: | OPPO, Xidian, Kwai |  |  |

_____________________________

# Abstract

This contribution reports the combination test results for the additional IntraTMP search areas proposed by JVET-AD0084, JVET-AD0117, and JVET-AD-214. IntraTMP employs a pre-defined template of the current block to search for a candidate template of the same shape that best matches the template of the current block within a pre-determined search area. The corresponding reference block with the best matched template is selected as the prediction block for the current block. Currently, the search region of the intraTMP mode is composed of four different areas which contain the reconstructed samples from the top and left CTUs and part of the reconstructed samples from the same CTU that the current block belongs to. This contribution proposes to include the bottom-left and top-right areas adjacent to the current block as additional search areas for intraTMP. In addition, this contribution proposes to restrict the intraTMP search with a specified search range for all pre-determined search areas. On top of ECM-8.0, the simulation results are reported as follows:

AI(Y/U/V): -0.21%/-0.28%/-0.27%, EncT 103%, DecT 102%

RA(Y/U/V): -0.12%/-0.08%/-0.05%, EncT 101%, DecT 100%

On top of EE2-Test1.20j, the simulation results are reported as follows (relative to ECM-8.0 anchor):

AI(Y/U/V): -1.35%/-1.32%/-1.37%, EncT 105%, DecT 108%

RA(Y/U/V): -0.xx%/-0.xx%/-0.xx%, EncT xx%, DecT xx%

# Introduction

IntraTMP employs a pre-defined template of the current block to search for a candidate template of the same shape that best matches the template of the current block within a pre-determined search area. In order to search for the best template, a cost function, e.g., SAD is defined and both the encoder and decoder use the same search strategy to obtain the best matched block. Once the best matched block is found, the corresponding reference block will be selected as the prediction block for the current block.

As shown in Figure 1, the search area of the existing intraTMP is divided into four different regions R1, R2, R3 and R4 which contain the reconstructed samples from the top and left CTUs as well as part of the reconstructed samples within the current CTU that are located above and left to the current block. In Figure 1, W and H are the width and height of the current coding block and the gray blocks describe the region of the top-left corners of all considered reference blocks in the reconstructed area.

Figure 1 The search region of the intraTMP in ECM8.0

Additionally, in EE2 Test-1.14c [2], it is proposed to extend the regions R3 and R4 to include more reconstructed samples that are located above and left to the current block. The maximum horizontal and vertical search size, i.e., searchRangeWidth and searchRangeHeight, are set to max(5W, 64) and max(5H, 64) respectively. Figure 2 depicts the modified search area of the intraTMP in Test-1.14c.

Figure 2 The extended search region of the intraTMP in EE2 Test-1.14c

# Proposed method

The bottom-left and top-right areas relative to the current block may also be available and they are very close to the current block, this contribution proposes to include these two regions (R5 and R6) as additional search areas for intraTMP as shown on the left of Figure 3. In ECM-8.0, the region scan order is R4, R1, R2, and R3. In the proposed method, the region scan order is R4, R5, R6, R1, R2, and R3. In addition, the current ECM-8.0 always searches all available positions within the current CTU while it may be beyond the specified search range or width/height. This contribution further proposes to restrict all search areas with the specified search range or width/height. The search areas with the proposed search range restrictions are illustrated on the right of Figure 3.

Fig.3. Left: bottom-left and top-right added as additional search areas for intraTMP

Right: all available search areas restricted by specified search width/height range

# Simulation results

The proposed bottom-left and top-right areas are implemented, as additional search areas on top of the ECM-8.0 software [1], and tested based on the common test condition [2]. The test results are summarized in Table 1.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.09% | -0.08% | 106% | 100% |
| Class A2 | -0.40% | -0.57% | -0.53% | 106% | 106% |
| Class B | -0.30% | -0.29% | -0.27% | 102% | 102% |
| Class C | -0.07% | -0.14% | -0.15% | 102% | 101% |
| Class E | -0.21% | -0.35% | -0.36% | 102% | 101% |
| Overall | -0.21% | -0.28% | -0.27% | 103% | 102% |
| Class D | -0.08% | -0.35% | -0.26% | 101% | 101% |
| Class F | 0.02% | -0.03% | 0.07% | 103% | 100% |
| Class TGM | -0.18% | -0.17% | -0.13% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.07% | 101% | 100% |
| Class A2 | -0.21% | -0.16% | -0.09% | 101% | 100% |
| Class B | -0.22% | -0.09% | -0.08% | 100% | 100% |
| Class C | -0.02% | -0.05% | 0.01% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.08% | -0.05% | 101% | 100% |
| Class D | -0.02% | -0.07% | -0.10% | 100% | 100% |
| Class F | -0.02% | -0.57% | -0.44% | 100% | 100% |
| Class TGM | -0.07% | -0.09% | -0.07% | 101% | 102% |

Table 1. The proposed additional search areas over ECM-8.0

Furthermore, the proposed additional search areas and the search range restrictions are both implemented on top of the EE2-Test1.20j. The test results are shown in Table 2.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.23% | -0.11% | 105% | 106% |
| Class A2 | -1.86% | -2.09% | -2.18% | 103% | 109% |
| Class B | -1.57% | -1.56% | -1.60% | 106% | 110% |
| Class C | -0.60% | -0.46% | -0.51% | 105% | 101% |
| Class E | -2.62% | -2.41% | -2.62% | 108% | 112% |
| Overall | -1.35% | -1.32% | -1.37% | 105% | 108% |
| Class D | -0.71% | -0.96% | -0.94% | 109% | 110% |
| Class F | -0.61% | -0.56% | -0.65% | 105% | 107% |
| Class TGM | -1.16% | -0.93% | -0.86% | 100% | 105% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.19% | -0.18% | -0.12% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.25% | -0.01% | -0.34% | 102% | 101% |
| Class F | -0.36% | -0.59% | -0.37% | 101% | 101% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 2. The proposed additional search areas and search range restrictions on top of the EE2-Test1.20j over ECM-8.0

In addition, Table 3 shows the test results of EE2-Test1.20j with the additional search areas and the search range restrictions, compared with EE2-Test1.20j. Note: the running time comparison shown in Table 3 is not reliable; the running time is compared between Table 2 and the results reported in [4] which indicates 0% and 2% encoding/decoding increase for the proposed method.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over AD0086_Test1.20j |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.11% | -0.15% | 94% | 100% |
| Class A2 | -0.51% | -0.54% | -0.60% | 95% | 95% |
| Class B | -0.55% | -0.58% | -0.60% | 97% | 104% |
| Class C | -0.15% | -0.17% | -0.07% | 97% | 94% |
| Class E | -0.32% | -0.15% | -0.36% | 101% | 105% |
| Overall | -0.34% | -0.33% | -0.37% | 97% | 100% |
| Class D | -0.02% | -0.05% | -0.23% | 101% | 106% |
| Class F | -0.16% | 0.02% | 0.00% | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-AD0086_Test1.20j |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.02% | -0.06% | -0.11% | 87% | 84% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.04% | 0.05% | 0.10% | 91% | 78% |
| Class F | -0.04% | 0.21% | 0.00% | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 3. The proposed additional search areas and search range restrictions on top of the EE2-Test1.20j over EE2-Test1.20j

# Conclusions

This contribution proposes to include two additional search areas for IntraTMP. The proposed method brings 0.21% coding gain for AI and 0.12% coding gain for RA over ECM-8.0. Furthermore, the proposed method combined with search range restrictions brings extra 0.34% coding gain for AI and 0.xx% coding gain for RA on top of EE2-Test1.20j with 0% and 2% encoding and decoding increase, respectively. It is suggested to adopt the proposed method on top of EE2-Test1.20j.

# References

- ECM8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0

- V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-AC2024, Jan. 2023.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, July, 2022.

- F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang(OPPO), Y. Ma, H. Zhang, J. Huo, F. Yang (Xidian Univ.), P. Lin, J. Lin, V. Seregin, M. Karczewicz(Qualcomm), X. Li, R. Liao, J. Chen, Y. Ye (Alibaba group) ,“EE2-1.20i/j: Combination of IntraTMP tests”, JVET-AD0086, April, 2023.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).