| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE_notes_dG |
| --- | --- |

| Title: | Meeting Report of the 31st Meeting of the Joint Video Experts Team (JVET), Geneva, CH, 11–19 July 2023 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the chair of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen Melatener Straße 23 D-52074 Aachen | Tel: Email: | +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chair of JVET |  |  |

_____________________________

# Summary

The Joint Video Experts Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/‌SC 29 held its thirty-first meeting during 11–19 July 2023 at the ITU premises in Geneva, Switzerland. The meeting was held as a face-to-face meeting, but remote participation was provided on best-effort basis for experts who were unable to travel.

For ISO/IEC purposes, JVET is alternatively designated ISO/IEC JTC 1/‌SC 29/‌WG 5, and this was the twelfth meeting as WG 5. The JVET meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 2.16 of this document. It is further noted that work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued in JVET as a single joint team, and explorations towards possible future need of standardization in the area of video coding are also conducted by JVET, as negotiated by the parent bodies.

The JVET meeting began at approximately 0930 CEST (UTC+2) on Tuesday 11 July 2023. Meeting sessions were held on all days, including the weekend days of Saturday and Sunday 15 and 16 July 2023, until the meeting was closed at approximately 1215 hours CEST on Wednesday 19 July 2023. Approximately 316 people attended the JVET meeting (191 in person and 125 remotely), and approximately 165 input documents (not counting crosschecks, reports, and summary documents), 14 AHG reports, 2 EE summary reports, 3 BoG reports, and 1 incoming liaison document were discussed. The meeting took place in a collocated fashion with a meeting of SG16 – one of the two parent bodies of the JVET, under whose auspices this JVET meeting was held. Various SC29 Working Groups and Advisory Groups were also meeting with partial temporal overlap – where WG 5 is representing the Joint Video Coding Team(s) and their activities from the perspective of the SC 29 parent body. The subject matter of the JVET meeting activities consisted of work on further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, reference software and conformance testing packages. Further important goals were reviewing the results of the Exploration Experiment (EE) on Neural Network-based Video Coding, of the EE on Enhanced Compression beyond VVC capability, of other technical input on novel aspects of video coding technology, and to plan next steps for investigation of candidate technology towards further standard development.

As a primary goal, the JVET meeting reviewed the work that had been performed in the interim period since the thirtieth JVET meeting in producing the following documents:

- JVET-AD1003 Coding-independent code points for video signal type identification (Draft 2 of 3rd edition), also issued as WG 5 preliminary FDIS N 206

- JVET-AD1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

- JVET-AD1008 Additional colour type identifiers for AVC and HEVC (Draft 4), also issued as WG 5 preliminary WD N 200

- JVET-AD1012 Overview of IT systems used in JVET

- JVET-AD2002 Algorithm description for Versatile Video Coding and Test Model 20 (VTM 20)

- JVET-AD2005 New level and systems-related supplemental enhancement information for VVC (Draft 5)

- JVET-AD2006 Additional SEI messages for VSEI (Draft 4)

- JVET-AD2007 Guidelines for NNVC software development

- JVET-AD2016 Common test conditions and evaluation procedures for neural network-based video coding technology

- JVET-AD2017 Common test conditions and evaluation procedures for enhanced compression tool testing

- JVET-AD2019 Description of algorithms and software in neural network-based video coding (NNVC) version 3

- JVET-AD2021 Verification test plan for VVC multilayer coding

- JVET-AD2022 Draft plan for subjective quality testing of FGC SEI message

- JVET-AD2023 Exploration experiment on neural network-based video coding (EE1)

- JVET-AD2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

- JVET-AD2025 Algorithm description of Enhanced Compression Model 9 (ECM 9)

- JVET-AD2027 SEI processing order SEI message in VVC (Draft 4), also issued as WG 5 preliminary WD N 203

- JVET-AD2028 Additional conformance bitstreams for VVC multilayer configurations

- JVET-AD2030 Optimization of encoders and receiving systems for machine analysis of coded video content (draft 2), also issued as WG 5 preliminary WD N 199

- JVET-AD2031 Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content

As main results, the JVET produced 20 output documents from the current meeting:

- JVET-AE1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

- JVET-AE1006 New profiles, colour decriptors, and SEI messages for HEVC (draft 1), also issued as WG 5 CDAM N 226

- JVET-AE1011 HEVC multiview profiles supporting extended bit depth (draft 2)

- JVET-AE1013 Common test conditions of 3DV experiments

- JVET-AE1016 AVC with extensions and corrections (draft 1), also issued as WG 5 DIS of AVC 11th edition N 218

- JVET-AE2005 New level and systems-related supplemental enhancement information for VVC (Draft 6)

- JVET-AE2006 Additional SEI messages for VSEI (Draft 5)

- JVET-AE2016 Common test conditions and evaluation procedures for neural network-based video coding technology

- JVET-AE2017 Common test conditions and evaluation procedures for enhanced compression tool testing

- JVET-AE2019 Description of algorithms and software in neural network-based video coding (NNVC) version 4

- JVET-AD2020 Film grain synthesis technology for video applications (Draft 5), also issued as WG 5 DTR N 222

- JVET-AE2021 Verification test plan for VVC multilayer coding (update 1)

- JVET-AE2023 Exploration experiment on neural network-based video coding (EE1)

- JVET-AE2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

- JVET-AE2025 Algorithm description of Enhanced Compression Model 10 (ECM 10)

- JVET-AE2027 SEI processing order SEI message in VVC (Draft 5), also issued as WG 5 preliminary WD N 229

- JVET-AE2028 Additional conformance bitstreams for VVC multilayer configurations

- JVET-AE2030 Optimization of encoders and receiving systems for machine analysis of coded video content (draft 3), also issued as WG 5 preliminary WD N 224

- JVET-AE2031 Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content

- JVET-AE2032 Technologies under consideration for future extensions of VSEI (draft 1)

- The following 7 documents were produced as WG 5 documents only:

- WG 5 N 217 Request for ISO/IEC 14496-10 11th edition

- WG 5 N 219 Disposition of comments received on ISO/IEC 23002-7:2022 DAM 1

- WG 5 N 220 Text of ISO/IEC FDIS 23002-7:202x Versatile supplemental enhancement information messages for coded video bitstreams (3rd edition)

- WG 5 N 222 Disposition of comments received on ISO/IEC CDTR 23002-9

- WG 5 N 225 Request for ISO/IEC 23008-2:202x/Amd.1

- WG 5 N 227 Disposition of comments received on ISO/IEC 23090-3:2022 DAM 1

- WG 5 N 228 Text of ISO/IEC FDIS 23090-3:202x Versatile video coding (3rd edition)

- The following 5 draft revised ITU-T Recommendations were forwarded by JVET and Q6/16 for ITU-T Consent:

TD169/Plen ITU-T H.265 (V9) "High efficiency video coding" (Rev.)

TD171/Plen ITU-T H.266 (V3) "Versatile video coding" (Rev.)

TD159/Plen ITU-T H.266.1 (V2) "Conformance specification for ITU-T H.266 versatile video coding" (Rev.)

TD172/Plen ITU-T H.273 (V3) "Coding-independent code points for video signal type identification" (Rev.)

TD170/Plen (A.5: TD173/Plen) ITU-T H.274 (V3) "Versatile supplemental enhancement information messages for coded video bitstreams" (Rev.)

For the organization and planning of its future work, the JVET established 14 “ad hoc groups” (AHGs) to progress the work on particular subject areas. At this meeting, 2 Exploration Experiments (EE) were defined. The next eight JVET meetings were planned for 13 – 20 October 2023 under ISO/IEC JTC 1/‌SC 29 auspices in Hannover, DE; during 17 – 26 January 2024 under ISO/IEC JTC 1/‌SC 29 auspices, to be conducted as teleconference meeting; during 19 – 26 April 2024 under ITU-T SG16 auspices in Rennes, FR; during 12 – 19 July 2024 under ISO/IEC JTC 1/‌SC 29 auspices in Sapporo, JP; during October 2024 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.; during January 2025 under ITU-T SG16 auspices, date and location t.b.d.; during April 2025 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.; and during 26 June – 4 July 2025 under ISO/IEC JTC 1/‌SC 29 auspices in Daejeon, KR.

The document distribution site https://jvet-experts.org/ was used for distribution of all documents. It was noted that the previous sites http://phenix.int-evry.fr/jvet/, http://phenix.int-evry.fr/jct/, and http://phenix.int-evry.fr/jct3v/ are still accessible, but were converted to read-only.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Experts Team (JVET) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and ISO/IEC JTC 1/‌SC 29/‌WG 5. The parent bodies of the JVET are ITU-T WP3/16 and ISO/IEC JTC 1/‌SC 29.

The Joint Video Experts Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/‌SC 29 held its thirty-first meeting during 11–19 July 2023 at the ITU premises in Geneva, Switzerland. The meeting was held as a face-to-face meeting, but remote participation was provided on best-effort basis for experts who were unable to travel. For ISO/IEC purposes, JVET is alternatively designated ISO/IEC JTC 1/‌SC 29/‌WG 5, and this was the twelfth meeting as WG 5. The JVET meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of the Versatile Video Coding (VVC) and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI) standards, as well as associated conformance test sets, reference software, verification testing, and non-normative guidance information. Furthermore, starting from the twentieth meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on:

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

Furthermore, explorations towards possible future need of standardization in the area of video coding are also conducted by JVET. Currently, the following topics are under investigation:

- Exploration on Neural Network-based Video Coding

- Exploration on Enhanced Compression beyond VVC capability

This report contains three important annexes, as follows:

- Annex A contains a list of the documents of the JVET meeting

- Annex B contains a list of the meeting participants, as recorded by the teleconferencing tool used for the meeting

- Annex C contains the meeting recommendations of ISO/IEC JTC 1/‌SC 29/‌WG 5 for purposes of results reporting to ISO/IEC.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2023_07_AE_Geneva/.

## Primary goals

As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the thirtieth JVET meeting in producing the following documents:

- JVET-AD1003 Coding-independent code points for video signal type identification (Draft 2 of 3rd edition), also issued as WG 5 preliminary FDIS N 206

- JVET-AD1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

- JVET-AD1008 Additional colour type identifiers for AVC and HEVC (Draft 4), also issued as WG 5 preliminary WD N 200

- JVET-AD1012 Overview of IT systems used in JVET

- JVET-AD2002 Algorithm description for Versatile Video Coding and Test Model 20 (VTM 20)

- JVET-AD2005 New level and systems-related supplemental enhancement information for VVC (Draft 5)

- JVET-AD2006 Additional SEI messages for VSEI (Draft 4)

- JVET-AD2007 Guidelines for NNVC software development

- JVET-AD2016 Common test conditions and evaluation procedures for neural network-based video coding technology

- JVET-AD2017 Common test conditions and evaluation procedures for enhanced compression tool testing

- JVET-AD2019 Description of algorithms and software in neural network-based video coding (NNVC) version 3

- JVET-AD2021 Verification test plan for VVC multilayer coding

- JVET-AD2022 Draft plan for subjective quality testing of FGC SEI message

- JVET-AD2023 Exploration experiment on neural network-based video coding (EE1)

- JVET-AD2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

- JVET-AD2025 Algorithm description of Enhanced Compression Model 9 (ECM 9)

- JVET-AD2027 SEI processing order SEI message in VVC (Draft 4), also issued as WG 5 preliminary WD N 203

- JVET-AD2028 Additional conformance bitstreams for VVC multilayer configurations

- JVET-AD2030 Optimization of encoders and receiving systems for machine analysis of coded video content (draft 2), also issued as WG 5 preliminary WD N 199

- JVET-AD2031 Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content

Further important goals were reviewing the results of the EE on Neural Network-based Video Coding, of the EE on Enhanced Compression beyond VVC capability, of other technical input on novel aspects of video coding technology, and planning next steps for investigation of candidate technology towards further standard development.

## Documents and document handling considerations

### General

The document distribution site https://jvet-experts.org/ was used for distribution of all documents. It was noted that the previous site http://phenix.int-evry.fr/jvet/ was still accessible, but had been converted to read-only.

Document registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting follow the CEST timezone (local time in Geneva), except as otherwise noted.

Highlighting of recorded decisions in this report is practised as follows:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect one of the various software packages but have no normative effect on text are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in one of the test model descriptions such as VTM, HM, etc. (an error, oversight, or messiness) or in the associated software package are marked by the string “Decision (BF):”.

- Decisions that are merely editorial without effect on the technical content of a draft standard are marked by the string "Decision (Ed.):". Such editorial decisions are merely suggestions to the editor, who has the discretion to determine the final action taken if their judgment differs.

- Other parenthetical comments may be used for describing the impact or motivation of a decision. Some decisions are recorded with the word “agreed” rather than “Decision:”, especially for non-normative, editorial and planning matters.

This meeting report is based primarily on notes taken by the JVET chair. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report. Expressions such as “X.XX%” indicate that the desired results were not available at the time the information was recorded.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Tuesday, 4 July 2023. Any documents uploaded after 1159 hours Paris/Geneva time on Wednesday 5 July 2023 were considered “officially late”, with a grace period of 12 hours (to accommodate those living in different time zones of the world). The deadline does not apply to AHG reports and other such reports which can only be produced after the availability of other input documents.

All contribution documents with registration numbers higher than JVET-AE0183 were registered after the “officially late” deadline (and therefore were also uploaded late). However, some documents in the “late” range might include break-out activity reports that were generated during the meeting, or documents which were requested to be produced for the purpose of improving specification text, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions has been to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This has been agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered and/or uploaded late:

- JVET-AE0052 (a proposal on NNPF cascades and alternatives), uploaded 07-05,

- JVET-AE0095 (a proposal on object mask information SEI), uploaded 07-05,

- JVET-AE0106 (a proposal on missing value ranges in NNPFC SEI), uploaded 07-07,

- JVET-AE0120 (a proposal on Intra Angular Prediction Extension), uploaded 07-08,

- JVET-AE0140 (a proposal on LIC Extensions), uploaded 07-11,

- JVET-AE0184 (a proposal on TIMD with IntraTMP and IBC candidates), uploaded 07-05,

- JVET-AE0185 (a proposal on IntraTMP with neighbouring candidates), uploaded 07-05,

- JVET-AE0186 (a proposal on block vector guided chroma direct mode), uploaded 07-05,

- JVET-AE0189 (a proposal on nnpfa_target_base_flag in NNPFA), uploaded 07-06,

- JVET-AE0191 (a proposal on a HOP filter model), uploaded 07-06,

- JVET-AE0196 (a proposal on a HOP filter model), uploaded 07-06,

- JVET-AE0214 (a proposal on intra CIIP), uploaded 07-16,

- JVET-AE0238 (a proposal on a unified design of NN-based loop-filters), uploaded 07-13,

- JVET-AE0257 (a proposal on peak memory reduction in the ECM), uploaded 07-12,

- JVET-AE0278 (a proposal on memory reduction for the ECM encoder), uploaded 07-13,

- JVET-AE0279 (a proposal on an additional nnpfc_mode_idc), uploaded 07-13,

- JVET-AE0280 (a proposal on generative face video SEI), uploaded 07-13,

- JVET-AE0281 (a proposal on unified LOP filter design, training, and usage), uploaded 07-14,

- JVET-AE0287 (a proposal combination), uploaded 07-16,

- JVET-AE0289 (a proposal on HOP training process and models), uploaded 07-17,

- JVET-AE0291 (a proposal on HOP performance with quantized parameters), uploaded 07-17,

- JVET-AE0296 (a proposal on HEVC Multiview profiles), uploaded 07-18,

- JVET-AE0298 (a proposal combination), uploaded 07-18.

It may be observed that some of the above-listed contributions were submissions made in response to issues that arose in discussions during the meeting or from the study of other contributions, and thus could not have been submitted by the ordinary deadline.

The following other documents not proposing normative technical content, but with some need for consideration, were registered and/or uploaded late:

- JVET-AE0179 (a document presenting a renewed license statement for 3DV test materials), uploaded 07-05,

- JVET-AE0194 (a document presenting a SADL software update), uploaded 07-06,

- JVET-AE0195 (a document discussing ECM complexity assessment), uploaded 07-06,

- JVET-AE0198 (a document discussing extended search region for IntraTMP), uploaded 07-06,

- JVET-AE0227 (a document discussing scalable quality ladder visual testing), uploaded 07-11,

- JVET-AE0250 (a document presenting film grain synthesis material), uploaded 07-12,

- JVET-AE0251 (a document discussing denoising for film grain synthesis), uploaded 07-12,

- JVET-AE0266 (a document presenting a VVC implementation, VVenC), uploaded 07-16,

- JVET-AE0285 (a document presenting a content layering functional test), uploaded 07-16,

- JVET-AE0295 (a document describing the implementation status of multi-view HEVC ), uploaded 07-18.

All cross-verification reports at this meeting were registered late, and/or uploaded late. In the interest of brevity, these are not specifically identified here. Initial upload times for each document are recorded in Annex A of this report.

The following contribution registrations were noted that were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-AE0045.

The cross-verification reports JVET-AE0260, and JVET-AE0292 were still missing at the end of the meeting on Wednesday 19 July, but became available later. The following cross-verification reports were still missing three weeks after the end of the meeting: JVET-AE0215, JVET-AE0233, JVET-AE0269, JVET-AE0270, JVET-AE0299, and JVET-AE0300. These were marked as withdrawn by the JVET chair, assuming their registrations had become obsolete.

“Placeholder” contribution documents that were basically empty of content, or lacking any results showing benefit for the proposed technology, and obviously uploaded with an intent to provide a more complete submission as a revision, had been agreed to be considered unacceptable and to be rejected in the document management system until a more complete version was available (which would then be counted as a late contribution if the update was after the document deadline). At the current meeting, this situation applied with documents JVET-AE0XXX and … , which were categorized as late in the list above, based on the time of the first reasonable document upload; this case did not happen at the current meeting, but the sentence is kept for future use.

Contributions that had significant problems with uploaded versions were not observed at this meeting.

It was remarked that documents that are substantially revised after the initial upload can also be a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This can especially be a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision, although participants tend to seldom notice what is recorded there.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when there was a consensus to consider them and there was sufficient time available for their review. Again, an exception is applied for AHG reports, CE and HLS topic summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there may have been some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). Any such issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors may have also noticed in other initial document uploads (wrong document numbers or meeting dates or meeting locations in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

All output documents of the previous meeting, particularly the meeting report JVET-AD1000, the Coding-independent code points for video signal type identification (Draft 2 of 3rd edition) JVET-AD1003, the Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP JVET-AD1004, the Additional colour type identifiers for AVC, HEVC and Video CICP (Draft 4) JVET-AD1008, the Overview of IT systems used in JVET JVET-AD1012, the Algorithm description for Versatile Video Coding and Test Model 20 (VTM 20) JVET-AD2002, the New level and systems-related supplemental enhancement information for VVC (Draft 5) JVET-AD2005, the Additional SEI messages for VSEI (Draft 4) JVET-AD2006, the Guidelines for NNVC software development JVET-AD2007, the Common test conditions and evaluation procedures for neural network-based video coding technology JVET-AD2016, the Common test conditions and evaluation procedures for enhanced compression tool testing JVET-AD2017, the Description of algorithms and software in neural network-based video coding (NNVC) version 3 JVET-AD2019, the Verification test plan for VVC multilayer coding JVET-AD2021, the Draft plan for subjective quality testing of FGC SEI message JVET-AD2022, the Description of the EE on Neural Network-based Video Coding JVET-AD2023, the Description of the EE on Enhanced Compression beyond VVC capability JVET-AD2024, the Algorithm description of Enhanced Compression Model 9 (ECM 9) JVET-AD2025, the SEI processing order SEI message in VVC (Draft 4) JVET-AD2027, the Additional conformance bitstreams for VVC multilayer configurations JVET-AD2028, the Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 2) JVET-AD2030, and the Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content JVET-AD2031, had been completed and were approved. In a few cases, the corresponding WG 5 N-numbered documents had not yet been uploaded, and this was requested to be done as soon as possible. The software implementations of VTM versions 20.1, 20.2, and 21.0, ECM (versions 9.0 and 9.1), and NNVC (versions 5.0 and 5.1) were also approved.

Only minor editorial issues were found in the meeting report JVET-AD1000; no need to produce an update was identified (see section 2.14 for details).

The available output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annexes B1 and B2 of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 5 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

It was further announced that it is necessary to register for the meeting through the ISO Meetings website for ISO/IEC experts or through the Q6/16 rapporteur for ITU-T experts. The password for meeting access had been sent to registered participants via these channels. Links to the Zoom sessions (without the necessary password) were available in the posted meeting logistics information and the calendar of meeting sessions in the JVET web site.

The following rules were established for those participating remotely via Zoom teleconference meeting:

- Use the “hand-raising” function to enter yourself in the queue to speak (unless otherwise instructed by the session chair). If you are dialed in by phone, request your queue position verbally. The online queue will be interleaved with the room queue, though it may not always be guaranteed that the sequence perfectly follows the sequence by which hand raising occurred.

- Stay muted unless you have something to say. People are muted by default when they join and need to unmute themselves to speak. The chair may mute anyone who is disrupting the proceedings (e.g. by forgetting they have a live microphone while chatting with their family or by causing bad noise or echo).

- Identify who you are and your affiliation when you begin speaking. The same applies for speakers in the room to let online participants know who is speaking.

- Use your full name and company/organization and country affiliation in your joining information, since the participation list of Zoom would also be used to compile the online part of attendance records.

- Turn on the chat window and watch for chair communication and side commentary there as well as by audio.

- Generally do not use video for the teleconferencing calls in order to avoid overloading internet connections; enable only voice and screen sharing.

- Extensive use of screen sharing is encouraged, to enable participants to view the presented material and the meeting notes. At times, multiple sources of screen sharing may be enabled, so it may be necessary for participants to understand that this is happening and to understand how to select which shared screen they want to see.

## Agenda

The agenda for the meeting, for the further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, software and conformance packages, was as follows:

Opening remarks and review of meeting logistics and communication practices

Roll call of participants

Adoption of the agenda

Code of conduct policy reminder

IPR policy reminder and declarations

Contribution document allocation

Review of results of the previous meeting

Review of target dates

Reports of ad hoc group (AHG) activities

Report of exploration experiments on neural-network-based video coding

Report of exploration experiments on enhanced compression beyond VVC capability

Consideration of contributions on high-level syntax

Consideration of contributions and communications on project guidance

Consideration of video coding technology contributions

Consideration of contributions on conformance and reference software development

Consideration of contributions on coding-independent code points for video signal type identification

Consideration of contributions on film grain synthesis technology

Consideration of contributions on optimization of encoders and receiving systems for machine analysis of coded video content

Consideration of contributions on errata relating to standards in the domain of JVET

Consideration of contributions on technical reports relating to standards and exploration study activities in the domain of JVET

Consideration of contributions providing non-normative guidance relating to standards and exploration study activities in the domain of JVET

Consideration of information contributions

Consideration of future work items

Coordination of visual quality testing

Liaisons, coordination activities with other organizations

Review of project editor and liaison assignments

Approval of output documents and associated editing periods

Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments (if any), establishment of AHGs, future meeting planning, other planning issues

Other business as appropriate for consideration

Closing

- The agenda was approved as suggested.

- The times of meeting sessions followed the needs of the face-to-face meeting, with highest priority given to the aim of achieving the goals of the meeting. Typical meeting hours were expected to be 0830-1900 CEST with coffee breaks and lunch breaks as appropriate, however some early morning or late-night sessions were anticipated to be necessary. Sessions were announced in the online JVET calendar in advance as far as possible, but it was anticipated that some activities (such as breakout sessions) could be held at short notice.

## ISO and IEC Code of Conduct reminders

Participants were reminded of the ISO and IEC Codes of Conduct, found at

https://www.iso.org/publication/PUB100011.html.

https://www.iecapc.jp/F/IEC_Code_of_Conduct.pdf

These include points relating to:

- Complying with legal and statutory obligations

- Performing and acting in good faith, consistent with the purpose, policies and principles of the organization

- Behaving ethically

- Promoting and enabling all voices to be heard

- Engaging constructively in ISO and IEC activities

- Declaring actual and potential conflicts of interest and managing them appropriately

- Protecting confidential information

- Protecting ISO and IEC assets

- Avoiding and preventing any form of bribery or corruption

- Escalating and resolving disputes and upholding agreed resolutions

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that the VTM and ECM software implementation packages use the same software copyright license header as the HEVC reference software, where the latter had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time. This license header language is based on the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N 10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11 of June/July 2009. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software header is also used in the process of designing the VTM and ECM software, and for evaluating proposals for technology to be potentially included in these designs. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

These considerations apply to the 360Lib video conversion software and HDRTools as well. The SADL and NNVC packages for neural network-based video coding use the same licensing terms.

Software packages that had been developed in prior work of the JVT, the JCT-VC and the JCT-3V have similar considerations and are maintained according to the past practice in that work.

## Communication practices

The documents for the meeting can be found at https://jvet-experts.org/. It was noted that the previous site http://phenix.int-evry.fr/jvet/ is still accessible, but was converted to read-only. It was reminded to send a notice to the chairs in cases of changes to document titles, authors, etc.

JVET email lists are managed through the site https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers on the JVET email list was 1215 (as of 10 July 2023). All discussions (including those on AVC, HEVC, VVC, CICP, etc.) shall be conducted on the JVET reflector rather than any of the old reflectors (including JVT, JCT-VC, and JCT-3V) which are retained for archiving purposes.

For distribution of test sequences, a password-protected ftp site had been set up at RWTH Aachen University, with a mirror site at FhG-HHI. Accredited members of JVET may contact the responsible JVET coordinators to obtain the password information (but the site is not open for use by others).

It is further emphasized that the document JVET-AD1012 gives valuable hints about communication practices as well as other IT resources used in JVET, such as software, conformance, and test materials.

## Terminology

- ACT: Adaptive colour transform

- AFF: Adaptive frame-field

- AI: All-intra

- AIF: Adaptive interpolation filtering

- ALF: Adaptive loop filter

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component)

- AMVP: Adaptive motion vector prediction

- AMT or MTS: Adaptive multi-core transform, or multiple transform selection

- AMVR: (Locally) adaptive motion vector resolution

- APS: Adaptation parameter set

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR)

- ARMC: Adaptive re-ordering of merge candidates

- ARSS: Adaptive reference sample smoothing

- ATM: AVC-based multiview and 3D test model

- ATMVP or “subblock-based temporal merging candidates”: Alternative temporal motion vector prediction

- AU: Access unit

- AUD: Access unit delimiter

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10

- BA: Block adaptive

- BC: See CPR or IBC

- BCW: Biprediction with CU based weighting

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001)

- BDOF: Bi-directional optical flow (formerly known as BIO)

- BDPCM: Block-wise DPCM

- BL: Base layer

- BMS: Benchmark set (no longer used), a former preliminary compilation of coding tools on top of VTM, which provide somewhat better compression performance, but are not deemed mature for standardzation

- BoG: Break-out group

- BR: Bit rate

- BT: Binary tree

- BV: Block vector (used for intra BC prediction)

- CABAC: Context-adaptive binary arithmetic coding

- CBF: Coded block flag(s)

- CC: May refer to context-coded, common (test) conditions, or cross-component

- CCALF: Cross-component ALF

- CCLM: Cross-component linear model

- CCCM: Cross-component convolutional model

- CCP: Cross-component prediction

- CCSAO: Cross-component SAO

- CE: Core Experiment – a coordinated experiment conducted toward assessment of coding technology

- CG: Coefficient group

- CGS: Colour gamut scalability (historically, coarse-grained scalability)

- CIIP: Combined inter/intra prediction

- CIPF: CABAC initialization from the previous frame

- CL-RAS: Cross-layer random-access skip

- CPB: Coded picture buffer

- CPMV: Control-point motion vector

- CPMVP: Control-point motion vector prediction (used in affine motion model)

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- CST: Chroma separate tree

- CTC: Common test conditions

- CVS: Coded video sequence

- DCI: Decoder capability information

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics)

- DCTIF: DCT-derived interpolation filter

- DF: Deblocking filter

- DIMD: Decoder intra mode derivation

- DMVR: Decoder motion vector refinement

- DoCR: Disposition of comments report

- DPB: Decoded picture buffer

- DPCM: Differential pulse-code modulation

- DPS: Decoding parameter sets

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR)

- DT: Decoding time

- DQ: Dependent quantization

- ECS: Entropy coding synchronization (typically synonymous with WPP)

- EMT: Explicit multiple-core transform

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element)

- ECM: Enhanced compression model – a software codebase for future video coding exploration

- ECV: Extended Colour Volume (up to WCG)

- EL: Enhancement layer

- EOS: End of (coded video) sequence

- ET: Encoding time

- FRUC: Frame rate up conversion (pattern matched motion vector derivation)

- GCI: General constraints information

- GDR: Gradual decoding refresh

- GLM: Gradient linear model

- GOP: Group of pictures (somewhat ambiguous)

- GPM: Geometry partitioning mode

- GRA: Gradual random access

- HBD: High bit depth

- HDR: High dynamic range

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2

- HLS: High-level syntax

- HM: HEVC Test Model – a video coding design containing selected coding tools that conforms to the HEVC standard design (possibly with under-development extensions) – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM)

- HMVP: History based motion vector prediction

- HOP: High-complexity operating point for neural network-based filter.

- HRD: Hypothetical reference decoder

- HTM: HEVC-based multiview and 3D test model (developed by JCT-3V)

- HyGT: Hyper-cube Givens transform (a type of NSST)

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample)

- IBF: Intra boundary filtering

- ILP: Inter-layer prediction (in scalable coding)

- ILRP: Inter-layer reference picture

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC)

- IRAP: Intra random access picture

- ISP: Intra subblock partitioning

- JCCR: Joint coding of chroma residuals

- JCT-3V: Joint collaborative team on 3D video (for AVC and HEVC)

- JCT-VC: Joint collaborative team on video coding (for HEVC)

- JEM: Joint exploration model – a software codebase previously used for video coding exploration

- JM: Joint model – the primary software codebase that has been developed for the AVC standard

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions

- JVET: Joint video experts team (initially for VVC, later expanded)

- JVT: Joint video team (for AVC)

- KLT: Karhunen-Loève transform

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP

- LFNST: Low-frequency non-separable transform

- LIC: Local illumination compensation

- LM: Linear model

- LMCS: Luma mapping with chroma scaling (formerly sometimes called “in-loop reshaping”)

- LOP: Low-complexity operating point for neural network-based filter.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames

- LUT: Look-up table

- LTRP: Long-term reference picture

- MANE: Media-aware network element

- MC: Motion compensation

- MCP: Motion compensated prediction

- MCTF: Motion compensated temporal pre-filtering

- MDNSST: Mode dependent non-separable secondary transform

- MIP: Matrix-based intra prediction

- MMLM: Multi-model (cross component) linear mode

- MMVD: Merge with MVD

- MPEG: Moving picture experts group (an alliance of working groups and advisory groups in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET)

- MPM: Most probable mode (in intra prediction)

- MRL: Multiple reference line intra prediction

- MV: Motion vector

- MVD: Motion vector difference

- NAL: Network abstraction layer

- NNVC: Neural network-based video coding (experimental software package)

- NSQT: Non-square quadtree

- NSST: Non-separable secondary transform

- NUH: NAL unit header

- NUT: NAL unit type (as in AVC and HEVC)

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F)

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value

- OLS: Output layer set.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- ONNX: Open Neural Network Exchange – a format used to convert code from common neural network software packages into SADL code.

- operation point: A temporal subset of an OLS.

- PDPC: Position-dependent (intra) prediction combination.

- PERP: Padded equirectangular projection (a 360° projection format).

- PH: Picture header.

- PHEC: Padded hybrid equiangular cubemap (a 360° projection format).

- PMMVD: Pattern-matched motion vector derivation.

- POC: Picture order count.

- PoR: Plan of record.

- PROF: Prediction refinement with optical flow

- PPS: Picture parameter set (as in AVC and HEVC).

- PTL: Profile/tier/level combination.

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading (type of picture).

- RASL: Random-access skipped leading (type of picture).

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- RDPCM: Residual DPCM

- ROT: Rotation operation for low-frequency transform coefficients.

- RPL: Reference picture list.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SADL: Small adhoc deep learning library

- SAO: Sample-adaptive offset.

- SBT: Subblock transform.

- SbTMVP: Subblock based temporal motion vector prediction.

- SCIPU: Smallest chroma intra prediction unit.

- SD: Slice data; alternatively, standard-definition.

- SDH: Sign data hiding.

- SDT: Signal-dependent transform.

- SE: Syntax element.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIF: Switchable (motion) interpolation filter.

- SIMD: Single instruction, multiple data.

- SMVD: Symmetric MVD.

- SPS: Sequence parameter set (as in AVC and HEVC).

- STMVP: Spatial-temporal motion vector prediction.

- STRP: Short-term reference picture.

- STSA: Step-wise temporal sublayer access.

- TBA/TBD/TBP: To be announced/determined/presented.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- TIMD: Template-based intra mode derivation

- TM: Template matching.

- TMVP: Temporal motion vector prediction.

- TS: Transform skip.

- TSRC: Transform skip residual coding.

- TT: Ternary tree.

- UCBDS: Unrestricted center-biased diamond search.

- UGC: User-generated content.

- UWP: Unequal weight prediction.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VCM: Video coding for machines.

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- VQA: Visual quality assessment.

- VT: Verification testing.

- VTM: VVC Test Model.

- VUI: Video usability information.

- VVC: Versatile Video Coding, the standardization project developed by JVET.

- WAIP: Wide-angle intra prediction

- WCG: Wide colour gamut.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

- Block and unit names in VVC:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in a P or B slice or in an I slice that uses a single tree, and one CTB per luma CTU and two CTBs per chroma CTU in an I slice that uses separate trees.

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in a P or B slice or in an I slice that uses a single tree, containing only luma or only chroma in an I slice that uses separate trees), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component.

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape.

  - PB: Prediction block, a luma or chroma block of a PU.

  - PU: Prediction unit, has the same size as a CU in the VVC context.

  - TB: Transform block, a luma or chroma block of a TU.

  - TU: Transform unit, has the same size as a CU in the VVC context.

## Standards, TRs, supplements and technical papers approval and publication status

- MPEG-2 | H.262 (coding specification is common text)

  - ITU-T H.262 V3 was approved in 2012-02; Amd.1 was approved in 2013-03 and was not published separately; it was instead incorporated directly into the V3 text and published 2013-09

  - ISO/IEC 13818-2:2013 V3 FDIS ballot closed 2012-05-08; FDAM 1 ballot closed 2013-04-12 and was not published separately; it was instead incorporated directly into the V3 text and published 2013-10

  - Conformance testing (not joint with ITU-T)

    - ISO/IEC 13818-4:2004 V2 FDIS closed 2004-08-22, published 2004-12-12; it specifies conformance testing for Part 1 (Systems), Part 2 (Video), Part 3 (Audio), and Part 7 (AAC)

    - ISO/IEC 13818-4:2004/Amd 3:2009 Level for 1080@50p/60p conformance testing

    - Cor 1:2007, Cor 2:2009, Cor 3:2012, Cor 4:2011 may also have video relevance

  - Reference software (not joint with ITU-T)

    - ISO/IEC TR 13818-5:2005 V2 FDIS closed 2005-07-24, published 2005-10; it specifies reference software for Part 1 (Systems), Part 2 (Video), Part 3 (Audio), Part 7 (AAC) and Part 11 (IPMP)

- AVC (twin text)

  - ITU-T H.264 V14 was Consented at 22nd meeting on 2021-04-30 (with annotated regions, shutter interval, and miscellaneous corrections), approved 2021-08-22, published 2021-10-13

  - ISO/IEC 14496-10:2020 (Ed. 9) FDIS ballot closed 2020-11-27, published 2020-12-15

  - ISO/IEC 14496-10:2022 (Ed. 10), had been forwarded from DIS directly for publication 2022-01-21 (with annotated regions, shutter interval, and miscellaneous corrections) with an editing period, submitted to ITTF in 2022-05 after consultation with ISO staff on format of graphics files, upgraded to “DIS approved for registration” in ISO Project system 2022-07-04, published 2022-11-07

  - Preliminary draft text for YCgCo-Re and YCgCo-Ro issued at 26th meeting, second draft including SMPTE ST 2128 issued at 28th meeting 2022-10, third draft issued at 29th meeting 2023-01, fourth draft issued at 30th meeting 2023-04 (not yet formally requested as a project)

  - Conformance testing (twin text)

    - ITU-T H.264.1 V6 Approved 2016-02-13, published 2016-06-17

    - Various amendments of ISO/IEC 14496-4:2004, including:

      - ISO/IEC 14496-4:2004/AMD 6:2005 Advanced Video Coding conformance

      - ISO/IEC 14496-4:2004/AMD 9:2006 AVC fidelity range extensions conformance

      - ISO/IEC 14496-4:2004/AMD 30:2009 Conformance testing for new profiles for professional applications

      - ISO/IEC 14496-4:2004/AMD 31:2009 Conformance testing for SVC profiles

      - ISO/IEC 14496-4:2004/AMD 38:2010 Conformance testing for Multiview Video Coding

      - ISO/IEC 14496-4:2004/AMD 41:2014 Conformance testing of MVC plus depth extension of AVC

      - ISO/IEC 14496-4:2004/AMD 42:2014 Conformance testing of Multi-Resolution Frame Compatible Stereo Coding extension of AVC

      - ISO/IEC 14496-4:2004/AMD 43:2015 3D-AVC conformance testing

      - ISO/IEC 14496-4:2004/AMD 45:2016 Conformance Testing for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps Extension of AVC

  - Reference software (twin text)

    - ITU-T H.264.2 V7 Approved 2016-02-13, published 2016-05-30

    - Various amendments of ISO/IEC 14496-5:2001 have been published, including:

      - ISO/IEC 14496-5:2001/AMD 6:2005 Advanced Video Coding (AVC) and High Efficiency Advanced Audio Coding (HE AAC) reference software

      - ISO/IEC 14496-5:2001/AMD 8:2006 AVC fidelity range extensions reference software

      - ISO/IEC 14496-5:2001/AMD 15:2010 Reference software for Multiview Video Coding

      - ISO/IEC 14496-5:2001/AMD 18:2008 Reference software for new profiles for professional applications

      - ISO/IEC 14496-5:2001/AMD 19:2009 Reference software for Scalable Video Coding

      - ISO/IEC 14496-5:2001/AMD 33:2015 Reference software for MVC plus depth extension of AVC

      - ISO/IEC 14496-5:2001/AMD 34:2014 Reference software of the multi-resolution frame compatible stereo coding of AVC

      - ISO/IEC 14496-5:2001/AMD 35:2015 3D-AVC Reference software

      - ISO/IEC 14496-5:2001/AMD 39:2016 Reference software for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps of AVC

      - ISO/IEC 14496-5:2001/AMD 42:2017 Reference software for the alternative depth information SEI message extension of AVC

- HEVC (twin text)

  - ITU-T H.265 V7 approved 2019-11-29, published 2020-01-10

  - ISO/IEC 23008-2:2020 (Ed. 4) FDIS ballot closed 2020-07-16, published 2020-08-27

  - ITU-T H.265 V8 Consented at the 22nd meeting (shutter interval information SEI message and miscellaneous corrections), published 2020-10-13

  - ISO/IEC 23008-2:2020/AMD 1:2021 (shutter interval information SEI message) published 2021-07-12

  - ISO/IEC 23008-2:202x (Ed. 5) began as CDAM 2 High-range levels output of 25th meeting of January 2022, CDAM ballot closed 2022-04-15, conversion to 5th edition with miscellaneous corrections planned at 26th meeting of 2022-04, text submitted for DIS ballot 2022-07-10, DIS ballot closed 2023-01-10, FDIS issued 29th meeting of 2023-01, waiting for FDIS ballot (ready for Consent for H.265 at this meeting)

  - Preliminary draft text for YCgCo-Re and YCgCo-Ro issued at 26th meeting 2022-04, second draft including SMPTE ST 2128 issued at 28th meeting 2022-10, third draft at 29th meeting 2023-01, fourth draft at 30th meeting 2023-04 (not yet formally requested as a project)

  - Conformance testing (twin text)

    - ITU-T H.265.1 V3 approved 2018-10-14, published 2019-01-15

    - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06

    - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15

  - Reference software (twin text)

    - ITU-T H.265.2 V4 approved 2016-12-22, published 2017-04-10

    - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for high efficiency video coding, published 2017-03-01

    - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for screen content coding extensions, published 2017-11-09

- VVC (twin text)

  - ITU-T H.266 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23090-3:2021 (Ed. 1) published 2021-02-16

  - ITU-T H.266 V2 with operation range extensions, Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-06-06, published 2022-07-12

  - ISO/IEC 23090-3:2022 (Ed. 2) with operation range extensions, approval at WG level to proceed to FDIS 2022-01-21, published 2022-09-25

  - ISO/IEC 23090-3:202x (Ed. 2) / Amd.1 New level and systems-related supplemental enhancement information, CDAM 1 issued from 26th meeting, ballot closed 2022-07-14, DAM 1 issued from 27th meeting, ballot closed 2023-01-03 (ready for FDIS and H.266 v3 Consent at this meeting)

  - Conformance testing (twin text)

    - ITU-T H.266.1 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-15:2022 V1 approval at WG level to proceed to FDIS 2022-10-15, upgraded to “DIS approved for registration” in ISO Projects system 2021-10-24, upgraded to “FDIS registered for formal approval” 2022-07-11, FDIS ballot closed 2022-11-04, published 2022-11-24

    - ISO/IEC 23090-15:202x Amd.1 Operation range extensions – DAM 1 issued from 25th meeting 2022-01-21, upgraded to “CD approved for registration as DIS” status in ISO Projects system 2022-05-31, upgraded to “DIS registered” 2022-06-22, DAM ballot closed 2022-11-15, consolidated into FDIS issued as an output of the 29th meeting in January 2023 (remaining under preparation at the time of this meeting; ready for H.266.1 Consent at this meeting)

  - Reference software (twin text)

    - ITU-T H.266.2 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-16:2022 V1 approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-04-21, upgraded to “FDIS registered for formal approval” 2022-04-22, FDIS ballot initiated 2022-07-24, FDIS ballot closed 2022-09-19, published 2022-10-23

- VSEI (twin text)

  - ITU-T H.274 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23002-7:2021 (Ed. 1) published 2021-01-28

  - ITU-T H.274 V2 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-05-22 (after 1 Last Call comment and Additional Review), pre-published 2022-06-17, published 2022-07-25

  - ISO/IEC 23002-7:2022 (Ed. 2) approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-05-05 and “FDIS registered for formal approval” 2022-05-08, FDIS ballot closed 2022-09-27, published 2022-10-30

  - ISO/IEC 23002-7:202x (2nd Ed.) Amd.1 Request for new edition and CD for additional SEI messages issued at 27th meeting, ballot closed 2022-10-10, DAM registered 2022-11-13, DAM ballot closed 2022-04-06: comments in m62571 from Finland, Japan, US, ISO CS (minor) (ready for FDIS and H.274 Consent at this meeting)

- CICP (twin text)

  - ISO/IEC 23091-2:2021 (Ed. 2) had been forwarded from DIS directly for publication in 2021-04 and published 2021-10-18

  - ITU-T H.273 V2 (with 4:2:0 sampling alignment and corrections for range of values for sample aspect ratio, ICTCP equations for HLG, and transfer characteristics function for sYCC of IEC 61966-2-1) Consented on 2021-04-30, Last Call closed during the 23rd meeting with approval on 2021-07-14, published 2021-09-24

  - ISO/IEC 23091-2:202x (Ed. 3) Request for new edition and CD for new edition (including YCgCo-Re and YCoCg-Ro) issued at 27th meeting, ballot closed 2022-10-10, DIS registered 2022-11-13, DIS ballot closed 2023-04-06, preliminary draft text for including SMPTE ST 2128 issued at 28th meeting, incorporated into preliminary FDIS at 30th meeting 2023-04 (ready for FDIS and H.273 Consent at this meeting)

- Conversion and coding practices for HDR/WCG Y′CbCr 4:2:0 video with PQ transfer characteristics (twin text)

  - H.Sup15 V1, approved 2017-01-27, published 2017-04-12

  - ISO/IEC TR 23008-14:2018 published 2018-08

- Signalling, backward compatibility and display adaptation for HDR/WCG video coding (twin text)

  - H.Sup18 V1, approved 2017-10-27, published 2018-01-18

  - ISO/IEC TR 23008-15:2018 published 2018-08

- Usage of video signal type code points (twin text)

  - H.Sup19 V3 approved 2021-04-30, published 2021-06-04

  - ISO/IEC TR 23091-4 (Ed. 3) published 2021-05-23

- Working practices using objective metrics for evaluation of video coding efficiency experiments (twin text)

  - HSTP-VID-WPOM V1: approved 2020-07-03, published 2020-11

  - ISO/IEC TR 23002-8 (Ed. 1) published 2021-05-20

- Film grain synthesis technologies for video applications (twin text)

  - ISO/IEC TR 23002-9 Request for subdivision and WD 1 issued at 25th meeting 2022-01-21, WD 2 issued at 27th meeting, WD 3 issued at 28th meeting, CDTR issued at 29th meeting 2023-07, consultation period ended 2023-07-09 (ready for DTR at this meeting)

- The following freely available standards are published here in ISO/IEC:
https://standards.iso.org/ittf/PubliclyAvailableStandards/index.html as of 2023-05-26.

  - ISO/IEC 13818-4:2004 Conformance for MPEG-2

  - ISO/IEC 13818-4:2004/Amd 3:2009 Level for 1080@50p/60p conformance testing

  - ISO/IEC TR 13818-5:2005 Software simulation for MPEG-2

  - Various amendments of ISO/IEC 14496-4:2004 Conformance for AVC

  - Various amendments of ISO/IEC 14496-5:2001 Reference software for AVC

  - ISO/IEC 14496-10:2022 (Ed. 10) AVC

  - ISO/IEC 23002-7:2022 (Ed. 2) – VSEI

  - ISO/IEC 23008-2:2020 (Ed. 4) HEVC

  - ISO/IEC 23090-3:2022 (Ed. 2) VVC

  - ISO/IEC 23090-15:2022 (Ed. 1) Conformance for VVC

  - ISO/IEC 23090-16:2022 (Ed. 1) Reference software for VVC

  - ISO/IEC 23091-2:2021 (Ed. 2) Video CICP

- The following standards that have been intended by JVET to be publicly available were not available at https://standards.iso.org/ittf/PubliclyAvailableStandards/index.html as of 2023-05-26 and this was still the case as of 2023-08-03. (These should be checked for previously issued requests for free availability.)

  - ISO/IEC 23008-2:2020 (Ed. 4) Amd.1:2021: Shutter interval information SEI message, published 2021-07-12 (has not been requested)

  - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for high efficiency video coding, published 2017-03-01

  - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for screen content coding extensions, published 2017-11-09

  - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08, published 2018-08-06

  - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15

- It appears necessary to check if all older software and conformance packages are publicly available – it might be that it was never requested, e.g. for those that were produced by JCT-3V. This topic was left TBD until the next meeting – perhaps it would be best to compile a list of all relevant software and conformance parts of AVC, HEVC, MPEG-2 aka H.262, CICP, and request these in bulk.

## Draft standards progression status

- AVC and HEVC colour type indicators for YCgCo-Re, YCgCo-Ro, and SMPTE ST 2128 (IPT-PQ-C2) are drafted and pending formal action.

- HEVC new levels (from JVET-Z1005) – ISO/IEC 23008-2 DIS of new edition of HEVC was issued from the April 2022 26th meeting, incorporating Amd.1 and corrigenda items (ballot closed 2023-01-10, ballot comments in the Summary of Voting document m61834); note that Amd.1 = shutter interval SEI is already included in latest ITU-T edition of H.265. It is noted that there are potential additional items (corrigenda+tickets, YCgCo-Re and YCgCo-Ro draft, SMPTE ST 2128, multiview profiles draft) where only corrigenda items were included in the FDIS text based on ballot comments, ballot had not been started yet. ITU-T consent for a new edition is planned for July 2023. It was noted that the referencing of VSEI is also somewhat different in the ITU-T and ISO/IEC versions of HEVC and/or AVC, which might be aligned at the next convenient time (basically editorial – e.g., the ITU version of AVC specifies the annotated regions SEI message without referencing VSEI, whereas the ISO/IEC version references VSEI for the syntax and semantics of that SEI message). However, there is currently no other need for HEVC to reference the VSEI standard. An FDIS for HEVC was issued as an output of the 29th meeting in January 2023 (and it does not reference VSEI). Ready for H.265 Consent at this meeting.

- VVC new level and systems-related supplemental enhancement information (from JVET-AA2005) – VVC DAM was issued from 27th meeting, ballot closed 2023-01-03, ballot comments in the Summary of Voting document m61833. This was converted into a preliminary FDIS of VVC 3rd edition (WG 5 N 183) at the 29th meeting of January 2023, anticipating that some alignment will be necessary with the ongoing VSEI amendment. ITU-T H.266 Consent was expected for the current meeting when a new edition of VSEI is also planned to be consented, to keep VVC and VSEI aligned.

- VVC Conformance testing for operation range extensions – (from JVET-Y2026) – the DAM ballot closed 2022-11-15 (ballot comments in the Summary of Voting document m61832), and this was consolidated into an FDIS at the 29th meeting, but the ballot had not been started yet. ITU-T H.266.1 Consent was planned for the current meeting.

- VSEI additional SEI messages (from JVET-AB2006) – VSEI DAM (JVET draft 3) was issued from the 28th meeting and a DAM ballot was issued, Summary of Voting document is available as m62571. This was planned to be consolidated into FDIS of a new edition and reach ITU-T Consent at the current meeting.

- Film grain synthesis technology for video applications – JVET draft 4 was issued at the 29th meeting (JVET-AC2020), and the ISO/IEC 23002-9 CDTR was issued (a request to start work on the TR had been made at the 25th meeting), and the CDTR consultation period ended 2023-07-09 (ready for DTR at this meeting; it was noted that a second DTR could become necessary in case of comments). The publication limit date was reportedly 2023-08-09, so action to extend that date may be needed. No action in ITU-T at this meeting.

- Video CICP new edition with for YCgCo-Re and YCgCo-Ro (from JVET-Z1003), an ISO/IEC 23091-2 preliminary FDIS was issued from the 30th meeting and the Summary of Voting document was available as m62572 and a draft DoC had been issued as WG 5 N 205. A delay might be considered depending on status of SMPTE ST 2128, which was tentatively included in the preliminary FDIS, based on an NB comment. Video CICP colour type indicator for SMPTE ST 2128 had been drafted and incorporated into the preliminary FDIS issued at the 30th meeting of 2023-04. It had been reported that the specification was expected to become finalized in the SMPTE meeting in March 2023. FDIS and ITU-T Consent had been planned for the current meeting of July 2023.

- A request for free availability in ISO/IEC has to be made for each edition, amendment and corrigendum, and the request needs to be approved in the WG 5 Recommendations. A request form also needs to be filled out (but the form does not need to be issued as a WG 5 document). A freely available URL for the ITU publication should be provided for the following parts:

  - For the ongoing work items, when they become finalized

  - ISO/IEC 23008-2:2020/Amd.1:2021 – HEVC FDAM issued 20th meeting (October 2020), public availability not yet requested but may not be necessary as it becomes included in next edition

## Opening remarks

Remarks during the opening session of the meeting Tuesday 11 July at 0930 CEST were as follows.

- Timing and organization of the meeting and online access, calendar posting of session plans

  - The initial number of documents was lower than for the previous meeting (200->160) – parallel sessions might nevertheless be necessary, considering the preparation of various deliverables.

  - Scheduling of NNVC discussions – might be alignement with NNPF discussions, and starting from July 15 with JPEG-AI (JPEG meeting starts only next week and finishes after JVET)

- Plans for subsequent F2F meeting in October (Hannover), April (??), and July 2024 (Sapporo).

- The meeting logistics, agenda, working practices, policies, and document allocation considerations were reviewed.

  - Remote access to the meeting was provided using Zoom. This required discipline in the meeting room (no microphone to be switched on, podium and room microphones to be under central control).

  - Having text and software available is crucial (and not just arriving at the end of the meeting).

  - There were no objections voiced in the opening plenary to the consideration of late contributions.

- The results of the previous meeting and the meeting report JVET-AD1000 were reviewed. The following small issues in the meeting report were noted and were not considered sufficient to warrant issuing a revision. These are obviously left over from a previous report, and the correct information can be found in other places of the report:

  - The title of the document designates the meeting as teleconference meeting, whereas it had been held in face-to-face mode in Antalya, TR.

  - In the summary (section 1), the meeting of April 2025 was expressed to be held under ITU-T SG16 auspices, whereas regularly it should be expected to be held under ISO/IEC SC 29 auspices.

  - In the summary (section 1), JVET-AC2003 was wrongly listed as output from the meeting. The corresponding line should have been removed, and the total number of outputs was 20 instead of 21.

  - In 2.4.3 outputs of preceding meeting, document JVET-AD2027 (SEI processing order SEI message) was designated to be draft 2, whereas actually it had already been the third draft. Further below, a reference to section 2.12 about minor errors in the previous meeting report should have been referencing section 2.14 (opening remarks).

  - In section 8.2 (experiment planning), documents JVET-AC2023 and JVET-AC2024 are referred to instead of JVET-AD2023 and JVET-AD2024, respectively.

- There was a somewhat increased numberof late non-cross-check documents, compared to previous meetings.

- There were only few documents registered where authors’ given names were not abbreviated, and/or company affiliation was missing in the authors’ list. Participants were reminded to stick to JVET’s conventions.

- Experts were asked to inform the chair when the title of a document is changed, or if authors are added. Otherwise, that might not be correct in the meeting notes.

- The primary goals of the meeting were:

  - 2nd edition of VVC confomance (H.266.1) is originally planned to be delivered for ITU consent at this meeting, but the corresponding JVET document JVET-AC2026 and WG 5 FDIS 23090-15 N 185 have not been produced yet. To be clarified from AHG5 report.

  - Any action on a new version of VVC software as standards part – target April 2024, for inclusion of NNPF SEI, software from TRs, layered coding, bug fixes, etc.

  - VVC and VSEI third editions FDIS, and consent on next ITU editions

    - VVC DAM ballot response in m61833, draft DoCR (from last meeting) in MDS22705, and preliminary FDIS text in MDS22706.

    - VSEI DAM ballot response in m62571, draft DoCR (from last meeting) in MDS22702, and preliminary FDIS text in MDS22703.

  - New edition video CICP FDIS (DIS ballot response in m62572 draft DoCR in MDS22710, and preliminary FDIS text in MDS22711), and consent for next ITU next edition) – inclusion of ST 2128 descriptor needs clarification of the status, this may impose delay

  - TR on film grain synthesis technology for video applications – CD TR 23002-9 ballot response in m63723, plan to issue DTR in ISO, target ITU consent for April 2024

  - Optimization of encoders and receiving systems for machine analysis of coded video content – new WD of TR to be issued

  - Preparation of verification tests film grain and multilayer – expert viewing?

  - Any action items on reference software JM/HM/VTM? Status of MV-HEVC software and test conditions (refer to resolution of last meeting for the latter)?

  - Plan for new edition of AVC (both ISO and ITU)

    - Additional colour type identifiers for AVC and HEVC (Draft 3 in JVET-AD1008 was issued at the last meeting) – clarify for status of SMPTE ST 2128 (same as for CICP

    - Errata items from JVET-AD1004

    - Anything else? SEI messages?

    - Would be premature for ITU consent (could be targeted for April 2024), but ISO CD (or DIS) and request could be issued

  - Plan for new edition of HEVC (only ITU version; FDIS of ISO version was issued recently)

    - Additional colour type identifiers for AVC and HEVC (Draft 3 in JVET-AD1008 was issued at the last meeting) – clarify for status of SMPTE ST 2128 (may not be included)

    - Errata items from JVET-AD1004

    - Anything else? SEI messages? 10-bit Multiview profile (depends on SW and CT status)? Another H.265 edition could already follow in April 2024, aligned with AVC timeline.

  - Exploration Experiments

    - Neural network-based video coding

    - Enhanced compression beyond VVC

- Liaison communication:

  - Any incoming liaison statements? Any outgoing? One from JPEG was expected.

- Joint meetings were expected with AG 5 (on Monday 17 July) and possibly with other groups.

- Principles of standards development were discussed.

## Scheduling of discussions

- The times of meeting sessions followed the needs of the face-to-face meeting, with highest priority given to the aim of achieving the goals of the meeting. Typical meeting hours were in the range of 0830-1900 CEST with coffee breaks and lunch breaks as appropriate, however some early morning or late-night sessions were anticipated to be necessary. Sessions were announced in the JVET calendar and the ITU posting system in advance as far as possible, although it was acknowledged that some activities (such as breakout sessions) might be held at short notice.

Particular scheduling notes are shown below, although not necessarily 100% accurate or complete. Times are recorded in the local timezone of the meeting venue, except as otherwise noted:

- Tue. 11 July, 1st day

  - Morning session:

    - 0930–1025 Opening remarks, review of practices, agenda, IPR policy reminder

    - 1025–1300 Reports of AHGs 1–11

  - Afternoon session2:

    - 1430–1515 Reports of AHGs 12–14

    - 1515–1815 EE2 summary report

    - 1515–1800 BoG (G. Sullivan) on VSEI/VVC v3 SEI messages

- Wed. 12 July, 2nd day

  - Morning sessions:

    - 0830–1300 Review of section 5.2.1 EE2 summary, section 5.2.3 EE2 related, and section 5.2.4 non-EE2

    - 0830–1300 BoG (G. Sullivan) on VSEI/VVC v3 SEI messages

  - Afternoon sessions:

    - 1430–1540 Review of section 5.1.1 EE1 summary,

    - 1540-1940 Review of section 5.2.4 non-EE2

    - 1430–1800 BoG (G. Sullivan) on VSEI/VVC v3 SEI messages

- Thu. 13 July, 3rd day

  - Morning session:

    - 0830–1300 BoG report, coordination/planning; review EE1 (section 5.1.2)

  - Afternoon session:

    - 1500–2000 Review non-EE2 (section 5.2.4)

    - 1500–1830 BoG on NNVC / EE1 preparation (E. Alshina, F. Galpin)

- Fri. 14 July, 4th day

  - Morning session:

    - 0830–1300 SEI messages beyond v3 (sections 6.1 and 6.2)

    - 0900–1300 BoG on NNVC / EE1 preparation (E. Alshina, F. Galpin)

  - Afternoon session:

    - 1430–1530 NNVC BoG report

    - 1530–1800 SEI messages beyond v3 (sections 6.1 and 6.2); optimization for machine analysis (AHG8 / section 4.10)

    - 1615–1815 BoG on film grain (W. Husak)

    - 1800–2000 Review non-EE2 (section 5.2.4)

- Sat. 15 July, 5th day

  - Morning session:

    - 0900–1300 SEI messages beyond v3 (section 6.2); optimization for machine analysis (AHG8 / section 4.10)

  - Afternoon session:

    - 1430–2100 Film grain synthesis incl. BoG report (section 4.12); remaining EE2 (sections 5.2.4 and 5.2.5); ECM tool assessment (section 4.9); encoding algorithm optimization (section 4.11); optimization for machine analysis (AHG8 / section 4.10);

- Sun. 16 July, 6th day

  - Morning session:

    - 0900–1300 ECM CTC and ECM modifications (section 5.2.5); ECM tool assessment (section 4.9)); encoding algorithm optimization (section 4.11); all other sections 4.x except section 4.5

  - Afternoon session:

    - 1430–1800 Remaining doc review: Encoding algorithm optimization (4.11); all other section 4.x except section 4.5; revisits, further planning;

- Mon. 17 July, 7th day

  - 0900–1230 MPEG information sharing session (out of JVET)

  - Afternoon session:

    - 1300–1600 Revisits, remaining doc review, further planning (EEs, outputs, AHGs, etc.)

  - Joint Meetings:

    - 1615–1700 with WG 4 and VCEG on VCM and MIV/MPI

    - 1715–1745 with WG 2, WG 7 and VCEG on generative faces

    - 1800–1930 with AG 5 and VCEG on visual testing and related topics

- Tue 18 July, 8h day

  - Morning session:

    - 0830–1400 output planning, remaining doc review, revisits, output review EE and DoCs, further planning

  - Afternoon session:

    - 1500–1800 Planning AHGs, timelines software and experiments, a.o.b.

    - From 1530: Informal viewing for layered verification test (room M03B)

- Wed. 19 July, 9th day

  - 0900–1030 MPEG information sharing session (out of JVET)

  - 1035–1300 Closing plenary:

    - Review liaison doc

    - Approval of output docs

    - Establishment of AHGs

    - Approval of meeting recommendations

    - Future planning, a.o.b.

- Fri. 21 July

  - 1400–1600 MPEG information sharing session (out of JVET)

  - 1645–1655 WG 5 presentation of meeting recommendations

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows (note that the noted document counts do not include crosschecks and summary reports, and may not be completely accurate; documents which are allocated to multiple sections are only counted in one of them):

- AHG reports (14) (section 3)

- Project development (section 4)

  - AHG1: Deployment and advertisement of standards (1)

  - AHG2: Text development and errata reporting (1)

  - AHG3: Test conditions (0)

  - AHG3: Software development (1)

  - AHG4: Subjective quality testing and verification testing (5)

  - AHG4: Test Material (1)

  - AHG4: Quality assessment methodology (1)

  - AHG5: Conformance test development (1)

  - AHG7: ECM tool assessment (2)

  - AHG8: Optimization of encoders and receiving systems for machine analysis of coded video content (5)

  - AHG10: Encoding algorithm optimization (4)

  - AHG13: Film grain synthesis (2)

  - Implementation studies (2)

  - Profile/tier/level specification (2)

  - General aspects of standards development and applications of standards (0)

- Low-level tool technology proposals (section 5) with subtopics (number counts excluding BoG and summary reports)

  - AHG11/AHG14 and EE1: Neural network-based video coding (19) (section 5.1)

  - AHG6/AHG12 and EE2: Enhanced compression beyond VVC capability (78) (section 5.2)

- AHG9: High-level syntax (HLS) proposals (section 6) with subtopics

  - SEI messages on neural-network post filter (28) (section 6.1)

  - SEI messages on topics other than NNPF (13) (section 6.2)

  - Non-SEI HLS aspects (0) (section 6.3)

- Joint meetings, plenary discussions, BoG reports (3), liaison (1), summary of actions (section 7)

- Project planning (section 8)

- Establishment of AHGs (section 9)

- Output documents (section 10)

- Future meeting plans and concluding remarks (section 11)

The document counts above do not include cross-checks and summary reports.

# AHG reports (14)

These reports were discussed during 1025–1300 and 1430–1515 on Tuesday 11 July 2023 (chaired by JRO).

JVET-AE0001 JVET AHG report: Project Management (AHG1) [J.-R. Ohm (chair), G. J. Sullivan (vice-chair)]

Goals and activity

The work of the JVET overall had proceeded well in the interim period with lower number of input documents (as compared to the previous meeting) submitted to the current meeting. Intense discussion had been carried out on the group email reflector, and most output documents from the preceding meeting had been produced as planned.

Output documents from the preceding meeting had been made initially available at the JVET web site (https://jvet-experts.org/) or the ITU-based JVET site (http://wftp3.itu.int/av-arch/jvet-site/2023_04_AD_Antalya/). It is noted that the previous document sites http://phenix.int-evry.fr/jvet/, http://phenix.int-evry.fr/jct/, and http://phenix.int-evry.fr/jct3v/ are still accessible, but were converted to read-only.

The list of documents produced included the following, particularly:

- The meeting report (JVET-AD1000) [Posted 2023-05-26, also submitted as WG 5 N 196]

- JVET-AD1003 Coding-independent code points for video signal type identification (Draft 2 of 3rd edition) [Posted 2023-07-10, also issued as WG 5 preliminary FDIS N 206]

- JVET-AD1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP [Posted 2023-07-04]

- JVET-AD1008 Additional colour type identifiers for AVC and HEVC (Draft 4) [Posted 2023-07-10, also issued as WG 5 preliminary WD N 200]

- JVET-AD1012 Overview of IT systems used in JVET [Posted 2023-06-12]

- JVET-AD2002 Algorithm description for Versatile Video Coding and Test Model 20 (VTM 20) [Posted 2023-06-29]

- JVET-AD2005 New level and systems-related supplemental enhancement information for VVC (Draft 5) [Posted 2023-06-02]

- JVET-AD2006 Additional SEI messages for VSEI (Draft 4) [Posted 2023-05-29]

- JVET-AD2007 Guidelines for NNVC software development [Posted 2023-05-29]

- JVET-AD2016 Common test conditions and evaluation procedures for neural network-based video coding technology [Posted 2023-05-12]

- JVET-AD2017 Common test conditions and evaluation procedures for enhanced compression tool testing [Posted 2023-05-10, last update 2023-06-15]

- JVET-AD2019 Description of algorithms and software in neural network-based video coding (NNVC) version 3 [Posted 2023-05-29]

- JVET-AD2021 Verification test plan for VVC multilayer coding [Posted 2023-06-12]

- JVET-AD2022 Draft plan for subjective quality testing of FGC SEI message [Posted 2023-06-24]

- JVET-AD2023 Exploration experiment on neural network-based video coding (EE1) [Posted 2023-04-28, last update 2023-05-15]

- JVET-AD2024 Exploration experiment on enhanced compression beyond VVC capability (EE2) [Posted 2023-04-28, last update 2023-05-29]

- JVET-AD2025 Algorithm description of Enhanced Compression Model 9 (ECM 9) [Posted 2023-07-03]

- JVET-AD2027 SEI processing order SEI message in VVC (Draft 4) [Posted 2023-05-19, also issued as WG 5 preliminary WD N 203]

- JVET-AD2028 Additional conformance bitstreams for VVC multilayer configurations [Posted 2023-05-26]

- JVET-AD2030 Optimization of encoders and receiving systems for machine analysis of coded video content (draft 2) [Posted 2023-06-07, also issued as WG 5 preliminary WD N 199]

- JVET-AD2031 Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content [Posted 2023-05-11]

Furthermore, the following documents were submitted to the ISO/IEC JTC1/SC29 parent body on behalf of its WG 5:

- Recommendations of the 11th WG 5 meeting (WG 5 N 195)

- Draft disposition of comments received on ISO/IEC 23002-7:2022 DAM 1 (WG 5 N 197)

- Preliminary text of ISO/IEC FDIS 23002-7:202x Versatile supplemental enhancement information messages for coded video bitstreams (3rd edition) (WG 5 N 198)

- Draft disposition of comments received on ISO/IEC 23090-3:2022 DAM 1 (WG 5 N 201)

- Preliminary text of ISO/IEC FDIS 23090-3:202x Versatile video coding (3rd edition) (WG 5 N 202)

- Draft disposition of comments received on ISO/IEC DIS 23091-2:202X (WG 5 N 205)

- Liaison statement to SC 29/WG 1 (JPEG) on JPEG AI and neural network-based video coding (WG 5 N 211)

- List of AHGs established at the 11th WG 5 meeting (WG 5 N 212)

In a few cases, the corresponding WG 5 N-numbered documents corresponding to the JVET documents had not yet been uploaded, and this was requested to be done as soon as possible.

It is noted that documents JVET-AC2026 Conformance testing for VVC operation range extensions (Draft 4), and the corresponding FDIS 23090-15 as WG 5 N 185 (VVC conformance 2nd ed.) had still not been delivered yet as outputs from the 29th JVET (10th WG 5) meeting of January 2023.

The fourteen ad hoc groups had made progress, and reports from those activities had been submitted. Furthermore, two exploration experiments (EE) on neural network-based video coding and on enhanced compression beyond VVC capability were conducted.

The arrangements for the 31st meeting had been announced in the JVET reflector, in the JVET logistics document (https://www.itu.int/wftp3/av-arch/jvet-site/2023_07_AE_Geneva/JVET-AE_Logistics.docx), and in the WG 5 calling notice (N 213) and agenda (N 214) for the 12th WG 5 meeting.

Software integration was finalized approximately according to the plan.

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system. That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful.

Roughly 160 input contributions (not counting the AHG reports, summary reports and crosschecks) had been registered for consideration at the current meeting.

It is further noted that, starting from the twentieth JVET meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and minor enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

To retain a consistent numbering scheme, the number range of output documents starting from 1001 was reserved for the previous JCT-VC topic items listed above, whereas the number range starting from 2001 was retained for VVC, VSEI and exploration activities.

A preliminary basis for the document subject allocation and meeting notes for the 31st meeting had been made publicly available on the ITU-hosted ftp site as http://wftp3.itu.int/av-arch/jvet-site/2023_07_AE_Geneva/JVET-AE_notes_d0.docx.

Recommendations

The AHG recommended its continuation.

JVET-AE0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice-chairs)]

Output documents produced

JVET-AD1003 - Coding-independent code points for video signal type identification (Draft 2 of 3rd edition)

This document contains the draft text for the specification of additional colour type identifiers for CICP (Rec. ITU-T H.273 | ISO/IEC 23091-2). Text modifications are provided for specification of code point identifiers for YCgCo-R colour representation with equal luma and chroma bit depths and for Draft SMPTE ST 2128. The new code points for YCgCo-R are referred to as YCgCo-Re and YCgCo-Ro, where the number of bits added to a source RGB bit depth is 2 (i.e., even) and 1 (odd), respectively. Draft SMPTE ST 2128 specifies a colour representation referred to as IPT-PQ-C2. The affected sections are provided relative to the corresponding basis text of ISO/IEC DIS 23091-2:202x (3rd Ed.). Change marks are included to show the changes since the January output (JVET-AC1008), which reflect a renumbering of the new MatrixCoefficients values and some minor refinements and corrections since the previous draft.

JVET-AD1004 - Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

This document contains a list of reported errata items for VVC, VSEI, HEVC, AVC, Video CICP, and the TR on usage of video signal type code points, for tracking purposes. Some of the items have been confirmed by the JVET and have been agreed to require fixing. This document also provides publication status backgrounds of these standards.

Incorporated items at the JVET-AD meeting:

- For AVC, incorporated the following items:

  - JVET-AD0078 item 4 and 5 on fixing the equation to derive the filter hint cross correlation matrix.

JVET-AD1008 - Additional colour type identifiers for AVC and HEVC (Draft 4)

This document contains the draft text for the specification of additional colour type identifiers for AVC (Rec. ITU-T H.264 | ISO/IEC 14496-10) and HEVC (Rec. ITU-T H.265 | ISO/IEC 23008-2). Text modifications are provided for specification of code point identifiers for YCgCo-R colour representation with equal luma and chroma bit depths and for Draft SMPTE ST 2128. The new code points for YCgCo-R are referred to as YCgCo-Re and YCgCo-Ro, where the number of bits added to a source RGB bit depth is 2 (i.e., even) and 1 (odd), respectively. Draft SMPTE ST 2128 specifies a colour representation referred to as IPT-PQ-C2. The affected sections are provided relative to corresponding basis texts (based on the 2021-08 edition of Rec. ITU-T H.264 (the most recent edition), the 2020 edition of ISO/IEC 23008-2 (not the 5th edition). Equation numbers and their cross-references have been updated without revision marking. Change marks are included to show the changes since the January output (JVET-AC1008), which reflect a renumbering of the new matrix coefficients values and some minor refinements and corrections since the previous draft.

JVET-AD2002 - Algorithm description for Versatile Video Coding and Test Model 20 (VTM 20)

The JVET established the VVC Test Model 21 (VTM21) software at its 30th meeting (21-28 April 2023, Antalya, TR). This document serves as a source of general tutorial information on the VVC design and also provides an algorithm description and encoding method description of VTM21 software. It is noted that, as no update of the algorithm description document was released at the 27th JVET meeting, the numbering of the VTM software version is one higher than that of this description document. In the main body of the text, numbering refers to the software version. The VVC has been developed by a joint collaborative team of ITU-T and ISO/IEC experts known as the Joint Video Experts Team (JVET), which is a partnership of ITU-T Study Group 16 Question 6 (known as VCEG) and ISO/IEC JTC 1/SC 29/WG 11 (known as MPEG). This new standard has been designed with two primary goals. The first of these is to specify a video coding technology with a compression capability that is substantially beyond that of the prior generations of such standards, and the second is for this technology to be highly versatile for effective use in a broadened range of applications. In addition to the applications that have commonly been addressed by prior video coding standards, some key application areas for the use of this standard include in particular ultra-high-definition video (e.g., with 3840×2160 or 7620×4320 picture resolution and bit depth of 10 or 12 bits as specified in Rec. ITU-R BT.2100), video with a high dynamic range and wide colour gamut (e.g., with the perceptual quantization or hybrid log-gamma transfer characteristics specified in Rec. ITU-R BT.2100), and video for immersive media applications such as 360° omnidirectional video projected using a common projection format such as the equirectangular or cubemap projection format.

Ed. Notes:

VVC Test Model 20 (VTM20) algorithm description and encoding method v1

- General improvements to reference picture resampling sections

- Incorporated JVET-AD0169: AHG12: RPR filters for scale factors below 1.5x

Incorporated JVET-AD0045: AHG10: Encoder MV selections and DMVR revisited.

JVET-AD2005 - New level and systems-related supplemental enhancement information for VVC (Draft 5)

This document contains a draft amendment for changes to the Versatile Video Coding (VVC) standard (ITU-T H.266 | ISO/IEC 23090-3). This amendment includes the following: 1) the support of an unlimited level for video profiles; 2) some technical corrections and editorial improvements to the 2nd edition text of VVC; and 3) the VVC-specific supports for some supplemental enhancement information (SEI) messages that may be included in VVC bitstreams but are not to be specified in the VVC specification. These SEI messages include two systems-related SEI messages, one for signalling of "green metadata" as to be specified in ISO/IEC 23001-11 and the other for signalling of an alternative video decoding interface for immersive media as to be specified in ISO/IEC 23090-13, and five other SEI messages, namely the shutter interval information SEI message, the neural network post-filter characteristics SEI message SEI message, the neural-network post-processing filter activation SEI message, the phase indication SEI message, and the post filter hint SEI message, that are to be specified in a new edition of the Versatile Supplemental Enhancement Information messages for coded video bitstreams (VSEI) standard (ITU-T H.274 | ISO/IEC 23002-7).

Draft 5 items from the BoG report JVET-AD0362 yet to be integrated: None. Other draft 5 items yet to be integrated:

- Spec bug ticket #1594 at editors’ discretion

Draft 5 incorporated items:

- Bug ticket #1593: Replaced Figures 5 to 7 for correction to illustrated slice partitioning.

- JVET-AD0362, action on item 2)b.ii: Allowed input pictures to an NNPF activated for any current picture to come from an earlier CLVS than the CLVS containing the current picture, and for the derivation of which pictures are used as in the input pictures, instead of using picture order count values, plain language phrases like "the previous picture in output order" and "the n-th previous picture in output order" are used.

- JVET-AD0362, action on item 2)b,i.2: Added a constraint that NNPF shall not be activated so that it creates an interpolated picture before the first picture in the bitstream in output order.

- JVET-AD0362, action on item 2)d: At the end of a bitstream, added repeated inference of NNPF for creating interpolated pictures: When an NNPF is active until the end of the bitstream, the NNPF is applied up to but excluding a set of input pictures that would cause creation of any interpolated picture after the last picture of the bitstream in output order.

- JVET-AD0362, action on item 2)e.i: Limited the special treatment of temporal interleaving frame packing arrangement to picture rate upsampling only rather than for any multi-picture-input NNPF.

- JVET-AD0362, action on item 2)f with the change of "the CLVS" to "the bitstream": Fixed an asserted bug in the derivation of input pictures for the case when there are multiple input pictures for an NNPF activated for currCodedPic and currCodedPic is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5, taking into account that there can be multiple pictures preceding an input picture in output order.

- JVET-AD0362, action on item 2)g: Specified that any multi-picture-input NNPF can only be used when all input pictures have the same dimensions after applying super resolution post-filter applicable to the input pictures, if any, using text provided in JVET-AD0057 item 6.

- JVET-AD0362, action on item 6)g: In the VVC text, disallowed the NNPFA SEI message from being associated with a non-output picture and disallowed any input pictures being a non-output picture.

- JVET-AD0362, action on item 14)a: Specified an array of filtering strength control variables for multiple input pictures (as StrengthControlVal[ idx ] values for the input pictures with index idx in the range of 0 to numInputPics − 1, inclusive).

- JVET-AD0362, action on item 14)d: Changed the derivation of StrengthControlVal to be equal to ( SliceQpY + QpBdOffset ) ÷ ( 63 + QpBdOffset), instead of SliceQpY ÷ 63, where SliceQpY is that of the first slice of currCodedPic.

- JVET-AD0362, action on item 13)a: Allow inclusion of NNPFC and NNPFA SEI messages in suffix SEI NAL units.

- JVET-AD0362, action on item 13)b: Disallow the post-filter hint SEI message to be present in both a prefix SEI NAL unit and a suffix SEI NAL unit in the same picture unit.

- Editorial change from JVET-AD0077

- VclAssociatedSeiList consists of the payloadType values of the SEI messages that, when non-scalable-nested, infer constraints on the NAL unit header of the SEI NAL unit on the basis of the NAL unit header of the associated VCL NAL unit. It is proposed to add the payloadType value of the green metadata SEI message in VclAssociatedSeiList. (JVET-AD0057, item 3)

- PicUnitRepConSeiList consists of the payloadType values of the SEI messages that are subject to the restriction on 4 repetitions per picture unit (PU). It is proposed to add the payloadType values of post-filter hint, green metadata, scalability dimension information, NNPFC, NNPFA, and phase indication SEI messages in PicUnitRepConSeiList. (JVET-AD0057, item 4)

Alignments of SEI messages between VVC and HEVC, regarding inclusion in prefix and/or suffix SEI NAL units (including for the user data SEI messages), and inclusion into the lists VclAssociatedSeiList and PicUnitRepConSeiList. (In-meeting suggestion).

JVET-AD2006 - Additional SEI messages for VSEI (Draft 4)

This document contains the draft text for changes to the versatile supplemental enhancement information messages for coded video bitstreams (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7), to specify additional SEI messages, including the shutter interval information SEI message, neural-network post-filter characteristics SEI message, neural-network post-filter activation SEI message, phase indication SEI message, and post-filter hint SEI message.

Changes that had been integrated:

Items noted with adoptions in the JVET meeting notes but not in the BoG report JVET-AD0362-v4:

- None noted

Items noted with adoptions in the BoG report JVET-AD0362-v4:

- None noted.

JVET-AD2027 - SEI processing order SEI message in VVC (draft 4)

This document contains the draft text for changes to the Versatile Video Coding (VVC) standard (Rec. ITU-T H.266 | ISO/IEC 23090-3), to specify the SEI processing order SEI message.

Changes yet to be integrated:

Items noted with adoptions in the BoG report JVET-AD0362-v4: None noted.

Changes that had been integrated:

Items noted with adoptions in the BoG report JVET-AD0362-v4: 1d (using text in JVET-AD0386)

Related input contributions

The following input contributions were noted as relevant to the work of this ad hoc group:

- JVET-AE0054 AHG2/AHG9: Editorial improvements of annotated regions SEI message

- JVET-AE0101 [AHG2][AHG9] Neural network post filter and phase indication SEI messages for AVC and HEVC

- JVET-AE0155 AHG2/AHG9: Editor commentary on the post-filter hint SEI message semantics

Remaining bug tickets

- #1606 VVC v2 wrongly uses zero out block size

- #1602 Need clarification for 8.4.3 Derivation process for chroma intra prediction mode

Fixes are available, will be included in next edition.

Recommendations

The AHG recommended to:

- Approve JVET-AD1003, JVET-AD1004, JVET-AD1008, JVET-AD2002, JVET-AD2005, JVET-AD2006, and JVET-AD2027 documents as JVET outputs,

- Compare the VVC documents with the VVC software and resolve any discrepancies that may exist, in collaboration with the software AHG,

- Encourage the use of the issue tracker to report issues with the text of both the VVC specification text and the algorithm and encoder description,

- Continue to improve the editorial consistency of VVC text specification and Test Model documents,

- Ensure that, when considering changes to VVC, properly drafted text for addition to the VVC Test Model and/or the VVC specification text is made available in a timely manner,

- Review bug tickets, and other AHG2 related inputs and act on them if found to be necessary.

JVET-AE0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice-chairs)]

The software model versions prior to the start of the meeting were:

- VTM 21.0 (Jul. 2023)

- HM-18.0 (Apr. 2023)

- HM-16.21+SCM-8.8 (Mar. 2020)

- SHM 12.4 (Jan. 2018)

- HTM 16.3 (Jul. 2018)

- JM 19.1 (Apr. 2023)

- JSVM 9.19.15

- JMVC 8.5

- 3DV ATM 15.0 (no version history)

- HDRTools 0.24 (March 2023)

Software for MFC and MFCD is only available as published by ITU-T and ISO/IEC. It is planned to create repositories with the latest versions available in ITU-T H.264.2 (02/2016). All development history is lost.

Software development

Development was continued on the GitLab server, which allows participants to register accounts and use a distributed development workflow based on git.

The server is located at:

https://vcgit.hhi.fraunhofer.de

The registration and development workflow are documented at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/VVC-Software-Development-Workflow

Although the development process is described in the context of the VTM software, it can be applied to all other software projects hosted on the GitLab server as well.

VTM related activities

The VTM software can be found at

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

The software development continued on the GitLab server. VTM versions 20.1 and 20.2 were tagged on May 8, and VTM version 21.0 was tagged on Jul. 9. VTM 21.1 is expected during the 31st JVET meeting.

VTM 20.1 was tagged on May 8, 2023. Changes include:

- Undefine macro from HDRTools

- Fix build when GDR_ENABLED is 0

- Some fixes after !2541 (Use strong-typed enum for chroma format)

- Fix parsing of SDI SEI message

- Fix parsing of MAI SEI message

- Fix parsing of multiple SEI payloads in one NAL unit

- Add command line option to print per-frame encoding time in float format

- Fix #1595: Set ptl_multi_layer_enabled_flag when more than one layer is encoded

- Rollback SEI bits computation as it is erroneous

- Fix compilation when EXTENSION_360_VIDEO is enabled

- Shorten Neural Network Post Filter Characteristics SEI parameters

- Fix type of literal in assert expression

- Fixes for multilayer coding

- Port changes from JVET-Z0072 to performance low delay configuration file

- Revert !2573 and add MultiLayerEnabledFlag=1 to two_layers.cfg

VTM 20.2 was tagged on May 8, 2023. Changes include:

- Remove macros from previous cycle

VTM 21.0 was tagged Jul. 9, 2023. Changes include:

- Fix #1599: include frame rate info in bitstream when provided in y4m file

- JVET-AD0169: Downsampling filters in range 1.1 to 1.35 based on Kaiser(7) windowed sinc

- JVET-AD0386: Processing Order SEI message update

- Fix #1598: signal chroma location type when input is y4m

- Fix #1597 nnpfc_constant_patch_size_flag and nnpfc_overlap and nnpfc_overlap parsing.

- JVET-AD0045: Encoder MV selections and DMVR revisited

- Avoid seg fault Slice::constructRefPicList

- Code cleanup: fix variable names, use proper data types, etc.

- Replace EncAppCfg::OptionalValue with std::optional

- Remove JVET_O0549_ENCODER_ONLY_FILTER_POL macro

- Fix output frame rate when using --MaxTemporalLayer in decoder

- Rename df::program_options_lite as ProgramOptionsLite

- Rename and initialize m_RCCpbSaturationEnabled

- JVET-AD0056 item 5: don't signal nnpfc_input_pic_output_flag[i] flags when nnpfc_num_input_pics_minus1==0

- Fix variable names in VideoIOYuv

- Fix parsing of frame rate parameter

- JVET-AD0067 item2: Swap signalling order for nnpfc_aux_inp_idc and nnpfc_inp_order_idc

- JVET-AD0129: Fix padding

- JVET-AD0054: nnpfc_absent_input_pic_zero_flag

- JVET-AD0141: NNPFA is associated with picture that is an output picture

- JVET-AD0383: Signalling scaling ratio for output picture size instead of absolute width and height

- JVET-AD0067 Item 7: Include syntax element nnpfc_full_range_flag

- Fix #1596 on encoding crash when DeltaQpRD is used

- Avoid CU copy in EncModeCtrlMTnoRQT::useModeResult

- JVET-AD0056 Item 4: picture width and height constraints

- Fix default values of SEI NNFPC parameters to avoid crash and fix variable names

- Check config parameters after they have been set

- JVET-AD0056 Item 6: move nnpfc_inp_order_idc and nnpfc_out_order_idc

- JVET-AD0056 Item 1: move nnpfc_base_flag next to nnpfc_id

- JVET-AD0056 Item 11: nnpfc SEI metadata extension

- JVET-AD0056 Item 9: add condition for nnpfc padding signalling

- Fix compile issues with GCC 13 on Windows

- Remove singleComp function and clean up singleChan function

- JVET-AD0056 Item 7: nnpfc_inp_order_idc and nnpfc_out_order_idc constraints

- JVET-AD0056 Item 8: Move nnpfc_separate_colour_description_present_flag

- Add CHECK statement to avoid possible buffer overflow

- fix MSAN issues

- JVET-AD0091 items 4-6: add constraints for nnfpc padding

- Fix RPL construction

- JVET-AD0056 Item 12: remove redundant constraints on nnpfa_target_id

- JVET-AD0233 Items 1,2,6: Add NNFPC constraints and chroma sample location

- JVET-AD0056 Item 13: implementation of nnpfa_target_base_flag

- Fix #1603 Overflow in importMergeInfo()

- JVET-AD0057 item 6: multi-picture-input constraint

- Fix #1604: NNPFC constraints check

- JVET-AD0057 items 3 and 4: Update VclAssociatedSeiList and PicUnitRepConSeiList

- JVET-AD0057 item 2: disallow the post-filter hint SEI message to be present in both a prefix SEI NALU and a suffix SEI NALU in the same picture unit

- JVET-AD0388: add nnfpa_num_output_entries and nnfpa_output_flag syntax elements

VTM 21.1 is expected to be tagged during the 31st JVET meeting. Changes are expected to include the remaining implementations of the 30th meeting.

CTC Performance

The following tables shows VTM 20.0 performance over HM 17.0 (not updated).

|  |  |  | All Intra Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-17.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -29.50% | -32.81% | -33.83% | 1273% | 166% |
| Class A2 | -29.73% | -24.39% | -21.58% | 2142% | 174% |
| Class B | -22.32% | -27.21% | -30.98% | 2397% | 175% |
| Class C | -22.89% | -19.53% | -23.19% | 3348% | 179% |
| Class E | -26.04% | -25.90% | -24.15% | 1869% | 158% |
| Overall | -25.50% | -25.75% | -27.02% | 2187% | 171% |
| Class D | -18.80% | -13.85% | -13.68% | 4270% | 189% |
| Class F | -39.49% | -40.23% | -42.90% | 4220% | 171% |
|  |  |  | Random access Main 10 |  |  |
|  |  |  | Over HM-17.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -40.60% | -40.71% | -47.16% | 586% | 158% |
| Class A2 | -44.02% | -41.73% | -40.78% | 679% | 170% |
| Class B | -37.41% | -50.12% | -48.51% | 672% | 161% |
| Class C | -33.86% | -36.28% | -38.21% | 923% | 164% |
| Class E |  |  |  |  |  |
| Overall | -38.42% | -42.87% | -43.95% | 713% | 163% |
| Class D | -31.72% | -32.60% | -31.95% | 970% | 161% |
| Class F | -46.07% | -49.76% | -50.65% | 490% | 145% |
|  |  |  | Low delay B Main 10 |  |  |
|  |  |  | Over HM-17.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -30.40% | -37.43% | -35.12% | 629% | 152% |
| Class C | -28.18% | -21.23% | -21.06% | 817% | 167% |
| Class E | -31.16% | -35.72% | -28.79% | 309% | 140% |
| Overall | -29.85% | -31.60% | -28.85% | 575% | 154% |
| Class D | -26.75% | -16.50% | -14.88% | 881% | 179% |
| Class F | -42.17% | -44.02% | -44.14% | 435% | 137% |
|  |  |  | Low delay P Main 10 |  |  |
|  |  |  | Over HM-17.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -34.52% | -38.34% | -36.12% | 526% | 160% |
| Class C | -28.07% | -17.73% | -18.09% | 670% | 176% |
| Class E | -33.06% | -37.89% | -31.10% | 280% | 146% |
| Overall | -32.01% | -31.36% | -28.85% | 487% | 161% |
| Class D | -26.68% | -12.73% | -11.47% | 762% | 175% |
| Class F | -40.13% | -41.24% | -41.72% | 437% | 145% |

According to common test conditions in random access configuration HM is using a GOP size of 16 pictures compared to VTM using a GOP of 32 pictures. Random access points are inserted approximately every second aligned with a GOP boundary of GOP 32 in both VTM and HM. The VTM uses two more reference pictures in random access than HM (due to more memory being available in typical level settings).

There was no change in coding performance or run time between VTM 21.0 and VTM 20.1 using SDR CTC.

HDR and high bit depth CTC were stated. Partial results show no performance change as well.

The following tables show VTM 20.0 performance over HM 17.0 using HDR CTC (not updated):

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM17.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -39.54% | -37.56% | -36.67% | -54.98% | -48.45% | -33.68% | -49.48% | -40.84% | 286% | 97% |
| Class H2 |  |  |  |  |  | -31.97% | -57.69% | -63.36% | 271% | 97% |
| Overall | -39.54% | -37.56% | -36.67% | -54.98% | -48.45% | -33.06% | -52.47% | -49.03% | 280% | 97% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over HM17.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -41.45% | -27.31% | -26.79% | -57.74% | -52.79% | -24.00% | -52.93% | -45.13% | 1538% | 113% |
| Class H2 |  |  |  |  |  | -21.76% | -47.21% | -50.57% | 1320% | 109% |
| Overall | -41.45% | -27.31% | -26.79% | -57.74% | -52.79% | -23.18% | -50.85% | -47.11% | 1455% | 112% |

Issues in VTM affecting conformance

The following issues in VTM master branch may affect conformance:

- Missing HLS features (see sections below)

Status of implementation of proposals of previous JVET meetings

The following list contains all adoptions of the Q and R meetings that were not marked as merged (or submitted) or specification only change in the software coordinator tracking sheet:

- JVET-Q0112

- JVET-Q0154: Disallow mixing of GDR and IRAP (Disallow mixing of GDR with any non-GDR).

- JVET-Q0164

- JVET-Q0402

- JVET-R0178: Require that when no_aps_constraint_flag is equal to 1, sps_lmcs_enabled_flag and sps_scaling_list_enabled_flag shall be equal to 0

- JVET-R0221

- JVET-R0046: Change the description of the bitstream extraction process per the value of max_tid_il_ref_pics_plus1[ ][ ] (aspect 1.2 per JVET-R0046-v4).

- JVET-R0065: Specify that GDR AUs shall be complete – i.e., all of the layers in the CVS shall have a picture in the AU (as with IRAP AUs).

- JVET-R0191: Update the range value for num_ols_hrd_params_minus1.

- JVET-R0222 aspect 1: Infer vps_max_sublayers_minus1 to be equal to 6 when sps_video_parameter_set_id is equal to 0 (i.e. VPS is not present). The exact editorial expression is at the discretion of the editor.

- JVET-S0196 (JVET-S0144 item 17)

- JVET-S0227 (JVET-S0144 item 22)

- JVET-S0077 (JVET-S0139 item 5)

- JVET-S0174 aspect 2 (JVET-S0139 item 18.b)

- JVET-S0156 aspect 3 (JVET-S0139 item 21)

- JVET-S0139 item 26 (no source listed, text only?)

- JVET-S0188 aspect 1 (JVET-S0139 item 28)

- JVET-S0139 item 40 (item does not exist)

- JVET-S0042 (JVET-S0142 item 1.b)

- JVET-S0174 aspect 1 (JVET S0143 item 19)

- JVET-S0096 aspect 3 (JVET-S0140 item 10)

- JVET-S0096 aspect 4 (JVET-S0140 item 13)

- JVET-S0159 aspect 3 (JVET-S0140 item 16)

- JVET-S0171 (JVET-S0256)

- JVET-S0118 (JVET-S0141 item 7)

- JVET-S0102 (JVET-S0141 item 9.a)

- JVET-S0157 item 2 (JVET-S0141 item 13)

- JVET-S0157 item 4 (JVET-S0141 item 14)

- JVET-S0175 aspect 3 (JVET-S0141 item 16)

- JVET-S0175 aspect 1, 2 (JVET-S0141 item 17)

- JVET-S0175 aspects 4 and 5 (JVET-S0141 item 18)

- JVET-S0175 aspect 6 (JVET-S0141 item 19)

- JVET-S0198/ JVET-S0223 (JVET-S0141 item 24)

- JVET-S0173 aspect 2 (JVET-S0141 item 40.b)

- JVET-S0173 item 1 (JVET-S0141 item 51)

- JVET-S0173 item 3 (JVET-S0141 item 52)

- JVET-S0173 item 5 (JVET-S0141 item 53)

- JVET-S0173 item 6 (JVET-S0141 item 54)

- JVET-S0173 item 4 (JVET-S0141 item 56)

- JVET-S0176 item 4 (JVET-S0141 item 60)

- JVET-S0154 aspect 5 (JVET-S0141 item 68)

- JVET-S0154 aspect 6 (JVET-S0141 item 69)

- JVET-S0154 aspect 8 (JVET-S0141 item 71)

- JVET-S0095 aspect 5 (JVET-S0145 item 5)

- JVET-S0095 aspect 6 (JVET-S0145 item 6)

- JVET-S0100 aspect 1, depends on JVET-R0193 (JVET-S0147 item 2)

- FINB ballot comments

- Make high tier support up to 960.

HM related activities

HM 18.0 was tagged on Apr. 20, 2023. Changes include:

- Update to Ubuntu 22.04 build server

- Update the HDR configuration files to align with the HDR CTC document (JVET-AC2011)

- Update to FilmGrainCharacteristics SEI support, including applying film grain noise onto the decoded output.

The performance of HM 18.0 is identical to HM 17.0 under SDR and HDR CTCs. Initial results also indicate that the performance is identical to HM 17.0 under RExt non-4:2:0 and RExt high bit depth CTCs.

The following MRs are pending [with status indicated]:

- Port the Y4M support [one issue remains]

- Mark the current picture as short-term ref (for SCM) [need SCC expert reviewer]

The HM SCC (SCM) branch (HM-16.21+SCM-8.8) has not been updated for the recent HM versions. Updating SCM to, for example, HM-18.0+SCM-8.8 should be considered. It may though be helpful to move SCC related functionality into separate source files. Volunteer work towards merging the branches would be appreciated.

As reported in the previous reports, further information on lambda optimisation in HM would be appreciated, including comparison of allocation of bits within the GOP structures between HM and VTM.

Two newer tickets were filed for HM in relation to 360Lib and the NoOutPrior_A_Qualcomm_1 conformance stream.

Otherwise the HEVC bug tracker lists:

- 43 tickets for “HM”, most of which are more than 5 years,

- 1 ticket for “HM RExt”,

- 9 tickets for “HM SCC”, most of which are at least 3 years old,

Help to address these tickets would be appreciated.

One merge request (MR#40) is available related to HM SCC for ticket #1511 on SCC reference picture marking. Help was requested to confirm that the proposed change matches the SCC text.

360Lib related activities

The latest 360Lib software can be found at https://vcgit.hhi.fraunhofer.de/jvet/360lib

There had not been any further developments to 360Lib during this meeting cycle.

SCM related activities

There had not been any further developments to SCC’s SCM during this meeting cycle.

SHM related activities

There had not been any further developments to SHVC’s SHM during this meeting cycle.

HTM related activities

There had not been any releases of HTM of MV-HEVC and 3D-HEVC.

The next release will include the following changes:

- JVET-Z0209: Early termination during calculating RDcost of depth

HDRTools related activities

There had not been any further developments to HDRTools during this meeting cycle.

JM, JSVM, JMVM related activities

There had not been any further developments to JM, JSVM or JMVM during this meeting cycle.

For the previous release of the JM, only basic testing was performed to confirm that the code still works, but performance testing was pending. Performance changes are unlikely but may occur due to bug fixes. A possible encoder/decoder mismatch had been reported privately, but needed more investigation.

Bug tracking

The bug tracker for VTM and specification text is located at:

https://jvet.hhi.fraunhofer.de/trac/vvc

The bug tracker uses the same accounts as the HM software bug tracker. Users may need to log in again due to the different sub-domain. For spam fighting reasons account registration is only possible at the HM software bug tracker at

https://hevc.hhi.fraunhofer.de/trac/hevc

Bug tracking for HDRTools is located at:

https://gitlab.com/standards/HDRTools/-/issues

Contributors are requested to please file all issues related to the VVC reference software and HDRTools into the appropriate bug tracker. Try to provide all the details, which are necessary to reproduce the issue. Patches for solving issues and improving the software are always appreciated.

Software repositories

Git repositories that were previously assigned to the JCT-VC group on the GitLab server were re-assigned to the JVET group. The old URLs are still working and will forward the user to the new location, with the display of a warning suggesting to update bookmarks to the new location. The SVN repository for 360Lib was converted to git and development was moved to the GitLab server. Historical branches can still be accessed in the SVN repository.

CTC alignment and merging

There are currently 8 JVET CTC documents:

JVET-Y2010		VTM/HM 4:2:0 test conditions

JVET-Z2011		VTM/HM HDR test conditions

JVET-AA2018		VTM/HM high bit depth test conditions (without spreadsheet)

JVET-T2013		VTM non-4:2:0 test conditions

JVET-AA1100		HM non-4:2:0 test conditions

JVET-U2012		VTM 360 video test conditions

JVET-AC1009		SHVC test conditions

JVET-AC1015		SCM test conditions

Older CTC documents are:

JCT3V-G1100		3DV test conditions

JVET-AC1013 was a planned output document for updating JCT3V-G1100, which was not produced. JVET-AD0201 was submitted with a description of the problems that were found during editing.

Further merging of HM RExt CTC into the appropriate VVC CTC was investigated (non 4:2:0 chroma formats), but proper comparable HM configuration files were not yet available by the beginning of this meeting.

The HM HDR test conditions require additional post-processing stages, due to WPSNR, deltaE100 and PSNRL100 values not being generated directly by the HM encoder. Merging of respective functions from VTM is recommended to streamline the process.

Guidelines for reference software development

No further work was conducted on Guidelines for VVC and HEVC reference software development and documents JVET-AC2003 and JVET-AC1001 remain current.

Recommendations

The AHG recommended to:

- Continue to develop the reference software.

- Improve documentation, especially the software manual.

- Encourage people to test VTM and other reference software more extensively outside of common test conditions.

- Encourage people to report all (potential) bugs that they are finding.

- Encourage people to submit bit-streams/test cases that trigger bugs in VTM and other reference software.

- Encourage people to submit non-normative changes that either reduce encoder run time without significantly sacrificing compression performance or improve compression performance without significantly increasing encoder run time.

- Design and add configuration files to the VTM software for testing of HLS features.

- Review VTM-related contributions and determine whether features should be added (or removed) from the software.

- Continue to investigate the merging of branches.

- Continue to investigate merging of CTC documents.

- Verify correctness of CTC documents and issue updates as appropriate

- Keep common test conditions aligned for the different standards.

- Consider documents (including late documents) related to AHG3 activities.

It was noted that JVET-AE0179 renews licensing conditions for 3DV content, a JVET-AE1013 document could be issued.

JVET-AE0004 JVET AHG report: Test material and visual assessment (AHG4) [V. Baroncini, T. Suzuki, M. Wien (co-chairs), S. Iwamura, S. Liu, S. Puri, A. Segall, P. Topiwala, S. Wenger, J. Xu, Y. Ye (vice-chairs)]

Verification test preparation for VVC multilayer coding

A teleconference was organized by AG 5 and JVET AhG4 on 2023-05-26 07:00h UTC. The topic was the Verification Test preparation for VVC Multilayer Coding. The report of the AhG meeting is available in JVET-AE0041.

Test sequences

The test sequences used for CfP/CTC are available on ftp://jvet@ftp.ient.rwth-aachen.de in directory “/ctc” (accredited members of JVET may contact the JVET chair for login information).

In the report period, test material related to the CTC for SHVC and HEVC SCC has been made available in that directory.

Due to copyright restrictions, the JVET database of test sequences is only available to accredited members of JVET (i.e., members of ISO/IEC MPEG and ITU-T VCEG).

Related contributions

| JVET number | Title | Source |
| --- | --- | --- |
| JVET-AE0004 | JVET AHG report: Test material and visual assessment (AHG4) | V. Baroncini, T. Suzuki, M. Wien (co-chairs), S. Iwamura, S. Liu, S. Puri, A. Segall, P. Topiwala, S. Wenger, J. Xu, Y. Ye (vice-chairs) |
| JVET-AE0041 | AHG4: Report on AhG meeting on verification testing for VVC multilayer coding | M. Wien (AhG Co-Chair) |
| JVET-AE0092 | [AHG4] Occupancy-only PSNR calculations for V3C V-PCC coding evaluation | S. Schwarz, M. M. Hannuksela |
| JVET-AE0179 | AHG4: Renewed license statement for Ghost Town Fly and Undo Dancer 3D video test sequences | M. M. Hannuksela, N. Salonen (Nokia) |
| JVET-AE0219 | AHG4: Results of visual checking of SVVC VT streams with and without DMVR fix | M. Wien (RWTH Aachen University) |
| JVET-AE0227 | AHG4: experiments in preparation of scalable quality ladder visual testing | P. de Lagrange, F. Urban (InterDigital) |
| JVET-AE0250 | Film Grain Synthesis: Material Selection for Ground Truth Content Creation | D. Podborski, D. Ugur, B. Williams, L. Levinson, R. Molholm, A.M. Tourapis |
| JVET-AE0251 | Evaluating Denoising methods in the context of Film Grain Analysis/Synthesis | D. Podborski, D. Ugur, A. M. Tourapis |

Recommendations

The AHG recommended:

- To review and consider JVET-AE0219 in the finalization of the SVVC verification test plan.

- To review JVET-AD0179 and consider further steps towards resolving licensing issues with 3DV test content.

- To review and consider JVET-AE0250 and JVET-AE0251 in the development of the draft plan for subjective quality testing of FGC SEI message

- To collect volunteers to conduct further verification tests and subjective quality tests, including volunteers to encode.

- To continue to discuss and to update the non-finalized categories of the verification test plan and subjective quality test plan for FGS, including those which have not been addressed yet.

- To review the set of available test sequences for the verification tests as well as subjective quality tests and potentially collect more test sequences with a variety of content.

- To continue to collect new test sequences available for JVET with licensing statement.

A test for multi-layer coding was expected to occur after the meeting.

JVET-AE0005 JVET AHG report: Conformance testing (AHG5) [D. Rusanovskyy, I. Moccagatta (co-chairs), F. Bossen, K. Kawamura, T. Ikai, S. Iwamura, H.-J. Jhu, K. Sühring, Y. Yu (vice-chairs)]

Timeline

The progress on the conformance testing specification was proceeding per the timeline below:

- VVCv1 conformance:

  - ISO/IEC FDIS 23090-15 issued from 2021-10 meeting, FDIS registered for formal approval 2022-07-11, FDIS ballot closed 2022-11-04, standard published 2022-11-24

  - H.266.1 V1 Consent 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, standard published 2022-07-12.

- VVCv2 conformance:

  - ISO/IEC 23090-15/Amd.1 CDAM: 2021-10

  - ISO/IEC 23090-15/Amd.1 DAM: 2022-01

  - DAM ballot closed 2022-11-15, FDIS issued at 2023-01 meeting

  - H.266.1 V2 Consent planned for 2023-07

Status on bitstream submission

The status at the time of preparation of this report was as follows:

- Conformance bitstreams for VVC:

  - 104 bitstream categories have been identified

  - At least one bitstream has been submitted in each identified category

  - 283 total bitstreams have been provided, checked, and made available

  - As reported in JVET-AB0005, HRD_A_Fujitsu (HRD_A_Fujitsu_4) has been regenerated to avoid CPB underflow

  - The 10 bitstreams reported in Ticket #1581 have been re-generated

  - The problems reported in Ticket #1601 for 2 FDIS_r1 packages have been fixed

- Conformance bitstreams for VVC operation range extensions:

  - 57 bitstream categories had been identified

  - 1 bitstream of 1 identified category had been re-generated

  - 128 bitstreams of 57 identified categories had been cross-checked and uploaded.

  - No changes between 30th and 31st meeting.

VVC activities:

The remaining 3 of the 10 bitstreams reported in Ticket #1581 had been re-generated after the VTM problem reported in Ticket #1595 has been fixed

- OLS_A_Tencent_5

- OLS_B_Tencent_5

- OLS_C_Tencent_5

The re-generated bitstreams, HRD_A_Fujitsu_4, and the remaining conformance streams are decoded correctly using VTM-20.2 w/o range extension support. The streams had been uploaded to https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC/under_test/VTM-20.2/

Ticket #1601 had been filed to report that the .yuv.md5 and .opl files of 2 FDIS_r1 packages (STILL_B_1 and STILL444_B_1) are different from the same files in the corresponding FDIS packages (STILL_B_ ERICSSON_1 and STILL444_B_ ERICSSON_1). Test bitstreams and collateral files in the FDIS and FDIS_r1 directories are supposed to be the same except for removal of company names and email addresses. The differences reported in Ticket #1601 are unintentional and have been fixed. The fixed packages have been uploaded to https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC/under_test/VTM-20.2_r1/

VVC operation range extensions activities:

All bitstreams are decoded correctly using VTM-20.2 and are available at https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCv2/under_test/VTM-20.2/

VVC Multilayer activities:

The 6 additional conformance bitstreams for VVC multilayer configurations in JVET-AD2028 are decoded correctly using VTM-20.2 and are available in https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCMultilayer/under_test/VTM-20.2/

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de).

The AHG5 chairs and JVET chairs can be reached at jvet-conformance@lists.rwth-aachen.de. Participants should not subscribe to this list but may send emails to it.

Relevant contributions

JVET-AE0111 - Additional conformance tests of spatial scalability for multilayer coding [C. Salmon-Legagneur, P. de Lagrange, F. Urban].

JVET-AE0181 - Scaling window support for VTM [Shunsuke Iwamura, Shimpei Nemoto, Atsuro Ichigaya].

Ftp site information

The procedure to exchange the bitstream (ftp cite, bitstream files, etc.) is specified in Sec 2 “Procedure” of JVET-R2008. The ftp and http sites for downloading bitstreams are

- VVC:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVC

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC/

- VVC operation range extensions:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVCv2

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCv2

The ftp site for uploading bitstream files is as follows.

ftp://ftp3.itu.int/jvet-site/dropbox/

(user id: avguest, passwd: Avguest201007)

If using FileZilla, the following configuration is suggested:

In the Filezilla Edit  Settings  Connection menu, it may also be necessary to set the minimum TLS level to 1.0.

Recommendations

The AHG recommended the following:

- Proceed with the generation, cross-checking, and documentation of the additional conformance bitstreams for VVC, in particular conformance bitstreams for VVC multilayer configurations (JVET-AD2028).

- Maintain and update the conformance bitstream database and contribute to report problems in JVET document 1004.

- Start the generation, cross-checking, and documentation of the conformance streams for the HEVC multiview profiles supporting extended bit depth (JVET-AA1011).

All known issues with V2 bitstreams had been resolved. Text integration was started. This was intended to be submitted for ITU consent, with the ITU version of the integrated text to be provided by Wednesday 19 July. Gary Sullivan was requested to provide the basis text, and provide help in checking differences relative to the ISO version.

JVET-AE0006 JVET AHG report: ECM software development (AHG6) [V. Seregin (chair), J. Chen, F. Le Léannec, K. Zhang (vice-chairs)]

Software development

The ECM software repository is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM.

ECM software is based on VTM-10.0 with enabled MCTF including the update from JVET-V0056, and GOP32, which is very close to VTM-11.0.

VTM-11.0ecm anchor https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/VTM11_ANC is used for ECM performance evaluation.

The following adopted aspects were integrated into ECM-9.0:

- JVET-AD0202 (EE2-1.2): CCCM with multiple downsampling filters (MR 399)

- JVET-AD0208 (EE2 Test-1.8c): IBC adaptation for coding of natural content (MR 405)

- JVET-AD0195: High-Precision MV Refinement for BDOF (MR 406)

- JVET-AD0222 (EE2-4.4b): ALF: additional fixed filter, residual-based classifier, increase the number of taps that are applied to the fixed filter results (MR 402)

- JVET-AD0206: CABAC init at GDR picture (MR 407)

- JVET-AD0123 (EE2 Test 5.1): reference frame padding for GDR (MR 409)

- JVET-AD0193 (EE2 Test 2.11e): Adaptive OBMC control (MR 408)

- JVET-AD0213 (EE2-Test2.7b): bi-predictive LIC and the combination of LIC and OBMC (MR 404)

- JVET-AD0082: add configuration option for TMRL (MR 401)

- JVET-AD0169: Downsampling filters in range 1.1 to 1.35 based on Kaiser(7) windowed sinc (MR 412)

- JVET-AD0184: Removal of Division Operations (MR 413)

- JVET-AD0085: Combinations of MPM related tests (MR 400)

- JVET-AD0086 (EE2-1.20j combination test): enhanced intra TMP (MR 415)

- JVET-AD0188 (EE2 Test-1.6c): Non-local cross-component prediction and cross-component merge mode (MR 418)

- JVET-AD0105 (Aspect1): Cap the maximum NumSignPred to 4 for QP<=22 (AI and RA) (MR 403)

- JVET-AD0182(EE2-2.1l): Affine Parameter refinement and mode extension (MR 419)

- JVET-AD0239/AD0124: Fixes for IntraTMP template availability (MR 421)

- JVET-AD0120 (EE2 Test 1.3e): Local-Boosting Cross-Component Prediction (MR 411)

- JVET-AD0140: MVD (sign and suffix) prediction, test 2.12a (MR 424)

Bug fixes:

- Revert MR383 and fix (MR 396)

- Fix #45 on IBC AMVR off (MR 397)

The following adopted aspects were integrated into VTM-11ecm anchor:

- JVET-AD0169: Downsampling filters in range 1.1 to 1.35 based on Kaiser(7) windowed sinc (MR 422)

This does not change the VTM-11ecm anchor performance under CTC.

- ECM-9.0 and VTM-11ecm9.0 were tagged on May 24, 2023.

- The following bug fixes were integrated into ECM-9.0:

- Fix mismatch with TIMD is 0 (MR 434)

- Fix luma offsets in AD0188 (MR 436)

- RPR-related fixes for JVET_AD0140_MVD_PREDICTION (MR 435)

- CCCM code refactoring (MR 431)

- Fix mismatch when --EnableTMTools=0 (MR 438)

- Fix crash when --EnableTMTools=0 (MR 440)

- Avoid temporary cu copy in EncModeCtrlMTnoRQT::useModeResult (MR 441)

- Fix AD0188 for GDR (MR 439)

- RPR fixes for JVET_AD0213_LIC_IMP and JVET_AD0140_MVD_PREDICTION (MR 437)

ECM-9.0 was tagged on July 5, 2023.

CTC Performance

ECM test results following ECM CTC configuration described in JVET-AD2017 are summarized below.

The next tables show ECM-9.0 performance over ECM-8.1 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.70% | -3.10% | -4.16% | 122.0% | 114.0% |
| Class A2 | -2.48% | -6.26% | -6.55% | 121.9% | 115.1% |
| Class B | -1.91% | -5.16% | -4.85% | 121.8% | 115.6% |
| Class C | -1.16% | -2.24% | -2.41% | 126.2% | 122.0% |
| Class E | -3.50% | -5.92% | -5.33% | 128.6% | 126.1% |
| Overall | -1.90% | -4.48% | -4.56% | 123.9% | 118.3% |
| Class D | -1.20% | -2.49% | -2.55% | 127.9% | 126.2% |
| Class F | -2.80% | -4.30% | -4.39% | 120.8% | 126.2% |
| Class TGM | -2.62% | -4.09% | -4.04% | 120.8% | 137.6% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.22% | -1.88% | -3.64% | 116.3% | 111.1% |
| Class A2 | -1.96% | -3.81% | -3.63% | 117.9% | 109.4% |
| Class B | -1.51% | -3.75% | -3.55% | 125.2% | 110.3% |
| Class C | -1.19% | -1.24% | -1.37% | 126.7% | 117.3% |
| Class E |  |  |  |  |  |
| Overall | -1.46% | -2.72% | -3.00% | 122.3% | 112.1% |
| Class D | -1.40% | -1.18% | -1.79% | 131.8% | 116.7% |
| Class F | -2.53% | -3.40% | -3.91% | 134.4% | 119.7% |
| Class TGM | -2.90% | -4.48% | -4.16% | 119.3% | 113.4% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.78% | -1.87% | -1.75% | 125.4% | 113.1% |
| Class C | -0.99% | -0.35% | -0.65% | 120.0% | 115.8% |
| Class E | -0.83% | -1.10% | -3.12% | 123.6% | 113.0% |
| Overall | -0.86% | -1.17% | -1.73% | 123.1% | 114.0% |
| Class D | -0.75% | -2.08% | -0.28% | 129.7% | 119.9% |
| Class F | -1.72% | -3.46% | -2.95% | 131.6% | 123.5% |
| Class TGM | -2.67% | -3.96% | -3.70% | 119.3% | 113.2% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.41% | -1.24% | -1.57% | 123.1% | 106.7% |
| Class C | -0.52% | -0.42% | -0.80% | 126.5% | 116.1% |
| Class E | -0.85% | -1.96% | -3.58% | 133.4% | 116.8% |
| Overall | -0.56% | -1.15% | -1.81% | 126.7% | 112.2% |
| Class D | -0.52% | -1.77% | -0.13% | 138.7% | 120.6% |
| Class F | -1.19% | -2.47% | -2.56% | 130.1% | 120.6% |
| Class TGM | -2.65% | -4.36% | -3.91% | 114.0% | 108.0% |

The below tables show ECM-9.0 performance comparing to VTM-11.0ecm9.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.67% | -21.16% | -28.19% | 827.7% | 406.4% |
| Class A2 | -15.77% | -27.44% | -30.55% | 813.4% | 405.9% |
| Class B | -10.06% | -25.61% | -23.82% | 773.3% | 423.4% |
| Class C | -10.27% | -15.86% | -16.33% | 783.1% | 442.4% |
| Class E | -13.65% | -23.55% | -22.06% | 759.1% | 481.4% |
| Overall | -11.59% | -22.66% | -23.71% | 788.5% | 430.8% |
| Class D | -8.61% | -13.65% | -13.74% | 803.2% | 462.1% |
| Class F | -24.34% | -33.43% | -33.28% | 529.7% | 445.8% |
| Class TGM | -37.29% | -45.21% | -44.55% | 443.8% | 443.1% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -20.86% | -24.25% | -32.21% | 757.7% | 741.6% |
| Class A2 | -23.90% | -32.20% | -35.28% | 697.8% | 882.0% |
| Class B | -19.50% | -31.71% | -29.62% | 648.2% | 739.3% |
| Class C | -20.90% | -24.61% | -24.58% | 672.8% | 779.1% |
| Class E |  |  |  |  |  |
| Overall | -21.03% | -28.42% | -29.93% | 685.5% | 777.1% |
| Class D | -21.87% | -25.90% | -26.55% | 699.1% | 849.6% |
| Class F | -26.77% | -34.78% | -35.14% | 578.7% | 528.1% |
| Class TGM | -35.17% | -41.37% | -41.35% | 547.2% | 419.4% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -17.01% | -32.11% | -30.48% | 601.4% | 658.9% |
| Class C | -18.50% | -26.42% | -26.63% | 590.5% | 661.3% |
| Class E | -15.40% | -25.47% | -26.14% | 595.7% | 440.1% |
| Overall | -17.10% | -28.56% | -28.11% | 596.3% | 596.4% |
| Class D | -20.39% | -28.41% | -27.33% | 629.6% | 770.9% |
| Class F | -23.36% | -33.28% | -31.66% | 562.9% | 526.4% |
| Class TGM | -33.93% | -40.53% | -40.11% | 511.1% | 404.8% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -15.87% | -31.42% | -29.54% | 537.3% | 625.1% |
| Class C | -17.58% | -25.77% | -25.54% | 521.8% | 661.5% |
| Class E | -14.93% | -24.37% | -25.89% | 579.6% | 462.0% |
| Overall | -16.20% | -27.77% | -27.29% | 542.3% | 590.6% |
| Class D | -20.36% | -28.35% | -27.51% | 563.4% | 739.4% |
| Class F | -22.24% | -33.15% | -31.57% | 541.3% | 486.8% |
| Class TGM | -30.83% | -38.46% | -38.16% | 526.4% | 378.7% |

The next tables show ECM-9.1 performance over the ECM-9.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.22% | -0.36% | 101.8% | 105.0% |
| Class A2 | -0.03% | -0.11% | -0.08% | 101.9% | 106.8% |
| Class B | -0.02% | -0.22% | -0.02% | 101.4% | 103.8% |
| Class C | 0.00% | -0.13% | -0.06% | 101.7% | 100.3% |
| Class E | -0.02% | -0.05% | -0.06% | 99.1% | 98.1% |
| Overall | -0.02% | -0.15% | -0.10% | 101.2% | 102.7% |
| Class D | 0.00% | -0.05% | 0.03% | 97.9% | 98.7% |
| Class F | -0.02% | -0.16% | -0.03% | 98.3% | 99.7% |
| Class TGM | -0.03% | -0.06% | -0.04% | 98.3% | 102.9% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.21% | -0.07% | 103.8% | 104.0% |
| Class A2 | -0.02% | -0.21% | -0.09% | 103.7% | 103.8% |
| Class B | 0.00% | -0.16% | -0.07% | 100.5% | 102.2% |
| Class C | -0.01% | -0.12% | -0.02% | 99.1% | 99.7% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.17% | -0.06% | 101.4% | 102.2% |
| Class D | 0.00% | -0.33% | 0.16% | 99.2% | 99.1% |
| Class F | -0.02% | -0.14% | -0.06% | 95.6% | 93.3% |
| Class TGM | 0.01% | 0.04% | -0.03% | 101.4% | 105.5% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.09% | -0.04% | 103.0% | 104.3% |
| Class C | 0.02% | -0.42% | -0.59% | 99.3% | 102.2% |
| Class E | 0.13% | -0.16% | 1.16% | 99.1% | 101.7% |
| Overall | 0.06% | -0.22% | 0.08% | 100.8% | 102.9% |
| Class D | -0.12% | 1.38% | -0.03% | 99.0% | 99.3% |
| Class F | -0.09% | -0.02% | 0.29% | 96.9% | 97.6% |
| Class TGM | -0.01% | 0.08% | -0.06% | 103.4% | 108.5% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.37% | -0.01% | 103.9% | 108.4% |
| Class C | -0.03% | -0.16% | 0.08% | 100.1% | 102.4% |
| Class E | 0.02% | -0.61% | 0.87% | 98.2% | 99.3% |
| Overall | 0.00% | -0.05% | 0.24% | 101.2% | 104.0% |
| Class D | 0.07% | 1.22% | 1.05% | 99.9% | 100.9% |
| Class F | 0.15% | 0.38% | -0.40% | 98.9% | 97.4% |
| Class TGM | -0.07% | -0.06% | -0.16% | 107.8% | 110.8% |

The Excel files with the complete test results are attached to the AHG report.

ECM memory consumption

ECM memory consumption is provided in ECM encoder log files and is summarized in the table below.

|  | RA | LD | AI |
| --- | --- | --- | --- |
| Class A | 25 GB |  | 12 GB |
| Class B | 14 GB | 12 GB | 10 GB |
| Class C | 6 GB | 6 GB | 6 GB |
| Class E |  | 8 GB | 7 GB |
| Class D | 5 GB | 5 GB | 5 GB |
| Class F | 14 GB | 12 GB | 11 GB |
| Class TGM | 16 GB | 14 GB | 11 GB |

It is encouraged to take care about memory allocation when developing and integrating tools into the ECM. In particular, it is strongly recommended to re-use already allocated memory wherever possible, rather than systematically allocating new memory.

Recommendations

The AHG recommended to:

- Continue to develop ECM software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues.

- Encourage people to submit merge requests fixing identified bugs.

JVET-AE0007 JVET AHG report: ECM tool assessment (AHG7) [X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, H.-J. Jhu, X. Li, H. Wang (vice-chairs)]

Group off tests

Test settings and crosschecking

Based on the discussion in the previous meeting, the same four groups were used in this meeting cycle.

- Group 1: Inter template matching tools

- Group 2: Coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process

- Group 3: Intra and IBC template matching (with search) related tools

- Group 4: Tools that require more processing on the neighbouring reconstructed samples than VVC

Accordingly, five group-off tests were performed and crosschecked on top of ECM-9.1. The two anchors are ECM 9.0 and VTM-11ECM9.0. The cfg files used are also attached with this AHG report.

The testers and crosscheckers are summarized in the table below. All the tests below have been crosschecked.

| Tests | Tester | Crosschecker |
| --- | --- | --- |
| Group 1 off | Jonathan Gan (v-jonathan.gan@oppo.com) | Charles Salmon-Legagneur (charles.salmon-legagneur@interdigital.com) |
| Group 2 off | Jonathan Gan (v-jonathan.gan@oppo.com) | Xinwei Li (sid.lxw@alibaba-inc.com) |
| Group 3 off | Xiang Li (xlxiangli@google.com) | Zhipin Deng (zhipin.deng@bytedance.com) |
| Group 4 off | Xiang Li (xlxiangli@google.com) | Hong-Jheng Jhu (jhuhong-jheng@kwai.com) |
| Group 1-4 off | Xiang Li (xlxiangli@google.com) | Lien-Fei Chen (lienfei.chen@global.tencent.com) Hongtao Wang (hongtaow@qti.qualcomm.com) |

Group 1 off

Group 1 includes inter template matching tools. The attached offgroup1.cfg was used in addition to ECM CTC settings.

Group 2 off

Group 2 includes coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process. The attached offgroup2.cfg was used in addition to ECM CTC settings.

Group 3 off

Group 3 includes intra and IBC template matching (with search) related tools. The attached offgroup3.cfg was used in addition to ECM CTC settings. Note that some parts of template matching for intra and IBC cannot be correctly switched off in ECM-9. The impacted tools are LBCCP and ARMC.

Group 4 off

Group 4 includes tools that require more processing on the neighbouring reconstructed samples than VVC.

The attached offgroup4.cfg was used in addition to ECM CTC settings.

Group 1-4 off

In this test, all the tools in the group 1-4 are switched off. The attached offgroup1-4.cfg was used in addition to ECM CTC settings.

Summary

The average RA results compared to ECM-9 are summarized below. Note that SCC results are not included. More figures, such as using ECM-9 as the anchor, may be found in the spreadsheet summary attached to the AHG report.

Figure: BD-rate Y (over VTM11-ECM9) vs Encoding Time

Note that weighting factor 6:1:1 is used in the figure below. In the attached summary, different weighting factors may be specified.

Figure: Weighted BD-rate YUV (over VTM11-ECM9) vs Encoding Time

Figure: Bd-rate Y (over VTM11-ECM9) vs Decoding Time

Resolved issues and new issues

Resolved issues

- Software issues #27 and #45 with tool off tests were resolved

- It has been verified in the tests that the memory usage ratio is relatively stable

New issues

- In ECM-9 some parts of template matching for intra and IBC cannot be correctly switched off, which impact several coding tools, such as LBCCP, ARMC

- Memory usage ratio keeps 100% in the group off tests, though 7-11% memory increase by ECM-9 over ECM-8.1 was observed

  - Two possible reasons 1) The memory usage increase in ECM-9 over ECM-8 was not introduced by the tested tools; 2) Memory was always allocated even a tool is disabled

- The variation of runtime over VTM11ECM9 is relatively large, for which the version of compiler matters

Input contributions

| JVET-AE0174 | AHG7: On TM control for non-inter tools | Z. Deng, K. Zhang, L. Zhang (Bytedance) |
| --- | --- | --- |
| JVET-AE0180 | On ECM SW memory consumption | R. Chernyak, S. Liu (Tencent), Y. Yasugi, T. Ikai (Sharp) |
| JVET-AE0195 | AhG7: ECM stages complexity assessment | F. Galpin, F. Leleannec, C. Salmon-Legagneur, E. François (InterDigital) |

Recommendations

The AHG recommended to:

- Continue and improve tool assessment

- Report memory usage ratio in the ECM CTC spread sheet, as suggested in the BoG report in JVET-AD0401

- Resolve identified software issues related to the tool assessment

- Review all the input contributions

It was commented that recording the run times with two digits after decimal point may not be useful in assessing complexity.

JVET-AE0008 JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) [C. Hollmann, S. Liu, S. Wang, M. Zhou (AHG chairs)]

Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, for email discussion. There were a few emails exchanged on the reflector uner the context of AHG 8 during this period. There were more email exchanges and discussions among co-chairs, editors and interested experts related to technical report preparation, common test conditions and anchor generation, etc. There was one joint meeting with WG 4 held on 2023-06-21. There are 12 input contriubtions related to AHG 8 mandates submitted to this meeting. They are listed in Section 3.

Common Test Conditions

Common test conditions (CTC) for optimization of encoders and receiving systems for machine analysis of coded video content, are summarized in output document JVET-AD2031. This document includes detailed descriptions of test datasets, anchor software and configurations, anchor generation processes, machine task networks used, test and training conditions, evaluation methodologies and metrics. A reporting template in Excel format is enclosed in the document package. This output document package was uploaded on 2023-05-11, with updated anchor results, and is also available at https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/ofm-ctc.

Test datasets

Two video datasets, referred as SFU-HW and TVD, are included in the CTC and results on both are expected to be reported. In addition, three image datasets, referred as TVD (image), OpenImageV6 and FLIR, are included in the CTC and can be tested using intra-only configuration. The results on the image datasets are optional.

It was decided in the last JVET meeting to add “Kimono” sequence (back to) the SFU-HW dataset, as a separate optional (O) class, and thus the SFU-HW dataset now consist of 14 test video sequences, as shown in the table below. These sequences can be found on ftp://hevc@mpeg.tnt.uni-hannover.de. The annotations are available at https://data.mendeley.com/datasets/hwm673bv4m/1.

Test sequences in SFU-HW dataset

| Class | Sequence name | Frame count | Frame rate | Bit depth | Frames skipped | Frames coded | Intra | RA | LD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Traffic | 150 | 30 | 8 | 117 | 33 | M | M | M |
| B | ParkScene | 240 | 24 | 8 | 207 | 33 | M | M | M |
| B | Cactus | 500 | 50 | 8 | 403 | 97 | M | M | M |
| B | BasketballDrive | 500 | 50 | 8 | 403 | 97 | M | M | M |
| B | BQTerrace | 600 | 60 | 8 | 471 | 129 | M | M | M |
| C | RaceHorses | 300 | 30 | 8 | 235 | 65 | M | M | M |
| C | BQMall | 600 | 60 | 8 | 471 | 129 | M | M | M |
| C | PartyScene | 500 | 50 | 8 | 403 | 97 | M | M | M |
| C | BasketballDrill | 500 | 50 | 8 | 403 | 97 | M | M | M |
| D | RaceHorses | 300 | 30 | 8 | 235 | 65 | M | M | M |
| D | BQSquare | 600 | 60 | 8 | 471 | 129 | M | M | M |
| D | BlowingBubbles | 500 | 50 | 8 | 403 | 97 | M | M | M |
| D | BasketballPass | 500 | 50 | 8 | 403 | 97 | M | M | M |
| O | Kimono | 240 | 24 | 8 | 207 | 33 | O | O | O |

Note: M – mandatory; O – optional.

It was decided in the last JVET meeting to replace some of the compressed source video sequences in Tencent Video Dataset (TVD) by their uncompressed versions. The updated source video sequences are renamed, as shown in the next table below, and can be downloaded from https://multimedia.tencent.com/resources/tvd with corresponding annotations.

Test sequences in TVD

| Sequence name | Frame count | Frame rate | Bit depth | Frames skipped | Frames coded | Intra | Random access | Low delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TVD-01-1 | 3000 | 50 | 8 | 1500 | 500 | M | M | M |
| TVD-01-2 | 3000 | 50 | 8 | 2000 | 500 | M | M | M |
| TVD-01-3 | 3000 | 50 | 8 | 2500 | 500 | M | M | M |
| TVD-02-1 | 636 | 50 | 10 | 0 | 636 | M | M | M |
| TVD-03-1 | 2334 | 50 | 10 | 0 | 500 | M | M | M |
| TVD-03-2 | 2334 | 50 | 10 | 500 | 500 | M | M | M |
| TVD-03-3 | 2334 | 50 | 10 | 1000 | 500 | M | M | M |

As the original sequences are available in .mp4 format, they shall be converted to YUV420 using FFmpeg. The format conversion process was cleaned up as follows:

ffmpeg -i {input.mp4} {output.yuv}

The TVD (image) dataset is an image dataset of 166 images of 1920x1080 resolution that have annotations for object detection and instance segmentation. The dataset with corresponding annotations is available at https://multimedia.tencent.com/resources/tvd.

The OpenImages dataset consists of around 9 million images. A subset of the validation set of its version 6 containing 5000 images are selected for testing object detection in this activity. The dataset with corresponding annotations is available at https://storage.googleapis.com/openimages/web/index.html.

The FLIR dataset used in the WG 4 VCM activities is a dataset consisting of 300 infrared images. The images, annotations and the fine-tuned model for thermal images can be found on the MPEG content repository (https://content.mpeg.expert/data/).

More detailed information about these test datasets can be found in JVET-AD2031.

Anchor software and configuration

Version 12.0 of the VTM software is used for generating anchor results. The VTM software is available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/.

In addition, version 4.2.2 of the FFmpeg is used for the format conversion. The FFmpeg software is available at https://ffmpeg.org/releases.

Three test conditions are used for experimenting encoder and receiving system optimizations on video coding for machine consumptions, i.e., random-access, low-delay and all-intra. The default configuration files provided with the VTM software are used for anchor generation.

- “Random access” (RA): encoder_randomaccess_vtm.cfg

- “Low delay” (LD): encoder_lowdelay_vtm.cfg

- “All Intra” (AI): encoder_intra_vtm.cfg

It was decided in the last JVET meeting that temporal subsampling is disabled for “All Intra” configuration for this activity, thus the following line

TemporalSubsampleRatio : 8

in “encoder_intra_vtm.cfg” needs to be modified to

TemporalSubsampleRatio : 1

A subset of these configurations might be used for a particular experiment. However, as these test conditions are for video coding experiments, results for at least one of the random access or low delay configurations are expected to be provided.

Anchor generation

The anchor generation process is illustrated in the figure below. The task network Faster R-CNN X101-FPN which is part of Detectron2 (https://github.com/facebookresearch/detectron2) is used to evaluate object detection (SFU-HW dataset and image datasets), the model is available here. The task network JDE-1088x608 which is part of the Towards Realtime MOT framework (https://github.com/Zhongdao/Towards-Realtime-MOT) is used to evaluate object tracking (TVD), the model is available at this Google Drive link. More details about anchor generation process and these machine task networks are referred to JVET-AD2031.

Figure: Anchor generation pipeline

Anchor results were generated following the CTC described in JVET-AD2031 and crosschecked by Ericsson and Tencent.

Evaluation and reporting

Proposed technologies are evaluated based on their compression performance, measured by bitrate, PSNR, mAP and MOTA, as well as encoding and decoding runtime to reflect the complexity of the proposed technology, to some extent. Definitions and detailed descriptions of these metrics can be found in JVET-AD2031.

It is noted that the mean Average Precision (mAP) is used to measure object detection performance, and Multiple Object Tracking Accuracy (MOTA) is used to measure object tracking performance. For the purpose of reporting encoding and decoding running times, the anchor and proposal should be simulated on the same platform, e.g. similar CPU and GPU configuration, to have reliable time comparison. Parallel encoding and decoding as described in JVET-B0036 may be applied for RA configurations.

In addition, relevant inference and training information should be reported if the proposed technology consists of learning-based components, such as network structure, number of network parameters, precision of network parameters, number of multiply–accumulate operations (MAC) per pixel, patch size, batch size, epoch, training time, training datasets, lost function, number of iterations, optimizer, and any pre- and post-processings used. More details can be found in JVET-AD2031.

A reporting template in Excel file format has been prepared to illustrate results following CTC and evaluation methodology described in JVET-AD2031. It is enclosed in JVET-AD2031 package.

Reporting template summary

Technical Report

The draft 2 of the technical report (TR) has been prepared and uploaded as JVET-AD2030 based on discussions in the last JVET meeting on 2023-06-07. More descriptions were added to use cases and applications, pre-processing technologies and encoding technologies. Two software implementation examples were included in the TR draft 2 as Annex.

Git Management

AHG 8 related software and documents can be found at https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm. This repository contains two projects, one (https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/ofm-ctc) containing instrucitons and information for conducting experiements and evaluation, such as evaluation scripts, machine task networks, CTC and reporting template with anchor results, while the other (https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/vtm-ofm) containing implementation examples. Two software example packages have been uploaded to the git in separate branches:

- JVET-AB0275: a region of interest-based method that uses adaptive QP to reduce the quality in background areas

- JVET-AC0086: a method that uses a pre-analysis to perform content adaptive machine vision oriented preprocessing

AHG Coordination

Collaborative discussions were held between this AHG and the VCM AHG in WG4 to synchronize common test conditions including test datasets, anchor QP points, encoder configurations, etc. A joint meeting was held on 2023-06-21 0500UTC to discuss metrics, existing issues and potential solutions for machine task performance evaluation. Two contributions were presented and discussed during this meeting:

m63676 - [VCM] Proposed performance evaluation method for VCM CE1 (ETRI, Konkuk Univ., Myongji Univ.)

This contribution proposes a performance evaluation method for VCM CE1.

Regarding the performance evaluation for VCM CE1, there are some difficulties as follows:

- The current BD-rate metric provided by the reporting template is not able to produce a value if either the anchor or a proposal is non-monotonic.

- To compensate for this limitation, the reporting template requires submission of BD-rate for multiple subcases (combination of 4 or 5 points out of 6 total points), but it is still difficult to make a direct comparison between proposals since the metric is not singular.

The following rules were proposed as the performance evaluation method for VCM CE1:

- For each test case, a proponent should submit 4 points that correspond to the high 4 rate points of the VTM anchor.

- Only if the result at the high 4 rate points is non-monotonic, then it is allowed to adjust QP in a range of [-2, +2] to produce a monotonic result.

- If a monotonic result cannot be obtained even with the adjustment of QP in the range, then it is not considered as a candidate technology in this round of CE1 evaluation.

- Evaluate the proposed technologies using the summed scores based on the class-wise (SFU) or sequence-wise (TVD) BD-rate rankings of the proposals.

JVET-AE0107 / m63692 - [VCM] Improvements of the BD-rate model using monotonic curve-fitting method (Tencent)

This contribution provides improvements of the BD-rate model used in current Excel templates. Specifically, the contribution proposes a monotonic fitted curve to replace the PCHIP (Piecewise Cubic Hermit Interpolation) curve for BD-rate and BD-metric calculation, such as BD-PSNR, BD-mAP, BD-MOTA, etc. The fitted curve guarantees monotonicity even if the input points are nonmonotonic, thus makes it possible to calculate BD-rate and BD-metric values in various testing conditions.

This contribution was also submitted to JVET (AHG 8) as document JVET-AE0107.

Input contributions

There were 12 input contributions related to AHG 8 mandates. They are listed below.

| Report |  |  |
| --- | --- | --- |
| JVET-AE0008 | JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) | C. Hollmann, S. Liu, S. Wang, M. Zhou (AHG chairs) |
| Proposal |  |  |
| JVET-AE0053 | AHG8/AHG9: Neural-network post-filter regions SEI message | T. Chujoh, Y. Yasugi, T. Ikai (Sharp) |
| JVET-AE0064 | AHG8/AHG9: Signalling encoder preprocessing and human / machine viewing indications | C. Kim, D. Gwak, Hendry, J. Lim, S. Kim (LGE), M. M. Hannuksela, F. Cricri, H. Zhang (Nokia) |
| JVET-AE0079 | AHG8/AHG9: Source picture timing information SEI message | S. McCarthy, G. J. Sullivan, P. Yin (Dolby) |
| JVET-AE0081 | [AHG8] De-noising filter as pre-processing for machine task | C. Kim, D. Gwak, J. Lim (LGE) |
| JVET-AE0090 | AHG8/AHG9: On machine vision indication | J. Gao, H.-B. Teo, C.-S. Lim, K. Abe (Panasonic) |
| JVET-AE0095* | AHG8/AHG9: proposed changes to the candidate new object mask information SEI message | P. de Lagrange, E. François, D. Doyen (InterDigital), J. Chen, S. Wang, Y. Ye (Alibaba) |
| JVET-AE0096 | [AHG8] Study on using different VTM versions | C. Hollmann, J. Ström (Ericsson) |
| JVET-AE0099 | [AHG8] NNPF and post-filter hint SEI messages for the technical report | C. Hollmann, M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson) |
| JVET-AE0107 | AHG8: Improvements of the BD-rate model using monotonic curve-fitting method | H. Wang, X. Pan, Z. Liu, X. Xu, S. Liu (Tencent) |
| JVET-AE0143 | AHG8: A spatial resampling algorithm and an exemplar software implementation | S. Wang, B. Li, J. Chen, Y. Ye (Alibaba), S. Wang (CityU) |
| Crosscheck |  |  |
| JVET-AE0234 | AHG8: Crosscheck of JVET-AE0107 | Honglei Zhang (Nokia) |

* Late.

Recommendations

The AHG recommended to:

- Review all input contributions.

- Discuss and refine test conditions, evalution and reporting procedures.

- Discuss the non-monotonic issue and potential solutions.

- Continue investigating non-normative technologies and their suitability for machine analysis applications.

- Continue developing draft technical report on optimization of encoders and receiving systems for machine analysis of coded video content.

- Continue collecting new test materials.

It was pointed out that a potential timeline for the technical report should be discussed. This however may also relate to the level of completeness in terms of using video compression standards for a variety of machine usage applications, which somewhat relies on the availability of test material of sufficient variety.

JVET-AE0009 JVET AHG report: SEI message studies (AHG9) [S. McCarthy, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, C. Fogg, Hendry, P. de Lagrange, G. J. Sullivan, A. Tourapis, S. Wenger (vice-chairs)]

Related contributions

A total of 42 contributions and 1 bug report were identified relating to the mandates of AHG9. Some contributions also relate to the work of AHG2 and AHG8.

The number of contributions relating to each AHG9 mandate is as follows

- 31 contributions relate to the mandate to study the SEI messages in VSEI, VVC, HEVC, and AVC. 2 of the 31 contributions relate to more than one SEI message.

  - 28 contributions relate to the neural-network post-filter SEI messages, including 1 summary of proposals on NNPF SEI messages.

  - 1 contribution relates to the annotated region SEI message

  - 2 contribution relates to the post filter hint SEI message

  - 1 contribution relates to the phase indication SEI message

  - 1 contribution relates to the SEI processing order SEI message

- 1 contribution relates to the mandate to collect software and showcase information for SEI messages;

- 10 contributions relate to the mandate to identify potential needs for additional SEI messages.

  - 2 contributions relate to human/machine viewing

  - 1 contribution relates to multiplane image information

  - 2 contributions relate to alternative picture timing

  - 3 contributions relate to generative face video

  - 1 contribution relates to object masking information

  - 1 contribution relates to object-wave compression and computer-generated hologram use

- 1 bug report relates to the mandate to coordinate with AHG3 for software support of SEI messages

The following is a list of contributions related to the mandates of AHG9.

Study the SEI messages in VSEI, VVC, HEVC and AVC

Neural-network post filter characteristics and activation SEI messages

Summary of proposals on NNPF SEI messages

JVET-AE0187 AHG9: A summary of SEI proposals on NNPF [Y.-K. Wang (Bytedance)]

Proposals on NNPF SEI messages

JVET-AE0048 AHG9: Miscellaneous VSEI changes on neural-network post-filter SEI messages [M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia)]

JVET-AE0049 AHG9: Miscellaneous VVC changes on neural-network post-filter SEI messages [M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia)]

JVET-AE0050 AHG9: On NNPF input picture selection [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AE0051 AHG9: On persistent NNPF activation [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AE0052 AHG9: NNPF cascades and alternatives [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AE0053 AHG8/AHG9: Neural-network post-filter regions SEI message [T. Chujoh, Y. Yasugi, T. Ikai (Sharp)]

JVET-AE0060 [AHG9] Comments on NNPFC [S. Deshpande (Sharp)]

JVET-AE0061 [AHG9] On NNPFC Application Purpose [S. Deshpande (Sharp)]

JVET-AE0062 [AHG9] On NNPF for Deinterlacing [A. Sidiya, S. Deshpande (Sharp)]

JVET-AE0063 [AHG9] On operations for multiple NNPFs [L. Chen, O. Chubach, Y.-W. Huang, S. Lei (MediaTek)]

JVET-AE0068 AHG9: On extensibility of purpose syntax element in NNPFC SEI message [Hendry, J. Nam, S. Kim, J. Lim (LGE)]

JVET-AE0069 AHG9: On the signalling of output pictures in NNPFA SEI message [Hendry, J. Nam, S. Kim, J. Lim (LGE)]

JVET-AE0070 AHG9: On input pictures that are not present in the bitstream for NNPF SEI messages [Hendry, J. Nam, S. Kim, J. Lim (LGE)]

JVET-AE0099 [AHG8] NNPF and post-filter hint SEI messages for the technical report [C. Hollmann, M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

(JVET-AD0099 also relates to the post-filter hint SEI message)

JVET-AE0101 [AHG2][AHG9] Neural network post filter and phase indication SEI messages for AVC and HEVC [T. Ikai, T. Chujoh (Sharp), Y.-K. Wang, J. Xu, W. Jia (Bytedance)]

(JVET-AD0101 also relates to the phase indication SEI message)

JVET-AE0106 AHG9: On missing value ranges for some syntax elements in the NNPFC SEI message [C. Lin, Y.-K. Wang, J. Xu, W. Jia, J. Li, Y. Li, K. Zhang, L. Zhang (Bytedance)]

JVET-AE0113 AHG9: Extendibility and code word length of nnpfc_purpose [R. Sjöberg, M. Pettersson (Ericsson)]

JVET-AE0126 AHG9: NNPF cleanup and editorial changes for VSEI [Y.-K. Wang, W. Jia, J. Xu, C. Lin (Bytedance)]

JVET-AE0127 AHG9: NNPF editorial changes for VVC [Y.-K. Wang, W. Jia, J. Xu (Bytedance)]

JVET-AE0128 AHG9: On NNPFC extensibility and base filter signalling [Y.-K. Wang (Bytedance)]

JVET-AE0134 AHG9: Align the design of NNPF with multiple input pictures to NNPF including picture rate upsampling [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AE0135 AHG9: On NNPF picture rate upsampling constraints [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AE0141 AHG9: Fix a bug in NNPFC SEI message for colourization [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AE0142 AHG9: On derivation of NNPF input pictures and the value of nnpfc_purpose [W. Jia, Y.-K. Wang, J. Xu, L. Zhang (Bytedance)]

JVET-AE0173 [AHG9] Clarification and improvements of signalling of NNPF update [Y. Lim (Samsung)]

JVET-AE0175 [AHG9] Editorial improvements of nnpfc_mode_idc [Y. Lim (Samsung)]

JVET-AE0189 AHG9: On the design of nnpfa_target_base_flag in NNPFA SEI message [Hendry (LGE)]

Annotated regions SEI message

JVET-AE0054 AHG2/AHG9: Editorial improvements of annotated regions SEI message [T. Chujoh, T. Aono, T. Ikai (Sharp)]

Post filter hint SEI message

JVET-AE0099 [AHG8] NNPF and post-filter hint SEI messages for the technical report [C. Hollmann, M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

(JVET-AD0099 also relates to NNPF SEI messages)

JVET-AE0155 Editor commentary on the post-filter hint SEI message semantics [G. J. Sullivan, Y.-K. Wang (Editors)

Phase indication SEI message

JVET-AE0101 [AHG2][AHG9] Neural network post filter and phase indication SEI messages for AVC and HEVC [T. Ikai, T. Chujoh (Sharp), Y.-K. Wang, J. Xu, W. Jia (Bytedance)]

(JVET-AD0101 also relates to NNPF SEI messages)

SEI processing order SEI message

JVET-AE0156 AHG9: Message wrapping and importance indication for the SEI processing order SEI message [G. J. Sullivan, S. McCarthy, P. Yin (Dolby Labs)]

Collect software and showcase information for SEI messages

JVET-AE0143 AHG8: A spatial resampling algorithm and an exemplar software implementation [S. Wang, B. Li, J. Chen, Y. Ye (Alibaba), S. Wang (CityU)]

Identify potential needs for additional SEI messages

Human/machine viewing

JVET-AE0064 AHG8/AHG9: Signalling encoder preprocessing and human / machine viewing indications [C. Kim, D. Gwak, Hendry, J. Lim, S. Kim (LGE), M. M. Hannuksela, F. Cricri, H. Zhang (Nokia)]

JVET-AE0090 AHG8/AHG9: On machine vision indication [J. Gao, H.-B. Teo, C.-S. Lim, K. Abe (Panasonic)]

Multiplane image information

JVET-AE0066 AHG9: Multiplane Image Information SEI [T. Lu, P. Yin, G. Su, D. Lee, T. Huang, S. McCarthy, W. Husak, G. J. Sullivan (Dolby)]

Alternative picture timing

JVET-AE0079 AHG8/AHG9: Source picture timing information SEI message [S. McCarthy, G. J. Sullivan, P. Yin (Dolby)]

JVET-AE0089 AHG9: Alternative Output Timing Hint SEI [H.-B. Teo, J. Gao, C.-S. Lim, K. Abe (Panasonic)]

Generative face video

JVET-AE0080 AHG9: Generative Face Video SEI message [S. McCarthy, P. Yin, G.-M. Su, A. K. Choudhury, W. Husak (Dolby)]

JVET-AE0083 AHG9: Common SEI Message of Generative Face Video [B. Chen, J. Chen, Y. Ye (Alibaba), S. Wang (CityU)]

JVET-AE0088 AHG9: A study on Generative Face Video SEI Message [H.-B. Teo, J.-Y Thong, K. Jayashree, C.-S. Lim, K. Abe (Panasonic)]

Object mask information

JVET-AE0095 AHG8/AHG9: proposed changes to the candidate new object mask information SEI message [P. de Lagrange, E. François, D. Doyen (InterDigital), J. Chen, S. Wang, Y. Ye (Alibaba)]

Object-wave compression and computer-generated hologram use

JVET-AE0182 AHG9: SEI message extension of VVC for object-wave compression and computer-generated hologram use [K. Nonaka, R. Koiso, H. Kojima, K. Kawamura, H. Kato (KDDI)]

Coordinate with AHG3 for software support of SEI messages

Bug report https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/1605

Activities

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de) with [AHG9] in message headers. There were no emails sent to the JVET reflector during the AHG period with [AHG9] in the message header.

Recommendations

The AHG recommended to:

- Review all related contributions; and

- Continue SEI messages studies.

It was agreed that no technical aspects regarding new functionality should be added to the NNPF SEI message for v3; the main focus was on bug fixes. In terms of purpose, no new items should be added to v3, as long as extensibility is possible.

A BoG on VSEI V3 aspects and VVC V3 SEI message aspects was established (coordinated by G. Sullivan), to meet from Tuesday afternoon. The BoG was also requested to sort out which proposals in the NNPF category would rather be candidates for a future version.

JVET-AE0010 JVET AHG report: Encoding algorithm optimization (AHG10) [P. de Lagrange, A. Duenas, R. Sjöberg, A. Tourapis (AHG chairs)]

Related contributions

A total of 5 contributions, not including cross-checks, were identified relating to AHG10, and summarized in the following sections.

Adaptive resolution

JVET-AE0104 - AHG10: GOP-based RPR encoder control using parallel resolution encoding

This contribution proposes multi-pass GOP-based adaptive resolution across full-resolution and 4/5, 2/3, and ½ reduced resolutions, using a rate-distortion cost based on squared error (=PSNR). The reported BD-rate difference in RA class A (using CTC QPs) are about -1.5% when compared to regular full-resolutions, and around -1% when compared to the current single-pass GOP-based RPR technique implemented in the VTM. When adding a lower quality point on top of the CTC QPs (QP=42 for UHD), gain increases compared to regular coding, but decreases compared to current GOP-based RPR. Gains are generally lower for lower resolutions and for LD and AI configurations.

Low delay

JVET-AE0114 - AHG10: Low-Delay configuration improvements

This contribution proposes changes to the low-delay configurations regarding pictures and QPs for improved coding efficiency. In addition to a -3 QP offset to the first intra picture (instead of -1), it is proposed to introduce a multi-level QP structure with a periodicity of 16 pictures, plus a picture with a -2 offset every 64 pictures. Reference picture list is changed to take advantage of higher quality reference pictures. A BD-rate difference of around -6% is reported.

Bug fixes, more encoder control

JVET-AE0111 - Additional conformance tests of spatial scalability for multilayer coding

Besides proposing an additional conformance bitstream, this contribution reports about fixing a bug in reference picture list when combining scalability and low-delay configuration: first picture of enhancement layer is intra-coded (not using the base layer as a reference), and for the remaining pictures, intra-layer prediction is removed when inter-layer referencing is activated; the proposed fix is in MR !2633.

JVET-AE0122 - AHG10: GOP-based RPR and scalable coding

This contribution proposes a bug fix and more control over the encoder when combining RPR and multi-layer. One use case is to restrict reduced resolution coding to the base layer. A marginal BD-rate gain is reported when testing the proposed changes using 1.5x scalability and GOP-based RPR for the base layer, over the test sequences currently considered for scalability visual testing.

JVET-AE0181 - Scaling window support for VTM

This contribution reports about adding control of the scaling window to the VTM, with additional configuration parameters. It also reports about an experiment using dual-layer coding with the enhancement layer containing a scaled-down, windowed version of the base layer. When coding without scaling window, affine coding mode is mostly used, while merge mode is used when using scaling window. A lower rate with a higher PSNR is obtained.

Recommendation

The AHG recommends that the related input contributions are reviewed and to further continue the study of encoding algorithm optimizations in JVET.

JVET-AE0011 JVET AHG report: Neural network-based video coding (AHG11) [E. Alshina, S. Liu, A. Segall (co chairs), F. Galpin, J. Li, R.-L. Liao, D. Rusanovskyy, T. Shao, M. Wien, P. Wu (vice-chairs)]

Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, for email. Forty-one emails were exchanged on the reflector related to the AHG mandates.

Common Test Conditions

Document

The AHG released revised common test conditions as decided at the 30th meeting. The final version was uploaded as document JVET-AD2016 on May 12, 2023.

Anchor Encoding

Anchors for the NN-based video coding activity made available on the Git repository used for the AHG activity: https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master.

EE Coordination

The AHG finalized, conducted and discussed the EE on NN based video coding. The final version of the EE description was uploaded to the document repository on May 15, 2023.

A summary report for the EE is available at this meeting as:

| JVET-AE0023 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang, Z. Xie (EE coordinators) |
| --- | --- | --- |

Teleconferences

The AHG conducted two teleconferences during the interim period. The teleconferences were held on June 14, 2023, and June 28, 2023 and attended by approximately 20 participants.

A summary report for the teleconferences is available at this meeting as:

| JVET-AE0042 | AhG14 & AHG11: Report on AhG teleconference on high operation point (HOP) unified filter training | E. Alshina, F. Galpin |
| --- | --- | --- |

Workshop on Neural Network-Based Technologies

The AHG contributed to the “Third AG4 Workshop on JPEG and MPEG Emerging Activities” that was conducted by the JPEG and MPEG Collaboration Advisory Group, or ISO/IEC JTC1/SC29/AG4.

The workshop took place on June 1st, 2023, and its agenda was:

| Speaker | Topic |
| --- | --- |
| Elena Alshina | AI-based Video Coding |
| Werner Bailer | Neural Network Compression |
| Marc Antonini | Coding for DNA Storage |
| Tim Bruylants | Event-based Vision |
| Arianne Hids | Scene-based Interchange and Support of Immersive Displays |

The AHG contributed the presentation titled “AI-based Video Coding”. Slides for the presentation are available in JVET-AE0237.

Performance Evaluation

The performance of the NNVC-5.0 anchor compared to NNVC-4.0 is reported below.

|  | Random access Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | -7.23% | -4.86% | -6.00% | -8.72% | -4.05% | -5.06% | 134% | 7667% | 96% |
| Class A2 | -6.56% | -5.78% | -5.09% | -6.53% | -4.14% | -2.90% | 130% | 7183% | 97% |
| Class B | -6.29% | -8.19% | -7.53% | -5.97% | -6.06% | -5.18% | 132% | 7834% | 98% |
| Class C | -6.62% | -10.53% | -9.55% | -6.26% | -7.91% | -5.58% | 132% | 7422% | 98% |
| Class E | - | - | - | - | - | - | - | - | - |
| Overall | -6.62% | -7.66% | -7.28% | -6.71% | -5.77% | -4.81% | 132% | 7557% | 97% |
| Class D | -7.57% | -9.45% | -9.68% | -5.76% | -6.81% | -5.94% | 129% | 8193% | 98% |
| Class F | -3.07% | -5.88% | -4.94% | -3.05% | -4.76% | -3.54% | 148% | 9739% | 99% |
| Class H | - | - | - | - | - | - | - | - | - |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | - | - | - | - | - | - | - | - | - |
| Class A2 | - | - | - | - | - | - | - | - | - |
| Class B | -4.85% | -8.13% | -7.84% | -5.15% | -6.16% | -6.65% | 134% | 6812% | 99% |
| Class C | -5.24% | -11.13% | -9.00% | -5.61% | -10.74% | -4.02% | 126% | 6233% | 99% |
| Class E | -4.88% | -4.09% | -5.17% | -5.64% | -2.18% | -3.36% | 162% | 9877% | 98% |
| Overall | -4.98% | -8.12% | -7.56% | -5.42% | -6.69% | -4.95% | 138% | 7257% | 99% |
| Class D | -6.54% | -10.76% | -9.77% | -5.75% | -9.48% | -6.83% | 125% | 6400% | 98% |
| Class F | -2.05% | -4.88% | -4.46% | -2.29% | -3.08% | -4.12% | 148% | 8941% | 99% |
| Class H | - | - | - | - | - | - | - | - | - |
|  | All Intra Main 10 |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | -8.46% | -7.23% | -8.67% | -10.00% | -8.00% | -7.50% | 210% | 5134% | 97% |
| Class A2 | -6.61% | -8.04% | -7.94% | -6.90% | -6.44% | -5.44% | 204% | 4416% | 98% |
| Class B | -7.03% | -8.58% | -8.71% | -7.04% | -7.61% | -6.99% | 203% | 4493% | 98% |
| Class C | -7.34% | -9.76% | -9.92% | -7.61% | -8.38% | -7.64% | 190% | 3770% | 98% |
| Class E | -10.29% | -9.04% | -8.85% | -10.14% | -7.52% | -6.38% | 199% | 5149% | 96% |
| Overall | -7.81% | -8.60% | -8.87% | -8.16% | -7.64% | -6.86% | 201% | 4507% | 97% |
| Class D | -7.57% | -8.11% | -9.33% | -7.16% | -6.81% | -7.56% | 180% | 3760% | 98% |
| Class F | -4.55% | -5.97% | -5.65% | -4.24% | -4.75% | -4.38% | 155% | 3961% | 98% |
| Class H | - | - | - | - | - | - | - | - | - |

Input contributions

There are 47 input contributions related to the AHG mandates. Ten of the contributions are part of the EE activity, while the remaining 37 contributions are related to AHG11 but not part of the EE. The list of input contributions is provided below.

EE and Related Input Contributions

| Reporting |  |  |
| --- | --- | --- |
| JVET-AE0023 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang, Z. Xie |
| EE Technology |  |  |
| JVET-AE0067 | EE1-4.1: Neural-network loop filters with further complexity reduction | J. N. Shingala, A. Shyam, A. Suneja, S. Badya (Ittiam), T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, Sean McCarthy (Dolby) |
| JVET-AE0112 | EE1-5.1: Deep Reference Frame Generation for Inter Prediction Enhancement | W. Bao, W. Meng, J. Jia, Y. Zhang, H. Wang, Z. Chen (Wuhan Univ.), Z. Liu, X. Xu, S. Liu (Tencent) |
| JVET-AE0144 | EE1-6.1: neural network-based intra prediction with reduced complexity | T. Dumas, F. Galpin, P. Bordes (Interdigital) |
| JVET-AE0160 | EE1-1.5: Optimization for complexity-performance trade-off of HOP network | R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AE0164 | EE1-1.2 Complexity-performance tradeoff of decomposition | D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm) |
| JVET-AE0165 | EE1-4.4: Low complexity NN filter with design elements of Unified Filter Architecture and EE1-1.2 and EE1-1.3 | Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm) |
| JVET-AE0191 | AhG11: EE1-0 High Operation Point model | F. Galpin (InterDigital), S. Eadie, D. Rusanovskyy (Qualcomm), Y. Li, J. Li (ByteDance), L. Wang, R. Chang (Tencent), Z. Xie (Oppo), E. Alshina (Huawei) |
| Cross Checks |  |  |
| JVET-AE0183 | Crosscheck of JVET-AE0067 (EE1-4.1: Neural-network loop filters with further complexity reduction) | J. Ström (Ericsson) |
| JVET-AE0229 | Crosscheck of JVET-AE0112(EE1-5.1: Deep Reference Frame Generation for Inter Prediction Enhancement) | X.Jie (OPPO) |

Non-EE Input Contributions

| Reporting |  |  |
| --- | --- | --- |
| JVET-AE0011 | JVET AHG report: Neural network-based video coding (AHG11) | E. Alshina, S. Liu, A. Segall (co chairs), F. Galpin, J. Li, R.-L. Liao, D. Rusanovskyy, T. Shao, M. Wien, P. Wu (vice chairs) |
| JVET-AE0042 | AhG14 & AHG11: Report on AhG teleconference on high operation point (HOP) unified filter training | E. Alshina, F. Galpin |
| JVET-AE0237 | Presentation of AI-based video Coding in "Third AG4 Workshop on JPEG and MPEG Emerging Activities" | E. Alshina |
| Loop Filtering |  |  |
| JVET-AE0072 | [AHG11] On residual adjustments for NNLF | Z. Dai, Y. Yu, H. Yu, D. Wang (OPPO) |
| JVET-AE0093 | AHG11: Content-adaptive neural network loop-filter | R. Yang, M. Santamaria, F. Cricri, H. Zhang, J. Lainema, R. G. Youvalari, M. M. Hannuksela (Nokia) |
| JVET-AE0161 | AHG11: Input and output rotation of model for NNVC in-loop filter | R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AE0171 | AHG11: Neural network-based in-loop filter with layer normalization | Y. Li, K. Zhang, L. Zhang (Bytedance) |
| Post Filtering |  |  |
| JVET-AE0048 | AHG9: Miscellaneous VSEI changes on neural-network post-filter SEI messages | M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia) |
| JVET-AE0049 | AHG9: Miscellaneous VVC changes on neural-network post-filter SEI messages | M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia) |
| JVET-AE0050 | AHG9: On NNPF input picture selection | M. M. Hannuksela, F. Cricri (Nokia) |
| JVET-AE0051 | AHG9: On persistent NNPF activation | M. M. Hannuksela, F. Cricri (Nokia) |
| JVET-AE0052 | AHG9: NNPF cascades and alternatives | M. M. Hannuksela, F. Cricri (Nokia) |
| JVET-AE0053 | AHG8/AHG9: Neural-network post-filter regions SEI message | T. Chujoh, Y. Yasugi, T. Ikai (Sharp) |
| JVET-AE0060 | [AHG9] Comments on NNPFC | S. Deshpande (Sharp) |
| JVET-AE0061 | [AHG9] On NNPFC Application Purpose | S. Deshpande (Sharp) |
| JVET-AE0062 | [AHG9] On NNPF for Deinterlacing | A. Sidiya, S. Deshpande (Sharp) |
| JVET-AE0063 | [AHG9] On grouping and operations for multiple NNPFs | L. Chen, O. Chubach, Y.-W. Huang, S. Lei (MediaTek) |
| JVET-AE0068 | AHG9: On extensibility of purpose syntax element in NNPFC SEI message | Hendry, J. Nam, S. Kim, J. Lim (LGE) |
| JVET-AE0069 | AHG9: On the signalling of output pictures in NNPFA SEI message | Hendry, J. Nam, S. Kim, J. Lim (LGE) |
| JVET-AE0070 | AHG9: On input pictures that are not present in the bitstream for NNPF SEI messages | Hendry, J. Nam, S. Kim, J. Lim (LGE) |
| JVET-AE0101 | [AHG2][AHG9] Neural network post filter and phase indication SEI messages for AVC and HEVC | T. Ikai, T. Chujoh (Sharp), Y.-K. Wang, J. Xu, W. Jia (Bytedance) |
| JVET-AE0106 | AHG9: On missing value ranges for some syntax elements in the NNPFC SEI message | C. Lin, Y.-K. Wang, J. Xu, W. Jia, J. Li, Y. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AE0113 | AHG9: Extendibility and code word length of nnpfc_purpose | R. Sjöberg, M. Pettersson (Ericsson) |
| JVET-AE0126 | AHG9: NNPF cleanup and editorial changes for VSEI | Y.-K. Wang, W. Jia, J. Xu, C. Lin (Bytedance) |
| JVET-AE0127 | AHG9: NNPF editorial changes for VVC | Y.-K. Wang, W. Jia, J. Xu (Bytedance) |
| JVET-AE0128 | AHG9: On NNPFC extensibility and base filter signalling | Y.-K. Wang (Bytedance) |
| JVET-AE0134 | AHG9: Align the design of NNPF with multiple input pictures to NNPF including picture rate upsampling | J. Xu, Y.-K. Wang (Bytedance) |
| JVET-AE0135 | AHG9: On NNPF picture rate upsampling constraints | J. Xu, Y.-K. Wang (Bytedance) |
| JVET-AE0141 | AHG9: Fix a bug in NNPFC SEI message for colourization | J. Xu, Y.-K. Wang (Bytedance) |
| JVET-AE0142 | AHG9: On derivation of NNPF input pictures and the value of nnpfc_purpose | W. Jia, Y.-K. Wang, J. Xu, L. Zhang (Bytedance) |
| JVET-AE0155 | AHG2/AHG9: Editor commentary on the post-filter hint SEI message semantics | G. J. Sullivan, Y.-K. Wang (Editors) |
| JVET-AE0173 | [AHG9] Clarification and improvements of signalling of NNPF update | Y. Lim (Samsung) |
| JVET-AE0175 | [AHG9] Editorial improvements of nnpfc_mode_idc | Y. Lim (Samsung) |
| JVET-AE0187 | AHG9: A summary of SEI proposals on NNPF | Y.-K. Wang (Bytedance) |
| JVET-AE0189 | AHG9: On the design of nnpfa_target_base_flag in NNPFA SEI message | Hendry (LGE) |
| Test Conditions |  |  |
| JVET-AE0162 | AHG11/AHG14: Fix MS-SSIM calculation for SR | R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |
| Cross Checks |  |  |
| JVET-AE0230 | Crosscheck of JVET-AE0162(AHG11/AHG14 : Fix MS-SSIM calculation for SR) | Z. Xie (OPPO) |
| JVET-AE0232 | Crosscheck of JVET-AE0072 ([AHG11] On residual adjustments for NNLF) | T. Shao (OPPO) |

Recommendations

The AHG recommended to:

- Review all input contributions.

- Continue investigating neural network-based video coding tools, including coding performance and complexity.

- Encourage EE1 to continue using a workflow similar to the one in JVET-AE0191 with unified training, clearly documented NNVC technology and transparent training process performed by multiple parties in parallel with regular information sharing.

JVET-AE0012 JVET AHG report: Enhanced compression beyond VVC capability (AHG12) [M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, H. Yang (vice-chairs)]

Activities

The Common Test Conditions were updated (JVET-AD2017). The run time precision in the reporting template has been increased to 1 digit after decimal point. The primary activity of the AHG was the “Exploration experiment on enhanced compression beyond VVC capability” (JVET-ACD024). The combined improvements of the ECM-9.0 over VTM-11.0ecm8.0 anchor for AI, RA and LB configurations are:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.67% | -21.16% | -28.19% | 827.7% | 406.4% |
| Class A2 | -15.77% | -27.44% | -30.55% | 813.4% | 405.9% |
| Class B | -10.06% | -25.61% | -23.82% | 773.3% | 423.4% |
| Class C | -10.27% | -15.86% | -16.33% | 783.1% | 442.4% |
| Class E | -13.65% | -23.55% | -22.06% | 759.1% | 481.4% |
| Overall | -11.59% | -22.66% | -23.71% | 788.5% | 430.8% |
| Class D | -8.61% | -13.65% | -13.74% | 803.2% | 462.1% |
| Class F | -24.34% | -33.43% | -33.28% | 529.7% | 445.8% |
| Class TGM | -37.29% | -45.21% | -44.55% | 443.8% | 443.1% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -20.86% | -24.25% | -32.21% | 757.7% | 741.6% |
| Class A2 | -23.90% | -32.20% | -35.28% | 697.8% | 882.0% |
| Class B | -19.50% | -31.71% | -29.62% | 648.2% | 739.3% |
| Class C | -20.90% | -24.61% | -24.58% | 672.8% | 779.1% |
| Class E |  |  |  |  |  |
| Overall | -21.03% | -28.42% | -29.93% | 685.5% | 777.1% |
| Class D | -21.87% | -25.90% | -26.55% | 699.1% | 849.6% |
| Class F | -26.77% | -34.78% | -35.14% | 578.7% | 528.1% |
| Class TGM | -35.17% | -41.37% | -41.35% | 547.2% | 419.4% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -17.01% | -32.11% | -30.48% | 601.4% | 658.9% |
| Class C | -18.50% | -26.42% | -26.63% | 590.5% | 661.3% |
| Class E | -15.40% | -25.47% | -26.14% | 595.7% | 440.1% |
| Overall | -17.10% | -28.56% | -28.11% | 596.3% | 596.4% |
| Class D | -20.39% | -28.41% | -27.33% | 629.6% | 770.9% |
| Class F | -23.36% | -33.28% | -31.66% | 562.9% | 526.4% |
| Class TGM | -33.93% | -40.53% | -40.11% | 511.1% | 404.8% |

The rate reduction for natural sequences over VTM in RA configuration for {Y, U, V} increased from ECM-8.0’s {-19.86%, -26.50%, -26.67%} to ECM-9.0’s {-21.03%, -28.42%, -29.93%} with the encoder complexity increasing from 601% to 686%. For SCC sequences (class TGM) the rate reduction for RA configuration increased from ECM-8.0’s {-33.27%, -38.66%, -38.66%} to ECM-9.0’s {-35.17%, -41.66%, -41.35%}.

Contributions

In addition to 24 EE2 contributions, 52 (comparing to 55 last meeting) EE2-related and ECM-related contributions were received and can be subdivided as follows:

Intra (19)

- JVET-AE0071, "AHG12: DIMD Merge", S. Blasi, I. Zupancic, J. Lainema (Nokia)

- JVET-AE0073, "Non-EE2: IBC-LIC Model Merge mode", L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO)

- JVET-AE0075, "Non-EE2: On IntraTMP block vector", Y. Yu, L. Zhang, L. Xu, H. Yu, J. Gan, F. Wang, Z. Xie, D. Wang (OPPO)

- JVET-AE0085, "Non-EE2: Direct block vector (DBV) mode extension", M. Hong, J. Choi, N. Park, J. Lim, S. Kim (LGE)

- JVET-AE0098, "AHG12: On the chroma DBV mode", H. Huang, H. Wang, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-AE0110, "Non-EE2: An improved method for IntraTMP fusion", S. Peng, D. Jiang, J. Lin, C. Zheng, K. Fu, P. Zhang (Dahua)

- JVET-AE0120, "Non-EE2: Intra Angular Prediction Extension", J. Fu, J. Zhang, C. Jia, S. Ma (PKU)

- JVET-AE0124, "Non-EE2: Fixes related to Intra TMP", J.-L Lin, P.-H Lin, Y.-J Chang, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-AE0130, "Non-EE2: DIMD with filtered template", C. Zhou, Z. Lv (vivo)

- JVET-AE0137, "Non-EE2: On TIMD fusion", P. Andrivon", M. Blestel (Ofinno)

- JVET-AE0138, "Non-EE2: Reference sample interpolation for intra prediction", G. Rath, F. Le Léannec, F. Urban, F. Racape, T. Dumas (InterDigital)

- JVET-AE0158, "AHG12: LUT-based angle calculation for DIMD", A. Aminlou, J. Lainema (Nokia)

- JVET-AE0166, "AHG12: IntraTMP and IBC search area harmonization", D. Ruiz Coll, B. Chen, P. Andrivon, M. Blestel (Ofinno)

- JVET-AE0167, "AHG12: DBV improvement", L. Xu, Y. Yu, H. Yu, D. Wang (OPPO)

- JVET-AE0184, "AHG12: TIMD with IntraTMP and IBC candidates", K. Naser, F. Le Léannec, P. Bordes, F. Galpin, A. Robert (InterDigital)

- JVET-AE0185, "AHG12: IntraTMP with Neighbouring Candidates", K. Naser, F. Le Léannec, A. Robert, F. Galpin (InterDigital)

- JVET-AE0186, "Non-EE2-2.1: Block Vector Guided Chroma Direct Mode", K. Naser, F. Le Léannec, P. Bordes, Y. Chen, G. Rath (InterDigital)

- JVET-AE0198, "AHG12: Extended Search Region for IntraTMP", K. Naser, P. Bordes, F. Le Léannec, F. Galpin (InterDigital)

- JVET-AE0214, "AHG12: on Intra CIIP", K. Naser, P. Bordes, F. Galpin, K. Reuze (InterDigital)

Inter (11)

- JVET-AE0047, "AHG12: On GPM-MMVD", R. Yu (Ericsson)

- JVET-AE0105, "Non-EE2: Local illumination compensation with multiple templates", Y. Wang, K. Zhang, Y. He, H. Liu, L. Zhang (Bytedance)

- JVET-AE0108, "AHG12: DMVR with robust MV derivation", K. Andersson, R. Yu (Ericsson)

- JVET-AE0109, "Non-EE2: LIC flag derivation of merge candidates with template costs", N. Zhang, K. Zhang, H. Liu, Y. Wang, L. Zhang (Bytedance)

- JVET-AE0117, "Non-EE2: Enhanced subblock-based motion compensation", L. Zhao, K. Zhang, L. Zhang (Bytedance)

- JVET-AE0129, "AHG12 Template-based CIIP weight derivation", M.-H. Jia, Y.-L. Hu, S.-W. Xie, Y. Gao, C. Huang (ZTE)

- JVET-AE0136, "Non-EE2: Fix on TM-based reordering for affine MMVD mode", D. Kim, W. Lim, J. Kim, S.-C. Lim, J. S. Choi (ETRI)

- JVET-AE0140, "Non-EE2: LIC extensions",	A. Filippov, V. Rufitskiy, K. Suverov (Ofinno)

- JVET-AE0148, "Non-EE2: Affine subblock BDOF refinement", Z. Zhang, H. Huang, J.-L Lin, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-AE0168, "Non-EE2: Sample-based BDOF for Chroma", C.-C. Chen, H. Huang, Z. Zhang, C. S. Coban, V. Seregin, M. Karczewicz

- JVET-AE0177, "Non-EE2: Local illumination compensation with slope adjustment", Y. Wang, K. Zhang, Y. He, H. Liu, L. Zhang (Bytedance)

Cross Component Prediction (6)

- JVET-AE0074, "Non-EE2: improvement on cross-component prediction merge mode", H. Huang, Y. Yu, H. Yu, D. Wang (OPPO)

- JVET-AE0097, "AHG12: On the cross-component merge mode", H. Huang, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-AE0115, "Non-EE2: Unified intra CC-models parameters precision", P. Bordes, K. Naser, F. Galpin, F. Leleannec (InterDigital)

- JVET-AE0170, "Non-EE2: Enhancements on CCP merge", K. Zhang, Z. Deng, L. Zhang (Bytedance)

- JVET-AE0176, "EE2-related: Enhancements on CCRM", Z. Deng, K. Zhang, L. Zhang (Bytedance)

- JVET-AE0178, "Non-EE2: Cross-component prediction merge mode for chroma inter coding", M.-S. Chiang, H.-Y. Tseng, C.-M. Tsai, C.-Y. Chuang, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, O. Chubach, Y.-W. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

In-Loop Filters (3)

- JVET-AE0044, "AHG12: Dynamic Scaling of Bilateral Filter (BIF)", V. Shchukin, P. Wennersten, J. Ström (Ericsson)

- JVET-AE0121, "Non-EE2: Luma Residual Taps in Chroma-ALF and CCALF", W. Yin, K. Zhang, Z. Deng, L. Zhang (Bytedance)

- JVET-AE0131, "Non-EE2: Variance based Classification for In-loop Filtering", W. Yin, K. Zhang, L. Zhang (Bytedance)

Entropy Coding (1)

- JVET-AE0058, "AHG12: Spatial CABAC tuning", J. Lainema, A. Aminlou, P. Astola", R. G. Youvalari, D. Bugdayci Sansli (Nokia)

Partitioning (1)

- JVET-AE0082, "AHG12: Non-square quadtree partitioning", Y. Ahn, J. Nam, N. Park, J. Lim, S. Kim (LGE)

GDR (2)

- JVET-AE0145, "AHG 12: Flexible GDR", L. Wang, S. Hong, K. Panusopone (Nokia)

- JVET-AE0146, "AHG 12: Fixes H-CCP Table for CUs in Refreshed Areas of GDR/Recovering Pictures", S. Hong, L. Wang, K. Panusopone (Nokia)

RPR (2)

- JVET-AE0103, "Weighted Edge Enhancement Filtering for Picture Upscaling and RPR", T. ClaÃŸen, M. Wien (RWTH Aachen University)

- JVET-AE0153, "Non-EE2: Enabling template-based inter tools for scaled reference pictures in the RPR", X. Xiu, H.-J. Jhu, C.-W. Kuo, C. Ma, N. Yan, W. Chen, X. Wang (Kwai)

Encoder/CTC (7)

- JVET-AE0055, "AHG12: Sign prediction parameter configuration for low-delay conditions", Y. Yasugi, T. Ikai (Sharp)

- JVET-AE0056, "Non-EE2: Changes on TIMD and SGPM for reducing ECM decoder complexity", Z. Fan, Y. Yasugi, T. Ikai (Sharp)

- JVET-AE0057, "MTT split modes early termination", W. Ahmad, P. Wennersten, K. Andersson (Ericsson)

- JVET-AE0133, "AhG12: On Common Test Conditions change based on EE2 Test 1.1", G. Laroche, P. Onno (Canon)

- JVET-AE0147, "AHG12: Updating context model parameters for ECM", R.-L. Liao, Y. Ye, J. Chen, X. Li (Alibaba)

- JVET-AE0163, "AHG12: modified CTC proposal for low-delay configuration", S. Puri, C. Bonnineau, F. Le Leannec, T. Poirier, E. Francois (InterDigital)

- JVET-AE0180, "On ECM SW memory consumption", R. Chernyak, S. Liu (Tencent), Y. Yasugi, T. Ikai (Sharp)

Recommendations

The AHG recommended to:

- To review all the related contributions.

It was pointed out that the gain gap between luma and chroma became again larger with ECM 9. It was however asserted to be careful shifting gain from chroma to luma without careful study, e.g. in terms of visual impact and rate dependency.

JVET-AE0013 JVET AHG report: Film grain technologies (AHG13) [W. Husak, M. Radosavljević (co-chairs), A. Duenas, D. Grois, Y. He, P. de Lagrange, X. Meng, A. Segall, A. Tourapis, W. Zhang (vice-chairs)]

Discussion

Much of this AHG period, ISO/IEC 23002-9 CDTR was out for comment. The document was issued on May 15. The expected response date was July 9 – just before the 31st Meeting (Geneva). The editors met on a roughly biweekly cadence throughout the period.

The discussions were related to content for film grain, external activities on film grain technologies and additional software being developed for film grain. There are several companies that are independently exploring content. The content is expected to be used for demonstrations and tests. The following characteristics were enumerated for further study:

- video representative of what could be found on streaming platforms

- 10 or 15s length

- consistent grain / no scene cut if possible

  - perhaps cross-fades?

- balanced scene lighting (not everything dark or bright)

- decent resolution and video quality, so that video quality can be rated

- reasonable motion (not too fast, not too blurry, not fully still)

- reasonable spatial content (not empty sky, not packed with excessive texture)

- "normal" grain (not just blurry blobs or blocks, or just scratches)

- color / BW?

The group agreed the content should include a “ground truth”. However, it was recognized that the “de-noising” process could be controversial. Several content repositories were identified that could be a source of open-source content. In addition, some companies are exploring providing content they own and can assign the rights.

Two applications SDOs have opened or are exploring study items/missions on film grain. 3GPP has started a study item that is scheduled to be completed at the end of 2023. DVB is also exploring opening a study mission on film grain. There is also an expectation that other SDOs will follow suit and explore film grain. The group discussed potential support for those groups.

There were two industry demonstrations with film grain during the AHG period. The first was at Mile High Video 2003 (MHV2003) and the second was at DVB World. The second supported the launch and discussion of the DVB study mission.

Finally, several additional software packages were discussed. Several software packages were updated and some new features were added, such as an autoregressive parameter translator.

Related contributions

Three contribution related to AHG13 was identified as of 07/10/2023.

- The one contribution was the AHG report:

  - JVET-AE0013 JVET AHG report: Film grain technologies (AHG13)

- Two other contributions were added but at present without an abstract

  - JVET-AE0250 Film Grain Synthesis: Material Selection for Ground Truth Content Creation

  - JVET-AE0251 Evaluating Denoising methods in the context of Film Grain Analysis/Synthesis

Contributions

There were no contributions uploaded as of the report time other than the AHG report.

Recommendations

The AHG recommended that:

- the related input contributions are reviewed;

- testing of FGC be discussed; and

- continue the study of film grain technologies in JVET.

It was pointed out that the software packages mentioned could be useful in the visual assessment activity.

BoG (W. Husak) to develop the DoCR (ballot response in m63723), and define which changes will be made in draft 5 relative to draft 4 (draft 5 will also be submitted as WG 5 DTR).

Potential viewing regarding the aspects of JVET-AE0250 and JVET-AE0251.

JVET-AE0014 JVET AHG report: NNVC software development (AHG14) [S. Eadie, F. Galpin, Y. Li, J. Shingala, L. Wang, Z. Xie (AHG chairs)]

Software development

Location

NNVC repository is located at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM

NNVC software is based on VTM-11.0 with enabled MCTF including the update from JVET-V0056, GOP32, and enabling deblocking in the RDO.

NNVC-5.1 anchor at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM is used for NNVC performance evaluation (Note: NNVC-5.0 should produce the same results for anchor generation).

Software changes

The following changes were integrated:

- Integration of JVET-AD0107 – simplified parameter selection filterset 1

- Integration of JVET-AD0156 – Low complexity filterset

- Integration of JVET-AD0380 – High Operation Point Filterset

- Integration of JVET-AD0163 – content adaptive NN post-filter

- Integration of JVET-AD0164 – sadl v5 and v5.1

- Integration of JVET-AC0089 Use qp slice for inter slcie

- Various fixes (21 issues resolved)

Other changes:

- New configurations files for anchors

- Improvement of the data loader for training

- Improvement of training dataset preparation scripts

- Improvement of training dataset creation scripts

Software version

- NNVC-5.1 was tagged xxx, 2023.

- NNVC-5.0 was tagged May 11th, 2023.

- NNVC-3.0 (a.k.a VTM-11.0_nnvc3.0) was tagged December 1st 2022.

- NCS-1.0 (a.k.a NNVC-3.0wip2) was tagged September 4th 2022 (first release containing the FilterSets, using NNVC 2.0 as a base).

- VTM-11.0_nnvc-2.0 was tagged August 4th 2022 (add deblocking in RDO).

- VTM-11.0_nnvc-1.0 was tagged May 6th 2021 (VTM-11.0 base with MCTF enabled).

CTC performance

See the configurations section for naming convention.

NNVC-4.0 vs NNVC-5.0 VTM mode

NNVC-5.0 in VTM mode performance are the same as the ones in NNVC-4.0 VTM.

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 VTM anchor |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 92% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 93% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 94% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 96% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 94% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 95% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 93% |

|  | All Intra Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 VTM anchor |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 93% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 93% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 95% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 95% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 94% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 98% |

|  | Low delay B Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 VTM anchor |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | – | – | – | – | – | – | – | – |
| Class A2 | – | – | – | – | – | – | – | – |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 92% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 94% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 86% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 91% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 95% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 102% | 91% |

NNVC-4.0 vs NNVC-5.0 anchor

The NNVC-5.0 anchor included LC filter and Intra Prediction tools activated.

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc4.0_VTM-anchor |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -7.23% | -4.86% | -6.00% | -8.72% | -4.05% | -5.06% | 134% | 7667% |
| Class A2 | -6.56% | -5.78% | -5.09% | -6.53% | -4.14% | -2.90% | 130% | 7183% |
| Class B | -6.29% | -8.19% | -7.53% | -5.97% | -6.06% | -5.18% | 132% | 7834% |
| Class C | -6.62% | -10.53% | -9.55% | -6.26% | -7.91% | -5.58% | 132% | 7422% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -6.62% | -7.66% | -7.28% | -6.71% | -5.77% | -4.81% | 132% | 7557% |
| Class D | -7.57% | -9.45% | -9.68% | -5.76% | -6.81% | -5.94% | 129% | 8193% |
| Class F | -3.07% | -5.88% | -4.94% | -3.05% | -4.76% | -3.54% | 148% | 9739% |

|  | Low delay B Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc4.0_VTM-anchor |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -4.85% | -8.13% | -7.84% | -5.15% | -6.16% | -6.65% | 134% | 6812% |
| Class C | -5.24% | -11.13% | -9.00% | -5.61% | -10.74% | -4.02% | 126% | 6233% |
| Class E | -4.88% | -4.09% | -5.17% | -5.64% | -2.18% | -3.36% | 162% | 9877% |
| Overall | -4.98% | -8.12% | -7.56% | -5.42% | -6.69% | -4.95% | 138% | 7257% |
| Class D | -6.54% | -10.76% | -9.77% | -5.75% | -9.48% | -6.83% | 125% | 6400% |
| Class F | -2.05% | -4.88% | -4.46% | -2.29% | -3.08% | -4.12% | 148% | 8941% |

|  | All Intra Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc4.0_VTM-anchor |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -8.46% | -7.23% | -8.67% | -10.00% | -8.00% | -7.50% | 210% | 5134% |
| Class A2 | -6.61% | -8.04% | -7.94% | -6.90% | -6.44% | -5.44% | 204% | 4416% |
| Class B | -7.03% | -8.58% | -8.71% | -7.04% | -7.61% | -6.99% | 203% | 4493% |
| Class C | -7.34% | -9.76% | -9.92% | -7.61% | -8.38% | -7.64% | 190% | 3770% |
| Class E | -10.29% | -9.04% | -8.85% | -10.14% | -7.52% | -6.38% | 199% | 5149% |
| Overall | -7.81% | -8.60% | -8.87% | -8.16% | -7.64% | -6.86% | 201% | 4507% |
| Class D | -7.57% | -8.11% | -9.33% | -7.16% | -6.81% | -7.56% | 180% | 3760% |
| Class F | -4.55% | -5.97% | -5.65% | -4.24% | -4.75% | -4.38% | 155% | 3961% |

NNVC-5.0 VTM mode vs NNVC-5.1 VTM mode + HOP (HOP only)

These early results are obtained using model of epoch 18 of the HOP training process in float.

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 VTM anchor |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.65% | -18.14% | -22.49% | 100.00% | 100.00% | 100.00% | – | – |
| Class A2 | -10.08% | -22.08% | -17.55% | 100.00% | 100.00% | 100.00% | – | – |
| Class B | -8.92% | -23.34% | -22.97% | 100.00% | 100.00% | 100.00% | – | – |
| Class C | -9.65% | -24.98% | -24.39% | 100.00% | 100.00% | 100.00% | – | – |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -9.49% | -22.48% | -22.17% | 100.00% | 100.00% | 100.00% | – | – |
| Class D | -10.89% | -24.02% | -24.54% | 100.00% | 100.00% | 100.00% | – | – |
| Class F | -5.30% | -16.51% | -14.82% | 100.00% | 100.00% | 100.00% | – | – |

|  | All Intra Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -7.04% | -17.89% | -20.06% | 100.00% | 100.00% | 100.00% | – | – |
| Class A2 | -6.97% | -19.25% | -16.77% | 100.00% | 100.00% | 100.00% | – | – |
| Class B | -7.00% | -18.24% | -19.89% | 100.00% | 100.00% | 100.00% | – | – |
| Class C | -7.94% | -18.64% | -21.26% | 100.00% | 100.00% | 100.00% | – | – |
| Class E | -10.39% | -20.44% | -21.57% | 100.00% | 100.00% | 100.00% | – | – |
| Overall | -7.77% | -18.81% | -19.98% | 100.00% | 100.00% | 100.00% | – | – |
| Class D | -7.83% | -17.73% | -21.23% | 100.00% | 100.00% | 100.00% | – | – |
| Class F | -5.53% | -16.45% | -16.17% | 100.00% | 100.00% | 100.00% | – | – |

NNVC-5.0 anchor vs NNVC-5.1 HOP

These early results are obtained using model of epoch 18 of the HOP training process in float.

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 anchor |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | – | – | – | 100.00% | 100.00% | 100.00% | – | – |
| Class A2 | – | – | – | 100.00% | 100.00% | 100.00% | – | – |
| Class B | -4.32% | -17.27% | -17.62% | 100.00% | 100.00% | 100.00% | – | – |
| Class C | -4.65% | -16.79% | -17.31% | 100.00% | 100.00% | 100.00% | – | – |
| Class E |  |  |  |  |  |  |  |  |
| Overall | – | – | – | 100.00% | 100.00% | 100.00% | – | – |
| Class D | -4.82% | -16.61% | -17.22% | 100.00% | 100.00% | 100.00% | – | – |
| Class F | -3.33% | -12.04% | -10.99% | 100.00% | 100.00% | 100.00% | – | – |
|  | All Intra Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-5.0 anchor |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | – | – | – | 100.00% | 100.00% | 100.00% | – | – |
| Class A2 | – | – | – | 100.00% | 100.00% | 100.00% | – | – |
| Class B | -2.88% | -13.90% | -15.16% | 100.00% | 100.00% | 100.00% | – | – |
| Class C | -3.50% | -12.91% | -15.35% | 100.00% | 100.00% | 100.00% | – | – |
| Class E | -4.40% | -16.48% | -17.90% | 100.00% | 100.00% | 100.00% | – | – |
| Overall | – | – | – | 100.00% | 100.00% | 100.00% | – | – |
| Class D | -2.90% | -13.35% | -15.66% | 100.00% | 100.00% | 100.00% | – | – |
| Class F | – | – | – | 100.00% | 100.00% | 100.00% | – | – |

NNVC-5.1 HOP final

TBA

NNVC-5.1 VTM mode with Post-filter

Results are extracted from JVET-AD0163.

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 NNPF OFF |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -6.40% | -13.70% | -20.30% | -8.39% | -16.51% | -21.17% | 171% | 22100% |
| Class A2 | -4.22% | -18.38% | -13.60% | -3.45% | -18.07% | -11.95% | 165% | 20486% |
| Class B | -5.13% | -20.59% | -16.95% | -3.25% | -20.82% | -16.74% | 170% | 23102% |
| Class C | -3.59% | -18.92% | -15.99% | -2.40% | -17.62% | -15.89% | 149% | 22203% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -4.79% | -18.32% | -16.69% | -4.09% | -18.56% | -16.44% | 164% | 22119% |
| Class D | -3.60% | -18.30% | -17.35% | 0.28% | -16.57% | -15.64% | 154% | 22471% |
| Class F | -1.42% | -13.15% | -10.64% | -1.24% | -14.56% | -12.42% | 215% | 14088% |

Discussions

Previous discussions

The following points were discussed or are still discussed in the group:

- SOLVED: Official configuration files for NNVC software and official configuration files for EE1 anchors.

- SOLVED: To enforce adopted contributions to provide training scripts to train a model from scratch

- SOLVED A MR (MR68) merged.

New discussions

- Final training strategy for HOP model is still work in progress.

- Some aspects of encoder optimization (eg RDO models) are not yet aligned with HOP model.

Configurations

The following configurations is used to generate the different NNVC results.

The column “tested” is read as follow:

- Y: the configuration has been tested using the new NNVC-5.0 software

- P: the results are the ones from previous NNVC software basis

- N: not tested.

The column “xcheck” is read as follow:

- Y: the test has been cross-checked

- P: no cross-checked performed but results are consistent with previous version on NNVC

- N: no cross-check available

| Name | Tools | Configuration | Tested | Xcheck |
| --- | --- | --- | --- | --- |
| NNVC-5.0 VTM mode | none | encoder_xxx_vtm.cfg | Y | Y |
| NNVC 5.0 Anchor/EE1 | Intra Pred + LC filterset | encoder_xxx_nnvc.cfg | Y | Y |
| NNVC-5.1. HOP | Intra Pred + HOP | encoder_xxx_nnvc.cfg + nn-based/HOP.cfg | P |  |
| HOP only | HOP | encoder_xxx_vtm.cfg + nn-based/HOP.cfg | Y |  |
| set0 | Loop filter set #0 | encoder_xxx_vtm.cfg + nn-based/NnlfOption_1.cfg | P | Y |
| set1 | Loop filter set #1 | encoder_xxx_vtm.cfg + nn-based/NnlfOption_2.cfg | P | Y |
| Intra | Intra prediction | nn-based/encoder_xxx_nnvc.cfg | P | Y |
| set0+intra | Loop filter set #0+ Intra prediction | nn-based/encoder_xxx_nnvc.cfg + nn-based/NnlfOption_1.cfg | P | Y |
| set1+intra | Loop filter set #1+ Intra prediction | nn-based/encoder_xxx_nnvc.cfg + nn-based/NnlfOption_2.cfg | P | Y |
| set0+rdo+intra | Loop filter set #0 + Rdo + Intra prediction | nn-based/encoder_xxx_nnvc.cfg + nn-based/NnlfOption_1.cfg + --EncNnlfOpt=1 | P | Y |
| set1+rdo+intra+temporal filter | Loop filter set #1 + Rdo + Intra prediction + Temporal filter | nn-based/encoder_xxx_nnvc.cfg + nn-based/NnlfOption_2.cfg + --EncNnlfOpt=1 + --NnlfSet1Multiframe=1 | P | Y |
| Sr | Super-resolution | encoder_xxx_vtm.cfg + nn-based/nnsr.cfg + nn-based/nnsr_classAx.cfg | P | N |
| set0+sr | Loop filter set #0+Super-resolution | encoder_xxx_vtm.cfg + nn-based/NnlfOption_1.cfg + nn-based/nnsr.cfg + nn-based/nnsr_classAx.cfg | N | N |
| set1+sr | Loop filter set #1+Super-resolution | encoder_xxx_vtm.cfg + nn-based/NnlfOption_2.cfg + nn-based/nnsr.cfg + nn-based/nnsr_classAx.cfg | N | N |
| sr+intra | Super-resolution +Intra prediction | nn-based/encoder_xxx_nnvc.cfg + nn-based/nnsr.cfg + nn-based/nnsr_classAx.cfg | P | Y |
| set0+sr+intra | Loop filter set #0+Super-resolution+Intra prediction | nn-based/encoder_xxx_nnvc.cfg + nn-based/NnlfOption_1.cfg + nn-based/nnsr.cfg + nn-based/nnsr_classAx.cfg | N | N |
| set1+sr+intra | Loop filter set #01+Super-resolution+Intra prediction | nn-based/encoder_xxx_nnvc.cfg + nn-based/NnlfOption_2.cfg + nn-based/nnsr.cfg + nn-based/nnsr_classAx.cfg | N | N |
| Pf | Adaptive post-filters | encoder_xxx_vtm.cfg + nn-based/nnpf_int16.cfg | P | P |
| Intra+pf | Intra prediction + adaptive post-filter | nn-based/encoder_xxx_nnvc.cfg + nn-based/nnpf_int16.cfg | P | N |

Note: current HOP results used early model of stage2/epoch18.

Recommendations

The AHG recommended to:

- Continue to develop NNVC software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/issues

- Encourage people to submit merge requests fixing identified bugs.

It was asked whether the low-operation-point filter also used three-stage training before it was adopted in the last meeting. Yes, it was confirmed that it did.

# Project development (19)

## AHG1: Deployment and advertisement of standards (1)

Contributions in this area were discussed at 1620–1630 on Sunday 16 July 2023 (chaired by JRO).

JVET-AE0154 MC-IF VVC technical guidelines [L. Litwic (Ericsson), S. McCarthy (Dolby), S. Wenger (Tencent), J. Ridge (Nokia), B. Bross (Fraunhofer HHI), D. Rusanovskyy (Qualcomm), Alan Stein (InterDigital)]

This input contribution provides an update on the Media Coding Industry Forum (MC-IF) and ongoing work on VVC technical guidelines. Since the launch of the VVC standard (standardized by ITU-T as Recommendation H.266 and in ISO and IEC as International Standard 23090-3 (MPEG-I Part 3)), several application-oriented standards developing organizations and industry fora worldwide have been examining inclusion of VVC-based profiles and corresponding receiver capabilities. The technical guidelines, currently developed in MC-IF as an informative resource to implementors, developers and service providers, aim to:

- cover best practices of VVC configuration in terms of functionality and compression performance for industry relevant VVC-based profiles and interoperability points,

- provide guidance and up to date information on VVC operating bitrate ranges for selected applications,

- provide information on the usage of VVC with accompanying technologies e.g., such as Versatile Supplemental Enhancement Information messages,

- advocate interoperability and seek commonality of VVC usage across transport and application layer standards and relevant industry guidelines.

The initial scope of the technical guidelines document is limited to VVC usage in video broadcast and streaming applications.

## AHG2: Text development and errata reporting (1)

Contributions in this area were discussed at 1635–1650 on Sunday 16 July 2023 (chaired by JRO).

JVET-AE0155 Editor commentary on the post-filter hint SEI message semantics [G. J. Sullivan, Y.-K. Wang (Editors)]

This contribution was addressed by the BoG reported in JVET-AE0272 on SEI messages for VSEI v3 and VVC v3.

The authors provided this contribution to address issues encountered in the specification of the post-filter hint SEI message during the finalization editing of WG 5 N 179 (FDIS ISO/IEC 23008-2 5th edition) and JVET-AD2006 (Additional SEI messages for VSEI (Draft 4)) after consideration of item 4 of JVET-AD0078 (AHG9: On comments for the draft of VSEI [T. Chujoh, Y. Yasugi, T. Ikai (Sharp)]).

- The semantics do not say that the described post-filtering should be applied.

In discussion, it was noted that some other similar messages (e.g. film grain and colour remapping) are also not saying they should be applied (but tone mapping uses “should” and NNPF uses “may”).

- It does not seem clear whether the “16-bit precision” description applies only to the cross-correlation matrix case or also to the filter coefficient cases.

It was commented that the first version of this SEI message was in the 2009 edition [actually, see the H.264 2007-04 Amendment 2 pre-publication and H.264 2007-11 edition] of AVC and the original proposal was from Thomas Wedi and Steffen Wittmann of Panasonic. A couple of changes were introduced in the 2010 edition [H.264 2010-03] which may have introduced problems [it corrected a missing value range, although apparently more generously than necessary, and tried to clarify the scaling of the cross-correlation matrix data but accidentally basically scaled down by 216 rather than scaling up by that factor]. In the JVET plenary review of the first version of the BoG, it was commented that a JVT contribution JVT-U035 contained a software showcase that might help clarify the intent. Other contributions before that were JVT-S030 and JVT-T039.

- A specified 32-bit range may be excessive for a syntax element that has 16-bit precision (see remarks on previous aspect for the history on this).

- Much of the text is written from the encoder perspective. We generally do not specify things from the encoder perspective. The decoder-side processing is not clearly described or is discussed only vaguely. For example, no equations are given for the decoder’s filtering process.

- It does not seem clear whether the decoder should divide the filter tap values (or equivalently, the result of the filtering process) by the sum of the filter tap values or if the encoder is responsible for the filter normalization.

- There are issues relating to how to interpret the filter depending on whether the filter length is even (in which case the output samples may presumably be located spatially between input samples) or odd (in which case the location of an output sample may presumably have a location corresponding to the centre tap).

- The handling of the boundaries of the picture and cropping window are not described and seems unclear.

- It seems unclear why should it matter whether a horizontal filter or a vertical filter is applied first if both are to be applied. Perhaps integer rounding is expected to be applied between the two filtering operations, but this is not stated.

- The discussion of Wiener post-filtering is a bit vague (e.g., what exactly is a Wiener filter, and how would a decoder derive one from the provided data?).

- A decoder could use a subset of a provided cross-correlation matrix to derive a Wiener post-filter with a smaller region of support than the provided statistics would enable. It does not seem clear whether this is intended to be an acceptable or possible practice.

The contribution was written to point out the issues and does not contain an exact recommendation about what to do about them.

BoG recommendation (action considering text specification quality): Do not include this SEI message in VVC v3 and VSEI v3 due to its apparent lack of clarity.

Further study was recommended for future clarification/action.

It was discussed whether the film grain and colour remapping and NNPF SEI messages should say they “should” be used.

Another potential phrasing was “is suggested” rather than “should”.

The BoG initially recommended, as a clarification of existing intent, to recommend that “should” be used for the film grain and NNPF SEI messages; but not to say this for colour remapping (since that has an ID value that could indicate whether it is appropriate or not appropriate for a particular decoding system to apply, e.g. depending on the target display). In the JVET plenary review of the first version of the BoG report, there was discussion of the meaning of “should” and the potential implications of using that word (and of changing a draft to introduce it). Discussed alternatives included “is suggested” and “is intended”. Some participants suggested that application standards are the appropriate place to provide guidance on usage and are customized to the application environment. Film grain synthesis was suggested to be an especially important functionality to encourage.

Tone mapping was another message to consider about phrasing. It is not in VSEI, but in AVC and HEVC it uses “should”.

JVET decision: Use “should” for film grain synthesis and retain prior drafted language for other SEI messages.

Further study toward some profile / conformance expression to include film grain synthesis support was encouraged.

JVET decision: Include issues from JVET-AE0155 in JVET-AE1004.

A correction can later be implemented in the next AVC edition and HEVC amendment/edition.

For VSEI, more evidence might be needed if the old post-filter hint SEI message has any use together with VVC, which has more powerful loop filters, such that its benefit might be limited.

## AHG3: Test conditions (0)

This section is kept as a template for future use.

## AHG3: Software development (1)

Contributions in this area were discussed at 1540–1600 on Sunday 16 July 2023 (chaired by JRO).

JVET-AE0181 Scaling window support for VTM [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

This contribution is a follow-up contribution of JVET-AD0101 on scaling window support for VTM. Scaling window functionality is reported to be useful for content layering application which is one of the effective use cases of multilayer coding. However, the latest version of VTM does not yet support the scaling window. To enable the evaluation of scaling windows functionality, this contribution proposes to integrate implementation of scaling window into VTM. The merge request was submitted to VTM repository as a draft implementation as MR!2635. In revision 3, the experiments with QP ranging from 22 to 37 are conducted and BD-rate saving {Y: -36.0%, U: -50.3%, V: -50.2%} was observed.

Scaling window is a functionality that is supported with VVC, but currently there is no encoder part in the VTM. The software coming with the contribution only supports static position of the scaling, whereas generally each picture might have its own scaling window position and size.

It was asked whether something like that could be done at the systems layer, which might be more efficient in a streaming application.

Decision (SW/non-CTC): Adopt JVET-AE0181.

JVET-AE0275 Cross-check of JVET-AE0181 (Scaling window support for VTM) [F. Urban (InterDigital)] [late]

## AHG4: Subjective quality testing and verification testing (5)

Contributions in this area were discussed at 1825–2000 on Monday 17 July 2023 (chaired by JRO and MW, joint meeting with AG 5).

JVET-AE0041 AHG4: Report on AhG meeting on verification testing for VVC multilayer coding [M. Wien (AhG4 co-chair)]

There was no need for presentation of this.

JVET-AE0219 AHG4: Results of visual checking of SVVC VT streams with and without DMVR fix [M. Wien, K. Andersson, P. de Lagrange (test coordinators)]

This document reports observations made on bitstreams prepared for the verification tests for dual-layer VVC coding comparing single-layer to a spatially scalable representation with 1.5× and 2× scaling ratio between base and enhancement layer.

The bitstreams were encoded using the VTM at commit “b6a20aa7d1” (interim status after VTM-20.2) by Ericsson and exchanged via the JVET ftp site. The encoder configuration follows the Verification Test plan document JVET-AD2021 with the DMVR encoder parameter DMVREncMvSelect switched on and switched off, respectively.

The goal of the visual checking is to inspect the reconstructed video for the visual impact and potential issues with the modified encoder configuration in both the single-layer and the dual-layer configurations.

The effect is observed at QP points >32, which are substantially used for visual testing.

It was agreed to use the version with the fix in the tests, for all single layer, upsampled base layer and multi-layer.

JVET-AE0227 AHG4: experiments in preparation of scalable quality ladder visual testing [P. de Lagrange, F. Urban (InterDigital)] [late]

This document reports about some experiments in preparation of VVC multilayer verification tests planned in JVET-AD2021, especially the 2nd test category involving a quality ladder, and more specifically the definition of the anchor. Encodings at candidate resolutions and overlapping bit rates were performed, PSNR and VMAF metrics of upscaled decoded pictures are reported, along with visual checks to estimate expected scores and to compare resolutions at comparable bitrate. The intent is to provide data to help define rate points and resolutions of the above-mentioned anchor.

There was a large variety of material, not all visually assessed yet.

Informal viewing for rough pre-selection was planned for Tuesday.

JVET-AE0285 AHG4: Preparation for content layering functional test [S. Iwamura, D. Arai, S. Nemoto, A. Ichigaya (NHK)] [late]

This contribution reports preparation of content layering functional test to be conducted in 31th JVET meeting at Geneva.

A block diagram of the equipment for the test is illustrated in the figure below. A multilayer bitstream encapsulated in CMAF over MMT is output from the streamer and a VVC decoder supporting multilayer decodes the bitstream in real time. Viewers can select which contents (main or sub) to play out under control of a tablet PC. VVC multilayer real-time decoder and CMAF/MMT streamer, signal converter for the functional test are prepared by NHK and a 4K monitor was provided by ITU-T.

Figure: Equipment for the functional test

Specifications of multilayer VVC decoder

| Profile | Main 10, Multilayer Main 10 |
| --- | --- |
| Level | 6.3 |
| Number of supported layers | 4 layers |
| Input Format | CMAF (ISO/IEC 23000-19) on MMT (ISO/IEC 23008-1) |
| Output Format | Resolution: 1080p, 2160p, 4320p Frame rate [Hz]: 59.94, 60, 119.88, 120 |

A demo was planned for Wednesday 17 July.

JVET-AE0288 Results of expert viewing for the spatial scalability category of the VVC multilayer VT [M. Wien (RWTH Aachen University), K. Andersson (Ericsson), P. de Lagrange (InterDigital)]

This document reports the results of an on-site expert viewing performed at the 31st JVET meeting in Geneva, CH.

The evaluation targets at the spatial scalability category of the test plan document JVET-AD2021, comparing the compression performance of dual layer (DL) coding with a downscaled base layer at scaling ratio 2 to single layer coding at the low resolution with upscaling to the target resolution using the VVC interpolation filters (UP) as well as single layer coding at the target resolution (SL).

A total of 29 experts participated in the expert viewing. Based on this larger database, the results reveal small confidence intervals and can reportedly be considered quite reliable.

With respect to the goals of this experiment, the following observations were made:

- DL shows a slight benefit over UP for DrivingPOV3 (overlapping confidence intervals), very minor for MountainBay2.

- No benefit or slight loss for DL compared to UP for Procession.

- More loss for DL compared to UP for CorsaireJoy, Marathon2, and, partially significant, for TallBuilding2.

- Relation of SL, DL, UP somewhat according to expectations on MountainBay2 (SL gaining at high rates, while DL, UP gaining at low rates.

- ISSUE observed for TallBuildings2: UP better than SL and DL for lower two rate points. Possible reason: less pumping artefacts for UP streams which are coded at a lower QP. Better detail of SL or DL does not compensate for this issue

- ISSUE observed for CorsaireJoy: UP rated better, although more detail is present in DL version. Inspection indicates that DL version also shows partially more visible artefacts.

The observed issues appear to be related to suboptimum bit allocation in the SL and DL configurations. It is recommended to take these results into account when setting up the set of four rate points for a formal evaluation.

It was asked whether the bit rates might be too low for demonstrating the benefit of coding at full resolution (both for SL and dual layer).

It was commented that potentially a DCR (degradation category rating) test is not most suitable to see the benefit of higher detail.

It was suggested to potentially look at intra frames in terms of quality and bit rate allocation between the different cases.

It was asked whether it would be possible to combine this analysis with test results from Antalya to conclude whether there are more cases where the upsampled low-res is consistently better than single layer.

Are the rate points appropriately selected? Does it make sense to go to very low quality with scalable coding? Is it possible to find a perform tests over a wide range of rates/qualities at same full resolution and get consistent results?

Statistics about rate allocation to base/enhancement should also be taken into consideration.

An informal viewing session was planned for Wednesday for interested participants.

## AHG4: Test material (1)

Contributions in this area were discussed at 1600–1610 on Sunday 16 July 2023 (chaired by JRO).

JVET-AE0179 AHG4: Renewed license statement for Ghost Town Fly and Undo Dancer 3D video test sequences [M. M. Hannuksela, N. Salonen (Nokia)] [late]

In this document, Nokia provides a renewed license statement for Ghost Town Fly and Undo Dancer 3D video test sequences. The license statement allows the use of the test sequences for developing, testing and promulgating standards by ITU-T/ISO/IEC.

## AHG4: Quality assessment methodology (1)

Contributions in this area were discussed at 1530–1540 on Sunday 16 July 2023 (chaired by JRO).

JVET-AE0092 [AHG4] Occupancy-only PSNR calculations for V3C V-PCC coding evaluation [S. Schwarz, M. M. Hannuksela (Nokia)]

This contribution discusses occupancy-only PSNR calculations to simplify V3C coding efficiency evaluation. Objective evaluations resulted in correlation coefficients of 0.9 and higher between the proposed evaluation metric and established end-to-end metrics. Therefore, it is proposed to recommend the use of occupancy-only PSNR in V3C V-PCC coding evaluations provided to JVET.

The metric is currently not used by MPEG WG 7 (3D Graphics and Haptics Coding), but is being proposed there as well. The benefit of the metric is that it is only assessing the quality of the video texture and geometry information, but has high correlation with end-to-end metrics.

It was asked if the benefit of the ECM over the VTM (using it as a codec for V-PCC) was again evaluated. This was not the case. Further study was requested on this.

Regarding the metric, JVET should not define own metrics in a domain that falls under the competence of WG 7. If WG 7 adopts the metric, BD results based on that metric would be welcome for this kind of data.

## AHG5: Conformance test development (1)

Contributions in this area were discussed at 1610–1620 on Sunday 16 July 2023 (chaired by JRO).

JVET-AE0111 Additional conformance tests of spatial scalability for multilayer coding [C. Salmon-Legagneur, P. de Lagrange, F. Urban (InterDigital)]

This contribution proposes an additional conformance test of spatial scalability functionality for multilayer coding. This bitstream exercises VVC with a multilayer configuration, and particularly interlayer-prediction from the first picture, and multiple reference frames together with inter-layer prediction scalability. The proposed conformance bitstreams, generated with VTM 20.2 are attached in the contribution.

Decision: Include the additional bitstream in an output JVET-AE2028.

JVET-AE0286 Crosscheck of JVET-AE0111 (Additional conformance tests of spatial scalability for multilayer coding) [S. Iwamura] [late]

## AHG7: ECM tool assessment (2)

Contributions in this area were discussed at 1200–1240 on Sunday 16 July 2023 (chaired by JRO).

JVET-AE0174 AHG7: On TM control for non-inter tools [Z. Deng, K. Zhang, L. Zhang (Bytedance)]

This proposal aims at solving an identified issue reported by AHG7. It is asserted that template costs are calculated in several IBC and intra techniques, but there is no option to turn them off correctly in the ECM. In ECM-9.0, three issues are observed. Firstly, ARMC of serval IBC techniques are erroneously turned off by the inter TM SPS flag, which messed up the tool evaluation results of inter TM tools (group 1) and non-inter TM tools (group 3). Secondly, the same IBC ARMC aspects are erroneously turned on even if they are required to be turned off in group 3 off test. Thirdly, the template aspect of LBCCP is not disabled in group 3 off test. This contribution proposes to introduce an SPS control flag to solve such identified issues in the ECM. The proposed changes were implemented on top of AHG7 base software. As compared to the AHG7 tool off tests, the simulation results are summarized as follows:

- Group 1 off test with the proposed change

|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class F | -0.29% | -0.19% | -0.15% | 101.0% | 99.9% |  |  |  |  |  |
| Class TGM | -1.09% | -1.17% | -1.12% | 102.3% | 104.5% |  |  |  |  |  |

- Group 3 off test with the proposed change

|  | All Intra Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Overall | 0.03% | 0.13% | 0.13% | 99.4% | 99.9% |  |  |  |  |  |
| Class F | 1.61% | 1.41% | 1.35% | 92.6% | 79.0% | 0.97% | 0.81% | 0.78% | 96.3% | 96.1% |
| Class TGM | 3.08% | 3.00% | 3.00% | 93.4% | 75.8% | 2.41% | 2.65% | 2.92% | 98.9% | 98.2% |

An additional flag is requested. It is asserted that this is the only practical way to configure the intented tool-off test appropriately. Furthermore, in this exploration, flags are rather a practical way to manage exploration of tools, rather than in a real standardization effort where they ar an important part of HLS. It is noted than ECM already has many flags for controlling tools that might not be used in a practical standard.

Decision (SW): Adopt JVET-AE0174, flag to control non-inter TM tools.

JVET-AE0212 Cross-check of JVET-AE0174 On TM control for non-inter tools [X. Li (Google)] [late]

JVET-AE0256 Crosscheck of JVET-AE0174 (On TM control for non-inter tools) [H. Wang (Qualcomm)] [late]

JVET-AE0195 AhG7: ECM stages complexity assessment [F. Galpin, F. Le Léannec, C. Salmon-Legagneur, E. François (InterDigital)] [late]

This contribution contains information about the current complexity of ECM-9.0 on some sample sequences in comparison with previously developed reference software. The contribution presents a more detailed view of the complexity evolution by showing the complexity at different stages, namely “parsing” (decoding the bins), “decoding” (computing the values of a CU), “prediction” (computing the prediction), “residuals” (computing the residuals), “loop-filters” (deblocking, SAO, ALF, etc.). Results are shown on HM-16.10, JEM-7.0, VTM 19.2 and ECM 9.0.

The following analysis is based on instructions independent on implementation (CPU, SIMD, etc.)

The figures below show the complexity ratio during the decoding. As expected, the decoding part increased more for the ECM.

Figure: Decoding complexity ratios

Figure: Evolution of complexity per stage

The chart above shows the evolution of the complexity across the software.

It was not fully clear during the discussion what “other” is – list construction, CABAC init, etc. It is interesting to see that this has a relatively large amount in the explorations.

This contribution was submitted for information – no action was needed, further study was welcome.

## AHG8: Optimization of encoders and receiving systems for machine analysis of coded video content (5)

Contributions in this area were discussed at 1230–1300 and 1845–2100 on Saturday 15 July 2023 (chaired by JRO).

### Non-SEI contributions

JVET-AE0081 [AHG8] De-noising filter as pre-processing for machine task C. Kim, D. Gwak, J. Lim (LGE)

This contribution reports the machine analysis performance of applying a de-noising filter as a pre-processing step. The Gaussian blur function consisting of 3x3 and 5x5 kernels is exploited to reduce the level of noise. Experimental results show that de-noising filtering on input video can give following changes.

- SFU-HW: -14.07%, -17.39%, -18.63% for RA, LD and AI respectively (Pareto-mAP)

- TVD: -18.75%, -22.30% for RA and AI respectively (Pareto-mAP)

Preprocessing was applied to entire frames.

Also the combination with JVET-AD0122 was presented (changing QP at high temporal layers), showing additional gain.

It was asked whether the preprocessing already improves the performance. This was not known. If it has better MAP, this would be OK, otherwise it would be necessary to apply less aggressive blurring at high rates.

It was pointed out that the blurring might not be appropriate for any machine vision task, e.g. license plate detection.

In the current CTC, there was an obvious advantage, but it may not generalize to any machine application.

Decision: It was agreed that this should be included in the next version of the draft technical report, but it should be made clear that it is not advised to be used generally on the frame level. It was also commented that de-noising approaches based on Gaussian filters (e.g., bilateral filter, anisotropic diffusion filters) exist which preserve local detail while removing noise.

JVET-AE0221 Crosscheck of JVET-AE0081 ([AHG8] De-noising filter as pre-processing for machine task) [C. Hollmann (Ericsson)] [late]

JVET-AE0096 [AHG8] Study on using different VTM versions [C. Hollmann, J. Ström (Ericsson)]

In this contribution the results of a study on different VTM versions in the context of machine analysis of coded video content are reported. Eight different versions of VTM from VTM-13 to VTM-20 are compared to the VTM-12 anchor.

It was reported that even though the variation in PSNR between different VTM version is rather small, the variation in mAP/MOTA performance is in some cases substantial.

It was proposed to replace the current VTM-12 anchor with a more recent one such as VTM-20.

Extreme fluctuations between subsequent VTM versions were noted, in particular diverging per sequence. In general, newer VTMs are better than VTM11 on average.

A significant improvement was detected at VTM 16->17, where the QP offsets over temporal levels was adjusted. Also MCTF modifications may have an impact.

Results indicate that the metric of BD-rate based on MAP is quite sensitive. With divergent behaviour between sequences as reaction on relatively small encoder changes, this might make the average numbers computed out of them somewhat random.

A 6-point BD rate with Pareto correction was used.

It was also commented that MAP is very low in some cases – should be considered to update the machine analysis algorithms used to determine MAP.

It was suggested to change the anchor from VTM12 to VTM20. It was later agreed to put the VTM20 anchor as an additional anchor into the CTC.

JVET-AE0099 [AHG8] NNPF and post-filter hint SEI messages for the technical report [C. Hollmann, M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

In this contribution new subsections for the draft technical report on optimizations for encoders and receiving systems for machine analysis of coded video content are proposed. The new subsections describe the neural-network post-filter SEI messages and the post-filter hint SEI message.

It was pointed out that no software implementation for post-filter hint exists (only for AVC, but also not in official JM). Further, it is questionable whether it would have much effect in particular for VVC, where ALF (which has more adaptation capabilities) would take over that role.

It was noted that JVET-AE0143 is about a related software implementation.

Decision: It was agreed to add NNPF SEI messages to the TR as an element of VSEI useful for machine analysis.

JVET-AE0107 AHG8: Improvements of the BD-rate model using monotonic curve-fitting method [H. Wang, X. Pan, Z. Liu, X. Xu, S. Liu (Tencent)]

This contribution provides improvements of the BD-rate model used in current Excel templates. Specifically, the contribution proposes a monotonic fitted curve to replace the PCHIP (Piecewise Cubic Hermite Interpolation) curve for BD-rate and BD-metric calculation, such as BD-PSNR, BD-mAP, BD-MOTA, etc. The fitted curve guarantees monotonicity even if the input points are nonmonotonic, thus makes it possible to calculate BD-rate and BD-metric values in various testing conditions.

It was pointed out that a curve that is convex would be more appropriate than an S-shaped model one as proposed here. RD curves should be convex in an optimum case, as well as ROC curves in classification.

It was asked whether the non-convex behaviour at the low end might also be coming from a deficiency in the machine analysis, e.g. overtraining.

It was pointed out that generating more bit rate points could be beneficial for curve fitting.

Bit rate is plotted on a log axis.

It is generally agreed that some kind of improved curve fitting is a good way to resolve the problem of finding a better metric.

Further discussion with WG 4 was suggested.

JVET-AE0234 AHG8: Crosscheck of AE-0107 [H. Zhang (Nokia)] [late]

JVET-AE0143 AHG8: A spatial resampling algorithm and an exemplar software implementation [S. Wang, B. Li, J. Chen, Y. Ye (Alibaba), S. Wang (CityU)]

Sections on using spatial resampling to improve coding efficiency for video coding for machines have been included in JVET-AD2030 “Optimization of encoders and receiving systems for machine analysis of coded video content (draft 2)” at the conceptual level, but without providing any detail. This contribution proposes a spatial resampling algorithm and a spatial resampling software implementation for inclusion in JVET-AD2030. The proposed algorithm can achieve 6.95%, 6.56% and 5.78% average Pareto BD-rate improvements for random access (RA), all intra (AI) and low delay (LD) configurations for SFU-HW dataset under the common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content as defined in JVET-AD2031. The proposed method is only activated for larger resolution sequences, where the gains can reach as high as 13.16% and 64.65%.

Results above come from 5 sequences with resolution HD and higher (two of which give results in last sentence), but average is computed over all. The benefit is highly different per sequence, which may again be due to the metric. This is called “option 2”, which gives bit rate reduction for 4 out of 5 sequences. Another set of results called “option 1” applies the network at the decoder end only for 2 out of the 5 sequences.

Sequences with resolution less than HD were not downsampled. The pre-analysis network is performing an analysis whether downsampling is done, which is then done by a subsequent network at the encoder side. This decision can be adjusted by a threshold (options 1 and 2 use different thresholds)

Overhead for the decoder-side upsampling network is roughly 42 Kbyte for NNPF, static model.

Pre-analysis model at encoder is also static.

Some gain is probably due to rate reduction by coding a downsampled video. However, the bit rate is significantly lower than for the anchor, and are only partially overlapping.

The main benefit is that comparable MAP can be achieved as with anchor with significantly lower rate.

It was asked whether also adaptation of the network would be possible.

It was also asked how the decoder-side upsampling compares against NN based SR approaches.

Candidate for adoption in SW codebase and exampole implementation annex of TR

Crosschecks were requested. It was also requested to show the RD graphs of all 5 HD sequences.

RD graphs were shown on Tuesday 18 July at 11:00. The only case with clear benefit was Cactus. Otherwise, RD graphs are either overlapping in a rather small range, and also crossing in that range. The proposed method goes into much lower bit rates than the anchor, but also the MAP is very bad in that range.

Further study was recommended. This was agreed to be included for further experimentation in an AHG8 software repository.

JVET-AE0293 Cross-check of JVET-AE0143: [AHG8] A spatial resampling algorithm and an exemplar software implementation [Z. Huang (PKU)] [late]

Partial results were reported on Tuesday 18 July: AI for the two lowest QP points.

It was commented that availability of RA results (at least partial) would be relevant.

JVET-AE0297 Crosscheck of JVET-AE0143 ([AHG8] A spatial resampling algorithm and an exemplar software implementation) [C. Hollmann (Ericsson)] [late]

### Contributions related to SEI messages (see section 6.2):

JVET-AE0053 AHG8/AHG9: Neural-network post-filter regions SEI message [T. Chujoh, Y. Yasugi, T. Ikai (Sharp)]

JVET-AE0064 AHG8/AHG9: Signalling encoder preprocessing and human / machine viewing indications [C. Kim, D. Gwak, Hendry, J. Lim, S. Kim (LGE), M. M. Hannuksela, F. Cricri, H. Zhang (Nokia)]

JVET-AE0079 AHG8/AHG9: Source picture timing information SEI message [S. McCarthy, G. J. Sullivan, P. Yin (Dolby)]

JVET-AE0090 AHG8/AHG9: On machine vision indication [J. Gao, H.-B. Teo, C.-S. Lim, K. Abe (Panasonic)]

JVET-AE0095 AHG8/AHG9: Proposed changes to the candidate new object mask information SEI message [P. de Lagrange, E. François, D. Doyen (InterDigital), J. Chen, S. Wang, Y. Ye (Alibaba)] [late]

## AHG10: Encoding algorithm optimization (4)

Contributions in this area were discussed at 1240–1300 and 1430–1530 on Sunday 16 July 2023 (chaired by JRO).

JVET-AE0104 AHG10: GOP-based RPR encoder control using parallel resolution encoding [D. Arai, S. Nemoto, S. Iwamura, A. Ichigaya (NHK)]

This contribution aims to improve the performance of JVET-AB0080 in controlling the encoder for GOP-based Reference Picture Resampling (RPR). The GOP-based selection method utilizes parallel resolution encoding for Rate-Distortion (RD) optimization. This method performs multi-pass encoding on the downscaled pictures and then calculates the RD cost for each resolution to determine the optimal resolution in each GOP.

The BD-rate performance (Y/Cb/Cr) compared to CTC with VTM-20.2 as anchor is as follows:

- RA: -0.65%/1.52%/0.48%

- LDB: -0.06%/-0.12%/-0.33%

- AI: -0.21%/4.83%/2.23%

This method is especially beneficial for 4K resolution in the RA configuration (-2.24%/-0.18%/-2.90% for ClassA1, -1.14%/7.42%/5.36% for ClassA2).

The method also improves over current GOP-based RPR (except for AI, where gains are experienced in some classes); this also comes with a run time increase (50% in RA, 86% in LB).

It was noted that in AI this is not using RPR in the loop, it is using VVC interpolation filters for upsampling.

As RD optimization is used per RPR including I picture, parallel encoding of GOPs in RA is not possible.

It was commented that weights of 8:1:1 for RD cost calculation may not be appropriate.

Further study was recommended along the questions asked above.

JVET-AE0277 Cross-check of JVET-AE0104 (AHG10: GOP-based RPR encoder control using parallel resolution encoding) [K. Andersson (Ericsson)] [late]

JVET-AE0114 AHG10: Low-Delay configuration improvements [J. Enhorn, P. Wennersten (Ericsson)]

This document proposes changes to CTC low-delay configurations including Intra QP offset, periodic QP offsets every 64th picture, and changes to GOP structure and reference picture lists.

Reported results on CTC on top of VTM and ECM are as follows:

- VTM LD-B Y/U/V: -5.62%/-10.34%/-11.24%

- VTM LD-P Y/U/V: -4.94%/-9.76%/-10.58%

- ECM LD-B Y/U/V: -4.49%/-9.08%/-9.47% (estimated)

- ECM LD-P Y/U/V: -3.57%/-7.94%/-8.29%

The basic idea is providing different qualities to the lowest layer of the B picture hierarchy in low delay in a periodic fashion, plus increasing quality of the I picture in the the beginning.

Proposed structure (current and proposed):

Are there visual artefacts? “Less than expected”

How much benefit from the three aspects (intra, boost, GOP)? Roughly 1/3 each.

It was pointed out that the variations between sequences are non-negligible, some sequences have losses.

It was commented that this increases the bit rate variations at picture level, which is undesirable in low delay applications.

It was also commented that averaging MSE over pictures (rather than averaging PSNR) should be investigated.

Further study with configurations of less variation, impact on visual quality, investigation on bit rate variation, benefit of the three elements.

JVET-AE0283 Cross check of JVET-AE0114 (AHG10: Low-Delay configuration improvements) [S. Hong (Nokia)] [late]

JVET-AE0122 AHG10: GOP-based RPR and scalable coding [K. Andersson (Ericsson)]

The purpose with the proposal is to provide a fix so that GOP-based RPR works together with scalable coding in the VVC reference software. The fix consists of some updates to the PPS usage in VTM for GOP-based RPR and RPR in general. It is also suggested to include layer-wise control of GOP-based RPR such that it for example only can be used in the base layer. The contribution also provides compression performance of scalable coding with and without GOP-based RPR in the base layer for the case of 1.5x scalability for some candidate VT sequences.

Objective performance (BDR) versus VTM-20.2+ commit 85ea835777eb:

Impact of GOP-based RPR in base layer for 1.5x: Y -0.07%, U -0.65%, V -0.84%

It was recommended to adopt the fix for GOP-based RPR such that RPR also can be used for scalable coding.

This was a desirable bug fix to avoid encoder crashes in this uncommon combination.

Decision (BF/SW): Adopt JVET-AE0122.

JVET-AE0284 Crosscheck of JVET-AE0122 (AHG10: GOP-based RPR and scalable coding) [D. Arai, S. Nemoto, S. Iwamura, A. Ichigaya (NHK)] [late]

JVET-AE0123 AHG10: Lagrange multiplier optimization for chroma ALF and CCALF [S.-W. Xie, Y. Gao, M.-H. Jia, Y.-L. Hu, C. Huang, P. Wu (ZTE)]

This contribution reports further results for the RDO method in ALF, which was proposed in JVET-AD0136. Regarding the proposed method, the chroma Lagrange multiplier will be adjusted by a scaling factor when the luma ALF APS transmission flag is ON and the number of history chroma ALF APS is insufficient. With VTM20.0 as anchor and test platform, the reported overall PSNR-Y,U,V BD-rate and {EncT, DecT} variations of this proposal are as follows:

- LDB: 0.05%, -0.80%, -1.18% {100%, 100%}

- LDP: 0.02%, -0.89%, -1.03% {100%, 101%}

As a general policy, encoder optimizations of this kind should equally be applied to the VTM and ECM. In the last meeting, it had specifically been requested to improve the performance of the ECM (where apparently losses had been observed, according to the JVET-AD1000 meeting notes. It was verbally reported that the ECM performance would be similar with the new proposal, but no results were provided.

For VTM, the results look beneficial, chroma gains outweigh the small luma losses.

It was pointed out that for ECM it might be necessary to adjust the multiplier differently.

Further study was encouraged on implementing the method on top of the ECM.

JVET-AE0262 Crosscheck of JVET-AE0123 (AHG10: Lagrange multiplier optimization for chroma ALF and CCALF) [H. Zhang (Tencent)] [late]

## AHG13: Film grain synthesis (2)

Contributions in this area were discussed in the BoG reported in JVET-AE0294, and at 1445 on Saturday 15 July 2023 (chaired by JRO).

JVET-AE0250 Film Grain Synthesis: Material Selection for Ground Truth Content Creation [D. Podborski, D. Ugur, B. Williams, L. Levinson, R. Molholm, A. M. Tourapis (Apple)] [late]

This document introduces some of the selected material to be used for the creation of new film grain content based on the “ground truth” method presented in contribution JVET-AD0369. The material consists of either newly captured content or scenes taken from the OpenMovie project “Tears of Steel”. An initial description of the material is included in this document.

JVET-AE0251 Evaluating Denoising methods in the context of Film Grain Analysis/Synthesis [D. Podborski, D. Ugur, A. M. Tourapis (Apple)] [late]

This contribution proposes a methodology for evaluating the performance and appropriateness of different video denoising techniques in the context of film grain modelling/synthesis for video compression. The results from performance such an evaluation could potentially be utilized for selecting a denoising system for further work on film grain modelling/synthesis within JVET or other standardization groups within MPEG or elsewhere.

JVET-AE0294 Film Grain BOG Report [W. Husak (BoG coordinator)]

The results of first BoG meeting were presented in JVET on Saturday 15 July at 1445, as shown below.

Elements needed included a Denoiser, FG Analyser, and FG Synthesizer. With the GT approach, the denoiser could in the first place be skipped, and it could be investigated whether the artistic film grain comes with the content could be equivalently generated with the FGS SEI. This would not be fully realistic, but denoiser and analysis could be added later.

Another application is coding artefact hiding by dithering. In that case, an objective metric is more urgently needed.

It was mentioned that in practice also quant matrices are often used together with film grain (example x.265?).

The GT data set could be used in the subjective quality testing of SEI for category 2, and also for category 1 -> viewing if possible on Wednesday, to be further discussed in AG 5 and at an AHG intermediate meeting.

The DoCR was to be produced by a responsible editor (W. Husak taking the lead). With regard to the comment on external references, it should be responded that it is anticipated that the reference document will become publicly available before the ballot ends, and filling it in will be purely editorial in publication.

Decision (BF): W.r.t. the reported error in equation 28 / figure 18 of VSEI (and correspondingly in AVC/HEVC), it was agreed to integrate the following change in VSEI v3, the AVC CD, and the HEVC PDAM:

Figure 18 in the DTR also needs similar consideration.

## Implementation studies (2)

Contributions in this area were discussed at 1630–1640 on Sunday 16 July 2023 and 1400–1410 on Monday 17 July 2023 (chaired by JRO).

JVET-AE0172 Update on multilayer coding support for VVenC [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

This contribution reports an information update on multilayer coding support for VVenC, which is firstly reported in JVET-AB0086, where the initial support of multilayer coding was implemented on VVenC-1.4.0. The base software was updated to VVenC-1.6.1 and the latest results of VVenC-multilayer over VTM-20.2 is reported as follows:

- Faster		BD-rate: {36.69%, 33.07%, 35.95%}, Encoder speedup:170.8x

- Fast		BD-rate: {29.02%, 24.08%, 29.52%}, Encoder speedup: 94.2x

- Medium	BD-rate: {10.59%, 10.54%, 10.37%}, Encoder speedup: 35.4x

- Slow		BD-rate: {5.07%, 4.16%, 3.25%}, Encoder speedup: 10.5x

- Slower		BD-rate: {0.52%, 0.29%, -0.98%}, Encoder speedup: 1.4x

The latest version of multilayer support on top of VVenC-1.6.1 is available at GitHub managed by NHK (https://github.com/nhkrd/vvenc.git).

JVET-AE0266 Update on open, optimized VVC implementations VVenC and VVdeC [A. Wieckowski, J. Brandenburg, C. Bartnik, V. George, J. Güther, G. Hege, C. Helmrich, A. Henkel, T. Hinz, C. Lehmann, C. Stoffers, B. Bross, H. Schwarz, D. Marpe, T. Schierl (HHI)] [late]

This document provides updated information on features, coding efficiency and runtime for version 1.9.0-rc2 of the open VVC software encoder VVenC released in July 2023 and version 2.1.0 of the open VVC software decoder VVdeC released in July 2023.

The main changes for VVenC v1.9.0-rc2 since version 1.8.0 include:

- New features and improvements:

  - Rate capping for rate control encoding (capping at minimum 1.5x target rate),

  - First pass speedups by the means of temporal and spatial subsampling (latter is experimental),

  - Initial, experimental support for frames-in-flight parallelism.

- Various bugfixes and cleanups.

Without QP adaptation for subjective optimization and 8 threads the following PSNR-based YUV BD-rates compared to HM-17.0 (GOP16+MCTF) as well as speedup factors compared to HM-17.0 and VTM-19.2 (GOP32+MCTF) are reported for different presets and JVET class B (HD), class A (UHD) as well as both (HD4K) test sequences:

| Preset | HD |  |  | UHD |  |  | HD4K |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PSNRYUV BD-rate vs. HM-17.0 | Speedup vs. HM-17.0 | Speedup vs. VTM-19.2 | PSNRYUV BD-rate vs. HM-17.0 | Speedup vs. HM-17.0 | Speedup vs. VTM-19.2 | PSNRYUV BD-rate vs. HM-17.0 | Speedup vs. HM-17.0 | Speedup vs. VTM-19.2 |
| FASTER | -11.% | 340x | 2200x | -15.2% | 410x | 2500x | -13.3% | 380x | 2400x |
| FAST | -24.8% | 140x | 940x | -26.7% | 160x | 1000x | -25.9% | 150x | 970x |
| MEDIUM | -34.2% | 30x | 200x | -36.7% | 46x | 280x | -35.6% | 38x | 240x |
| SLOW | -38.4% | 11x | 70x | -40.1% | 16x | 95x | -39.3% | 13x | 83x |
| SLOWER | -41.4% | 2.3x | 15x | -43.1% | 3.5x | 21x | -42.3% | 2.9x | 18x |

With QP adaptation for subjective optimization and 8 threads, the following MS-SSIM-based YUV BD-rates compared to HM-17.0 (GOP16+MCTF) as well as speedup factors compared to HM-17.0 and VTM-19.2 (GOP32+MCTF) are reported for different presets:

| Preset | HD |  |  | UHD |  |  | HD4K |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MS-SSIMYUV BD-rate vs. HM-17.0 | Speedup vs. HM-17.0 | Speedup vs. VTM-19.2 | MS-SSIMYUV BD-rate vs. HM-17.0 | Speedup vs. HM-17.0 | Speedup vs. VTM-19.2 | MS-SSIMYUV BD-rate vs. HM-17.0 | Speedup vs. HM-17.0 | Speedup vs. VTM-19.2 |
| FASTER | -20.3% | 310x | 2100x | -20.0% | 370x | 2500x | -20.1% | 340x | 2300x |
| FAST | -31.6% | 130x | 930x | -31.0% | 150x | 1100x | -31.3% | 150x | 1000x |
| MEDIUM | -38.4% | 29x | 200x | -40.1% | 43x | 300x | -39.3% | 36x | 250x |
| SLOW | -42.0% | 10x | 71x | -43.4% | 15x | 100x | -42.8% | 13x | 87x |
| SLOWER | -44.4% | 2.2x | 15x | -46.2% | 3.2x | 22x | -45.4% | 2.7x | 19x |

Two versions of VVdeC (v2.0.0 and v2.1.0) had been released since v1.6.1 (reported in JVET-AD0266) with minor fixes, speedups and improvements. Two most important new features include initial error resilience and initial native support for ARMv8 architecture.

## Profile/tier/level specification (2)

Contributions in this area were discussed at 1005 on Tuesday 18 July 2023 (chaired by JRO).

JVET-AE0295 [AHG5] HEVC Multiview Profiles: Implementation Status [S. Paluri, D. Podborksi, A. Tourapis (Apple)] [late]

This contribution aims to provide an update on the implementation status of the new HEVC Multiview profiles described in JVET-AA0239. An implementation in the HM reference software for the Multiview Main 10 profile is provided in this contribution. This implementation was verified with internal production level SW and HW. An implementation for the other Multiview profiles that are specified in JVET-AA0239 is still in progress.

Bitstreams and software are included in the contribution. KDDI and Dolby had been performing crosschecks, and verbally confirmed that everything is correct.

This is asserted to be sufficient evidence that a software implementation exists, and conformance testing is being developed.

JVET-AE0296 HEVC Multiview profiles supporting extended bit depth (Draft 2) [S. Paluri, A. Tourapis (Apple), W. Husak (Dolby)] [late]

This document contains the draft text for changes to the High Efficiency Video Coding (HEVC) standard (ITU-T H.265 | ISO/IEC 23008-2), for the addition of new multiview profiles supporting extended bitdepth.

Draft 2 incorporated items:

- Addition of new multiview profiles supporting extended bitdepth (JVET-AA0239)

- Correction in the profile definitions

A typo in Table F.3 “monochrome10”->”monochrome12” shall be corrected.

Monochrome profile is intended to be used for coding of depth maps, which sometimes require more precision than the video.

It was suggested to also include a Multiview Monochrome 10 profile, as it can be expected that more encoder implementations exist that would support that.

Decision: Adopt JVET-AE0296 with an additional Multiview Monochrome 10 profile (see typo correction above).

## Proposed modification of system interface (0)

This section is kept as a template for future use.

## General aspects of standards development and applications of standards (0)

This section is kept as a template for future use.

# Low-level tool technology proposals

## AHG11/AHG14: Neural network-based video coding (17)

### Summary, BoG reports, and information documents (3)

Contributions in this area were discussed at 1430–1540 on Wednesday 12 July 2023 (chaired by JRO).

JVET-AE0023 EE1: Summary report of exploration experiment on neural network-based video coding [E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang, Z. Xie (EE coordinators)]

This report summarizes the activities of the Exploration Experiment 1 (EE1) performed between the JVET-AD and JVET-AE meetings to evaluate Neural Network-based Video Coding (NNVC) technologies, analyze their performance, evaluate their complexity aspects, and clarify training procedure. Two teleconferences had been conducted during this meeting cycle.

In total seventeen test been planned for this EE1 round, but only seven were completed. Major reason for the delay is unified filter architecture for high operation point (HOP) and unified training procedure (both greed at JVET-AD meeting and documented in the JVET-AD0380 and JVET-AD2023) implemented from scratch and trained in parallel by multiple companies with careful cross-check of all intermediate steps and performance tracking. In addition, many unfinished EE tests are supposed to be conducted on top of the HOP model, which was not fully ready yet.

EE1 conducted two teleconferences jointly with AhG11 and AhG14 to discuss training of HOP unified filter (EE1-0). The teleconferences were held on June 14 and 28, 2023 and were attended by approximately 20 participants.

In the JVET-AD meeting, a low complexity operation point NN-based filter was agreed to be default configuration of NNVC-5.0. Also it was agreed to enable NN-Intra by default in NNVC-5.0 anchor. EE1 tests participant targeting for adoption to Common SW base NNVC are requested to report performance relatively to NNVC-5.0 default configuration.

In order to track NNVC work progress, results relative to VVC (NNVC-5.0 with all tools disabled) are shown in the next figure below.

Figure: Complexity performance trade-off of each NNVC tool in common SW base (old filters shown for the last time).

Unified Filter Training

EE1-0 Agreed unified filter design for High Operation Point

JVET-AE0191 AhG11: EE1-0 High Operation Point model [ F. Galpin (InterDigital), S. Eadie, D. Rusanovskyy (Qualcomm), Y. Li, J. Li (ByteDance), L. Wang, R. Chang (Tencent), Z. Xie (Oppo), E. Alshina (Huawei)]

Figure: Unified filter architecture (HOP).

Unified filter architecture (EE1.0) was documented in JVET-AD2023 and is shown in the figure above. Computational complexity: 477 kMAC/pxl; Total Number of parameters: 1.45 M;

Unified training procedure of EE1.0 was also documented in JVET-AD2023 and consisted of following stages

- Training Model stage I (Intra model)

  - All Intra coding of DIV2K data coding with vanilla VTM (NN Tools off)

  - Training data dumping and DIV2K unified database creation size of 2.6TB

  - Model stage I training from scratch for Intra data (40 epochs, 10 days)

- Training Model stage II (Intra & Inter model)

  - Random Access coding of BVI and TVD data with NNVC5.0 (NN Tools off, HOP Stage I model enabled for Intra Slices)

  - Training data dumping, BVI and TVD data unified databases creation size of 1.5TB

  - Model stage II training with DIV2K, BVI and TVD databases (20 epochs, 10 days)

  - Testing (5 days)

- Training stage III (Intra & Inter model)

  - Random Access coding of BVI and TVD data with NNVC5.0 (NN Tools off, HOP Stage II model enabled for Intra and B slices)

  - Training data dumping, BVI and TVD data unified databases creation size of 1.5TB

  - Model stage III training with DIV2K, BVI and TVD databases (20 epochs, 10 days)

It is noted that collaborative development of HOP filter (EE1-0) allows achieving significant improvement for the complexity-performance trade-off of the NN-based filter at the high operation point.

| HOP filters: past and future | Parameters, M | kMAC /pixel | Random Access vs VVC |  |  | All Intra vs VVC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | Cb | Cr | Y | Cb | Cr |
| EE1-0 (Stage I) | 1.5 | 477 | -4.2% | -17.4% | -16.2% | -7.8% | -19.2% | -20.2% |
| EE1-0 (Stage II) | 1.5 | 477 | -9.5% | -22.4% | -22.1% | -7.8% | -18.8% | -20.0% |
| filter set#0 (fully trained) | 1.9 | 615 | -9.8% | -21.1% | -20.5% | -7.5% | -16.8% | -17.4% |
| filter set#1 (fully trained) | 3.1 | 673 | -9.6% | -20.9% | -21.5% | -7.4% | -18.3% | -19.9% |

Only after two (out of three) stages training unified HOP filter (EE1-0) achieves almost the same performance with ste#0/set#1 filters in NNVC 4.0 with more than 20% lower computational complexity. Roughly 1% gain is expected after completion Stage II training (expected to finish during JVET-AE meeting).

Filter architecture EE1.0 and unified dataset and training process are recommended for adoption to the next version of NNVC common SW base.

New finding during joint training of HOP are recommended to be presented with details (JVET-AE0191).

NOTE: “Skip connection” missed in HOP filter diagram, shall be added to diagrams as depicted on slide #6 of attached presentation.

Unified Filter Architectural changes

EE1-1.2 Complexity-performance tradeoff of decomposition

JVET-AE0164 EE1-1.2 Complexity-performance tradeoff of decomposition [D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm)]

Figure: Back Bone Block: (above) EE1-0 design, all 24 BBB sare the same; (below) proposed design: every odd BBBs are T1, even BBBs are T2.

Proposed modification for unified filter architecture is shown in the figure above. Type 1 and Type 2 Back Bone Blocks feature reduced complexity, comparing to the anchor BBB, and are alternating (T1 and T2) in order. The model was trained following HOP training procedure and test set. Results for Stage1 have been provided, training for latter stages of HOP is ongoing, results to be provided once available.

Computational complexity: 426 kMAC/pxl (11% reduction compared to EE1-0); Total Number of parameters: 1.3M;

In all Intra cfg performance difference compare to EE1-0 (after same stage of training) is 0.0% (Y), 0.2%(Cb), 0.1%(Cr).

Suggestion: review in the BOG to plan study in EE1, e.g. provide results with 3 stages training, perform training cross-check.

EE1-1.5 Optimization for complexity-performance trade-off of network

JVET-AE0160 EE1-1.5: Optimization for complexity-performance trade-off of HOP network [R. Chang, L. Wang, X. Xu, S. Liu (Tencent)]

The proposed modification for the unified filter architecture is shown in the next figure below.

Figure: Proposed split Luma – Chroma filter architecture. Key changes: split architecture for luma and chroma compared to HOP architecture.

Three variants of the residual block are proposed. They are shown in the next figure below.

Figure: Proposed variants of Residual Block.

Based on filter0 training scripts and dataset, retrain the EE1-0 residual block and two improved residual blocks in Test 1, and retrain the EE1-0 architecture and split architecture in Test 2. Compared to the EE1-0 residual block, 11% kMAC/pixel reduction with 0.1% luma gain loss for RA. Compared to the EE1-0 architecture, 34% kMAC/pixel reduction with 0.5% luma gain loss for RA.

| Test | Parameters, M | kMAC /pixel | Random Access vs NNVC-5.0 anchor |  |  | All Intra vs NNVC-5.0 anchor |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | Cb | Cr | Y | Cb | Cr |
| NNIntra & EE1-1.5.1-subtest 1 | 1.4 | 461 | -5.1% | -14.6% | -14.5% | -3.1% | -12.4% | -13.5% |
| NNIntra & EE1-1.5.1-subtest 2 | 1.2 | 402 | -4.8% | -14.9% | -14.1% | -2.8% | -12.0% | -13.0% |
| NNIntra & EE1-1.5.1-subtest 3 | 1.3 | 408 | -5.0% | -14.2% | -13.8% | -2.9% | -11.8% | -13.0% |
| NNIntra & EE1-1.5.2-subtest 1 | 1.5 | 480 | -3.6% | -12.1% | -13.7% | -2.3% | -11.4% | -12.2% |
| NNIntra & EE1-1.5.2-subtest 2 | 0.9 | 316 | -3.1% | -11.6% | -11.5% | -2.0% | -10.6% | -11.5% |

| Test | Subtest | Architecture | Backbone block | Training strategy | Training set | Interface | kMAC/ pxl |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EE1-0 (HOP) |  | HOP | HOP resblock | HOP | HOP | HOP | 477 |
| EE1-1.5.1 | subtest1 (*) | Filter set#0 | Same as HOP resblock | Filter set#0 | Filter set#0 | Same as HOP | 461 |
|  | subtest2 |  | HOP resblock with depth-wise separable |  |  |  | 408 |
|  | subtest3 |  | HOP resblock with group convolution |  |  |  | 402 |
| EE1-1.5.2 | subtest1 (**) | Same as HOP | Same as HOP resblock |  |  |  | 480 |
|  | subtest2 | HOP with Split |  |  |  |  | 316 |

It was suggested to review this in the BOG to plan study in EE1, e.g. utilizing HOP unified dataset and training process.

Training set modification

No specific study was conducted in this category. Test EE1-1.5 uses different training data and training strategy.

Unified filter usage aspects

No tests were possible at this point, since the HOP filter was not yet available.

It was recommended to conduct scheduled tests in the next EE round.

Improvement for Low complexity NN-filter

EE1-4.1 Neural-network loop filters with further complexity reduction

JVET-AE0067 EE1-4.1: Neural-network loop filters with further complexity reduction [J. N. Shingala, A. Shyam, A. Suneja, S. Badya (Ittiam), T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, Sean McCarthy (Dolby)]

Filter architecture of low operation point (LOP) filter in NNVC-5.0 is shown in the next figure below. Computational complexity is 16 kMAX/pxl. Number of parameters 0.21 M.

Figure: Low Operation Point (LOP) filter in NNVC-5.0.

Proposed modification for LOP are shown in the next figure below. Parameters of the architecture are following: number of input feature sets M=64, K=24, R=24. Computational complexity is 10 kMAX/pxl. Number of parameters 0.13 M.

Figure: Proposed modification for LOP filter.

Complexity reduced drastically, performance degradation is 0.3...0.4% (performance gain of LOP filter is ~5%). Only a floating point model is provided. Performance test results are summarized in the table below.

Performance of low-complexity filter EE1-4.1 in comparison with LOP filter in NNVC-5.0

| Test | Random Access vs VVC |  |  | All Intra vs VVC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | Cb | Cr | Y | Cb | Cr |
| NNIntra & EE1-4.1 | -6.2% | -10.9% | -9.4% | -7.5% | -11.0% | -10.8% |
| Anchor(NNIntra & LOP NNVC-5.0) | -6.6% | -7.7% | -7.3% | -7.8% | -8.6% | -8.9% |
| NNIntra | -1.8% | -0.5% | -0.9% | -3.6% | -3.2% | -3.3% |
| LOP NNVC5.0 | -5.2% | -7.5% | -6.6% | -4.6% | -5.7% | -5.7% |

Suggestion: to continue study in EE1, provide quantized model and cross-check training.

EE1-4.4 Low complexity NN filter with design elements of UF and EE1-1.2 and EE1-1.3

JVET-AE0165 EE1-4.4: Low complexity NN filter with design elements of Unified Filter Architecture and EE1-1.2 and EE1-1.3 [Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm) (Dolby)]

In this contribution, Unified Filter Architecture of HOP was adjusted to meet complexity constraints of LOP. This was achieved by reducing complexity of the headblock, backbone, and tailblock and utilizing the CP4 decomposition in the backbone blocks. The model was trained End-to-End following HOP training procedure and test set. Results for Stage1 have been provided, training for latter stages of HOP is ongoing, results to be provided once available.

Training for latter stages of HOP is ongoing, results to be provided once available. Proposed filter, comparing to the EE1.0 anchor is shown in the following figure (upper: EE1-0 filter, below: proposed low complexity filter).

Achieved model complexity in kMAC/pixel: 14.8 (frame-wise); 16.7 (block-wise of block 256x256); 18.7 (block-wise of block 128x128), total amount of parameters is 0.05 M (compared to 1.45 for HOP).

Low complexity filter following concept of unified HOP.

Performance test results are available for all intra configuration only, shown in the table below. The training procedure follows EE1 recommendation, so results are easily comparable with the rest of EE1 tests.

Performance of low-complexity filter EE1-4.4 in comparison with LOP filter in NNVC-5.0

| Test | Random Access vs VVC |  |  | All Intra vs VVC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | Cb | Cr | Y | Cb | Cr |
| NNIntra & EE1-4.4 |  |  |  | -7.7% | -13.6% | -14.3% |
| Anchor (NNIntra & LOP NNVC-5.0) | -6.6% | -7.7% | -7.3% | -7.8% | -8.6% | -8.9% |

The channels number in some point of the processing pipeline have been selected to match complexity/number of channels of the LOP anchor. Due to this, they are multiple of 16 (even not multiple of 8), which could be sub-optimal for some implementations.

Suggestion: review in the BOG to plan study in EE1, e.g for architecture optimization, provide results with HOP 3 stages training, perform training cross-check.

EE1-4.1 and EE1-4.4 still require training cross-check. This should be performed in the next round of EE. It was pointed out that proponents might think about a unification with the target of an improved low operation point filter, as this might speed up the process of training cross-check, and invoke a similar approach of multiple training stages as was done with the high operation point. If the proposals are kept independently, an equivalent training method should be used. See the BoG report JVET-AE0282.

NN-based Inter

EE1-5.1 NN-based inter prediction with lower complexity

JVET-AE0112 EE1-5.1: Deep Reference Frame Generation for Inter Prediction Enhancement [ W. Bao, W. Meng, J. Jia, Y. Zhang, H. Wang, Z. Chen (Wuhan Univ.), Z. Liu, X. Xu, S. Liu (Tencent)]

The framework of the proposed DRF method is depicted in the next figure below. In this approach, two reconstructed frames chosen from the DPB are inputted into the deep reference frame generation network to synthesize a new frame. This synthesized frame is then added to both reference picture list0 and list1, using the same POC as the current encoding frame. It serves as an additional reference frame for subsequent encoding processes.

Position of NN-Inter in video codec (test EE1-5)

Changes of design compared to previous EE1 tests: removal of PFRB module and ResBlocks, IFRNet small IFRNet, SADL implementation (32 float).

Computational complexity is 504 kMAC/pxl (was 986 before), total number of parameters is 14.9 M (was 19.3 M).

Performance in random access configuration test (compared to previous design) is shown the table below.

Performance of reduced complexity NN-Inter (EE1-5) compared to previously studied design.

Performance after complexity reduction (EE1-5): kMAC/xpl 504, Number of parameters 14.9M.

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_NNVC-5.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -3.2% | -7.3% | -6.6% | -3.9% | -9.3% | -8.4% | 700% | 1030222% |
| Class C | -4.5% | -7.0% | -7.2% | -6.0% | -9.7% | -10.0% | 441% | 761956% |
| Class E |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |
| Class D | -5.5% | -5.9% | -6.5% | -5.7% | -10.4% | -10.1% | 751% | 1300136% |

Performance demonstrated in JVET-AD0162: kMAC/xpl 989, Number of parameters 19.3M.

|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -3.7% | -8.0% | -8.1% | -4.1% | -9.4% | -9.0% | 157% | 65726% |
| Class A2 | -5.4% | -11.7% | -9.4% | -6.2% | -11.5% | -9.2% | 147% | 60992% |
| Class B | -4.1% | -10.3% | -9.8% | -4.7% | -11.2% | -10.8% | 159% | 79960% |
| Class C | -5.7% | -13.4% | -13.6% | -7.3% | -13.5% | -14.3% | 123% | 47989% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -4.7% | -11.0% | -10.4% | -5.6% | -11.5% | -11.0% | 146% | 63562% |
| Class D | -7.2% | -14.2% | -15.6% | -6.8% | -14.8% | -15.2% | 113% | 32008% |

Reduction of computational complexity roughly twice leads to ~1 % compression performance degradation.

Suggestion: Get opinion from cross-checker during the meeting

completion of cross-check was not possible during the meeting (training completed and confirmed, inference ongoing).

Still floating point – integerization should be studied in the next round of EE.

See further notes in the BoG report JVET-AE0282.

Also the interdependency with loop filter should be studied – how much of the gain is retained in combination with low-operation loop filter (which will be one of the anchors).

NN-based intra

EE1-6.1 Neural network-based intra prediction with reduced complexity

JVET-AE0144 EE1-6.1: neural network-based intra prediction with reduced complexity [T. Dumas, F. Galpin, P. Bordes (Interdigital)]

NN-Intra is enabled by default in NNVC-5.0 Design of NN-Intra is show in the next figure below.

Note that all the mentioned layers are fully-connected. No convolutional layers exist in the presented neural networks. Changes compared to NN-Intra in common SW base (NNVC0) are summarized in the table below. The percentage of non-zero weights in the layer returning repIdx becomes 10% for the neural network predicting blocks of size 4×4, and 50% for the other six neural networks.

Computational complexity is different for different block sizes, the worst case is 4.8 kMAC/pixel (7.8 in NNVC). Total number of parameters is 1.3 M (1.5 in NNVC).

In AI, full test results show ~0.2% BD-rate performance degradation compared to NN-intra in NNVC-5.0. In RA, partial test results (full test results will be available by start of the meeting) show ~0.1% (RA) BD-rate performance degradation compared to NN-intra in NNVC-5.0.

Training cross-check could not be completed during the meeting. See further notes under the BoG report JVET-AE0282.

Figure: Prediction of the current w×h block from its context of reference samples via the neural network fh, w . , θh,w, parametrized by θh,w, belonging to the neural network-based intra prediction mode. Here, w=8 and h=4. The dimensions of the context of reference samples, “preprocessing”, “postprocessing”, repIdx, grpIdx1, grpIdx2 are explained in NNVC description document.

Architecture policy for each neural network in the common software base for NNVC and proposed solution (EE1-6).

| Architecture | NN-Intra in common software base | EE1-6 |
| --- | --- | --- |
| Number of hidden layers | For 16x16, 3. For the other six neural networks, 2. | Same. |
| Number of neurons per hidden layer | 1216. | For 4x4, 8x4, and 16x16, 1216 for the last hidden layer, and 640 for the other hidden layers. For the other four neural networks, 1216. |
| Biases | For all layers. | Only for the layer returning repIdx and the layer returning grpIdx1 and grpIdx2. |

There was a suggestion for looking at Bytedance’s training cross-check to decide.

Recommendations:

- Present the unified HOP filter (JVET-AE0191) in detail

- Adopt HOP filter from JVET-AE0191 (after training is finished) to NNVC-6.0.

- Continue investigating pending EE tests of this round (supposed to be conducted on top of the HOP model) in the next round of EE.

- Conduct BoG to review input contributions and plan EE1 tests, identify potential for joint training in the next EE1 round (could be two designs for LOP and HOP).

- Conduct training cross-check for low-complexity NN-Intra (EE1-6)

Review of EE1 & related documents (5.1.2/5.1.3) was on Thursday morning; there was a BoG (E. Alshina, F. Galpin) after that with the mandate of preparing the next EE, software development, etc.

JVET-AE0299 Crosscheck of JVET-AE0144 (EE1-6.1: Neural network-based intra prediction with reduced complexity) [J. Li (Bytedance)] [late] [miss]

JVET-AE0300 Crosscheck of JVET-AE0144 (EE1-6.1: Neural network-based intra prediction with reduced complexity) [Z. Dai (OPPO)] [late] [miss]

JVET-AE0042 AhG14 & AHG11: Report on AhG teleconference on high operation point (HOP) unified filter training [E. Alshina (AhG11 co-chair)]

There was no need for presentation of this – it was included in the AHG report JVET-AE0011.

JVET-AE0237 Presentation of AI-based video Coding in "Third AG4 Workshop on JPEG and MPEG Emerging Activities" [E. Alshina]

This was for information – no need for presentation.

### EE1 contributions: Neural network-based video coding (7)

Contributions in this area were partially discussed at 1130–1340 on Thursday 13 July 2023 (chaired by JRO), with follow-up in the BoG reported in JVET-AE0282 (corresponding notes are from BoG report).

JVET-AE0191 AhG11 - EE1-0 High Operation Point model [F. Galpin (InterDigital), S. Eadie, D. Rusanovskyy (Qualcomm), Y. Li, J. Li (ByteDance), L. Wang, R. Chang (Tencent), Z. Xie (Oppo), E. Alshina (Huawei)] [late]

A Unified Filter Architecture for the High-performance Operation Point (HOP) was defined at JVET-AD0380 and includes filter architecture, training process and training test set. Every design element and training procedure for this architecture were carefully selected though multiple rounds of earlier EE1 tests. Following JVET-AD meeting, a unified dataset and unified training procedure was implemented by multiple participants, and training conducted in parallel independently to allow cross-check.

This was initially discussed in JVET on July 13.

This contribution reports on the progress of unified filter implementation and achieved simulation results.

Intermediate results demonstrate the performance of the model: -7.78%,	-18.81%, -19.98% BD-rate change in Y, U and V components for AI configuration and -9.49%, -22.48%, -22.17% BD-rate change in Y, U and V components for RA configuration.

It is suggested to adopt EE1-0 into NNVC common software and enable it by default for HOP. It is also suggested to make training strategy defined for EE1-0 a part of NNVC SW. It is also suggested to continue practice of joint training activity for most promising NN-based tools studied in EE1. Identified dis-convergence of training at Stage II can be one of aspects to study in further EE1 tests.

In stage 2, problem with switch from L1 to L2 loss function: In the first epoch (18), the loss is still decreasing, and then it increases. It was suggested that a solution might be reduction of the learning rate. Results above are with the model coming from epoch 18 of stage 2.

In each stage, training is performed from scratch. Currently, training of stage 3 is running, which uses the filter model from stage 2 only for generating decoded data, but then a completely new model is trained from scratch.

It was agreed that a unified training strategy shall be used in EE to make proposals in a given category comparable. In the same category (e.g., directly competing methods, such as filter for HOP), identical number of epochs shall be used, training parameters (loss function, batch size, switching point, learning rate decay) shall be identical.

For proposals that are exclusive (e.g., HOP and LOP loop filters), the following shall apply:

- It would be desirable (to be clarified in BoG if mandatory already in the next round of EE) to define an equivalent mandatory strategy for LOP (the current LOP was trained somewhat similarly). Data set should be identical, but number of epochs may be different, as well as training parameters.

- not only the data set, but also augmentation, generation of coded data (QP points) should be identical.

Proponents are free to provide additional results with a different training strategy.

In terms of the HOP model for next NNVC SW the following was agreed.

Decision:

- Inference results for integerized stage 2 are expected during the meeting, such that a decision could be made. D. Rusanovskyy will send the model to cross-checkers, and provide an input when results are available. This could become the “fallback candidate” for HOP in NNVC6 SW.

- Training for stage 3 is running (plus cross-checks), but will not be available before the end of the meeting. An integerization strategy for stage 3 should be negotiated in the BoG. If the integerized stage 3 model provides better results within 14 days after the meeting, it will replace the “fallback candidate” in NNVC6 (to be confirmed in telco on Aug. 2)

It was later agreed (JVET Monday 17 July afternoon session, see notes under JVET-AE0291) to shift the deadline above and Telco by one week, August 9.

JVET-AE0291 AhG11: Performance of the NNVC HOP with quantized Stage II model [D. Rusanovskyy (Qualcomm), F. Galpin (InterDigital)] [late]

This was presented in JVET on July 17 afternoon (not in the BoG).

This contribution reports on the progress of the NNVC HOP Stage II models inference testing and cross-check conducted during JVET-AE meeting. This information is provided to facilitate selection of NNVC HOP anchor model from the JVET-AE.

It is reported that quantized HOP Stage 2 model provides no BD-rate penalty { 0.0%, -0.3%, -0.3%} on AI test configuration, and around {0.1%, 0.6%, 0.3%} on RA test configuration. Noticeable run-time reduction is reported with int16 implementation for both, encoder and decoders.

It is proposed to utilize quantized Stage II model and int16 implementation for generation of the Stage III data set. It is expected that use of quantized HOP model would allow bit-wise identical results for Stage III dataset reproduction by multiple parties.

It was agreed to use the integer model for stage 3 training. However, as this requires restarting the stage 3 training processes, it was further agreed to delay the deadline for finalization by one week (Aug. 9).

Comparative results of int16 vs. float model, NNVC HOP

|  | Int16 vs float, st2.ep18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  |  |  | Random Access |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.03% | 0.07% | -0.04% | 97% | 89% | 0.07% | 0.71% | 0.21% | 96% | 92% |
| Class A2 | 0.11% | 0.16% | -0.29% | 97% | 85% | 0.06% | 0.50% | -0.20% |  |  |
| Class B | 0.02% | -1.15% | -0.36% | 98% | 89% | 0.10% | 0.53% | 0.30% | 96% | 86% |
| Class C | 0.00% | -0.23% | -0.35% | 104% | 102% | 0.13% | 0.76% | 0.59% | 102% | 96% |
| Class E | 0.02% | -0.04% | -0.11% | 105% | 99% |  |  |  |  |  |
| Overall | 0.02% | -0.34% | -0.25% | 99% | 91% | 0.09% | 0.62% | 0.26% |  |  |
| Class D | 0.00% | -0.05% | -0.18% | 101% | 90% | -0.02% | -0.21% | -0.16% | 103% | 96% |

The quantization is performed such that float values are multiplied by 2^11, same as used in stage 1.

Decision: Adopt JVET-AE0191 and JVET-AE0291 to NNVC6 SW and JVET-AE2019, subject to possible further changes from stage 3 training.

(It was noted that the HOP model agreed in last meeting was already included in JVET-AD2019, but there may be changes in details.)

JVET-AE0067 EE1-4.1: Neural-network loop filters with further complexity reduction [J. N. Shingala, A. Shyam, A. Suneja, S. Badya (Ittiam), T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, Sean McCarthy (Dolby)]

This was presented in JVET plenary Thu 13 July.

This contribution reports the experimental results of EE1-4.1. The models for complexity reduction in this EE test are originally proposed in JVET-AD0157. The model uses CP decomposition, fusing adjacent 1x1 convolution and split architecture for luma and chroma. The EE tests focus on two aspects: 1) test different combination of model parameters with further complexity reduction compared to NNVC 5.0 LOP (worse case block level complexity is 16.2 KMAC/Pixel); 2) 16x SIMD friendly. The new proposed model is implemented on top of NNVC common software NNVC-5.0.

The AI and RA results for the new proposed model of {Y, Cb, Cr} BD-Rate gain over NNVC-5.0 tools off are as follows:

EE1-4.1: CP-decomposition with fusing and split luma chroma (16L, 16C), worst case block level complexity of 9.87 KMAC/Pixel

- SADL, fp32: AI { -4.14%, -7.90%, 7.64%}, RA { -4.58%, -10.14%, -8.37%}

The AI and RA BD-Rate results over NNVC-5.0 anchor (NnIntraPred and Nnlf LOP tool on) are as follows:

- SADL, fp32: AI { 0.35%, -2.59%, -2.07%}, RA { 0.48%, -3.29%, -2.14% }

The next figure below shows NNVC 5.0 NNLF LOP (Low Operation Point) architecture: CP-decomposition of 3x3 convolution layers and fusion of adjacent 1x1 pointwise convolutions. The 3x3 convolutions of each hidden layer are decomposed into 4 layers with rank R followed by fusion of adjacent 1x1 convolution:

- 1st layer: 1x1xKxR pointwise convolution

- 2nd layer: 3x1xRxR separable convolution

- 3rd layer: 1x3xRxR separable convolution

- 4th layer: 1x1xRxK pointwise convolution

Figure: NNVC 5.0 LOP: CP Decomposition + fusion of 1x1 conv layers

The next figure below shows the EE1-4.1 model: split luma, chroma architecture combined with CP decomposition and fusing of adjacent 1x1 conv layer. Various 16x SIMD friendly channel count combinations have been tested at about 10KMAC/Pixel worst case block-level complexity. The proposed new model setting is shown in the table below.

Figure: Split luma and chroma model + CP Decomposition + fusion of 1x1 conv layer (EE1-4.1)

EE1-4.1 Split Luma Chroma (16L, 16C) setting and complexity

| Model/Exp | K | Ky | Kc | M | My | Mc | Ry | Rc | Ny | Nc | kmac |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| split luma chroma (16L, 16C) | 32 | 16 | 16 | 64 | 48 | 16 | 16 | 16 | 10 | 6 | 9.87 |

There were 2 models for inter and 2 models for intra.

Proponents used the same training strategy as for current LOP filter. It was requested to provide slides for the BoG to understand the commonalities and differences with the HOP strategy.

This showed loss in luma, but gain in chroma. It was suggested to try different weights, or modify number of channels (less in chroma).

JVET-AE0183 Crosscheck of JVET-AE0067 (EE1-4.1: Neural-network loop filters with further complexity reduction) [J. Ström (Ericsson)] [late]

JVET-AE0112 EE1-5.1: Deep Reference Frame Generation for Inter Prediction Enhancement [W. Bao, W. Meng, J. Jia, Y. Zhang, H. Wang, Z. Chen (Wuhan Univ.), Z. Liu, X. Xu, S. Liu (Tencent)]

See EE summary report – the next EE was to be defined in the BoG.

JVET-AE0229 Crosscheck of JVET-AE0112(EE1-5.1: Deep Reference Frame Generation for Inter Prediction Enhancement) [Z. Xie (OPPO)] [late]

JVET-AE0144 EE1-6.1: neural network-based intra prediction with reduced complexity [T. Dumas, F. Galpin, P. Bordes (Interdigital)]

Following the 29th JVET meeting, 11-20 January 2023, three additional neural network-based coding tools (JVET-AC0055, JVET-AC0196, and JVET-AC0116) were integrated into VTM-11-NNVC (JVET-AC2019). In JVET-AC0116, a neural network-based intra prediction mode is added to the set of intra prediction modes in VTM-11-NNVC. To limit the complexity of the neural network-based intra prediction mode, each neural network in this mode features a relatively small architecture and weight sparsity is enforced. However, the architecture and weight sparsity policy were not tuned to each neural network.

To address this, JVET-AD0212, during the 30th JVET meeting (21-28 April 2023), proposed a modified architecture and weight sparsity policy adapted to each neural network in this neural network-based intra prediction mode. This contribution reports EE1-6.1 associated to JVET-AD0212.

It is reported that VTM-11-NNVC-5 in which the models of the neural network-based intra prediction mode are replaced by the models with refined architecture and weight sparsity policy yields 0.22%, 0.16%, 0.10% and 0.08%, -0.01%, 0.02% in AI and RA respectively with respect to VTM-11-NNVC-5. In VTM-11-NNVC-5, the worst-case complexity of the neural network-based intra prediction mode is estimated at 7.8 kMACs/pixel whereas, in VTM-11-NNVC-5 containing the models with refined architecture and weight sparsity policy, the worst-case complexity of this mode becomes 4.8 kMACs/pixel.

Notes were derived from the BoG report JVET-AE0282:

It was agreed that JVET-AE0144 could be included in NNVC6, provided that the training crosscheck would be successfully completed by the date of the AHG telco meeting (Aug. 9).

Post-meeting note: As the training crosscheck was not successful, a new test (EE1-3.1) for NN-Intra with training cross-check was added per a decision of that AHG meeting.

Cross-checkers were requested to register their document, even though results will only be available after the meeting. It should be uploaded in due time before the AHG telco that is planned for Aug. 9.

JVET-AE0160 EE1-1.5: Optimization for complexity-performance trade-off of HOP network [R. Chang, L. Wang, X. Xu, S. Liu (Tencent)]

From BoG notes (recommendation confirmed by JVET):

This contribution reports the EE-1.5 result of JVET-AE0160. Based on the unified network architecture of the High Operation Point (HOP), this contribution proposes further optimization including new resblock and split network design to improve the complexity-performance trade-off of HOP network. Specifically, based on the depthwise separable convolution and group convolution, two novel resblocks are proposed to reduce the computational complexity without affecting the coding performance too much. Besides, the split network design for luma and chroma components is introduced into HOP network. Based on NNVC-5.1, the test results are shown in order of RA and AI configurations as follows.

Compared with NNVC-5.0 anchor:

Test 1.5.1 (Test designed resblocks with depthwise separable convolution and group convolution):

- Subtest 1: HOP resblock (as base comparison method for Test 1.5.1)

RA: -5.11%	-14.62%	-14.53%	EncT: 129%	DecT: 502%

AI : -3.06%	-12.38%	-13.54%	EncT: 102%	DecT: 439%

- Subtest 2: HOP res-block with depthwise separable convolution

RA: -4.82%	-14.92%	-14.18%	EncT: 140%	DecT: 640%

AI : -2.81%	-12.01%	-13.02%	EncT: 106%	DecT: 546%

- Subtest 3: HOP res-block with group convolution

RA: -4.97%	-14.16%	-13.79%	EncT: 153%	DecT: 822%

AI : -2.90%	-11.79%	-12.99%	EncT: 110%	DecT: 693%

Test 1.5.2 (Test split network design):

- Subtest 1: HOP network (as base comparison method for Test 1.5.2)

RA: -3.63%	-12.05%	-13.74%	EncT: 139%	DecT: 623%

AI : -2.28%	-11.44%	-12.15%	EncT: 105%	DecT: 535%

- Subtest 2: HOP network with split architecture for luma and chroma

RA: -3.12%	-11.64%	-11.47%	EncT: 249%	DecT: 2139%

AI : -1.96%		-10.56%	-11.48%	EncT: 145%	DecT: 1769%

Test EE1-1.5.1 Two ways of Back Bone Block simplification: 1) depth-wise separable convolution (~0.3% drop, 461408 kMAC/pxl); 2) group (size 4) convolutions (0.1%...0.2% drop 461402 kMAC/pxl).

Test EE1-1.5.2 Split for HOP architecture: 0.5% drop (480316 kMAC/pxl).

It was recommended to study both aspects of JVET-AE0160 in EE1, using single modification of HOP.1 architecture in each sub-test, usage aspects and training strategy to keep the same as HOP.1.

JVET-AE0255 Crosscheck of JVET-AE0160 (EE1-1.5: Optimization for complexity-performance trade-off of HOP network) [D. Liu (Ericsson)] [late]

The cross-checker supported study for both aspects in EE1.

JVET-AE0164 EE1-1.2 Complexity-performance tradeoff of decomposition [D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm)]

From BoG notes (recommendation confirmed by JVET):

A Unified Filter Architecture for the High-performance Operation Point (HOP) was defined at the JVET-AC meeting and includes model architecture, training process and training test set. In test EE1-1.2 complexity-performance trade-off optimization of the convolution decomposition in unfied filter model have been studied.

This test introduces reduced ranking of the two-components decomposition in residue blocks of the backbone. With parameter C31 reduced to 48, complexity of the results model is reduced by 11% to 426kMAC/pixel (block-wise 128x128). To mitigate the impact of the rank reduction on decomposition symmetry, order of the decomposition (horizontal-vertical or vertical-horizontal) is alternated every block in the backbone. The method was tested with HOP Stage 1 training and AI results for quantized model are reported. Training for latter stages of HOP is ongoing, results to be provided once available.

Comparing to EE1-1.0 HOP (float), proposed filter (int16) provides {0.0%, 0.3%, 0.1%} bd-rate penalty in AI configuration with 11% complexity reduction in kMAC/pixel terms. Comparing to NNVC5.0 with NN tools OFF, proposed filter provides: {-7.9%, -19.0%, -20.2%} bd-rate gain in AI configuration. Comparing to NNVC-5.0 (NN Intra and LOP enabled), proposed filter being tested with NN Intra enabled provides: {-3.2%, -14.4%, -15.3%}} bd-rate gain in AI configuration.

It was proposed to study the proposed method in the next round of EE.

Two aspects: 1) alternation of 3x1 & 1x3 and 1x3 & 3x1 convolutions in Back Bone Block, gain in class A1, no impact in average.

Recommendation: Study JVET-AE0164 in EE1. Complete training (same as HOP.I) till Stage III.

JVET-AE0165 EE1-4.4: Low complexity NN filter with design elements of Unified Filter Architecture and EE1-1.2 and EE1-1.3 [Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm)]

This was presented in JVET on July 13.

A Unified Filter Architecture for the High-performance Operation Point (HOP) was defined at JVET-AC meeting and includes model architecture, training process and training test set. Unified data set creation, training, and performance assessment of the HOP architecture have been ongoing between JVET-AD and AE meetings, and Stage1 model and its performance was fully evaluated (on AI results) at the time of submission of this contribution. A Low-complexity Operation Point (LOP) in-loop filter, with complexity of 16.2 kMAC/pixel (frame-wise), was also adopted in JVET-AC meeting to the NNVC software to serve as an anchor for further HOP development.

In this contribution, Unified Filter Architecture of HOP was adjusted to meet complexity constraints of LOP. This was achieved by reducing complexity of the headblock, backbone and utilizing the CP decomposition in the backbone blocks. End-to-end training of this model was conducted following the process specified for HOP Stage1. Training for latter stages of HOP is ongoing, results to be provided once available.

Achieved model complexity in kMAC/pixel: 14.8 (frame-wise); 16.7 (block-wise of block 256x256); 18.7 (block-wise of block 128x128).

Comparing to NNVC-5.0 HOP Anchor (NN Intra and LOP filter enabled), the proposed filter (quantized model) provides: {0.08%, -5.41%, -5.83%} in AI test configuration. For the joint YCbCr metric, the BD-rate gain is -1.1%.

Comparing to NNVC-5.0 (NN tools OFF), the proposed filter (quantized model) provides: {-4.42%, -10.55%, -11.17%} in AI test configuration.

Comparing to NNVC-5.0 (NN tools OFF), the proposed filter (quantized model) with NN Intra enabled provides: {-7.73%, -13.64%, -14.28%} in AI test configuration.

It is proposed to study the proposed method in the next round of EE.

Figure: The proposed filter architecture. In the blocks marked with “Separable”, depth-wise convolution is performed.

The information for the inference and the training stage is presented in the next two tables below, respectively.

Network information for the inference

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | HW environment: |  |  |
|  | GPU Type |  | N/A |
|  | Framework: |  | SADL |
|  | Number of GPUs per Task |  | 0 |
|  | Total Parameter Number |  | 56544 /model |
|  | Parameter Precision (Bits) |  | 32 or 16 |
|  | Memory Parameter (MB) |  |  |
|  | Multiply Accumulate (kMAC/pixel) block (frame) based |  | 18.7 (14.8 frame-wise) |
|  | Total Conv. Layers |  | 67 |
|  | Total FC Layers |  | 0 |
|  | Total Memory (MB) | Float |  |
|  |  | Int16 | 0.125/model |
|  | Batch size: |  | 1 |
|  | Patch size |  | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage |  |  |
|  | Other information: |  |  |

Network information for the training

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla V100-SXM2-32GB |
|  | Framework: | PyTorch v1.8 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 130 |
|  | Batch size: | 64 |
|  | Training time: | ~240h |
|  | Training data information: | DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Backbone blocks are not exactly identical to the HOP model, for the benefit of complexity reduction.

Many more epochs were used in training than for the HOP model.

It was requested to upload the modified software to EE repository.

It was suggested to merge convolutions at the input, as there is no non-linearity.

From the BoG notes:

It was suggested and agreed to use unified filter design approach for LOP. Unification includes:

- Training strategy,

- Filter usage aspects,

- Filter architecture,

- PyTorch implementation for training (Tensor flow I used currently)

A joint contribution from EE1-4.1 and EE1-4.4 was expected to be ready for review by July 14.

### EE1 related contributions: Neural network-based video coding (5)

Contributions in this area were discussed in the BoG reported in JVET-AE0282 (notes are from BoG report)

JVET-AE0072 [AHG11] On residual adjustments for NNLF [Z. Dai, Y. Yu, H. Yu, D. Wang (OPPO)]

This was reviewed in the BoG with recommendation approved by JVET.

This contribution proposes residual adjustments for NNLF’s output based on low complexity operation point (LOP) NN-based filter in the NNVC reference software. In JVET-AD0156, a residual network based NNLF was proposed to predict the residual between the reconstructed image and the original image to get better coding performance. It is proposed to adjust the residuals at the frame level, which is signalled by a flag.

Compared with the NNVC-5.1rc2 default configuration (enabling NN-intra and low complexity NN-filter), it is reported that the proposed method can further improve the coding efficiency {Y, U, V, EncT, DecT} with the following results:

RA: Y -0.05%, U -0.59%, V -0.49%, EncT 100%, DecT 99%.

LDB: Y -0.20%, U -1.44%, V -0.90%, EncT 100%, DecT 99%.

AI: Y -0.03%, U -0.32%, V -0.24%, EncT 100%, DecT 98%.

This contribution proposes a two-step residual adjustment method. Firstly, a frame level residual offset can be used to adjust the output of the NNLF in the inference stage.

Secondly, a block level residual averaging can be used to adjust the output of the NNLF in the inference stage.

It was recommended to study JVET-AE0072 in EE1 (“filter usage” category). Apply for both LOP and HOP. Improve encoder decision targeting no loss for all sequences. Only inference cross-check.

JVET-AE0232 Crosscheck of JVET-AE0072 ([AHG11] On residual adjustments for NNLF) [T. Shao (Dolby)] [late]

JVET-AE0093 AHG11: Content-adaptive neural network loop-filter [R. Yang, M. Santamaria, F. Cricri, H. Zhang, J. Lainema, R. G. Youvalari, M. M. Hannuksela (Nokia)]

This was reviewed in the BoG with recommendation confirmed by JVET.

This contribution presents a content-adaptive neural network loop-filter. The content adaptation is achieved by means of overfitting a base loop-filter on the test video. The base loop-filter was described in JVET-AD0156 and it is the default (low-complexity) neural network loop-filter in NNVC 5.0. The generated weight-updates are coded with the MPEG-NNR standard and the bitrate overhead is considered into BD-rate calculations. The inference was done using SADL library with int16 precision and evaluated against NNVC 5.0 in RA configuration, with neural network intra prediction and low-complexity neural network loop-filter enabled. It is reported that, the average coding gains are -1.21% (Y), -6.43% (Cb) and -5.52% (Cr) with negligible impact to the decoding times. Thus, it is reported that the proposed content-adaptive loop-filter outperforms NNVC 5.0. It is proposed to include the method into the next EE1 round for a training and inference cross-check, and to eventually adopt it into the next version of NNVC anchor.

The proposed content-adaptation is performed by overfitting multipliers applied after each convolutional block, shown as the following equation:

σW*x+b∙m,

where W is the kernel, * is the convolution operator, x is the input, b is the bias, m is the multiplier and σ is the activation function. The resulting weight update is coded using the MPEG-NNR standard once per sequence. Overfitting takes ~30% of encoding (must be noticed that encoding is performed in CPU, overfitting in GPU).

In anchor 4 base models are used, in test 3 base models + 1 overfitted model are used. Gain (RA cfg) 1.2% (Y), 6.4%(Cb), 5.5%(Cr).

It was noticed that decoder run time increases (107% compared to the anchor), reason is extra multipliers (which comes from bit-stream).

Reference to MPEG-NNR standard is needed in this tools goes to the standard developed by JVET.

It would be interesting to show how often overfitted model is selected.

It was recommended to study JVET-AE0093 in EE1 (“filter usage” category) for LOP only. Improve encoder decision targeting no loss for all sequences. Only inference (and overfitting) cross-check.

JVET-AE0161 AHG11: Input and output rotation of model for NNVC in-loop filter [R. Chang, L. Wang, X. Xu, S. Liu (Tencent)]

This was reviewed in the BoG with recommendation confirmed by JVET.

In this contribution, the rotation operations are applied to the inputs and outputs of NNVC HOP network to further improve the coding performance. In the rotation process, all the input samples including image information and side information are rotated and then fed into network to perform the inference. After that, the output of network is rotated back to restore the original order of filtered samples. Based on NNVC-5.1 the partial test results show 0.14% BD-rate gain (RA cfg, luma) .

In the proposed method, all the input samples may be rotated 90° to the left or right and then fed into the network. If rotation is performed on the inputs, the output of network would be rotated back to restore the original order of the filtered samples. The operations of rotating left and right are shown as follows.

Fig. 2: Rotation to the left

Fig. 3: Rotation to the right

Specifically, a slice-level flag is signalled to indicate the rotation type including no rotation, rotation to the left and rotation to the right. All the CTUs in a slice are applied the same rotation operation before and after network inference.

Besides, to reduce the encoding time, only partial CTUs are used in the RDO process of selecting the best rotation type.

It was recommended to study JVET-AE0161 in EE1 (“filter usage” category) for both HOP and LOP. Only inference cross-check.

JVET-AE0276 Crosscheck of JVET-AE0161 (AHG11: Input and output rotation of model for NNVC in-loop filter) [Z. Xie (OPPO)] [late]

JVET-AE0171 AHG11: Neural network-based in-loop filter with layer normalization [Y. Li, K. Zhang, L. Zhang (Bytedance)]

This was reviewed in the BoG.

This contribution proposes to introduce the layer normalization technique into the NN-based filtering design. The proposed method uses separate models for luma and chroma. BD-rate changes on top of NNNC-5.0 VTM anchor and NNVC-5.0 anchor, as well as model complexities, are reportedly summarized as below:

Compared with NNVC-5.0 VTM anchor: RA: -11.55%, -27.24%, -26.78%, AI: -9.05%, -23.28%, -23.64%

Compared with NNVC-5.0 anchor: RA: -6.73%, -21.68%, -21.61%, AI: -4.52%, -18.97%, -19.03%

kMAC/pixel of frame basis: 396 (luma: 370, chroma: 26), #parameters: 2.5M (luma: 1.9M, chroma: 0.6M)

Filter set #1 used as basis. Separate Luma and Chroma filters. Layer normalization (used in this contribution is not supported by SADL).

Suggestion is to use HOP and only add normalization layer.

This was difficult to study in EE1-1 because SADL implementation was not ready yet. Proponent was asked to share code if possible.

Unified training strategy

Figure: Training steps for unified model creation

Number of epochs in unified training strategy for HOP.1 is:

- Stage I: 40, stage II: 20 (19), stage III: 20 (1 accurate check at loss-function change point).

Unified training which was used for HOP-1 is reproducible. User can start from

- Stage I (only training scripts and access to training set is needed),

- Stage II (additionally intermediate model of Stage I – HOP.1sI is needed),

- Stage III (additionally intermediate model of Stage II – HOP.1sII is needed).

and performance of re-trained HOP.1 model was expected to be within 0.1% BD-rate difference for JVET-AE0191 (as was confirmed in joint training acitvity).

From BoG discussion:

Proponents of JVET-AE0191 are mandated to prepare the document with training instructions starting from Stage I, or Stage II or Stage III. This document must include the link to the intermediate models (HOP.1sI, HOP.1sII) and test results expected after each stage of training. This document can be late input to the JVET-AE meeting and become a part of EE1 description (or even NNVC CTC document).

EE1 proponent will be given a freedom to start from any training stage. For minimal changes in architecture likely just Stage III is enough. EE1 test description and input contributions must clearly state which stage(s) been used in training.

(See further notes under JVET-AE0281.)

JVET-AE0238 AHG11: A unified design of NN-based loop-filters at low and high operation points [J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang (Bytedance)] [late]

This was reviewed in the BoG with the recommendation confirmed by JVET.

This contribution proposes to unify the implementation of NN-based loop-filters at the low operation point (LOP) and the high operation point (HOP). The unified design is conducted based on the interface of HOP, and model selection in slice level is introduced to adapt various filtering models. The performance of HOP maintains the same as before. Based on partial test results the BD-rate gain of LOP filter improved ~0.2 % (Y) in random access cfg.

Filter usage of unified HOP is applied to LOP.

It was recommended to study JVET-AE0238 in EE1 (“filter usage” category) for LOP only, improving the encoder decision targeting to balance luma/chroma performance.

Only the inference was cross-checked.

JVET-AE0281 Unified LOP filter design, training procedure and filter usage [D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm), T. Shao, P.Yin, S. McCarthy (Dolby), J. N. Shingala, A. Shyam(Ittiam), J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang (Bytedance)] [late]

This was reviewed in the BoG, with the recommendation confirmed by JVET.

A Unified Filter Architecture for the High-performance Operation Point (HOP) was defined at JVET-AD meeting and includes model architecture, training process and training test set. Additionally, a Low-complexity Operation Point (LOP) in-loop filter, with complexity of 16.2 kMAC/pixel (block-wise), was also adopted in JVET-AD meeting to the NNVC 5.0 software to serve as an anchor for further HOP development.

For JVET-AE meeting, two EE1 tests and one AHG11 proposal have been submitted in LOP category: EE1-4.1 (JVET-AE0067), EE1-4.4 (JVET-AE0165), and JVET-AE0238. EE1-4.1 tests presented a significant complexity reduction achieved by luma-chroma split in the NN architecture, whereas EE1-4.1 presented HOP based architecture of LOP complexity and utilizes HOP training process. JVET-AE0238 presented study on unified filter usage interfaces of HOP and LOP filters.

This contribution proposes a unified filter architecture and filter usage for LOP study in EE1. Proposed architecture combines key components of EE1-4.1, EE1-4.4, and JVET-AE0238 tests and is configured to meet complexity constraints of the current NNVC5.0 LOP anchor (16kMAC/pixel) and employs unified filter usage for LOP.

Defined training is HOP aligned: utilizes HOP dataset, training script and training parameters with changes limited to the scheduler. Filter usage, output interface is subject of study in sub-test.

It is proposed to introduce this architecture as EE1 test for study on LOP.

Summary of unified design:

- Filter architecture is combination of current EE1-4.1 (split aspect) and EE1-4.4 (simplified Back Bone Block aspect).

- Filter usage aspect as HOP (JVET-AE0238)

- Training strategy from EE1-4.4 (identical to HOP but different number of epochs, switching point and learning rate).

Parameters were selected to keep computational complexity at 16 kMAC/pxl.

Training starts from Stage I (from the scratch).

Beside of this architecture proponents will train 2-3 more variations (as back up). In those back up variants proponents with 1) optimize the headblock design toward improving hardware friendliness, 2) optimize of the split luma and chroma backbone design toward better complexity-performance tradeoff.

After Stage II training based on intermediate performance test results only one model will be selected. Teleconference is expected (mid/end of August).

All intermediate tests during training will be done with filter usage aspects identical to HOP.

For only one final unified LOP additional study of necessity for filter usage modification will be investigated (as sub-test).

Volunteers for joint training: Bytedance, Dolby, Itiamm, Qualcomm, InterDigital (?).

SW for joint LOP training will be hosted in EE1 repository.

After SW is mature enough and ready to start training the announcement to JVET reflector is requested.

During join training it is suggested to regularly exchange data on validation loss (for earlier detection of abnormal training behaviour and bugs). Same can be useful for all EE1 test with training.

Recommendation: To include joint training for unified LOP as proposed in JVET-AE0281 to EE1.

### Improvements of NNVC software beyond EE1 (4)

Contributions in this area were discussed in the BoG reported in JVET-AE0282 (notes were from the BoG report, with recommendations confirmed by JVET in the context of review of the BoG report).

JVET-AE0119 AHG14: The extension of SADL library [W. Bao, H. Wang, Z. Chen (Wuhan Univ.)]

This was reviewed in the BoG.

This contribution presents the extensions in the Small AdHoc Deep Learning (SADL) library from Wuhan University. These extensions include features such as PReLU, Slicing, and ScatterND, along with a bug fix. All of these extensions have been verified and merged.

Bug fix relevant not only to NN-Inter, common use.

This was an excellent example of contribution to AhG14.

JVET-AE0162 AHG11/AHG14: Fix MS-SSIM calculation for SR [R. Chang, L. Wang, X. Xu, S. Liu (Tencent)]

This was reviewed in the BoG.

In current NNVC-5.1 software, the MS-SSIM calculation for super resolution (SR) coding tools is not supported. An invalid MS-SSIM result is obtained when enabling those coding tools, such as reference picture resampling (RPR) and neural-network based super resolution (NNSR). This contribution proposes a fix for the MS-SSIM calculation both for conventional RPR and NNSR. This fix does not affect the PSNR results.

MS-SSIM results with bugfix for NNSR:

- RA: -3.99%	-1.83%		-1.74%		EncT: 106%	DecT: 448%

- AI : -3.56%	-3.12%		-1.96%		EncT: 113%	DecT: 420%

MS-SSIM results without bugfix for NNSR:

- RA: -16.77%	-10.71%	Unknown	EncT: 107%	DecT: 489%

- AI : -17.76%	-12.99%	-12.02%	EncT: 113%	DecT: 455%

Recommendation: Adopt to NNVC-6.0 SW (only SW change), consider to fix this bug in VTM, ECM, will be useful for RPR tests.

JVET-AE0230 Crosscheck of JVET-AE0162(AHG11/AHG14: Fix MS-SSIM calculation for SR) [Z. Xie (OPPO)] [late]

This confirms the results and supports adoption.

JVET-AE0194 AhG14: SADL update [F. Galpin, T. Dumas, P. Bordes, E. François (InterDigital)] [late]

This was reviewed in the BoG.

This contribution presents updates in the Small AdHoc Deep Learning (SADL) library. The update contains both some fixes and new features.

Changes are available in the repository at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/sadl.

Please note that internal file format has been upgraded to SADL0004.

Old files are backward compatible with the SADL library but newly generated file cannot be used with older version of the library.

Version v5.1 has been tagged on July 13th 2023.

New layers

The following new layers were added:

- scatterND (JVET-AD0365)

- extend slicing support (JVET-AD0365)

- extend PreLU to multi-slope (JVET-AD0365)

- extend slicing in other dimensions, (JVET-AD0163)

Improvements and fixes

- Speed-up separated convolution for HOP model (x4 speed-up).

- Fix potential bug in shared weights for convolution (thanks to W. Bao)

- Count PreLU/leakyReLU as 1 MAC

- Other minor improvements

Comment: All changes are already in NNVC-5, no extra action is needed.

JVET-AE0289 AhG11: HOP training process and models [F. Galpin (InterDigital), S. Eadie, D. Rusanovskyy, Y. Li (Qualcomm), Y. Li, J. Li (ByteDance), L. Wang, R. Chang (Tencent), Z. Xie (Oppo), E. Alshina (Huawei)] [late]

This document provides detailed information about how to conduct the training when starting from stage 2 or stage 3. It shall be made available along with the model, and should be placed in the software repository as well.

There was no need for presentation of this contribution.

It was suggested that later more generic guidelines for training of NNVC models (not only HOP) could be developed from parts of this contribution.

### Other aspects of neural network-based video coding (0)

Section kept as a template for future use.

## AHG6/AHG12: Enhanced compression beyond VVC capability (78)

### Summary and BoG reports

Contributions in this area were discussed at 1515–1815 on Tuesday 11 July 2023 and at 0830–1045 on Wednesday 12 July 2023 (chaired by JRO).

JVET-AE0024 EE2: Summary report of exploration experiment on enhanced compression beyond VVC capability [V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang (EE coordinators)]

This document provides a summary report of Exploration Experiment on Enhanced Compression beyond VVC capability. The tests are categorized as partitioning, intra prediction, inter prediction, transform and coefficient coding, and in-loop filtering.

The software basis for this EE is ECM-9.0, released at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-9.0. ECM-9.0 is used as an anchor in the tests.

Software for EE tests is released in the corresponding branches at https://vcgit.hhi.fraunhofer.de/ecm/jvet-ad-ee2/ECM/-/branches.

Test results can be found in input JVET contributions, cross-check results are uploaded to https://vcgit.hhi.fraunhofer.de/ecm/jvet-ad-ee2/simulation-results if cross-check reports are not submitted as they are optional for EE tests.

List of tests

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Partitioning |  |  |  |  |
| 1.1a | Partitioning prediction | Canon G. Laroche JVET-AE0132 | InterDigital S. Puri |
| 1.1b | Partitioning prediction with unconditional MTT depth increment | Canon G. Laroche JVET-AE0132 | InterDigital S. Puri |
| 1.1c | Encoder only partitioning prediction | Canon G. Laroche JVET-AE0132 | InterDigital F. Le Léannec |
| 2 Intra prediction |  |  |  |
| 2.1a | Block vector guided CCCM with IBC BV | Nokia R. Youvalari | B<>com M. Abdoli |
| 2.1b | Block vector guided CCCM with IBC and IntraTMP BV | Nokia R. Youvalari | B<>com M. Abdoli |
| 2.2a | Extended IBC-GPM with two IBC predictions | Kwai C. Ma | withdrawn |
| 2.2b | Bi-predictive IBC-GPM | KDDI Y. Kidani JVET-AE0169 | withdrawn |
| 2.2c | Bi-predictive IBC-GPM | Kwai C. Ma KDDI Y. Kidani JVET-AE0169 | Bytedance W. Yin JVET-AE0197 |
| 2.3a | IBC BVP-merge | KDDI Y. Kidani JVET-AE0169 |  |
| 2.3b | Test 2.3a + bi-predictive IBC merge without IBC-GPM | KDDI Y. Kidani JVET-AE0169 | Ericsson R. Yu JVET-AE0205 Bytedance W. Yin JVET-AE0197 |
| 2.3c | Bi-predictive IBC merge without + Test 2.3a + Test 2.2c | KDDI Y. Kidani Kwai C. Ma | withdrawn |
| 2.4a | IBC MBVD list derivation | Qualcomm Z. Zhang JVET-AE0169 | Ofinno D. Ruiz Coll |
| 2.4b | Test 2.4a + Test 2.3a | Qualcomm Z. Zhang KDDI Y. Kidani | withdrawn |
| 2.4c | Test 2.4a + Test 2.3b + Test 2.2c | Qualcomm Z. Zhang KDDI Y. Kidani Kwai C. Ma JVET-AE0169 | Ericsson R. Yu JVET-AE0205 |
| 2.5a | Filtered IBC | Kwai H.-J. Jhu JVET-AE0159 | withdrawn |
| 2.5b | Filtering IBC predicted blocks | Qualcomm B. Ray JVET-AE0159 | withdrawn |
| 2.5c | Test 2.5a + Test 2.5b | Kwai H.-J. Jhu Qualcomm B. Ray JVET-AE0159 | OPPO Y. Yu L. Zhang JVET-AE0288 |
| 2.6a | IBC with non-adjacent spatial candidates | Kwai C. Ma | withdrawn |
| 2.6b | Non-adjacent spatial candidates for IBC | Bytedance Y. Wang | withdrawn |
| 2.6c | IBC with non-adjacent spatial candidates (Test 2.6a + Test 2.6b) | Kwai C. Ma Bytedance Y. Wang JVET-AE0094 | Alibaba J. Chen |
| 2.7 | Cross-component merge mode with temporal candidates | MediaTek H.-Y. Tseng | InterDigital P. Bordes |
| 2.8 | An extrapolation filter-based intra prediction mode | OPPO L. Xu | Kwai H.-J. Jhu JVET-AE0217 |
| 2.9 | Extended search areas for IntraTMP mode | OPPO Y. Yu Xidian Y. Ma, H.Zhang Kwai X. Xiu JVET-AE0077 | InterDigital K. Naser JVET-AE0210 Qualcomm P.-H Lin JVET-AE0216 Alibaba X. Li JVET-AE0245 |
| 2.9b | Extended search areas for IntraTMP mode with scan order #2 | OPPO Y. Yu Xidian Y. Ma, H.Zhang Kwai X. Xiu | withdrawn |
| 2.9c | IntraTMP mode with partial extended search areas with scan order #1 | OPPO Y. Yu Xidian Y. Ma H. Zhang Kwai X. Xiu | withdrawn |
| 2.9d | IntraTMP mode with partial extended search areas with scan order #2 | OPPO Y. Yu Xidian Y. Ma H. Zhang Kwai X. Xiu | withdrawn |
| 2.10a | IBC-LIC extension without large block-size constraint | OPPO Z. Xie JVET-AE0078 | Bytedance W. Yin JVET-AE0157 |
| 2.10b | ECM IBC-LIC without large block-size constraint | OPPO Z. Xie JVET-AE0078 | Bytedance W. Yin JVET-AE0157 |
| 2.11a | Harmonization between IBC HMVP and IBC-LIC | Bytedance N. Zhang JVET-AE0084 | Kwai C. Ma JVET-AE0222 |
| 2.11b | Test 2.11a + Test 2.10 | Bytedance N. Zhang OPPO Z. Xie JVET-AE0084 | Kwai C. Ma JVET-AE0222 |
| 3 Inter prediction |  |  |  |
| 3.1a | Cross-component residual model | Nokia P. Astola JVET-AE0059 | InterDigital F. Le Léannec Ittiam J. Raj |
| 3.1b | Cross-component residual model with complexity reductions | Nokia P. Astola JVET-AE0059 | InterDigital F. Le Léannec Ittiam J. Raj |
| 3.2 | Bi-predictive GPM | Ericsson R. Yu JVET-AE0046 | LGE Y. Ahn JVET-AE0199 KDDI Y. Kidani |
| 3.3a | Additional TM refinement for bi-prediction | Bytedance Y. Wang JVET-AE0087 | Alibaba J. Chen JVET-AE0239 |
| 3.3b | 16-point diamond search pattern for TM | Bytedance Y. Wang JVET-AE0087 | Alibaba J. Chen JVET-AE0239 |
| 3.3c | Enabling TM for bi-prediction under DMVR condition | Bytedance Y. Wang JVET-AE0087 | Alibaba J. Chen JVET-AE0239 |
| 3.3d | Test 3.3a + Test 3.3c | Bytedance Y. Wang JVET-AE0087 | Alibaba J. Chen JVET-AE0239 |
| 3.3e | Test 3.3a + Test 3.3b + Test 3.3c | Bytedance Y. Wang JVET-AE0087 | Alibaba J. Chen JVET-AE0239 |
| 3.4 | HPel flag and BCW weight usage in OBMC | InterDigital A. Robert JVET-AE0196 | withdrawn |
| 3.5 | Iterative BDOF pass in multi-pass DMVR | Bytedance M. Salehifar Alibaba J. Chen JVET-AE0065 | vivo Z. Lv JVET-AE0200 |
| 3.6 | Affine AMVP mode with one MVD | Qualcomm H. Huang | withdrawn |
| 3.7a | RPR with new filters, scale factor 1.25x | Sharp J. Samuelsson-Allendes JVET-AE0150 | Qualcomm Z. Zhang JVET-AE0204 |
| 3.7b | RPR with new filters, scale factor 1.33x | Sharp J. Samuelsson-Allendes JVET-AE0150 | Qualcomm Z. Zhang JVET-AE0204 |
| 3.8 | Combination of Test 3.3e and Test 3.5 | Bytedance M. Salehifar Alibaba J. Chen JVET-AE0091 | Kwai C. Ma JVET-AE0223 |
| 4 Transform and coefficient coding |  |  |  |  |
| 4.1 | Shifting quantizer center | InterDigital M. Balcilar JVET-AE0125 | Qualcomm M. Coban JVET-AE0149 |
| 4.2 | Large NSPT | LGE M. Koo JVET-AE0086 | Qualcomm P. Garus JVET-AE0118 |
| 4.3 | Context modelling for transform coefficients for LFNST/NSPT | Qualcomm P. Nikitin JVET-AE0102 | IRT b-com M.Abdoli JVET-AE0225 |
| 4.4a | InterMTS is enabled for IBC-coded blocks in AMVP mode. | Qualcomm P. Garus JVET-AE0116 | InterDigital K. Naser JVET-AE0211 |
| 4.4b | Test 4.4a + IntraTMP using interMTS instead of intraMTS kernels | Qualcomm P. Garus JVET-AE0116 | InterDigital K. Naser JVET-AE0211 |
| 4.4c | IntraMTS disabled for IntraTMP | Qualcomm P. Garus JVET-AE0116 | InterDigital K. Naser JVET-AE0211 |
| 5 In-loop filtering |  |  |  |
| 5.1a | CCSAO with temporal history offset | Kwai C.-W. Kuo JVET-AE0151 | Qualcomm N. Hu JVET-AE0209 |
| 5.1b | Test 5.1a + extended edge classifier | Kwai C.-W. Kuo JVET-AE0151 | Qualcomm N. Hu JVET-AE0209 |
| 5.2a | Applying fixed filters to samples before DBF | Qualcomm N. Hu JVET-AE0139 | Kwai C.-W. Kuo JVET-AE0193 |
| 5.2b | Test 5.2a + extended classifiers for fixed filters | Qualcomm N. Hu JVET-AE0139 | Kwai C.-W. Kuo JVET-AE0193 |
| 5.2c | Test 5.2b + applying the second fixed filter to outputs of the first fixed filter | Qualcomm N. Hu JVET-AE0139 | Kwai C.-W. Kuo JVET-AE0193 |
| 5.3 | Combination of Test 5.1b and Test 5.2c | Kwai C.-W. Kuo Qualcomm N. Hu JVET-AE0152 | Alibaba J. Chen |

Category 1: Partitioning prediction

Test 1.1: Partitioning prediction (JVET-AE0132)

In this test, a temporal partitioning is introduced, where for each block, the allowed partitioning splits are predicted according to the minimum QT/MTT split and the average QT/MTT split obtained from a temporal area:

- if the current QT depth is less than the temporal minimum QT depth minus 1, only the QT split is allowed,

- if the current QT depth is less than the temporal average QT depth minus 1, no split, QT and TT splits are allowed, and BT split is allowed if TT is selected in a parent node,

- if the temporal maximum MTT depth and less than the maximum MTT depth of the current block, the depth is decreased except if the current QP is less than the temporal QP,

- if the temporal maximum MTT depth is large than the maximum MTT depth of the current block and if the current depth is equal to the current QT depth, the maximum MTT depth is incremented,

- if the maximum QT depth of the current frame is larger than the maximum multi-tree depth and if the temporal maximum MTT depth is equal to the maximum MTT depth of the current frame, and if the temporal QT depth is less than the QT depth of the current frame, the maximum MTT depth of the current block is decreased.

The modifications on the maximum MTT depth are enabled only if the palette mode is disabled.

The temporal area is a collocated area from the same reference frame used for the temporal motion predictor.

JVET-AD0147 has memory requirement analysis where the largest memory which is required to store temporal depths for class A is estimated as 40.5 KB.

Test 1.1a: Partitioning prediction (as described above)

Test 1.1b: Test 1.1a where MTT depth increment is enabled unconditionally. It affects classes A and B, class A1 encoder runtime reaches 105.8%.

Test 1.1c: Use partition prediction as encoder only optimization.

Questions:

- Is the partitioning only from the reference frame? A: Yes.

- What is the additional memory requirement? A: Approx. 40 Kbyte per reference picture in 4K.

- Does CABAC parsing require the temporal partitioning info? A; Yes.

It was commented that the memory requirement is significantly less than for motion vectors.

The cross-checkers pointed that the implicit splitting performed at the picture boundary is useless (no gain when removing it, but more complicated).

It was commented that for LB there is no gain (or small losses).

Even though there was no strong objection, from the questions raised the implementation of the idea, it appears not mature. The gain is relatively low, even though it could be attractive in terms of run time, it introduces additional temporal dependencies that might be undesirable.

No action was taken on this.

Category 2: Intra prediction

Test 2.1: Block vector guided CCCM (JVET-AE0100)

In this test, a co-located luma BV is used to determine the reference area for calculating the CCCM parameters. Then the reference area in luma and corresponding area in chroma channel is used to calculate the CCCM parameters. The prediction uses the calculated model parameters and co-located luma samples to do the CCCM prediction as follows:

predChromaVal = c0C + c1N + c2S + c3E + c4W + c5P(C) + c6P(N) + c7P(S) + c8P(W) + c9P(E)+ c10B,

where spatial components are as shown in the next figure, the nonlinear term P is represented as power of two of the corresponding luma sample, and B is the bias term.

The mode is enabled only in intra slices and is controlled by SPS flag.

Test 2.1a: the mode only uses BVs of the IBC coded blocks from co-located luma area.

Test 2.1b: the mode can use BVs of both IBC and IntraTMP coded blocks from co-located luma area.

Test 2.2: Bi-predictive GPM (JVET-AE0169)

In ECM-9.0, IBC GPM uses uni prediction for one partition and intra mode for the other one. In the test, the core GPM design is kept unchanged which uses 48 GPM modes and IBC merge candidate list, while it enables both partitions being predicted using IBC with different BVs.

Two flags are signalled to indicate the prediction modes of two partitions, the first flag indicates whether the first partition is intra predicted, and if not then the second flag is signalled to indicate whether intra prediction is used for the second partition.

The method is applied to SCC only.

Test 2.3: IBC BVP-merge and bi-predictive IBC merge (JVET-AE0169)

In Test 2.3a, IBC-BVP-merge, which is similar to AMVP-merge, derives one BV from IBC block vector prediction (BVP) and the second BV from IBC merge to form bi-prediction for IBC. Two different indices for the IBC BVP and the IBC merge candidates are signalled.

In Test 2.3b, bi-predictive IBC merge is introduced, and it is enabled together with the existed in the ECM MBVD and uni-merge (currently disabled by the encoder configuration for non-SCC classes). In bi-predictive IBC merge, two BVs from the existing IBC merge candidate list are derived, utilizing two different indices, which are signalled. Bi-predictive IBC merge is applied to IBC regular merge and IBC MBVD. Bi-predictive IBC merge, IBC MBVD, and IBC uni-merge are enabled for non-SCC classes and is tested together with Test 2.3a.

Test 2.4: IBC MBVD list derivation (JVET-AE0169)

In the test 2.4a, adaptive BVD offsets along MVBD directions and enabled for IBC MBVD mode. The MBVD candidates search is a two-step process, which starts with checking template SAD costs of offsets added to BVP along each direction with the interval of 1-pel. The second step of the search checks template SAD costs with 1/4-pel interval for the candidates around the selected candidates from the first step. For the integer MBVD (when existed in the ECM ph_fpel_mbvd_enabled_flag is 0), those intervals are multiplied by 4. The candidates with the lowest TM cost are included into the final MBVD list.

Test 2.4c is a combination of Test 2.4a, Test 2.3b, and Test 2.2c.

Test 2.5: Filtered IBC (JVET-AE0159)

In the test, additional filtered IBC mode is introduced, where a filter is applied to IBC predictor, which is derived by minimizing MSE between current and reference template.

Output of the filter is calculated as follows:

predLumaVal = c0C + c1N + c2S + c3E + c4W + c5P + c6B

The nonlinear term P is represented as power of two of the center sample C and scaled to the sample value range of the content:

P = ( C*C + midVal ) >> bitDepth

The bias term B represents a scalar offset between the input and output and is set to middle luma value (512 for 10-bit content).

This filtered mode is used as an additional mode for non-merge IBC blocks, and it is not used together with IBC-LIC, IBC-CIIP or RR-IBC. For IBC merge modes, this filtering mode is inherited when merge mode list is constructed.

In Test 2.5a, the mode flag is signalled conditioned on the IBC-LIC flag, so when the IBC-LIC flag is true, the flag is signalled and used to indicate whether the tested mode is applied to the current block or not.

In Test 2.5b, the mode flag is signalled before IBC-LIC flag, and the model does not have non-linear term.

In Test 2.5c, the model of Test 2.5a and signalling of Test 2.5b are used, different aspects of encoder design is chosen from both tests considering the best trade-off.

Test 2.6: IBC with non-adjacent spatial candidates (JVET-AE0094)

In the test, several new candidates obtained from the BVs of non-adjacent spatial neighbouring blocks are added to the candidate lists of IBC merge modes and IBC AMVP. A pattern similar to the non-adjacent spatial candidates of regular inter is used to obtain non-adjacent BVs shown in the below figure. The obtained BVs are inserted between the adjacent spatial candidates and the HBVP candidates for both IBC merge and IBC AMVP. Additionally, the restriction that spatial candidates are not used to construct IBC merge candidate list for a 44 CU is removed.

In Test 2.6a, the same reference area as for non-adjacent regular merge is reused for the IBC, where up to 4 rows/columns of spatial neighbouring blocks (in terms of the CU height/width) of the CU can be used as reference for IBC AMVP, and up to 7 rows/columns of the spatial neighbouring blocks (in terms of the CU height/width) of the CU are used for IBC merge.

In Test 2.6b, for both IBC AMVP and IBC merge, the spatial neighbouring blocks located up to 4 rows/columns of spatial neighbouring blocks (in terms of the CU height/width) of the CU can be used as reference. Additionally, the restriction that spatial candidates are not used for the IBC merge of a 44 CU in ECM-9.0 is removed.

Test 2.6c is the combination of Test 2.6a and Test 2.6b, where the sampling method in Test 2.6a is applied. Additionally, the removal of the restriction that spatial candidates cannot be used for IBC merge of a 4x4 CU from Test 2.6b is used.

Test 2.7: Cross-component merge mode with temporal candidates (JVET-AE0043)

In the current CCP merge mode, a CCP merge candidate list contains spatial adjacent, spatial non-adjacent, and history-based candidates coded in CCLM, MMLM, CCCM, GLM, chroma fusion, and CCP merge modes. After including these candidates, default models can be included to fill the remaining empty positions in the merge list if necessary. To remove redundant CCP models in the merge list, pruning operations are applied. The candidates in the list are reordered based on the SAD costs obtained using the neighbouring template of the current block.

In the test, two types of candidates, namely temporal candidates and shifted temporal candidates, are additionally included in the merge list for non-intra slices. Temporal candidates are added to the merge list after the spatial adjacent candidates. Shifted temporal candidates are added after the history-based candidates.

CCP merge mode is allowed to be applied in non-intra slices for chroma blocks with sizes less than or equal to 16.

Temporal candidates

Temporal candidates are selected from the collocated picture, the positions and inclusion order of the temporal candidates, which are the same as those for temporal candidates in inter merge mode of ECM-9.0, is shown in the next figure below.

Figure: Positions of the temporal candidates

An inclusion order of the temporal candidates is C01  C02  …  C010. If C0i is outside of the picture/slice boundary and C1i is inside of the picture/slice boundary, C1i will be used instead of C0i, where 1 ≤ i ≤ 10. Otherwise, the next inclusion position is checked.

Shifted temporal candidates

Shifted temporal candidates are also selected from the collocated picture. As depicted in the next figure below, the position of the collocated block is shifted by a selected neighbouring motion vector. Consequently, the positions of C0i and C1i are shifted by the same neighbouring motion vector. The inclusion order of shifted temporal candidates is the same as that of temporal candidates.

The adjacent motion vector is selected from the motion vectors of the neighbouring blocks ordered as follows: L0B1  L1B1  L0A1  L1A1  L0B0  L1B0  L0A0  L1A0  L0B2  L1B2. The first motion vector, which uses the collocated picture as the reference picture, is selected. If no such motion vector is found, no shifted temporal candidate is added.

Figure: Positions of the shifted candidates (left) and selecting the neighbouring motion vector (right)

Test 2.8: Extrapolation filter-based intra prediction mode (JVET-AE0076)

In the test, extrapolation filter-based intra prediction is introduced with 3 types of reconstructed areas and 3 filter shapes as shown in the next figure below, a choice of reconstructed area and filter shape is signalled.

The size of reconstructed area depends on the min(blockWidth, blockHeight) and the selected filter shape. For example, when the current block is an 8x16 block and the selected filter shape is 4x4. The aboveSize of reconstructed area is min(8, 16) + 4 – 1 = 11, and the leftSize of reconstructed area is min(8, 16) + 4 – 1 = 11.

Figure: Three types of reconstructed areas (top) and three filter shapes (bottom)

The selected filter moves in the selected reconstructed area with a one-sample step to collect input samples and output samples, then the filter coefficients are derived using CCCM solver.

In the current block, the recursive predictor is derived from the top-left to the bottom-right position by a diagonal prediction order as shown in the next figure below, where the predicted results of the previous diagonal are used.

Figure: Example of generating predictions for different positions in the current block by a diagonal order

To reduce the prediction error, the min and max values from the neighbouring reconstructed area are applied to restrict the output range of each predicted value.

The predicted samples are calculated as follows,

predx,y=clip(min,max,i=014(ci×(tx-xoffset, y-yoffset-offset))+offset)

where predx,y is the predicted value at (x, y) in the current block. min,  max and offset are derived in the selected reconstructed area, ci is the ith coefficient of the derived filter, tx-xoffset, y-yoffset is reconstructed or predicted value used for prediction in the current position.

The tested mode is applied in I-slices only and is only used for the block sizes less than or equal to 32x32.

Test 2.9: Extended search areas for IntraTMP mode (JVET-AE0077)

In the test, IntraTMP search areas are extended by including the bottom-left R5 and top-right R6 areas relative to the current block as they may also be available as shown in the next figure below. In ECM-9.0, the region scan order is R4, R1, R2, and R3. In the tested method, the region scan order is R4, R5, R6, R1, R2, and R3.

Figure: Extended IntraTMP search areas (R5 and R6)

In addition, the current ECM-9.0 always searches all available positions within the current CTU while it may be beyond the specified search range or width/height. In the test, all search areas are restricted to be within the specified search range or width/height specified by (searchRangeWidth, searchRangeHeight) as illustrated in the next figure below. The maximum horizontal and vertical search sizes, i.e., searchRangeWidth and searchRangeHeight, are set to max(5W, 64) and max(5H, 64) respectively. It is not allowed to search any positions outside the specified search range even those available positions are within the current CTU.

Figure: Search areas restriction

Test 2.10: IBC-LIC extension (JVET-AE0078)

In Test 2.10a, IBC-LIC is extended by including 3 additional modes:

- Use only left or use only above LIC template in addition to the current L-shape template,

- Extend the MMLM to IBC-LIC, which allows IBC-LIC to have two linear models in one CU for L-shape template.

Large block-size constraint for ECM IBC-LIC, tested separately in Test 2.10b, is additionally removed in Test 2.10a.In Test 2.10b, large block-size constraint for ECM IBC-LIC is removed, so IBC-LIC can be applied to the CU whose block size is larger than or equal to 32.

Test 2.11: Harmonization of IBC HMVP and IBC-LIC (JVET-AE0084)

In ECM-9.0, the inter LIC flag can be inherited from a HMVP candidate. However, the IBC-LIC flag is not inherited when the motion information from an IBC HMVP candidate is used.

In Test 2.11a, IBC-LIC flag is inherited from an IBC HMVP candidate, which is similar to the inter LIC case.

Test 2.11b: Test 2.11a + Test 2.10a

Results for AI/RA

Test 2.1 is a chroma tool, main benefit on screen content. The gain is small, but it has no impact on run time, and implementation is straightforward according to cross-checkers. Adoption was also supported by independent experts.

Decision: Adopt JVET-AE0100, test 2.1b.

Tests 2.2..2.4 target several aspects of improving IBC in context with GPM and bi-prediction, 2.3b also provides gain on camera-captured content (but had been reported previously to not give gain in case of screen content). In the combination 2.4c (2.2c+2.3b+2.4a), 2.3b is disabled for screen content, and 2.2c is disabled for camera-captured content, whereas 2.4a is enabled for both. gives best gain overall, including screen content. However, the results show that 2.4a does not have benefit for camera capured content when combined with 2.3b, whereas for screen content the gain from 2.2c and 2.4a is somewhat additive.

Decision: Adopt JVET-AE0169 Test 2.2c, in CTC enabled only for screen content.

Decision: Adopt JVET-AE0169 Test 2.3b, in CTC enabled only for camera-captured content.

Decision: Adopt JVET-AE0169 Test 2.4a, in CTC enabled only for screen content.

For 2.5c, a discussion was performed about the benefit of a combination test when the individual results of the elements being combined are not available. Proponents are strongly discouraged to withdraw proposal elements when still including them in a combination, as this will not allow to judge the individual benefits.

Individual results of 2.5a and 2.5b were presented in an update of the summary report on Wednesday July 12.

Gains appear to be somewhat additive, but encoding run time is significantly higher in 2.5b than it is in 2.5c. According to proponents, an encoder with less RDO checks was used in 2.5c. This is also the likely reason for losing a slight bit rate reduction in 2.5c, compared to 2.5b.

It was asked whether the encoder search strategy of 2.5c is the same as 2.5a, such that it could be confirmed that the additional benefit of 2.5c comes by the signalling method of 2.5b, and its non-linear filter term? According to proponents, main gain comes from signalling.

Results were matched according to cross-checker (verbally reported in session, document to be uploaded).

Decision: Adopt JVET-AE0159 Test 2.5c (enabled only for screen content).

Same situation was initially found with 2.6c.I Individual results of 2.6a and 2.6b were presented in an update of the summary report on Wednesday July 12.

Results indicate that the combination of 2.6c provides more gain than the individual parts 2.6a and 2.6b. Gains cannot be expected to be additive, as the basic approach is identical. It can however be concluded that the element of 2.6b (removing the restriction on 4x4 blocks) is also beneficial in combination with 2.6a. The proposal was supported by independent experts (including cross-checkers).

Decision: Adopt JVET-AE0094 Test 2.6c (enabled only for screen content).

Test 2.7 is targeting improvement of chroma, by adding additional temporal candidates in cross-component merge (which had been adopted in the last meeting). Candidates are stored in an 8x8 grid.

It was asked what the benefit of shifted candidates is. According to proponents, it had been 10% of the total gain in the original contribution. It was argued that this element of the proposal might be difficult to implement in hardware. It was however asserted by JVET that this is not of highest important in this stage of exploration, and technology investigated in EE should normally be adopted as proposed.

Decision: Adopt JVET-AE0043 test 2.7.

Test 2.8: It was asked how the mode is used when reconstructed area is not available. It is disabled. It was further pointed out that the gain seems to be very small for small blocks, where the pixel dependency might be critical in real-world implementation. The cross-checker confirms that such dependency is probably present, but would nevertheless support this tools at this stage of exploration.

2.5% increase of encoder run time is pointed out to be not a good tradeoff for 0.16% bit rate reduction in AI. For RA, the gain is less, as it is only applied in I slices currently.

Further study was recommended (in the EE) to reduce encoder run time, and also apply in inter slices.

Reduction of pixel dependency also would be important, but probably is more difficult to achieve.

Test 2.9: Increase in decoder run time by 2.5% by additional search operations. Encoder run time also slightly increased. One cross-checker points out that the increase of decoder run time may be caused by pixel-wise availability checks. It was also pointed out that this is straightforward extension of search range relative to ECM 9, and the tradeoff between gain and run time is still in an attractive range.

Decision: Adopt JVET-AE 0077 test 2.9.

Tests 2.10/2.11 are only beneficial for screen content. From comparing 2.10a/b, it cannot be concluded that removing the large block size constraint is beneficial, as there are no results for the IBC-LIC extension with the large block size constraint of current ECM (such a test had originally been planned, but was not conducted). Proponents are requested to present results of a variant of 2.10a with the constraint, and also in combination 2.11.b to assess whether the gain is additive with 2.11a.

Test 2.11a is a minor change which gives some benefit in class F without impact on run time. However, it should also be enabled for camera-captured content to avoid another high-level flag (according to proponents, no losses would be expected). Therefore, the new combination test 2.11b should be conducted such that the method 2.11a is also used for camera-captured content.

A revised version of JVET-AE0024 was presented on July 16 1700. From the new results, it became obvious that the large block size constraint had no impact on run time, but results are slightly better without the constraint. From the results, 2.10a or 2.11b (combining 2.10a with 2.11a) would be candidates. According to the cross-checker, 2.11a is straightforward to implement. Also the runtime variations are low.

Decision: Adopt JVET-AE0084 Test 2.11b. In the combination, both 2.10a and 2.11a should be enabled for camera captured content, also in CTC.

Inter prediction

Test 3.1: Cross-component residual model for inter prediction (JVET-AE0059)

Cross-component residual model to predict chroma samples from reconstructed luma samples when the block uses inter prediction or intra block copy (IBC) is tested. As illustrated in the next figure below, the cross-component filters are derived using the prediction blocks of luma and chroma. The derived filters are applied to the reconstructed luma block and blended with the prediction blocks of chroma to produce the final chroma prediction blocks. In the blending process the filtered reconstructed luma blocks use blending weight of 0.75 and chroma prediction blocks use blending weight of 0.25.

Figure: Cross-component residual model

Model uses 8-tap filter consisting of 6 spatial luma samples shown in the next figure below, a nonlinear term, and a bias term as follows:

predChromaVal = c0 L0+ c1L1 + c2L2 + c3L3 + c4L4 + c5L5 + c6 nonlinear((L0+L3+1) >> 1) + c7 B,

Figure: 2 Luma samples L0,..,L5 in relation to the chroma sample C.

The filter coefficients are derived using ECM’s division-free Gaussian elimination method and the necessary offsets are applied to samples prior to filter derivation.

For filter coefficient derivation at most 256 chroma samples are used.

The mode flag is only signalled if the TU’s luma Cbf is non-zero and the CU’s predMode is either MODE_INTER or MODE_IBC.

Test 3.1a: Cross-component residual model, where offsets for division free operation are obtained by averaging the samples in the block.

Test 3.1b: Cross-component residual model, where offsets for division free operation are obtained by averaging the four points correspond to the top-left, top-right, bottom-left and bottom-right corners of the blocks.

Test 3.2: Bi-predictive GPM (JVET-AE0046)

In the test, the existing uni-predictive GPM is extended to allow usage of bi-predictive motion vectors for generating motion compensated prediction samples for inter partitions. The method consists of the following elements:

The first element conditionally invokes the existing extraction process that extracts uni-predictive motion vectors from the initial list. The extraction process is invoked only for small blocks 8x8, 16x8 and 8x16. For other larger blocks, the extraction process is bypassed, so the initial list (which may contain merged Bi-MVs) is directly used as the final GPM merge list. The generation of the initial list is the same as before (i.e., the normal merge list generation without any candidate reordering) except that when generating the initial list for larger blocks (i.e., blocks with the extraction process bypassed), the motion vector difference threshold for controlling whether a candidate can be added into the initial list is increased to be one full sample distance.

The second element modifies GPM-MMVD to support bi-predictive motion vector as the base vector. For low-delay pictures, the signalled MVD is applied on top of the L0 and L1 motion vector as in the existing merge MMVD design. For non-low-delay pictures, the bi-predictive motion vector is converted into a uni-predictive motion vector first and then the MVD is applied on top.

The third element modifies GPM-TM to also support bi-predictive motion vectors.

The last element is to enable the BDOF based mv refinement as in the multi-pass DMVR on top of the associated bi-predictive motion vectors for each inter partition.

Test 3.3: High-Accuracy template matching (JVET-AE0087)

In the ECM, template matching is a decoder-side MV derivation method to refine the motion information of the current CU by finding the closest match between a template (i.e., top and/or left neighbouring blocks of the current CU) in the current picture and a block (i.e., same size to the template) in a reference picture.

When TM is used for bi-prediction, the following steps are applied:

- The initial motion vector of list 0 (MV0) is refined to derive a refined MV (MV’0) and a TM cost C0 is calculated;

- The initial motion vector of list 1 (MV1) is refined to derive a refined MV (MV’1) and a TM cost C1 is calculated;

- If C0 is larger than C1, MV’1 is used to derive a further refined MV (MV’’0) by refining MV’0; Otherwise, MV’0 is used to derive a further refined MV (MV’’1) by refining MV’1 .

In high-accuracy template matching, three aspects are tested:

- In Test 3.3a, an additional refinement is applied to TM for bi-prediction, when MV’0 is refined in step 3), MV’’0 is used to derive MV’’1 by refining MV’1; Otherwise, MV’’1 is used to derive MV’’0 by refining MV’0.

- In Test 3.3b, the diamond search pattern used in TM is modified from 8-point to 16-point.

- In Test 3.3c, TM for bi-prediction is enabled when DMVR condition is satisfied.

Test 3.3d: Test 3.3a + Test 3.3c.

Test 3.3e: Test 3.3a + Test 3.3b + Test 3.3c.

Test 3.4: OBMC with HPel flag and BCW weights (JVET-AE0196)

In the current OBMC design, the motion compensation of the neighbouring blocks and subblocks performed by OBMC only considers the motion vectors and reference pictures.

In the test, HPel flags and BCW weights of the neighbouring blocks and subblocks are considered in the OBMC. When processing the top and left borders, the neighbouring BCW weights and HPel flags are used in addition to the motion vectors and reference pictures. The neighbouring HPel flag allows setting the correct AMVR index. For the internal subblock boundaries, the BCW and AMVR indexes of the current CU are also used.

It has to be noticed that the integration of adopted contribution JVET-AD0213 (Bi-prediction LIC) turns out to already include main aspect of this test (BCW weights in OBMC) in the ECM-9.0 which was not the case in ECM-8.0.

Test 3.5: Iterative BDOF pass in multi-pass DMVR (JVET-AE0065)

In the ECM, a multi-pass DMVR has three passes of motion refinement. In the first pass, bilateral matching (BM) is applied to the coding block (CB) to get the block level refined MV pair. In the second pass, the block is divided into 16×16 subblocks and on top of block level refined MV pair obtained in the first pass, BM is applied on each 16×16 subblock to get a 16×16 subblock level refined MV pair. In the third pass, BDOF based MV refinement is applied on the 4×4 or 8×8 subblocks depending on the CB size.

In the test, the current multi-pass DMVR is extended by adding another BDOF-based motion refinement pass as the 4th pass of the multi-pass DMVR (i.e., the 2nd round of BDOF-based DMVR).

Also, the subblock size of the proposed round of the BDOF-based DMVR is adaptively selected depending on the area (i.e., width×height) of the coding block. For blocks smaller than 1024 pixels, the subblock size of 4×4, and otherwise 8×8 is used.

Test 3.7: RPR with new filters for scale factor 1.25x and 1.33x (JVET-AE0150)

In the test, additional set of 8-bit precision RPR filters consisting of three tables (respectively 12-tap for luma, 6-tap for chroma, and 10-tap for affine) is introduced with the target to improve the performance for scaling ratios 1.25x and 1.33x. These new set of filters is used for scaling ratios in between 1.1x and 1.35x.

In the tests, two PSNR values are used:

- PSNR1: it is measured on the decoded picture size by calculating MSE for each picture, accumulating MSEs for all pictures and calculating average PSNR from the accumulated MSE.

- PSNR2: it is measured after upsampling of the decoded picture if the decoded picture size is in smaller resolution, the existed ECM interpolation filters are used for the upsampling.

Test 3.8: Combination of Test 3.3e and Test 3.5 (JVET-AE0091)

Results for RA/LB

Test 3.1: Increase of encoder run time should be expected in test 3.1b (RA results not complete yet).

It was commented that the method should better be called “cross component chroma prediction in inter”, as it is not the residual that is predicted.

It was also commented that a blending with 0.75/0.25 weights was newly introduced, which according to proponents provides better performance in blending. It was requested to provide quantitative information about the gain by blending

Results were presented Wednesday 8:30. Test 3.1b has slightly less encoder run time (which is explainable by having less checks), and bit rate reduction is almost identical. This was also confirmed by cross-checker.

It was asked why a different cross-component model was used than the one in the ECM. It was answered by proponent that downsampling is avoided. The unification might be desirable, but is not of prior importance at this moment.

Decision: Adopt JVET-AE0059 Test 3.1b.

Test 3.2: 0.2% bit rate reduction in RA with slight increase in encoding time. According to one cross-checker it may increase the worst-case memory access, which may however not be too severe, as the proposal is not used in case of small block size, and multi-hypothesis is also used in other tools of ECM. Several experts (including cross-checkers) supported the proposal. It was however suggested to modify the software such that the memory for motion vector storage is reduced in the encoder.

Decision: Adopt JVET-AE0046 Test 3.2.

Test 3.3x, 3.5, and combination 3.8: Test 3.3c seems to have no benefit standalone, but according to proponents has some benefit in combination with 3.3a. In combination, gains of 3.3e and 3.5 are somewhat additive in RA, whereas only 3.3e contributes gain LB (BDOF of 3.5 not applicable in LB). Several experts supported the combination, and no objection was raised.

Decision: Adopt JVET-AE0091 Test 3.8.

Test 3.4 does not show benefit in compression, might be due to new adoptions in ECM 9. No action was taken on this.

Test 3.7 is about RPR (non-CTC test conditions deduced from RPR specific VTM test conditions which only used 1.5x and 2x). The results indicate that improvement is possible by using different filters for small factors of resolution change. Though it is not of prior importance for the exploration, it may be interesting to provide such implementation in the software for experimentation of interested parties.

Decision (SW): Adopt JVET-AE0150 (not in CTC, not in the ECM description).

Transforms and coefficient coding

Test 4.1: Shifting quantizer center (JVET-AE0125)

In the test, a quantization offset is added to the quantized level, the offset is quantization index dependent, a look-up table is used to derive the offset T[yi].

xi=1024-T[yi]*Q-1yi+T[yi]*Q-1y'i>>10,

where x is the dequantized coefficient, y and y’ are the quantization indices, Q-1 is the dequantization operation, T is a look-up table

T=[0,63, 31, 21, 15, 12, 10, 9, 7, 7, 6, 5, 5, 4, 4, 4, 3, 3, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 1,…,1].

Test 4.2: Large NSPT (JVET-AE0086)

The large NSPT kernels are tested for 4x32/32x4 and 8x32/32x8 blocks, of which kernel matrix dimensions are 20x128 and 24x256. Therefore, 20 and 24 transform coefficients are generated by applying the two types of kernel matrices, respectively, which are placed from DC position following scan order. The remaining 108 and 232 positions in each transform block are zeroed-out, respectively.

Large NSPT kernels are applied in the same way as for other block sizes 4x4, 4x8/8x4, 4x16/16x4, 8x8, and 8x16/16x8 and those NSPT kernels are not changed.

Test 4.3: Context modelling for transform coefficients for LFNST/NSPT (JVET-AE0102)

In the ECM, the causal 2D neighbourhood of coefficients is used to model the context to parse the sig_coeff_flag, gt1_flag, gt2_flag as shown in the next figure below. For LFNST, DCT-II coefficients are placed into the coefficient block using diagonal reordering.

Figure: Context modelling for LFNST coefficients in the ECM

In the test, when LFNST/NSPT is applied, the previous 5 coefficients in the coding order are used for context derivation instead of 2D neighbourhood.

Figure: Context modelling for LFNST coefficients in TEST4.3

In the ECM, the lfnstIdx is signalled after all the coefficients in a CU. In the test, lfnstIdx is signalled after all last_sig_coeff_pos syntax elements in a CU since lfsntIdx is required for parsing the transform coefficients.

Test 4.4: InterMTS for IBC and IntraTMP (JVET-AE0116)

InterMTS enabled for IBC and IntraTMP is tested. The following tests are performed:

Test 4.4a: InterMTS is enabled for IBC-coded blocks in AMVP mode. MTS is not enabled for IBC merge.

Test 4.4b: InterMTS is enabled for IBC-coded blocks in AMVP mode as in Test 4.4a. In addition, IntraTMP is utilizing InterMTS instead of IntraMTS. Consequently, the number of MTS candidates for IntraTMP has been fixed to 4 instead of adaptively setting it to 1, 4 or 6 as in IntraMTS.

Test 4.4c: IntraMTS is disabled for IntraTMP coded blocks.

Results for AI/RA (LFNST not enabled in LB)

Test 4.1: Shifting quantizer center provides 0.1% gain without any complexity impact.

Decision: Adopt JVET-AE0125 Test 4.1.

Test 4.2: Reasonable tradeoff runtime vs. bit rate reduction, additional memory consumption of larger kernels not relevant in cntext of exploration.

Decision: Adopt JVET-AE0086 Test 4.2.

Test 4.3: It was commented that the usage of immediately preceding coefficients might have some latency impact (VVC did not use coefficients from same diagonal for that reason), but for the purpose of exploration several experts (including cross-checkers) supported the proposal.

Decision: Adopt JVET-AE0102 Test 4.3.

Test 4.4x: Enabling MTS for IBC does not provide relevant gain to justify the increase of encoding time (caused by additional RDO checks). No action was taken on this.

Loop filtering

Test 5.1: CCSAO with extended edge classifiers and history offsets (JVET-AE0151)

In ECM-9.0, CCSAO uses band and edge classifiers switched at CTB level. Classifier parameters/offsets are derived at encoder and signalled for each slice independently.

In Test 5.1a, to reduce the signalling overhead, CCSAO inheritance scheme is introduced, where the offsets/parameters of some coded pictures are stored at both encoder and decoder which are allowed to be used as the CCSAO offsets/classifiers of future pictures. An index is signalled in SH to indicate which candidate in the CCSAO parameter storage is selected for the current slice. The candidates in the CCSAO parameter storage are updated in a FIFO manner and refreshed at IDR pictures.

In Test 5.1b, on top of Test 5.1a, beside the existing CCSAO edge classification, one new edge classification is added, which is a subset of the original one with less edge range divisions.

orginal: iE=d<0?d<-T?0:1:d<T?2:3added:  iE=                     d<T?0:1

where iE is calculated by comparing a sample difference d in one direction with a threshold T.

Additionally, the component used for edge classification can be selected from one of all three components. Same to the existing CCSAO design, the selected edge classifier and edge component are decided by encoder and signalled in the SH.

Test 5.2: Improved fixed filters for ALF (JVET-AE0139)

In ECM-9.0, two Laplacian-based classifiers (one for each fixed filter) are applied to a 2x2 block. In each classifier, activity and directionality values are derived based on vertical, horizontal, and diagonal gradients using a window surrounding each 2x2 block. Then a class index is determined based on the activity and directionality values. Two 13x13 diamond shaped fixed filters are selected from the two filter sets by using the derived two class indices. Then the two fixed filters (fi with i=0, 1) are applied to the ALF input samples. Finally, a signalled filter is applied to the ALF input samples, samples before the deblocking filter (DBF), outputs of the two fixed filters, output of a gaussian filter and the residual data.

In Test 5.2a, both fixed filters are applied to samples before DBF and ALF input, where additional diamond 9x9 filter is used for the samples before DBF. The shape of the first fixed filter applied to the ALF input samples is reduced from 13x13 to 9x9, and the shape of the second fixed filter, which is 13x13, applied to ALF input is unchanged as shown in the table below.

|  | ECM-9.0 | Proposed method |  |
| --- | --- | --- | --- |
|  | ALF input | Samples before DBF | ALF input |
| Fixed filer f0 | 13x13 | 9x9 | 9x9 |
| Fixed filer f1 | 13x13 | 9x9 | 13x13 |

In Test 5.2b, on top of Test 5.2a, the classifiers of the fixed filters are extended. For each 2x2 block, the mean value of a surrounding window is calculated. Then, for each sample of this window, the difference between the sample value and the mean value is calculated. A scaling factor is determined based on the activity value derived from a Laplacian classifier. The square root of the sum of the squared differences is further quantized to C' by a scaling factor. The value of C' is an integer between 0 and 7, inclusively. With i=0, 1, let Ci denote the classifier from the classifier of i-th fixed filter in ECM-9.0. Then the proposed class index Ci' is derived as

Ci'= C'*896+Ci.

The total number of the fixed filters is not changed.

In Test 5.2c, on top of Test 5.2b, fixed filter f1 is applied to outputs of f0 (instead of ALF input) and samples before DBF.

Test 5.3: Combination of Test 5.1b and Test 5.2c (JVET-AE0152)

One expert commented that the changes in 5.1a and 5.1b are straightforward and supported it.

It was report that an additional buffer of approcimately 5Kbyte is necessary for storage.

It was confirmed that the data flow in 5.2x is not changed relative to the current ECM.

It can be expected that the gains of CCSAO modifications and ALF modifications are non-overlapping, and it was also reported that preliminary results of 5.3 indicate additive gains.

Decision: Adopt JVET-AE0151 Test 5.1b.

Decision: Adopt JVET-AE0139 Test 5.2c.

It was commented that the results of 5.3 (combination test) are expected to be provided for information.

### EE2 contributions: Enhanced compression beyond VVC capability (24)

There was no presentation or discussion about specific proposals in this category.

For actions decided to be taken, see section 5.2.1, unless otherwise noted.

JVET-AE0043 EE2-2.7: Cross-component merge mode with temporal candidates [H.-Y. Tseng, C.-M. Tsai, C.-Y. Chuang, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, O. Chubach, Y.-W. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-AE0220 Crosscheck of JVET-AE0043 (EE2-2.7: Cross-component merge mode with temporal candidates) [P. Bordes (InterDigital)] [late]

JVET-AE0046 EE2-3.2: Bi-predictive GPM [R. Yu, P. Wennersten, J. Enhorn, K. Andersson (Ericsson)]

JVET-AE0199 Crosscheck of JVET-AE0046 (EE2-3.2: Bi-predictive GPM) [Y. Ahn (LGE)] [late]

JVET-AE0249 Crosscheck of JVET-AE0046 (EE2-3.2: Bi-predictive GPM) [Y. Kidani, K. Kawamura (KDDI)] [late]

JVET-AE0059 EE2-3.1: Cross-component residual model (CCRM) for inter prediction [P. Astola, J. Lainema (Nokia)]

JVET-AE0236 Cross-check of JVET-AE0059 “EE2-3.1: Cross-component residual model (CCRM) for inter prediction” [F. Le Léannec (InterDigital)] [late]

JVET-AE0253 Cross-check of JVET-AE0059 (EE2-3.1: Cross-component residual model (CCRM) for inter prediction) [J. Arumugam (Ittiam)] [late]

JVET-AE0065 EE2-3.5: Iterative BDOF pass in multi-pass DMVR [M. Salehifar, Y. He, K. Zhang, H. Liu, L. Zhang (Bytedance), J. Chen, R.-L. Liao, X. Li, Y. Ye (Alibaba)]

JVET-AE0200 Crosscheck of JVET-AE0065 (EE2-3.5: Iterative BDOF pass in multi-pass DMVR) [Z. Lv (vivo)] [late]

JVET-AE0076 EE2-2.8: An extrapolation filter-based intra prediction mode [L. Xu, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AE0217 Crosscheck of JVET-AE0076 (EE2-2.8: An extrapolation filter-based intra prediction mode) [H.-J. Jhu (Kwai)] [late]

JVET-AE0077 EE2-2.9: Extended search areas for IntraTMP mode [Y. Yu, L. Zhang, F. Wang, J. Gan, H. Yu, L. Xu, Z. Xie, D. Wang (OPPO), Y. Ma, H. Zhang, J. Huo, F. Yang, X. Gao (Xidian University), X. Xiu, N. Yan, C. Ma, H. Jhu, C. Kuo, W. Chen (Kwai)]

JVET-AE0210 Crosscheck of JVET-AE0077 EE2-2.9: Extended search areas for IntraTMP mode [K. Naser (InterDigital)] [late]

JVET-AE0216 Crosscheck of JVET-AE0077 (EE2-2.9: Extended search areas for IntraTMP mode) [P.-H Lin (Qualcomm)] [late]

JVET-AE0245 Crosscheck of JVET-AE0077 (EE2-2.9: Extended search areas for IntraTMP mode) [X. Li (Alibaba)] [late]

JVET-AE0078 EE2-2.10: IBC-LIC extension [Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AE0157 Crosscheck of JVET-AE0078(EE2-2.10: IBC-LIC extension) [W. Yin (Bytedance)] [late]

JVET-AE0084 EE2-2.11: Harmonization of IBC HMVP and IBC-LIC [N. Zhang, K. Zhang, L. Zhang (Bytedance), Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AE0222 Crosscheck of JVET-AE0084 (EE2-2.11: Harmonization of IBC HMVP and IBC-LIC) [C. Ma (Kwai)] [late]

JVET-AE0086 EE2-4.2: Large NSPT [M. Koo, J. Zhao, J. Lim, S. Kim (LGE)]

JVET-AE0118 Crosscheck of JVET-AE0086 (EE2-4.2: Large NSPT) [P. Garus (Qualcomm)] [late]

JVET-AE0087 EE2-3.3: High-Accuracy template matching [Y. Wang, K. Zhang, L. Zhang (Bytedance)]

JVET-AE0239 Crosscheck of JVET-AE0087 (EE2-3.3: High-Accuracy template matching) [J. Chen (Alibaba)] [late]

JVET-AE0091 EE2-3.8: Combination of Test 3.3e and Test 3.5 [M. Salehifar, Y. Wang, Y. He, K. Zhang, H. Liu, L. Zhang (Bytedance), J. Chen, R.-L. Liao, X. Li, Y. Ye (Alibaba)]

JVET-AE0223 Crosscheck of JVET-AE0091 (EE2-3.8: Combination of Test 3.3e and Test 3.5) [C. Ma (Kwai)] [late]

JVET-AE0094 EE2-2.6: IBC with non-adjacent spatial candidates [C. Ma, X. Xiu, W. Chen, H.-H. Jhu, C.-W. Kuo, N. Yan, X. Wang (Kwai), Y. Wang, K. Zhang, L. Zhang (Bytedance)]

JVET-AE0240 Crosscheck of JVET-AE0094 (EE2-2.6c: IBC with non-adjacent spatial candidates) [J. Chen (Alibaba)] [late]

JVET-AE0100 EE2-2.1: Block vector guided CCCM [R. G. Youvalari, D. Bugdayci Sansli, P. Astola, J. Lainema (Nokia)]

JVET-AE0226 Crosscheck of JVET-AE0100 (EE2-2.1: Block vector guided CCCM) [M. Abdoli (IRT b-com)] [late]

JVET-AE0102 EE2-4.3: Context modelling for transform coefficients for LFNST/NSPT [P. Nikitin, M. Coban, C. S. Coban, B. Ray, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AE0225 Crosscheck of JVET-AE0102 (EE2-4.3: Context modelling for transform coefficients for LFNST/NSPT) [M. Abdoli (IRT b-com)] [late]

JVET-AE0116 EE2-4.4: InterMTS for IBC and IntraTMP [P. Garus, M. Coban, B. Ray, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AE0211 crosscheck of JVET-AE0116 EE2-4.4: InterMTS for IBC and IntraTMP [K. Naser (InterDigital)] [late]

JVET-AE0125 EE2-4.1: Shifting quantization center [M. Balcilar, K. Naser, F. Galpin, F. Le Léannec (InterDigital)]

JVET-AE0149 Crosscheck of JVET-AE0125 (EE2-4.1 Shifting Quantizer Center) [M. Coban (Qualcomm)] [late]

JVET-AE0132 EE2-1.1: Partitioning prediction [G. Laroche, P. Onno (Canon)]

JVET-AE0235 Cross-check of JVET-AE0132 “EE2-1.1: Partitioning prediction” [R. Utida, F. Le Léannec (InterDigital)] [late]

JVET-AE0139 EE2-5.2: Improved fixed filters for ALF [M. Karczewicz, N. Hu, H. Wang, V. Seregin (Qualcomm)]

JVET-AE0193 Crosscheck of JVET-AE0139 (EE2-5.2: Improved fixed filters for ALF) [C.-W. Kuo (Kwai)] [late]

JVET-AE0150 EE2-3.7 RPR with new filters for scale factor 1.25x and 1.33x [J. Samuelsson-Allendes, S. Deshpande (Sharp)]

JVET-AE0204 Crosscheck of JVET-AE0150 (EE2-3.7 RPR with new filters for scale factor 1.25x and 1.33x) [Z. Zhang (Qualcomm)] [late]

JVET-AE0151 EE2-5.1: CCSAO with extended edge classifiers and history offsets [C.-W. Kuo, X. Xiu, W. Chen, H.-J. Jhu, N. Yan, C. Ma, X. Wang (Kwai)]

JVET-AE0209 Crosscheck of JVET-AE0151 (EE2-5.1: CCSAO with extended edge classifiers and history offsets) [N. Hu (Qualcomm)] [late]

JVET-AE0152 EE2-5.3: Combination of Test 5.1b and Test 5.2c [C.-W. Kuo, X. Xiu, W. Chen, H.-J. Jhu, N. Yan, C. Ma, X. Wang (Kwai), M. Karczewicz, N. Hu, H. Wang, V. Seregin (Qualcomm)]

JVET-AE0241 Crosscheck of JVET-AE0152 (EE2-5.3: Combination of Test 5.1b and Test 5.2c) [J. Chen (Alibaba)] [late]

JVET-AE0159 EE2-2.5: Filtered Intra Block Copy (FIBC) [H.-J. Jhu, X. Xiu, C.-W. Kuo, W. Chen, N. Yan, C. Ma, X. Wang (Kwai), B. Ray, M. Coban, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AE0228 Crosscheck of JVET-AE0159 (EE2-2.5: Filtered Intra Block Copy (FIBC) [L. Zhang, Y. Yu (OPPO)] [late]

JVET-AE0169 EE2-2.2/EE2-2.3/EE2-2.4: Bi-predictive IBC GPM, bi-predictive IBC, and IBC MBVD list derivation for camera captured and screen contents [Y. Kidani, H. Kato, K. Kawamura (KDDI), C. Ma, X. Xiu, W. Chen, H.-J. Jhu, C.-W. Kuo, N. Yan, X. Wang (Kwai), G. Verba, Z. Zhang, P. Nikitin, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AE0197 Crosscheck of JVET-AE0169 (EE2-2.2c: Bi-predictive IBC-GPM and EE2-2.3c: Combination of 2.3b and 2.2c)) [W. Yin (Bytedance)] [late]

JVET-AE0205 Crosscheck of JVET-AE0169 (Test EE2-2.3c and EE2-2.4c) [R. Yu (Ericsson)] [late]

JVET-AE0196 EE2-3.4: OBMC with HPel flag and BCW weights [A. Robert, F. Le Léannec, F. Galpin, T. Poirier, Y. Chen (InterDigital)] [late]

### EE2 related contributions (2)

Contributions in this area were discussed at 1100–1125 on Wednesday 12 July 2023 (chaired by JRO).

JVET-AE0176 EE2-related: Enhancements on CCRM [Z. Deng, K. Zhang, L. Zhang (Bytedance)]

This proposal presents the results of a multi-model CCRM mode and a CCRM merge mode. In EE2-3.1a, a single-model CCRM is investigated. In this contribution, two methods are presented to further improve the coding efficiency of CCRM. In method #1, a multi-model CCRM mode is proposed. Samples are separated into two groups and each group can apply its own CCRM model. There is no flag signalling involved in the proposed method, and the final CCRM mode is implicitly determined by comparing decoder derived costs of the two CCRM modes with single or multiple models. In method #2, a CCRM merge mode is proposed, wherein the CCRM model is inherited from a candidate in the CCRM merge list. Simulation results based on ECM-9.0 are reported as below:

Method #1 :

RA { -0.20%, -1.66%, -1.89%, 101.8%, 100.0% }; LDB { -0.01%, -3.86%, -3.46%, 101.9%, 99.7% }

Method #2 :

RA {}; LDB {}

Simulation results versus EE2-3.1a are listed as below :

Method #1 :

RA { -0.03%, -0.07%, -0.07%, 100.4%, 100.1% }; LDB { -0.12%, 0.39%, 0.42%, 100.7%, 99.8% }

Method #2 :

RA {}; LDB {}

Relative to EE test 3.1a, small benefit in RA; in LB, some gain in luma, but loss in chroma.

According to proponents of 3.1, it would be a straightforward add-on, and might be interesting to investigate on top of the adopted method 3.1b.

It was agreed to investigate this in the upcoming EE.

JVET-AE0186 Non-EE2-2.1: Block Vector Guided Chroma Direct Mode [K. Naser, F. Le Léannec, P. Bordes, Y. Chen, G. Rath (InterDigital)] [late]

Chroma direct mode (DM) is a special intra mode for chroma component where the same prediction mode of the collocated luma block is used for the current chroma block. In ECM-9.0, it is asserted that when the collocated luma block is coded with IBC or IntraTMP, DM is set to planar. This contribution proposes to use a method similar to EE2-2.1, where the collocated luma block vectors are used to obtain its reference block intra mode. The following results are obtained:

AI -0.03%	-0.15%	-0.10% with EncT 100% and DecT 100%

Only the co-located BV is used.

According to proponents, this should be independent from adoptions to ECM10, and also give some gain on camera-captured content. Several experts expressed interest to study this further.

It was commented that there is a drop in class D, and also some inconsistency in terms of chroma.

It was requested to provide per-sequence results, and RA results in xls spreadsheet.

Was initially considered candidate for EE, per-sequence results were results were presented in v2, and relatively large variations.

Further study was encouraged, not in the EE.

### ECM modifications and software improvements beyond EE2 (46)

Contributions in this area were discussed at 1125–1300 and 1430–1940 on Wednesday 12 July 2023, at 1500–2000 on Thursday 13 July 2023, at 1800–2000 on Friday 14 July 2023 (chaired by JRO, and at 1600–1830 on Saturday 15 July 2023 (chaired by JRO and Yan Ye).

#### Intra (18)

JVET-AE0056 Non-EE2: Changes on TIMD and SGPM for reducing ECM decoder complexity [Z. Fan, Y. Yasugi, T. Ikai (Sharp)]

This proposal aims to reduce the complexity of the ECM decoder. There are two changes made in this proposal for ECM.

- The first change is replacing timdHor and timdVer with dimdHor and dimdVer in the SGPM candidate list. This allows for the SGPM method to be used without the need to run the additional template matching process of TIMD.

- The second change involves reducing the number of template matching / number of candidate in TIMD, reducing the time required for the TIMD method.

Based on these changes, and the experiment results are as follows:

AI : 0.04%	0.08%	0.10%	99%	95%
RA: 0.05% -0.01% -0.03% 100% 101%

It was commented that reduction of decoder complexity is not of primary importance in this exploration, and the proposal would incur performance losses.

No action was taken on this.

JVET-AE0201 Crosscheck of JVET-AE0056 (Non-EE2: Changes on TIMD and SGPM for reducing ECM decoder complexity) [F. Wang (OPPO)] [late]

JVET-AE0071 AHG12: DIMD Merge [S. Blasi, I. Zupancic, J. Lainema (Nokia)]

This contribution proposes to merge the Decoder-side Intra Mode Derivation (DIMD) information from neighbouring blocks to predict the current block. When neighbouring blocks encoded with DIMD are available, the DIMD histograms are combined to form a new DIMD Merge histogram for the current block. DIMD Merge modes and weights are computed based on this merged histogram.

It is reported that adding the proposed DIMD Merge to ECM in AI configuration provides -0.08%, 0.01% and 0.07% BD-rate impact for Y, U and V, respectively. In RA configuration the impact is reportedly x%, y%, z% for Y, U and V, respectively. The encoder and decoder runtimes are reportedly minimally changed with respect to ECM.

Gain is slightly higher for larger resolution (A classes)

Is it necessary to store histograms of neighbouring PUs? Yes, up to 13. Methods to reduce this are under consideration according to proponents. It was recommended to investigate reduction of storage needs.

Usage of this mode is up to 45%.

It was agreed to investigate this in the EE. Also analysis of memory needs is requested.

JVET-AE0073 Non-EE2: IBC-LIC Model Merge mode [L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO)]

This contribution proposes an IBC-LIC model merge mode that inherits IBC-LIC model parameters from spatial adjacent and non-adjacent neighbours, history candidates, or default models. The proposed method is implemented on top of ECM-9.0 software, it shows coding gains as follows:

AI: { -0.56% Y, -0.50% U, -0.45% V, 101.9% EncT, 100.9% DecT } for class F

{ -0.17% Y, -0.16% U, -0.15% V, 101.9% EncT, 101.3% DecT } for class TGM

RA: { -0.39% Y, -0.37% U, -0.35% V, 100.6% EncT, 102.3% DecT } for class F

{ -0.04% Y, -0.08% U, -0.09% V, 101.1% EncT, 102.4% DecT } for class TGM

It was asked why the gain is lower in class TGM. Probably, current IBC-LIC already works well in that class.

Maximum 12 candidates are used.

It was pointed out that depending on the potential adoption of EE2 2.10/2.11, some gain might be lost. Proponents believe that the multiple models still would have benefit.

Proponents were asked to run tests in combination with the EE2 candidates and provide results. This could be candidate for EE if it still provides gain.

Results were provided in v3 of the contribution. In combination with the adopted method EE2-2.11b, gain is -0.29/-0.28/-0.59 for class F, and -0.13/-0.16/-0.13 for TGM for AI (RA not available).

For class F, only half of the gain is retained.

There was no attractive tradeoff with almost 2% encoder runtime increase.

Further study was recommended.

JVET-AE0268 Crosscheck of JVET-AE0073 (Non-EE2: IBC-LIC Model Merge mode) [Y. Wang (Bytedance)] [late]

JVET-AE0075 Non-EE2: On IntraTMP block vector [Y. Yu, L. Zhang, L. Xu, H. Yu, J. Gan, F. Wang, Z. Xie, D. Wang (OPPO)]

IntraTMP block vectors (BVs) are stored for coding future blocks. Currently, an intraTMP BV may have a quarter-pel resolution but it is always saved in a full-pel resolution after coding an intraTMP block. This contribution proposes to store IntraTMP BVs in their full resolution. In addition, it is also proposed that intraTMP BVs are stored for HMVP. On top of ECM-9.0, the simulation results are reported as follows:

AI: 		-0.07% (Y), -0.04% (U), -0.08% (V), 98.1% (EncT), 99.9% (DecT) for class F

-0.10% (Y), -0.11% (U), -0.14% (V), 99.0% (EncT), 99.1% (DecT) for class TGM

RA: 		-0.07% (Y), -0.05% (U), -0.11% (V), 101.8% (EncT), 99.9% (DecT) for class F

-0.03% (Y), 0.00% (U), -0.07% (V), 101.5% (EncT), 100.0% (DecT) for class TGM

There was a very small gain, and results were reported only for screen content. It was pointed out that it might be more beneficial in case of camera-captured content, where also IBC merge will be enabled in next ECM.

It was agreed to investigate this in the EE. The two aspects should be reported separately, and also results with camera content should be reported.

JVET-AE0246 Crosscheck of JVET-AE0075 (Non-EE2: On IntraTMP block vector) [X. Li (Alibaba)] [late]

JVET-AE0202 Crosscheck of JVET-AE0075 (Non-EE2: On IntraTMP block vector) [W. Lim, S.-C. Lim (ETRI)] [late]

JVET-AE0085 Non-EE2: Direct block vector (DBV) mode extension [M. Hong, J. Choi, N. Park, J. Lim, S. Kim (LGE)]

This contribution proposes direct block vector (DBV) mode extension. Firstly, the number of locations to find a block vector are extended from 5 to 17 keeping the existing search range. Also, DBV candidate list is composed of block vector of collocated luma blocks that is coded with IBC or IntraTMP mode, and DBV candidate list is sorted in ascending order of SAD cost. The best one of DBV candidate list is used to apply block prediction for chroma. Experimental results of the proposed method for class F and class TGM are reported as follows:

For AI configuration:

- F: -0.08%/-0.19%/-0.09%, 100.1%(EncT), 100.1%(DecT),

- TGM: -0.10%/-0.15%/-0.24%, 99.3%(EncT), 99.4%(DecT).

For RA configuration:

- F: -0.10%/0.06%/-0.49%, 100.4%(EncT), 100.0%(DecT),

- TGM: -0.03%/-0.05%/-0.02%, 99.7%(EncT), 99.6%(DecT).

The additional complexity is not justified by the extremely small gain (<0.1% for screen content only).

No action was taken on this.

JVET-AE0263 Crosscheck of JVET-AE0085 (Non-EE2: Direct block vector (DBV) mode extension) [H. Huang (Qualcomm)] [late]

JVET-AE0098 AHG12: On the chroma DBV mode [H. Huang, H. Wang, V. Seregin, M. Karczewicz (Qualcomm)]

This contribution proposes to enable chroma DBV mode in single-tree configuration and to remove the template matching based refinement in chroma DBV mode. The simulation on top of ECM-9.0 reports the following results {Y, U, V, EncT, DecT}:

- AI: -0.01%, -0.03, 0.02%, 99.9%, 94.5%

- RA: *-0.02%, -0.06%, 0.00%, 97.9%, 98.3%

- AI (single tree): -0.19%, -0.24%, -0.19%, 100.3%, 100.3%

The results in the abstract are including removal of the refinement. Table 3 of the contribution includes results without removing the refinement, but not complete yet.

Single Tree would be non-CTC.

Some concern is raised about removing the refinement, as in some classes losses occur, the overall gain is minor, and it is not critical in terms of complexity. This may not be worthwhile to consider.

The aspect of enabling chroma DBV in single-tree mode appears interesting, but this would only affect inter slices in current CTC (for which no results are available). Complete results were presented in v3 of the contribution (including those without removal of refinement). In this context, beyond the removal of TM based refinement, also an element of removing some of the regular intra modes was investigated separately.

One argument made against considering the removal of TM refinement and some of the regular intra modes is rather small, so it may not be worthwhile considering.

On the other hand, it was argued by proponents that in particular the TM refinement has some impact on processing and appears useless in terms of compression benefit.

Enabling chroma DBV in single-tree mode is generally agreed to be beneficial. In CTC, this would only have influence on inter slices. In non-CTC, it would also have benefit on intra slices, but single-tree mode in intra would still be worse in performance than dual-tree mode.

It was commented that there are probably many elements in the ECM which would give tiny gains when removed under certain configurations or combinations with other (most likely later adopted) tools. Taking action on these would be necessary in a real standardization activity, but is not important in an exploration.

It was agreed to investigate enabling of DBV in single tree in the EE.

JVET-AE0203 Crosscheck of JVET-AE0098 (AHG12: On the chroma DBV mode) [L. Xu, Y. Yu (OPPO)] [late]

JVET-AE0292 Crosscheck of JVET-AE0098 (AHG12: On the chroma DBV mode) [M. Hong (LGE)] [very late]

Still missing at the end of the meeting, but provided later.

JVET-AE0110 Non-EE2: An improved method for IntraTMP fusion [S. Peng, D. Jiang, J. Lin, C. Zheng, K. Fu, P. Zhang (Dahua)]

This contribution proposes a modified method for IntraTMP fusion. In this method, the filtered matched blocks are used in the IntraTMP fusion process. The proposed method was implemented on top of ECM-9.0, it reportedly provides {Y, U, V} BD-reduction with encoder and decoder runtimes as follows:

- AI: -0.05%, -0.01%, -0.02%, 104.4% (EncT), 102.8% (DecT)

No reasonable tradeoff for gain vs. computation time was shown (both encoder and decoder), which is caused by substantial additional processing.

No action was taken on this.

JVET-AE0120 Non-EE2: Intra Angular Prediction Extension [J. Fu, J. Zhang, C. Jia, S. Ma (PKU)] [late]

In the ECM, current angular intra prediction is based on linear projection. 1 of 67 intra prediction modes is chosen to be the angle of projection, and the location of reference pixel are calculated, then the value of predicting pixel is calculated through 6-tap intra filter and the value of reference pixel.

In this contribution, it is proposed to extend the current linear projection function into quadratic projection function that takes two intra prediction modes. When two intra prediction modes are belonging to the set of vertical intra prediction mode, the location of reference pixel is inferred by the location of predicting pixel and the quadratic function generated by two intra prediction modes.

This document illustrates test results of the proposed method.

On top of the ECM-9.0, simulation result of the proposed method is reported as:

- AI: -0.02%, -0.01%, 0.21%, 124.6%, 107.9%

The coding gain was low, and not consistent for the classes. The run time was much too large.

This was an interesting idea, but not relevant for action as long as the performance is not better.

JVET-AE0124 Non-EE2: Fixes related to Intra TMP [J.-L Lin, P.-H Lin, Y.-J Chang, V. Seregin, M. Karczewicz (Qualcomm)]

In ECM-9.0, it is asserted that the cost list for Intra TMP candidates is not updated correctly and the BV storage in Intra TMP is not aligned with the IBC. This contribution proposes to fix those issues, and the simulation results on top of ECM-9.0 are summarized as follows:

TEST1:

- AI(Y/U/V): 0.00%/ 0.01 %/ 0.03 %, EncT 100 %, DecT 99 %

- RA(Y/U/V): X.XX %/ X.XX %/ X.XX %, EncT XX %, DecT XX %

TEST2:

- AI(Y/U/V): -0.01%/ 0.03 %/ 0.02 %, EncT 100 %, DecT 104* %

- RA(Y/U/V): X.XX %/ X.XX %/ X.XX %, EncT XX %, DecT XX %

TEST3:

- AI(Y/U/V): 0.00%/ 0.03 %/ 0.04 %, EncT 101 %, DecT 103* %

- RA(Y/U/V): X.XX %/ X.XX %/ X.XX %, EncT XX %, DecT XX %

For “Test 1”, it is not obvious that a problem exists with the current implementation. The “Fix 1” even has a (very small) loss in chroma. No action on this.

“Test 2” is somewhat equivalent to an aspect of JVET-AE0075, which was also promoted to be investigated in EE. The results on camera-captured content indicate that with current ECM (not using IBC merge in camera-captured classes) no gain can be achieved, but that might change due to upcoming EE adoptions.

Decoding time is not measured reliably.

It was agreed to investigate Test 2 in the EE together with JVET-AE0075.

JVET-AE0247 Crosscheck of JVET-AE0124 (Non-EE2: Fixes related to Intra TMP) [X. Li (Alibaba)] [late]

JVET-AE0258 Crosscheck of JVET-AE0124 aspect 1 (Non-EE2: Fixes related to Intra TMP) [F. Wang, Y. Yu (OPPO)] [late]

JVET-AE0130 Non-EE2: DIMD with filtered template [C. Zhou, Z. Lv (vivo)]

This contribution proposes a template filtering method for DIMD mode. The template is filtered using a 3x3 filter operator before obtaining the gradient histogram. The 3x2 and 2x3 gradient operators are used instead when calculating a gradient. The experimental results are summarized as follows:

On top of ECM-9.0

- AI: -0.09% (Y), 0.01% (U), 0.04% (V), 106% (EncT),  100% (DecT)

- RA : -0.xx% (Y), 0.xx% (U), 0.xx% (V), 10x% (EncT), 10x% (DecT)

This has a large increase in encoding time (likely caused by one more RD check for the decision of enabling or not enabling the filtering). Without the enabling flag, loss would occur.

In the current version, the tradeoff between compression and run time is not reasonable. It was agreed to investigate this in the EE, but the encoding time needs to be largely reduced, without any substantial loss in performance.

It would also be desirable to avoid the modification of gradient operators, e.g. by using unfiltered samples for gradient computation at poisitions where no filtered samples are available.

It was agreed to investigate this in the EE.

JVET-AE0242 Crosscheck of JVET-AE0130 (Non-EE2: DIMD with filtered template) [J. Chen (Alibaba)] [late]

JVET-AE0137 Non-EE2: On TIMD fusion [P. Andrivon, M. Blestel (Ofinno)]

The contribution proposes to conditionally introduce in the TIMD fusion process a non-angular intra predictor (DC or Planar) additionally to the two selected TIMD modes. On top of ECM-9.0, experimental results reportedly show the following results:

- AI: -0.04% / 0.06% / 0.04%, with 100% EncT and 101% DecT

Why is loss observed for class F? Could be caused by smoothing which is not good for screen content which has more sharp edges. It was also pointed out that in some cases losses occur in chroma.

From the preliminary results, it also appears that more cases of loss occur in RA.

Further study was encouraged to make the benefit more consistent over different classes.

JVET-AE0138 Non-EE2: Reference sample interpolation for intra prediction [G. Rath, F. Le Leannec, F. Urban, F. Racape, T. Dumas (InterDigital)]

For intra prediction along angular directions, this contribution reports the results of using a simple 4-tap filter in the case of DIMD, TIMD, ISP, and wide angular modes instead of the existing 6-tap filters in ECM 9. The filtering can be implemented as a linear interpolation together with an update term, where the update term is excluded if the two nearest reference samples are sufficiently close. Besides BD-rate performance gain, the advantages of the proposed method are lower complexity, no memory storage of filter coefficients, and a uniform method for both Luma and Chroma components.

It is reported that, on top of ECM-9.0, the proposed interpolation together with unified PDPC yields -0.10%, -0.14%, -0.05% and -0.02%, -0.17%, -0.12% in AI and RA configurations respectively for encoding and decoding runtimes of 100% and 99%, and 100% and 99%, respectively.

Standalone gain of 4-tap filter would be 0.05% (with decreased runtime), unified PDPC gives 0.04% standalone (with increased run time).

It was questioned if the filter (which has some additional computation for adaptation) is really simpler than the current 6-tap cubic filter, implementation-wise. Further, it was asked why with the “unified PDPC” the filter is only used at the top side.

The 6-tap filter is not replaced everywhere in the ECM. Therefore, yet another filter is introduced.

For RA, performance is not consistent over different sequences/classes. Also, a somewhat different PDPC is used there. Definitely, the same design should be used in AI and RA.

Various concerns were raised whether the design is appropriate.

Further study was recommended for a more consistent design, and more consistent results over different sequences and configurations.

JVET-AE0208 Crosscheck of JVET-AE0138 (Non-EE2: Reference sample interpolation for intra prediction) [P. Nikitin (Qualcomm)] [late]

JVET-AE0158 AHG12: LUT-based angle calculation for DIMD [A. Aminlou, J. Lainema (Nokia)]

In the current version of DIMD in ECM9.0, for each neighbouring sample of the block, a ratio value is calculated based on the gradient in horizontal and vertical directions, and then the intra mode is derived by searching the ratio value inside an angle table. The ratio has high dynamic range between 0 and 65536 (i.e., 16 bits), and searching the ratio value inside the angle table is a sequential process. This contribution proposes to calculate the ratio value in logarithm-like domain and replace the ratio searching with a look-up-table. The following BD-rate and runtime impact observed when it is implemented in ECM-9:

- AI { 0.01%, 0.05%, 0.05%, 99%, 97%}

- RA { } // RA simulations were still running.

Actual reduction in decoding time should be in the range of 0.5% (numbers above not reliable).

It was considered not of primary importance to make DIMD more “implementation friendly” at the current stage. It is also pointed out that a major code cleanup of the overall design would be desirable.

Further study was recommended.

JVET-AE0166 AHG12: IntraTMP and IBC search area harmonization [D. Ruiz Coll, B. Chen, P. Andrivon, M. Blestel (Ofinno)]

This contribution explores the search area harmonization of the IBC and IntraTMP tools. IBC and IntraTMP use the reconstructed samples of the current picture to select the prediction block for the current block. The IBC search area is fixed to a specific number of CTUs according to the current block locations, while the IntraTMP search area is based on the block size, which exceeds the IBC search region for some block sizes. In order to harmonize the decoded buffer for both tools, this contribution investigates two aspects, the constraint of the current IntraTMP search regions and the extent of such regions to the IBC search region.

On top of ECM-9.0, simulation results are reported as follows:

Test-1: IntraTMP search sub-regions constraint to IBC search area.

- AI {-0.00% Y, -0.00% U, -0.00% V, 99.3% EncT, 99.1% DecT};

Test-2: Adjustment of the IntraTMP sub-regions to the IBC search area.

- AI: Class F : {-0.44%, -0.57%, -0.38%, 102.4%, 109.5%},

- AI: Class TGM : {-0.68%, -0.68 %, -0.69%, 102.5%, 114.5%};

The presentation deck was to be provided. It was commented that “test 1” seems to have some relation to elements of EE2-2.9. Harmonization is not of prior importance at this moment, and the benefit in runtime is relatively small.

For test 2, it was commented that the tradeoff compression vs. encoder/decoder run time is not appropriate. In adoptions from EE, screen content gains around 0.5% usually came with almost no increase in run time.

Not attractive for EE, further study was encouraged for a better tradeoff.

JVET-AE0167 AHG12: DBV improvement [L. Xu, Y. Yu, H. Yu, D. Wang (OPPO)]

When dual-tree block partition is enabled for an intra slice in ECM9, DBV is one of the chroma coding modes if one of the five corresponding luma positions is coded with either intraTMP or IBC. If the current luma block is coded with IBC or IntraTMP and DBV is not used, this contribution proposes to use one of the 5 prediction modes, i.e., DIMD, DC, Planar, Horizontal, and Vertical, without the DM flag to code the corresponding chroma block. On top of ECM-9.0, the simulation results are reported as follows:

- AI: 		-0.02% (Y), -0.09% (U), -0.08% (V), xx% (EncT), xx% (DecT)

- RA: 		-0.01% (Y), -0.13% (U), -0.01% (V), xx% (EncT), xx% (DecT)

Encoding/decoding times are not reliable,

Somewhat similar approach to JVET-AE0186, but that proposal showed slightly better (though still low) results. Proponents of JVET-AE0186 believe that the two proposals are not overlapping and might give additive gain.

It was agreed to investigate this in the EE.

JVET-AE0265 Crosscheck of JVET-AE0167 (AHG12: DBV improvement) [H. Huang (Qualcomm)] [late]

JVET-AE0184 AHG12: TIMD with IntraTMP and IBC candidates [K. Naser, F. Le Léannec, P. Bordes, F. Galpin, A. Robert (InterDigital)] [late]

TIMD is a an intra tool that uses template analysis to generate the prediction signal. Specifically, a set of intra candidate modes are tested on the template and the template distance (SATD) is measured to deduce up to two best modes. This contribution proposes extending the candidate modes with IBC and IntraTMP candidates obtained from neighbouring blocks. The proposed modification is tested on top of ECM-9.0 and the following results are obtained:

- AI: -0.10%	-0.13%	-0.16%	with EncT 101% and DecT 102%

IBC and IntraTMP are added as candidates into TIMD search, which is then performed as in current ECM. Up to 6 additional candidates.

Several experts expressed interest, as this is relatively straightforward and tradeoff is reasonable.

It was requested to provide per-sequence results, and RA results in xls spreadsheet.

It was agreed to investigate this in the EE.

JVET-AE0185 AHG12: IntraTMP with neighbouring candidates [K. Naser, F. Le Léannec, A. Robert, F. Galpin (InterDigital)] [late]

In ECM-9.0, IntraTMP search is performed within predefined search area to obtain the best candidate block vectors. This contribution proposes to augment the TM search with the block vectors of neighbouring PU’s that are coded with IBC or IntraTMP. The purpose is to improve the IntraTMP by further including probable candidates during the TM search. On top of ECM-9.0, the following results are obtained:

- AI -0.09%	0.18%	0.17%	100%	100%

Five additional candidates in IntraTMP. Search area is not going beyond IBC search range. Only uses BVs that are already stored in current ECM.

Where does chroma loss for some cases come from? This was not analysed.

Tradeoff is reasonable.

It was requested to provide per-sequence results, and RA results in xls spreadsheet, and an provide an answer to the question on partial chroma losses.

In an update (v3) following further investigations by proponents, it was stated that some negative interaction with DBV was found which was responsible for the inconsistency in chroma losses. Was also sent to cross-checkers, but not completed yet.

Are additional candidates checked in the updated version? Yes, likely more than 5 candidates could be checked as maximum (not exactly known), but it was claimed still search area is not going beyond IBC search range, and only uses BVs that are already stored in current ECM.

Run times of the new method are not reported. It was also commented that the new changes are not sufficiently described in v3.

It was also commented that there might be interaction with some of the new ECM adoptions.

Interest was expressed in the benefit of the method, but considering that this was a very late contribution (in terms of the new elements that were added, which were not documented yet), further study was recommended until the next meeting.

JVET-AE0198 AHG12: Extended Search Region for IntraTMP [K. Naser, P. Bordes, F. Le Léannec, F. Galpin (InterDigital)] [late]

This contribution proposes an extension to the search range of IntraTMP tools. The extended search range is the same one as of IBC mode. It is based on adding new search region with specific subsampling factor to control the number of added SAD checks.

On top of ECM-9.0, simulation results are reported as follows:

AI:

CTC: 		-0.03%	0.24%	0.02%	102%	103%

classF: 		-0.40%	-1.66%	-0.28%	102%	103%

ClassTGM:	-0.99%	-1.38%	-1.20%	103%	104%

It was commented that the tradeoff compression vs. encoder/decoder run time is not appropriate. In adoptions from EE, screen content gains around 0.5% usually came with almost no increase in run time.

Search region is not enlarged, but somewhat re-arranged in sub-regions. Similar concepts as in JVET-AE0166, but various aspects (subsampling, clipping) ar different.

This had interesting gain for screen content, but no good tradeoff with encoder runtime (in particular for CTC).

Results were somewhat inhomogeneous over classes, and there were chroma losses in several cases.

It was requested to provide per-sequence results, and RA results in xls spreadsheet.

This was not attractive for the EE, further study was encouraged for a better tradeoff.

JVET-AE0214 AHG12: on Intra CIIP [K. Naser, P. Bordes, F. Galpin, K. Reuzé (InterDigital)] [late]

IntraCIIP is a method proposed in JVET-AC0201 and studied in last EE under the test of EE2-1.18 (JVET-AD0177). This tool was not tested on top of the adoption of several IntraTMP improvement of EE2-1.20j (JVET-AD0086). It is reported that IntraCIIP provides further coding gain compared to last EE as it benefits from the overall improvement of IntraTMP. The following result are obtained on top of ECM-9.0

- AI: -0.08%	-0.02%	0.07%	100%	99%

It was commented that the contribution contains practically no description of the technology, just references to previous proposals. This makes it difficult to understand (see section 8.4, to which a remark about this problem has been included).

It was requested to provide per-sequence results, and RA results in xls spreadsheet.

Reasonable tradeoff, would be interesting to investigate whether the gain is retained with ECM10.

It was further commented that there might be interdependency with JVET-AE0184, these should be tested in combination.

It was confirmed that the method is the same as with InterCIIP, logic could be re-used.

It was agreed to investigate this in the EE.

#### Inter (11)

JVET-AE0047 AHG12: On GPM-MMVD [R. Yu (Ericsson)]

GPM-MMVD provides a way to adjust motion vectors for inter partitions of GPM blocks. Similarly to merge-MMVD, GPM-MMVD explicitly signals MVD which are applied on top of merged motion vectors. The MVD magnitudes that can be signalled in GPM-MMVD is limited within sets which always include fractional MVDs. It is proposed to let GPM-MMVD follow the merge-MMVD design to allow possibilities of disabling those fractional MVDs. The proposed modification is reported to be implemented in ECM-9.0 and tested for ClassA1 and ClassA2 under the ECM RA CTC. The BD-rate impact is reported to be -0.01% for ClassA1 and 0.01% for ClassA2.

In v2 of the contribution, additional results for disabling fractional MVD for GPM-MMVD on screen contents are provided.

It was commented that a per-sequence switch might give more gain. Currently, a switch is only performed for A classes.

The cross-checker mentioned that there might be more advantage in the context of the newly adopted bi-predictive GPM.

Further study was encouraged (not in the EE from this meeting).

JVET-AE0218 Crosscheck of JVET-AE0047 (AHG12: On GPM-MMVD) [Z. Deng (Bytedance)] [late]

JVET-AE0105 Non-EE2: Local illumination compensation with multiple templates [Y. Wang, K. Zhang, Y. He, H. Liu, L. Zhang (Bytedance)]

This contribution presents a method of local illumination compensation with multiple templates. Apart from using a template comprising neighbouring samples both top to and left to the current block, two new LIC modes using top-only or left-only template are proposed to derive the parameters of LIC. On top of ECM-9.0, simulation results of the proposed method are reported (estimated) as below:

- RA: {-0.09%%, -0.20%, -0.16%; 109.4%, 100.1%};

- LDB*: {-0.06%, -0.15%, -0.32%; 117.6%, 99.6%}.

Different shapes could be used for different motion vectors in same location.

Is there dependency of templates between adjacent blocks? Not currently exploited.

Possibility to reduce encoding time? No concrete ideas or preliminary experimentation were available for this yet.

This is also applied to chroma.

Several experts expressed this is an interesting approach. Further study was recommended on largely reducing encoding time while keeping the benefit in compression.

JVET-AE0224 Crosscheck of JVET-AE0105 (Non-EE2: Local illumination compensation with multiple templates) [C. Ma (Kwai)] [late]

JVET-AE0108 AHG12: DMVR with robust MV derivation [K. Andersson, R. Yu (Ericsson)]

In JVET-AD0045, it was reported that DMVR in VVC can sometimes produce dislocated subblocks with unreliable motion vectors which leads to subjective artefacts. It is found that multi-pass DMVR in the ECM can also have such a problem. It is therefore proposed to include additional criterions on top of bilateral block matching cost calculation for cases where there is a risk to have an unreliable motion vector to make sure that selected motion vectors result in subblocks that are aligned with neighbouring subblocks. The proposed method inherits low spatial activity check, subblock motion vector difference check and boundary difference checks from JVET-AD0045. When there is a risk for unreliable motion vectors, a boundary distortion metric is added to the block matching cost to bias the selection of motion vectors to produce subblocks that are aligned with above and left neighbouring subblocks.

Objective performance (BDR) versus ECM-9.0 is reported as:

For CTC at normal QP range:

- RA: Y: 0,00%, U -0,03%, V -0,01%	 Enc:100%	Dec:101%

For CTC at higher QP range QP= 40, 43, 47, 50:

- RA: Y -0,07%, U -0,06%, V -0,02%	 Enc: 100%	Dec:101%

It is also asserted that the proposal can improve subjective quality.

It was asked if it would help to apply deblocking at DMVR subblock boundary? Was tried, but did not help. The DMVR outlier also influences the inner part of the block.

Threshold is QP adaptive.

Why is there some gain for high QP? Might be due to better consistency of MVs in combination with almost no residual updates.

The contribution solves a problem that is known from VTM and was resolved by an encoder modification (which reduces the compression performance). In VVC, it could not be done decoder-side, as the problem was only detected after finalization of the standard.

It is asserted that it would be valuable to solve the problem in the ECM, where a decoder-side solution could be applied which comes without bit rate overhead.

It was agreed to investigate this in the EE. Comparison could be made in terms of bit rate saving against an encoder-only solution (in the ECM, similar to the one of VTM), and some subjective comparison should also be performed (possibly for subjective comparison also using non-CTC sequences, e.g. from verification tests).

JVET-AE0109 Non-EE2: LIC flag derivation of merge candidates with template costs [N. Zhang, K. Zhang, H. Liu, Y. Wang, L. Zhang (Bytedance)]

In the ECM, the LIC flag is inherited for a merge candidate. It is proposed to derive the LIC flag of a merge candidate based on template costs. A SAD-based template cost, denoted as C0, and a Mean Removal SAD (MRSAD)-based template cost, denoted as C1, are calculated. The LIC flag is set to be false, if C0 < C1 and is set to be true, if C0 >= C1.

On top of ECM9.0, simulation results of the proposed method are reported as below:

- RA: -0.01%, -0.06%, -0.01%, 99%, 99%; LB: -0.12%, -0.12%, -0.26%, 100%, 101%.

This showed interesting gain for LB.

One expert mentioned that the criterion is interesting. It was asked why the criterion is not effective for RA.

It was pointed out that potentially a similar approach could be used for LIC flag generally, not only in the merge case.

It was agreed to study this in the EE.

JVET-AE0117 Non-EE2: Enhanced subblock-based motion compensation [L. Zhao, K. Zhang, L. Zhang (Bytedance)]

This proposal presents the combined solution which was initially proposed in JVET-AC0187 and JVET-AD0235. Three aspects are proposed to enhance the subblock-based motion compensation.

- Aspect #1: TM-based subblock motion refinement.

- Aspect #2: Interweaved affine prediction.

- Aspect #3: RMVF candidate derivation with multiple CUs as input.

It is reported that on top of ECM-9.0, simulation results of the proposed method are as below:

- RA: -0.10 % (Y), -0.11 % (U), -0.01 % (V), 102.3% (EncT), 100.8% (DecT)

- LB: -0.05% (Y), -0.05% (U), -0.15 % (V), 106.8% (EncT), 102.7% (DecT)

This showed a better tradeoff than the previous proposal, but in particular for LB it was not yet attractive

It was asked what is the impact on the decoder complexity. Probably, subblock processing in generating the prediction becomes more complex.

What is the benefit of the three aspects, and what is their complexity impact?

Only applied to first candidate, out of complexity considerations

It was agreed to investigate this in the EE, but beyond answering the questions above, the encoder run time should be further reduced to make it attractive.

JVET-AE0129 AHG12 Template-based CIIP weight derivation [M.-H. Jia, Y.-L. Hu, S.-W. Xie, Y. Gao, C. Huang (ZTE)]

This contribution proposes to derive the inter weight and intra weight of CIIP based on template matching cost. Given a CIIP coded CU, the TM cost values are calculated with different CIIP weights setting. Then, the CIIP weight with minimum TM cost value is selected to fusion intra prediction and inter prediction of CIIP.

There were incomplete results, and classes B/C/D come with bit rate losses.

Further study was requested.

JVET-AE0136 Non-EE2: Fix on TM-based reordering for affine MMVD mode [D. Kim, W. Lim, J. Kim, S.-C. Lim, J. S. Choi (ETRI)]

This contribution addresses a bug in the implementation of template matching (TM)-based reordering for affine merge with motion vector difference (MMVD) mode in ECM-9.0. In the affine MMVD mode, the refinement positions for the base candidate motion are reordered based on template costs calculated using the sum of absolute difference (SAD). However, during the reordering process, there is an issue where the incorrect reference picture index is used when calculating the picture order count (POC) of the reference picture from list 1, leading to inaccuracies in template SAD cost calculations. To resolve this issue, the proposed fix involves replacing the reference picture index of list 0 with the correct reference picture index of list 1 in "sortAffineMergeCandidates" function. Experimental results reportedly shows that the proposed fix has a negligible impact on coding performance in the RA case. Therefore, it is recommended to incorporate the proposed fix in the next version of ECM.

Decision (BF/SW): Adopt JVET-AE0136.

JVET-AE0213 Crosscheck of JVET-AE0136 (Non-EE2: Fix on TM-based reordering for affine MMVD mode) [M. Salehifar (Bytedance)] [late]

JVET-AE0140 Non-EE2: LIC extensions [A. Filippov, V. Rufitskiy, K. Suverov (Ofinno)] [late]

The contribution proposes an additional method of filtering (interpolated) reference samples for inter predicted blocks. This method is complementary to LIC that utilizes 2-parametric linear model whereas the proposed technique involves high-order models like done in CCCM.

Preliminary results (RA) indicate -0.16%, -0.24%, -0.18% with 102.2% encoder and 100.6% decoder runtimes.

Higher-order models are more than an extension, mixture of explicit and implicit signalling, for the latter a “probe samples template” is introduced including also one more line/column of reference samples.

Up to 9 models were proposed (not all may be used).

More detail about models: Non-linear/linear/offset terms?

This was very interesting technology; more detail was requested, further study was recommended.

The method was assessed to be interesting for investigation in EE. However, provided that substantial detail is missing in the contribution, and it is not possible to clearly specify which aspects to be tested, further study was recommended until the next meeting.

JVET-AE0148 Non-EE2: Affine subblock BDOF refinement [Z. Zhang, H. Huang, J.-L Lin, V. Seregin, M. Karczewicz (Qualcomm)]

This contribution proposes to apply BDOF to refine subblock MV for an affine coded block. When an affine coded block meets the BDOF enabling condition and it is decided to do subblock MC, BDOF is applied to refine each subblock MV and do sample adjustments. The refined MVs are stored for future block MV prediction. The proposed method is also applied to SbTMVP mode.

It also proposes the AMVP-Merge mode for affine block. When a block is decided to be AMVP-Merge mode, a flag is signalled to further indicate the block is an affine block or not.

The proposed method was implemented on top of ECM-9.0 and the test results are as follows:

BDOF affine subblock MV refinement:

RA : -0.14% (Y), -0.17% (U), -0.07% (V), 103.0% (EncT), 104.4% (DecT)

BDOF affine subblock MV refinement and AMVP-Merge mode for affine:

RA : -0.19% (Y), -0.21% (U), -0.13% (V), 107.4% (EncT), 104.3% (DecT)

PROF will not be applied in case when BDOF is applied to affine subblocks. It is also not applied in case of pixel-based MC.

Blocksize cannot be lower than 4x4.

It was asked if the method including AMVP merge for affine is simpler for decoder?

It was suggested to test AMVP merge for affine separately (without BDOF for affine)

Can encoding time be reduced? There are some options, e.g. storage of computed MVs, skipping checks in certain cases.

It was agreed to investigate this in the EE.

JVET-AE0168 Non-EE2: Sample-based BDOF for Chroma [C.-C. Chen, H. Huang, Z. Zhang, C. S. Coban, V. Seregin, M. Karczewicz (Qualcomm)]

This contribution proposed to extend the notion of sample-based bi-directional optical flow (BDOF) to refining bi-prediction samples of chroma components. The same process of sample-based BDOF in the ECM is applied to each chroma sample in a coding unit, except that its per-sample motion refinement is inferred from its co-located luma components in the same coding unit. In other words, the per-sample motion refinement of a luma sample is shared across all color components. Then, each bi-prediction chroma sample is adjusted using an offset value computed by the same BDOF function of gradient vector and motion refinement. The experimental results on top of ECM-9.0 were reported, as follows:

- Class B: (Y) 0.01%, (U) -0.13%, (V) -0.03%, (Enc) 100.3%, (Dec) 100.2%

- Class C: (Y) -0.01%, (U) -0.15%, (V) -0.13%, (Enc) 99.6%, (Dec) 99.3%

- Class D: (Y) 0.00%, (U) -0.45%, (V) -0.35%, (Enc) 100.3%, (Dec) 99.9%

- Class F: (Y) -0.02%, (U) -0.11%, (V) -0.09%, (Enc) 100.3%, (Dec) 100.0%

- Class TGM: (Y) 0.01%, (U) -0.08%, (V) -0.08%, (Enc) 100.4%, (Dec) 99.5%

Results for A1/A2 were not available yet.

It is claimed that the gain comes for free, but other experts expressed opinion that additional processing (or logic in hardware) would be necessary. It was surprising that decoder run time is decreased; the measurements may be unreliable.

The opinion was expressed that the gain is too low to be considered. Apparently, BDOF does not have a significant benefit on chroma.

Class D has higher gain. Perhaps chroma has more structure in low resolution than in high resolution. Also most gain was from the basketballpass sequence.

No action was taken on this.

JVET-AE0177 Non-EE2: Local illumination compensation with slope adjustment [Y. Wang, K. Zhang, Y. He, H. Liu, L. Zhang (Bytedance)]

This contribution presents a method of local illumination compensation with slope adjustment, in which an adjustment parameter is used to modify parameters of LIC. On top of ECM-9.0, simulation results of the proposed method are reported (estimated) as below:

- RA*: {-0.10%, -0.22%, -0.17%; 112.9%, 100.3%};

- LDB*: {-0.10%, -0.18%, -0.31%; 120.8%, 100.4%}.

Slope adjustment was similar to CCLM.

Several experts said this is interesting and straightforward, but the tradeoff is not reasonable.

Further study was recommended on largely reducing encoding time while keeping the benefit in compression, and potentially further improvements in compression.

JVET-AE0248 Crosscheck of JVET-AE0177 (Non-EE2: Local illumination compensation with slope adjustment) [X. Li (Alibaba)] [late]

#### Cross Component Prediction (5)

JVET-AE0074 Non-EE2: Improvement on cross-component prediction merge mode [H. Huang, Y. Yu, H. Yu, D. Wang (OPPO)]

This contribution presents methods for the cross-component prediction (CCP) merge mode. The CCP merge mode is proposed to be applied to the linear-model part of the chroma fusion mode. The cross-component model parameters applied to the linear-model predictor of chroma fusion mode are inherited from the spatial adjacent, spatial non-adjacent, history-based, or default CCP models. In addition, a filter flag for local boosting is also proposed to be inherited in the CCP merge mode. Therefore, the prediction samples generated by the CCP merge mode may also be filtered with neighbouring samples.

On top of ECM-9.0, the simulation results of the proposed method are reported as follows:

AI: {-0.01%, -0.34%, -0.33%, 100.9%, 99.9%}; RA:{ -0.xx%, -0.xx%, -0.xx%, xxx%, xxx%}

The two aspects should be investigated separately.

There are similar aspects in JVET-AE0170

Further study was recommended if more gain can be achieved, potentially also by combination.

JVET-AE0254 Crosscheck of JVET-AC0074 (Non-EE2: Improvement on cross-component prediction merge mode) [X. Li (Alibaba)] [late]

JVET-AE0260 Crosscheck of JVET-AE0074 (Non-EE2: Improvement on cross-component prediction merge mode) [K. Zhang (Bytedance)] [late] [very late]

Still missing at the end of the meeting, but provided later.

JVET-AE0097 AHG12: On the cross-component merge mode [H. Huang, V. Seregin, M. Karczewicz (Qualcomm)]

This contribution proposes to remove the template-based offset update process for the cross-component merge candidate.

On top of ECM-9.0, the average BD-rate impact, encoder and decoder run times are summarized as follows {Y, U, V, EncT, DecT}:

AI: -0.01%, 0.01, 0.00%, 98.8%, 97.0%; class TGM -0.39%, -1.01%, -1.02%, 99.4%, 99.7%

RA: -0.02%, 0.00%, -0.22%, 99.8%, 99.9%

The run time numbers are likely unreliable.

The results are confirmed by two cross-checks, and the change is straightforward due to the cross-checkers. It was further mentioned that when the template based offset update process was adopted, it had only been tested on AI and not on screen content and not on RA, and in the meantime some other elements in cross-component processing had been added.

Decision: Adopt JVET-AE0097.

JVET-AE0188 Crosscheck of JVET-AE0097 (AHG12: On the cross-component merge mode) [K. Zhang (Bytedance)] [late]

JVET-AE0243 Crosscheck of JVET-AE0097 (AHG12: On the cross-component merge mode) [C.-M. Tsai (MediaTek)] [late]

JVET-AE0115 Non-EE2: Unified intra CC-models parameters precision P. Bordes, K. Naser, F. Galpin, F. Le Léannec (InterDigital)]

In ECM-9.0, several intra coding tools use cross-component (CC) prediction. However, some of these tools use 64-bits precision for storage and applying CC-models, while others use 32-bits. It is proposed to unify the CC-model parameters to use 32-bits precision. It is reported that the proposed change does not impact the RD performance of ECM-9.0: AI 0.00% -0.01% 0.01%, RA xx% xx% xx%.

It was asked which would be the actual reduction in memory usage?

It was asked what the actual dynamic range of the parameters is? Is 32 bits necessary? Is it necessary to specify a shifting additionally, depending on which the value range of a given parameter is.

Several experts supported the proposal, being straightforward to implement.

It was further argued that a more clean solution would be to compute the parameters in 32 bits from the beginning, rather than leaving the 64 bit computation unchanged and convert them into 32 in a second step, as proposed here.

Without doubt, this would be necessary for a standard, but then a more consistent overall design would be necessary. It was considered not urgent in the exploration stage.

JVET-AE0244 Crosscheck of JVET-AE0115 (Non-EE2: Unified intra CC-models parameters precision) [C.-Y. Chuang (MediaTek)] [late]

JVET-AE0170 Non-EE2: Enhancements on CCP merge [K. Zhang, Z. Deng, L. Zhang (Bytedance)]

This proposal presents the results of the enhanced CCP merge mode. The fusion prediction can be generated by the CCP-merge prediction and the MM-CCCM prediction as a weighted sum. Besides, CCP merge mode is harmonized with LB-CCP by including the LB-CCP flag in the CCP merge candidate. Simulation results based on ECM-9.0 are reported as below:

- AI {-0.03%, -0.43%, -0.45%, 101%, 100%}; RA {}.

The two aspects should be studied separately.

There are similar aspects in JVET-AE0074

Further study was recommended if more gain can be achieved, potentially also by combination.

JVET-AE0261 Crosscheck of JVET-AE0170 (Non-EE2: Enhancements on CCP merge) [H. Huang (OPPO)] [late]

JVET-AE0178 Non-EE2: Cross-component prediction merge mode for chroma inter coding [M.-S. Chiang, H.-Y. Tseng, C.-M. Tsai, C.-Y. Chuang, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, O. Chubach, Y.-W. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

In ECM-9.0, when the current chroma intra block is coded in cross-component prediction (CCP) merge mode, the cross-component model parameters of this chroma block can be inherited from previously coded blocks, as described in JVET-AD0188. This contribution extends the CCP merge mode to inter coding blocks, where the final chroma inter prediction combines motion-compensation predicted signals and cross-component predicted signals derived using an inherited CCP model.

On top of ECM-9.0, the simulation results of the proposed method are reported below:
RA: {Y BD-rate = 0.xx%, Cb BD-rate = -0.xx%, Cr BD-rate = -0.xx%, EncT = 10x%, DecT = 10x%}

Some preliminary results from the PowerPoint deck:

| Over ECM-9.0 |  |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y |  | Cb | Cr |  |  | EncT |  | DecT |
| Class A1 | -0.48% |  | -1.97% |  |  | -2.64% | 107.1% |  | 103.1% |  |
| Class A2: Not available |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.04% |  | -1.22% |  |  | -1.07% | 104.7% |  | 100.6% |  |
| Class C | 0.04% |  | -0.73% |  |  | -0.74% | 105.7% |  | 99.7% |  |

The method uses the CCP merge mechanism from intra and applies it to inter coded blocks, and allows weighted combination with MC prediction. A temporal candidate is added to the merge list.

High gain is reported for class A, significantly higher for Campfire.

It was commented that some of the gain may be reduced due to adoption of EE2-3.1b, which also takes benefit from cross-component in intra.

The tradeoff of runtime vs. compression was not attractive in the current version.

Experts supported investigation in the EE, possibly in combination with changes made to CCP merge list construction in ECM10, and reduction of encoder runtime. This should test the impact of adding the temporal candidate to the merge list.

#### In-Loop Filters (3)

JVET-AE0044 AHG12: Dynamic Scaling of Bilateral Filter (BIF) [V. Shchukin, P. Wennersten, J. Ström (Ericsson)]

This contribution proposes three additional techniques for the Bilateral Filter (BIF):

- The TU scale factor depends on the TU shape size.

- The TU scale factor depends on the mean absolute difference (MAD) of the TU.

- The BIF LUTs are interpolated.

The proposed algorithm is implemented on the top of ECM-9.0. According to simulation results under common test conditions (CTC), the BD-rate PSNR impact is reported to be -0.14%/-0.11% for AI/RA for Luma, the average decoding run time is reported to be 100.5%/99.8% for AI/RA. For AI, the average encoding run time is reported to be 101.4%. For RA, the encoding run time averaged over class B and class C is reported to be 100.5%.

There was some complexity increase, but OK for exploration.

Several experts supported this as an interesting approach.

It was agreed to investigate this in the EE.

JVET-AE0190 Crosscheck of JVET-AE0044 (AHG12: Dynamic Scaling of Bilateral Filter) [W. Yin (Bytedance)] [late]

JVET-AE0121 Non-EE2: Luma Residual Taps in Chroma-ALF and CCALF [W. Yin, K. Zhang, Z. Deng, L. Zhang (Bytedance)]

In the current adaptive loop filter (ALF) design of ECM-9.0, luma residual values are stored and used in the luma-ALF. In this contribution, luma residual values are further introduced into chroma-ALF and CCALF.

On top of ECM-9.0, simulation results of the proposed method are reported as below:

- RA: *0.02%, -0.56%, -0.67%, 99.8%, 100.0%.

- LB: %, %, %, %, %.

The preliminary results (not including class B yet) indicated higher chroma gain for LB than in RA.

It was found that no benefit (some chroma losses) would be present in AI. The proponents suggest this might be caused by different characteristics of intra residual. In case of luma, the benefit was also lower for AI, but no loss.

Experts supported study in the EE. Separate results should be provided for chroma ALF and CCALF, and also the AI configuration should be studied.

Single tree operation might be studied in AI, in case that the separate tree is the cause of losses.

JVET-AE0231 Crosscheck of JVET-AE0121 (Non-EE2: Luma Residual Taps in Chroma-ALF and CCALF) [Z. Xie (OPPO)] [late]

JVET-AE0131 Non-EE2: Variance based classification for in-loop filtering [W. Yin, K. Zhang, L. Zhang (Bytedance)]

In this contribution, a variance-based classification for in-loop filtering is proposed. In the proposed method, variance is computed and used as an additional classification rule for BF and ALF. The existing classification methods in BF and ALF are kept unchanged.

On top of ECM-9.0, simulation results (estimated) of the proposed methods are reported as below:

- AI: *-0.02%, -0.02%, 0.03%, 100.0%, 100.1%.

- RA: *-0.04%, -0.12%, 0.07%, 100.0%, 99.8%.

- LB: %, %, %, %, %.

How was the weight derived? Boundary info from partitioning, dependency on variance and combination with existing classification are only vaguely described and the proponents were requested to provide an update of the contribution.

Two filters were modified, but gain is not significant enough to consider the proposal.

Further study was encouraged to determine if more benefit can be achieved.

The cross-checker found an increase of decoding time, but sees potential for improvement in gain.

JVET-AE0207 Crosscheck of JVET-AE0131 (Non-EE2: Variance based Classification for In-loop Filtering) [V. Shchukin (Ericsson)] [late]

#### Entropy coding and transform coefficient coding (2)

JVET-AE0058 AHG12: Spatial CABAC tuning [J. Lainema, A. Aminlou, P. Astola, R. G. Youvalari, D. Bugdayci Sansli (Nokia)]

This contribution asserts that the current CABAC context update mechanism may lead to “overfitting” the probability estimates to the statistical properties of the CTU that is preceding the current CTU in scan order. To help overcome such bias, it is proposed to update the CABAC contexts using syntax element values retrieved from the above CTU when starting to process a new CTU. A limited number of bins (e.g., 512) of syntax elements belonging to the bottom CUs of the above CTU are fed into the context update mechanism of the shorter window probability estimator to achieve this.

It is reported that the proposed approach has the following BD-rate and runtime impact when implemented in ECM-9:

- AI { -0.04%, 0.00%, -0.01%, 99%, 97%}

- RA { -0.06%, -0.18%, -0.02%, 100%, 100%}

It is further reported that an alternative configuration with larger maximum number of stored bins (1024 instead of 512) per CTU and updating both shorter and longer window probability estimators has the following BD-rate and runtime impact:

- AI { -0.05%, -0.03%, -0.04%, 99%, 100%}

- RA { -0.07%, -0.18%, -0.07%, 101%, 100%}

Only cases are stored where bins are actually appearing in bitstream. Indices need to be stored along with the bin.

How much storage is needed at maximum? Roughly 6000 bits per CTU, for entire row of CTUs.

Would this cause some latency problem? Maximum of 32 bins per context needs to be done sequentially, update can be done in parallel for different contexts. Limitation of maximum number might be necessary. Analysis would be desirable how many updates are actually performed at certain bit rates.

Would it be effective in case of mixed content, where characteristics change? Statistically likely yes, according to proponents.

It was agreed to investigate this in the EE, also reporting about aspects mentioned above.

JVET-AE0147 AHG12: Updating context model parameters for ECM [R.-L. Liao, Y. Ye, J. Chen, X. Li (Alibaba)]

This contribution proposes to update context initialization parameters for I-slice. Specifically, the initial probabilities, window sizes and adaptive weights for I-slice context models are retrained and updated. It is reported that on top of ECM-9.0, the overall coding performance impact for {Y, U, V} is {-0.10%, 0.07%, 0.15%} for AI, {-0.01%, -0.01%, 0.03%} for RA and {0.05%, -0.22%, 0.32%} for LB.

The training script was used that was once used during in VVC development, and adapted to ECM accordingly. It was reported that currently, the script is not properly working for PB slices, likely due to temporal inheritance. Contexts are also more complex in the ECM, so the modifications necessary may not be trivial.

It was questioned whether the gain in intra is justified, while loss occurs in LB. It is also observed that the gain is quite sequence dependent, some sequences with high.

It was reported that in case of VTM, also other content such as HDR was used.

Several experts expressed that it would be beneficial to update initialization regularly. However, the proposed approach does not seem to do it in an optimum way, and in particular not for PB slices yet.

It was agreed to study this in the EE, provided that the modified script could be shared and if possible further improved.

Further study was recommended for the application to PB slices.

JVET-AE0252 Crosscheck of JVET-AE0147 (AHG12: Updating context model parameters for ECM) [Z. Deng (Bytedance)] [late]

#### Partitioning (1)

JVET-AE0082 AHG12: Non-square quadtree partitioning [Y. Ahn, J. Nam, N. Park, J. Lim, S. Kim (LGE)]

In the current VVC and ECM, quadtree partitioning is applied only to square blocks while binary and ternary tree partitioning are allowed to be applied to both square and non-square blocks. This contribution allows quadtree partitioning to also be applied to non-square blocks of which shapes are 2N×N and N×2N. The proposed partitioning method was implemented on top of ECM-9.0 and the summarized test results are shown as follows:

- AI:   -0.17%/-0.29%/-0.26% (Y/U/V), 137% (EncT), 100% (DecT)

- RA:  -0.67%/-0.92%/-0.82% (Y/U/V), 167% (EncT), 100% (DecT)

- LDB: -0.31%/-0.90%/-0.54% (Y/U/V), 144% (EncT), 99% (DecT)

Minimum non-square QT sizes are 8x16, 16x8

This has no new block sizes that are non-existing.

As there are no new partition options, to show benefit, it should be compared with encoders using very similar encoding times. It was suggested to release in the ECM the constraint for maximum MTT depth, which would increase the encoding time of ECM and would make the comparison more fair. However, to be attractive for adoption, the encoding time shall be in the ballpark of current ECM.

Currently, there were no fast decisions in the code.

It was agreed to investigate this in the EE. See notes above about comparison.

JVET-AE0206 Crosscheck of JVET-AE0082 (AHG12: Non-square quadtree partitioning) [R. Yu (Ericsson)] [late]

Categories from here discussed Saturday 15 July 1605-1830

#### GDR (2)

JVET-AE0145 AHG 12: Flexible GDR [L. Wang, S. Hong, K. Panusopone (Nokia)]

In the current GDR design in VVC/ECM, an initial refreshed area starts on the left of a GDR picture and gradually expends over the associated recovering pictures. The most meaningful content information of a video sequence, however, may not necessarily always be in the left part of pictures. This contribution proposes a flexible GDR, where an initial refreshed area can be in any location in a GDR picture. To support such flexible GDR, each virtual boundary in GDR/recovering picture is assigned a flag (new syntax element), specifying which side of the virtual boundary is in refreshed area. The proposed flexible GDR is implemented in ECM-9.0. Simulations were conducted for the flexible GDR with an initial refreshed area starting from the left, the middle and the center of GDR pictures. The middle/center GDR present interesting content information much earlier than the left GDR. The overall BD rate measurements of the middle/center GDR over the left GDR were not yet available.

Bit rate increased due to larger number of virtual boundary (at least if more activity in the center, such as in videoconferencing). Up to 4 would be supported by the proposal, instead of one currently.

The encoder decides where to start the refresh, including left/right/top/bottom, four corners, etc.

It was commented that this would not be favorable if the memory consumption becomes larger (JVET-AE0180) reports unnecessary memory consumption by GDR (in the report on JVET-AE0180, this was later corrected to be caused by wraparound MC, not GDR).

It was commented that GDR access of a user can start at any frame (not only IRAP, as demonstrated here). However, in VVC, a GDR picture is not possible at any frame, and a decoder has to wait until seeing a GDR picture.

It was pointed out that some of these aspects can be done non-normatively, e.g. by using subpictures

The effect may be very specific to specific content, such as videoconference with one person in the middle.

GDR is not of high importance in the ECM, and the software should not become overcomplicated by even more complicated variants of it that may not be needed practically. According to the software coordinator, GDR impacts many tools in the ECM, which likely would become more complicated with additional VBs.

No action was taken on this.

JVET-AE0267 Crosscheck of JVET-AE0145 (AHG 12: Flexible GDR) [J. Enhorn (Ericsson)] [late]

JVET-AE0146 AHG 12: Fixes H-CCP Table for CUs in Refreshed Areas of GDR/Recovering Pictures [S. Hong, L. Wang, K. Panusopone (Nokia)]

This contribution presents a solution to address the GDR leak caused by the adoption of JVET-AD0188 (JVET-AC0176 and JVET-AC0315) in the ECM. JVET-AD-0188 introduces a H-CCP table that retains CCP model information of the most recent coded CUs in CCP model. The entries of the H-CCP table may be included in the CCP merge candidate list. In the case of GDR, if an entry in the H-CCP table originates from coded CUs in a non-refreshed area of a GDR/recovering picture, it may result in a leak. To avoid such leak issue, a separate H-CCP is proposed for CUs in the refreshed area, which would encompass only the CCP model information of the last coded CUs in the refreshed area. This fix had been implemented in ECM 9.0 master. commit 2e1d547d. The simulations over JVET test sequences revealed no instances of leak occurring after implementing the proposed fix.

The cross-checker reported a deviation in results, which is present when a new adoption of ALF in ECM9 is enabled (From JVET-AD0222).

Otherwise, it is confirmed by the cross-checker that the GDR leak is avoided by the contribution.

It was asked if this problem could also be resolved non-normatively by making the encoder not use the candidates that are affected. Further study was requested on this.

It was commented that it is likely that similar problems may occur with other tools of ECM, e.g. when subpictures would be enabled. It is not of high importance in an exploration for coding efficiency to make tools fully compliant with functionalities which would typically only be implemented in a real standard development.

JVET-AE0273 Crosscheck of JVET-AE0146 (AHG 12: Fixes H-CCP Table for CUs in Refreshed Areas of GDR/Recovering Pictures) [L. Zhao (Bytedance)] [late]

#### RPR (2)

(The discussion of contributions in this section was chaired by Y. Ye.)

JVET-AE0103 Weighted Edge Enhancement Filtering for Picture Upscaling and RPR [T. Claßen, M. Wien (RWTH Aachen University)]

This input contribution discusses Reference Picture Resampling (RPR) and picture rescaling in the ECM. An adaptive filtering method for upscaled video content is proposed. It proposes an adaptive filtering method for upscaled video content, suggesting the application of the same filter for both RPR upscaling and picture upscaling. The proposed method employs a locally weighted linear filter, consisting of two components. The first component involves computing the local weighting from the upscaled video content. The second component is a linear filter that is RD-optimized and transmitted in the bitstream. The results obtained from these operations are multiplied and added to the upscaled frame or block. The reported results are based on upscaling 1.5x and 2.0x, respectively, using the RPR test conditions outlined in JVET-Q0822, which defines the 0.5sec resolution switch. These changes result in average BD-Rate savings of -7.69%/-1.44%/-1.33% for 2.0x LDP and -7.27%/-0.57%/-0.32% for 2.0x LDB in ECM8.0. The input suggests further investigation in an exploration experiment (EE).

Discussion of this contribution was chaired by Y. Ye.

For the case of applying the proposed filter to AI configuration, every other picture is downsampled by a factor of 2, and the proposed filter is applied to recovered the downsampled pictures to the original resolution (applied after normal post-processing upsampling in the ECM is performed). This is outside of the RPR CTC, which does not include AI. The filter is applied as post processing.

For the case of applying the proposed filter to RA configuration, pictures are downsampled every 0.5 second (in alignment with RPR CTC), which means that some intra-coded pictures may be downsampled as well depending the test sequence’s frame rate. For those intra pictures that have been downsampled, they are upsampled using the proposed filter in the same way as in the AI case.

For the case of applying the proposed filter to LDB and LDP configurations, the operation conforms to RPR CTC, that is, the first intra picture is coded as is. The following tables show partial results for AI and RA.

|  | All Intra Main 10 (switch every frame) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT* | DecT* |
| Class A1 | #WERT! | #WERT! | #WERT! | #DIV/0! | #DIV/0! |
| Class A2 | #WERT! | #WERT! | #WERT! | #DIV/0! | #DIV/0! |
| Class B | -4.89% | -0.80% | -1.11% | 125% | 169% |
| Class C | -3.71% | -0.90% | -1.05% | 136% | 160% |
| Class E | -8.35% | -0.25% | -0.02% | 153% | 197% |
| Overall | #WERT! | #WERT! | #WERT! | #ZAHL! | #ZAHL! |
| Class D | -3.12% | -0.50% | -1.09% | 148% | 178% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT* | DecT* |
| Class A1 | #WERT! | #WERT! | #WERT! | #DIV/0! | #DIV/0! |
| Class A2 | #WERT! | #WERT! | #WERT! | #DIV/0! | #DIV/0! |
| Class B | -6.02% | -0.65% | -0.96% | 125% | 132% |
| Class C | -4.17% | -1.22% | -1.36% | 89% | 100% |
| Class E |  |  |  |  |  |
| Overall | #WERT! | #WERT! | #WERT! | #DIV/0! | #DIV/0! |
| Class D | -3.05% | -0.30% | -1.99% | 66% | 84% |
| Class F | -8.08% | -5.03% | -6.40% | 110% | 151% |

When a downsampled picture is used as refernce for inter prediction, the proposed filter is applied after the normal upsampling (which is done by regular motion compensation filters in the ECM) has been applied as an additional step to correct/recover high frequency information.

In terms of signalling, one filter per picture is signalled if the picture is downsampled.

Decoder also needs to compute weighting map. In the current implementation, this is computed for the entire picture.

It was commented that this is generally a very interesting idea, and may also bring benefits to inter prediction in general even when RPR is not used (that is, for some blocks, applying this after inter prediction may enhance the prediction quality).

It was commented that if this is combined with GOP-based RPR, the gains from both may be additive.

The proposed filter is applied in two parts of the coding chain, the first part is normative (i.e. when lower-resolution reference picture is used), the second is “non-normative” (i.e., when lower-resolution reconstructed picture is upsampled, the proposed filter is used as a post processing step, although filter coefficients still need to be signalled). These two parts should be studied separately.

The proponent suggests that if this is investigated in EE, then the RPR CTC is the most suitable condition to be used. For AI configuration, the proponent suggests that additional results using the every-other-picture-resolution-change condition may also be made available.

The results reported are based PNSR in full resolution, i.e. what is referred to as “PSNR2” in RPR tests.

Tests that should be performed are:

- Normative aspect: only perform the proposed filtering after motion compensation, the post-processing upsampling is not changed, and kept exactly the same as ECM

- Both aspects: perform the proposed filtering after motion compensation, and perform the proposed filtering after reconstruction

The proponent remarked that although only 2x results are shown, the method can be applied to 1.5x in a straightforward manner.

It was agreed to investigate this in the EE. Use RPR CTC (2x and 1.5x) for LDB. RA, AI and LDP results could be provided optionally. Separate results should be brought for the two tests as described above.

JVET-AE0153 Non-EE2: Enabling template-based inter tools for scaled reference pictures in the RPR [X. Xiu, H.-J. Jhu, C.-W. Kuo, C. Ma, N. Yan, W. Chen, X. Wang (Kwai)]

This contribution proposes to improve the coding efficiency of the RPR in ECM-9.0 by enabling the template-based inter reordering tools (including ARMC, MMVD, affine MMVD, template-based BCW derivation, reference picture list reordering and MVD sign and magnitude-suffix prediction) and the LIC for scaled reference pictures. Additionally, the contribution identifies one encoder crash issue of ECM-9.0 in RA configuration when the RPR is enabled and provides the fix. The proposed changes were implemented on top of ECM-9.0 with MR437. The simulation results are summarized as follows:

- RPR-1.5x

  - LB PSNR1: -0.89% (Y), -1.04% (U), -0.67% (V), 104.3% (Enc), 104.1% (Dec)

  - LB PSNR2: -0.86% (Y), -1.07% (U), -0.68% (V), 104.3% (Enc), 104.1% (Dec)

- RPR-2.0x

  - LB PSNR1: -0.77% (Y), -0.67% (U), -0.54% (V), 104.4% (Enc), 103.8% (Dec)

  - LB PSNR2: -0.65% (Y), -0.72% (U), -0.59% (V), 104.4% (Enc), 103.8% (Dec)

Discussion of this contribution was chaired by Y. Ye.

The cross checker confirmed the reported results.

There are two fixes provided by the proponent as a part of this contribution: MR437 and MR447. Both fixes have been merged into the ECM master.

There are two normative changes proposed in this contribution, one relates to using RPR filters to perform template matching based reordering, the other relates to LIC.

The cross checker suggested to test these two changes separately.

It was asked if TM-AMVP has been modified in this contribution, the proponent answered no, because internal test showed no gain.

Multiple experts expressed support to test this in the EE.

It was agreed to investigate this in the EE, testing the two proposed changes separately.

JVET-AE0259 Crosscheck of JVET-AE0153 (Non-EE2: Enabling template-based inter tools for scaled reference pictures in the RPR) [Z. Zhang (Qualcomm)] [late]

JVET-AE0287 Non-EE2: A combination of JVET-AE0074 and JVET-AE0170 [K. Zhang, Z. Deng, L. Zhang (Bytedance), H. Huang, Y. Yu, H. Yu, D. Wang (OPPO)] [late]

This proposal presents the results of a combination of JVET-AE0074 and JVET-AE0170. The combination comprises three aspects.

- Aspect #1 (from both JVET-AE0074 and JVET-AE0170): CCP merge mode is harmonized with LB-CCP by inheriting the LB-CCP flag in the CCP merge candidate.

- Aspect #2 (from JVET-AE0074): The prediction of a fusion mode can be generated by the CCP-merge prediction and the angular intra-prediction.

- Aspect #3 (from JVET-AE0170): The prediction of a fusion mode can be generated by the CCP-merge prediction and the MM-CCCM prediction.

Results were reported as :

- AI {-0.02%, -0.70%, -0.68%, 102%, 100%}

- RA {}.

For the individual proposals, it had been assessed that the relatively low gain in chroma is not justifying the encoder run time increase that both of them had individually. From this contribution, it can be found that coding gains are almost additive, but run times are also additive, therefore it can be concluded that it does not have a better tradeoff than the individual contributions.

Further study was recommended for better tradeoff (better compression and/or decreased run time).

JVET-AE0290 Crosscheck of JVET-AE0287 (Non-EE2: A combination of JVET-AE0074 and JVET-AE0170) [Z. Lv (vivo)] [late]

### CTC for EE2/ECM and ECM improvements (7)

Contributions in this area were discussed at 0900–1200 on Sunday 16 July 2023 (chaired by JRO).

JVET-AE0055 AHG12: Sign prediction parameter configuration for low-delay conditions [Y. Yasugi, T. Ikai (Sharp)]

In JVET-AD0105, the common test condition with a modification to the sign prediction parameter to AI and RA was adopted. In this contribution, the same configuration that the NumSignPred parameter in QP22 is set to four rather than eight is also applied for the low-delay conditions (LB and LP), thus it will be used in all conditions. Experimental results reportedly show that the proposed modification reduced the encoding time in QP22 on the low-delay conditions by 2%.

Very small variation (<=0.02), sometimes gain, sometimes loss per sequence.

The same setting was already used in AI and RA.

Decision (CTC): Adopt JVET-AE0055.

JVET-AE0271 Crosscheck of JVET-AE0055 (AHG12: Sign prediction parameter configuration for low-delay conditions) [Y.-J. Chang (Qualcomm)] [late]

JVET-AE0057 MTT split modes early termination [W. Ahmad, P. Wennersten, K. Andersson (Ericsson)]

This contribution presents an early termination method that is claimed to reduce encoding runtime for ECM. The cost of coding the CU without splitting the CU at all is compared to a threshold. If the cost is higher than the threshold, MTT split will not be considered, which is claimed to reduce encoding runtime. The proposal is implemented on top of ECM 9.0 software and compared to ECM-9.0, the average BDR loss and runtimes reported in this contribution are as follows:

Test1 (Proposed condition is applied to all QPs)

Y 0.03%, U 0.06%,	V 0.11%, EncTime 97.0%, DecTime 99.9% for the RA configuration,

Y 0.05%, U -0.06%, V -0.20%, EncTime 96.8%, DecTime 100.1% for the LDB configuration,

Test2 (Proposed condition is applied to QP 22 only)

Y 0.00%, U 0.00%,	V 0.02%, EncTime 98.8%, DecTime 99.9% for the RA configuration. Overall EncT & DecT for QP22 are 95.5% , 99.7% respectively

Y -0.01%, U -0.02%, V 0.02%, EncTime 98.6%, DecTime 100.1% for the LDB configuration. Overall EncT & DecT for QP22 are 95.5% , 100.4% respectively.

For the worst-case RA encoding time, it is claimed that the proposed scheme reduces the runtime by 3% for the slowest chunk in Parkrunning3 QP22. For the worst-case LDB encoding time, it is claimed that the proposed scheme reduces the runtime by 5% for BQTerrace QP22.

It is proposed to include the Test2 in the ECM for RA and LDB camera captured contents.

It was asked if the formula (including a term (QP-22)) was tested for QPs below 22.? No, but it is believed that the threshold by tendency must be higher in lower QPs, which would be the case.

It was asked if the threshold should be bit depth dependent? Was not considered so far.

Test 1 is not QP dependent, and not class dependent.

Test 2 is QP dependent, and not class dependent.

concern was expressed by one expert that this introduces a new QP dependent encoder optimization, of which some aspects are not yet fully understood.

For AI, there would be no effect, as MTT starts at 32x32.

Generally, the tradeoff of test 1 (encoder runtime decrease vs. bit rate increase, which is consistent over classes) is asserted to be attractive.

Performance for screen content? For test 1 Class F -0.02, TGM +0.19% (which may be interpreted as very small loss, as gain/loss in TGM is usually much larger than in other classes)

Decision (SW/CTC): Adopt JVET-AE0057 Test 1. (CTC also for optional classes). Also for VTM with encoder config option, but not yet CTC for VTM, as results are not yet complete.

It was requested to conduct further study on behaviour for lower QP, and bit depth other than 10, to make an assessment about adoption for VTM CTC also for extended ranges of bit depth and QP.

Post-meeting note: A crosscheck on this contribution had originally been registered as JVET-AE0215, but was never delivered. According to a verbal report of the crosschecker, partial results were matching, and the implementation was straightforward.

JVET-AE0133 AhG12: On EE2 Common test conditions change based on EE2 Test 1.1 [G. Laroche, P. Onno (Canon)]

This contribution proposes a modification of the current EE2 common test conditions on top of the EE2 Test 1.1b. The aim of this proposed configuration is to reduce the encoding run time of the ECM software for Random access configuration.

The current EE2 common test conditions use several maximum MTT hierarchy depths settings depending on the Classes and QP. In this contribution only one setting for all classes and QP is proposed.

Compared to ECM-9.1, the average BDR gains and runtimes for this contribution were not available.

There was no need for presentation of this, as the EE2 tested modification was not adopted.

JVET-AE0163 AHG12: modified CTC proposal for low-delay configuration [S. Puri, C. Bonnineau, F. Le Léannec, T. Poirier, E. François (InterDigital)]

In JVET-AD0128, a modification to the CTC for low delay configuration was proposed, to significantly reduce the worst-case encoding runtime for LDB/LDP. It consisted of modifying encoder configuration as follows. The maximum MTT hierarchy depth is set to 4 in the I slice. For QP 22 of HD sequences, it is decreased from 3 to 2 for slices with temporal depth higher or equal to 2.

This contribution proposes a change in the above configuration. The maximum MTT hierarchy depth change from 3 to 2 for slice temporal depth greater than 2 is not applied to the QP 22 of the AOV sequence to avoid applying a configuration at sequence level. Instead, this change is only applied to Class B HD sequences only for QP 22. Class F remains untouched in this contribution.

Further this contribution provides the individual test results for the following two set of encoder parameter modifications:

- Test 1: same as JVET-AD0128 with the above modification

- Test 2: Test 1 with no maximum MTT depth changes to I-slice

The results are summarized below:

LDB (Test 1):

- 99.9% average encoding time relative to ECM-9.0 CTC, with -0.10% luma coding gain

- 68% encoding time relative to ECM-9.0 for the worst case (BQTerrace QP 22)

LDB (Test 2):

- 95.7% average encoding time relative to ECM-9.0 CTC, with 0.04% luma coding loss.

- 65% encoding time relative to ECM-9.0 for the worst case (BQTerrace QP 22)

Similar encoder time savings are observed in LDP configuration in ECM-9.0. The contribution proposed to adopt this CTC change for the LDB and LDP configurations.

This is a modification of test 1 from the last meeting: The change in QP 22 is only applied in class B, whereas it had also been used for one HD sequence in class F previously. The change of MTT hierarchy depth in I slices is applied over all classes, which causes a run time increase for other classes than B.

Test 2 is without the change in MTT hierarchy depth, therefore the encoder run time decrease is larger.

Overall, the saving in average encoding time comes from class B QP 22 in both cases.

It was commented that there are already other class-specific and QP-specific configuration changes in CTC.

It was commented that worst case run time in LDB has increased significantly (>20%) from ECM8 to ECM9. From that, the proposal might not fully help to break such tendency.

Several experts supported test 2.

Decision (CTC): Adopt JVET-AE0163 Test 2.

JVET-AE0264 Crosscheck of JVET-AE0163 (AHG12: Modified CTC proposal for low-delay configuration) [H. Huang (Qualcomm)] [late]

JVET-AE0180 On ECM SW memory consumption [R. Chernyak, S. Liu (Tencent), Y. Yasugi, T. Ikai (Sharp)]

This document provides an ECM SW memory consumption analysis. It also proposes a straightforward non-normative ECM SW modification that losslessly reduces ECM memory consumption worst case for about 2 GiB under CTC.

It was found that larger memory is consumed by covariance matrices in ALF, and memory allocation for wraparound MC, even if the latter is not used in the ECM CTC.

It was commented that this is a step in the right direction, but much more effort would be necessary for reduction of memory consumption. It was further suggested that it would be good if proponents would be enforced to report (in the excel sheet) the additional memory consumption of the proposal.

One expert commented that covariance matrices consume a maximum which is lower than reported here.

It was commented that a similar analysis for decoder would also be useful.

Decision (SW): Adopt the change of unnecessary memory allocation for wraparound MC in cases where the tool is disabled (for ECM, and to the discretion of SW coordinators to also be implemented in VTM and NNVC).

It was further agreed that reporting the additional memory usage of a new element proposed will be made mandatory (e.g., by adding a corresponding column to the excel sheet). It is understood that this is purely informative and not meant as a mechanism to reject a proposal, but rather identify newly added elements that cause excessive additional memory consumption, such that a solution for possible reduction can be sought by a coordinated activity between proponents and software coordinators.

This should be the “VmPeak” memory consumption output from ECM.

It should also be reported in cross-checks.

It was commented that a similar output for the decoder would be desirable, which would require an implementation in the ECM.

JVET-AE0192 Crosscheck JVET-AE0180 (On ECM SW memory consumption) [C.-W. Kuo, X. Xiu (Kwai)] [late]

JVET-AE0257 Non-EE2: Peak Memory Reduction for ECM [W. Yin, K. Zhang, L. Zhang (Bytedance)] [late]

In the current design of adaptive loop filter (ALF) and cross-component adaptive loop filter (CCALF) in ECM-9.0, the filtering unit size is set equal to the CTU size. The calculation of co-variance matrix for each CTU requests is a high memory cost at encoder. In this contribution, it is proposed to allow the filtering unit size larger than the CTU size. By doubling the dimensions of the filtering unit for 4K sequences in RA conditions, the peak required total memory at encoder is reduced from 23.2 GB to 19.6 GB, reportedly. The encoding time is reportedly reduced by 1~2% since the encoder requires less training rounds and determinations for ALF and CCALF.

On top of ECM-9.0, simulation results of the proposed method are reported/estimated as below:

- RA: *0.02%, 0.08%, 0.21%, 99.4%, 99.9%.

The loss is higher for A classes where the change has an effect.

Final results (incomplete simulation currently) may have higher bit rate increase.

Solutions for memory reduction without normative changes are preferable, normative solutions should not be considered until the problem is better understood.

JVET-AE0274 Crosscheck of JVET-AE0257 (Non-EE2: Peak Memory Reduction for ECM) [Y. Yasugi (Sharp)] [late]

JVET-AE0278 AHG6/Non-EE2: Suggestions on memory reduction for ECM encoder [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)] [late]

In the encoder of ALF and CCALF of ECM-9.0, the co-variance matrices of CTUs require a high amount of memory. It is proposed to optimize ECM-9.0 encoder to reduce that memory usage.

The following changes are proposed:

- It is proposed to define the co-variance matrix of a filter based on the actual filter size.

In ECM-9.0, the numbers of coefficients of a luma ALF filter, a chroma ALF filter and a CCALF filter are 39, 20 and 24, respectively. Defining a co-variance matrix by assuming the numbers of coefficients of all filters are 42 allocates unnecessary memory.

- It is proposed to remove the non-linearity of co-variance matrices for CCALF.

- To avoid storing identical information in two co-variance matrices, it is proposed to use one big matrix to store both the identical info (the ALF input samples, samples before the deblocking filter, output of a Gaussian filter and the residual data) and different info (outputs of fixed filters from different candidate).

- It is proposed to change the data type of a co-variance matrix into float from double.

Decision (software): Changes 1-3 should be lossless; it was agreed that they can be implemented via a merge request for code optimization. Change 4 would change the performance, but without knowing results, no action could be taken (a similar change had been proposed earlier in JVET-Z0150 on an older ALF version, but not adopted at that meeting).

It was requested to announce the related merge request(s) via the email reflector.

# High-level syntax (HLS) and related proposals (41)

## AHG9: SEI messages on neural-network post filter (28)

Contributions in this area were discussed in the BoG reported in JVET-AE0272 (those related to v3 VVC/VSEI), and otherwise at 0830–1215 on Friday 14 July 2023 (chaired by JRO).

JVET-AE0272 BoG report on SEI messages for VSEI v3 and VVC v3 [G. J. Sullivan (BoG coordinator)]

This document is a break-out-group report on SEI messages for VSEI v3 and VVC v3. The BoG met from 15:15 to 18:30 on Tuesday 11 July 2023 and from 08:30 to 12:30 and 14:30 to 18:30 on Wednesday 12 July 2023 in Salle 3 of the Geneva CICG facility. About 15-20 people participated in person and another 10-15 participated remotely.

The BoG reviewed the relevant proposals for NNPF SEI messages based primarily on the summary of NNPF-related SEI messages provided in JVET-AE0187-v3, which formed the original basis of the structure of this report.

No specific plans were recommended for topics that were agreed not to be within the scope of VSEI v3 and VVC v3 (except to encourage progress toward future support for VSEI v3 SEI messages for NNPF and filtering phase indication in HEVC and AVC).

Actions recommended by the BoG are highlighted in this report. Five items were marked to revisit for further discussion; these five were suggested to be relatively minor and may not require a further BoG session (if the BoG scope is not expanded to consider additional topics).

Notes of the JVET plenary review of the first version of the BoG report were included in the -v2 version of the report (with revision marking). In the -v2 version, one item remained marked for further discussion for text review. See the notes for JVET-AE2006 for the disposition of that topic.

The proposals on NNPF-related SEI messages were classified by JVET-AE0187-v3 into the following 14 categories based on contributions in section 6 of the draft JVET meeting notes:

1)	On missing value ranges for ue(v)-coded syntax elements

2)	On order, value inference, conditional presence, etc., of syntax elements and semantics

3)	On the persistence scope of NNPF SEI messages

4)	On presence of multiple NNPFC and NNPFA SEI messages and on NNPF activation

5)	On NNPF input pictures and repeated NNPF inference

6)	On NNPF generated pictures and NNPF output pictures

7)	On NNPF purpose(s) determined by the application

8)	On extensibility

9)	On base NNPF and NNPF updates

10)	Editorial changes

11)	On new NNPF purposes/functionalities

12)	On NNPF cascades, alternatives, etc.

13)	On region-based NNPF operations

14)	On NNPF SEI messages and phase indication SEI messages for AVC and HEVC

The following two additional contributions on other SEI topics potentially relevant to VSEI v3 and VVC v3 were also discussed in the BoG:

15)	JVET-AE0054 AHG2/AHG9: Editorial improvements of annotated regions SEI message [T. Chujoh, T. Aono, T. Ikai (Sharp)]

16)	JVET-AE0155 Editor commentary on the post-filter hint SEI message semantics [G. J. Sullivan, Y.-K. Wang (Editors)]

Below is the list of design questions / proposals and their discussion and BoG-recommended dispositions.

- On missing value ranges for ue(v)-coded syntax elements

This item was discussed in the BoG at the beginning of its discussions on 11 July

  - Add missing value ranges for resampling syntax elements nnpfc_pic_width_num_minus1, nnpfc_pic_width_denom_minus1, nnpfc_pic_height_num_minus1, and nnpfc_pic_height_denom_minus1 to be in the range of 0 to 65535, inclusive. (JVET-AE0048 item 1)

BoG recommendation (correction to define missing value range): Decoders shall allow nnpfc_pic_width_num_minus1, nnpfc_pic_width_denom_minus1, nnpfc_pic_height_num_minus1, and nnpfc_pic_height_denom_minus1 to be in the range of 0 to 65535, inclusive.

  - Add missing value ranges for nnpfc_inp_format_idc and nnpfc_out_format_idc. (JVET-AE0048 item 2, JVET-AE0106)

Note: This is only for what values decoders would be required to allow; encoders only have a couple of values currently defined.

BoG recommendation (correction to define missing value range): 0 to 255, inclusive.

    - To be in the range of 0 to 255, inclusive. (JVET-AE0048 item 2)

    - To be in the range of 0 to 7, inclusive. (JVET-AE0106)

- On order, value inference, conditional presence, etc., of syntax elements and semantics

  - Arrange the definitions of DeriveInputTensors(), the syntax elements related to overlap and padding type, and the definition of the InpSampleVal function so that forward references are avoided by either of the following (JVET-AE0048 item 3):

BoG recommendation (Ed. cleanup): Proposed approach ii, since that approach is only editorial.

    - Move all the overlap and padding type syntax elements earlier in the syntax, between the syntax elements defining the input format and the syntax elements defining the output format, i.e., just before nnpfc_out_format_idc. Similarly, move the related semantics and function derivations respectively.

    - Move the derivations of strengthControlScaledVal (Formula 87), DeriveInputTensor (Formula 88), and StoreOutputTensor (91) to a later position that immediately follows the derivation of the InpSampleVal function.

  - Clarify that indicated or inferred values of vui_colour_primaries, vui_transfer_characteristics, vui_matrix_coeffs, and vui_full_range_flag apply when nnpfc_separate_colour_description_present_flag is equal to 0. Here, the possibility of inferring these syntax elements is made explicit when compared to JVET-AD2006. (JVET-AE0048 item 4)

BoG recommendation (Ed. clarification of existing intent): Adopt.

  - Signal nnpfc_chroma_loc_info_present_flag only when nnpfc_out_order_idc is not equal to 0. Infer nnpfc_chroma_loc_info_present_flag to be equal to 0 when not present. (JVET-AE0060 item 1)

BoG recommendation (BF): Adopt.

  - Move the syntax present when nnpfc_mode_idc is equal to 1 to be immediately after the syntax present when nnpfc_mode_idc is equal to 0, such that information about the network topology and parameters of the NNPF is after the NNPF properties regardless of the value of nnpfc_mode_idc. This also allows for sharing of the byte alignment syntax for nnpfc_mode_idc equal to 0 and nnpfc_mode_idc equal to 1, including merging of two syntax elements nnpfc_reserved_zero_bit_a and nnpfc_reserved_zero_bit_b into one syntax element nnpfc_reserved_zero_bit. (JVET-AE0126 item 1)

It was commented that if the tag is not understood, with the current scheme, other syntax can be skipped.

It was also commented that having the URI appear early in the syntax could be helpful for prefixing.

No action was recommended on this subtopic by the BoG.

  - Make either of the following changes: (JVET-AE0175)

    - Clarify that the base NNPF can only be in the ISO/IEC 15938-17 bitstream format and the update can be any formats.

    - Clarify that the nnpfc_mode_idc only indicates whether it is the base NNPF or updates relative to the base NNPF and the format exclusively indicates the format of the data regardless of the value of nnpfc_mode_idc.

The current sentence: “nnpfc_mode_idc equal to 0 indicates that this SEI message contains an ISO/IEC 15938-17 bitstream that specifies a base NNPF (when nnpfc_base_flag is equal to 1) or is an update relative to the base NNPF with the same nnpfc_id value (when nnpfc_base_flag is equal to 0).”

Suggestion 0: “nnpfc_mode_idc equal to 0 indicates that this SEI message contains an ISO/IEC 15938-17 bitstream. This ISO/IEC 15938-17 bitstream specifies a base NNPF (when nnpfc_base_flag is equal to 1) or is an update relative to the base NNPF with the same nnpfc_id value (when nnpfc_base_flag is equal to 0).”

Suggestion 1: “nnpfc_mode_idc equal to 0 indicates that the stuff is in the SEI message. nnpfc_mode_idc equal to 1 indicates the stuff is in a location identified by a URI. When nnpfc_mode_idc is equal to 0, the stuff is in the format specified by ISO/IEC 15938-17.”

BoG recommendation (Ed. clarification): Clarify using the approach in suggestion 1.

  - Change nnpfc_reserved_zero_bit(_a/_b) to nnpfc_alignment_zero_bit(_a/_b). It is asserted that there is no need to reserve the value 1 for this byte alignment bit, as the VVC and VSEI specs almost always just simply specify it as "shall be equal to 0". Only one of all cases in the latest VVC and VSEI specs is specified as a reserved bit, that is ptl_reserved_zero_bit. Even that one is not really necessary; however, that one is a bit special, as the bits in the PTL syntax structure are way more precious than other places, because the PTL syntax structure are often directly copied into top-level systems signalling/negotiation of capabilities. (JVET-AE0126 item 2)

BoG recommendation (cleanup/consistency/simplicity): Adopt.

  - Change the condition for the constraint on either nnpfcOutputPicWidth is not equal to CroppedWidth or nnpfcOutputPicHeight is not equal to CroppedHeight from "when nnpfc_pic_width_num_minus1, nnpfc_pic_width_denom_minus1, nnpfc_pic_height_num_minus1, and nnpfc_pic_height_denom_minus1 are present" to "when resolutionResamplingFlag is equal to 1". From the meaning point of view, it should be better to use resolutionResamplingFlag equal to 1 as the condition instead of the presence of the syntax elements, as the essence herein is really to disallow not changing either the width or height when the NNPF purpose includes resolution resampling. (JVET-AE0126 item 5)

BoG recommendation (Ed. simplification of expression): Adopt.

  - Move nnpfa_target_base_flag to be immediately after nnpfa_persistence_flag, such that the persistence flag, when present, immediately follows the cancel flag, thus these two flags specifying the persistence scope are together, same as all other SEI message syntax containing a cancel flag and a persistence flag. (JVET-AE0126 item 7)

BoG recommendation (cleanup/consistency): Adopt (subject to disposition of item 1 of JVET-AE0189; this was confirmed in review of the first version of the BoG report by the JVET plenary).

  - Update the constraint on when an NNPFC SEI message shall be a repetition of the first NNPFC SEI message in the CLVS to be as follows, such that it is clearly specified that the current CLVS is the CLVS containing the NNPFC SEI message nnpfcB, and that the first NNPFC SEI message nnpfcA is within the current CLVS: (JVET-AE0128 item 2)

When an NNPFC SEI message nnpfcB is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, which contains nnpfcB, and the value of nnpfc_base_flag is equal to 1, the NNPFC SEI message shall be a repetition of the first NNPFC SEI message nnpfcA with the same nnpfc_id value, in decoding order, within the current CLVS, i.e., the payload content of nnpfcB shall be the same as that of nnpfcA.

BoG recommendation (Ed. clarification and simplification of existing intent): State that “All NNPFC SEI messages in a CLVS that have a particular nnpfc_id value and nnpfc_base_flag equal to 1 shall have identical SEI payload content.” (or editorial equivalent)

  - To allow colourization on non-4:0:0 video, which was claimed to be the intent agreed at the previous meeting, remove "ChromaFormatIdc or" from the following constaint: (JVET-AE0141)

When ChromaFormatIdc or chromaUpsamplingFlag is not equal to 0, colourizationFlag shall be equal to 0.

BoG recommendation (Ed. correction to express existing intent): Remove ChromaFormatIdc from the constraint for when colourizationFlag shall be equal to 0.

- On persistence scopes of the NNPF SEI messages

Discussed in the BoG at 15:40 on 11 July

  - Cancel the persistence of an NNPFA SEI message with any subsequent NNPFA SEI message with the same nnpfa_target_id, Here, when compared to JVET-AD2006, the requirement on nnpfa_cancel_flag equal to 1 is removed for cancelling the persistence. (JVET-AE0048 item 5)

BoG recommendation (Ed. correction to express existing intent): Adopt. The editors are also requested to check/clarify the existing intent that if the current picture contains an SEI message (any SEI message that uses the typical persistence scope scheme) with a cancel flag equal to 1, the current picture is not within the scope of the previous SEI message (regardless of whether the SEI message is a suffix or prefix SEI message).

  - Update the constraint related to nnpfcTargetPictures and nnpfaTargetPictures by also considering the value of nnpfa_target_base_flag, such that the constraint is as follows: (JVET-AE0189 item 3)

Let the nnpfcTargetPictures be the set of pictures to which the NNPFC SEI message corresponding to the target NNPF pertains. Let nnpfaTargetPictures be the set of pictures for which the target NNPF is activated by the current NNPFA SEI message. It is a requirement of bitstream conformance that any picture included in nnpfaTargetPictures shall also be included in nnpfcTargetPictures.

BoG recommendation (Ed. correction to express existing intent): Adopt as expressed above (or editorial equivalent).

- On presence of NNPFC and NNPFA SEI messages and on NNPF activation

  - Allow multiple NNPFC and NNPFA SEI messages with different content in the same picture unit by excluding NNPFC and NNPFA SEI messages from the existing constraint in JVET-AD2005, subclause D.3.1. If item 3)a (i.e., JVET-AE0048 item 5) is agreed the NNPFA SEI message is constrained in a manner that the payload shall be identical for the same nnpfa_target_id value. Otherwise, an NNPFA SEI message cancelling the previous NNPFA SEI message followed by an NNPFA SEI with the same nnpfa_target_id for activating an NNPF are allowed for the same picture unit. (JVET-AE0049 item 1).

BoG recommendation (Ed. correction to express existing intent): Adopt.

BoG suggested editor action item: Editors are requested to make sure the blanket constraint in VVC on constraining SEI payload content to be the same within a PU does not impose a problem for SEI messages that contain ID values. No such blanket constraint appears to exist in HEVC, and this was intentional for some SEI messages such as tone mapping information and colour remapping information SEI messages.

  - If the design intent is that both a base NNPF and an update of the NNPF can be activated at the same time for the same picture (which is not the design intent), act on 4)b.i; otherwise (the intent is that only one of a base NNPF and an update of the NNPF can be activated for a picture), act on 4)b.ii. (JVET-AE0189 item 1)

    - Move nnpfa_target_base_flag to be not conditioned on the value of nnpfa_cancel_flag, and specify that, when nnpfa_cancel_flag is equal to 1, the NNPFA SEI message cancels the persistence of the target NNPF that was previously activated with the same value of nnpfa_target_id and nnpfa_target_base_flag.

    - Add a constraint such that there shall not be two or more NNPFA SEI message active (i.e, with nnpfa_cancel_flag equal to 0) for the same picture regardless their value of nnpfa_target_base_flag.

The BoG agreed that no action is needed on this, since the issue is resolved by the action on 4)a.

BoG suggested editor action item: Editors are requested to make sure it is clear that activating an “update” can (and basically always would) require using information from a “base” filter in order to interpret the update.

  - If it is intended to be disallowed for an NNPFA SEI message with nnpfa_target_base_flag equal to 0 to activate a base NNPF (and yes, that is the intent), add a constraint as in 4)c.i. Otherwise, add a NOTE as in 4)c.ii: (JVET-AE0189 item 2)

    - When nnpfa_target_base_flag in an NNPFA SEI message is equal to 0, there shall be at least one NNPFC SEI message with nnpfc_id equal to nnpfa_target_id and nnpfc_base_flag equal to 0 that precedes the NNPFA SEI message in decoding order.

    - NOTE –When nnpfa_target_base_flag is equal to 0, the target NNPF can be a base NNPF in the case where there is no NNPFC SEI message with nnpfc_id equal to nnpfa_target_id and nnpfc_base_flag is equal to 0 that precedes the NNPFA SEI message in decoding order.

BoG recommendation (Ed. correction to express existing intent): Adopt 4)c.i.

This topic marked the end of the BoG discussion of 11 July at 18:30 in BoG.

- On NNPF input pictues and repeated NNPF inference

This topic was addressed at the beginning of the BoG discussion of 12 July at 08:30 in BoG

  - Constrain the repeated inference to generate interpolated pictures up to the end of the bitstream only to NNPFs that perform only picture rate upsampling. (JVET-AE0049 item 2)

BoG recommendation (Ed. correction/simplification to express existing intent): Adopt.

  - Change the index range in input picture assignment for repeated inference at the end of a bitstream: it is proposed to remove "− j" from "When numInputPics is greater than 1, the following applies for each value of i in the range of j + 1 to numInputPics − j − 1, inclusive, in increasing order of i:" (JVET-AE0049 item 3, JVET-AE0142 item 1)

BoG recommendation (Ed. correction to express existing intent): Adopt.

  - As an asserted bug fix, in the following sentence in the NNPFC interface text, replace "inputPic" with "resampledPic": (JVET-AE0142 item 2)

If sourceWidth is equal to CroppedWidth and sourceHeight is equal to CroppedHeight, inputPic is set to be the same as sourcePic.

BoG recommendation (Ed. correction to express existing intent): Adopt.

  - As an asserted bug fix, in the NNPFC interface text, condition the following constraint on "when pictureRateUpsamplingFlag is equal to 1", because currently this constraint can also be applied for cases where pictureRateUpsamplingFlag is equal to 0: (JVET-AE0142 item 3)

It is a requirement of bitstream conformance that num_interpolated_pics[ i − 1 ] shall not be greater than 0.

It was asked if this should instead be for when the nnpf_purpose is equal to 8, but this was not advisable because of cases other than the end of the bitstream.

BoG recommendation (Ed. correction to express existing intent): Adopt (and num_interpolated_pics[ i − 1 ] shoud be nnpf_interpolated_pics[ i − 1 ]).

  - Add a syntax element nnpfa_no_prev_clvs_flag to the NNPFA SEI message. When equal to 1, nnpfa_no_prev_clvs_flag specifies that input pictures that would originate from a previous CLVS in output order are treated as if they were not present in the bitstream. When nnpfa_no_prev_clvs_flag is equal to 0, the current operation in JVET-AD2005 and JVET-AD2006 is kept unchanged. The value of nnpfa_no_prev_clvs_flag can be changed from 0 to 1, when the current CLVS is spliced from another bitstream next to the previous CLVS in output order and this NNPFA SEI message would cause one or more input pictures to be selected from the previous CLVS in output order. (JVET-AE0050 item 1)

The inferred values of luma and chroma equal to 0 associated with nnpfc_absent_input_pic_zero_flag equal to 1 were discussed, with a participant questioning the sensibility of these values. Another participant said a filter could recognize these as special values and do something different in response to them. In the JVET plenary review of the first version of the BoG report, this was further discussed, with no change recommended.

The next subitem is closely related.

  - Add a new flag named nnpfa_broken_link_flag in the NNPFA SEI message to specify whether the result of applying the target NNPF may result in broken-link / mismatch (i.e., filtered picture in decoder that is different from is corresponding filtered picture at encoder). When the bitstream is spliced at a CLVS that resulted in one or more input pictures of the target NNPF becomes unavailable, the value of nnpfa_broken_link_flag is set to be equal to 1. The nnpfa_broken_link_flag can be used by decoder to decide whether or not to activate / apply the NNPF. (JVET-AE0070 item 1)

The basic difference between the two approaches is that in approach “e”, the syntax indicates what to do, while in approach “f”, the syntax would just provide a warning without an indication of how the decoder should react to that warning.

BoG recommendation (cleanup of corner-case behaviour): Adopt approach 5)e.

  - Add nnpfa_no_foll_clvs_flag to the NNPFA SEI message, gated by nnpfa_persistence_flag being equal to 1. nnpfa_no_foll_clvs_flag equal to 1 specifies that when this NNPFA SEI message persists for the last PU of a CLVS in output order, the NNPFA SEI message is treated as if it persisted for the last PU of the current layer within the bitstream. In this case, the picture rate upsampling post-filter may be invoked for multiple sets of input pictures until pictures are interpolated between the last pair of input pictures within the CLVS in output order. When nnpfa_no_foll_clvs_flag is equal to 0, the current operation in JVET-AD2005 and JVET-AD2006 is kept unchanged. (JVET-AE0050 item 2)

The next subitem is related (but not the same).

BoG recommendation (cleanup of corner-case behaviour): Adopt.

  - At the end of a bitstream, add repeated inference of NNPF for creating pictures corresponding to input pictures (regardless of the purpose indicated by nnpfc_purpose except for picture rate upsampling) when there are multiple input pictures to which the NNPF applies: When an NNPF is active until the end of the bitstream and the current picture for which the NNPF is activated is the last picture in output order in the bitstream, the NNPF is applied up to but excluding a set of input pictures that would cause creation of any NNPF-generated picture after the last picture of the bitstream in output order. (JVET-AE0134 item 2)

It was commented that it would be more correct to use the nnpfa_output_flag rather than the nnpfc_input_pic_output_flag in this modification.

BoG recommendation (cleanup of corner-case behaviour): Adopt (subject to offline checking of the suggested modification, later confirmed in JVET plenary review of the first version of the BoG report to use nnpfa_output_flag).

- On NNPF generated pictures and NNPF output pictures

  - Enable persistent activation of an NNPF that outputs multiple filtered input pictures or interpolates pictures between multiple pairs of input pictures as follows: (JVET-AE0051)

    - In the NNPFA SEI semantics of the VSEI text: Derivation of the pictures for which the NNPF is activated when nnpfa_persistence_flag is equal to 1 at the smallest interval such that the same picture is not output by the NNPF process more than once in subsequent NNPF activations and interpolated pictures are not generated between the same pair of input pictures in subsequent NNPF activations.

    - In the NNPF SEI use of the VVC text: The repeated inference to generate interpolated pictures up to the end of the bitstream is performed at the smallest interval that does not cause interpolated pictures to be generated between the same pair of input pictures in subsequent NNPF inferences.

It was commented that the text changes for this seem a bit too complicated and it might really be more of a proposed extra functionality, and thus its consideration could be deferred to v4 or beyond.

The BoG thus recommended no action on this (at least for v3).

Another participant questioned the usefulness of NNPFA persistence in general, saying it could be simpler to just send an NNPFA for every picture (since they are small). It was noted that on the previous day it had been pointed out that persistence doesn’t really operate the same way for NNPF as it does for other SEI messages. In the interest of stability of the planned approach, this line of thinking was not further discussed.

  - Add the following NOTE to clarify an implied, hidden constraint on picture-rate-upsampling NNPFs with persistent activation: (JVET-AE0135 item 1)

NOTE – When pictureRateUpsamplingFlag is equal to 1 for an NNPF and the NNPFA SEI message that activated this NNPF has nnpfa_persistence_flag equal to 1, nnpfc_interpolated_pics[ i ] is greater than 0 only for a single value of i in the range of 0 to numInputPics − 1, inclusive.

It was asked whether the text should also express a similar observation for other cases, for example, when nnpfc_purpose indicates resolution upsampling only, when there are multiple input pictures and multiple output pictures. This suggestion was supported.

BoG recommendation (Ed.): Adopt (editorial only, with expansion to consider all possible cases that have a similar implication).

  - Act on one of the following three suggestions for picture-rate-upsampling NNPFs, to avoid the seemingly vagueness and easy violation of the constraint on no more than one NNPF output picture for any particular output time instance: (JVET-AE0135 item 2)

    - Clearly specify that the interpolated pictures are uniformly located between each pair of consecutive input pictures.

    - Explicitly indicate an output time instance for each interpolated picture, e.g., through signalling a POC delta between the POC values of the interpolated picture and the input picture closest and succeeding in output order.

It was commented that POC is not intended as timing information, although it could be used to establish relative order.

    - Disallow generating NNPF output pictures between any particular pair of consecutive input pictures more than once.

It was suggested that unless some strong need to do otherwise is identified, approach 6)c.iii seems like the practical action to take.

BoG recommendation (BF / clarification): Approach 6)c.iii, pending text review (remaining open at the time of the JVET plenary review of the first version of the BoG report).

  - Add the following constraint to ensure that there is at least one NNPF output picture when an NNPF is activated and applied: (JVET-AE0069)

When nnpfc_purpose & 0x08 is equal to 0 for the associated NNPFC [SEI message] and nnpfa_num_output_entries is equal to NumInpPicsInOutputTensor, nnpfa_output_flag[ i ] shall be equal to 1 for at least one value of i in the range of 0 to nnpfa_num_output_entries − 1, inclusive.

BoG recommendation (BF / clarification of existing intent): Adopt (with some editorial refinement).

  - Add the following constraint as part of the semantics of nnpfa_output_flag[ i ], to ensure that input pictures that are not present in the bitstream do not have a corresponding NNPF output picture: (JVET-AE0070 item 2)

When the target NNPF takes multiple input pictures and one or more of the input pictures are not present in the bitstream (i.e., unavailable input pictures) and the unavailable input pictures have associated output pictures (i.e., the value of their associated nnpfc_input_pic_output_flag[ i ] in NNPFC SEI message is equal to 1), for each of those unavailable input pictures that has associated output picture, the nnpfa_output_flag[ i ] associated with such input picture shall be present and be equal to 0.

It was commented that item f is related. See further notes for that item.

  - Add the following constraint as part of the process for derivation of the NNPF input pictures in the interface text, to ensure that input pictures that are not present in the bitstream do not have a corresponding NNPF-generated picture (and consequently do not have a corresponding NNPF output picture): (JVET-AE0134 item 1)

It is a requirement of bitstream conformance that, when inputPresentFlag[ i ] is equal to 0, nnpfc_input_pic_output_flag[ i ] shall be equal to 0.

It was commented that the desire is to prohibit output of unavailable pictures that are before the first picture in the bitstream. The control of the pictures that are output is in the NNPFA SEI message.

It was suggested that since a similar issue is addressed in the VVC interface text, it would be best to express this constraint there as well.

BoG recommendation (BF / clarification of existing intent): Adopt approach 6)e, expressed in the VVC interface text. This was agreed in the JVET plenary review of the first version of the BoG report to use the nnpfa_output_flag rather than the nnpfc_input_pic_output_flag in this modification.

- On NNPF purpose(s) determined by the application

  - Signal a new syntax element nnpfc_application_purpose_tag_uri when nnpfc_purpose is equal to 0 (i.e. when the NNPF may be used as determined by the application) to provide more details about the application-specific purpose. (JVET-AE0061 item 1)

As proposed, the tag URI would be mandatory to be present when nnpfc_purpose is equal to 0.

It was commented that nnpfc_purpose equal to 0 is for an application-specific purpose that may not benefit from the extra tag signalling; others said the tag could be useful for some external organization to document/indicate a particular purpose.

One participant indicated that allowing indication of an application-specific purpose should be sufficient, without needing to make additional provisions for support of information about application-specific purposes – e.g., additional information could be signalled in user data.

It was commented that this may be new functionality that could be deferred for later consideration beyond v3. There is a metadata extension capability that could indicate this.

It was suggested to make the tag presence optional. There was some support for that.

No action was recommended on this subtopic by the BoG for v3; this could be considered for a future version.

  - Signal one bit in the nnpfc_purpose syntax element to specify some additional purpose of the NNPF as determined by the application, for use in combination with other pre-defined NNPF usages. (JVET-AE0061 item 2)

It was commented that this could cause confusion with nnpfc_purpose equal to 0. The proponent pointed out that the proposal has a constraint to prevent setting only this bit equal to 1 without also indicating some defined purpose.

The proponent said another possibility is to allow this and remove the nnpfc_purpose equal to 0 (without the constraint that some other purpose needs to be indicated). Others indicated that text/design stability at this stage should be preferred over considering some significant change of how an application-specific purpose would be indicated.

Examples suggested by the proponent were dehazing (not a current defined purpose) together with some existing defined purpose such as chroma upsampling.

Participants other than the proponent did not see a clear need for this simultaneous indication of application-specific purpose along with other currently defined purposes. Such supplemental indications could potentially be accomplished in other ways (e.g. user data or system data).

It was commented that having this flag equal to 1 might cause decoders to become confused about whether to use the filter if they don’t understand the application-specific purpose, which could basically make the indication the same as only indicating an application-specific purpose.

No action was recommended on this subtopic by the BoG, at least for v3.

- On extensibilities

  - Add u(8)-coded nnpfc_purpose_len just before nnpfc_purpose, and change the coding of nnpfc_purpose from u(16) to (v). (JVET-AE0068)

One participant commented that by the time we use up all of the existing purpose bits, it may be unlikely we would want to keep using the same SEI message.

Another commented that there are other ways to deal with this, such as reserving an “escape” value; v3 decoders are already required to ignore the message if they don’t understand the indicated purpose.

Subitem b is related.

No action was recommended on subtopics a and b by the BoG.

  - Change the coding of nnpfc_purpose from u(16) to u(24) or ue(v). Additionally, this contribution demonstrates a mechanism on how to add additional NNPFC purposes in a future v4 version of VSEI such that, according to the proponents, a v4 decoder would decode v3 NNPFC SEI messages to indicate “unknown” for those additional purposes. The mechanism is claimed to work without any text changes to the v3 text, so no other change but the change to u(24) or ue(v) is being proposed. (JVET-AE0113)

Subitem a is related. See notes for that.

  - Specify the metadata extension mechanism, enabled by the syntax elements nnpfc_metadata_extension_num_bits and nnpfc_reserved_metadata_extension, to be applied only when nnpfc_mode_idc is equal to 0, by making the following changes: (JVET-AE0128 item 1)

    - The signalling of the syntax element nnpfc_metadata_extension_num_bits is skipped when nnpfc_mode_idc is not equal to 0.

    - It is specified that, when nnpfc_metadata_extension_num_bits is not present, its value is inferred to be equal to 0.

The proponent noted that the existing general SEI message extension mechanism is available for use when nnpfc_mode_idc is not equal to 0.

Another participant said it may be nicer to be able to use the same mechanism for multiple modes.

The proposed modification is not really necessary; there is no clear bug.

No action was recommended on this subtopic by the BoG.

The BoG stopped for a lunch break here at 12:30 on 12 July.

- On base NNPF and NNPF updates

The BoG resumed is discussions here at 14:30 on 12 July.

  - As an asserted bug fix, specify that all NNPFC SEI messages with a particular value of nnpfc_id within a CLVS shall have the same value of nnpfc_purpose. (JVET-AE0142 item 4)

BoG recommendation (BF / sensibility correction of existing intent): Adopt. This was further discussed in the JVET plenary review of the BoG report. The persistence of an NNPFC is for a CLVS and the indicated purpose is part of the NNPFC, so this constraint is to require alignment of the two. We already have a constraint that an NNPFC update that has property parameters signalled shall have the same indicated purpose as the base NNPF. It was suggested that “purpose” might really mean “capability”.

  - Make the following changes: (JVET-AE0173, updated)

    - Move nnpfa_target_base_flag out of the syntax condition "if( !nnpfa_cancel_flag )".

We can activate a base even after sending an update. The intended interpretation of the current syntax was discussed.

BoG recommendation (clarification of existing intent): Clarify, as necessary, that an NNPFA message can activate a base with a particular ID when an update is active, which will switch the filter from the updated filter to the base filter.

    - NNPFA message with nnpfa_cancel_flag equal to 0 and nnpfa_target_base_flag equal to 0 shall only be sent to the NNPF which is not active. The NNPFA messages with nnpfa_cancel_flag equal to 0 and nnpfa_target_base_flag equal to 1 shall be ignored if the NNPF whose nnpfc_id equal to nnpfa_target_id of the NNPFA message is not active when the message is received by the receiver.

It was commented that it should be OK to receive a cancel indication without having previously received relevant data with the same ID.

Other SEI messages use the word “any” to cover this possibility. The same word is used for this new one.

No action was recommended on this subtopic by the BoG.

    - (from updated contribution) Adding a flag nnpfa_cumulative_update_flag such that if nnpfa_cumulative_update_flag is equal to 1, the target NNPF is the NNPF specified by combination of all NNPFC SEI messages with nnpfc_id equal to nnpfa_target_id that precedes the first VCL NAL unit of the current picture in decoding order and after the NNPFC SEI message contains the base NNPF which is not a repetition.

It was commented that this proposes adding an additional mode of operation and we do not have evidence of benefit of the proposed cumulative update behaviour, and also that cumulative update could cause a loss resilience problem.

No action was recommended on this subtopic by the BoG, at least for v3.

    - Target of NNPFA message with nnpfa_cancel_flag is decided by the value of nnpfa_target_base_flag. If nnpfa_target_base_flag is equal to 0 then the target NNPF is completely deactivated and is no longer used until it is activated by another NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 0. If nnpfa_target_base_flag is equal to 1 then the updates applied to the base NNPF is deactivated, i.e. the base NNPF is applied to the pictures following the current NNPFA SEI message in decoding order.

This aspect had been clarified in the context of discussion of the first aspect.

No action was recommended on this subtopic by the BoG.

- Editorial changes.

BoG recommendation (Ed.): Delegate these minor refinements to the editors for consideration.

  - Clarify the description of the InpIdx[ idx ] variable to by adding "in inverse output order" as follows: (JVET-AE0048 item 5)

InpIdx[ idx ] specifying the input picture index of the idx-th picture in inverse output order that is present in the output tensor of the NNPF and has a corresponding input picture

  - Rename the syntax element nnpfc_input_pic_output_flag[ i ] to be nnpfc_input_pic_filtering_flag[ i ] and change the semantics accordingly, as this flag indicates whether for the i-th input picture the NNPF generates a corresponding filtered picture stored in the output tensor, not whether the NNPF outputs a corresponding picture, which is controlled by the nnpfa_output_flag[ i ] instead. This also includes changing of the name of the variable numOutputPics to numPicsInOutputTensor. (JVET-AE0126 item 3)

  - Change the local variables chromaUpsamplingFlag, resolutionResamplingFlag, pictureRateUpsamplingFlag, bitDepthUpsamplingFlag, and colourizationFlag to be global variables ChromaUpsamplingFlag, ResolutionResamplingFlag, PictureRateUpsamplingFlag, BitDepthUpsamplingFlag, and ColourizationFlag, because most, if not all, of them are used in more than one (sub)clauses of the text. (JVET-AE0126 item 4)

  - Change all instances of "Values of se_a greater than y shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use" to "The value of se_a shall be in the range of 0 to y, inclusive", as the latter is a wording consistent with the usual way of expressing the value range for a ue(v)-coded syntax element in AVC/HEVC/VVC/VSEI specifications. (JVET-AE0126 item 6)

  - Other miscellaneous proposed editorial changes not covered above (found in JVET-AE0126 attachment, JVET-AE0127 attachment, and ticket #1605)

- On new NNPF purposes/functionalities (the BoG suggested these are not for v3 consideration, as further study seemed necessary and late addition of new features seems undesirable)

  - Add decolourization (converting colour video to monochrome) as a new nnpfc_purpose. (JVET-AE0060 item 2)

It was suggested that “recolourization” may be different from “colourization”.

  - Add deinterlacing as a new NNPF purpose, by the following: (JVET-AE0062)

    - Signal a flag that indicates the number of output pictures for each interlaced input picture to the NNPF.

    - Signal if the input needs to be flipped vertically so that a neural network trained on top field first can work on bottom field first content and vice-versa.

It was commented that an NNPF might not only deinterlace content that is known to be interlaced, but might involve detecting whether the content is interlaced.

- On NNPF cascades, alternatives, etc. (the BoG and the contributions themselves suggested these are not for v3 consideration, as further study seemed necessary and late addition of new features seems undesirable)

  - Add support of NNPF cascades and alternatives as follows: (JVET-AE0052)

    - Add a new SEI message named the NNPF group characteristics (NNPFGC) SEI message, which can be used to define either a cascade of NNPFs or indicate a set of alternative NNPFs or alternative NNPF cascades.

    - Add a new SEI message named the NNPF group activation (NNPFGA) SEI message, which is used for activating an NNPF cascade.

    - Update the general post-processing filtering process using NNPFs to take into account the proposed NNPFGC and NNPFGA SEI messages.

  - Add a new SEI message NNPF group SEI message to indicate multiple NNPFs as a group/list and related operations for a picture. The related operations for the indicated multiple NNPFs consist of four types: cascade, alternate, coherence and option. (JVET-AE0063)

- On region-based NNPF operations (the BoG and the contributions themselves suggested these are not for v3 consideration, as further study seemed necessary and late addition of new features seems undesirable)

  - Add a new SEI message named the NNPF regions (NNPFR) SEI message. The NNPFR SEI message synchronizes the annotated regions SEI message with the NNPF SEI messages, identified by nnpfr_target_id, for post-processing filtering of regions in pictures. The regions in the pictures indicated by the annotated regions SEI message are activated by the NNPFA SEI message and post-filtered by the NNPFC SEI messages. (JVET-AE0053)

- On NNPF SEI messages and phase indication SEI messages for AVC and HEVC (the BoG suggested no immediate incorporation into these for Consent at this meeting, but encouraged further development, with possibly issuing some draft text(s) at this meeting and ISO/IEC timeline planning)

  - Proposes adding the support of the NNPFC SEI message, the NNPFA SEI message, and the phase indication SEI message to AVC and HEVC. (JVET-AE0101)

Non-NNPF topics reviewed in the BoG follow as items 15 and 16 below.

- JVET-AE0054 AHG2/AHG9: Editorial improvements of annotated regions SEI message

For the BoG consideration of this topic, see the notes in section 6.2.

- JVET-AE0155 Editor commentary on the post-filter hint SEI message semantics [G. J. Sullivan, Y.-K. Wang (Editors)]

For the BoG consideration of this topic, see the notes in section 4.2.

The BoG discussions ended with this topic at 1830 on 12 July.

JVET-AE0187 AHG9: A summary of SEI proposals on NNPF [Y.-K. Wang (Bytedance)] [late]

JVET-AE0048 AHG9: Miscellaneous VSEI changes on neural-network post-filter SEI messages [M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia)]

JVET-AE0049 AHG9: Miscellaneous VVC changes on neural-network post-filter SEI messages [M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia)]

JVET-AE0050 AHG9: On NNPF input picture selection [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AE0051 AHG9: On persistent NNPF activation [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AE0052 AHG9: NNPF cascades and alternatives [M. M. Hannuksela, F. Cricri (Nokia)] [late]

This was discussed in JVET on Friday July 14.

This input document proposes the following aspects to be included in a working draft of a VSEI amendment:

- NNPF group characteristics (NNPFGC) SEI message, which can be used to define either a cascade of NNPFs or indicate a set of alternative NNPFs or alternative NNPF cascades.

- NNPF group activation (NNPFGA) SEI message, which is used for activating an NNPF cascade.

- An update of the general post-processing filtering process using NNPFs to take into account the proposed NNPFGC and NNPFGA SEI messages.

The NNPF group characteristics (NNPFGC) SEI message contains:

  - An identifier of the NNPF group

  - A grouping type of the NNPF group, where 0 indicates a cascade of NNPFs and 1 indicates alternative NNPFs or alternative NNPF cascades

  - The purpose of the NNPF group, the presence of which is conditioned by the grouping type indicating a cascade of NNPFs.

Note: When the grouping type indicates alternative filters, their purpose is available in the NNPFC SEI messages defining the filters.

  - The number of members in the NNPF group, where each member can be an NNPF or an NNPF group of a cascade of NNPFs

  - The identifiers of the NNPFs or NNPF groups belonging to this NNPF group

  - A flag indicating if the complexity information for the NNPF group is present

  - Complexity information specified like in the NNPFC SEI message

The general post-processing filtering process using NNPFs is proposed to be modified as follows:

  - It is clarified that the bitstream may be filtered multiple times to produce different results.

  - Selecting between alternatives indicated in an NNPFGC SEI message is added.

  - Applying an NNPF cascade as indicated in NNPFGC and NNPFGA SEI messages is added.

The NNPFGA SEI message contains:

- nnpfga_target_id, nnpfga_cancel_flag, and nnpfga_persistence_flag specified like nnpfa_target_id, nnpfa_cancel_flag, and nnpfa_persistence_flag, respectively, but applying to an NNPF group rather than to a single NNPF.

- The number of filters in the NNPF group

- For each filter in the NNPF group, the NNPFGA SEI message contains:

    - nnpfga_target_base_flag[ i ] specified for the i-th NNPF in the NNPF group like nnpfa_target_base_flag

    - An indication if the input pictures for the i-th NNPF in the NNPF group are formed from the list of candidate input pictures without skipping. If not, the following is indicated:

      - The number of input pictures for the i-th NNPF in the NNPF group

      - The picture count that is skipped in the list of candidate input pictures when selecting input pictures for the i-th NNPF in the NNPF group

- nnpfga_num_output_entries[ i ] and nnpfga_output_flag[ i ][ j ] specified for the i-th NNPF in the NNPF group like nnpfa_num_output_entries and nnpfa_output_flag[ j ]

The relation with SEI processing order SEI message was discussed. For NNPF, specific information is needed, for example, about the complexity, input/output signalling of temporal picture positions. Another aspect is that the NNPFGC can offer several alternative groups, and the receiver selects one.

It was pointed out that the example of hierarchical temporal upsampling in a cascade may not be used, as no new information is added to the intermediate picture. This example is the only one from the contribution which requires signalling of picture position. There could be other examples where several pictures are used as input to a subsequent neural network.

It was asked if the needed functionality (signalling of temporal position of input/output, alternative groups, operation) could also be achieved by extending the concept of SEI processing order SEI, rather than defining it specifically for the NNPF case.

JVET-AE0053 AHG8/AHG9: Neural-network post-filter regions SEI message [T. Chujoh, Y. Yasugi, T. Ikai (Sharp)]

This was discussed in JVET on Friday July 14.

The neural-network post-filter characteristic and activation SEI messages process picture by picture. In several past meetings, there were several proposals to activate regions in the picture, and it has been recommended to use the annotated regions SEI message, which carries parameters that identify annotated regions using bounding boxes representing the size and location of identified objects. However, information on using regions and labels specified in the annotated regions SEI message for post-processing is not defined. Therefore, this contribution considers a minimum specification of how to post-process regions and introduces a new SEI message indicating an ID for the annotated regions SEI message. This proposal is expected to apply to the VSEI (V4) and is asserted to be useful not only for ROI but also encoders and receiving systems for machine analysis of coded video content.

It was commented that region based activation of NNPF is a useful functionality. However, the combination with annotated regions SEI message may not be the best way to implement this, as its original purpose is different (more in the semantics of content, rather than processing). Furthermore, such an SEI message should support multiple regions, and also using different NNPF for different regions.

The functionality of region based processing is not supported in v3, and would be interesting for a future version. Further study was recommended.

JVET-AE0060 [AHG9] Comments on NNPFC [S. Deshpande (Sharp)]

JVET-AE0060 item 2 was further discussed in JVET on Friday July 14.

Some modifications are proposed to the Neural-Network Post-Filter Characteristics SEI Message. Following is proposed:

- Proposal 1: (discussed in BoG)

- Proposal 2: It is proposed to add decolourization (converting colour video to monochrome) as a new nnpfc_purpose.

It was asked why this would not be done at the encoder side

It would also be possible to use colourization, which also has three components as input, and produces an output which has zero chroma.

There was no evidence that this is needed.

JVET-AE0061 [AHG9] On NNPFC Application Purpose [S. Deshpande (Sharp)]

Item 1 was discussed in JVET on Friday July 14.

Some modifications are proposed to the Neural-Network Post-Filter Characteristics SEI Message related to the application determined purpose. The following is proposed:

- Proposal 1: It is proposed to signal a new syntax element nnpfc_application_purpose_tag_uri when nnpfc_purpose is equal to 0 (i.e. when the NNPF may be used as determined by the application) to provide more details about the application-specific purpose.

It was commented that the presence of the nnpfc_application_purpose_tag_uri syntax element should be gated by a flag, to allow future extensions.

Decision: Include in “TuC for VSEI”, with an additional flag.

JVET-AE0062 [AHG9] On NNPF for Deinterlacing [A. Sidiya, S. Deshpande (Sharp)]

This was discussed in JVET on Friday July 14.

It is proposed to allow specifying deinterlacing as a new NNPF purpose. For this following is proposed:

- It is proposed to signal a flag that indicates the number of output pictures for each interlaced input picture to the NNPF.

- It is proposed to signal if the input needs to be flipped so that a neural network trained on top field first can work on bottom field first content and vice-versa.

It was commented that this would be beneficial for legacy AVC content, which is valuable. In HEVC and VVC, deinterlacing would mostly be done before encoding.

It was commented that rotating would be inappropriate for the top-/bottom-field first problem, as also within the lines the scan from left to right would be converted right-to-left. It might be more general to use different networks for the two cases.

It was asked whether more than a pair of fields can be used as input. This is the case, as in the current NNPF for frame interpolation. It was reported that the rotation method also works in case of more than 2 fields as input.

It was commented that the necessity of having this is not clear, as de-interlacing is typically done in a display when the decoder output is in interlaced formats. What would be use cases that a content provider would send information about a recommended de-interlacing algorithm?

Further study was requested.

JVET-AE0063 [AHG9] On operations for multiple NNPFs [L. Chen, O. Chubach, Y.-W. Huang, S. Lei (MediaTek)]

This was discussed in JVET on Friday July 14.

This contribution proposes an SEI message indicating multiple NNPFs as a group/list and related operations for a picture in response to the four questions on item 1)c in JVET-AD0362. The related operations for the indicated multiple NNPFs consist of four types of cascade, alternate, coherence and optional.

(coherence=parallel)

Conceptually this is similar to JVET-AE0052. Parallel is not explicitly in JVET-AE0052. Another aspect is that the encoder can take provision that only a single filter is activated at one time. No specific group activation SEI is needed in JVET-AE0063

Parallel operation might, for example, be useful to generate attention mechanism input for a subsequent network. This is something that would not appear in other SEI messages.

Use cases for parallel operation need more study, including the question how many could be operated in parallel.

Further study required if it is possible to bring those aspects into the SEI processing order SEI message (in VVC), or if this is so specific to NNPF that it would be a burden for implementers who only want to use SEI processing order for simpler cases of SEI messages.

It was agreed to produce an output document “Technology under consideration for future extensions of VSEI”. Proponents of JVET-AE0052 and JVET-AE0063 should combine the aspects from both contributions, and propose a text part.

It was also commented that a software implementation demonstrating different options would be desirable.

JVET-AE0298 AHG9: Combined text of JVET-AE0052 and JVET-AE0063 [M. M. Hannuksela, F. Cricri (Nokia), L. Chen, O. Chubach, Y.-W. Huang, S. Lei (MediaTek)] [late]

This document contains a combination of JVET-AE0052 and JVET-AE0063 for potential inclusion to Technologies under Consideration for VSEI.

The NNPF group design in this section contains the following aspects:

NNPF group characteristics (NNPFGC) SEI message, which can be used to define any of the following:

- a cascade of NNPFs

- a group of NNPFs which are activated in an alternating manner so that at most one of these NNPFs is activated by NNPFA SEI messages for any picture unit

- a set of alternative NNPFs or alternative NNPF groups out of which only one is to be selected in the decoder side

- a set of NNPFs or NNPF groups intended to be applied in parallel

- NNPFs or NNPFs groups that are optional

NNPF group activation (NNPFGA) SEI message, which is used for activating an NNPF group.

Extension of the NNPFA SEI message semantics so that it can activate an NNPF group

An update of the general post-processing filtering process using NNPFs to take into account the proposed NNPFGC and NNPFGA SEI messages.

The combined text was presented in JVET Tuesday 18 July 1145.

The text contains a concept where the existing NNPFA SEI can be used to activate a group. It was asked whether a separate group activation SEI message also included in the text is needed at all. This is left for further study.

Decision: Adopt JVET-AE0298 into JVET-AE2032 TuC.

JVET-AE0068 AHG9: On extensibility of purpose syntax element in NNPFC SEI message [Hendry, J. Nam, S. Kim, J. Lim (LGE)]

JVET-AE0069 AHG9: On the signalling of output pictures in NNPFA SEI message [Hendry, J. Nam, S. Kim, J. Lim (LGE)]

JVET-AE0070 AHG9: On input pictures that are not present in the bitstream for NNPF SEI messages [Hendry, J. Nam, S. Kim, J. Lim (LGE)]

JVET-AE0101 [AHG2][AHG9] Neural network post filter and phase indication SEI messages for AVC and HEVC [T. Ikai, T. Chujoh (Sharp), Y.-K. Wang, J. Xu, W. Jia (Bytedance)]

This was discussed in JVET on Friday July 14.

Neural network post filter (NNPF) SEI messages, the NNPF characteristic SEI and the NNPF activation SEI, and phase indication SEI messages been developed and incorporated into the VVC draft text and VSEI draft text. On the other hand, AVC and HEVC have not defined those SEI messages in the specifications while they have been widely deployed in various applications and those SEI messages are applicable if the specifications have the corresponding payload type numbers and interface definitions using the normative references. It is proposed to support the NNPF and phase indication SEI messages in the AVC and HEVC specifications.

A new edition of H.265 is planned to be submitted for ITU consent, but it would be premature to include by last minute, and the corresponding edition 5 of 23008-2 is already under FDIS ballot.

According to opening remarks, another edition of H.265 could be submitted in April 2024, and the three SEI messages (NNPFC, NNPFA, phase indication) should be enabled in that. It was agreed to implement this by establishing a link to VSEI (v3). Another edition of 23008-2 could be initiated for 2024 once it becomes clear that the 5th ed. would be published in 2023. At this moment, an amendment (CDAM1) on top of the 5th edition could be started, which would contain the colour descriptors from JVET-AD1008, and the proposed text from JVET-AE0101 (possibly some refinements). Decision: This was agreed.

According to opening remarks, another edition of H.264 could be submitted in April 2024. Furthermore, it is planned to start working on the 11th edition of 14496-10, with CD at this meeting. This should include various corrigenda items from JVET-AD1004, the colour descriptors from JVET-AD1008, and the proposed text from JVET-AE0101 (possibly some refinements). This was

Further study should be performed if other SEI messages existing in HEVC and VSEI should also be included in AVC. Decision: This was agreed.

It was noted that inclusion of new SEI messages in HEVC and AVC also requires software implementation in HM and JM, respectively.

JVET-AE0106 AHG9: On missing value ranges for some syntax elements in the NNPFC SEI message [C. Lin, Y.-K. Wang, J. Xu, W. Jia, J. Li, Y. Li, K. Zhang, L. Zhang (Bytedance)] [late]

JVET-AE0113 AHG9: Extendibility and code word length of nnpfc_purpose [R. Sjöberg, M. Pettersson (Ericsson)]

JVET-AE0126 AHG9: NNPF cleanup and editorial changes for VSEI [Y.-K. Wang, W. Jia, J. Xu, C. Lin (Bytedance)]

JVET-AE0127 AHG9: NNPF editorial changes for VVC [Y.-K. Wang, W. Jia, J. Xu (Bytedance)]

JVET-AE0128 AHG9: On NNPFC extensibility and base filter signalling [Y.-K. Wang (Bytedance)]

JVET-AE0134 AHG9: Align the design of NNPF with multiple input pictures to NNPF including picture rate upsampling [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AE0135 AHG9: On NNPF picture rate upsampling constraints [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AE0141 AHG9: Fix a bug in NNPFC SEI message for colourization [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AE0142 AHG9: On derivation of NNPF input pictures and the value of nnpfc_purpose [W. Jia, Y.-K. Wang, J. Xu, L. Zhang (Bytedance)]

JVET-AE0173 [AHG9] Clarification and improvements of signalling of NNPF update [Y. Lim (Samsung)]

JVET-AE0175 [AHG9] Editorial improvements of nnpfc_mode_idc [Y. Lim (Samsung)]

JVET-AE0189 AHG9: On the design of nnpfa_target_base_flag in NNPFA SEI message [Hendry (LGE)] [late]

JVET-AE0279 AHG9: New nnpfc_mode_idc for reuse of NNPF definition for new purpose [Y. Lim (Samsung)] [late]

This was presented in JVET on Friday July 14.

An NNPF can be associated with a single purpose and ID within a particular CLVS regardless of its capability. Sometime it might be efficient and useful to reuse NNPF data with new purpose and ID. This contribution proposes a new nnpfc_mode_idc for referencing an NNPF already defined with a purpose and ID for new purpose and ID without sending the data again.

(This is related to the discussion regarding disallowing changes of purpose, and “purpose” vs. “capability” as per BoG report JVET-AE0172 v2.)

Several experts saw some inconsistency w.r.t. the concept of updating vs. base.

Further study was requested.

## AHG9: SEI messages on topics other than NNPF (13)

Contributions in this area were discussed at 1215–1300 and 1530–1800 on Friday 14 July 2023, and 0900–1225 on Saturday 15 July 2023 (chaired by JRO).

JVET-AE0054 AHG2/AHG9: Editorial improvements of annotated regions SEI message [T. Chujoh, T. Aono, T. Ikai (Sharp)]

This was addressed by the BoG on SEI messages for VSEI v3 and VVC v3 as reported in JVET-AE0272.

This contribution was discussed in the BoG at 16:30 on Wednesday 12 July.

The annotated regions SEI message carries parameters that identify annotated regions using bounding boxes representing the size and location of identified objects and has been introduced in VSEI, HEVC and AVC. After reviewing the current their texts, several editorial issues have been found. They should be improved when revising VSEI, HEVC and AVC. This contribution proposes revised texts for each VSEI, HEVC and AVC.

The following problems exist in the texts of the annotated regions SEI message.

- The “ar_” prefix of several syntax elements is lacking. BoG recommendation (Ed.): Agreed.

- The offset variables for the conformance window are used in the semantics, but they are not needed because the input picture of the SEI message has been cropped.

The BoG disagreed with the interpretation and instead suggested only to clarify the existing specification as follows.

BoG recommendation (Ed.): Add a NOTE saying (approximately) “NOTE – Although some variables used in this subclause are related to picture cropping, the coordinates used in these semantics are relative to the entire decoded picture prior to cropping.”

- The name of the chroma format indicator differs between VSEI and HEVC/AVC.

ChromaArrayType does not exist in VVC. In HEVC and AVC, ChromaArrayType is assigned as follows:

–	If separate_colour_plane_flag is equal to 0, ChromaArrayType is set equal to chroma_format_idc.

–	Otherwise (separate_colour_plane_flag is equal to 1), ChromaArrayType is set equal to 0.

BoG recommendation (Ed. BF): Change draft VSEI to use ChromaFormatIdc instead of ChromaArrayType. If HEVC and/or AVC are changed to refer to VSEI instead of having an in-line definition of the annotated regions SEI message, we need to define that variable in the interface text when using the SEI message.

- In the case of VVC, the offset variables of the conformance window are defined on PPS, but in the case of HEVC/AVC, they are defined on SPS.

In VVC, no action is needed since the interface description does not refer to where the cropping window is specified.

BoG recommendation (Ed. BF): In VSEI, remove the phrase “specified by the active SPS” (since that is not correct for VVC).

- It is reported that many unknown characters are inserted in the test of Word versions of HEVC and AVC. It was pointed out that these are no-width optional line breaks that are being used intentionally, so no action was considered needed on this by the BoG.

JVET-AE0064 AHG8/AHG9: Signalling encoder preprocessing and human / machine viewing indications [C. Kim, D. Gwak, Hendry, J. Lim, S. Kim (LGE), M. M. Hannuksela, F. Cricri, H. Zhang (Nokia)]

This contribution proposes a new SEI message for signalling encoder preprocessing type that is applied to video bitstream and indication whether the optimization is targeting for human viewing and / or machine analysis. The proposed SEI message contains the following information:

- Encoding pre-processing type applied to picture(s) associated with the SEI message.

- Indication whether the pre-processing for the picture(s) is optimized for human viewing.

- Indication whether the pre-processing for the pictures(s) is optimized for machine analysis.

The proposed SEI message is the result of further study on contributions made in previous JVET meetings that include JVET-AD0060 and JVET-AD0137.

It was asked which action a decoder would execute when receiving this SEI message. One example given would be signalling of key pictures optimized for object detection.

Hints on spatial and temporal subsampling would be mainly useful for decoders that would like to conceal something that has been optimized for machine consumption w.r.t. reconstruction for human viewing.

The proponents do not claim that the list of preprocessing steps is preliminary, and would need to be further extended depending on work done in WG 4 and AHG8.

It was clarified that after cancellation another SEI message could follow. In this context, the interrelation between persistence and cancellation should be more clarified.

It was commented that the high-level information “with object based optimization” is not useful. It should be more specifically say what was done, e.g. QP adjustment, blurring, omission of certain parts, etc. It is likely that more information needs to be conveyed for this optimization type.

In terms of syntax, it was commented that defining optimization type as u(16) and then re-defining as flags is inappropriate and could be either right away defined as flags, or by numbers.

Joint meeting with WG 4 to clarify responsibility of defining metadata supporting legacy standards used for the purpose of machine consumption of decoder output.

JVET-AE0090 has similar target but is more a subset in its “option 1”.

This was discussed in a joint meeting with WG4 (see notes in section 7.3).

Decision: Adopt JVET-AE0064 into TuC for VSEI extensions JVET-AC2032.

JVET-AE0066 AHG9: Multiplane Image Information SEI [T. Lu, P. Yin, G. Su, D. Lee, T. Huang, S. McCarthy, W. Husak, G. J. Sullivan (Dolby)]

This contribution proposes a multiplane image information (MPII) SEI message be included in the next version of VSEI. The proposed MPII SEI message is intended to enable virtual reality/immersive experiences on existing consumer devices using existing video codecs such as HEVC and VVC that were developed for conventional two-dimensional video.

It has up to 16 planes and codes an alpha map plus video

Is there any relation with region-wise packing?

It was asked if there is relationship with MIV MPI approach? The following table was shown on this aspect:

It was requested to put this information into an update of the contribution.

Application scope? Mainly consumer

Following comments were made in detail on syntax (YK Wang):

- Is view id useful in single layer bitstream

- Usage of ue(v)

- Usage of cancel flag

- Some details

Otherwise, the editorial quality was considered sufficient.

This was discussed with WG 4 in a joint meeting. As MIV supports various variants (single-view, multi-view, etc.) of MPI, further study in WG 4 was agreed, to identify the possibilities of supporting this very simple variant in an efficient way.

JVET-AE0079 AHG8/AHG9: Source picture timing information SEI message [S. McCarthy, G. J. Sullivan, P. Yin (Dolby)]

This contribution proposes a source picture timing information (SPTI) SEI message for inclusion in the next version of VSEI for use with VVC, HEVC, and AVC. The proposed SPTI SEI message indicates the actual temporal distance between source pictures associated with the corresponding decoded output pictures prior to encoding, e.g., for camera-captured content, the temporal distance between source pictures is the difference between the time at which an image sensor was exposed to produce a source picture associated with the current decoded picture and the time at which the image sensor was exposed to produce the source picture associated with a previous decoded picture in output order. The proposed SPTI SEI message conveys information that supports various post-decoder processes. As examples, the proposed SPTI SEI message could facilitate the following: playback of slow-motion video at actual capture rate; playback of high-speed and time-lapse imaging for human viewing whilst providing data for scientific, industrial, and machine analysis; indicating synthesized pictures and corresponding timing information for frame rate conversion applications; playback of reverse-time video in original causal time; conveyance of data to facilitate machine analysis of actual motion and physical dynamics for applications including high-speed imaging, security, defense, traffic and collision analysis; and conveyance of data to facilitate conventional and AI-based post-decode processing such as frame rate conversion and synthesis of video using generative AI technologies, for example.

This has use cases in machine analysis, but also far beyond.

Could the signalling better be done in enumeration rather than bit mask? Some of the variants are mutually exclusive, but many combinations are possible, such that bit mask is easier semantics-wise.

Further study if some combinations should be prohibited by encoder constraint, or if additional information is needed how to interpret certain combinations.

Related to cancel flag: The absolute time of first picture cannot be signalled, would be necessary to get from systems level.

The proposal resolves the problem with HRD constraint that was found in the previous proposal from Panasonic on the same topic (follow-up of that is JVET-AE0089).

Decision: Adopt to TuC on VSEI extensions.

JVET-AE0080 AHG9: Generative Face Video SEI message [S. McCarthy, P. Yin, G.-M. Su, A. K. Choudhury, W. Husak (Dolby)]

This contribution proposes a generative face video (GFV) SEI message for inclusion in the next version of VSEI for use with VVC. The proposed GFV SEI message conveys information for facial features extracted from source video prior to encoding that can be applied to a previously decoded reference picture to reconstruct a face picture at the current time. The proposed GFV SEI message also specifies a neural network, for example, a generative adversarial network (GAN), that could be used to generate face pictures based on the face feature information conveyed in the GFV SEI message. As noted in previous related JVET contributions JVET-AC0088 and JVET-AD0051, the GFV SEI message is intended to enable face picture reconstruction processes to use VVC-coded pictures as reference pictures. It is also noted that extracting face features on the encoder-side can reduce receiver-side complexity. The present contribution proposes additions and refinements to previously proposed syntax and semantics to address issues noted by experts during previous JVET meetings. The proposed GFV SEI message is intended for ultra low bitrate face video compression applications such as video conferencing, live entertainment, and face animation.

When a receiver ignores the SEI message, the base image would be shown.

It was asked whether this would not rather be something to standardize as a specific standard rather than SEI message. The benefit of the technology is well recognized, very low bit rate for video conferencing.

One expert mentions that this technology has a very limited scope. The proponents say this is intentional.

Is it an advantage to couple this to VVC/HEVC? If somebody is interested only in this functionality, they might not want to implement a full VVC decoder.

Would it not be better to define a profile if the intent is limited scope?

The relationship with VPC is also pointed out, but this is more generic, not only using VVC as base codec. According to last meeting’s report, WG 7 also seems to have some plans about Avatar coding. Unclear if that is based on neural networks or old MPEG-4 technology. Joint meeting?

Usage of other legacy codecs is also important in the opinion of another expert.

Complexity issue? It was pointed out that real-time implementations on smartphone exist, which also fulfills real-time requirements (see https://arxiv.org/abs/2012.00328)

One expert suggested that an exploration experiment might be started.

Interoperability between encoder and decoder could be important in this context – can this be provided by an SEI message? This role could be taken over by application standards who could define a specific encoder/decoder pair.

JVET-AE0083 AHG9: Common SEI Message of Generative Face Video [B. Chen, J. Chen, Y. Ye (Alibaba), S. Wang (CityU)]

At the 29th JVET meeting, JVET-AC0088 proposed a generative face video SEI message to allow VVC-coded pictures to be used as base pictures to drive a deep generative network to generate additional images, therefore enabling ultra low rate face video compression. At the 30th JVET meeting, JVET-AD0051 generalized the generative face video SEI message to include syntax elements that correspond to a variety of facial representations, such as 2D keypoints, 2D landmarks, 3D keypoints, facial semantics and other formats.

Compared with previous contributions, in this contribution the following changes were made

- refine the syntax and semantics of the proposed SEI message

- define the interface between the decoder (which decodes the base pictures and parses the SEI message) and the generative neural network

- provide further information on available software implementations.

The Powerpoint presentation was requested to be provided in an update of the contribution.

The dummy picture is not sent; the decoder uses the previous base picture.

The mechanism to signal the kind of algorithm was unclear.

The “undefined matrix” seemed vague.

It was questioned whether an SEI message is an appropriate mechanism to generate an output video.

A software implementation was available and was demonstrated.

JVET-AE0088 AHG9: A study on Generative Face Video SEI Message [H.-B. Teo, J.-Y Thong, K. Jayashree, C.-S. Lim, K. Abe (Panasonic)]

A generative face video SEI message was presented in JVET-AC0088 at the 29th JVET meeting for compression of talking-face videos at ultra low bitrate, where at the 30th JVET meeting JVET-AD0051 further expanded its scope to encompass multiple facial representations. This contribution demonstrates the effectiveness of 3-D facial landmarks for generating talking-face videos. This document further highlights the framework’s independence of number of landmarks used for generation, and its interoperability of landmark representations across various systems.

JVET-AE0089 AHG9: Alternative Output Timing Hint SEI [H.-B. Teo, J. Gao, C.-S. Lim, K. Abe (Panasonic)]

This contribution is a further follow-up proposal on JVET-AC0141 and JVET-AD0161. This contribution proposes an Alternative Output Timing Hint SEI message for VSEI. In the case if the video bitstream contains slow-motion scenes, this SEI can indicate the actual motion speed of the content at captured time. Two aspects are included. In the first aspect, a time scale factor is introduced. This time scale factor is used to calculate the alternative output time using the DPB output time. In the second aspect, recommended level_idc for every temporal sublayer is added. These recommended level_idc values are to inform the decoder the minimum level that is needed to decode each temporal sublayer using the alternative output time. In the 30th JVET meeting, it was asked how to ensure HRD functions correctly if there is no conformance test of the additional operations enabled by the proposed SEI. The evidence for HRD simulation with time scale factor is provided in this contribution. The proposal suggests to adopt either aspect 1, or aspect 1 and aspect 2 into VSEI specification.

This was similar to JVET-AE0079, and had no need for presentation.

JVET-AE0090 AHG8/AHG9: On machine vision indication [J. Gao, H.-B. Teo, C.-S. Lim, K. Abe (Panasonic)]

This contribution proposes to add an indication for machine vision in VSEI. The Annotated Regions SEI contains a flag to indicate whether the decoded image is not optimized for human vision. This human vision indication flag can’t cover the scenario where the image is not optimized for machine vision. It is proposed to add machine vision indication in VSEI by one of the following options:

- Option 1: Add machine vision indication SEI message in VSEI. The machine vision indication SEI message carries information to identify the possible usage of the decoded image associated with this SEI message.

- Option 2:  Add extension mechanism to Annotated Regions SEI message. The extension data contains ar_extension_flag and ar_not_optimized_for_machine_vision_flag. ar_not_optimized_for_machine_vision_flag indicates whether the decoded image is not optimized for machine vision.

- Option 3: Change ar_not_optimized_for_viewing_flag to ar_not_optimized_for_viewing_idx in the Annotated Regions SEI message. ar_not_optimized_for_viewing_idx identifies the possible usage of the image. The new value of 2 indicates that the decoded image that the Annotated Regions SEI message applies to is not optimized for machine viewing, but rather is optimized for some other purpose such as user viewing.

Option 1 is basically a subset of JVET-AE0064.

Options 2 and 3 should not be used, as the original purpose of annotated regions is different, even though it also can be used for machine consumption purposes.

The annotated regions SEI message has a flag saying it is optimized for machine vision.

JVET-AE0095 AHG8/AHG9: proposed changes to the candidate new object mask information SEI message [P. de Lagrange, E. François, D. Doyen (InterDigital), J. Chen, S. Wang, Y. Ye (Alibaba)] [late]

This contribution proposes an update to the candidate new object mask information (OMI) SEI message proposed in contribution JVET-AD0175. The OMI SEI message is used in conjunction with auxiliary pictures that code object masks. Objects can be detected from the auxiliary pictures thanks to their ID, signalled in the SEI message. One potential issue is when lossy coding is used for those pictures, in which case the sample values inside the mask auxiliary pictures are not guaranteed to perfectly match the object IDs. The proposal aims at addressing this issue, by adding in the SEI a parameter specifying a tolerance range around the object ID values. In addition, bounding box parameters that delimit the masks’ location are also proposed to be included.

The purpose of using an alpha map to reconstruct binary object shapes (rather than “grey-level shapes”) via an auxiliary picture is well recognized and important. An open question is whether the method of defining a threshold in case of lossy coding of the map is more efficient (and practical from the encoder view point) than right away doing lossless coding (e.g. of a picture that just contains 0/1 values). For complicated shapes, it might be difficult to determine at which QP lossy coding needs to be performed, to guarantee that no points of the object are lost, and no points outside of the object are interpreted as object points.

To some extent, perfect lossless coding may not be necessary, and post processing of the lossy decoded shape could also be performed.

Further study was recommended, bringing examples/results with real-world examples rather than hand-crafted examples.

This would have potential applications in machine consumption tasks, but even more for general purposes of segmented video, such as video conferencing.

It was pointed out that V-PCC has some similar concepts of coding the occupancy map, using either lossy or lossless compression. Comparison would be beneficial.

Decision: Adopt JVET-AE0095 into TuC for VSEI extension, without the thresholding.

JVET-AE0156 AHG9: Message wrapping and importance indication for the SEI processing order SEI message [G. J. Sullivan, S. McCarthy, P. Yin (Dolby Labs)]

This contribution proposes four modifications of the draft SEI processing order SEI message for VVC, relative to draft 4 (JVET-AD2027).

- Enabling a “wrapping” capability, controlled by a wrapping flag, so that rather than having the SEI messages always carried in a way that they will be potentially interpreted by decoders that do not understand the SEI processing order SEI message, the encoder can place them where they will only be interpreted in the proper context.

- Adding an importance flag to indicate whether a stage of the sequential processing is needed for proper output of the indicated sequence of SEI message processing operations.

- Adding an explicit indication of the number of SEI messages that are in the indicated sequence, to avoid needing to infer this from the amount of data in the SEI message and to ensure that at least two messages are in the indicated sequence.

- Constraining the order indication values so that SEI messages are not sent in an order that differs from their natural sequential order (and reducing the size of these values to 8 bits).

- Removing the constraint that SEI processing order SEI messages cannot change in a CVS.

Finally, the contribution points out that the SEI processing order message can be useful not only for indicating stages of processing operations, but also for indicating the properties and intended uses of the video content after particular stages of processing (e.g. by sending CLL, ATC, CCV or AVE SEI messages). The consequence of this observation is that the SEI processing order SEI message could be useful for more purposes than may have been previously appreciated.

Examples were explained on why those changes are considered appropriate.

Comments were made about wrapping, as this would hide SEI messages from legacy decoders, as the legacy decoder might be able to process the content. It could hypothetically happen that a decoder knows all SEI messages that are wrapped, and would by itself process them in the right order (e.g., the order they are appearing in the bitstream), but is prevented from doing so by the encoder which has decided to wrap them.

It was suggested that a note could be included that this should only be used if it is expected that legacy decoders would execute processing that would not lead to intended results.

Another comment is made about persistence. Previously it was the entire CVS, which would no longer be clear when the SEI message is changed. Further study on this.

It was commented that reducing the order indication to u(8) instead of u(16) may be too restrictive. It was agreed after discussion that this should be sufficient.

Decision: Adopt JVET-AE0156 (JVET-AE2027). Leave out the change in CVS (for further study), and add a note about the usage of wrapping.

It was commented that this SEI message would be relevant not only for VVC, but also for AVC and HEVC, where new standardization actions were initiated. It was however discussed that it would be premature to include processing order in AVC CD and HEVC CDAM at this meeting, as in the context of discussion about NNPF grouping, it might be further extended for additional functionality in the context of NNPF.

JVET-AE0182 AHG9: SEI message extension of VVC for object-wave compression and computer-generated hologram use [K. Nonaka, R. Koiso, H. Kojima, K. Kawamura, H. Kato (KDDI)]

This contribution proposes an SEI message to be used with VVC for realizing computer-generated holography use. Transmission of object wave, which is the intermidiate data representation of computer-generatad hologram, is said to be suitable for the practical use in terms of high computational complexity on object wave generation and the data flexibility of the object wave. The contribution focuses on how to transmit object wave by using the existing video codec. This document describes the flow of the object wave transmission for computer-generated hologram use and the results of preliminary study of the flow by using the exsisting video codec. The proposed extension is reported to provide the ability to compress the object wave and convert the object wave into computer-generated holograms tailored to the playback device, enabling a wide range of use of computer-generated holograms.

It was asked why the RD curve is saturating at the low end (PSNR staying constant at around 30 db).

The parameters of the SEI message (mapping from float to integer) are individually adapted at the frame level. It was asked whether this will make inter prediction less efficient.

The real and imaginary part are packed in one image. It was asked what happens when a motion vector points from one into the other, and whether layered coding could be better used.

Is there any relation between real and imaginary part, e.g. in terms of motion? E.g., interleaving, or using 3D-HEVC where coherence between left and right view motion is utilized.

What is the market need, e.g. in terms of holographic displays? WG 2 (MPEG requirements) has some investigation on this – seek communication with them. Also there may be some relation with MIV.

Is 8 bit enough?

More information is requested about the properties of object wave data. Could this also be represented as polar coordinates, e.g., amplitude and phase.

Would be interesting to see results for moving holograms, performing inter prediction.

Further study and information was welcome.

JVET-AE0280 AHG9: Common text for proposed generative face video SEI message [B. Chen, J. Chen, Y. Ye (Alibaba), S. Wang (CityU), S. McCarthy, P. Yin, G.-M. Su, A. K. Choudhury, W. Husak (Dolby)] [late]

This contribution proposes common text based on JVET-AE0080 and JVET-AE0083 for a generative face video (GFV) SEI message for inclusion in the next version of VSEI. The proposed GFV SEI message conveys information for facial features extracted from source video prior to encoding that can be applied to a previously decoded reference picture to reconstruct a face picture at the current time. The proposed GFV SEI message is intended for ultra low bitrate face video compression applications such as video conferencing, live entertainment, and face animation.

The application of translating a real person into an avatar does not require precise encoder/decoder match, synthesis models would translate the parametric description into the avatar’s movements. Ultimately, a decoder would need to be able to interpret different combinations of parameters.

The application of “video realistic” generation of video would require precise match of encoder/decoder. The examples shown in the demo are not fully reconstructing all movements such as eye closing (at 2 kbit/s), but could be adjusted with increased number of landmarks, etc.

This was further discussed in a joint meeting with WG2/WG4/WG7/VCEG (see section 7.4). The differences and commonalities with avatar technology and implicit NN technology for generative NNs were discussed, and it was agreed to perform further study and investigate requirements.

## Non-SEI HLS aspects (0)

Section kept as a template for future use.

# Plenary meetings, joint meetings, BoG reports, and liaison communications

## JVET plenaries

No intermediate plenaries were held, as document review and decisions were made in single-track mode at this meeting (with some BoG activity as noted). Further detail on scheduling is recorded in section 2.15.

Information sharing sessions with other WGs and AGs of the MPEG community were held on Monday 17 July 0900–1130, Wednesday 19 July 0900–1030, and Friday 21 July 1400–1600.

Joint meetings involving JVET were held as follows:

- JVET, WG 4, Q6/16 (VCEG) on MPI/MIV & VCM related topics, on Monday 17 July at 1615-1735

- JVET, MPEG WG2 Requirements, MPEG WG4 Video, MPEG WG7 3D Graphics and Gaptics, and Q6/16 (VCEG) on generative face video, on Monday 17 July at 1715-1800

- JVET, MPEG AG 5 Visual Quality Assessment & Q6/16 (VCEG) on verification testing, on Monday 17 July at 1815-2000

- Further detail about these sessions with other groups is provided in the other subsections of this section.

General plenary wrap-up discussions are recorded under sections 8, 9, and 10.

## Information sharing meetings

Information sharing sessions with other WGs and AGs of the MPEG community were held on Monday 17 July 0900–1130, Wednesday 19 July 0900–1030, and Friday 21 July 1400–1600. The status and plans for the work in the MPEG WGs and AGs was reviewed at these information sharing sessions.

## Joint meeting of JVET, WG 4, Q6/16 (VCEG) on MPI/MIV & VCM related topics

This joint session was held on Monday 17 July at 1615-1735. About 150 people were present in the room and about 180 were connected on Zoom (substantially overlapping with those present in the room).

Agenda

- MIV (MPEG immersive video) and MPI (multiplane image) relationship

- VCM

On the MIV and MPI relationship topic, a proposal to support MPI using SEI was submitted to JVET as JVET-AE0066. JVET-AE0066 proposes a multiplane image information (MPII) SEI message for inclusion in the next version of VSEI. The proposed MPI SEI message is intended to enable virtual reality/immersive experiences on existing consumer devices using existing video codecs such as HEVC and VVC that were developed for conventional two-dimensional video.

There was discussion of whether this proposal would provide additional functionality or other improvement relative to what the MIV standard can do and whether implementing MIV rather than the proposed SEI approach would be burdensome.

- It was reported that multiplane image (MPI) and multisphere (MSI) are supported in the “MIV Extended – restricted geometry” profile or sub-profile.

- For MIV video transport, it was reported that all necessary information can be put into a single video bitstream and used with a single decoder instantiation.

- There was discussion of whether MIV supports irregular spacing of depths, and it was reported that it does.

- There was discussion of how many layers are needed for effective MPI usage and whether the number of layers can be restricted sufficiently for an application. It was said that there are “levels” defined for dealing with this.

Further study in MPEG WG4 was suggested to clarify these issues.

For VCM the following issues were suggested to be discussed

- Anchors

- Test sequences, conditions and metrics

- Architecture of a VCM “normative” (pursued in WG4 with a core codec that may differ somewhat from a generic codec) versus a “non-normative” approach (e.g. in the technical report planned to be developed in JVET)

- The role of SEI messages (e.g. JVET-AE0064, also submitted to WG4 as m64518).

Further study of these issues was suggested, with WG4 and JVET AHG activity to take place between meetings.

There was discussion about proposed SEI messages (or modifications of existing SEI messages) that are related to machine consumption of video content. It was suggested that SEI messages developed in JVET should be “generic” rather than specialized to machine analysis.

It was suggested to use a different NAL unit type for important data related to system usage.

JVET planned to include JVET-AE0064 in a “technology under consideration” output document (as candidate technology for a future version of VSEI or similar), which does not include discussion of a specific usage procedure – it is basically sending flags indicating whether the video is optimized for machine consumption or human consumption.

WG4 is expected to develop “messages” (or some form of signalling) and specific processing methods for machine usage of video.

Source picture timing indication was also suggested to be potentially relevant to machine usage of video content (as this information could be useful for machine analysis), but it is general purpose, not targeted specifically for machine analysis.

## Joint JVET, MPEG WG2 Requirements, MPEG WG4 Video, MPEG WG7 3D Graphics and Gaptics, and Q6/16 (VCEG) on generative face video

This joint session was held on Monday 17 July at 1715-1800, continuing in the same room as the joint session described above, with a similar number of people participating.

Several approaches for technology to synthesize moving faces were discussed.

- JVET has received proposals for “generative face video” SEI messages including JVET-AC0088, JVET-AD0051, JVET-AE0080, JVET-AE0083, JVET-AE0088, and JVET-AE0280. The concept is to have an ordinary 2D picture and some data that guides the generation of additional 2D pictures from the basis picture.

- WG7 has been working on “avatar” technology based on 3D graphics technology and parameters extracted from analysis of 2D video content of faces or a full body. This provides parameters that are used for an animated 3D mesh representation. Requirements for avatar technology standardization are under study in WG2.

- WG4 has been developing “implicit neural network” technology, where a neural network is coded as a neural network representation and coded feature data that can generate animated pictures.

Further study is planned, esp. in MPEG Requirements WG2.

## Joint JVET, MPEG AG 5 Visual Quality Assessment & Q6/16 (VCEG) on verification testing

This joint session was held on Monday 17 July at 1815-2000, in a different room than the two sessions described above. About 100 people were present in the room and a similar number were connected on Zoom (substantially overlapping with those present in the room).

This joint session was to discuss verification testing preparations, particularly including test materials for film grain.

JVET-AE0219, reporting results of visual checking of scalable VVC verification testing streams, was discussed. Artefacts related to decoder motion vector refinement (DMVR) were noted for low bit rates, for which a fix called DMVREncMvSelect was available and planned to be used (despite a small loss in average PSNR performance).

JVET-AE0288, reporting results of expert viewing for the spatial scalability category of the VVC multilayer VT, was discussed. It was reported that higher fidelity encoding of a base layer that is upsampled is sometimes visually superior to two-layer coding of 4K content, although the two-layer coding may have better PSNR results.

The problem was reported to be the temporal effect of periodic quality “pumping” in the two-layer coded content. It was commented that single-layer coding can also exhibit “pumping”. Lower-resolution coding would not have such pumping.

It was pointed out in the discussion that upscaling could be done with a neural network, which would further improve the visual quality of the upscaled base layer. Another comment was that neural network post-processing of the multilayer coding could also be used.

It was asked whether chroma phase was being used correctly, which seemed to be the case.

It was commented that using the lowest quality coding in the range may not be appropriate, but the test was designed to span a wide range of quality.

Analysis of the seating positions of the viewers and confidence intervals, and possibly the preferences of individual viewers, was suggested.

It was commented that the amount of bit rate to be allocated to the enhancement layer should perhaps be reduced. However, the enhancement layer was said to already be getting a relatively low percentage of the total bit rate.

It was commented that the content may not have enough real high frequency signal in it.

It was noted that the high resolution content may contain noise that is removed by the downsampling.

Further study of these issues was needed before an effective test can be performed.

Further informal viewing in AG5 and AHG activity was planned.

## BoGs (3)

The following break-out groups were established at this meeting to conduct discussion and develop recommendations on particular subjects.

JVET-AE0272 BoG report on SEI messages for VSEI v3 and VVC v3 [G. J. Sullivan (BoG coordinator)]

This document is a break-out-group report on SEI messages for VSEI v3 and VVC v3. The BoG met from 15:15 to 18:30 on Tuesday 11 July 2023 and from 08:30 to 12:30 and 14:30 to 18:30 on Wednesday 12 July 2023 in Salle 3 of the Geneva CICG facility.

The BoG reviewed the relevant proposals for NNPF SEI messages based primarily on the summary of NNPF-related SEI messages provided in JVET-AE0187-v3, which formed the original basis of the structure of this report.

No specific plans were recommended for topics that were agreed not to be within the scope of VSEI v3 and VVC v3 (except to encourage progress toward future support for VSEI v3 SEI messages for NNPF and filtering phase indication in HEVC and AVC).

The first version of the BoG report was presented in JVET plenary on Thursday July 13 at 0830. The changes made during discussion in the JVET plenary were reflected in version 2 of JVET-AE0272.

Actions recommended by the BoG are recorded in section 6.1 of this report, with a few additional notes in sections 4.2 and 6.2.

All BoG recommendations as per v2 were confirmed by the JVET plenary. For further detail, see section 6.

JVET-AE0282 BoG report on NNVC work planning [E. Alshina, F. Galpin (BoG coordinators)]

The mandate of this BoG was preparing the next EE1 on NNVC and reviewing documents from section 5.1 of the JVET meeting notes relevant to EE1 planning.

The BoG met twice on July, 13 (Thursday) 14:30, and on July, 14 (Friday) 09:00. Up to 81 experts attended the second BoG discussion in-person or remotely.

In total eight  contributions were reviewed on July 13 (JVET-AE0072,  JVET-AE0093, JVET-AE0161, JVET-AE0238, JVET-AE0171, JVET-AE0160, JVET-AE0255, JVET-AE0164).

Five more contributions were reviewed on July 14 (JVET-AE0119, JVET-AE0162 , JVET-AE0230, JVET-AE0194, JVET-AE0281.

The BoG made the following recommendations:

- Adopt JVET-AE0191 to NNVC-6.0 as first generation of unified filter design for high operation point HOP.1 (disabled by default). Keep study of alternative HOP designs as EE1 tests.  No joint training for HOP expected this time. The strongest candidate for joint HOP training to be decided based on EE1 results during JVET-AF meeting.

- Use existing low operation point filter in NNVC-5.0 as first generation of LOP (LOP.1).

- AhG11, EE1 (and non-EE1) proposals which study filter usage aspects (such as residual adjustment, input/output rotation, overfitting with signalling), or additional functionalities (such as considering filter in RDO, extra models dealing with input from different POC) must use HOP.1 or/and LOP.1 as base.

- All EE1 tests which were not conducted due to delay of HOP.1 to be included into next EE1. Test description to be provided to EE1 coordinators before end of the day Monday, July 17.

- Document with training instructions, which allow reproduction of HOP.1, starting from Stage I, or Stage II, or Stage III, with the link to intermediate models needed for reproducing the training, to be provided by proponent of JVET-AE0191 and corresponding text to be included to the next EE1 description (later can be considered for NNVC CTC).

- Conduct AhG11/AhG14/EE1 teleconference on August 2, reviewing HOP.1 after Stage III training, model quantization and test (all-intra and random access cfg) are completed. JVET-AE0191 authors requested to report Stage III results (float and integer 16) before August, 2. The best performing model will be selected during teleconference. Publish test results of HOP.1 in EE1 description as anchor for HOP tests.

- To train unified design for LOP jointly based on design and procedure described in JVET-AE0281: filter architecture design -  combination of JVET-AE0067 (split aspect) and JVET-AE0165 (simplified Back Bone Block); training - JVET-AE0165 (aligned with HOP except number of epochs); filter usage – JVET-AE0238 (aligned with HOP).

- Establish following tests in EE1:

  - EE1-0.x: Unified LOP (JVET-AE0281);

  - EE1-1.1.x category: architectural changes for HOP (JVET-AE0160 , JVET-AE0164), more tests from old EE1 to be added.

  - EE1-1.2.x category: filter usage aspects (JVET-AE0072, JVET-AE0093, JVET-AE0161, JVET-AE0238), more tests from old EE1 to be added.

  - EE1-1.3.x category: additional model for HOP (place holder for “temporal filter”???).

  - EE1-2.x NN-Inter (JVET-AE0112 investigating complexity reduction possibilities).

  - EE1-3.x NN-Intra (JVET-AE0144 with training in cross-check).

- Adopt to NNVC SW MS-SSIM bug fix from JVET-AE0162. Bug affects only Super-Resolution of RPR tests. Suggest this bug fix to VTM, ECM.

The BoG report v1 version was presented in the JVET plenary on Friday 14 July 1430-1530.

All BoG recommendations above were confirmed by JVET.

It was agreed that JVET-AE0144 could be included in NNVC6, provided that the training crosscheck would be successfully completed by the date of the telco meeting (Aug. 9) (conditional adoption)

Decision (SW/BF): Adopt the JVET-AE0162 bug fix for MS-SSIM in case of SR/RPR. This should be implemented in NNVC, ECM, and the VTM.

JVET-AE0294 Film Grain BOG Report [W. Husak (BoG coordinator)]

See section 4.12.

## Liaison communications (1)

JPEG liaison m64537 provided an update on the status of work in the JPEG AI project. The JPEG AI VM2 evaluation reportedly showed 28% and 32% compression efficiency gains over the VVC Intra anchor, for the tools-off and tools-on configurations, respectively, requiring approximately 800 kMAC/pxl at the decoder side. A lightweight model was also adopted targeting mobile devices, reportedly providing 10% compression efficiency gains (tools off) over VVC at 20 kMAC/pxl and 15% gains (tools on) at ~30 kMAC/pxl.

DIS was scheduled for Oct. 2023.

A reply was drafted as WG 5 N 234, and was reviewed in JVET on Wednesday 19 July 1045. The draft reply was also presented in the AG 3 meeting Thursday 20 July at 1000.

# Project planning

## Software timeline

ECM 10 software (including all adoptions) was planned to be available 3 weeks after the meeting.

The NNVC 6.0 codebase software (integrating HOP loop filter with stage 2 optimization plus some bug fixes) was planned to be available 2 weeks after the meeting. An update 6.1 (including verified HOP stage 3 and verified simplified intra prediction) was planned to be available 4 weeks after the meeting.

VTM22.0 software was planned to be available on 2023-09-01. (Note that further updates may be released later)

Updates on top of HM17.0 software were not planned, but might be released after merging pending requests, as appropriate.

## Core experiment and exploration experiment planning

An EE on neural network-based video coding was established, as recorded in output document JVET-AE2023.

An EE on enhanced compression technology beyond VVC capability using techniques other than neural-network technology was also established, as recorded in output document JVET-AE2024.

Initial versions of these documents were presented and approved.

## Drafting of specification text, encoder algorithm descriptions, and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM/VTM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- Normative contributions (relating to changes in bitstream/decoder) shall include draft specification text

- Proposals shall contain all details relevant for understanding and be self-contained. In cases where the document is a follow-up of a previous contribution, the overall concept and the novelties should be highlighted at minimum

- Coding tool and encoder optimization proposals shall contain Excel sheets that allow assessment on a per-sequence basis

- Algorithm description text is strongly encouraged for non-normative contributions that are intended to be included in model description documents (VTM, ECM, etc.), and that is required for inclusion in TR drafts.

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions (nott exercised currently)

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for experiments

It was emphasized that those rules which had been set up or refined during the 12th JVET meeting should be observed. In particular, for some CEs of some previous meetings, results were available late, and some changes in the experimental setup had not been sufficiently discussed on the JVET reflector.

Group coordinated experiments have been planned as follows:

- “Core experiments” (CEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of a draft standard by the next meeting or in the near future.

- “Exploration experiments” (EEs) are also coordinated experiments. These are conducted on technology which is not foreseen to become part of a draft standard in the near future. The investigating methodology for assessment of such technology can also be an important part of an EE. (Further general rules for EEs, as far as deviating from the CE rules below, should be discussed in a future meeting. For the current meeting, procedures as described in the EE description document are deemed to be sufficient.)

- A CE is a test of a specific fully described technology in a specific agreed way. It is not a forum for thinking of new ideas (like an AHG). The CE coordinators are responsible for making sure that the CE description is complete and correct and has adequate detail. Reflector discussions about CE description clarity and other aspects of CE plans are encouraged.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. The experiment description document should provide the names of individual people, not just company names.

- Software for tools investigated in a CE will be provided in one or more separate branches of the software repository. Each CE will have a “fork” of the software, and within the CE there may be multiple branches established by the CE coordinator. The software coordinator will help coordinate the creation of these forks and branches and their naming. All JVET members will have read access to the CE software branches (using shared read-only credentials as described below).

- During the experiment, revisions of the experiment plans can be made, but not substantial changes to the proposed technology. Withdrawing parts of experiments that were intended to show the individual benefits of a tool or parts of a tool is strongly discouraged. Combination tests may not be considered in such cases. Any changes made to individual tools in a combination shall be documented.

- The CE description must match the CE testing that is done. The CE description needs to be revised if there has been some change of plans.

- The CE summary report must describe any changes that were made in the process of finalizing the CE.

- By the next meeting it is expected that at least one independent cross-checker will report a detailed analysis of each proposed feature that has been tested and confirm that the implementation is correct. Commentary on the potential benefits and disadvantages of the proposed technology in cross-checking reports is highly encouraged. Having multiple cross-checking reports is also highly encouraged (especially if the cross-checking involves more than confirmation of correct test results). The reports of cross-checking activities may (and generally should) be integrated into the CE report rather than submitted as separate documents.

- It is mandatory to report encoder optimizations made for the benefit of a tool, and if an equivalent optimization could be applied on the anchor, a comparison against the improved anchor shall be provided.

- A new proposal can be included in a CE based on group decision, regardless if an independent party has already performed a cross-check in the meeting when it was first proposed.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the group test model software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments for SDR video are described in the prior output document JVET-T2010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a proponent perspective – e.g. not referring to methods as “improved”, “optimized”, “enhanced”, etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions contain a basic description of the technology under test, but should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in documents that are referenced in the CE description that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

The CE development workflow is described at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/Core-experiment-development-workflow

CE read access is available using shared accounts: One account exists for MPEG members, which uses the usual MPEG account data. A second account exists for VCEG members with account information available in the TIES informal ftp area (IFA) system at:

https://www.itu.int/ifa/t/2017/sg16/exchange/wp3/q06/vceg_account.txt

Some agreements relating to CE activities were established as follows:

- Only qualified JVET members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting. Participation is requested by contacting the CE coordinator.

- All software, results, and documents produced in the CE should be announced and made available to JVET in a timely manner.

- A JVET CE reflector will be established and announced on the main JVET reflector. Discussion of logistics arrangements, exchange of data, minor refinement of the test plans, and preparation of documents shall be conducted on the JVET CE reflector, with subject lines prefixed by “[CEx: ]”, where “x” is the number of the CE. All substantial communications about a CE other than such details shall take place on main JVET reflector. In the case that large amounts of data are to be distributed, it is recommended to send a link to the data rather than the data itself, or upload the data as an input contribution to the next meeting.

General timeline for CEs

T1= 3 weeks after the JVET meeting: To revise the CE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. Initially assigned description numbers shall not be changed later. If a test is skipped, it is to be marked as “withdrawn”.

T2 = Test model software release + 2 weeks: Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector.

- Initial study by cross-checkers can begin.

- Proponents may continue to modify the software in this branch until T3.

- 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes

T3: 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later: Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. A software version tag should be created at this time. The name of the cross-checkers and list of specific tests for each tool under study in the CE plan description shall be documented in an updated CE description by this time.

T4: Regular document deadline minus 1 week: CE contribution documents including specification text and complete test results shall be uploaded to the JVET document repository (particularly for proposals targeting to be promoted to the draft standard at the next meeting).

The CE summary reports shall be available by the regular contribution deadline. This shall include documentation about crosscheck of software, matching of CE description and confirmation of the appropriateness of the text change, as well as sufficient crosscheck results to create evidence about correctness (crosscheckers must send this information to the CE coordinator at least 3 days ahead of the document deadline). Furthermore, any deviations from the timelines above shall be documented. The numbers used in the summary report shall not be changed relative to the description document.

CE reports may contain additional information about tests of straightforward combinations of the identified technologies. Such supplemental testing needs to be clearly identified in the report if it was not part of the CE plan.

New branches may be created which combine two or more tools included in the CE document or the VTM (as applicable).

It is not necessary to formally name cross-checkers in the initial version of the CE description document. To adopt a proposed feature at the next meeting, JVET would like to see comprehensive cross-checking done, with analysis of whether the description matches the software, and a recommendation of the value of the tool and given tradeoffs.

The establishment of a CE does not indicate that a proposed technology is mature for adoption or that the testing conducted in the CE is fully adequate for assessing the merits of the technology, and a favourable outcome of CE does not indicate a need for adoption of the technology into a standard or test model.

Availability of specification text is important to have a detailed understanding of the technology and also to judge what its impact on the complexity of the specification will be. There must also be sufficient time to study this in detail. CE contributions without sufficiently mature draft specification text in the CE input document should not be considered for adoption.

Lists of participants in CE documents should be pruned to include only the active participants. Read access to software will be available to all members.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JVET reflector (jvet@lists.rwth-aachen.de).

Review of AHG plans was conducted during the plenary on Tuesday 18 July 2023 at 1500, with some refinements on Wednesday 19 July 2023 at 1045.

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Project Management (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate overall JVET interim efforts. - Supervise AHG and experiment studies. - Report on project status to JVET reflector. - Provide a report to the next meeting on project coordination status. - Supervise processing and delivery of output documents | J.-R. Ohm (chair), G. J. Sullivan (vice-chair) | N |
| Draft text and test model algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize draft text outputs of the meeting (JVET-AE1006, JVET-AE1016, JVET-AE2005, JVET-AE2006, and JVET-AE2027. - Collect reports of errata for the VVC, VSEI, HEVC, AVC, CICP, and the published related technical reports and produce the JVET-AE1004 errata output collection. - Coordinate with the test model software development AhG to address issues relating to mismatches between software and text. - Collect and consider errata reports on the texts. | B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| Test model software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of test models (VTM, HM, SCM, SHM, HTM, MFC, MFCD, JM, JSVM, JMVM, 3DV-ATM, 360Lib, and HDRTools) software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Enable software support for recently standardized additional SEI messages. - Discuss and make recommendations on the software development process. - Perform comparative tests of test model behaviour using common test conditions, including HDR, high bit depth and high bit rate. - Suggest configuration files for additional testing of tools. - Investigate how to minimize the number of separate codebases maintained for group reference software. - Coordinate with AHG on Draft text and test model algorithm description editing (AHG2) to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate. - Prepare drafts of merged and updated CTC documents for HM and VTM, as applicable. | F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Consider plans for additional verification testing of VVC capability, particularly target conducting a first test for VVC multi-layer features by the next meeting, and update the test plan according to subsequent tests. - Coordinate with AHG13 on assessing new test material; improve and update the draft test plan for subjective quality testing of the FGC SEI message. - Maintain the video sequence test material database for testing the VVC and HEVC standards and potential future extensions, as well as exploration activities. - Study coding performance and characteristics of available and proposed video test material. - Identify and recommend appropriate test material for testing the VVC standard and potential future extensions, as well as exploration activities. - Identify and characterize missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material, in coordination with other AHGs, as appropriate. - Maintain and update the directory structure for the test sequence repository, as necessary. - Collect information about test sequences that have been made available by other organizations. - Prepare and conduct expert viewing for purposes of subjective quality evaluation. - Coordinate with AG 5 in studying and developing further methods of subjective quality evaluation, e.g. based on crowd sourcing. - Prepare availability of viewing equipment and facilities arrangements for future meetings. | V. Baroncini, T. Suzuki, M. Wien (co-chairs), W. Husak, S. Iwamura, P. de Lagrange, S. Liu, S. Puri, A. Segall, S. Wenger (vice-chairs) | Y (tel., 2 weeks notice) |
| Conformance testing (AHG5) (jvet@lists.rwth-aachen.de) - Produce and finalize the draft of additional conformance bitstreams for VVC multilayer configurations JVET-AE2028, and investigate the need for future improvements of conformance testing specifications. - Study the conformance needs for HEVC multi-view profiles, and develop a set of conformance bitstreams as appropriate. - Study the requirements of VVC, HEVC, and AVC conformance testing to ensure interoperability. - Maintain and update the conformance bitstream database, and contribute to report problems, and suggest actions to resolve these. - Study additional testing methodologies to fulfil the needs for VVC conformance testing. | I. Moccagatta (chair), F. Bossen, K. Kawamura, P. de Lagrange, T. Ikai, S. Iwamura, H.-J. Jhu, S. Paluri, K. Sühring, Y. Yu (vice-chairs) | N |
| ECM software development (AHG6) (jvet@lists.rwth-aachen.de) - Coordinate development of the ECM software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Prepare and deliver ECM-10.0 software version and the reference configuration encodings according to the ECM common test conditions. - Investigate encoder speedup and other software optimization such as reduction of memory consumption. - Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate. | V. Seregin (chair), J. Chen, R. Chernyak, F. Le Léannec, K. Zhang (vice-chairs) | N |
| ECM tool assessment (AHG7) (jvet@lists.rwth-aachen.de) - Investigate methodology of tool assessment. - Coordinate with AHG6 on resolving tool-off test related software issues (missing tool controls and software bugs). - Prepare configuration files and generate bitstreams and results of tool-on/tool-off testing. - Prepare reporting of tool assessment results. - Collect simulation results on non-CTC sequences, and report any issues identified with non-CTC sequences - Develop methodology of more reliable runtime measurement | X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, H.-J. Jhu, X. Li, H. Wang (vice-chairs) | N |
| Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) (jvet@lists.rwth-aachen.de) - Solicit and study non-normative encoder and receiving systems technologies that enhance performance of machine analysis tasks on coded video content. - Identify and collect test materials that are suitable to be used by JVET for machine analysis tasks. - Generate anchors according to the common test conditions JVET-AE2031. - Discuss improvements on the evaluation framework, including evaluation procedures and methodologies. - Coordinate software development, and continue to migrate the software basis used in AHG8 to newest VTM version. - Coordinate experiments on optimization of encoders and receiving systems for machine analysis of coded video content. - Maintain the software implementation example algorithms in the repository, including sufficient documentation in terms of operation and performance. - Evaluate proposed technologies and their suitability for machine analysis applications. - Propose improvements to the draft technical report JVET-AE2030 on optimization of encoders and receiving systems for machine analysis of coded video content. - Study the potentials of using SEI messages for the purpose of machine analysis in coordination with AHG9. - Coordinate with WG 4 VCM AHG on aspects such as common test conditions, evaluation metrics, test and training materials, usage of SEI messages, and on studying characteristics and requirements of targeted machine analysis tasks, etc. | C. Hollmann, S. Liu, S. Wang, M. Zhou (AHG chairs) | Y (tel., 2 weeks notice) |
| SEI message studies (AHG9) (jvet@lists.rwth-aachen.de) - Study the SEI messages in VSEI, VVC, HEVC and AVC. - Discuss the document for technologies under consideration for VSEI JVET-AE2032, and propose improvements as appropriate. - Collect software and showcase information for SEI messages, including encoder and decoder implementations and bitstreams for demonstration and testing. - Identify potential needs for additional SEI messages, including the study of SEI messages defined in HEVC and AVC for potential use in the VVC context. - Study the alignments of the same SEI messages in different standards - Coordinate with AHG8 and WG 4 to study mechanisms for signalling metadata in the context of machine analysis of coded video content. - Coordinate with AHG3 for software support of SEI messages. | S. McCarthy, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, C. Fogg, Hendry, P. de Lagrange, G. J. Sullivan, A. Tourapis, S. Wenger (vice-chairs) | N |
| Encoding algorithm optimization (AHG10) (jvet@lists.rwth-aachen.de) - Study the impact of using techniques such as tool adaptation and configuration, and perceptually optimized adaptive quantization for encoder optimization. - Study the impact of non-normative techniques of preprocessing for the benefit of encoder optimization. - Study encoding techniques of optimization for objective quality metrics and their relationship to subjective quality. - Study optimized encoding for reference picture resampling and scalability modes in VTM. - Study optimized encoding and tool combinations for low latency and low complexity. - Consider neural network-based encoding optimization technologies for video coding standards. - Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures and multi-pass encoding. - Study methods of rate control and rate-distortion optimization and their impact on performance, subjective and objective quality. - Study the potential of defining default or alternate software configuration settings and test conditions optimized for either subjective quality, or higher objective quality, and coordinate such efforts with AHG3 and AHG6. - Study the effect of varying configuration parameters depending on temporal layer, such as those related to deblocking, partitioning, chroma QP. | P. de Lagrange, A. Duenas, R. Sjöberg, A. Tourapis (AHG chairs) | N |
| Neural network-based video coding (AHG11) (jvet@lists.rwth-aachen.de) - Evaluate and quantify the performance improvement potential of NN-based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools and novel architectures. - Discuss potential refinements of the test conditions for NN-based video coding in JVET-AE2016. Generate and distribute anchor encoding, and develop supporting software as needed. - Study the impact of training (including the impact of loss functions) on the performance of candidate technologies, and identify suitable material for testing and training. - Assess the results on 3rd stage of HOP filter training and simplified intra prediction training crosscheck for a possible inclusion in NNVC6 software. - Analyse complexity characteristics, perform complexity analysis, and develop complexity reductions of candidate technology. - Finalize and discuss the EE on neural network-based video coding. - Coordinate with other groups, including SC29/AG5 on the evaluation and assessment of visual quality, and AHG12 on the interaction with ECM coding tools. If possible, prepare encodings with combinations of tools included in the NNVC software for visual quality assessment at the next meeting. - Coordinate with AHG14 on items related to NNVC software development. | E. Alshina, F. Galpin, S. Liu, A. Segall (co-chairs), J. Li, R.-L. Liao, D. Rusanovskyy, T. Shao, M. Wien, P. Wu (vice-chairs) | Y (tel., 2 weeks notice), first on Aug. 9 |
| Enhanced compression beyond VVC capability (AHG12) (jvet@lists.rwth-aachen.de) - Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC. - Discuss and propose refinements to the ECM10 algorithm description JVET-AE2025. - Coordinate with AHG7 to study the performance and complexity tradeoff of these video coding tools. - Coordinate with AHG6 on ECM software development. - Support AHG6 in generating anchors according to the test conditions in JVET-AE2017. - Analyse the results of exploration experiments described in JVET-AE2024 in coordination with the EE coordinators. - Coordinate with AHG11 to study the interaction with neural network-based coding tools. | M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, Y. Yu (vice-chairs) | Y (tel., 2 weeks notice) |
| Film grain technologies (AHG13) (jvet@lists.rwth-aachen.de) - Study the benefits and characteristics of film grain technologies, including autoregressive and frequency-filtering technologies. - Study alternative film grain models and their associated documentation. - In consultation with AHG4, study and define content characteristics and test conditions that are desirable for the study and testing of film grain technologies, and perform an assessment of newly available test materials in that regard. - Given the study of desirable content characteristics, solicit or create new test material for further determining the operational characteristics of, testing, and developing any related technologies. - Study preprocessing and encoder technologies for determining values for FGC (Film Grain Characteristics) SEI message syntax elements. - Identify potential need for additional film grain technology and signalling, if needed. - Coordinate development of film grain technology software and configuration files. - Coordinate with AG 5 on improving the draft plan for subjective quality testing of the FGC SEI message JVET-AD2022, and conduct preparations for such testing. - Coordinate with AHG3 for software support of the FGC SEI message. | W. Husak, P. de Lagrange (co-chairs), A. Duenas, D. Grois, Y. He, X. Meng, M. Radosavljević, A. Segall, A. Tourapis, W. Zhang (vice-chairs) | Y (tel., 2 weeks notice) |
| NNVC software development (AHG14) (jvet@lists.rwth-aachen.de) - Coordinate development of the NNVC software and associated configuration files. - Prepare and deliver NNVC-6.0 software version and the reference configuration encodings according to the NNVC common test conditions as described in JVET-AE2016. - Investigate combinations of tools included in the NNVC software, prepare and release anchor data for all configurations of the software, including anchors for High and Low Operation Point (HOP/LOP) configurations. - Study and maintain the SADL (Small Adhoc Deep-Learning Library). Identify gaps in functionality and develop improvements as needed. - Coordinate with NNVC algorithm and software description (JVET-AE2019) editors to identify any mismatches between software and description document, suggest further updates to the description document as appropriate. - Coordinate with AHG11 on items related to NNVC activities. | F. Galpin (chair), Y. Li, Y. Li, J. Shingala, L. Wang, Z. Xie (vice chairs) | Y (tel., 2 weeks notice), first on Aug. 9 |

It was confirmed that the rules which can be found in document ISO/IEC JTC 1/‌SC 29/‌AG 2 N 046 “Ad hoc group rules for MPEG AGs and WGs” (available at https://www.mpegstandards.org/adhoc/), are consistent with the operation mode of JVET AHGs. It is pointed out that JVET does not maintain separate AHG reflectors, such that any JVET member is implicitly a member of any AHG. This shall be mentioned in the related WG Recommendations. The list above was also issued as a separate WG 5 document (ISO/IEC JTC 1/‌SC 29/‌WG 5 N 235) in order to make it easy to reference.

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production. Where applicable, dates of planned finalization and corresponding parent-body document numbers are also noted.

It was reminded that in cases where the JVET document is also made available as a WG 5 output document, a separate version under the WG 5 document header should be generated. This version should be sent to GJS and JRO for upload.

The list of JVET ad hoc groups was also issued as a WG 5 output document WG 5 N 235, as noted in section 9.

JVET-AE1000 Meeting Report of the 31st JVET Meeting [J.-R. Ohm] [WG 5 N 216] (2023-08-16)

Initial versions of the meeting notes (d0 … d8) were made available on a daily basis during the meeting.

Remains valid – not updated: JVET-AC1001 Guidelines for HM-based software development [K. Sühring, F. Bossen, X. Li (software coordinators)]

Remains valid – not updated: JVET-Y1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 16 [C. Rosewarne (primary editor), K. Sharman, R. Sjöberg, G. J. Sullivan (co-editors)] [WG 5 N 103]

Remains valid – not updated: JVET-AD1003 Coding-independent code points for video signal type identification (Draft 2 of 3rd edition) [WG 5 preliminary FDIS N 206] [G. J. Sullivan, A. Tourapis] (2023-06-30)

The technical content was submitted for ITU consent (but will not be published until ST 2128 is available); ISO FDIS was to be delayed until it is available.

Post-meeting note: Expected de facto primary editor for ITU consent text: G. J. Sullivan.

JVET-AE1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP [Y.-K. Wang, B. Bross, I. Moccagatta, C. Rosewarne, G. J. Sullivan] (2023-09-30, near next meeting)

Post-meeting note: De facto primary editor: Y.-K. Wang.

Errata on post-filter hint SEI message in AVC and HEVC to be added from JVET-AE0155.

Remains valid – not updated: JVET-Z1005 New levels for HEVC (Draft 3) [T. Suzuki, A. Tourapis, Y.-K. Wang]

The content of this document (along with some errata corrections from JVET-AD1004) was included in a new edition of HEVC submitted for ITU consent (and had previously been included in the FDIS submitted as WG 5 N 179 issued from the January 2023 meeting).

Post-meeting note: De facto primary editor for ITU consent text: Y.-K. Wang.

(JVET-Z1005 can be removed after publication of the new edition of ISO/IEC 23008-2.)

JVET-AE1006 New profiles, colour descriptors, and SEI messages for HEVC (draft 1) [WG 5 CDAM N 226] [B. Bross, T. Ikai, G. J. Sullivan, A. Tourapis, Y.-K. Wang] (2023-08-11) [2023-08-11]

Post-meeting note: De facto primary editor for this document and WG 5 N 226: Y.-K. Wang.

NNPF and phase indication SEI messages from JVET-AE0101, colour descriptors from JVET-AD1008, fix for FGC SEI message (see notes under BoG report JVET-AE0294), and new multiview profiles from JVET-AE0296, plus an additional Multiview Monochrome 10 profile.

A request document was issued as WG 5 N 225.

Remains valid – not updated: JCTVC-V1007 SHVC Test Model 11 (SHM 11) Introduction and Encoder Description [G. Barroux, J. Boyce, J. Chen, M. M. Hannuksela, Y. Ye] [WG 11 N 15778]

Remains valid – not updated: JVET-AD1008 Additional colour type identifiers for AVC and HEVC (Draft 4) [G. J. Sullivan, W. Husak, A. Tourapis] [WG 5 Preliminary WD N 200] (2023-06-30)

Remains valid – not updated: JCTVC-AC1009 Common test conditions for SHVC [K. Sühring]

Remains valid – not updated JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan]

JVET-AE1011 HEVC multiview profiles supporting extended bit depth (draft 2) [S. Paluri, W. Husak, A. Tourapis] [2023-08-11]

From JVET-AE0296. The specification of these profiles was also included in JVET-AE1006 and WG 5 N 226. This document, basically duplicating part of JVET-AE1006, was not issued as a separate WG 5 N document. See JVET-AE1006 for editorship note.

Draft 1 had been issued as preliminary WD WG 5 N 143.

(Number 1011 can be re-used when JVET-AE1006 progresses.)

Remains valid – not updated: JVET-AD1012 Overview of IT systems used in JVET [J.-R. Ohm, I. Moccagatta, K. Sühring, M. Wien] (2023-05-19)

Remains valid – not updated: JCT3V-G1003 3D-AVC Test Model 9 [	D. Rusanovskyy, F. C. Chen, L. Zhang, T. Suzuki] [WG 11 N 14239]

Remains valid – not updated: JCT3V-K1003 Test Model 11 of 3D-HEVC and MV-HEVC [Y. Chen, G. Tech, K. Wegner, S. Yea] [WG 11 N 15141]

JVET-AE1013 Common test conditions of 3DV experiments [K. Sühring, M. Wien] [2023-09-01]

New licensing available from JVET-AE0179. Other owners have not responded, therefore we can assume that they don’t have a problem that the sequences are used.

Remains valid – not updated JCTVC-V1014 Screen Content Coding Test Model 7 Encoder Description (SCM 7) [R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye] [WG 11 N 16049]

Remains valid – not updated: JVET-AC1015 Common test conditions for SCM-based screen content coding [K. Sühring]

This requires an update, as the previous version referred to an outdated location of test sequences.

JVET-AE1016 AVC with extensions and corrections (draft 1) [WG5 CD of 11th ed. N 218] [B. Bross, T. Ikai, G. J. Sullivan, A. Tourapis, Y.-K. Wang] [2023-08-11]

Post-meeting note: De facto primary editor and WG 5 N 218: B. Bross.

Target 15th edition of ITU-T H.264 in April 2024.

NNPF and phase indication SEI messages from JVET-AE0101, colour descriptors from JVET-AD1008, fix for FGC SEI message (see notes under BoG report JVET-AE0294 and corrigenda items from JVET-AD1004.

A request document was issued as WG 5 N 217.

No output: JVET-Axx1017 through JVET-Axx1099

Remains valid – not updated JVET-AA1100 Common Test Conditions for HM Video Coding Experiments [K. Sühring, K. Sharman]

This specifies only the CTC for non-4:2:0 colour formats. The corresponding document for VVC is JVET-T2013, with no unification yet.

No output: JVET-Axx2001

Remains valid – not updated: JVET-AD2002 Algorithm description for Versatile Video Coding and Test Model 20 (VTM 20) [A. Browne, Y. Ye, S. Kim] [WG 5 N 204] (2023-07-07, near next meeting)

(Text kept for future use.) It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Remains valid – not updated: JVET-AC2003 Guidelines for VTM-based software development [F. Bossen, X. Li, K. Sühring]

Remains valid – not updated: JVET-T2004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 12) [Y. Ye, J. Boyce]

JVET-AE2005 New level and systems-related supplemental enhancement information for VVC (Draft 6) [B. Bross, E. François, M. M. Hannuksela, A. Tourapis, Y.-K. Wang] (2023-08-18)

This was also delivered for ITU-T Consent as part of H.266 3rd ed.

The content of this delta (amendment style) document was included in a new edition of VVC. A DoCR on ISO/IEC 23090-3/DAM1 was issued as WG 5 N 227 (reviewed Tuesday 18 July 1245), and the FDIS text was issued as WG 5 N 228 with delivery date 2023-09-15.

Post-meeting note: De facto primary editor for this document, the corresponding ITU consent text, and the corresponding FDIS text WG 5 N 228: B. Bross.

See BoG report JVET-AE0272 for SEI elements included.

JVET-AE2006 Additional SEI messages for VSEI (Draft 5) [S. McCarthy, T. Chujoh, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang] (2023-08-18)

Post-meeting note: De facto primary editor for this document, the corresponding ITU consent text, and the corresponding FDIS text WG 5 N 220: Y.-K. Wang.

Text related to the remaining revisit from the BoG report JVET-AE0272 was presented on Tuesday 18 July at 1035. This is related to the issue that for frame interpolation between frames A and B only one activation shall be performed for all frames that are to be interpolated altogether. It was commented that the purpose may be difficult to understand for implementers. It was suggested to add a NOTE explaining the purpose of that constraint.

Decision (Ed./clarification): Adding such a note was agreed.

The technical content of this was also delivered for ITU-T consent as part of draft Rec. H.274 3rd ed.

The content of this delta (amendment style) document was included in a new edition of VSEI. A DoCR on ISO/IEC 23007-7/DAM1 was issued as WG 5 N 219 (reviewed Tuesday 18 July 1255), and the FDIS text was issued as WG 5 N 220 with delivery date 2023-09-15.

See BoG report JVET-AE0272 for elements included. Additionally, this should include the fix for the FGC SEI message (see notes under BoG report JVET-AE0294).

Remains valid – not updated: JVET-AD2007 Guidelines for NNVC software development [F. Galpin, S. Eadie, L. Wang, Z. Xie, Y. Li] (2023-05-26)

Remains valid – not updated: JVET-X2008 Conformance testing for versatile video coding (Draft 7) [J. Boyce, F. Bossen, K. Kawamura, I. Moccagatta, W. Wan]

Remains valid – not updated: JVET-Y2009 Reference software for versatile video coding (Draft 3) [F. Bossen, K. Sühring, X. Li]

Remains valid – not updated JVET-AB2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video [F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring]

Remains valid – not updated: JVET-AC2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video [A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy]

Remains valid – not updated: JVET-U2012 JVET common test conditions and evaluation procedures for 360° video [Y. He, J. Boyce, K. Choi, J.-L. Lin]

Remains valid – not updated: JVET-T2013 VTM common test conditions and software reference configurations for non-4:2:0 colour formats [Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu]

Remains valid – not updated: JVET-Q2014 JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding [T.-C. Ma, A. Nalci, T. Nguyen]

Remains valid – not updated: JVET-Q2015 JVET functionality confirmation test conditions for reference picture resampling [J. Luo, V. Seregin]

JVET-AE2016 Common test conditions and evaluation procedures for neural network-based video coding technology [E. Alshina, R.-L. Liao, S. Liu, A. Segall] (2023-07-28)

This includes some editorial updates.

JVET-AE2017 Common test conditions and evaluation procedures for enhanced compression tool testing [M. Karczewicz, Y. Ye] (2023-08-04)

This is to include a modified Excel sheet, and mention the requirement of documenting memory consumption (see discussion under JVET-AE0180).

Remains valid – not updated: JVET-AA2018 Common test conditions for high bit depth and high bit rate video coding [A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu, Y. Yu]

JVET-AE2019 Description of algorithms and software in neural network-based video coding (NNVC) version 4 [F. Galpin, Y. Li, D. Rusanovskyy, J. Ström, L. Wang] [WG 5 N 231] (2023-09-01)

New elements from notes elsewhere in this report:

- Decision: Adopt JVET-AE0191 and JVET-AE0291 to NNVC6 SW and JVET-AE2019, subject to possible further changes from stage 3 training (post-meeting note: it was later confirmed in an AHG meeting on Aug. 9 that the model from stage 3 training will be included).

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

JVET-AE2020 Film grain synthesis technology for video applications (Draft 5) [D. Grois, Y. He, W. Husak, P. de Lagrange, A. Norkin, M. Radosavljević, A. Tourapis, W. Wan] [WG 5 DTR N 223] (2023-09-08)

It was remarked that we would want the ballot results in time for the January meeting.

A DoCR on ISO/IEC 23007-9/CDTR was issued as WG 5 N 222 (reviewed Tuesday 18 July 1305-1350).

JVET-AE2021 Verification test plan for VVC multilayer coding (update 1) [S. Iwamura, P. de Lagrange, M. Wien] (2023-09-29)

See notes under section 4.5 for updates.

Remains valid – not updated: JVET-AD2022 Draft plan for subjective quality testing of FGC SEI message [P. de Lagrange, W. Husak, M. Radosavljević, M. Wien] (2023-06-16)

As new sequences were not available yet for viewing during the JVET meeting, an update of this document of this document was agreed to be postponed.

JVET-AE2023 Exploration experiment on neural network-based video coding (EE1) [E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang, Z. Xie] [WG 5 N 230] (2023-08-11)

An initial draft of this document was reviewed and approved at 1200-1210 on Tuesday 18 July.

Categories are:

- Unified LOP

- Architectural changes of HOP

- Filter usage aspects of LOP and HOP

- Additional models for HOP (e.g., temporal filter)

- Inter prediction

JVET-AE2024 Exploration experiment on enhanced compression beyond VVC capability (EE2) [V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang] [WG 5 N 232] (2023-08-18)

An initial draft of this document was reviewed and approved at 1210-1240 on Tuesday 18 July.

Categories are:

- Partitioning

- Intra prediction

- Inter prediction

- RPR

- In-loop filters

- Entropy coding

It was requested to add details to test 6.2, which changes are intended for the training script. Also a combination (retraining 6.1a with method from 6.2) was requested.

JVET-AE2025 Algorithm description of Enhanced Compression Model 10 (ECM 10) [M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang] [WG 5 N 233] (2023-09-29)

New elements from notes elsewhere in this report:

- Decision: Adopt JVET-AE0100, test 2.1b.

- Decision: Adopt JVET-AE0169 Test 2.2c, in CTC enabled only for screen content.

- Decision: Adopt JVET-AE0169 Test 2.3b, in CTC enabled only for camera-captured content.

- Decision: Adopt JVET-AE0169 Test 2.4a, in CTC enabled only for screen content.

- Decision: Adopt JVET-AE0159 Test 2.5c (enabled only for screen content).

- Decision: Adopt JVET-AE0094 Test 2.6c (enabled only for screen content).

- Decision: Adopt JVET-AE0043 test 2.7.

- Decision: Adopt JVET-AE 0077 test 2.9.

- Decision: Adopt JVET-AE0084 Test 2.11b. In the combination, both 2.10a and 2.11a should be enabled for camera captured content, also in CTC.

- Decision: Adopt JVET-AE0059 Test 3.1b.

- Decision: Adopt JVET-AE0046 Test 3.2.

- Decision: Adopt JVET-AE0091 Test 3.8.

- Decision: Adopt JVET-AE0125 Test 4.1.

- Decision: Adopt JVET-AE0086 Test 4.2.

- Decision: Adopt JVET-AE0102 Test 4.3.

- Decision: Adopt JVET-AE0151 Test 5.1b.

- Decision: Adopt JVET-AE0139 Test 5.2c.

- Decision: Adopt JVET-AE0097.

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Remains valid – not updated: JVET-AC2026 Conformance testing for VVC operation range extensions (Draft 4) [D. Rusanovskyy, T. Hashimoto, H.-J. Jhu, I. Moccagatta, Y. Yu] (2023-04-14)

This was integrated with v1 (JVET-X2008) and delivered for ITU-T consent as H.266.1 2nd ed.

Post-meeting note: De facto primary editor for the ITU consent text: I. Moccagatta.

JVET-AE2027 SEI processing order SEI message in VVC (draft 5) [S. McCarthy, M. M. Hannuksela, Y.-K. Wang] [WG 5 preliminary WD 5 N 229] (2023-09-01)

Updated from JVET-AE0156.

JVET-AE2028 Additional conformance bitstreams for VVC multilayer configurations [S. Iwamura, P. de Lagrange, I. Moccagatta] (2023-09-01)

This is to include the new bitstream from JVET-AE0111.

Remains valid – not updated: JVET-AB2029 Visual quality comparison of ECM/VTM encoding [V. Baroncini, J.-R. Ohm, M. Wien] [AG 5 N 75]

JVET-AE2030 Optimization of encoders and receiving systems for machine analysis of coded video content (draft 3) [J. Chen, C. Hollmann, S. Liu] [WG 5 N 224)] (2023-09-15)

See notes under JVET-AE0081, JVET-AE0081, and JVET-AE0099.

JVET-AE2031 Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content [S. Liu, C. Hollmann] (2023-08-11)

This is to contain editorial improvements, plus anchors from VTM20 as per JVET-AE0096 as additional anchors in the Excel sheet.

JVET-AE2032 Technologies under consideration for future extensions of VSEI (draft 1) [M. M. Hannuksela, J. Chen, S. Deshpande, S. McCarthy] [WG 5 N 221)] (2023-09-15)

Elements from JVET-AE0061, JVET-AE0298, JVET-AE0064, JVET-AE0079, JVET-AE0095; see detailed notes under these document numbers.

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines (assuming face-to-face meetings):

- Meeting under ITU-T SG16 auspices when it meets (ordinarily starting meetings on the Tuesday or Wednesday of the first week and closing it on the Wednesday of the second week of the SG16 meeting – a total of 8 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/‌SC 29 auspices when its MPEG WGs meet (ordinarily starting meetings on the Friday prior to the main week of such meetings and closing it on the same day as other MPEG WGs – a total of 8 meeting days).

In cases where an exceptionally high workload is expected for a meeting, an earlier starting date may be defined. In cases of online meetings, no sessions should be held on weekend days, such that meetings would typically start two days earlier.

Some specific future meeting plans (to be confirmed) were established as follows:

- During Fri. 13 – Fri. 20 October 2023, 32nd meeting under ISO/IEC JTC 1/‌SC 29 auspices in Hannover, DE,

- During Wed. 17 – Fri. 19 and Mon. 22 – Fri. 26 January 2024, 33rd meeting under ISO/IEC JTC 1/‌SC 29 auspices, to be held as teleconference meeting,

- During Fri. 19 – Fri. 26 April 2024, 34th meeting under ITU-T SG16 auspices in Rennes, FR,

- During Fri. 12 – Fri. 19 July 2024, 35th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Sapporo, JP,

- During October 2024, 36th meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.,

- During January 2025, 37th meeting under ITU-T SG16 auspices, date and location t.b.d.,

- During April 2025, 38th meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.,

- During Thu. 26 June – Fri. 4 July 2025, 39th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Daejeon, KR.

The agreed document deadline for the 32nd JVET meeting was planned to be Friday 6 October 2023.

Marius Preda was thanked for his support in maintaining the document site jvet-experts.org, as well as the document sites of JCT-VC and JCT-3V. Institut Mines-Télécom is thanked for hosting the sites.

Apple was thanked for providing test materials that can be used in assessing and developing technology in the context of film grain synthesis applications.

The following companies for thanked for agreeing (by written or verbal statement, or by abstaining from any disagreement according to recommendation 4.1.2 of the 11th WG 5 meeting) to continued usage of test materials that were originally provided to JCT-3V: GIST, Nagoya University, NICT, Nokia, and Poznan University of Technology.

Simão Campos was thanked for the help in setting up the 31st JVET meeting in Geneva. The following members of ITU staff were thanked for their dedication and continuous help in the technical setup of meeting facilities: Gent Bajrami, Marc Antoine Zanou, Ilia Londo, and Paul Marko.

The 31st JVET meeting was closed at approximately 1215 hours CEST (UTC+2) on Wednesday 19 July 2023.

# Annex A to JVET report:
List of documents

(Dates and times in the table below are in Paris/Geneva time.)

| JVET-VC number | MPEG number | Created | First upload | Last upload | Title | Authors |
| --- | --- | --- | --- | --- | --- | --- |
| JVET-AE0001 | m63840 | 2023-07-04 17:19:10 | 2023-07-10 20:48:24 | 2023-07-10 20:48:24 | JVET AHG report: Project Management (AHG1) | J.-R. Ohm (chair), G. J. Sullivan (vice-chair) |
| JVET-AE0002 | m63841 | 2023-07-04 17:21:18 | 2023-07-11 06:29:33 | 2023-07-11 06:29:33 | JVET AHG report: Draft text and test model algorithm description editing (AHG2) | B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice-chairs) |
| JVET-AE0003 | m63842 | 2023-07-04 17:24:59 | 2023-07-11 09:36:33 | 2023-07-11 09:36:33 | JVET AHG report: Test model software development (AHG3) | F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice chairs) |
| JVET-AE0004 | m63843 | 2023-07-04 17:26:52 | 2023-07-10 09:52:06 | 2023-07-11 11:19:59 | JVET AHG report: Test material and visual assessment (AHG4) | V. Baroncini, T. Suzuki, M. Wien (co-chairs), S. Iwamura, S. Liu, S. Puri, A. Segall, P. Topiwala, S. Wenger, J. Xu, Y. Ye (vice-chairs) |
| JVET-AE0005 | m63844 | 2023-07-04 17:28:46 | 2023-07-10 17:17:45 | 2023-07-10 17:17:45 | JVET AHG report: Conformance testing (AHG5) | D. Rusanovskyy, I. Moccagatta (co-chairs), F. Bossen, K. Kawamura, T. Ikai, S. Iwamura, H.-J. Jhu, K. Sühring, Y. Yu (vice-chairs) |
| JVET-AE0006 | m63667 | 2023-06-06 02:56:13 | 2023-06-06 02:56:41 | 2023-07-10 19:05:24 | JVET AHG report: ECM software development (AHG6) | V. Seregin (chair), J. Chen, F. Le Léannec, K. Zhang (vice-chairs) |
| JVET-AE0007 | m63845 | 2023-07-04 17:31:46 | 2023-07-10 15:37:46 | 2023-07-10 15:37:46 | JVET AHG report: ECM tool assessment (AHG7) | X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, H.-J. Jhu, X. Li, H. Wang (vice-chairs) |
| JVET-AE0008 | m63847 | 2023-07-04 17:33:39 | 2023-07-10 07:39:24 | 2023-07-11 09:54:21 | JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) | C. Hollmann, S. Liu, S. Wang, M. Zhou (AHG chairs) |
| JVET-AE0009 | m63849 | 2023-07-04 17:35:03 | 2023-07-11 08:24:39 | 2023-07-11 14:51:15 | JVET AHG report: SEI message studies (AHG9) | S. McCarthy, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, C. Fogg, Hendry, P. de Lagrange, G. J. Sullivan, A. Tourapis, S. Wenger (vice-chairs) |
| JVET-AE0010 | m63852 | 2023-07-04 17:36:53 | 2023-07-11 08:02:47 | 2023-07-16 16:27:21 | JVET AHG report: Encoding algorithm optimization (AHG10) | P. de Lagrange, A. Duenas, R. Sjöberg, A. Tourapis (AHG chairs) |
| JVET-AE0011 | m63853 | 2023-07-04 17:39:22 | 2023-07-10 21:37:14 | 2023-07-11 08:11:06 | JVET AHG report: Neural network-based video coding (AHG11) | E. Alshina, S. Liu, A. Segall (co chairs), F. Galpin, J. Li, R.-L. Liao, D. Rusanovskyy, T. Shao, M. Wien, P. Wu (vice chairs) |
| JVET-AE0012 | m63854 | 2023-07-04 17:40:52 | 2023-07-09 03:23:16 | 2023-07-09 03:23:16 | JVET AHG report: Enhanced compression beyond VVC capability (AHG12) | M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, H. Yang (vice-chairs) |
| JVET-AE0013 | m63856 | 2023-07-04 17:42:28 | 2023-07-11 14:31:36 | 2023-07-15 10:46:01 | JVET AHG report: Film grain technologies (AHG13) | W. Husak, M. Radosavljević (co-chairs), A. Duenas, D. Grois, Y. He, P. de Lagrange, X. Meng, A. Segall, A. Tourapis, W. Zhang (vice-chairs) |
| JVET-AE0014 | m63857 | 2023-07-04 17:44:39 | 2023-07-07 14:46:32 | 2023-07-19 16:21:00 | JVET AHG report: NNVC software development (AHG14) | S. Eadie, F. Galpin, Y. Li, J. Shingala, L. Wang, Z. Xie (AHG chairs) |
| JVET-AE0023 | m63858 | 2023-07-04 17:47:53 | 2023-07-09 22:08:30 | 2023-07-11 22:37:50 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang, Z. Xie |
| JVET-AE0024 | m63859 | 2023-07-04 17:49:10 | 2023-07-11 10:54:51 | 2023-07-16 19:46:52 | EE2: Summary report of exploration experiment on enhanced compression beyond VVC capability | V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang |
| JVET-AE0041 | m63654 | 2023-05-28 22:37:10 | 2023-05-28 22:43:21 | 2023-05-28 22:56:04 | AHG4: Report on AhG meeting on verification testing for VVC multilayer coding | M. Wien (AhG Co-Chair) |
| JVET-AE0042 | m63682 | 2023-06-14 17:06:25 | 2023-06-14 19:28:17 | 2023-06-28 20:27:02 | AhG14 & AHG11: Report on AhG teleconference on high operation point (HOP) unified filter training | E. Alshina, F. Galpin |
| JVET-AE0043 | m63713 | 2023-06-30 03:36:01 | 2023-06-30 11:27:23 | 2023-07-11 10:34:49 | EE2-2.7: Cross-component merge mode with temporal candidates | H.-Y. Tseng, C.-M. Tsai, C.-Y. Chuang, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, O. Chubach, Y.-W. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) |
| JVET-AE0044 | m63730 | 2023-06-30 10:03:50 | 2023-07-04 17:59:15 | 2023-07-12 09:54:30 | AHG12: Dynamic Scaling of Bilateral Filter (BIF) | V. Shchukin, P. Wennersten, J. Ström (Ericsson) |
| JVET-AE0045 | m63742 | 2023-07-02 19:38:51 |  |  | Withdrawn |  |
| JVET-AE0046 | m63743 | 2023-07-03 08:13:51 | 2023-07-04 15:34:37 | 2023-07-09 21:07:32 | EE2-3.2: Bi-predictive GPM | R. Yu, P. Wennersten, J. Enhorn, K. Andersson (Ericsson) |
| JVET-AE0047 | m63744 | 2023-07-03 08:14:33 | 2023-07-04 17:13:31 | 2023-07-12 11:01:55 | AHG12: On GPM-MMVD | R. Yu (Ericsson) |
| JVET-AE0048 | m63745 | 2023-07-03 08:47:53 | 2023-07-04 18:56:29 | 2023-07-05 06:08:43 | AHG9: Miscellaneous VSEI changes on neural-network post-filter SEI messages | M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia) |
| JVET-AE0049 | m63746 | 2023-07-03 08:49:01 | 2023-07-04 18:57:04 | 2023-07-05 06:09:16 | AHG9: Miscellaneous VVC changes on neural-network post-filter SEI messages | M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia) |
| JVET-AE0050 | m63747 | 2023-07-03 08:54:22 | 2023-07-04 18:57:58 | 2023-07-05 06:09:48 | AHG9: On NNPF input picture selection | M. M. Hannuksela, F. Cricri (Nokia) |
| JVET-AE0051 | m63748 | 2023-07-03 08:55:33 | 2023-07-04 18:58:38 | 2023-07-05 06:10:21 | AHG9: On persistent NNPF activation | M. M. Hannuksela, F. Cricri (Nokia) |
| JVET-AE0052 | m63749 | 2023-07-03 08:57:34 | 2023-07-05 20:29:02 | 2023-07-05 20:29:02 | AHG9: NNPF cascades and alternatives | M. M. Hannuksela, F. Cricri (Nokia) |
| JVET-AE0053 | m63751 | 2023-07-03 11:38:59 | 2023-07-04 11:02:16 | 2023-07-04 12:49:18 | AHG8/AHG9: Neural-network post-filter regions SEI message | T. Chujoh, Y. Yasugi, T. Ikai (Sharp) |
| JVET-AE0054 | m63752 | 2023-07-03 11:39:39 | 2023-07-04 11:02:42 | 2023-07-04 11:02:42 | AHG2/AHG9: Editorial improvements of annotated regions SEI message | T. Chujoh, T. Aono, T. Ikai (Sharp) |
| JVET-AE0055 | m63753 | 2023-07-03 11:45:42 | 2023-07-03 14:36:10 | 2023-07-15 12:39:26 | AHG12: Sign prediction parameter configuration for low-delay conditions | Y. Yasugi, T. Ikai (Sharp) |
| JVET-AE0056 | m63754 | 2023-07-03 11:52:46 | 2023-07-03 12:02:07 | 2023-07-04 04:47:02 | Non-EE2: Changes on TIMD and SGPM for reducing ECM decoder complexity | Z. Fan, Y. Yasugi, T. Ikai (Sharp) |
| JVET-AE0057 | m63755 | 2023-07-03 12:04:59 | 2023-07-04 19:27:40 | 2023-07-15 12:28:28 | AHG10/AHG12: MTT split modes early termination | W. Ahmad, P. Wennersten, K. Andersson (Ericsson) |
| JVET-AE0058 | m63756 | 2023-07-03 12:23:11 | 2023-07-04 15:50:51 | 2023-07-12 09:31:36 | AHG12: Spatial CABAC tuning | J. Lainema, A. Aminlou, P. Astola, R. G. Youvalari, D. Bugdayci Sansli (Nokia) |
| JVET-AE0059 | m63761 | 2023-07-03 14:46:54 | 2023-07-04 13:04:35 | 2023-07-11 22:51:32 | EE2-3.1: Cross-component residual model (CCRM) for inter prediction | P. Astola, J. Lainema (Nokia) |
| JVET-AE0060 | m63762 | 2023-07-03 18:39:01 | 2023-07-04 22:33:23 | 2023-07-04 22:33:23 | [AHG9] Comments on NNPFC | S. Deshpande (Sharp) |
| JVET-AE0061 | m63763 | 2023-07-03 18:40:22 | 2023-07-04 22:34:13 | 2023-07-04 22:34:13 | [AHG9] On NNPFC Application Purpose | S. Deshpande (Sharp) |
| JVET-AE0062 | m63764 | 2023-07-03 18:42:21 | 2023-07-04 22:39:19 | 2023-07-04 22:39:19 | [AHG9] On NNPF for Deinterlacing | A. Sidiya, S. Deshpande (Sharp) |
| JVET-AE0063 | m63765 | 2023-07-03 20:02:55 | 2023-07-04 19:32:07 | 2023-07-04 19:32:07 | [AHG9] On grouping and operations for multiple NNPFs | L. Chen, O. Chubach, Y.-W. Huang, S. Lei (MediaTek) |
| JVET-AE0064 | m63766 | 2023-07-03 20:37:08 | 2023-07-03 20:41:24 | 2023-07-03 20:41:24 | AHG8/AHG9: Signalling encoder preprocessing and human / machine viewing indications | C. Kim, D. Gwak, Hendry, J. Lim, S. Kim (LGE), M. M. Hannuksela, F. Cricri, H. Zhang (Nokia) |
| JVET-AE0065 | m63767 | 2023-07-03 20:39:01 | 2023-07-03 20:52:03 | 2023-07-03 20:52:03 | EE2-3.5: Iterative BDOF pass in multi-pass DMVR | M. Salehifar, Y. He, K. Zhang, H. Liu, L. Zhang (Bytedance), J. Chen, R.-L. Liao, X. Li, Y. Ye (Alibaba) |
| JVET-AE0066 | m63768 | 2023-07-03 21:35:20 | 2023-07-04 10:05:37 | 2023-07-15 12:05:47 | AHG9: Multiplane Image Information SEI | T. Lu, P. Yin, G. Su, D. Lee, T. Huang, S. McCarthy, W. Husak, G. J. Sullivan (Dolby) |
| JVET-AE0067 | m63769 | 2023-07-03 21:44:09 | 2023-07-04 23:37:13 | 2023-07-13 08:58:25 | EE1-4.1: Neural-network loop filters with further complexity reduction | J. N. Shingala, A. Shyam, A. Suneja, S. Badya (Ittiam), T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy (Dolby) |
| JVET-AE0068 | m63770 | 2023-07-03 22:09:52 | 2023-07-03 22:12:57 | 2023-07-03 22:12:57 | AHG9: On extensibility of purpose syntax element in NNPFC SEI message | Hendry, J. Nam, S. Kim, J. Lim (LGE) |
| JVET-AE0069 | m63771 | 2023-07-03 22:14:36 | 2023-07-03 22:17:29 | 2023-07-03 22:17:29 | AHG9: On the signalling of output pictures in NNPFA SEI message | Hendry, J. Nam, S. Kim, J. Lim (LGE) |
| JVET-AE0070 | m63772 | 2023-07-03 22:18:07 | 2023-07-03 22:21:19 | 2023-07-03 22:21:19 | AHG9: On input pictures that are not present in the bitstream for NNPF SEI messages | Hendry, J. Nam, S. Kim, J. Lim (LGE) |
| JVET-AE0071 | m63773 | 2023-07-03 22:28:46 | 2023-07-04 20:47:44 | 2023-07-12 10:26:18 | AHG12: DIMD Merge | S. Blasi, I. Zupancic, J. Lainema (Nokia) |
| JVET-AE0072 | m63774 | 2023-07-03 22:50:00 | 2023-07-04 08:17:56 | 2023-07-12 11:13:11 | [AHG11] On residual adjustments for NNLF | Z. Dai, Y. Yu, H. Yu, D. Wang (OPPO) |
| JVET-AE0073 | m63775 | 2023-07-03 22:51:31 | 2023-07-04 06:39:12 | 2023-07-16 12:22:45 | Non-EE2: IBC-LIC Model Merge mode | L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO) |
| JVET-AE0074 | m63776 | 2023-07-03 22:52:20 | 2023-07-04 06:41:10 | 2023-07-13 17:42:57 | Non-EE2: Improvement on cross-component prediction merge mode | H. Huang, Y. Yu, H. Yu, D. Wang (OPPO) |
| JVET-AE0075 | m63777 | 2023-07-03 22:53:21 | 2023-07-04 07:54:18 | 2023-07-12 10:14:11 | Non-EE2: On IntraTMP block vector | Y. Yu, L. Zhang, L. Xu, H. Yu, J. Gan, F. Wang, Z. Xie, D. Wang (OPPO) |
| JVET-AE0076 | m63778 | 2023-07-03 22:54:18 | 2023-07-04 03:08:17 | 2023-07-11 14:34:57 | EE2-2.8: An extrapolation filter-based intra prediction mode | L. Xu, Y. Yu, H. Yu, D. Wang (OPPO) |
| JVET-AE0077 | m63779 | 2023-07-03 22:54:44 | 2023-07-04 02:41:34 | 2023-07-04 02:41:34 | EE2-2.9: Extended search areas for IntraTMP mode | Y. Yu, L. Zhang, F. Wang, J. Gan, H. Yu, L. Xu, Z. Xie, D. Wang (OPPO), Y. Ma, H. Zhang, J. Huo, F. Yang, X. Gao, N. Wang (Xidian University), X. Xiu, N. Yan, C. Ma, H. Jhu, C. Kuo, W. Chen, X. Wang (Kwai) |
| JVET-AE0078 | m63780 | 2023-07-03 22:55:15 | 2023-07-04 07:47:25 | 2023-07-16 13:19:35 | EE2-2.10: IBC-LIC extension | Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO) |
| JVET-AE0079 | m63781 | 2023-07-04 01:53:13 | 2023-07-04 20:54:41 | 2023-07-04 20:54:41 | AHG8/AHG9: Source picture timing information SEI message | S. McCarthy, G. J. Sullivan, P. Yin (Dolby) |
| JVET-AE0080 | m63782 | 2023-07-04 01:59:55 | 2023-07-04 20:55:11 | 2023-07-04 20:55:11 | AHG9: Generative Face Video SEI message | S. McCarthy, P. Yin, G.-M. Su, A. K. Choudhury, W. Husak (Dolby) |
| JVET-AE0081 | m63783 | 2023-07-04 04:07:02 | 2023-07-04 04:24:24 | 2023-07-15 15:39:48 | [AHG8] De-noising filter as pre-processing for machine task | C. Kim, D. Gwak, J. Lim (LGE) |
| JVET-AE0082 | m63784 | 2023-07-04 04:20:24 | 2023-07-04 04:26:20 | 2023-07-14 16:33:22 | AHG12: Non-square quadtree partitioning | Y. Ahn, J. Nam, N. Park, J. Lim, S. Kim (LGE) |
| JVET-AE0083 | m63785 | 2023-07-04 06:34:16 | 2023-07-04 18:01:04 | 2023-07-04 18:01:04 | AHG9: Common SEI Message of Generative Face Video | B. Chen, J. Chen, Y. Ye (Alibaba), S. Wang (CityUniv HK) |
| JVET-AE0084 | m63786 | 2023-07-04 06:43:08 | 2023-07-04 13:50:18 | 2023-07-16 11:30:30 | EE2-2.11: Harmonization of IBC HMVP and IBC-LIC | N. Zhang, K. Zhang, L. Zhang (Bytedance), Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO) |
| JVET-AE0085 | m63787 | 2023-07-04 06:48:23 | 2023-07-04 13:33:56 | 2023-07-12 11:22:03 | Non-EE2: Direct block vector (DBV) mode extension | M. Hong, J. Choi, N. Park, J. Lim, S. Kim (LGE) |
| JVET-AE0086 | m63788 | 2023-07-04 07:33:33 | 2023-07-04 08:03:03 | 2023-07-04 08:03:03 | EE2-4.2: Large NSPT | M. Koo, J. Zhao, J. Lim, S. Kim (LGE) |
| JVET-AE0087 | m63789 | 2023-07-04 07:40:17 | 2023-07-05 03:38:41 | 2023-07-08 03:26:27 | EE2-3.3: High-Accuracy template matching | Y. Wang, K. Zhang, L. Zhang (Bytedance) |
| JVET-AE0088 | m63790 | 2023-07-04 08:08:49 | 2023-07-04 19:16:21 | 2023-07-13 23:50:30 | AHG9: A study on Generative Face Video SEI Message | H.-B. Teo, J.-Y Thong, K. Jayashree, C.-S. Lim, K. Abe (Panasonic) |
| JVET-AE0089 | m63791 | 2023-07-04 08:11:00 | 2023-07-04 08:35:14 | 2023-07-04 08:35:14 | AHG9: Alternative Output Timing Hint SEI | H.-B. Teo, J. Gao, C.-S. Lim, K. Abe (Panasonic) |
| JVET-AE0090 | m63792 | 2023-07-04 08:14:57 | 2023-07-04 19:06:12 | 2023-07-13 23:45:08 | AHG8/AHG9: On machine vision indication | J. Gao, H.-B. Teo, C.-S. Lim, K. Abe (Panasonic) |
| JVET-AE0091 | m63793 | 2023-07-04 09:05:33 | 2023-07-05 03:39:48 | 2023-07-08 03:27:44 | EE2-3.8: Combination of Test 3.3e and Test 3.5 | M. Salehifar, Y. Wang, Y. He, K. Zhang, H. Liu, L. Zhang (Bytedance), J. Chen, R.-L. Liao, X. Li, Y. Ye (Alibaba) |
| JVET-AE0092 | m63797 | 2023-07-04 09:57:25 | 2023-07-04 13:34:47 | 2023-07-15 17:18:55 | [AHG4] Occupancy-only PSNR calculations for V3C V-PCC coding evaluation | S. Schwarz, M. M. Hannuksela (Nokia) |
| JVET-AE0093 | m63798 | 2023-07-04 10:24:00 | 2023-07-04 16:19:42 | 2023-07-12 18:34:04 | AHG11: Content-adaptive neural network loop-filter | R. Yang, M. Santamaria, F. Cricri, H. Zhang, J. Lainema, R. G. Youvalari, M. M. Hannuksela (Nokia) |
| JVET-AE0094 | m63800 | 2023-07-04 11:14:58 | 2023-07-04 11:24:06 | 2023-07-12 07:48:38 | EE2-2.6: IBC with non-adjacent spatial candidates | C. Ma, X. Xiu, W. Chen, H.-H.Jhu, C.-W. Kuo, N. Yan, X. Wang (Kwai), Y. Wang, K. Zhang, L. Zhang (Bytedance) |
| JVET-AE0095 | m63801 | 2023-07-04 11:17:14 | 2023-07-05 15:15:58 | 2023-07-13 15:09:39 | AHG8/AHG9: Proposed changes to the candidate new object mask information SEI message | P. de Lagrange, E. François, D. Doyen (InterDigital), J. Chen, S. Wang, Y. Ye (Alibaba) |
| JVET-AE0096 | m63802 | 2023-07-04 11:18:00 | 2023-07-04 11:34:02 | 2023-07-04 11:34:02 | [AHG8] Study on using different VTM versions | C. Hollmann, J. Ström (Ericsson) |
| JVET-AE0097 | m63803 | 2023-07-04 11:41:53 | 2023-07-04 12:04:10 | 2023-07-12 20:24:17 | AHG12: On the cross-component merge mode | H. Huang, V. Seregin, M. Karczewicz (Qualcomm) |
| JVET-AE0098 | m63804 | 2023-07-04 11:44:00 | 2023-07-04 12:05:54 | 2023-07-16 17:27:02 | AHG12: On the chroma DBV mode | H. Huang, H. Wang, V. Seregin, M. Karczewicz (Qualcomm) |
| JVET-AE0099 | m63805 | 2023-07-04 12:01:31 | 2023-07-04 12:10:39 | 2023-07-04 12:10:39 | [AHG8] NNPF and post-filter hint SEI messages for the technical report | C. Hollmann, M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson) |
| JVET-AE0100 | m63806 | 2023-07-04 12:02:24 | 2023-07-04 15:32:17 | 2023-07-06 12:20:59 | EE2-2.1: Block vector guided CCCM | R. G. Youvalari, D. Bugdayci Sansli, P. Astola, J. Lainema (Nokia) |
| JVET-AE0101 | m63807 | 2023-07-04 12:03:31 | 2023-07-04 12:20:42 | 2023-07-04 12:20:42 | [AHG2][AHG9] Neural network post filter and phase indication SEI messages for AVC and HEVC | T. Ikai, T. Chujoh (Sharp), Y.-K. Wang, J. Xu, W. Jia (Bytedance) |
| JVET-AE0102 | m63808 | 2023-07-04 12:11:45 | 2023-07-04 23:57:54 | 2023-07-11 06:55:31 | EE2-4.3: Context modelling for transform coefficients for LFNST/NSPT | P. Nikitin, M. Coban, C. S. Coban, B. Ray, V. Seregin, M. Karczewicz (Qualcomm) |
| JVET-AE0103 | m63809 | 2023-07-04 12:31:35 | 2023-07-04 13:38:55 | 2023-07-14 17:31:30 | Weighted Edge Enhancement Filtering for Picture Upscaling and RPR | T. Claßen, M. Wien (RWTH Aachen University) |
| JVET-AE0104 | m63810 | 2023-07-04 12:57:05 | 2023-07-04 13:53:26 | 2023-07-15 12:55:01 | AHG10: GOP-based RPR encoder control using parallel resolution encoding | D. Arai, S. Nemoto, S. Iwamura, A. Ichigaya (NHK) |
| JVET-AE0105 | m63811 | 2023-07-04 13:26:09 | 2023-07-05 03:42:23 | 2023-07-13 08:11:19 | Non-EE2: Local illumination compensation with multiple templates | Y. Wang, K. Zhang, Y. He, H. Liu, L. Zhang (Bytedance) |
| JVET-AE0106 | m63812 | 2023-07-04 13:29:47 | 2023-07-07 08:47:01 | 2023-07-07 08:47:01 | AHG9: On missing value ranges for some syntax elements in the NNPFC SEI message | C. Lin, Y.-K. Wang, J. Xu, W. Jia, J. Li, Y. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AE0107 | m63813 | 2023-07-04 13:32:30 | 2023-07-04 13:42:40 | 2023-07-09 15:36:19 | AHG8: Improvements of the BD-rate model using monotonic curve-fitting method | H. Wang, X. Pan, Z. Liu, X. Xu, S. Liu (Tencent) |
| JVET-AE0108 | m63815 | 2023-07-04 13:37:05 | 2023-07-04 17:03:31 | 2023-07-04 17:03:31 | AHG12: DMVR with robust MV derivation | K. Andersson, R. Yu (Ericsson) |
| JVET-AE0109 | m63816 | 2023-07-04 14:00:06 | 2023-07-04 15:02:24 | 2023-07-12 08:38:19 | Non-EE2: LIC flag derivation of merge candidates with template costs | N. Zhang, K. Zhang, H. Liu, Y. Wang, L. Zhang (Bytedance) |
| JVET-AE0110 | m63817 | 2023-07-04 14:01:28 | 2023-07-04 16:14:38 | 2023-07-11 14:55:57 | Non-EE2: An improved method for IntraTMP fusion | S. Peng, D. Jiang, J. Lin, C. Zheng, K. Fu, P. Zhang (Dahua) |
| JVET-AE0111 | m63818 | 2023-07-04 14:02:28 | 2023-07-04 14:28:58 | 2023-07-08 09:43:00 | Additional conformance tests of spatial scalability for multilayer coding | C. Salmon-Legagneur, P. de Lagrange, F. Urban (InterDigital) |
| JVET-AE0112 | m63823 | 2023-07-04 14:47:59 | 2023-07-04 15:13:16 | 2023-07-11 01:05:25 | EE1-5.1: Deep Reference Frame Generation for Inter Prediction Enhancement | W. Bao, W. Meng, J. Jia, Y. Zhang, H. Wang, Z. Chen (Wuhan Univ.), Z. Liu, X. Xu, S. Liu (Tencent) |
| JVET-AE0113 | m63825 | 2023-07-04 14:57:55 | 2023-07-04 15:02:00 | 2023-07-04 15:02:00 | AHG9: Extendibility and code word length of nnpfc_purpose | R. Sjöberg, M. Pettersson (Ericsson) |
| JVET-AE0114 | m63826 | 2023-07-04 15:36:21 | 2023-07-04 17:43:21 | 2023-07-15 12:29:00 | AHG10: Low-Delay configuration improvements | J. Enhorn, P. Wennersten (Ericsson) |
| JVET-AE0115 | m63829 | 2023-07-04 16:09:15 | 2023-07-04 17:33:07 | 2023-07-12 18:10:00 | Non-EE2: Unified intra CC-models parameters precision | P. Bordes, K. Naser, F. Galpin, F. Le Léannec (InterDigital) |
| JVET-AE0116 | m63830 | 2023-07-04 16:09:25 | 2023-07-04 19:42:14 | 2023-07-11 16:22:39 | EE2-4.4: InterMTS for IBC and IntraTMP | P. Garus, M. Coban, B. Ray, V. Seregin, M. Karczewicz (Qualcomm) |
| JVET-AE0117 | m63831 | 2023-07-04 16:10:59 | 2023-07-05 03:38:30 | 2023-07-12 19:12:47 | Non-EE2: Enhanced subblock-based motion compensation | L. Zhao, K. Zhang, L. Zhang (Bytedance) |
| JVET-AE0118 | m63832 | 2023-07-04 16:17:31 | 2023-07-11 11:01:40 | 2023-07-11 11:01:40 | Crosscheck of JVET-AE0086 (EE2-4.2: Large NSPT) | P. Garus (Qualcomm) |
| JVET-AE0119 | m63834 | 2023-07-04 16:43:10 | 2023-07-04 17:43:50 | 2023-07-04 17:43:50 | AHG14: The extension of SADL library | W. Bao, H. Wang, Z. Chen (Wuhan Univ.) |
| JVET-AE0120 | m63835 | 2023-07-04 16:54:11 | 2023-07-08 02:27:02 | 2023-07-12 04:24:02 | Non-EE2: Intra Angular Prediction Extension | J. Fu, J. Zhang, C. Jia, S. Ma (PKU), Y. Gao, C. Huang (ZTE) |
| JVET-AE0121 | m63836 | 2023-07-04 16:54:48 | 2023-07-04 23:21:03 | 2023-07-13 15:52:10 | Non-EE2: Luma Residual Taps in Chroma-ALF and CCALF | W. Yin, K. Zhang, Z. Deng, L. Zhang (Bytedance) |
| JVET-AE0122 | m63837 | 2023-07-04 17:04:56 | 2023-07-04 21:58:46 | 2023-07-15 10:37:42 | AHG10: GOP-based RPR and scalable coding | K. Andersson (Ericsson) |
| JVET-AE0123 | m63838 | 2023-07-04 17:06:39 | 2023-07-04 17:32:32 | 2023-07-06 14:40:06 | AHG10: Lagrange multiplier optimization for chroma ALF and CCALF | S.-W. Xie, Y. Gao, M.-H. Jia, Y.-L. Hu, C. Huang, P. Wu (ZTE) |
| JVET-AE0124 | m63839 | 2023-07-04 17:18:47 | 2023-07-04 17:30:08 | 2023-07-12 11:50:12 | Non-EE2: Fixes related to Intra TMP | J.-L Lin, P.-H Lin, Y.-J Chang, V. Seregin, M. Karczewicz (Qualcomm) |
| JVET-AE0125 | m63846 | 2023-07-04 17:33:12 | 2023-07-04 17:36:45 | 2023-07-04 17:36:45 | EE2-4.1: Shifting quantization center | M. Balcilar, K. Naser, F. Galpin, F. Le Léannec (InterDigital) |
| JVET-AE0126 | m63848 | 2023-07-04 17:34:35 | 2023-07-04 17:56:14 | 2023-07-04 17:56:14 | AHG9: NNPF cleanup and editorial changes for VSEI | Y.-K. Wang, W. Jia, J. Xu, C. Lin (Bytedance) |
| JVET-AE0127 | m63850 | 2023-07-04 17:35:26 | 2023-07-04 18:00:01 | 2023-07-04 18:00:01 | AHG9: NNPF editorial changes for VVC | Y.-K. Wang, W. Jia, J. Xu (Bytedance) |
| JVET-AE0128 | m63851 | 2023-07-04 17:35:50 | 2023-07-04 18:02:30 | 2023-07-04 18:02:30 | AHG9: On NNPFC extensibility and base filter signalling | Y.-K. Wang (Bytedance) |
| JVET-AE0129 | m63855 | 2023-07-04 17:41:55 | 2023-07-04 18:03:33 | 2023-07-12 11:31:13 | AHG12 Template-based CIIP weight derivation | M.-H. Jia, Y.-L. Hu, S.-W. Xie, Y. Gao, C. Huang (ZTE) |
| JVET-AE0130 | m63860 | 2023-07-04 17:53:53 | 2023-07-04 17:59:00 | 2023-07-12 12:01:01 | Non-EE2: DIMD with filtered template | C. Zhou, Z. Lv (vivo) |
| JVET-AE0131 | m63861 | 2023-07-04 17:58:01 | 2023-07-04 23:23:05 | 2023-07-13 15:52:38 | Non-EE2: Variance based Classification for In-loop Filtering | W. Yin, K. Zhang, L. Zhang (Bytedance) |
| JVET-AE0132 | m63862 | 2023-07-04 18:02:38 | 2023-07-04 18:24:11 | 2023-07-07 19:20:03 | EE2-1.1: Partitioning prediction | G. Laroche, P. Onno (Canon) |
| JVET-AE0133 | m63863 | 2023-07-04 18:02:49 | 2023-07-04 18:26:41 | 2023-07-07 11:31:48 | AhG12: On Common Test Conditions change based on EE2 Test 1.1 | G. Laroche, P. Onno (Canon) |
| JVET-AE0134 | m63864 | 2023-07-04 18:10:56 | 2023-07-04 19:12:06 | 2023-07-04 19:12:06 | AHG9: Align the design of NNPF with multiple input pictures to NNPF including picture rate upsampling | J. Xu, Y.-K. Wang (Bytedance) |
| JVET-AE0135 | m63865 | 2023-07-04 18:11:28 | 2023-07-04 19:12:26 | 2023-07-04 19:12:26 | AHG9: On NNPF picture rate upsampling constraints | J. Xu, Y.-K. Wang (Bytedance) |
| JVET-AE0136 | m63866 | 2023-07-04 18:21:57 | 2023-07-04 20:33:50 | 2023-07-13 14:51:38 | Non-EE2: Fix on TM-based reordering for affine MMVD mode | D. Kim, W. Lim, J. Kim, S.-C. Lim, J. S. Choi (ETRI) |
| JVET-AE0137 | m63867 | 2023-07-04 18:40:44 | 2023-07-04 18:59:05 | 2023-07-12 13:00:56 | Non-EE2: On TIMD fusion | P. Andrivon, M. Blestel (Ofinno) |
| JVET-AE0138 | m63868 | 2023-07-04 18:46:20 | 2023-07-04 19:03:37 | 2023-07-12 16:10:40 | Non-EE2: Reference sample interpolation for intra prediction | G. Rath, F. Le Léannec, F. Urban, F. Racape, T. Dumas (InterDigital) |
| JVET-AE0139 | m63869 | 2023-07-04 18:53:54 | 2023-07-04 22:29:26 | 2023-07-11 00:14:26 | EE2-5.2: Improved fixed filters for ALF | M. Karczewicz, N. Hu, H. Wang, V. Seregin (Qualcomm) |
| JVET-AE0140 | m63870 | 2023-07-04 18:54:12 | 2023-07-11 03:21:33 | 2023-07-13 12:23:47 | Non-EE2: LIC extensions | A. Filippov, V. Rufitskiy, K. Suverov (Ofinno) |
| JVET-AE0141 | m63871 | 2023-07-04 18:57:42 | 2023-07-04 19:12:53 | 2023-07-04 19:12:53 | AHG9: Fix a bug in NNPFC SEI message for colourization | J. Xu, Y.-K. Wang (Bytedance) |
| JVET-AE0142 | m63872 | 2023-07-04 18:59:53 | 2023-07-04 19:38:45 | 2023-07-04 19:38:45 | AHG9: On derivation of NNPF input pictures and the value of nnpfc_purpose | W. Jia, Y.-K. Wang, J. Xu, L. Zhang (Bytedance) |
| JVET-AE0143 | m63873 | 2023-07-04 19:23:37 | 2023-07-04 19:28:16 | 2023-07-18 07:15:26 | AHG8: A spatial resampling algorithm and an exemplar software implementation | S. Wang, B. Li, J. Chen, Y. Ye (Alibaba), S. Wang (CityUniv HK) |
| JVET-AE0144 | m63874 | 2023-07-04 19:32:03 | 2023-07-04 19:35:37 | 2023-07-15 17:46:16 | EE1-6.1: Neural network-based intra prediction with reduced complexity | T. Dumas, F. Galpin, P. Bordes (Interdigital) |
| JVET-AE0145 | m63875 | 2023-07-04 19:36:50 | 2023-07-04 19:41:05 | 2023-07-04 19:41:05 | AHG 12: Flexible GDR | L. Wang, S. Hong, K. Panusopone (Nokia) |
| JVET-AE0146 | m63876 | 2023-07-04 19:38:26 | 2023-07-04 19:43:12 | 2023-07-13 19:34:47 | AHG 12: Fixes H-CCP Table for CUs in Refreshed Areas of GDR/Recovering Pictures | S. Hong, L. Wang, K. Panusopone (Nokia) |
| JVET-AE0147 | m63877 | 2023-07-04 19:53:49 | 2023-07-04 19:56:20 | 2023-07-13 16:41:32 | AHG12: Updating context model parameters for ECM | R.-L. Liao, Y. Ye, J. Chen, X. Li (Alibaba) |
| JVET-AE0148 | m63878 | 2023-07-04 20:22:11 | 2023-07-04 23:59:42 | 2023-07-13 09:00:15 | Non-EE2: Affine subblock BDOF refinement | Z. Zhang, H. Huang, J.-L Lin, V. Seregin, M. Karczewicz (Qualcomm) |
| JVET-AE0149 | m63879 | 2023-07-04 20:26:40 | 2023-07-11 07:31:09 | 2023-07-11 07:31:09 | Crosscheck of JVET-AE0125 (EE2-4.1 Shifting Quantizer Center) | M. Coban (Qualcomm) |
| JVET-AE0150 | m63880 | 2023-07-04 22:04:16 | 2023-07-04 23:34:52 | 2023-07-11 09:53:54 | EE2-3.7 RPR with new filters for scale factor 1.25x and 1.33x | J. Samuelsson-Allendes, S. Deshpande (Sharp) |
| JVET-AE0151 | m63881 | 2023-07-04 22:24:25 | 2023-07-04 22:32:57 | 2023-07-11 08:04:49 | EE2-5.1: CCSAO with extended edge classifiers and history offsets | C.-W. Kuo, X. Xiu, W. Chen, H.-J. Jhu, N. Yan, C. Ma, X. Wang (Kwai) |
| JVET-AE0152 | m63882 | 2023-07-04 22:24:55 | 2023-07-04 22:41:12 | 2023-08-01 17:42:55 | EE2-5.3: Combination of Test 5.1b and Test 5.2c | C.-W. Kuo, X. Xiu, W. Chen, H.-J. Jhu, N. Yan, C. Ma, X. Wang (Kwai), M. Karczewicz, N. Hu, H. Wang, V. Seregin (Qualcomm) |
| JVET-AE0153 | m63883 | 2023-07-04 22:25:54 | 2023-07-04 23:52:43 | 2023-07-15 15:45:28 | Non-EE2: Enabling template-based inter tools for scaled reference pictures in the RPR | X. Xiu, H.-J. Jhu, C.-W. Kuo, C. Ma, N. Yan, W. Chen, X. Wang (Kwai) |
| JVET-AE0154 | m63884 | 2023-07-04 22:30:03 | 2023-07-04 23:19:50 | 2023-07-06 09:23:47 | MC-IF VVC technical guidelines | L. Litwic (Ericsson), S. McCarthy (Dolby), S. Wenger (Tencent), J. Ridge (Nokia), B. Bross (Fraunhofer HHI), D. Rusanovskyy (Qualcomm), A. Stein (InterDigital) |
| JVET-AE0155 | m63885 | 2023-07-04 22:36:01 | 2023-07-04 23:03:40 | 2023-07-04 23:03:40 | AHG2/AHG9: Editor commentary on the post-filter hint SEI message semantics | G. J. Sullivan, Y.-K. Wang (Editors) |
| JVET-AE0156 | m63886 | 2023-07-04 22:38:28 | 2023-07-04 23:04:15 | 2023-07-11 11:22:22 | AHG9: Message wrapping and importance indication for the SEI processing order SEI message | G. J. Sullivan, S. McCarthy, P. Yin (Dolby Labs) |
| JVET-AE0157 | m63887 | 2023-07-04 22:49:37 | 2023-07-10 19:58:04 | 2023-07-10 19:58:04 | Crosscheck of JVET-AE0078(EE2-2.10: IBC-LIC extension) | W. Yin (Bytedance) |
| JVET-AE0158 | m63888 | 2023-07-04 22:56:54 | 2023-07-04 23:05:49 | 2023-07-12 16:06:58 | AHG12: LUT-based angle calculation for DIMD | A. Aminlou, J. Lainema (Nokia) |
| JVET-AE0159 | m63889 | 2023-07-04 23:02:50 | 2023-07-04 23:29:22 | 2023-07-12 08:26:47 | EE2-2.5: Filtered Intra Block Copy (FIBC) | H.-J. Jhu, X. Xiu, C.-W. Kuo, W. Chen, N. Yan, C. Ma, X. Wang (Kwai), B. Ray, M. Coban, V. Seregin, M. Karczewicz (Qualcomm) |
| JVET-AE0160 | m63890 | 2023-07-04 23:04:31 | 2023-07-04 23:14:47 | 2023-07-13 08:45:13 | EE1-1.5: Optimization for complexity-performance trade-off of HOP network | R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AE0161 | m63891 | 2023-07-04 23:04:46 | 2023-07-04 23:15:05 | 2023-07-13 15:01:46 | AHG11: Input and output rotation of model for NNVC in-loop filter | R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AE0162 | m63892 | 2023-07-04 23:05:02 | 2023-07-04 23:15:21 | 2023-07-13 16:34:26 | AHG11/AHG14: Fix MS-SSIM calculation for SR | R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AE0163 | m63893 | 2023-07-04 23:12:20 | 2023-07-04 23:40:27 | 2023-07-15 15:34:33 | AHG12: Modified CTC proposal for low-delay configuration | S. Puri, C. Bonnineau, F. Le Léannec, T. Poirier, E. François (InterDigital) |
| JVET-AE0164 | m63894 | 2023-07-04 23:23:17 | 2023-07-04 23:47:06 | 2023-07-13 12:19:54 | EE1-1.2 Complexity-performance tradeoff of decomposition | D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm) |
| JVET-AE0165 | m63895 | 2023-07-04 23:30:53 | 2023-07-04 23:48:35 | 2023-07-20 11:52:51 | EE1-4.4: Low complexity NN filter with design elements of Unified Filter Architecture and EE1-1.2 and EE1-1.3 | Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm) |
| JVET-AE0166 | m63896 | 2023-07-04 23:34:54 | 2023-07-05 02:33:05 | 2023-07-12 23:11:53 | AHG12: IntraTMP and IBC search area harmonization | D. Ruiz Coll, B. Chen, P. Andrivon, M. Blestel (Ofinno) |
| JVET-AE0167 | m63897 | 2023-07-04 23:43:41 | 2023-07-05 07:27:14 | 2023-07-12 14:43:44 | AHG12: DBV improvement | L. Xu, Y. Yu, H. Yu, D. Wang (OPPO) |
| JVET-AE0168 | m63898 | 2023-07-04 23:45:15 | 2023-07-04 23:53:13 | 2023-07-13 15:20:59 | Non-EE2: Sample-based BDOF for Chroma | C.-C. Chen, H. Huang, Z. Zhang, C. S. Coban, V. Seregin, M. Karczewicz (Qualcomm) |
| JVET-AE0169 | m63899 | 2023-07-04 23:59:16 | 2023-07-05 00:03:08 | 2023-07-11 01:07:54 | EE2-2.2/EE2-2.3/EE2-2.4: Bi-predictive IBC GPM, bi-predictive IBC, and IBC MBVD list derivation for camera captured and screen contents | Y. Kidani, H. Kato, K. Kawamura (KDDI), C. Ma, X. Xiu, W. Chen, H.-J. Jhu, C.-W. Kuo, N. Yan, X. Wang (Kwai), G. Verba, Z. Zhang, P. Nikitin, H. Huang, V. Seregin, M. Karczewicz (Qualcomm) |
| JVET-AE0170 | m63900 | 2023-07-05 00:35:45 | 2023-07-05 03:20:54 | 2023-07-17 13:48:12 | Non-EE2: Enhancements on CCP merge | K. Zhang, Z. Deng, L. Zhang (Bytedance) |
| JVET-AE0171 | m63902 | 2023-07-05 01:42:33 | 2023-07-05 03:16:19 | 2023-07-12 07:07:27 | AHG11: Neural network-based in-loop filter with layer normalization | Y. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AE0172 | m63903 | 2023-07-05 01:43:11 | 2023-07-05 11:08:05 | 2023-07-12 17:41:35 | Update on multilayer coding support for VVenC | S. Iwamura, S. Nemoto, A. Ichigaya (NHK) |
| JVET-AE0173 | m63904 | 2023-07-05 01:52:09 | 2023-07-05 01:55:46 | 2023-07-12 11:16:05 | [AHG9] Clarification and improvements of signalling of NNPF update | Y. Lim (Samsung) |
| JVET-AE0174 | m63905 | 2023-07-05 02:00:58 | 2023-07-05 04:12:11 | 2023-07-15 15:32:11 | AHG7: On TM control for non-inter tools | Z. Deng, K. Zhang, L. Zhang (Bytedance) |
| JVET-AE0175 | m63906 | 2023-07-05 02:03:12 | 2023-07-05 02:11:10 | 2023-07-05 02:11:10 | [AHG9] Editorial improvements of nnpfc_mode_idc | Y. Lim (Samsung) |
| JVET-AE0176 | m63907 | 2023-07-05 02:55:34 | 2023-07-05 04:12:55 | 2023-07-12 09:51:26 | EE2-related: Enhancements on CCRM | Z. Deng, K. Zhang, L. Zhang (Bytedance) |
| JVET-AE0177 | m63909 | 2023-07-05 03:46:54 | 2023-07-05 03:50:52 | 2023-07-13 12:01:32 | Non-EE2: Local illumination compensation with slope adjustment | Y. Wang, K. Zhang, Y. He, H. Liu, L. Zhang (Bytedance) |
| JVET-AE0178 | m63911 | 2023-07-05 05:05:17 | 2023-07-05 05:52:54 | 2023-07-14 16:34:11 | Non-EE2: Cross-component prediction merge mode for chroma inter coding | M.-S. Chiang, H.-Y. Tseng, C.-M. Tsai, C.-Y. Chuang, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, O. Chubach, Y.-W. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) |
| JVET-AE0179 | m63912 | 2023-07-05 06:14:29 | 2023-07-05 15:13:30 | 2023-07-05 15:13:30 | AHG4: Renewed license statement for Ghost Town Fly and Undo Dancer 3D video test sequences | M. M. Hannuksela, N. Salonen (Nokia) |
| JVET-AE0180 | m63913 | 2023-07-05 09:23:52 | 2023-07-05 09:33:31 | 2023-07-16 00:55:49 | On ECM SW memory consumption | R. Chernyak, S. Liu (Tencent), Y. Yasugi, T. Ikai (Sharp) |
| JVET-AE0181 | m63914 | 2023-07-05 10:18:56 | 2023-07-05 11:07:31 | 2023-07-16 09:34:41 | Scaling window support for VTM | S. Iwamura, S. Nemoto, A. Ichigaya (NHK) |
| JVET-AE0182 | m63915 | 2023-07-05 10:20:11 | 2023-07-05 10:41:34 | 2023-07-17 13:30:01 | AHG9: SEI message extension of VVC for object-wave compression and computer-generated hologram use | K. Nonaka, R. Koiso, H. Kojima, K. Kawamura, H. Kato (KDDI) |
| JVET-AE0183 | m63916 | 2023-07-05 11:20:23 | 2023-07-11 09:40:04 | 2023-07-11 09:40:04 | Crosscheck of JVET-AE0067 (EE1-4.1: Neural-network loop filters with further complexity reduction) | J. Ström (Ericsson) |
| JVET-AE0184 | m63917 | 2023-07-05 12:16:26 | 2023-07-05 12:19:54 | 2023-07-16 14:38:05 | AHG12: TIMD with IntraTMP and IBC candidates | K. Naser, F. Le Léannec, P. Bordes, F. Galpin, A. Robert (InterDigital) |
| JVET-AE0185 | m63918 | 2023-07-05 12:24:16 | 2023-07-05 12:27:02 | 2023-07-16 16:37:53 | AHG12: IntraTMP with Neighbouring Candidates | K. Naser, F. Le Léannec, A. Robert, F. Galpin (InterDigital) |
| JVET-AE0186 | m63923 | 2023-07-05 15:24:33 | 2023-07-05 15:32:32 | 2023-07-16 14:38:52 | Non-EE2-2.1: Block Vector Guided Chroma Direct Mode | K. Naser, F. Le Léannec, P. Bordes, Y. Chen, G. Rath (InterDigital) |
| JVET-AE0187 | m63931 | 2023-07-05 21:42:29 | 2023-07-06 02:40:58 | 2023-07-10 23:17:11 | AHG9: A summary of SEI proposals on NNPF | Y.-K. Wang (Bytedance) |
| JVET-AE0188 | m63932 | 2023-07-06 01:53:44 | 2023-07-17 14:02:46 | 2023-07-17 14:02:46 | Crosscheck of JVET-AE0097 (AHG12: On the cross-component merge mode) | K. Zhang (Bytedance) |
| JVET-AE0189 | m63941 | 2023-07-06 07:16:21 | 2023-07-06 07:18:37 | 2023-07-06 07:18:37 | AHG9: On the design of nnpfa_target_base_flag in NNPFA SEI message | Hendry (LGE) |
| JVET-AE0190 | m63947 | 2023-07-06 09:59:37 | 2023-07-10 20:00:21 | 2023-07-10 20:00:21 | Crosscheck of JVET-AE0044(AHG12: Dynamic Scaling of Bilateral Filter) | W. Yin (Bytedance) |
| JVET-AE0191 | m63951 | 2023-07-06 10:46:35 | 2023-07-06 10:58:36 | 2023-07-25 17:57:42 | AhG11: EE1-0 High Operation Point model | F. Galpin (InterDigital), S. Eadie, D. Rusanovskyy, Y. Li (Qualcomm), Y. Li, J. Li (ByteDance), L. Wang, R. Chang (Tencent), Z. Xie (Oppo), E. Alshina (Huawei) |
| JVET-AE0192 | m63952 | 2023-07-06 10:49:40 | 2023-07-13 18:05:41 | 2023-07-13 18:05:41 | Crosscheck JVET-AE0180 (On ECM SW memory consumption) | C.-W. Kuo, X. Xiu (Kwai) |
| JVET-AE0193 | m63954 | 2023-07-06 12:06:12 | 2023-07-11 07:21:10 | 2023-07-11 07:21:10 | Crosscheck of JVET-AE0139 (EE2-5.2: Improved fixed filters for ALF) | C.-W. Kuo (Kwai) |
| JVET-AE0194 | m63960 | 2023-07-06 13:26:02 | 2023-07-06 13:31:45 | 2023-07-13 09:59:25 | AhG14: SADL update | F. Galpin, T. Dumas, P. Bordes, E. François (InterDigital) |
| JVET-AE0195 | m63962 | 2023-07-06 15:05:00 | 2023-07-06 18:05:21 | 2023-07-11 12:30:48 | AhG7: ECM stages complexity assessment | F. Galpin, F. Leleannec, C. Salmon-Legagneur, E. François (InterDigital) |
| JVET-AE0196 | m63965 | 2023-07-06 17:08:17 | 2023-07-06 17:10:27 | 2023-07-06 17:10:27 | EE2-3.4: OBMC with HPel flag and BCW weights | A. Robert, F. Le Léannec, F. Galpin, T. Poirier, Y. Chen (InterDigital) |
| JVET-AE0197 | m63967 | 2023-07-06 17:10:58 | 2023-07-10 20:35:39 | 2023-07-10 20:35:39 | Crosscheck of JVET-AE0169 (EE2-2.2c: Bi-predictive IBC-GPM and EE2-2.3c: Combination of 2.3b and 2.2c)) | W. Yin (Bytedance) |
| JVET-AE0198 | m63968 | 2023-07-06 17:43:22 | 2023-07-06 17:48:33 | 2023-07-06 17:48:33 | AHG12: Extended Search Region for IntraTMP | K. Naser, P. Bordes, F. Le Léannec, F. Galpin (InterDigital) |
| JVET-AE0199 | m63975 | 2023-07-07 01:32:12 | 2023-07-11 12:54:00 | 2023-07-11 12:54:00 | Crosscheck of JVET-AE0046 (EE2-3.2: Bi-predictive GPM) | Y. Ahn (LGE) |
| JVET-AE0200 | m63976 | 2023-07-07 02:24:53 | 2023-07-11 09:58:33 | 2023-07-11 09:58:33 | Crosscheck of JVET-AE0065 (EE2-3.5: Iterative BDOF pass in multi-pass DMVR) | Z. Lv (vivo) |
| JVET-AE0201 | m63977 | 2023-07-07 03:21:13 | 2023-07-12 09:42:54 | 2023-07-12 09:42:54 | Crosscheck of JVET-AE0056 (Non-EE2: Changes on TIMD and SGPM for reducing ECM decoder complexity) | F. Wang (OPPO) |
| JVET-AE0202 | m63988 | 2023-07-07 04:39:52 | 2023-07-12 09:27:02 | 2023-07-12 09:27:02 | Crosscheck of JVET-AE0075 (Non-EE2: On IntraTMP block vector) | W. Lim, S.-C. Lim (ETRI) |
| JVET-AE0203 | m63995 | 2023-07-07 09:24:17 | 2023-07-12 09:12:48 | 2023-07-12 09:12:48 | Crosscheck of JVET-AE0098 (AHG12: On the chroma DBV mode) | L. Xu, Y. Yu (OPPO) |
| JVET-AE0204 | m63999 | 2023-07-07 11:35:04 | 2023-07-11 10:04:31 | 2023-07-11 10:04:31 | Crosscheck of JVET-AE0150 (EE2-3.7 RPR with new filters for scale factor 1.25x and 1.33x) | Z. Zhang (Qualcomm) |
| JVET-AE0205 | m64010 | 2023-07-07 14:32:02 | 2023-07-11 11:14:00 | 2023-07-11 11:14:00 | Crosscheck of JVET-AE0169 (Test EE2-2.3c and EE2-2.4c) | R. Yu (Ericsson) |
| JVET-AE0206 | m64011 | 2023-07-07 14:34:06 | 2023-07-12 10:19:00 | 2023-07-12 10:19:00 | Crosscheck of JVET-AE0082 (AHG12: Non-square quadtree partitioning) | R. Yu (Ericsson) |
| JVET-AE0207 | m64017 | 2023-07-07 16:17:20 | 2023-07-13 15:34:27 | 2023-07-13 15:34:27 | Crosscheck of JVET-AE0131 (Non-EE2: Variance based Classification for In-loop Filtering) | V. Shchukin (Ericsson) |
| JVET-AE0208 | m64018 | 2023-07-07 18:36:06 | 2023-07-11 10:01:39 | 2023-07-11 10:01:39 | Crosscheck of JVET-AE0138 (Non-EE2: Reference sample interpolation for intra prediction) | P. Nikitin (Qualcomm) |
| JVET-AE0209 | m64019 | 2023-07-07 19:22:22 | 2023-07-12 09:36:26 | 2023-07-12 09:36:26 | Crosscheck of JVET-AE0151 (EE2-5.1: CCSAO with extended edge classifiers and history offsets) | N. Hu (Qualcomm) |
| JVET-AE0210 | m64029 | 2023-07-08 00:22:56 | 2023-07-11 00:45:21 | 2023-07-11 00:45:21 | crosscheck of JVET-AE0077 EE2-2.9: Extended search areas for IntraTMP mode | K. Naser (InterDigital) |
| JVET-AE0211 | m64030 | 2023-07-08 00:24:48 | 2023-07-11 00:53:35 | 2023-07-11 00:53:35 | crosscheck of JVET-AE0116 EE2-4.4: InterMTS for IBC and IntraTMP | K. Naser (InterDigital) |
| JVET-AE0212 | m64031 | 2023-07-08 01:18:04 | 2023-07-13 09:18:00 | 2023-07-13 09:18:00 | Cross-check of JVET-AE0174 On TM control for non-inter tools | X. Li (Google) |
| JVET-AE0213 | m64032 | 2023-07-08 01:29:07 | 2023-07-10 20:54:37 | 2023-07-10 20:54:37 | Crosscheck of JVET-AE0136 (Non-EE2: Fix on TM-based reordering for affine MMVD mode) | M. Salehifar (Bytedance) |
| JVET-AE0214 | m64037 | 2023-07-08 15:33:11 | 2023-07-08 15:41:17 | 2023-07-16 14:40:37 | AHG12: on Intra CIIP | K. Naser, P. Bordes, F. Galpin, K. Reuzé (InterDigital) |
| JVET-AE0215 | m64038 | 2023-07-08 15:42:17 |  |  | Withdrawn |  |
| JVET-AE0216 | m64040 | 2023-07-08 18:40:07 | 2023-07-11 10:18:30 | 2023-07-11 10:18:30 | Crosscheck of JVET-AE0077 (EE2-2.9: Extended search areas for IntraTMP mode) | P.-H Lin (Qualcomm) |
| JVET-AE0217 | m64048 | 2023-07-09 07:13:45 | 2023-07-11 08:30:49 | 2023-07-11 08:30:49 | Crosscheck of JVET-AE0076 (EE2-2.8: An extrapolation filter-based intra prediction mode) | H.-J. Jhu (Kwai) |
| JVET-AE0218 | m64055 | 2023-07-09 11:06:57 | 2023-07-13 08:50:50 | 2023-07-13 08:50:50 | Crosscheck of JVET-AE0047 (AHG12: On GPM-MMVD) | Z. Deng (Bytedance) |
| JVET-AE0219 | m64064 | 2023-07-09 17:23:42 | 2023-07-13 17:28:10 | 2023-07-13 17:28:10 | AHG4: Results of visual checking of SVVC VT streams with and without DMVR fix | M. Wien (RWTH Aachen University), K. Andersson (Ericsson), P. de Lagrange (InterDigital) |
| JVET-AE0220 | m64132 | 2023-07-10 10:44:49 | 2023-07-11 09:02:47 | 2023-07-11 09:02:47 | Crosscheck of JVET-AE0043 (EE2-2.7: Cross-component merge mode with temporal candidates) | P. Bordes (InterDigital) |
| JVET-AE0221 | m64137 | 2023-07-10 10:51:55 | 2023-07-13 11:42:30 | 2023-07-13 11:42:30 | Crosscheck of JVET-AE0081 ([AHG8] De-noising filter as pre-processing for machine task) | C. Hollmann (Ericsson) |
| JVET-AE0222 | m64138 | 2023-07-10 10:56:51 | 2023-07-10 13:49:19 | 2023-07-10 13:49:19 | Crosscheck of JVET-AE0084 (EE2-2.11: Harmonization of IBC HMVP and IBC-LIC) | C. Ma (Kwai) |
| JVET-AE0223 | m64139 | 2023-07-10 11:03:25 | 2023-07-11 05:16:25 | 2023-07-11 05:16:25 | Crosscheck of JVET-AE0091 (EE2-3.8: Combination of Test 3.3e and Test 3.5) | C. Ma (Kwai) |
| JVET-AE0224 | m64140 | 2023-07-10 11:06:47 | 2023-07-18 15:53:21 | 2023-07-18 15:53:21 | Crosscheck of JVET-AE0105 (Non-EE2: Local illumination compensation with multiple templates) | C. Ma (Kwai) |
| JVET-AE0225 | m64149 | 2023-07-10 11:22:39 | 2023-07-10 11:46:32 | 2023-07-10 13:58:41 | Crosscheck of JVET-AE0102 (EE2-4.3: Context modelling for transform coefficients for LFNST/NSPT) | M. Abdoli (IRT b-com) |
| JVET-AE0226 | m64151 | 2023-07-10 11:24:21 | 2023-07-10 16:23:56 | 2023-07-10 16:23:56 | Crosscheck of JVET-AE0100 (EE2-2.1: Block vector guided CCCM) | M. Abdoli (IRT b-com) |
| JVET-AE0227 | m64155 | 2023-07-10 11:28:54 | 2023-07-11 11:54:50 | 2023-07-17 17:11:53 | AHG4: Experiments in preparation of scalable quality ladder visual testing | P. de Lagrange, F. Urban (InterDigital) |
| JVET-AE0228 | m64237 | 2023-07-10 16:43:58 | 2023-07-10 23:05:32 | 2023-07-12 12:59:33 | Crosscheck of JVET-AE0159 (EE2-2.5c: Filtered Intra Block Copy (FIBC) | L. Zhang, Y. Yu (OPPO) |
| JVET-AE0229 | m64248 | 2023-07-10 16:50:22 | 2023-07-10 16:58:14 | 2023-07-10 16:58:14 | Crosscheck of JVET-AE0112(EE1-5.1: Deep Reference Frame Generation for Inter Prediction Enhancement) | Z. Xie (OPPO) |
| JVET-AE0230 | m64249 | 2023-07-10 16:53:15 | 2023-07-14 09:23:56 | 2023-07-14 09:23:56 | Crosscheck of JVET-AE0162(AHG11/AHG14: Fix MS-SSIM calculation for SR) | Z. Xie (OPPO) |
| JVET-AE0231 | m64250 | 2023-07-10 16:54:25 | 2023-07-12 09:19:09 | 2023-07-12 09:19:09 | Crosscheck of JVET-AE0121(Non-EE2: Luma Residual Taps in Chroma-ALF and CCALF) | Z. Xie (OPPO) |
| JVET-AE0232 | m64263 | 2023-07-10 17:44:30 | 2023-07-10 17:55:14 | 2023-07-10 17:55:14 | Crosscheck of JVET-AE0072 ([AHG11] On residual adjustments for NNLF) | T. Shao (Dolby) |
| JVET-AE0233 | m64306 | 2023-07-10 20:25:34 |  |  | Withdrawn |  |
| JVET-AE0234 | m64317 | 2023-07-10 21:06:31 | 2023-07-10 21:10:44 | 2023-07-10 21:10:44 | AHG8: Crosscheck of JVET-AE0107 | H. Zhang (Nokia) |
| JVET-AE0235 | m64346 | 2023-07-10 23:25:17 | 2023-07-11 10:33:04 | 2023-07-11 14:59:56 | Cross-check of JVET-AE0132 â€œEE2-1.1: Partitioning predictionâ€ | R. Utida, F. Le Léannec (InterDigital) |
| JVET-AE0236 | m64350 | 2023-07-10 23:27:31 | 2023-07-11 10:28:19 | 2023-07-11 10:28:19 | Cross-check of JVET-AE0059 â€œEE2-3.1: Cross-component residual model (CCRM) for inter predictionâ€ | F. Le Léannec (InterDigital) |
| JVET-AE0237 | m64386 | 2023-07-11 00:34:13 | 2023-07-11 01:55:55 | 2023-07-11 01:55:55 | Presentation of AI-based video Coding in "Third AG4 Workshop on JPEG and MPEG Emerging Activities" | E. Alshina |
| JVET-AE0238 | m64398 | 2023-07-11 07:25:29 | 2023-07-13 08:34:38 | 2023-07-13 15:41:59 | AHG11: A unified design of NN-based loop-filters at low and high operation points | J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang |
| JVET-AE0239 | m64403 | 2023-07-11 09:25:33 | 2023-07-16 17:23:11 | 2023-07-16 17:23:11 | Crosscheck of JVET-AE0087 (EE2-3.3: High-Accuracy template matching) | J. Chen (Alibaba) |
| JVET-AE0240 | m64404 | 2023-07-11 09:26:51 | 2023-07-16 17:41:53 | 2023-07-16 17:41:53 | Crosscheck of JVET-AE0094 (EE2-2.6c: IBC with non-adjacent spatial candidates) | J. Chen (Alibaba) |
| JVET-AE0241 | m64405 | 2023-07-11 09:27:50 | 2023-07-16 17:56:07 | 2023-07-17 15:56:58 | Crosscheck of JVET-AE0152 (EE2-5.3: Combination of Test 5.1b and Test 5.2c) | J. Chen (Alibaba) |
| JVET-AE0242 | m64406 | 2023-07-11 09:29:14 | 2023-07-17 14:29:21 | 2023-07-17 14:29:21 | Crosscheck of JVET-AE0130 (Non-EE2: DIMD with filtered template) | J. Chen (Alibaba) |
| JVET-AE0243 | m64407 | 2023-07-11 09:44:07 | 2023-07-13 09:25:49 | 2023-07-13 09:25:49 | Crosscheck of JVET-AE0097 (AHG12: On the cross-component merge mode) | C.-M. Tsai (MediaTek) |
| JVET-AE0244 | m64408 | 2023-07-11 09:44:59 | 2023-07-11 10:51:39 | 2023-07-11 10:51:39 | Crosscheck of JVET-AE0115 (Non-EE2: Unified intra CC-models parameters precision) | C.-Y. Chuang (MediaTek) |
| JVET-AE0245 | m64410 | 2023-07-11 09:53:36 | 2023-07-11 10:17:29 | 2023-07-11 10:17:29 | Crosscheck of JVET-AE0077 (EE2-2.9: Extended search areas for IntraTMP mode) | X. Li (Alibaba) |
| JVET-AE0246 | m64411 | 2023-07-11 09:54:29 | 2023-07-12 11:26:54 | 2023-07-12 11:26:54 | Crosscheck of JVET-AE0075 (Non-EE2: On IntraTMP block vector) | X. Li (Alibaba) |
| JVET-AE0247 | m64412 | 2023-07-11 09:54:43 | 2023-07-12 11:27:14 | 2023-07-12 11:27:14 | Crosscheck of JVET-AE0124 (Non-EE2: Fixes related to Intra TMP) | X. Li (Alibaba) |
| JVET-AE0248 | m64413 | 2023-07-11 09:54:57 | 2023-07-13 14:50:41 | 2023-07-13 14:50:41 | Crosscheck of JVET-AE0177 (Non-EE2: Local illumination compensation with slope adjustment) | X. Li (Alibaba) |
| JVET-AE0249 | m64414 | 2023-07-11 10:00:11 | 2023-07-11 10:01:57 | 2023-07-11 10:01:57 | Crosscheck of JVET-AE0046 (EE2-3.2: Bi-predictive GPM) | Y. Kidani, K. Kawamura (KDDI) |
| JVET-AE0250 | m64415 | 2023-07-11 10:00:47 | 2023-07-12 00:20:46 | 2023-07-12 00:20:46 | Film Grain Synthesis: Material Selection for Ground Truth Content Creation | D. Podborski, D. Ugur, B. Williams, L. Levinson, R. Molholm, A. M. Tourapis |
| JVET-AE0251 | m64416 | 2023-07-11 10:09:09 | 2023-07-12 06:43:50 | 2023-07-12 06:43:50 | Evaluating Denoising methods in the context of Film Grain Analysis/Synthesis | D. Podborski, D. Ugur, A. M. Tourapis |
| JVET-AE0252 | m64419 | 2023-07-11 10:19:46 | 2023-07-13 09:44:20 | 2023-07-13 09:44:20 | Crosscheck of JVET-AE0147 (AHG12: Updating context model parameters for ECM) | Z. Deng (Bytedance) |
| JVET-AE0253 | m64427 | 2023-07-11 11:34:41 | 2023-07-11 11:41:10 | 2023-07-11 11:41:10 | Cross-check of JVET-AE0059 (EE2-3.1: Cross-component residual model (CCRM) for inter prediction) | J. Arumugam (Ittiam) |
| JVET-AE0254 | m64430 | 2023-07-11 15:20:16 | 2023-07-13 15:05:35 | 2023-07-13 15:05:35 | Crosscheck of JVET-AC0074 (Non-EE2: Improvement on cross-component prediction merge mode) | X. Li (Alibaba) |
| JVET-AE0255 | m64435 | 2023-07-11 20:57:56 | 2023-07-12 11:15:40 | 2023-07-14 15:33:33 | Crosscheck of JVET-AE0160 (EE1-1.5: Optimization for complexity-performance trade-off of HOP network) | D. Liu (Ericsson) |
| JVET-AE0256 | m64445 | 2023-07-12 05:27:01 | 2023-07-14 06:28:17 | 2023-07-14 06:28:17 | Crosscheck of JVET-AE0174 (On TM control for non-inter tools) | H. Wang (Qualcomm) |
| JVET-AE0257 | m64446 | 2023-07-12 06:15:59 | 2023-07-12 08:25:28 | 2023-07-13 15:53:04 | Non-EE2: Peak Memory Reduction for ECM | W. Yin, K. Zhang, L. Zhang (Bytedance) |
| JVET-AE0258 | m64453 | 2023-07-12 08:44:55 | 2023-07-13 12:51:27 | 2023-07-13 12:51:27 | Crosscheck of JVET-AE0124 aspect 1 (Non-EE2: Fixes related to Intra TMP) | F. Wang, Y. Yu (OPPO) |
| JVET-AE0259 | m64454 | 2023-07-12 08:56:15 | 2023-07-14 19:01:57 | 2023-07-14 19:01:57 | Crosscheck of JVET-AE0153 (Non-EE2: Enabling template-based inter tools for scaled reference pictures in the RPR) | Z. Zhang (Qualcomm) |
| JVET-AE0260 | m64455 | 2023-07-12 09:14:38 | 2023-07-25 20:52:11 | 2023-07-25 20:52:11 | Crosscheck of JVET-AE0074 (Non-EE2: Improvement on cross-component prediction merge mode) | K. Zhang (Bytedance) |
| JVET-AE0261 | m64456 | 2023-07-12 09:21:57 | 2023-07-17 11:07:20 | 2023-07-17 11:07:20 | Crosscheck of JVET-AE0170 (Non-EE2: Enhancements on CCP merge) | H. Huang (OPPO) |
| JVET-AE0262 | m64457 | 2023-07-12 09:33:07 | 2023-07-15 00:17:18 | 2023-07-15 00:17:18 | Crosscheck of JVET-AE0123 (AHG10: Lagrange multiplier optimization for chroma ALF and CCALF) | H. Zhang (Tencent) |
| JVET-AE0263 | m64458 | 2023-07-12 09:34:04 | 2023-07-17 15:30:04 | 2023-07-17 15:30:04 | Crosscheck of JVET-AE0085 (Non-EE2: Direct block vector (DBV) mode extension) | H. Huang (Qualcomm) |
| JVET-AE0264 | m64459 | 2023-07-12 09:34:37 | 2023-07-15 16:43:51 | 2023-07-15 16:43:51 | Crosscheck of JVET-AE0163 (AHG12: Modified CTC proposal for low-delay configuration) | H. Huang (Qualcomm) |
| JVET-AE0265 | m64460 | 2023-07-12 09:35:33 | 2023-07-14 10:03:31 | 2023-07-14 10:03:31 | Crosscheck of JVET-AE0167 (AHG12: DBV improvement) | H. Huang (Qualcomm) |
| JVET-AE0266 | m64461 | 2023-07-12 09:38:58 | 2023-07-16 13:04:30 | 2023-07-17 14:23:49 | Update on open, optimized VVC implementations VVenC and VVdeC | A. Wieckowski, J. Brandenburg, C. Bartnik, V. George, J. Güther, G. Hege, C. Helmrich, A. Henkel, T. Hinz, C. Lehmann, C. Stoffers, B. Bross, H. Schwarz, D. Marpe, T. Schierl (HHI) |
| JVET-AE0267 | m64463 | 2023-07-12 11:23:16 | 2023-07-13 15:44:04 | 2023-07-13 15:44:04 | Crosscheck of JVET-AE0145 (AHG 12: Flexible GDR) | J. Enhorn (Ericsson) |
| JVET-AE0268 | m64464 | 2023-07-12 11:28:39 | 2023-07-14 09:49:25 | 2023-07-17 08:11:42 | Crosscheck of JVET-AE0073 (Non-EE2: IBC-LIC Model Merge mode) | Y. Wang (Bytedance) |
| JVET-AE0269 | m64465 | 2023-07-12 11:54:39 |  |  | Withdrawn |  |
| JVET-AE0270 | m64469 | 2023-07-12 15:48:04 |  |  | Withdrawn |  |
| JVET-AE0271 | m64471 | 2023-07-12 17:19:40 | 2023-07-13 19:04:12 | 2023-07-13 19:04:12 | Crosscheck of JVET-AE0055 (AHG12: Sign prediction parameter configuration for low-delay conditions) | Y.-J. Chang (Qualcomm) |
| JVET-AE0272 | m64478 | 2023-07-12 22:09:44 | 2023-07-12 23:48:18 | 2023-07-13 17:19:19 | BoG report on SEI messages for VSEI v3 and VVC v3 | G. J. Sullivan (BoG coordinator) |
| JVET-AE0273 | m64488 | 2023-07-13 07:15:31 | 2023-07-15 16:21:23 | 2023-07-15 16:21:23 | Crosscheck of JVET-AE0146 (AHG 12: Fixes H-CCP Table for CUs in Refreshed Areas of GDR/Recovering Pictures) | L. Zhao (Bytedance) |
| JVET-AE0274 | m64491 | 2023-07-13 10:03:03 | 2023-07-16 10:19:15 | 2023-07-16 10:19:15 | Crosscheck of JVET-AE0257 (Non-EE2: Peak Memory Reduction for ECM) | Y. Yasugi (Sharp) |
| JVET-AE0275 | m64492 | 2023-07-13 10:17:49 | 2023-07-13 10:22:59 | 2023-07-13 10:22:59 | Cross-check of JVET-AE0181 (Scaling window support for VTM) | F. Urban (InterDigital) |
| JVET-AE0276 | m64494 | 2023-07-13 11:46:20 | 2023-07-14 09:24:20 | 2023-07-14 09:24:20 | Crosscheck of JVET-AE0161 (AHG11: Input and output rotation of model for NNVC in-loop filter) | Z. Xie (OPPO) |
| JVET-AE0277 | m64495 | 2023-07-13 11:50:14 | 2023-07-13 11:55:19 | 2023-07-13 11:55:19 | Cross-check of JVET-AE0104 (AHG10: GOP-based RPR encoder control using parallel resolution encoding) | K. Andersson (Ericsson) |
| JVET-AE0278 | m64498 | 2023-07-13 14:30:53 | 2023-07-13 14:50:05 | 2023-07-13 14:50:05 | AHG6/Non-EE2: Suggestions on memory reduction for ECM encoder | N. Hu, V. Seregin, M. Karczewicz (Qualcomm) |
| JVET-AE0279 | m64499 | 2023-07-13 16:14:28 | 2023-07-13 16:29:41 | 2023-07-13 16:29:41 | AHG9: New nnpfc_mode_idc for reuse of NNPF definition for new purpose | Y. Lim (Samsung) |
| JVET-AE0280 | m64502 | 2023-07-13 18:11:37 | 2023-07-13 18:24:27 | 2023-07-16 14:50:03 | AHG9: Common text for proposed generative face video SEI message | B. Chen, J. Chen, Y. Ye (Alibaba), S. Wang (CityU), S. McCarthy, P. Yin, G.-M. Su, A. K. Choudhury, W. Husak (Dolby), |
| JVET-AE0281 | m64507 | 2023-07-14 02:37:50 | 2023-07-14 09:20:33 | 2023-07-14 10:16:18 | Unified LOP filter design, training procedure and filter usage | D.Rusanovskyy, Y. Li, M.Karczewicz (Qualcomm), T. Shao, P. Yin, S. McCarthy (Dolby), J. N. Shingala, A. Shyam (Ittiam), J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang (Bytedance) |
| JVET-AE0282 | m64509 | 2023-07-14 11:32:52 | 2023-07-14 11:35:24 | 2023-07-14 11:35:24 | BoG report on NNVC work planning | E. Alshina, F. Galpin |
| JVET-AE0283 | m64513 | 2023-07-14 16:03:29 | 2023-07-14 16:15:02 | 2023-07-14 16:15:02 | Cross check of JVET-AE0114 (AHG10: Low-Delay configuration improvements) | S. Hong (Nokia) |
| JVET-AE0284 | m64515 | 2023-07-15 09:30:56 | 2023-07-15 10:34:44 | 2023-07-15 10:34:44 | Crosscheck of JVET-AE0122 (AHG10: GOP-based RPR and scalable coding) | D. Arai, S. Nemoto, S. Iwamura, A. Ichigaya (NHK) |
| JVET-AE0285 | m64516 | 2023-07-15 10:45:22 | 2023-07-16 09:01:50 | 2023-07-17 16:59:27 | AHG4: Preparation for content layering functional test | S. Iwamura, D. Arai, S. Nemoto, A. Ichigaya (NHK) |
| JVET-AE0286 | m64517 | 2023-07-15 11:22:57 | 2023-07-16 09:17:38 | 2023-07-16 09:17:38 | Crosscheck of JVET-AE0111 (Additional conformance tests of spatial scalability for multilayer coding) | S. Iwamura (NHK) |
| JVET-AE0287 | m64521 | 2023-07-16 03:47:21 | 2023-07-16 04:06:13 | 2023-07-16 04:06:13 | Non-EE2: A combination of JVET-AE0074 and JVET-AE0170 | K. Zhang, Z. Deng, L. Zhang (Bytedance), H. Huang, Y. Yu, H. Yu, D. Wang (OPPO) |
| JVET-AE0288 | m64522 | 2023-07-16 09:03:54 | 2023-07-17 10:32:42 | 2023-07-17 10:32:42 | Results of expert viewing for the spatial scalability category of the VVC multilayer VT | M. Wien (RWTH Aachen University), K. Andersson (Ericsson), P. de Lagrange (InterDigital) |
| JVET-AE0289 | m64523 | 2023-07-16 09:29:03 | 2023-07-17 14:50:42 | 2023-07-26 17:52:25 | AhG11: HOP training process and models | F. Galpin (InterDigital), S. Eadie, D. Rusanovskyy, Y. Li (Qualcomm), Y. Li, J. Li (ByteDance), L. Wang, R. Chang (Tencent), Z. Xie (Oppo), E. Alshina (Huawei) |
| JVET-AE0290 | m64524 | 2023-07-16 09:29:18 | 2023-07-17 17:37:26 | 2023-07-17 17:37:26 | Crosscheck of JVET-AE0287 (Non-EE2: A combination of JVET-AE0074 and JVET-AE0170) | Z. Lv (vivo) |
| JVET-AE0291 | m64527 | 2023-07-16 12:40:27 | 2023-07-17 14:18:46 | 2023-07-17 14:18:46 | AhG11: Performance of the NNVC HOP with quantized Stage II model | D. Rusanovskyy, F. Galpin |
| JVET-AE0292 | m64528 | 2023-07-16 14:48:05 | 2023-07-24 02:12:04 | 2023-07-24 02:12:04 | Crosscheck of JVET-AE0098 (AHG12: On the chroma DBV mode) | M. Hong (LGE) |
| JVET-AE0293 | m64533 | 2023-07-16 17:39:49 | 2023-07-16 17:47:29 | 2023-07-18 06:48:19 | Cross-check of JVET-AE0143: [AHG8] A spatial resampling algorithm and an exemplar software implementation | Z. Huang (PKU) |
| JVET-AE0294 | m64547 | 2023-07-17 13:06:26 | 2023-07-17 13:22:14 | 2023-07-17 14:08:17 | Film Grain BOG Report | W. Husak |
| JVET-AE0295 | m64550 | 2023-07-17 16:36:03 | 2023-07-17 17:36:00 | 2023-07-17 17:36:00 | [AHG5] HEVC Multiview Profiles: Implementation Status | S. Paluri, D. Podborksi, A. Tourapis (Apple) |
| JVET-AE0296 | m64551 | 2023-07-17 17:08:04 | 2023-07-18 08:31:38 | 2023-07-18 12:04:29 | HEVC Multiview profiles supporting extended bit depth (Draft 2) | S. Paluri, A. Tourapis (Apple), W. Husak(Dolby) |
| JVET-AE0297 | m64552 | 2023-07-17 17:08:31 | 2023-07-19 10:50:09 | 2023-07-19 10:50:09 | Crosscheck of JVET-AE0143 ([AHG8] A spatial resampling algorithm and an exemplar software implementation) | C. Hollmann (Ericsson) |
| JVET-AE0298 | m64563 | 2023-07-18 11:40:16 | 2023-07-18 12:03:20 | 2023-07-18 12:03:20 | AHG9: Combined text of JVET-AE0052 and JVET-AE0063 | M. M. Hannuksela, F. Cricri (Nokia), L. Chen, Y.-W. Huang, S. Lei (MediaTek) |
| JVET-AE0299 | m64655 | 2023-07-26 20:34:05 |  |  | Withdrawn |  |
| JVET-AE0300 | m64656 | 2023-07-27 02:48:36 |  |  | Withdrawn |  |
| JVET-AE1000 | m64589 | 2023-07-19 12:27:57 |  |  | Meeting Report of the 31st JVET Meeting | J.-R. Ohm |
| JVET-AE1004 | m64590 | 2023-07-19 12:30:06 | 2023-07-26 21:08:21 | 2023-07-26 21:08:21 | Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP | Y.-K. Wang, B. Bross, I. Moccagatta, C. Rosewarne, G. J. Sullivan |
| JVET-AE1006 | m64591 | 2023-07-19 12:34:33 |  |  | New profiles, colour decriptors, and SEI messages for HEVC (draft 1) | B. Bross, T. Ikai, G. Sullivan, A. Tourapis, Y.-K. Wang |
| JVET-AE1011 | m64592 | 2023-07-19 12:35:20 |  |  | HEVC multiview profiles supporting extended bit depth (draft 2) | S. Paluri, W. Husak, A. Tourapis |
| JVET-AE1013 | m64593 | 2023-07-19 12:35:46 |  |  | Common test conditions of 3DV experiments | K. Sühring, M. Wien |
| JVET-AE1016 | m64594 | 2023-07-19 12:37:20 |  |  | AVC with extensions and corrections (draft 1) | B. Bross, T. Ikai, G. Sullivan, A. Tourapis, Y.-K. Wang |
| JVET-AE2005 | m64595 | 2023-07-19 12:37:50 | 2023-08-11 15:53:12 | 2023-08-11 15:53:12 | New level and systems-related supplemental enhancement information for VVC (Draft 6) | E. François, B. Bross, M. M. Hannuksela, A. Tourapis, Y.-K. Wang |
| JVET-AE2006 | m64596 | 2023-07-19 12:38:14 | 2023-07-21 01:34:26 | 2023-07-21 01:34:26 | Additional SEI messages for VSEI (Draft 5) | S. McCarthy, T. Chujoh, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang |
| JVET-AE2016 | m64597 | 2023-07-19 12:47:08 | 2023-08-01 02:25:42 | 2023-08-01 02:25:42 | Common test conditions and evaluation procedures for neural network-based video coding technology | E. Alshina, R.-L. Liao, S. Liu, A. Segall |
| JVET-AE2017 | m64598 | 2023-07-19 12:47:30 | 2023-08-01 01:33:51 | 2023-08-01 01:33:51 | Common test conditions and evaluation procedures for enhanced compression tool testing | M. Karczewicz, Y. Ye |
| JVET-AE2019 | m64599 | 2023-07-19 12:47:55 |  |  | Description of algorithms and software in neural network-based video coding (NNVC) version 4 | F. Galpin, Y. Li, D. Rusanovskyy, J. Ström, L. Wang |
| JVET-AE2020 | m64600 | 2023-07-19 12:49:15 |  |  | Film grain synthesis technology for video applications (Draft 5) | D. Grois, Y. He, W. Husak, P. de Lagrange, A. Norkin, M. Radosavljević, A. Tourapis, W. Wan |
| JVET-AE2021 | m64601 | 2023-07-19 12:50:08 |  |  | Verification test plan for VVC multilayer coding (update 1) | S. Iwamura, P. de Lagrange, M. Wien |
| JVET-AE2023 | m64555 | 2023-07-17 18:44:20 | 2023-07-17 18:46:31 | 2023-08-10 13:55:44 | Exploration Experiments on Neural Network-based Video Coding (EE1) | E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang, Z. Xie |
| JVET-AE2024 | m64564 | 2023-07-18 12:01:26 | 2023-07-18 12:10:27 | 2023-07-18 12:10:27 | Exploration experiment on enhanced compression beyond VVC capability (EE2) | V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang |
| JVET-AE2025 | m64602 | 2023-07-19 12:51:07 |  |  | Algorithm description of Enhanced Compression Model 10 (ECM 10) | M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang |
| JVET-AE2027 | m64603 | 2023-07-19 12:51:33 |  |  | SEI processing order SEI message in VVC (draft 5) | S. McCarthy, M. M. Hannuksela, Y.-K. Wang |
| JVET-AE2028 | m64604 | 2023-07-19 12:52:02 |  |  | Additional conformance bitstreams for VVC multilayer configurations | S. Iwamura, P. de Lagrange, I. Moccagatta |
| JVET-AE2030 | m64605 | 2023-07-19 12:52:48 |  |  | Optimization of encoders and receiving systems for machine analysis of coded video content (draft 3) | J. Chen, C. Hollmann, S. Liu |
| JVET-AE2031 | m64606 | 2023-07-19 12:53:12 | 2023-08-11 09:13:45 | 2023-08-11 09:13:45 | Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content | S. Liu, C. Hollmann |
| JVET-AE2032 | m64607 | 2023-07-19 12:53:40 |  |  | Technologies under consideration for future extensions of VSEI (draft 1) | M. M. Hannuksela, J. Chen, S. Deshpande, S. McCarthy |

# Annex B1 to JVET report:
List of meeting participants attending in person

The participants who were personally present at the meeting site of the thirty-first meeting of the JVET, according to the registration with ITU-T for badge pickup (approximately 191 people in total), were as follows:

- Mohsen Abdoli (IRT b-com)

- Kiyofumi Abe (Panasonic)

- Yongjo Ahn (LG Electronics)

- Elena Alshina (Huawei)

- Alireza Aminlou (Nokia)

- Kenneth Andersson (Ericsson)

- Pierre Andrivon (Ofinno)

- Muhammet Balcilar (InterDigital)

- Junho Bang (Hanbat Nat. Univ.)

- Saverio Blasi (Nokia)

- Mederic Blestel (Ofinno)

- Charles Bonnineau (InterDigital)

- Frank Bossen (Bossen Tech.)

- Allan Brahim (BT Plc)

- Benjamin Bross (Fraunhofer HHI & IIS)

- Madhukar Budagavi (Samsung)

- Joohyung Byeon (Kwangwoon Univ.)

- Simão Campos Neto (ITU-T)

- Chih-Yuan Chen (FG Innovation)

- Chun-Chi Chen (Qualcomm)

- Hong Hui Chen (MediaTek)

- Jie Chen (Alibaba)

- Lulin Chen (MediaTek)

- Roman Chernyak (Tencent)

- Giyong Choi (Samsung)

- Haechul Choi (Hanbat Nat. Univ.)

- Hansol Choi (Kwangwoon Univ.)

- Kiho Choi (Kyung Hee Univ.)

- Kwang Pyo Choi (Samsung)

- Minseok Choi (Hanbat Nat. Univ.)

- Keiichi Chono (NEC)

- Chie Ming Chou (FG Innovation)

- Olena Chubach (MediaTek)

- Takeshi Chujoh (Sharp)

- Tim Claßen (RWTH Aachen Univ.)

- Kai Cui (Qualcomm)

- Arai Daichi (NHK)

- Philippe de Lagrange (InterDigital)

- Sachin Deshpande (Sharp)

- Ding Ding (Tencent)

- Tianyu Dong (Hanyang Univ.)

- Alberto Duenas (HBO - Time Warner)

- Jack Enhorn (Ericsson)

- Alexey Filippov (ICP)

- Per Frojdh (Ericsson)

- Franck Galpin (InterDigital)

- Jonathan Gan (OPPO)

- Jingying Gao (Panasonic)

- Dan Grois (Comcast)

- Heeji Han (Hanbat Nat. Univ.)

- Miska Matias Hannuksela (Nokia)

- Myungoh Hong (LG Electronics)

- Chih-wei Hsu (MediaTek)

- Yali Hu (ZTE)

- Hang Huang (OPPO)

- Yu-Wen Huang (MediaTek)

- Walt Husak (Dolby)

- Kenta Iida (NEC)

- Tomohiro Ikai (Sharp)

- Masaru Ikeda (Sony)

- Takaaki Ishikawa (Canon)

- Shunsuke Iwamura (NHK)

- Byeungwoo Jeon (Sungkyunkwan Univ.)

- Sanghun Jeon (Korea Aerospace Univ.)

- Rajan Joshi (Samsung)

- Kashyap Kammachi Sreedhar (Nokia)

- Jaeha Kang (Hanbat Nat. Univ.)

- Jewon Kang (Ewha Womans Univ.)

- Marta Karczewicz (Qualcomm)

- Kei Kawamura (KDDI)

- Steve Keating (Sony)

- Yoshitaka Kidani (KDDI)

- Chulkeun Kim (LG Electronics)

- Donghyun Kim (ETRI)

- Jiyoung Kim (Hanbat Nat. Univ.)

- Jongho Kim (ETRI)

- Jungsun Kim (Apple)

- Nam Uk Kim (Mediaexcel)

- Seonjae Kim (Dong-A Univ.)

- Seung-Hwan Kim (LG Electronics)

- Konstantinos Konstantinides (Dolby)

- Moonmo Koo (LG Electronics)

- Jani Lainema (Nokia)

- Pierre Larbier (ATEME)

- Guillaume Laroche (Canon)

- Fabrice Le Léannec (InterDigital)

- Jungkyung Lee (Ewha Womans Univ.)

- Minhun Lee (Kwangwoon Univ.)

- Taesik Lee (Dong-A Univ.)

- Yooho Lee (Dong-A Univ.)

- Binzhe Li (City Univ. HK)

- Xiang Li (Google)

- Xinwei Li (Alibaba)

- Yun Li (Qualcomm)

- Kai-Wen Liang (FG Innovation)

- Jaehyun Lim (LG Electronics)

- Sung-Chang Lim (ETRI)

- Sungwon Lim (KT Corp.)

- Woong Lim (ETRI)

- Youngkwon Lim (Samsung)

- Po Han Lin (Qualcomm)

- Lukasz Litwic (Ericsson)

- Du Liu (Ericsson)

- Shan Liu (Tencent)

- Zhuoyi Lv (Vivo)

- Sean McCarthy (Dolby)

- Dominik Mehlem (RWTH))

- Akira Minezawa (Mitsubishi)

- Gihwa Moon (Korea Aerospace Univ.)

- Kwon Naseong (Kwangwoon Univ.)

- Karam Naser (InterDigital)

- Shimpei Nemoto (NHK)

- Thi Hong Nhung Nguyen (Canon)

- Didier Nicholson (Ektacom)

- Pavel Nikitin (Qualcomm)

- Keisuke Nonaka (KDDI)

- Andrey Norkin (Netflix)

- Jens-Rainer Ohm (RWTH Aachen Univ.)

- Patrice Onno (Canon)

- Jeeyoon Park (Samsung)

- Min Woo Park (Samsung)

- Seanae Park (Kwangwoon Univ.)

- Jonathan Pfaff (Fraunhofer HHI & IIS)

- Pierrick Philippe (Orange)

- Yinji Piao (Samsung)

- Fangjun Pu (Dolby)

- Fabien Racapé (InterDigital)

- Justin Ridge (Nokia)

- Hyungmin Roh (Samsung)

- Christopher Rosewarne (Canon)

- Vasily Rufitskiy (Ofinno)

- Dmytro Rusanovskyy (Qualcomm)

- Jonatan Samuelsson-Allendes (Sharp)

- Yago Sanchez de la Fuente (Fraunhofer HHI & IIS)

- Maria Santamaria (Nokia)

- Johannes Sauer (Huawei)

- Heiko Schwarz (Fraunhofer HHI & IIS)

- Vadim Seregin (Qualcomm)

- Vladislav Shchukin (Ericsson)

- Masato Shima (Canon)

- Juyeon Shin (Ewha Womens Univ.)

- Jay Nitin Shingala (Ittiam)

- Anubhav Singh (Samsung)

- Jacob Ström (Ericsson)

- Karsten Sühring (Fraunhofer HHI & IIS)

- Gary J. Sullivan (Dolby)

- Teruhiko Suzuki (Sony Group)

- Hendry (LG Electronics)

- Chih-Yu Teng (FG Innovation)

- Han Boon Teo (Panasonic)

- Alexandros Tourapis (Apple)

- Hsinyi Tseng (MediaTek)

- Nikolay Tverdokhleb (Ateme)

- Kyohei Unno (KDDI)

- Gleb Verba (Qualcomm)

- Dong Wang (OPPO)

- Fan Wang (OPPO)

- Hanming Wang (China Telecom)

- Huifen Wang (China Telecom)

- Sheng-Po Wang (ITRI)

- Shurun Wang (Alibaba)

- Ahmad Waqas (Ericsson)

- Stephan Wenger (Tencent)

- Adam Wieckowski (Fraunhofer HHI & IIS)

- Mathias Wien (RWTH Aachen Univ.)

- Dongjae Won (Mediaexcel)

- Ping Wu (ZTE)

- Zijun Wu (China Telecom)

- Zhihuang Xie (OPPO)

- Jizheng Xu (Bytedance)

- Luhang Xu (OPPO)

- Yingzhan Xu (MIIT)

- Ruiying Yang (Nokia)

- Yu Chiao Yang (FG Innovation)

- Yukinobu Yasugi (Sharp)

- Yan Ye (Alibaba)

- Peng Yin (Dolby)

- Wenbin Yin (MIIT)

- Yong-Uk Yoon (Korea Aerospace Univ.)

- Haoping Yu (OPPO)

- Lu Yu (Zhejiang Univ.)

- Yue Yu (OPPO)

- Lai Zhang (OPPO)

- Yajun Zhang (Tencent)

- Yuan Zhang (China Telecom)

- Zhi Zhang (Qualcomm)

- Xin Zhao (Tencent)

- Fan Zheming (Sharp)

- Dai Zhenyu (OPPO)

- Chuan Zhou (Vivo)

- Nannan Zou (Nokia)

# Annex B2 to JVET report:
List of meeting participants attending remotely

The remote participants of the thirtieth meeting of the JVET, according to the participation records from the Zoom teleconferencing tool used for the meeting sessions (approximately 125 people in total, not including those who had attended the meeting in person at least part-time (see annex B1), and not including those who attended only the joint sessions with other groups), were as follows:

- xJukka Ahonen (Nokia – FI)

- Hussein Akbarzadeh (INSO – IR)

- Jeeva Raj Arumugam (Ittiam – IN)

- Pekka Astola (Nokia – FI)

- Tae Meon Bae (Sharp – US)

- Robbie Bancroft (BBC – UK)

- Weijie Bao (WHU – CN)

- Guillaume Boisson (InterDigital – FR)

- Philippe Bordes (InterDigital – FR)

- Adrian Browne (Sony – JP)

- Döne Bugdayci Sansli (Nokia – FI)

- Eric Chai (Ubilinx – US)

- Renjie Chang (Tencent – CN)

- Bolin Chen (Alibaba – CN)

- Man-Shu Chiang (MediaTek – US)

- Chih-Yao Chiu (MediaTek – US)

- Hyon-Gon Choo (ETRI – KR)

- Cheng-Yen Chuang (Mediatek – US)

- Tzu-Der Chuang (MediaTek – US)

- Muhammed Coban (Qualcomm – US)

- Francesco Cricri (Nokia – FI)

- Zhipin Deng (Bytedance – CN)

- Quockhanh Dinh (Samsung – KR)

- Didier Doyen (InterDigital – FR)

- Thierry Dumas (Interdigital – FR)

- Samuel Eadie (Qualcomm – US)

- Felix Fleckenstein (Fraunhofer IIS – DE)

- Chad Fogg (MovieLabs – US)

- Edouard François (InterDigital – FR)

- Jiaye Fu (PKU – CN)

- Patrick Garus (Qualcomm – US)

- Thomas Guionnet (Ateme – FR)

- Dong-Gyu Gwak (LGE – KR)

- Gyu-Woong Han (KAU – KR)

- Yong He (Qualcomm – US)

- Jin Heo (Hyundai – KR)

- Christopher Hollmann (Ericsson – SE)

- Shih-Ta Hsiang (MediaTek – US)

- Yuling Hsiao (MediaTek – US)

- Nan Hu (Qualcomm – US)

- Cheng Huang (ZTE – CN)

- Junyan Huo (Xidian Univ. – CN)

- Yongkai Huo (Univ. Southhampton – UK)

- Venkatesh Jatla (MediaTek – US)

- Hyunki Jeong (SKKU – KR)

- Hong-Jheng Jhu (Kwai – US)

- Tianying Ji (Sharp – US)

- Chuanmin Jia (PKU – CN)

- Ju-Hyung John (WILUS – CN)

- Ikram Jumakulyyev (Qualcomm – US)

- Kimihiko Kazui (Fujitsu – JP)

- Carlo Kneissl (Qualcomm – US)

- Ryota Koiso (KDDI – JP)

- Hiroki Kojima (KDDI – JP)

- Jacek Konieczny (TCL Research – PL)

- Madhu Krishnan (Tencent – US)

- Gosala Kulupana (BBC – UK)

- Che-Wei Kuo (Kwai – US)

- Hyukmin Kwon (HYU – KR)

- Chen-Yen Lai (MediaTek – US)

- Nam Le (Nokia – FI)

- Brian Lee (Dolby – US)

- Jin Young Lee (SJU – KR)

- Frédéric Lefebvre (InterDigital – FR)

- Ru-Ling Liao (Alibaba – CN)

- Chaoyi Lin (Bytedance – CN)

- Chih-Hsuan Lo (MediaTek – US)

- Federico Lo Bianco (InterDigital – FR)

- Taoran Lu (Dolby – US)

- Ajay Luthra (Picsel Labs – US)

- Changyue Ma (Kwai – US)

- Ismail Marzuki (InterDigital – CA)

- Wenhui Meng (WHU – CN)

- Philipp Merkle (HHI – DE)

- Koohyar Minoo (IR)

- Iole Moccagatta (Inte – US)

- Junghak Nam (LGE – KR)

- Rose Nguyen (Canon – AU)

- Seethal Paluri (Apple – US)

- Krit Panusopone (Nokia – US)

- Min Woo Park (Samsung – KR)

- Shuang Peng (Dahua – CN)

- Saurabh Puri (InterDigital – CA)

- Hongdong Qin (TCL – CN)

- Milos Radosavljević (InterDigital – FR)

- M. Rafie (vivo – US)

- Gagan Rath (InterDigital – FR)

- Bappaditya Ray (Qualcomm – US)

- Kevin Reuzé (InterDigital – FR)

- Antoine Robert (InterDigital – FR)

- Damian Ruiz Coll (Ofinno – US)

- Changwoo Ryu (Kaon – KR)

- Mehdi Salehifar (Bytedance – US)

- Charles Salmon-Legagneur (InterDigital – FR)

- Andrew Segall (Amazon – US)

- Cagan Selim Coban (Qualcomm – US)

- Juyeon Seo (HYU – KR)

- Tong Shao (Dolby – US)

- Ahmed Sidiya (Sharp – US)

- Yasser Syed (Comcast – US)

- Keiichiro Takada (Sharp – JP)

- Minhao Tang (Tencent – CN)

- Herbert Thoma (Fraunhofer IIS – DE)

- Jing Yuan Thong (Panasonic – SG)

- Chia-Ming Tsai (MediaTek – US)

- Fabrice Urban (InterDigital – FR)

- Renan Utida (InterDigital – CA)

- Biao Wang (Tencent – US)

- Sheng Po Wang (ITRI – US)

- Martin Winken (HHI – DE)

- Samuel Wong (INTEL – US)

- Shaowei Xie (ZTE – CN)

- Alisa Xing (Hisense – CN)

- Xiaoyu Xiu (Kwai – US)

- Yoichi Yagasaki (Sony – JP)

- Ning Yan (Kwai – US)

- Huei-Jiun Yang (ITRI – US)

- Jiedong Ye (HUST – CN)

- Ramin Youvalari (Nokia – FI)

- Weimin Zeng (Ubilinx – US)

- Han Zhang (Tencent – US)

- Jane Zhao (LGE – US)

- Zhou Zhi (TCL – CN)

- Yiqing Zhu (HUST – CN)

- Ivan Zupancic (Nokia – DE)

# Annex C to JVET report:
Recommendations of the 12th meeting of
ISO/IEC JTC 1/SC 29/WG 5 MPEG Joint Video Experts Team with ITU-T SG 16

ISO/IEC JTC 1/SC 29/WG 5 N 215

## 1. Reports

### 1.1 Meeting reports

### 1.1.1 WG 5 approves the following document

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
| 196 | Report of the 11th JTC 1/SC 29/WG 5 meeting | Jens-Rainer Ohm | N | 2023-05-26 | 22700 |

## 2. MPEG-4 (ISO/IEC 14496 - Coding of audio-visual objects)

### 2.1 Part 10 - Advanced video coding

### 2.1.1 WG 5 recommends approval of the following documents

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | ISO/IEC 14496-10 - Advanced video coding |  |  |  |  |
| 217 | Request for ISO/IEC 14496-10 11th edition | Gary Sullivan | N | 2023-07-21 | 22868 |
| 218 | Text of ISO/IEC CD 14496-10:202x Advanced video coding (11th edition) | Gary Sullivan | Y | 2023-08-11 | 22869 |

## 3. MPEG-C (ISO/IEC 23002 – MPEG video technologies)

### 3.1 Part 7 – Versatile supplemental enhancement information messages for coded video bitstreams

### 3.1.1 WG 5 recommends approval of the following documents

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | ISO/IEC 23002-7 – Versatile supplemental enhancement information messages for coded video bitstreams |  |  |  |  |
| 219 | Disposition of comments received on ISO/IEC 23002-7:2022 DAM 1 | Gary Sullivan | N | 2023-07-21 | 22870 |
| 220 | Text of ISO/IEC FDIS 23002-7:202x Versatile supplemental enhancement information messages for coded video bitstreams (3rd edition) | Gary Sullivan | N | 2023-09-15 | 22871 |
| 221 | Technologies under consideration for future extensions of VSEI | Sean McCarthy | Y | 2023-09-15 | 22872 |

| ### 3.1.2 |  | ### WG 5 requests to make ISO/IEC 23002-7:202x (3rd ed.) publicly available, anticipating public availability elsewhere of a corresponding twin text by ITU-T. |
| --- | --- | --- |

### 3.2 Part 9 – Film grain synthesis technology for video applications

### 3.2.1 WG 5 recommends approval of the following documents

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | ISO/IEC 23002-9 - Film grain synthesis technology for video applications |  |  |  |  |
| 222 | Disposition of comments received on ISO/IEC CDTR 23002-9 | Gary Sullivan | N | 2023-07-21 | 22873 |
| 223 | Text of ISO/IEC DTR 23002-9 Film grain synthesis technology for video applications | Walt Husak | N | 2023-09-08 | 22874 |

| ### 3.2.2 |  | ### WG 5 thanks Apple for providing test materials that can be used in assessing and developing technology in the context of film grain synthesis applications. |
| --- | --- | --- |

## 4. MPEG-AI (MPEG artificial intelligence)

### 4.1 Part 2 – Optimization of encoders and receiving systems for machine analysis of coded video content

### 4.1.1 WG 5 recommends approval of the following document

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | ISO/IEC TBD-2 – Optimization of encoders and receiving systems for machine analysis of coded video content |  |  |  |  |
| 224 | Preliminary working draft 3 of TR: Optimization of encoders and receiving systems for machine analysis of coded video content | Christopher Hollmann | Y | 2023-09-15 | 22875 |

## 5. MPEG-H (ISO/IEC 23008 – High efficiency coding and media delivery in heterogeneous environments)

### 5.1 Part 2 – High efficiency video coding

### 5.1.1 WG 5 recommends approval of the following document

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | ISO/IEC 23008-2 – High efficiency video coding |  |  |  |  |
| 225 | Request for ISO/IEC 23008-2:202x/Amd.1 | Gary Sullivan | N | 2023-07-21 | 22876 |
| 226 | Text of ISO/IEC 23008-2:202x (5th ed.) CDAM1 New profiles and SEI messages | Gary Sullivan | Y | 2023-08-11 | 22877 |

| ### 5.1.2 |  | ### Following up on recommendation 4.1.2 of the 11th meeting, WG 5 thanks the following companies and organizations for agreeing (by written or verbal statement, or by abstaining from any disagreement) to continued usage of test materials that were originally provided to JCT-3V: GIST, Nagoya University, NICT, Nokia, and Poznan University of Technology. As successor of JCT-3V in maintaining the legacy standards, JVET has updated the common test conditions (CTC) document accordingly. As far as updated usage conditions were received, these will be updated when making the materials accessible for members of the ITU-T and ISO/IEC standardization communities. |
| --- | --- | --- |

## 6. MPEG-I (ISO/IEC 23090 – Coded representation of immersive media)

### 6.1 Part 3 – Versatile video coding

### 6.1.1 WG 5 recommends approval of the following documents

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | ISO/IEC 23090-3 – Versatile video coding |  |  |  |  |
| 227 | Disposition of comments received on ISO/IEC 23090-3:2022 DAM 1 | Gary Sullivan | N | 2023-07-21 | 22878 |
| 228 | Text of ISO/IEC FDIS 23090-3:202x Versatile video coding (3rd edition) | Gary Sullivan | N | 2023-09-15 | 22879 |
| 229 | Preliminary working draft 5 of SEI processing order SEI message in VVC | Sean McCarthy | Y | 2023-09-01 | 22880 |

| ### 6.1.2 |  | ### WG 5 requests to make ISO/IEC 23090-3:202x (3rd ed.) publicly available, anticipating public availability elsewhere of a corresponding twin text by ITU-T. |
| --- | --- | --- |
| ### 6.1.3 |  | ### WG 5 requests additional support in financing or providing resources for the upcoming series of verification tests on multi-layer VVC coding, considering the excellent opportunity for demonstrating the superiority of VVC functionality over previous standards. |

| ### 6.1.4 |  | ### WG 5 welcomes participation in and contribution of resources and test material to the development of the dual-view (stereo) category in the verification test plan for VVC multilayer coding. In particular, 10-bit content with resolution higher than HD would be desirable. |
| --- | --- | --- |

## 7. Explorations

### 7.1 Neural network-based video coding

### 7.1.1 WG 5 recommends approval of the following documents

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | Explorations |  |  |  |  |
| 230 | Exploration experiment on neural network-based video coding (EE1) | Elena Alshina | Y | 2023-08-11 | 22881 |
| 231 | Description of algorithms and software in neural network-based video coding (NNVC) version 4 | Franck Galpin | Y | 2023-09-01 | 22882 |

### 7.2 Enhanced compression beyond VVC capability

### 7.2.2 WG 5 recommends approval of the following documents

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | Explorations |  |  |  |  |
| 232 | Exploration experiment on enhanced compression beyond VVC capability (EE2) | Vadim Seregin | Y | 2023-08-18 | 22883 |
| 233 | Algorithm description of enhanced compression model 10 (ECM 10) | Muhammed Coban | Y | 2023-09-29 | 22884 |

## 8. Management

### 8.1 Collaboration with ITU-T

| ### 8.1.1 |  | ### The JVET chair proposes to hold the 32nd JVET meeting during Fri. 13 – Fri. 20 October 2023 (with contribution deadline Fri. 6 Oct.) under SC 29 auspices in Hannover, DE; during Wed. 17 – Fri. 26 January 2024 under SC 29 auspices, to be conducted as a teleconference meeting; during Fri. 19 – Fri. 26 April 2024 under ITU-T SG16 auspices in Rennes, FR; during Fri. 12 – Fri. 19 July 2024 under SC 29 auspices in Sapporo, JP; during October 2024 under SC 29 auspices, exact date and location t.b.d.; during January 2025 under ITU-T SG16 auspices, exact date and location t.b.d.; during April 2025 under SC 29 auspices, exact date and location t.b.d.; and during Thu. 26 June – Fri. 4 July 2025 under SC 29 auspices in Daejeon, KR. |
| --- | --- | --- |

### 8.2 Liaisons

### 8.2.1 WG 5 recommends approval of the following liaison statement

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | Liaisons |  |  |  |  |
| 234 | Liaison statement to SC 29/WG 1 (JPEG) on JPEG AI, neural network-based video coding, and SEI messages for neural network-based post filtering | Gary Sullivan | N | 2023-07-21 | 22885 |

### 8.3 Ad hoc groups

### 8.3.1 WG 5 approves the following document

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | Ad hoc groups |  |  |  |  |
| 235 | List of AHGs established at the 12th WG 5 meeting | Jens-Rainer Ohm | N | 2023-07-21 | 22886 |

### 8.4 Expression of thanks

| ### 8.4.1 |  | ### WG 5 thanks Marius Preda for his support in maintaining the document site jvet-experts.org, as well as the document sites of JCT-VC and JCT-3V. Institut Mines-Télécom is thanked for hosting the sites. |
| --- | --- | --- |
| ### 8.4.2 |  | ### WG 5 thanks Simão Campos for the help in setting up the 31st JVET meeting in Geneva. The following members of ITU staff are thanked for their dedication and continuous help in the technical setup of meeting facilities: Gent Bajrami, Marc Antoine Zanou, Ilia Londo, and Paul Marko. |

The meeting was closed at 1215 CEST on 2023-07-19.