| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0006-v2 |
| --- | --- |

| Title: | JVET AHG report: ECM software development (AHG6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Fabrice Le Léannec Kai Zhang | Tel: Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com Fabrice.Leleannec@InterDigital.com zhangkai.video@bytedance.com |
| Source: | AHG6 on ECM software development |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AHG6 on ECM software development that has taken place between the 30th and 31st JVET meetings.

# Introduction

The mandates for the AHG are as follows:

- Coordinate development of the ECM software and associated configuration files.

- Produce documentation of software usage for distribution with the software.

- Prepare and deliver ECM-9.0 software version and the reference configuration encodings according to the ECM common test conditions.

- Investigate encoder speedup and other encoder software optimization.

- Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate.

# Software development

ECM software repository is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM.

ECM software is based on VTM-10.0 with enabled MCTF including the update from JVET-V0056, and GOP32, which is very close to VTM-11.0.

VTM-11.0ecm anchor https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/VTM11_ANC is used for ECM performance evaluation.

The following adopted aspects were integrated into ECM-9.0:

JVET-AD0202 (EE2-1.2): CCCM with multiple downsampling filters (MR 399)

JVET-AD0208 (EE2 Test-1.8c): IBC adaptation for coding of natural content (MR 405)

JVET-AD0195: High-Precision MV Refinement for BDOF (MR 406)

JVET-AD0222 (EE2-4.4b): ALF: additional fixed filter, residual-based classifier, increase the number of taps that are applied to the fixed filter results (MR 402)

JVET-AD0206: CABAC init at GDR picture (MR 407)

JVET-AD0123 (EE2 Test 5.1): reference frame padding for GDR (MR 409)

JVET-AD0193 (EE2 Test 2.11e): Adaptive OBMC control (MR 408)

JVET-AD0213 (EE2-Test2.7b): bi-predictive LIC and the combination of LIC and OBMC (MR 404)

JVET-AD0082: add configuration option for TMRL (MR 401)

JVET-AD0169: Downsampling filters in range 1.1 to 1.35 based on Kaiser(7) windowed sinc (MR 412)

JVET-AD0184: Removal of Division Operations (MR 413)

JVET-AD0085: Combinations of MPM related tests (MR 400)

JVET-AD0086 (EE2-1.20j combination test): enhanced intra TMP (MR 415)

JVET-AD0188 (EE2 Test-1.6c): Non-local cross-component prediction and cross-component merge mode (MR 418)

JVET-AD0105 (Aspect1): Cap the maximum NumSignPred to 4 for QP<=22 (AI and RA) (MR 403)

JVET-AD0182(EE2-2.1l): Affine Parameter refinement and mode extension (MR 419)

JVET-AD0239/AD0124: Fixes for IntraTMP template availability (MR 421)

JVET-AD0120 (EE2 Test 1.3e): Local-Boosting Cross-Component Prediction (MR 411)

JVET-AD0140: MVD (sign and suffix) prediction, test 2.12a (MR 424)

Bug fixes:

Revert MR383 and fix (MR 396)

Fix #45 on IBC AMVR off (MR 397)

The following adopted aspects were integrated into VTM-11ecm anchor:

JVET-AD0169: Downsampling filters in range 1.1 to 1.35 based on Kaiser(7) windowed sinc (MR 422)

It does not change VTM-11ecm anchor performance under CTC.

ECM-9.0 and VTM-11ecm9.0 were tagged on May 24, 2023.

The following bug fixes were integrated into ECM-9.0:

Fix mismatch with TIMD is 0 (MR 434)

Fix luma offsets in AD0188 (MR 436)

RPR-related fixes for JVET_AD0140_MVD_PREDICTION (MR 435)

CCCM code refactoring (MR 431)

Fix mismatch when --EnableTMTools=0 (MR 438)

Fix crash when --EnableTMTools=0 (MR 440)

Avoid temporary cu copy in EncModeCtrlMTnoRQT::useModeResult (MR 441)

Fix AD0188 for GDR (MR 439)

RPR fixes for JVET_AD0213_LIC_IMP and JVET_AD0140_MVD_PREDICTION (MR 437)

ECM-9.0 was tagged on July 5, 2023.

## CTC Performance

In this section, ECM test results following ECM CTC configuration descried in JVET-AD2017 are summarized.

Next tables show ECM-9.0 performance over ECM-8.1 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.70% | -3.10% | -4.16% | 122.0% | 114.0% |
| Class A2 | -2.48% | -6.26% | -6.55% | 121.9% | 115.1% |
| Class B | -1.91% | -5.16% | -4.85% | 121.8% | 115.6% |
| Class C | -1.16% | -2.24% | -2.41% | 126.2% | 122.0% |
| Class E | -3.50% | -5.92% | -5.33% | 128.6% | 126.1% |
| Overall | -1.90% | -4.48% | -4.56% | 123.9% | 118.3% |
| Class D | -1.20% | -2.49% | -2.55% | 127.9% | 126.2% |
| Class F | -2.80% | -4.30% | -4.39% | 120.8% | 126.2% |
| Class TGM | -2.62% | -4.09% | -4.04% | 120.8% | 137.6% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.22% | -1.88% | -3.64% | 116.3% | 111.1% |
| Class A2 | -1.96% | -3.81% | -3.63% | 117.9% | 109.4% |
| Class B | -1.51% | -3.75% | -3.55% | 125.2% | 110.3% |
| Class C | -1.19% | -1.24% | -1.37% | 126.7% | 117.3% |
| Class E |  |  |  |  |  |
| Overall | -1.46% | -2.72% | -3.00% | 122.3% | 112.1% |
| Class D | -1.40% | -1.18% | -1.79% | 131.8% | 116.7% |
| Class F | -2.53% | -3.40% | -3.91% | 134.4% | 119.7% |
| Class TGM | -2.90% | -4.48% | -4.16% | 119.3% | 113.4% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.78% | -1.87% | -1.75% | 125.4% | 113.1% |
| Class C | -0.99% | -0.35% | -0.65% | 120.0% | 115.8% |
| Class E | -0.83% | -1.10% | -3.12% | 123.6% | 113.0% |
| Overall | -0.86% | -1.17% | -1.73% | 123.1% | 114.0% |
| Class D | -0.75% | -2.08% | -0.28% | 129.7% | 119.9% |
| Class F | -1.72% | -3.46% | -2.95% | 131.6% | 123.5% |
| Class TGM | -2.67% | -3.96% | -3.70% | 119.3% | 113.2% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.41% | -1.24% | -1.57% | 123.1% | 106.7% |
| Class C | -0.52% | -0.42% | -0.80% | 126.5% | 116.1% |
| Class E | -0.85% | -1.96% | -3.58% | 133.4% | 116.8% |
| Overall | -0.56% | -1.15% | -1.81% | 126.7% | 112.2% |
| Class D | -0.52% | -1.77% | -0.13% | 138.7% | 120.6% |
| Class F | -1.19% | -2.47% | -2.56% | 130.1% | 120.6% |
| Class TGM | -2.65% | -4.36% | -3.91% | 114.0% | 108.0% |

The below tables show ECM-9.0 performance comparing to VTM-11.0ecm9.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.67% | -21.16% | -28.19% | 827.7% | 406.4% |
| Class A2 | -15.77% | -27.44% | -30.55% | 813.4% | 405.9% |
| Class B | -10.06% | -25.61% | -23.82% | 773.3% | 423.4% |
| Class C | -10.27% | -15.86% | -16.33% | 783.1% | 442.4% |
| Class E | -13.65% | -23.55% | -22.06% | 759.1% | 481.4% |
| Overall | -11.59% | -22.66% | -23.71% | 788.5% | 430.8% |
| Class D | -8.61% | -13.65% | -13.74% | 803.2% | 462.1% |
| Class F | -24.34% | -33.43% | -33.28% | 529.7% | 445.8% |
| Class TGM | -37.29% | -45.21% | -44.55% | 443.8% | 443.1% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -20.86% | -24.25% | -32.21% | 757.7% | 741.6% |
| Class A2 | -23.90% | -32.20% | -35.28% | 697.8% | 882.0% |
| Class B | -19.50% | -31.71% | -29.62% | 648.2% | 739.3% |
| Class C | -20.90% | -24.61% | -24.58% | 672.8% | 779.1% |
| Class E |  |  |  |  |  |
| Overall | -21.03% | -28.42% | -29.93% | 685.5% | 777.1% |
| Class D | -21.87% | -25.90% | -26.55% | 699.1% | 849.6% |
| Class F | -26.77% | -34.78% | -35.14% | 578.7% | 528.1% |
| Class TGM | -35.17% | -41.37% | -41.35% | 547.2% | 419.4% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -17.01% | -32.11% | -30.48% | 601.4% | 658.9% |
| Class C | -18.50% | -26.42% | -26.63% | 590.5% | 661.3% |
| Class E | -15.40% | -25.47% | -26.14% | 595.7% | 440.1% |
| Overall | -17.10% | -28.56% | -28.11% | 596.3% | 596.4% |
| Class D | -20.39% | -28.41% | -27.33% | 629.6% | 770.9% |
| Class F | -23.36% | -33.28% | -31.66% | 562.9% | 526.4% |
| Class TGM | -33.93% | -40.53% | -40.11% | 511.1% | 404.8% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -15.87% | -31.42% | -29.54% | 537.3% | 625.1% |
| Class C | -17.58% | -25.77% | -25.54% | 521.8% | 661.5% |
| Class E | -14.93% | -24.37% | -25.89% | 579.6% | 462.0% |
| Overall | -16.20% | -27.77% | -27.29% | 542.3% | 590.6% |
| Class D | -20.36% | -28.35% | -27.51% | 563.4% | 739.4% |
| Class F | -22.24% | -33.15% | -31.57% | 541.3% | 486.8% |
| Class TGM | -30.83% | -38.46% | -38.16% | 526.4% | 378.7% |

Next tables show ECM-9.1 performance over ECM-9.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.22% | -0.36% | 101.8% | 105.0% |
| Class A2 | -0.03% | -0.11% | -0.08% | 101.9% | 106.8% |
| Class B | -0.02% | -0.22% | -0.02% | 101.4% | 103.8% |
| Class C | 0.00% | -0.13% | -0.06% | 101.7% | 100.3% |
| Class E | -0.02% | -0.05% | -0.06% | 99.1% | 98.1% |
| Overall | -0.02% | -0.15% | -0.10% | 101.2% | 102.7% |
| Class D | 0.00% | -0.05% | 0.03% | 97.9% | 98.7% |
| Class F | -0.02% | -0.16% | -0.03% | 98.3% | 99.7% |
| Class TGM | -0.03% | -0.06% | -0.04% | 98.3% | 102.9% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.21% | -0.07% | 103.8% | 104.0% |
| Class A2 | -0.02% | -0.21% | -0.09% | 103.7% | 103.8% |
| Class B | 0.00% | -0.16% | -0.07% | 100.5% | 102.2% |
| Class C | -0.01% | -0.12% | -0.02% | 99.1% | 99.7% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.17% | -0.06% | 101.4% | 102.2% |
| Class D | 0.00% | -0.33% | 0.16% | 99.2% | 99.1% |
| Class F | -0.02% | -0.14% | -0.06% | 95.6% | 93.3% |
| Class TGM | 0.01% | 0.04% | -0.03% | 101.4% | 105.5% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.09% | -0.04% | 103.0% | 104.3% |
| Class C | 0.02% | -0.42% | -0.59% | 99.3% | 102.2% |
| Class E | 0.13% | -0.16% | 1.16% | 99.1% | 101.7% |
| Overall | 0.06% | -0.22% | 0.08% | 100.8% | 102.9% |
| Class D | -0.12% | 1.38% | -0.03% | 99.0% | 99.3% |
| Class F | -0.09% | -0.02% | 0.29% | 96.9% | 97.6% |
| Class TGM | -0.01% | 0.08% | -0.06% | 103.4% | 108.5% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.37% | -0.01% | 103.9% | 108.4% |
| Class C | -0.03% | -0.16% | 0.08% | 100.1% | 102.4% |
| Class E | 0.02% | -0.61% | 0.87% | 98.2% | 99.3% |
| Overall | 0.00% | -0.05% | 0.24% | 101.2% | 104.0% |
| Class D | 0.07% | 1.22% | 1.05% | 99.9% | 100.9% |
| Class F | 0.15% | 0.38% | -0.40% | 98.9% | 97.4% |
| Class TGM | -0.07% | -0.06% | -0.16% | 107.8% | 110.8% |

The Excel files with the complete test results are attached to this report.

# ECM memory consumption

ECM memory consumption is provided in ECM encoder log files and is summarized in the table below.

|  | RA | LD | AI |
| --- | --- | --- | --- |
| class A | 25 GB |  | 12 GB |
| class B | 14 GB | 12 GB | 10 GB |
| class C | 6 GB | 6 GB | 6 GB |
| class E |  | 8 GB | 7 GB |
| class D | 5 GB | 5 GB | 5 GB |
| class F | 14 GB | 12 GB | 11 GB |
| class TGM | 16 GB | 14 GB | 11 GB |

It is encouraged to care about memory allocation when developing and integrating tools into ECM. In particular, it is strongly recommended to re-use already allocated memory wherever possible, rather than systematically allocating new memory.

# Recommendations

The AHG recommends to:

- Continue to develop ECM software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues.

- Encourage people to submit merge requests fixing identified bugs.