| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0012-v1 |
| --- | --- |

| Title: | JVET AHG report: Enhanced compression beyond VVC capability (AHG12) |  |  |  |  |  |  |  |  | [Please include AHG & CE relation and reference doc number of a cross-check] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |  |  |  |  |  | [e.g., Input document to JVET or Output document approved by JVET] |
| Purpose: | AHG Report |  |  |  |  |  |  |  |  | [e.g, Proposal, Information, Draft text, Proposed draft, Report] |
| Author(s) or Contact(s): | M. Karczewicz Y. Ye L. Zhang B. Bross X. Li R. Chernyak K. Naser H. Yang | Author(s) or Contact(s): | martak@qti.qualcomm.com yan.ye@alibaba-inc.com lizhang.idm@bytedance.com benjamin.bross@hhi.fraunhofer.de xlxiangli@google.com chernyak@global.tencent.com karam.naser@interdigital.com haitao.yang@huawei.com |  | Name(s) Address line 1 Address line 2 Address line 3 |  | Tel: Email: | [phone contact number] [email address(es)] |
| Source: | AHG 12 |  |  |  |  |  |  |  |  | [Name of sourcing organization, AHG, CE, BoG, etc.] |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on enhanced compression beyond VVC capability (AHG12) between the 30th meeting in Antalya, TR, 21–28 April 2023, and the 31st meeting in Geneva, CH, 11–19 July 2023.

# Introduction

The mandates given to the AHG are:

- Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC.

- Discuss and propose refinements to the ECM9 algorithm description JVET-AD2025.

- Coordinate with AHG7 to study the performance and complexity tradeoff of these video coding tools.

- Coordinate with AHG6 on ECM software development.

- Support AHG6 in generating anchors according to the test conditions in JVET-AD2017.

- Analyse the results of exploration experiments described in JVET-AD2024 in coordination with the EE coordinators.

- Coordinate with AHG11 to study the interaction with neural network-based coding tools.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). A few emails were exchanged regarding EE2.

# Activities

The Common Test Conditions were updated (JVET-AD2017). The run time precision in the reporting template has been increased to 1 digit after decimal point. The primary activity of the AHG was the “Exploration experiment on enhanced compression beyond VVC capability” (JVET-ACD024). The combined improvements of the ECM-9.0 over VTM-11.0ecm8.0 anchor for AI, RA and LB configurations are:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.67% | -21.16% | -28.19% | 827.7% | 406.4% |
| Class A2 | -15.77% | -27.44% | -30.55% | 813.4% | 405.9% |
| Class B | -10.06% | -25.61% | -23.82% | 773.3% | 423.4% |
| Class C | -10.27% | -15.86% | -16.33% | 783.1% | 442.4% |
| Class E | -13.65% | -23.55% | -22.06% | 759.1% | 481.4% |
| Overall | -11.59% | -22.66% | -23.71% | 788.5% | 430.8% |
| Class D | -8.61% | -13.65% | -13.74% | 803.2% | 462.1% |
| Class F | -24.34% | -33.43% | -33.28% | 529.7% | 445.8% |
| Class TGM | -37.29% | -45.21% | -44.55% | 443.8% | 443.1% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -20.86% | -24.25% | -32.21% | 757.7% | 741.6% |
| Class A2 | -23.90% | -32.20% | -35.28% | 697.8% | 882.0% |
| Class B | -19.50% | -31.71% | -29.62% | 648.2% | 739.3% |
| Class C | -20.90% | -24.61% | -24.58% | 672.8% | 779.1% |
| Class E |  |  |  |  |  |
| Overall | -21.03% | -28.42% | -29.93% | 685.5% | 777.1% |
| Class D | -21.87% | -25.90% | -26.55% | 699.1% | 849.6% |
| Class F | -26.77% | -34.78% | -35.14% | 578.7% | 528.1% |
| Class TGM | -35.17% | -41.37% | -41.35% | 547.2% | 419.4% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -17.01% | -32.11% | -30.48% | 601.4% | 658.9% |
| Class C | -18.50% | -26.42% | -26.63% | 590.5% | 661.3% |
| Class E | -15.40% | -25.47% | -26.14% | 595.7% | 440.1% |
| Overall | -17.10% | -28.56% | -28.11% | 596.3% | 596.4% |
| Class D | -20.39% | -28.41% | -27.33% | 629.6% | 770.9% |
| Class F | -23.36% | -33.28% | -31.66% | 562.9% | 526.4% |
| Class TGM | -33.93% | -40.53% | -40.11% | 511.1% | 404.8% |

The rate reduction for natural sequences over VTM in RA configuration for {Y, U, V} increased from ECM-8.0’s {-19.86%, -26.50%, -26.67%} to ECM-9.0’s {-21.03%, -28.42%, -29.93%} with the encoder complexity increasing from 601% to 686%. For SCC sequences (class TGM) the rate reduction for RA configuration increased from ECM-8.0’s {-33.27%, -38.66%, -38.66%} to ECM-9.0’s {-35.17%, -41.66%, -41.35%}.

# Contributions

In addition to 24 EE2 contributions, 52 (comparing to 55 last meeting) EE2-related and ECM-related contributions were received and can be subdivided as follows:

## Intra (19)

JVET-AE0071, "AHG12: DIMD Merge", S. Blasi, I. Zupancic, J. Lainema (Nokia)

JVET-AE0073, "Non-EE2: IBC-LIC Model Merge mode", L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AE0075, "Non-EE2: On IntraTMP block vector", Y. Yu, L. Zhang, L. Xu, H. Yu, J. Gan, F. Wang, Z. Xie, D. Wang (OPPO)

JVET-AE0085, "Non-EE2: Direct block vector (DBV) mode extension", M. Hong, J. Choi, N. Park, J. Lim, S. Kim (LGE)

JVET-AE0098, "AHG12: On the chroma DBV mode", H. Huang, H. Wang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AE0110, "Non-EE2: An improved method for IntraTMP fusion", S. Peng, D. Jiang, J. Lin, C. Zheng, K. Fu, P. Zhang (Dahua)

JVET-AE0120, "Non-EE2: Intra Angular Prediction Extension", J. Fu, J. Zhang, C. Jia, S. Ma (PKU)

JVET-AE0124, "Non-EE2: Fixes related to Intra TMP", J.-L Lin, P.-H Lin, Y.-J Chang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AE0130, "Non-EE2: DIMD with filtered template", C. Zhou, Z. Lv (vivo)

JVET-AE0137, "Non-EE2: On TIMD fusion", P. Andrivon", M. Blestel (Ofinno)

JVET-AE0138, "Non-EE2: Reference sample interpolation for intra prediction", G. Rath, F. Le Léannec, F. Urban, F. Racape, T. Dumas (InterDigital)

JVET-AE0158, "AHG12: LUT-based angle calculation for DIMD", A. Aminlou, J. Lainema (Nokia)

JVET-AE0166, "AHG12: IntraTMP and IBC search area harmonization", D. Ruiz Coll, B. Chen, P. Andrivon, M. Blestel (Ofinno)

JVET-AE0167, "AHG12: DBV improvement", L. Xu, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AE0184, "AHG12: TIMD with IntraTMP and IBC candidates", K. Naser, F. Le Léannec, P. Bordes, F. Galpin, A. Robert (InterDigital)

JVET-AE0185, "AHG12: IntraTMP with Neighboring Candidates", K. Naser, F. Le Léannec, A. Robert, F. Galpin (InterDigital)

JVET-AE0186, "Non-EE2-2.1: Block Vector Guided Chroma Direct Mode", K. Naser, F. Le Léannec, P. Bordes, Y. Chen, G. Rath (InterDigital)

JVET-AE0198, "AHG12: Extended Search Region for IntraTMP", K. Naser, P. Bordes, F. Le Léannec, F. Galpin (InterDigital)

JVET-AE0214, "AHG12: on Intra CIIP", K. Naser, P. Bordes, F. Galpin, K. Reuze (InterDigital)

## Inter (11)

JVET-AE0047, "AHG12: On GPM-MMVD", R. Yu (Ericsson)

JVET-AE0105, "Non-EE2: Local illumination compensation with multiple templates", Y. Wang, K. Zhang, Y. He, H. Liu, L. Zhang (Bytedance)

JVET-AE0108, "AHG12: DMVR with robust MV derivation", K. Andersson, R. Yu (Ericsson)

JVET-AE0109, "Non-EE2: LIC flag derivation of merge candidates with template costs", N. Zhang, K. Zhang, H. Liu, Y. Wang, L. Zhang (Bytedance)

JVET-AE0117, "Non-EE2: Enhanced subblock-based motion compensation", L. Zhao, K. Zhang, L. Zhang (Bytedance)

JVET-AE0129, "AHG12 Template-based CIIP weight derivation", M.-H. Jia, Y.-L. Hu, S.-W. Xie, Y. Gao, C. Huang (ZTE)

JVET-AE0136, "Non-EE2: Fix on TM-based reordering for affine MMVD mode", D. Kim, W. Lim, J. Kim, S.-C. Lim, J. S. Choi (ETRI)

JVET-AE0140, "Non-EE2: LIC extensions",	A. Filippov, V. Rufitskiy, K. Suverov (Ofinno)

JVET-AE0148, "Non-EE2: Affine subblock BDOF refinement", Z. Zhang, H. Huang, J.-L Lin, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AE0168, "Non-EE2: Sample-based BDOF for Chroma", C.-C. Chen, H. Huang, Z. Zhang, C. S. Coban, V. Seregin, M. Karczewicz

JVET-AE0177, "Non-EE2: Local illumination compensation with slope adjustment", Y. Wang, K. Zhang, Y. He, H. Liu, L. Zhang (Bytedance)

## Cross Component Prediction (6)

JVET-AE0074, "Non-EE2: improvement on cross-component prediction merge mode", H. Huang, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AE0097, "AHG12: On the cross-component merge mode", H. Huang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AE0115, "Non-EE2: Unified intra CC-models parameters precision", P. Bordes, K. Naser, F. Galpin, F. Leleannec (InterDigital)

JVET-AE0170, "Non-EE2: Enhancements on CCP merge", K. Zhang, Z. Deng, L. Zhang (Bytedance)

JVET-AE0176, "EE2-related: Enhancements on CCRM", Z. Deng, K. Zhang, L. Zhang (Bytedance)

JVET-AE0178, "Non-EE2: Cross-component prediction merge mode for chroma inter coding", M.-S. Chiang, H.-Y. Tseng, C.-M. Tsai, C.-Y. Chuang, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, O. Chubach, Y.-W. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

## In-Loop Filters (3)

JVET-AE0044, "AHG12: Dynamic Scaling of Bilateral Filter (BIF)", V. Shchukin, P. Wennersten, J. Ström (Ericsson)

JVET-AE0121, "Non-EE2: Luma Residual Taps in Chroma-ALF and CCALF", W. Yin, K. Zhang, Z. Deng, L. Zhang (Bytedance)

JVET-AE0131, "Non-EE2: Variance based Classification for In-loop Filtering", W. Yin, K. Zhang, L. Zhang (Bytedance)

## Entropy Coding (1)

JVET-AE0058, "AHG12: Spatial CABAC tuning", J. Lainema, A. Aminlou, P. Astola", R. G. Youvalari, D. Bugdayci Sansli (Nokia)

## Partitioning (1)

JVET-AE0082, "AHG12: Non-square quadtree partitioning", Y. Ahn, J. Nam, N. Park, J. Lim, S. Kim (LGE)

## GDR (2)

JVET-AE0145, "AHG 12: Flexible GDR", L. Wang, S. Hong, K. Panusopone (Nokia)

JVET-AE0146, "AHG 12: Fixes H-CCP Table for CUs in Refreshed Areas of GDR/Recovering Pictures", S. Hong, L. Wang, K. Panusopone (Nokia)

## RPR (2)

JVET-AE0103, "Weighted Edge Enhancement Filtering for Picture Upscaling and RPR", T. ClaÃŸen, M. Wien (RWTH Aachen University)

JVET-AE0153, "Non-EE2: Enabling template-based inter tools for scaled reference pictures in the RPR", X. Xiu, H.-J. Jhu, C.-W. Kuo, C. Ma, N. Yan, W. Chen, X. Wang (Kwai)

## Encoder/CTC (7)

JVET-AE0055, "AHG12: Sign prediction parameter configuration for low-delay conditions", Y. Yasugi, T. Ikai (Sharp)

JVET-AE0056, "Non-EE2: Changes on TIMD and SGPM for reducing ECM decoder complexity", Z. Fan, Y. Yasugi, T. Ikai (Sharp)

JVET-AE0057, "MTT split modes early termination", W. Ahmad, P. Wennersten, K. Andersson (Ericsson)

JVET-AE0133, "AhG12: On Common Test Conditions change based on EE2 Test 1.1", G. Laroche, P. Onno (Canon)

JVET-AE0147, "AHG12: Updating context model parameters for ECM", R.-L. Liao, Y. Ye, J. Chen, X. Li (Alibaba)

JVET-AE0163, "AHG12: modified CTC proposal for low-delay configuration", S. Puri, C. Bonnineau, F. Le Leannec, T. Poirier, E. Francois (InterDigital)

JVET-AE0180, "On ECM SW memory consumption", R. Chernyak, S. Liu (Tencent), Y. Yasugi, T. Ikai (Sharp)

# Recommendations

The AHG recommends to:

- To review all the related contributions.