| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0041-v1 |
| --- | --- |

| Title: | AHG4: Report on AhG meeting on verification testing for VVC multilayer coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mathias Wien RWTH Aachen University Aachen Germany | Tel: Email: | +49-241-8027867 wien@lfb.rwth-aachen.de |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document contains the report on the AhG meeting held on 2023-05-26 in preparation of subjective testing for VVC multilayer coding.

AhG recommendations on dual-layer scalability tests:

- Expert reviewing of available streams (with and without DMVR cfg) in week of June 5th.

- Planning for dry run viewing with naïve subjects based on the outcome of this pre-screening.

- Input contribution to Geneva meeting presenting the outcome of this activity.

AhG recommendations on dual-layer scalability tests:

- Update test plan document according to presented changes.

- Review comparison to system-layer solution based on input to the Geneva meeting.

# Agenda

Review the progress and the draft plan.

# Discussion

Order of topics in discussion:

- Review of dual-layer scalability test

- Review of content layering

- (Status of quality ladder: no update yet)

- (Multiview category without activity)

## Dual-layer scalability test

Based on the results reported in JVET-AD0399, the results of previous SDR UHD verification tests in JVET-T2020 as well as JVET-AB0045, updated QP values to be used for SL and the 1.5x and 2x spatial BL+EL settings were discussed. A fixed relation between the SL and BL/EL QPs was suggested, following the rule:

- 1.5x scalability: QPBL = QPSL - 3

- 2x scalability: QPBL = QPSL - 6

- QPEL = QPSL + 1

Further aspects to address:

- Consideration of videos using upscaled base-layer resolution. Should this be tested for both 2x and 1.5x? It was mentioned that the number of test points must be considered. At the same time, having complete data available would be favorable.

- The set of points under consideration is 7 sequences x 4 QP points x <number of codec configurations, max 5> = 140

- Current intention is to do a dry run for the next meeting. It might be acceptable to do this dry run with a subset of test points and do the complete set in the next iteration.

- What type of upsampling should be used for upscaling the base-layer resolution sequences. It was suggested to use the same filter as for inter-layer prediction.

- At the Antalya meeting, an encoder fix for DMVR was adopted as an option for the VTM. The fix implies a slight loss in terms of BD-rate savings. The impact is higher on SL than in the scalable configurations due to the differences in QP settings. Artifacts induced by the regular encoder setting were reported to be more visible at high QPs and also to be more visible for the scalable setting. This will be integrated into VTM version 21.0 which has not been released yet.

- It was suggested to perform a visual pre-screening by experts before running a formal test with naïve subjects. This pre-screening would provide the decision on what to test in a formal dry run with naïve viewers. It was suggested to further double check the impact of the DMVR encoder modification in the context of this exercise and base the decision to use this parameter on the outcome of this.

Recommendation:

- Expert reviewing of available streams (with and without DMVR cfg) in week of June 5th.

- Planning for dry run viewing with naïve subjects based on the outcome of this pre-screening.

- Input contribution to Geneva meeting presenting the outcome of this activity.

## Content layering

An update of the text for this category was reviewed. Three new test sequences were suggested with the test including two or three output layer sets, i.e., switching between different additional layers. Updates for the encoder configuration were suggested including 0.5 s intra period for faster random access.

Two of the proposed sequences could be published on a website with a time limit (suggested to be probably less than two years).

It was suggested to include a description of the process to generate such content layered content to the VT report document. This would enable experts to easily replicate the experiment with other test sequences. It was mentioned that the software used to generate the demonstration sequences is intended to be made available to JVET.

Recommendation:

- Update test plan document according to presented changes.

- Review comparison to system-layer solution based on input to the Geneva meeting.

Meeting closed at 08.45h UTC.

# Participants

Fabrice Urban (InterDigital - FR)

Jens-Rainer Ohm (RWTH - DE)

Mathias Wien (RWTH - DE)

Philippe de Lagrange (InterDigital - FR)

Shunsuke Iwamura (NHK - JP)