| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0046-v1 |
| --- | --- |

| Title: | EE2-3.2: Bi-predictive GPM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu Per Wennersten Jack Enhorn Kenneth Andersson | Email: | ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution reports results for EE2 Test 3.2 on bi-predictive GPM. In EE2 Test 3.2, it is proposed to extend the existing uni-predictive GPM design to allow usage of bi-predictive motion vectors for generating motion compensated prediction samples for inter partitions. The test is reported to be carried on top of ECM-9.0. The BD-rate PSNR impact is reported to be -0.20% and -0.21% (estimated number, final number will be updated) under RA and LDB common test configurations. The average encoder run time number under RA is reported to be 101%. The average decoder run time number under RA is reported to be 100%.

# Introduction

At the JVET-AD meeting, JVET-AD0064 [1] was proposed to extend the current GPM design to allow usage of bi-predictive motion vectors. The proposal was agreed to be included in the EE2 Test 3.2 for further study and evaluation.

The detailed elements in the proposed modifications are as follows:

The first element conditionally invokes the extraction process that extracts uni-predictive motion vectors from the initial list. The extraction process is invoked only for small blocks 8x8, 16x8 and 8x16. For other larger blocks, the extraction process is bypassed. The generation of the initial list is the same as before (i.e., the normal merge list generation without any candidate reordering) except that when generating the initial list for larger blocks (i.e., blocks with the extraction process bypassed), the motion vector difference threshold for controlling whether a candidate can be added into the initial list is increased to be one full sample distance.

The second element modifies GPM-MMVD to support bi-predictive motion vector as the base vector. For low-delay pictures, the signaled MVD is applied on top of the L0 and L1 motion vector as in the existing merge MMVD design. For non-low-delay pictures, the bi-predictive motion vector is converted into a uni-predictive motion vector first and then the MVD is applied on top.

The third element modifies GPM-TM to also support bi-predictive motion vectors.

The last element is to enable the BDOF based mv refinement as in the multi-pass DMVR on top of the associated bi-predictive motion vectors for each inter partition.

# Experiment results

The EE2 Test 3.2 has been carried out in ECM-9.0 [2] and evaluated under the ECM RA and LDB common test configurations [3].

The following table reports the objective results. The reported encoder and decoder run time numbers are the average number from several simulation runs.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,15% | -0,46% | -0,09% | #NUM! | 100,0% |
| Class A2 | -0,27% | -0,30% | -0,26% | #NUM! | 99,4% |
| Class B | -0,20% | -0,23% | -0,32% | 100,9% | 99,8% |
| Class C | -0,19% | -0,33% | -0,14% | 101,0% | 100,0% |
| Class E |  |  |  |  |  |
| Overall | -0,20% | -0,32% | -0,21% | #NUM! | 99,8% |
| Class D | -0,13% | -0,06% | -0,16% | 99,7% | 100,0% |
| Class F | -0,08% | -0,22% | -0,17% | 100,7% | 99,5% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #NUM! |
| Class C | -0,23% | -0,68% | -0,58% | 101,6% | 100,5% |
| Class E | -0,17% | -0,01% | -0,46% | 101,4% | 101,9% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #NUM! |
| Class D | -0,06% | -0,05% | 0,10% | 102,6% | 102,0% |
| Class F | -0,23% | -0,49% | -0,28% | #DIV/0! | 100,6% |

# Conclusion

The contribution reports the results for EE2 Test 3.2 on bi-predictive GPM. It is asserted that the results indicate reasonable trade-off between BD-rate gains and run time impacts. It is proposed to include the proposed modifications from EE2 Test 3.2 into the next version of ECM.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).