| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0048-v2 |
| --- | --- |

| Title: | AHG9: Miscellaneous VSEI changes on neural-network post-filter SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Francesco Cricri, Maria Santamaria Nokia Technologies, Tampere, Finland | Email: | miska.hannuksela@nokia.com francesco.cricri@nokia.com maria.santamaria_gomez@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This input document proposes miscellaneous items for the VSEI text on the neural-network post-filter characteristics (NNPFC) and activation (NNPFA) SEI messages. The proposed changes are relative to JVET-AD2006. The following items are proposed:

- Adding missing value ranges for resampling syntax elements nnpfc_pic_width_num_minus1, nnpfc_pic_width_denom_minus1, nnpfc_pic_height_num_minus1, and nnpfc_pic_height_denom_minus1 to be in the range of 0 to 65535, inclusive.

- Adding missing value ranges for nnpfc_inp_format_idc and nnpfc_out_format_idc to be in the range of 0 to 255, inclusive.

- Arranging the definitions of DeriveInputTensors(), the syntax elements related to overlap and padding type, and the definition of the InpSampleVal function so that forward references are avoided by either of the following:

- Move all the overlap and padding type syntax elements earlier in the syntax, between the syntax elements defining the input format and the syntax elements defining the output format, i.e., just before nnpfc_out_format_idc. Similarly, move the related semantics and function derivations respectively.

- Move the derivations of strengthControlScaledVal (Formula 87), DeriveInputTensor (Formula 88), and StoreOutputTensor (91) to a later position that immediately follows the derivation of the InpSampleVal function.

- Clarifying that indicated or inferred values of vui_colour_primaries, vui_tranfer_characteristics, vui_matrix_coeffs, and vui_full_range_flag apply when nnpfc_separate_colour_description_present_flag is equal to 0. Here, the possibility of inferring these syntax elements is made explicit when compared to JVET-AD2006.

- Cancelling the persistence of an NNPFA SEI message with any subsequent NNPFA SEI message with the same nnpfa_target_id, Here, when compared to JVET-AD2006, the requirement on nnpfa_cancel_flag equal to 1 is removed for cancelling the persistence.

- Clarifying the description of the InpIdx[ idx ] variable to include "in inverse output order":

InpIdx[ idx ] specifying the input picture index of the idx-th picture in inverse output order that is present in the output tensor of the NNPF and has a corresponding input picture

# Adding missing value ranges for resampling syntax elements

## Problem statement

The syntax element value ranges are missing for nnpfc_pic_width_num_minus1, nnpfc_pic_width_denom_minus1, nnpfc_pic_height_num_minus1, and nnpfc_pic_height_denom_minus1

## Proposed changes

It is proposed to use a 16-bit value range for the resampling syntax elements. The yellow-highlighted additions are proposed as follows:

nnpfc_pic_width_num_minus1 plus 1 and nnpfc_pic_width_denom_minus1 plus 1 specify the numerator and denominator, respectively, for the resampling ratio of the NNPF output picture width relative to CroppedWidth. nnpfc_pic_width_num_minus1 and nnpfc_pic_width_denom_minus1 shall both be in the range of 0 to 65535, inclusive. The value of ( nnpfc_pic_width_num_minus1 + 1 ) ÷ ( nnpfc_pic_width_denom_minus1 + 1 ) shall be in the range of 1 ÷ 16 to 16, inclusive. When nnpfc_pic_width_num_minus1 and nnpfc_pic_width_denom_minus1 are not present, the values of nnpfc_pic_width_num_minus1 and nnpfc_pic_width_denom_minus1 are both inferred to be equal to 0.

nnpfc_pic_height_num_minus1 plus 1 and nnpfc_pic_height_denom_minus1 plus 1 specify the numerator and denominator, respectively, for the resampling ratio of the NNPF output picture height relative to CroppedHeight. nnpfc_pic_height_num_minus1 and nnpfc_pic_height_denom_minus1 shall both be in the range of 0 to 65535, inclusive. The value of ( nnpfc_pic_height_num_minus1 + 1 ) ÷ ( nnpfc_pic_height_denom_minus1 + 1 ) shall be in the range of 1 ÷ 16 to 16, inclusive. When nnpfc_pic_height_num_minus1 and nnpfc_pic_height_denom_minus1 are not present, the values of nnpfc_pic_height_num_minus1 and nnpfc_pic_height_denom_minus1 are both inferred to be equal to 0.

# Adding missing value ranges for input and output format indicator syntax elements

## Problem statement

The syntax element value ranges are missing for nnpfc_inp_format_idc and nnpfc_out_format_idc.

## Proposed changes

It is proposed to use an 8-bit value range for the input and output format indicators. The yellow-highlighted changes are proposed as follows:

Values of nnpfc_inp_format_idc greater than 1 in the range of 1 to 255, inclusive, are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages that contain reserved values of nnpfc_inp_format_idc. Values of nnpfc_inp_format_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

Values of nnpfc_out_format_idc greater than 1 in the range of 1 to 255, inclusive, are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages that contain reserved values of nnpfc_out_format_idc. Values of nnpfc_out_format_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

# Arranging syntax or semantics order so that syntax elements or derived functions appear before they are used

## Problem statement

The process definition of DeriveInputTensors() uses the syntax elements related to overlap and the InpSampleVal function whose semantics are specified after DeriveInputTensors().

## Proposed changes

Either of the following options is proposed:

- Move all the overlap and padding type syntax elements earlier in the syntax, between the syntax elements defining the input format and the syntax elements defining the output format, i.e., just before nnpfc_out_format_idc. Similarly, move the related semantics and function derivations respectively.

- Move the derivations of strengthControlScaledVal (Formula 87), DeriveInputTensor (Formula 88), and StoreOutputTensor (91) to a later position that immediately follows the derivation of the InpSampleVal function.

# Clarification related to inferred VUI parameters

## Problem statement

The following sentence leaves ambiguity if VUI parameters are allowed to be absent and hence inferred, when nnpfc_separate_colour_description_present_flag is equal to 0:

nnpfc_separate_colour_description_present_flag equal to 0 indicates that the combination of colour primaries, transfer characteristics, matrix coefficients, and scaling and offset values applied in association with the matrix coefficients for the picture resulting from the NNPF is the same as indicated in VUI parameters for the CLVS.

## Proposed changes

nnpfc_separate_colour_description_present_flag equal to 0 indicates that the combination of colour primaries, transfer characteristics, matrix coefficients, and scaling and offset values applied in association with the matrix coefficients for the picture resulting from the NNPF is the same as indicated in VUI parameters implied by the VUI parameters vui_colour_primaries, vui_tranfer_characteristics, vui_matrix_coeffs, and vui_full_range_flag that are indicated or inferred for the CLVS.

# Cancelling the persistence of an NNPFA SEI message with any subsequent NNPFA SEI message with the same nnpfa_target_id

## Problem statement

JVET-AD0056, item 14 proposed that any NNPFA SEI message regardless of the value of nnpfa_cancel_flag would cancel the persistence of the previous NNPFA SEI message with the same nnpfa_target_id value.

The proposed change is asserted to reduce the number of NNPFA SEI messages. For example, the activation of the base filter after a persistent activation of an updated filter of the same nnpfa_target_id value and the activation of an updated filter after a persistent activation of the base filter of the same nnpfa_target_id value are cases where the current text would require a specific NNPFA SEI message to cancel the persistence.

JVET-AD0056, item 14 was not adopted. In our recollection, the motivation for not adopting the proposal was to align the handling of the cancel flag in relation to the persistence flag equal to 1 to be the same among all SEI messages.

A cancel flag and a persistence flag are present in the following SEI messages of VSEI:

- Film grain characteristics

- Frame packing arrangement

- Content colour volume

- Equirectangular projection

- Generalized cubemap projection

- Sphere rotation

- Region-wise packing

- Omnidirectional viewport

- Sample aspect ratio information

- Display orientation

- Colour transform

All these SEI messages use the same phrasing for the semantics of persistence flag equal to 1, repeated below (where xyz is the syntax prefix for the SEI message and XYZ is the name of the SEI message):

xyz_persistence_flag equal to 1 specifies that the XYZ SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an XYZ SEI message is output that follows the current picture in output order.

To align with other SEI message, it is proposed to remove the dependency of nnpfa_cancel_flag being equal to 1 from the semantics of nnpfa_persistence_flag equal to 1.

## Proposed changes

nnpfa_persistence_flag equal to 1 specifies that the target NNPF may be used for post-processing filtering for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with a NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 1 is output that follows the current picture in output order.

NOTE 2 – The target NNPF is not applied for this subsequent picture in the current layer associated with a NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 1.

# Editorial clarification of the description of InpIdx variable

## Problem statement

Input pictures are indexed in inverse output order, i.e., the current picture is the 0-th input picture, and the i-th input picture precedes the (i − 1)-th input picture in output order for i greater than 0.

Pictures in the output tensor are indexed in output order.

InpIdx[ idx ] is an array that is used to map nnpfa_output_flag[ i ] to the respective input picture that has index InpIdx[ i ].

InpIdx[ idx ] is derived in the semantics of the NNPFC SEI message as follows. The description of the InpIdx[ idx ] may be misinterpreted so that idx would be an index of pictures in the output tensor, i.e., in output order, whereas Formula 80 defines it to be an index of pictures in inverse output order.

The variables NumInpPicsInOutputTensor, specifying the number of pictures that have a corresponding input picture and are present in the output tensor of the NNPF, InpIdx[ idx ] specifying the input picture index of the idx-th picture that is present in the output tensor of the NNPF and has a corresponding input picture, and numOutputPics, specifying the total number of pictures present in the output tensor of the NNPF, are derived as follows:

for( i = 0, numOutputPics = 0; i < numInputPics; i++ )
	if( nnpfc_input_pic_output_flag[ i ] ) {
		InpIdx[ numOutputPics ] = i
		numOutputPics++
	}					(80)
NumInpPicsInOutputTensor = numOutputPics
if( pictureRateUpsamplingFlag )
	for( i = 0; i  <=  numInputPics − 2; i++ )
		numOutputPics  +=  nnpfc_interpolated_pics[ i ]

In the semantics of nnpfa_output_flag[ i ], the inverse output order is assumed for InpIdx[ i ]. In other words, Formula 80 is correct for the semantics of nnpfa_output_flag[ i ].

nnpfa_output_flag[ i ] equal to 1 specifies that the NNPF-generated picture that corresponds to the input picture having index InpIdx[ i ] is output by the NNPF process activated by this NNPFA SEI message, where the NNPF process is specified in the semantics of the NNPFC SEI message. nnpfa_output_flag[ i ] equal to 0 specifies that the NNPF-generated picture that corresponds to the input picture having index InpIdx[ i ] is not output by the NNPF process activated by this NNPFA SEI message. When nnpfa_num_output_entries is less than NumInpPicsInOutputTensor, nnpfa_output_flag[ i ] is inferred to be equal to 1 for each value of i in the range of nnpfa_num_output_entries to NumInpPicsInOutputTensor − 1, inclusive.

## Proposal

It is proposed to clarify the description of InpIdx[ idx ] as follows:

The variables NumInpPicsInOutputTensor, specifying the number of pictures that have a corresponding input picture and are present in the output tensor of the NNPF, InpIdx[ idx ] specifying the input picture index of the idx-th picture in inverse output order that is present in the output tensor of the NNPF and has a corresponding input picture, and numOutputPics, specifying the total number of pictures present in the output tensor of the NNPF, are derived as follows:

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).