| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0050-v2 |
| --- | --- |

| Title: | AHG9: On NNPF input picture selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Francesco Cricri Nokia Technologies, Tampere, Finland | Email: | miska.hannuksela@nokia.com francesco.cricri@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution asserts that when a CLVS is concatenated after coded pictures of another bitstream, the output of an NNPF that would take input pictures across the CLVS boundary could be unpredictable, since the encoder did not expect the NNPF to be applied with inputs originating from different bitstreams. To enable splicing of bitstreams that define NNPFs inputting multiple pictures, the contribution proposes the following items:

- Addition of nnpfa_no_prev_clvs_flag in the NNPFA SEI message.

When equal to 1, nnpfa_no_prev_clvs_flag specifies that input pictures that would originate from the previous CLVS in output order are treated as if they were not present in the bitstream. When nnpfa_no_prev_clvs_flag is equal to 0, the current operation in JVET-AD2005 and JVET-AD2006 is kept unchanged.

The value of nnpfa_no_prev_clvs_flag can be changed from 0 to 1, when the current CLVS is spliced from another bitstream next to the previous CLVS in output order and this NNPFA SEI message would cause one or more input pictures to be selected from the previous CLVS in output order.

- Addition of nnpfa_no_foll_clvs_flag in the NNPFA SEI message, gated by nnpfa_persistence_flag being equal to 1. nnpfa_no_foll_clvs_flag equal to 1 specifies that when this NNPFA SEI message persists for the last PU of a CLVS in output order, the NNPFA SEI message is treated as if it persisted for the last PU of the current layer within the bitstream. In this case, the picture rate upsampling post-filter may be invoked for multiple sets of input pictures until pictures are interpolated between the last pair of input pictures within the CLVS in output order. When nnpfa_no_foll_clvs_flag is equal to 0, the current operation in JVET-AD2005 and JVET-AD2006 is kept unchanged.

# Introduction

It was concluded in the JVET-AD meeting to allow input pictures of an NNPF inference from more than one CLVS. Consequently, as an example, an encoder can indicate picture rate upsampling to apply across CLVS boundaries to achieve a constant picture rate as the result of filtering.

This contribution asserts that when a CLVS is concatenated after coded pictures of another bitstream, the output of an NNPF that would take input pictures across the CLVS boundary could be unpredictable, since the encoder did not expect the NNPF to be applied with inputs from different bitstreams. Consequently, the contribution proposes to indicate in the NNPFA SEI message how to handle situations when some input pictures of the NNPF would originate from a different CLVS than the current one.

Figure 1 presents an example of a picture rate upsampling NNPF that takes four pictures as input and interpolates one output picture between the middle pair of the input pictures. Values indicate a relative output order of pictures. It is remarked that even though the contribution presents an example using picture rate upsampling, similar problems are asserted to exist with an NNPF using multiple input pictures for other purposes.

| Input pictures | 0 |  | 2 |  | 4 |  | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ↓ |  |  |  |  |  |  |
|  | NNPF |  |  |  |  |  |  |
|  | ↓ |  |  |  |  |  |  |
| Output pictures |  |  |  | 3 |  |  |  |

Figure 1 – An example of a picture rate upsampling NNPF

Figure 2 illustrates the selection of input pictures when the example NNPF is applied across CLVSs according to JVET-AD2005 and JVET-AD2006. When an NNPFA SEI message activates the example NNPF for picture 6, input pictures 2, 4, 6 originate from the current CLVS, whereas input picture 0 originates from the previous CLVS in output order.

| Input pictures | 0 | CLVS boundary | 2 |  | 4 |  | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↓ |  |  |  |
|  |  |  | NNPF |  |  |  |  |
|  |  |  |  | ↓ |  |  |  |
| Output pictures |  |  |  | 3 |  |  |  |

Figure 2 – The example picture rate upsampling NNPF at the start of a CLVS.

Continuing the previous example, if the previous CLVS originated from a different bitstream than that of the current CLVS, it is asserted that all the input pictures should be selected from the current CLVS. Rather than using input picture 0 from the previous CLVS, a duplicate of input picture 2 (when nnpfc_absent_input_pic_zero_flag is equal to 0) or a picture with sample values equal to 0 (when nnpfc_absent_input_pic_zero_flag is equal to 1) should be used.

Figure 3 illustrates the selection of input pictures when an NNPFA SEI message activates the example NNPF for picture 6 and input pictures 0, 2, and 4 originate from the previous CLVS. If the previous CLVS originated from a different bitstream than that of the current CLVS, it is asserted that in order to interpolate picture 3 as presented in Figure 3, pictures 0, 2, and 4 should be used as input in addition to a duplicate of input picture 4 (when nnpfc_absent_input_pic_zero_flag is equal to 0) or a picture with sample values equal to 0 (when nnpfc_absent_input_pic_zero_flag is equal to 1).

| Input pictures | 0 |  | 2 |  | 4 | CLVS boundary | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↓ |  |  |  |
|  |  |  | NNPF |  |  |  |  |
|  |  |  |  | ↓ |  |  |  |
| Output pictures |  |  |  | 3 |  |  |  |

Figure 3 – The example picture rate upsampling NNPF at the end of a CLVS.

# Proposal

## Overview

This contribution proposes the following items:

- Addition of nnpfa_no_prev_clvs_flag in the NNPFA SEI message. When equal to 1, nnpfa_no_prev_clvs_flag specifies that input pictures that would originate from the previous CLVS in output order are treated as if they were not present in the bitstream. When nnpfa_no_prev_clvs_flag is equal to 0, the current operation in JVET-AD2005 and JVET-AD2006 is kept unchanged.

- Addition of nnpfa_no_foll_clvs_flag in the NNPFA SEI message, gated by nnpfa_persistence_flag being equal to 1. nnpfa_no_foll_clvs_flag equal to 1 specifies that when this NNPFA SEI message persists for the last PU of a CLVS in output order, the NNPFA SEI message is treated as if it persisted for the last PU of the current layer within the bitstream. In this case, the picture rate upsampling post-filter may be invoked for multiple sets of input pictures until pictures are interpolated between the last pair of input pictures within the CLVS in output order. When nnpfa_no_foll_clvs_flag is equal to 0, the current operation in JVET-AD2005 and JVET-AD2006 is kept unchanged.

If the proposed item 1 were not adopted, it is asserted that either or both of the following consequences might occur:

- If a CLVS originating from a second bitstream is spliced after a CLVS originating from a first bitstream and the splicer has no specific treatment of NNPFA SEI messages, an NNPFA SEI message that invokes an NNPF with input pictures across CLVSs may remain present and may cause undesirable filtering or interpolation.

- When a picture rate upsampling filter does not interpolate picture between the first pair of its input pictures and an encoder or splicer inserts NNPFA SEI messages so that input pictures originate from a single CLVS only, it will not be possible to interpolate pictures between one or more first pairs of pictures in the CLVS that is not the first CLVS of the current layer in the bitstream. In other words, it will not be possible to increase the picture rate right from the start of the CLVS.

If the proposed item 2 were not adopted, it is asserted that the following consequences might occur:

- When a picture rate upsampling filter does not interpolate pictures between the last pair of its input pictures and an encoder or splicer inserts NNPFA SEI messages so that input pictures originate from a single CLVS only, it will not be possible to interpolate pictures between one or more last pairs of pictures in the CLVS that is not the last CLVS of the current layer in the bitstream. In other words, it will not be possible to increase the picture rate until the end of the CLVS.

## Proposed VSEI additions

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_target_base_flag | u(1) |
| nnpfa_persistence_flag | u(1) |
| nnpfa_no_prev_clvs_flag | u(1) |
| if( nnpfa_persistence_flag ) |  |
| nnpfa_no_foll_clvs_flag | u(1) |
| nnpfa_num_output_entries | ue(v) |
| for( i = 0; i < nnpfa_num_output_entries; i++ ) |  |
| nnpfa_output_flag[ i ] | u(1) |
| } |  |
| } |  |

nnpfa_no_prev_clvs_flag equal to 1 specifies that the input pictures for the NNPF do not originate from the previous CLVS in output order. nnpfa_no_prev_clvs_flag equal to 0 specifies that the input pictures for the NNPF may or may not originate from the previous CLVS in output order.

NOTE 3 – The value of nnpfa_no_prev_clvs_flag can be changed from 0 to 1, when the current CLVS is spliced from another bitstream next to the previous CLVS in output order and this NNPFA SEI message would cause one or more input pictures to be selected from the previous CLVS in output order and therefore is likely to impact the output of the target NNPF negatively.

nnpfa_no_foll_clvs_flag equal to 1 specifies that when this NNPFA SEI message persists for the last PU of a CLVS in output order, the NNPFA SEI message is treated like it persisted for the last PU, in output order, of the current layer within the bitstream. When this NNPFA SEI message does not persist for the last PU, in output order, of a CLVS in output order or nnpfa_no_foll_clvs_flag is equal to 0, the value of nnpfa_no_foll_clvs_flag causes no specific impact.

NOTE 4 – The value of nnpfa_no_foll_clvs_flag can be  changed from 0 to 1 for a picture rate upsampling NNPF, when the following CLVS in output order is spliced from a different bitstream next to the current CLVS in output order. Consequently, the NNPF process interpolates pictures up to the end of the current CLVS using input pictures originating from the current CLVS only.

## Proposed VVC changes

The variable numInferences is derived as follows:

–	If pictureRateUpsamplingFlag is equal to 1, the NNPFA SEI message that activated this NNPF has nnpfa_persistence_flag equal to 1, nnpfc_interpolated_pics[ i ] is greater than 0 only for a single value of i that is greater than 0, and currPic is the last picture of the bitstream in output order that has nuh_layer_id equal to currLayerId or both currPic is the last picture in the CLVS in output order and nnpfa_no_foll_clvs_flag is equal to 1, the variable numPostRoll is set equal to the value of i such that nnpfc_interpolated_pics[ i ] is greater than 0 and the variable numInferences is set equal to 1 + numPostRoll.

–	Otherwise, the variable numInferences is set equal to 1.

For each value of j in the range of 0 to numInferences − 1, inclusive, the following applies:

–	The arrays inputPic[ i ] and inputPresentFlag[ i ] for i in the range of 0 to numInputPics − 1, inclusive, representing all the input pictures and the presence of input pictures within the bitstream, respectively, are specified as follows:

–	When j is greater than 0, for each value of k in the range of 0 to j − 1, inclusive, inputPic[ k ] is set to be currPic and inputPresentFlag[ k ] is set equal to 0.

–	The j-th input picture, inputPic[ j ], is set equal to be currPic and inputPresentFlag[ j ] is set equal to 1.

–	When numInputPics is greater than 1, the following applies for each value of i in the range of j + 1 to numInputPics − j − 1, inclusive, in increasing order of i:

–	If pictureRateUpsamplingFlag is equal to 1, currPic is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and a particular value of fp_current_frame_is_frame0_flag, and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId, precede inputPic[ i − 1 ] in output order, and are associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag, and nnpfa_no_prev_clvs_flag is equal to 0 or the coded picture corresponding to prevPic and the current picture are present in the same CLVS, the inputPic[ i ] is set to be prevPic and inputPresentFlag[ i ] is set equal to 1.

–	Otherwise, if pictureRateUpsamplingFlag is equal to 0 and there is a picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId and precede inputPic[ i − 1 ] in output order, and nnpfa_no_prev_clvs_flag is equal to 0 or the coded picture corresponding to prevPic and the current picture are present in the same CLVS, inputPic[ i ] is set to be prevPic and inputPresentFlag[ i ] is set equal to 1.

–	Otherwise, if currPic is not associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId and precede inputPic[ i − 1 ] in output order, and nnpfa_no_prev_clvs_flag is equal to 0 or the coded picture corresponding to prevPic and the current picture are present in the same CLVS, inputPic[ i ] is set to be prevPic and inputPresentFlag[ i ] is set equal to 1.

–	Otherwise, the following applies:

–	inputPic[ i ] is set to be the same picture as inputPic[ i − 1 ] and inputPresentFlag[ i ] is set equal to 0.

–	It is a requirement of bitstream conformance that num_interpolated_pics[ i − 1 ] shall not be greater than 0.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).