| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0055-v1 |
| --- | --- |

| Title: | AHG12: Sign prediction parameter configuration for low-delay conditions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi, Tomohiro Ikai | Tel: Email: | yasugi.yukinobu@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

In JVET-AD0105, the common test condition with a modification to the sign prediction parameter to AI and RA was adopted. In this contribution, the same configuration that the NumSignPred parameter in QP22 is set to four rather than eight is also applied for the low-delay conditions (LB and LP), thus it will be used in all conditions. Experimental results reportedly show that the proposed modification reduced the encoding time in QP22 on the low-delay conditions by 2%.

Introduction

In JVET-AD0105 (Aspect 1), a modification to the sign prediction parameter in the AI and RA conditions of the Common Test Conditions (CTC)[1] was adopted, and ECM-9.0 introduced a new option NumSignPredOverrideByQP to override NumSignPred only for specific QPs. This option is currently used only in AI and RA conditions and is set as follows:

NumSignPredOverrideByQP: 22 4

This configuration overrides the number of sign predictions in QP22 to four rather than eight. In this contribution, the authors report experimental results of applying this configuration to the LB and LP conditions.

Experimental results

Table 1 shows results of the encoding and decoding time ratio for QP22. The modified configuration reduced the encoding times of QP22 in the LB and LP conditions by 2% on average. For BQTerrace, the most time-consuming sequence, encoding time was reduced by 4%. This is about 16 hours in our computing environment. Decoding times also tended to decrease by 0-2% on average.

Table 1: Time ratio of encoding and decoding for QP=22

|  |  | LB-10 |  | LP-10 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | EncR | DecR | EncR | DecR |
| Class B | MarketPlace | 99% | 100% | 99% | 100% |
|  | RitualDance | 98% | 100% | 99% | 99% |
|  | Cactus | 98% | 98% | 96% | 99% |
|  | BasketballDrive | 98% | 98% | 96% | 97% |
|  | BQTerrace | 96% | 98% | 97% | 97% |
| Class C | BasketballDrill | 95% | 98% | 98% | 98% |
|  | BQMall | 100% | 99% | 97% | 98% |
|  | PartyScene | 102% | 98% | 96% | 99% |
|  | RaceHorses | 96% | 97% | 95% | 93% |
| Class D | BasketballPass | 96% | 102% | 97% | 96% |
|  | BQSquare | 101% | 99% | 97% | 93% |
|  | BlowingBubbles | 99% | 99% | 97% | 98% |
|  | RaceHorses | 98% | 99% | 96% | 98% |
| Class E | FourPeople | 98% | 98% | 100% | 98% |
|  | Johnny | 99% | 103% | 99% | 99% |
|  | KristenAndSara | 100% | 101% | 99% | 101% |
| Class F | ArenaOfValor | 98% | 101% | 97% | 98% |
|  | BasketballDrillText | 96% | 99% | 95% | 100% |
|  | SlideEditing | 99% | 106% | 96% | 99% |
|  | SlideShow | 98% | 101% | 97% | 98% |
| Average | (classB, C, E) | 98% | 99% | 98% | 98% |
| Average | (All) | 98% | 100% | 97% | 98% |

The modified configuration only affects QP=22 in the LB and LP conditions, not other QPs or the AI and RA conditions. It also does not affect VTM because VTM does not have the parameter. Table 2 shows experimental results in the LB and LP conditions, with the modified configuration. There was almost no impact on coding efficiency.

Table 2: Experimental results of modified configuration for low delay conditions

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.03% | 0.02% | 99% | 100% |
| Class C | 0.00% | -0.05% | 0.01% | 100% | 99% |
| Class E | 0.00% | -0.05% | 0.03% | 100% | 100% |
| Overall | 0.01% | -0.02% | 0.02% | 100% | 100% |
| Class D | 0.00% | -0.02% | -0.08% | 100% | 100% |
| Class F | -0.08% | -0.07% | -0.16% | 99% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.01% | -0.02% | 99% | 100% |
| Class C | 0.00% | 0.04% | 0.02% | 99% | 99% |
| Class E | 0.03% | -0.06% | -0.02% | 100% | 100% |
| Overall | 0.01% | 0.01% | -0.01% | 99% | 100% |
| Class D | 0.02% | -0.09% | -0.03% | 99% | 99% |
| Class F | -0.02% | 0.14% | -0.04% | 99% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusion

We suggest that the NumSignPredOverrideByQP parameter be added to the LB and LP conditions of CTC to be set the same as the current AI and RA conditions.

Acknowledgements

These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.

References

[1] M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, April 2023.

[2] Y. Yasugi and T. Ikai, “Modifications of common test condition and ECM software”, JVET-AD0105, April 2023.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).