| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0065 |
| --- | --- |

| Title: | EE2-3.5: Iterative BDOF pass in multi-pass DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar, Yuwen He, Kai Zhang, Hongbin Liu, Li Zhang Jie Chen, Ru-Ling Liao, Xinwei Li, Yan Ye | Email: | mehdi.salehifar@bytedance.com jiechen.cj@alibaba-inc.com |
| Source: | Bytedance Inc., Alibaba group |  |  |

_____________________________

# Abstract

This document reports test results for EE2-3.5.

In test EE2-3.5 (JVET-AD0196, JVET-AD0176), multi-pass DMVR is extended by introducing additional BDOF iteration.

On top of the ECM-9.0, simulation result of the proposed method is reported as:

RA: -0.11%, -0.19%, -0.11%, 100%, 98%.

# Introduction

In ECM, a multi-pass DMVR is applied. There are three passes of motion refinement. In the first pass, bilateral matching (BM) is applied to the coding block (CB) to get the block level refined MV pair. In the second pass, the block is divided into 16×16 subblocks and on top of block level refined MV pair obtained in the first pass, BM is applied on each 16×16 subblock to get a 16×16 subblock level refined MV pair. In the third pass, BDOF based MV refinement is applied on the 4×4 or 8×8 subblocks depending on the CB size.

# Proposed method

In this contribution (as disclosed in JVET-AD196 and JVET-AD176 [1-2]), it is proposed to extend the current multi-pass DMVR by adding another BDOF-based motion refinement pass as the 4th pass of the multi-pass DMVR (i.e., the 2nd round of BDOF-based DMVR).

Also, the subblock size of the proposed round of the BDOF-based DMVR is adaptively selected depending on the area (i.e., width×height) of the coding block. For blocks smaller than 1024 pixels, the subblock size of 4×4, and otherwise 8×8 is used.

# Experimental results

The proposed method has been implemented on top of the ECM-9.0 [3]. Testing results of EE2-3.5 under RA configuration is summarized in Table I. The simulations were performed following the common test conditions [4]. More detailed results can be found in the attached spreadsheet.

Table I: Performance of EE2-3.5

# Conclusions

In this contribution, iterative BDOF pass for multi-pass DMVR is proposed. With 2% reduction in decoder complexity, it provides -0.11% RA coding gain. It is recommended to adopt this method into ECM-10.0.

# References

[1] M. Salehifar, Y. He, K. Zhang, H. Liu, L. Zhang, “EE2-related: On Iterative BDOF and EE2-2.6 Improvement” JVET-AD0196, Apr. 2023.

[2] J. Chen, R. -L. Liao, X. Li, Y. Ye, “Non-EE2: Improvements on multi-pass DMVR” JVET-AD0176, Apr. 2023.

[3] ECM-9.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0

[4] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, Apr. 2023.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).