| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0066-v3 |
| --- | --- |

| Title: | AHG9: Multiplane Image Information SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taoran Lu, Peng Yin, Guan-Ming Su, Dae Yeol Lee, Tsung-wei Huang, Sean McCarthy, Walt Husak, Gary J. Sullivan (Dolby) | Email: | {tlu, pyin, smcca}@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a multiplane image information (MPII) SEI message be included in the next version of VSEI. The proposed MPII SEI message is intended to enable virtual reality/immersive experiences on existing consumer devices using existing video codecs such as HEVC and VVC that were developed for conventional two-dimensional video.

Version 3 of this contribution contains 3 changes with respect to version 1 in response to comments from JVET experts during discussion on July 14, 2023:

- Remove syntax elements mpii_cancel_flag and mpii_persistence_flag

- Remove syntax element mpii_view_id

- Add a table comparing the MPI in the context of MIV and the proposed MPII SEI message

# Introduction

Multiplane Image (MPI) scene representation is commonly used for three-dimensional (3D) scene reconstruction [1][2]. MPI is a state-of-the-art method for novel view synthesis which can model challenging occlusions and reflections. It requires very low computation for 3D scene reconstruction, making it possible to be implemented on existing devices to enable immersive image/video experiences.

MPI representation stores fronto-parallel planes of a scene at a discretely sampled fixed range of depths from the reference coordinate frame. Information stored in each plane contains the texture (in terms of R, G, B or Y, Cb, Cr value) and opacity/transparency (in terms of alpha (A) channel or opacity channel). This structure is advantageous for synthesizing images from novel viewpoints of a scene since it can avoid occlusion-related issues typically encountered in RGB images.

## Representation of a multiplane image

Figure 1 illustrates an MPI representation, MPI(s), using D layers of RGBA images [1] for the case of a single camera view s. The most distant layer is layer 0, and layer D-1 is the one closest to the reference camera position S. Denote the RGB value for the ith layer at camera location S as Ci(s) with dimension HxW. The sample value at (x, y) for the ith layer color channel c is represented as Ci(s)(x,y,c). The opacity/transparency value for the ith layer is Ai(s). The sample value at (x, y) for the ith alpha layer is represented as Ai(s)(x,y). The depth distance between the ith layer to reference camera position is di.

MPI(s) = {Cis(x,y,c), Ais(x,y)}, i = 0…D-1					(1)

The image from the source viewpoint (original camera reference view), denoted as R with sample values R(s)(x,y,c), is obtained by weighted composition as follows:

R(x,y,c)(s)=i=0D-1Cis(x,y,c)Wi(s) 							(2)

Where weight information for the ith alpha layer is

Wi(s)=Aisx,y*j=i+1D-1(1-Ajs(x,y))) 						(3)

It is straightforward to extend the MPI based image representation to video when the source viewpoint s is kept static overtime by adding a time dimension t:

MPI(s, t) = {Cis(x,y,c, t), Ais(x,y,t)}, i = 0…D-1					(4)

Figure 1 MPI representation (in perspective projection, all layers have the same dimensions HxW)

## Synthesis of a novel view from a reference MPI

The image from the novel virtual viewpoint (i.e., a viewpoint different from the original source viewpoint) is rendered from a reference MPI in two steps. First, each layer is warped from the source viewpoint to the desired virtual viewpoint. Second, the novel viewpoint image is obtained by weighted composition of the warped layers (this operation is the same as in Equations 3 and 4).

1) Warping

Each layer (Ci(s), Ai(s)) is warped from the source viewpoint position (s) to the virtual viewpoint position (v).

Ci(s→v)=Ts,v(σdi,Ci)											(5)

Ai(s→v)=Ts,v(σdi,Ai) 											(6)

The warping function, Ts,v(), computes the texture and alpha value at each sample in its output by sampling bilinearly from the input texture or alpha, respectively. This is done by applying a homography to each target sample’s coordinates (xv,yv) to obtain corresponding source coordinates (xs,ys) at which to sample:

xsys1=Ks(R-tnTa)(Kv)-1xvyv1									(7)

where Ks and Kv are the camera intrinsic camera model for source view and virtual view. R and t are the extrinsic camera model for rotation and translation. n is the normal vector [0 0 1]T. a is the distance to a plane that is fronto-parallel to the source camera at depth σdi.

2) Compositing

We can render the virtual view from those warped texture and alpha images via the following equations.

Wi(s→v)=(Ai(s→v)∙j=i+1D-1(1-Aj(s→v))) 						(8)

R(s→v)=i=0D-1Ci(s→v)Wi(s→v)									(9)

More details are explained in [1][2].

# Proposal

It is desirable to transmit and distribute the MPI format over existing video transmission infrastructure to enable virtual reality experience for end users using existing devices. This contribution proposes multiplane image information SEI message to help achieve this goal. The SEI and corresponding bitstream is specified for one camera view. (For multiple view case, each view’s bitstream can have its corresponding MPII SEI.)

To enable novel view rendering using MPI scene representation, the following information is required for the current view:

- MPI texture layers and opacity layers

- MPI depth information for each layer

- Camera information

## Aspect 1 – Coding of MPI texture and opacity layers

It is proposed to code MPI texture layers and opacity layers as 2D pictures using conventional video codecs such as VVC, HEVC, and AVC.

For each MPI picture, it is proposed that the associated N texture layers and N opacity layers be represented with the following configuration options where, K x M=N, K specifies the number of layers for texture and opacity layers in width, M specifies the number of texture and opacity layers in height:

- Constituent picture pairs comprising

  - Spatially packed texture layers in K x M arrangement (Figure 2(a))

  - Spatially packed opacity layers in K x M arrangement (Figure 2(b))

- Spatially packed texture and opacity layers in a top-bottom K x 2M arrangement (Figure 3)

- Spatially packed texture and opacity layers in a side-by-side 2K x M arrangement (Figure 4)

- Temporally interleaved texture and opacity layers in spatially packed K x M arrangements (Figure 5)

| - constituent picture 0: MPI texture layers KxM packing | - constituent picture 1: MPI opacity layers K*M packing |
| --- | --- |

Figure 2 spatial packing for MPI texture layers and opacity layers: K x M

Figure 3 MPI texture layers and opacity layers packing arrangement: top and bottom

Figure 4 MPI texture layers and opacity layers packing arrangement: side by side

Figure 5 MPI texture layers and opacity layers packing arrangement: temporal interleaving

## multiplane_image_information SEI syntax and semantics

| multiplane_image_information( payloadSize ) { | Descriptor |
| --- | --- |
| mpii_num_layers_minus1 | ue(v) |
| mpii_layer_depth_equal_distance_flag | u(1) |
| if( mpii_layer_depth_equal_distance_flag ) { |  |
| depth_rep_info_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| depth_rep_info_element( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| } else |  |
| for( i = 0; i < = mpii_num_layer_minus1; i++ ) |  |
| depth_rep_info_element( ZSign[ i ], ZExp[ i ], ZMantissa[ i ], ZManLen[ i ] ) |  |
| mpii_texture_opacity_interleave flag | u(1) |
| if( mpii_texture_opacity_interleave flag = = 0 ) |  |
| mpii_texture_opacity_arrangement_flag /* 0: TaB, 1:SbS */ | u(1) |
| mpii_picture_num_layers_in_height_minus1 | ue(v) |
| } |  |

## Multiplane image information SEI message semantics

The multiplane image information (MPII) SEI message specifies the MPI scene representation information that may be used for view synthesis.

When an MPII SEI message is present in any access unit of a CLVS, an MPII SEI message shall be present in the first access unit of the CLVS and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an MPII SEI message is output that follows the current picture in output order.

NOTE 1 – This SEI can work together with multiview acquisition information SEI message for view synthesis. multiview acquisition information SEI message specifies the intrinsic and extrinsic parameters for current camera view. When multiple video bitstreams are available, the reconstructed novel views can be rendered from nearby multiview MPIs.

Use of this SEI message requires the definition of the following variables:

–	Cropped decoded output picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	a chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

–	a cropped decoded picture array decPicCurr0[ cIdx ][ x ][ y ], with cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, x = 0..(cIdx = = 0 ) ? CroppedWidth : CroppedWidth / SubWidthC – 1, y = 0..(cIdx = = 0 ) ? CroppedHeight : CroppedHeight / SubHeightC – 1.

–	In output order a temporally following cropped decoded picture array decPicCurr1[ cIdx ][ x ][ y ], with cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, x = 0..(cIdx = = 0 ) ? CroppedWidth : CroppedWidth / SubWidthC – 1, y = 0..(cIdx = = 0 ) ? CroppedHeight : CroppedHeight / SubHeightC – 1.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

mpii_num_layers_minus1 plus 1 specifies the number of texture and opacity layers for MPI representation.

mpii_layer_depth_equal_distance_flag equal to 1 indicates the equal distance is used to generate MPI layers and depth parameter for each layer Z[ i ] can be derive using nearest depth value ZNear and farthest depth value ZFar.

The depth value for i-th MPI layer Z[ i ] = i * ( ZFar – Znear ) / (mpi_num_layers_minus1 ) + ZNear (xx)

mpii_layer_depth_equal_distance_flag equal to 0 indicates the depth information for each layer follows next in SEI.

The variables in the x column of Table xx are derived from the respective variables in the s, e, n and v columns of Table xx as follows:

–	If the value of e is in the range of 0 to 127, exclusive, x is set equal to (  − 1 )s * 2e - 31 * ( 1 + n ÷ 2v ).
–	Otherwise (e is equal to 0), x is set equal to (  − 1 )s * 2-( 30 + v ) * n.

NOTE 3 – The above specification is similar to that found in IEC 60559:1989.

Table xx – Association between depth parameter variables and syntax elements

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| Z[ i ] | ZSign[ i ] | ZExp | ZMantissa | ZManLen |

NOTE 4 – in some applications, disparity is used instead of depth (Disparity value D and depth value Z relationship is D = 1/Z). For (xx), instead of deriving depth value, the disparity value for i-th MPI layer D[ i ] = i * ( DFar – Dnear ) / (mpi_num_layers_minus1 ) + DNear.

mpii_texture_opacity_interleave_flag equal to 1 indicates decoded output pictures correspond to temporally interleaved texture and opacity constituent pictures in output order as illustrated in Figure 5. mpii_texture_opacity_interleave_flag equal to 0 indicates decoded output pictures correspond to spatially packed texture and opacity constituent pictures as illustrated in Figures 3 and 4.

mpii_texture_opacity_arrangement_flag equal to 0 indicates decoded output pictures represent texture and opacity constituent pictures in a top-bottom packing arrangement as illustrated in Figure 3. mpii_texture_opacity_arrangement_flag equal to 1 indicates decoded output pictures represent texture and opacity constituent pictures in a side-by-side packing arrangement as illustrated in Figure 4.

For each specified picture packing arrangement scheme, there are two constituent pictures that are referred to as picture 0 and picture 1. When mpii_texture_opacity_interleave_flag is equal to 0, the constituent picture associated with the upper-left sample of the decoded picture is considered to be constituent picture 0 and the other constituent picture is considered to be constituent picture 1. When mpii_texture_opacity_interleave_flag is equal to 1, the first decoded picture in the current CLVS is constituent picture 0 and the next decoded picture in output order is constituent picture 1 and the display time of the constituent picture 0 should be delayed to coincide with the display time of constituent picture 1. The two constituent pictures form the spatially packed texture and opacity picture of a MPI, with picture 0 being associated with the spatially packed texture picture and picture 1 being associated with the spatially packed opacity picture.

mpii_picture_num_layers_in_height_minus1 plus 1 specifies the number of spatially packed layers in height for picture 0 and picture 1. The variable hLayers is set equal to mpii_picture_num_layers_in_height_minus1 + 1 and the variable wLayers is set equal to (mpii_num_layers_minus1 + 1) / hLayers.

Let variable fWidth and fHeight specify the width and height of picture 0 and picture 1, respectively, and are derived as follows:

–	If mpii_texture_opacity_interleave_flag is equal to 1, the following applies:

fWidth = CroppedWidth

fHeight = CroppedHeight

–	Otherwise (mpii_texture_opacity_interleave_flag is equal to 0)

–	If mpii_texture_opacity_arrangement_flag is equal to 0, the following applies:

fWidth = CroppedWidth , fHeight = CroppedHeight / 2

–	Otherwise (mpii_texture_opacity_arrangement_flag is equal to 1), the following applies:

fWidth = CroppedWidth / 2 , fHeight = CroppedHeight

Let variable cWidth = fWidth / subWidthC and variable cHeight = fHeight / subHeightC.

Let array picture0[ cIdx ][ x ][ y ] specify samples in picture 0 and array picture1[ cIdx ][ x ][ y ] specify samples in picture 1, with cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, x = 0..(cIdx = = 0 ) ? fWidth: cWidth – 1, y = 0..(cIdx = = 0 ) ? fHeight : cHeight – 1 and are derived as follows:

–	If mpii_texture_opacity_interleave_flag is equal to 1, the following applies:

picture0[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y ]

picture1[ cIdx ][ x ][ y ] = decPicCurr1[ cIdx ][ x ][ y ]

–	Otherwise (mpii_texture_opacity_interleave_flag is equal to 0)

–	Let variable cW = (cIdx = = 0 )? fWidth : cWidth

–	Let variable cH = (cIdx = = 0 )? fHeight : cHeight

–	If mpii_texture_opacity_arrangement_flag is equal to 0, the following applies:

picture0[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y ]

picture1[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y + cH ]

–	Otherwise (mpii_texture_opacity_arrangement_flag is equal to 1), the following applies:

picture0[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y ]

picture1[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x + cW ][ y ]

Let variable layerWidth and layerHeight specify the width and height for decoded MPI layer, respectively. The variables are derived as follows:

layerWidth = fWidth/wLayers

layerHeight = fHeight/hLayers

The reconstruction of MPI process is described as follows:

The outputs of this process are:

–	a 4D mpi texture layer array recTextureLayer[ i ][ cIdx ][ w ][ h ] with i = 0..mpii_num_layers_minus1, cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, w = 0..(cIdx = = 0 ) ? layerWidth : layerWidth / SubWidthC − 1, and h = 0..(cIdx = = 0 ) ? layerHeight : layerHeight / SubHeightC – 1.

–	a 3D mpi opacity layer array recOpacityLayer[ i ][ w ][ h ] with i = 0..mpii_num_layers_minus1, x = 0..layerWidth − 1, and y = 0..layerHeight – 1.

The array recTextureLayer and array recOpacityLayer are derived as follows:

for( i = 0; i <= mpii_num_layers_minus1; i++ ) { 
	k = i % wLayers
	m = ( i – k ) / hLayers
	for ( cIdx = 0; cIdx < ChromaFormatIdc = = 0 ) ? 1 : 3; cIdx++ ) 
		for( h = 0; h < (cIdx = = 0 ) ? layerHeight : layerHeight / SubHeightC ; h++ ) 
			for( w = 0; w < (cIdx = = 0 ) ? layerWidth : layerWidth / SubWidthC ; w++ ) {
				u = k * (cIdx = = 0 ) ? layerWidth : layerWidth / SubWidthC + w
				v = m * (cIdx = = 0 ) ? layerHeight : layerHeight / SubHeightC + h
				recTextureLayer[ i ][ cIdx ][ w ][ h ] = picture0[ cIdx ][ u ][ v ]
			}
	for( h = 0; h < layerHeight; h++ ) 
		for( w = 0; w < layerWidth ; w++ ) 
			recOpacityLayer[ i ][ w ][ h ] = picture1[ 0 ][ k * layerWidth + w ][ m * layerHeight + h ]
}

## Aspect 2 – MPI layer depth information

For depth information for each layer, if constant/equal depth distance is used to generate MPI layers, only closest depth and farthest depth are required to be signalled. Otherwise, we will signal depth information for each layer. In VSEI, depth_rep_info_element( OutSign, OutExp, OutMantissa, OutManLen ) in the depth representation information (DRI) SEI can be used to signal such information and we propose to re-use this structure for depth-value signaling and incorporate the syntax/semantics into the proposed MPII SEI.

## Aspect 3 – Camera information

Camera information in terms of intrinsic and extrinsic matrices is needed for the warping process in order to preserve the real-world coordinates and synchronize multiple cameras. In that case, we propose that the multiview acquisition information (MAI) SEI message can be used to specify the camera information for current view required for synthesis of virtual viewpoints from a reference MPI. This is out of the scope of the MPI information SEI. For single view rendering, the renderer can perform view synthesis with a pre-defined, universal camera intrinsic matrix.

# Examples and demo

Examples are provided to illustrate the usability of the MPII SEI. Several clips are selected for the MPI compression demo (Figure 6). They are one second (30 frames) from the view 07 in Barn (copyright @ InterDigital), Breakfast (copyright @ InterDigital) and CBAbasketball (copyright @ Alibaba) from the MIV common test conditions [3]. AdaMPI [4] is used to generate the MPI texture and alpha layers with corresponding layer depth. For all the demos, the number of MPI layers are set to 16. An end-to-end MPI compression workflow is built to convert and pack these MPI layers into YCbCr420 10bit format, to perform VVC main10 compression, and to decode/de-pack to reconstructed MPI format at decoder side. Rendering of the reference view or novel view is done by invoking the rendering module of AdaMPI with the reconstructed MPI and camera pose as input. Figure 7 illustrates the demo set up. Demo videos are to be presented during presentation of slides.

Figure 6 clips used in MPII SEI demo

Figure 7 MPII SEI demo set up

Table 1 shows the bitrates of the compressed MPI bitstreams at three quality levels, encoded by VVC main10 @30fps. High quality level setting is QPT=22, mid quality level setting is QPT=32 and low quality level setting is QPT=37.QPT is qp used to code constituent texture pictures. QP used to code constituent opacity layer is QPT+5 for all cases. The packed MPI layers of the demo clips are illustrated in Figure 8.

Table 1 VVC main10 coding of packed MPI layers

|  | image width | image height | MPI layer arrangement | constituent picture width | constituent picture height | High BR (Mbps) | Mid BR (Mbps | Low BR (Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GuitarMan | 384 | 768 | 2x8, temporal interleaved | 3072 | 1536 | 10.2 | 5.8 | 3.2 |
| Barn | 896 | 512 | 4x4, temporal interleaved | 3584 | 2048 | 12.3 | 4.1 | 1.9 |
| Breakfast | 896 | 512 | 4x4, temporal interleaved | 3584 | 2048 | 8.6 | 3.1 | 1.6 |
| CBAbasketball | 896 | 512 | 4x4, temporal interleaved | 3584 | 2048 | 8.4 | 2.5 | 1.2 |

Figure 8 Packed MPI layers of the examples. Top row: constituent texture picture, bottom row: constituent opacity picture

For novel view rendering, the virtual camera position can be set freely (within a reasonable range) at device side to create desirable immersive experience. In the demo, different example novel view paths are illustrated, as shown in Figure 9. Some detailed image quality comparisons are provided in Annex A.

Figure 9 Example of virtual view paths for novel view rendering demos

Table 2 shows the bit count of the MPI info SEI with two examples: one using non-equal depth signaling and one using equal depth signaling for the layer depths. The examples both use 16-layer MPI representation with interleaved texture/opacity packing and 4x4 layer arrangement. It can be seen that even in the case of non-equal depth layer signaling, the MPII SEI is lightweight and transmission friendly. It is reasonable to update MPI info SEI at IRAP points to adapt to content variation. In a worst case that per-frame update is required, the bitrate is up to 20kbps@30fps.

Table 2 two examples of MPI info SEI bit count

# Comparison of MIV MPI and proposed MPII SEI message

|  | MIV MPI | MPI SEI |
| --- | --- | --- |
| Definition of “MPI” in glossary | Set of pairs of texture and transparency attribute frames, each associated with an implicit constant geometry frame [5] | Set of pairs of texture and transparency layers with implicitly/explicitly signaled depth values |
| Generation of MPI | Out of scope (unknown algorithm by Interdigital in house GPU) | Out of scope (Open-source algorithm by Dolby in house GPU) |
| Encoder input | MPI format is one special input type of the MIV encoder. A special storage format (PCS [6]) is designed to store MPI in a compact way to save memory footprint. | The packed constituent picture is treated as conventional 2D video for encoder input |
| Number of MPI layers | Typically > 100 [7], equal distanced[5] | Recommend small value, e.g. 16, layers can be equal or non-equal distanced |
| Number of views supported | single view reference camera [6] | single view reference camera |
| Elemental bitstream | Texture and transparency layers are coded in two independent elemental bitstreams. The coded pictures are “atlas” with patches. | One elemental bitstream including both texture and transparency layers. |
| Decoding and post process | Decode two bitstreams following MIV spec, restore MPI representation from patches on atlas. | Decode one bitstream and restore MPI representation from packed constituent picture. |

# Conclusion

This contribution proposes a multiplane image information (MPII) SEI message be adopted for inclusion in the next version of VSEI.

# References

- Richard Tucker and Noah Snavely, "Single-view view synthesis with multiplane images", CVPR 2020 [link]

- Ben Mildenhall, Pratul P. Srinivasan, Rodrigo Ortiz-Cayon, Nima Khademi Kalantari, Ravi Ramamoorthi, Ren Ng, and Abhishek Kar. 2019. Local light field fusion: practical view synthesis with prescriptive sampling guidelines. ACM Trans. Graph. 38, 4, Article 29 (August 2019), 14 pages. https://doi.org/10.1145/3306346.3322980

- MDS22394_WG04_N00307, “Common test conditions for MPEG immersive video”, Feb. 2023

- Yuxuan Han, Ruicheng Wang and Jiaolong Yang, “Single-View View Synthesis in the Wild with Learned Adaptive Multiplane Images”, SIGGRAPH 2022.

- ISO/IEC 23090-12:2021(E)/AMD.1:2022, Information technology — Coded representation of immersive media — Part 12: MPEG Immersive video

- ISO/IEC JTC 1/SC 29/WG 4 N0271, “Test Model 15 for MPEG immersive video”

- m55089, MIV CE1-Related - Activation of a transparency attribute and application to MPI encoding

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Annex A: detailed comparison of reconstructed images with MPI representation

| GuitarMan (© Dolby) Original image at reference view | Rendered image from MPI layers without compression |
| --- | --- |
| Rendered image from MPI layers compressed by High BR | Rendered image from MPI layers compressed by Low BR |

| Original image at reference view Barn (© T. Tapie, A.Schubert, R.Gendrot, G. Briand, F.Thudor, R. Doré (Interdigital) Barn new natural content proposal for MIV, ISO/IEC JTC1/SC29/WG11 MPEG134/M56632, April 2021) |  |
| --- | --- |
| Rendered image from MPI layers without compression |  |
| Rendered image from MPI layers compressed by High BR |  |
| Rendered image from MPI layers compressed by Low BR |  |
| Original image at reference view Breakfast (© T. Tapie, A.Schubert, R.Gendrot, G. Briand, F.Thudor, R. Doré (Interdigital) Breakfast new natural content proposal for MIV, ISO/IEC JTC1/SC29/WG11 MPEG134/M56730, April 2021) |  |
| Rendered image from MPI layers without compression |  |
| Rendered image from MPI layers compressed by High BR |  |
| Rendered image from MPI layers compressed by Low BR |  |
| Original image at reference view CBAbasketball (© Xiaojie Sheng (Alibaba Group Holding Limited) New Test Sequence for MPEG-I Visual, ISO/IEC JTC1/SC29/WG04 MPEG136/M58275, October 2021) |  |
| Rendered image from MPI layers without compression |  |
| Rendered image from MPI layers compressed by High BR |  |
| Rendered image from MPI layers compressed by Low BR |  |