| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0067 |
| --- | --- |

| Title: | EE1-4.1: Neural-network loop filters with further complexity reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jay N. Shingala, Ajay Shyam, Ajat Suneja, Siddarth P Badya Tong Shao, Arjun Arora, Peng Yin, Fangjun Pu, Taoran Lu, Sean McCarthy | Email: | jay.shingala@ittiam.com {tshao, pyin}@dolby.com |
| Source: | Ittiam Systems, Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the experimental results of EE1-4.1. The models for complexity reduction in this EE test are originally proposed in JVET-AD0157. The model uses CP decomposition, fusing adjacent 1x1 convolution and split architecture for luma and chroma. The EE tests focus on two aspects: 1) test different combination of model parameters with further complexity reduction compared to NNVC 5.0 LOP (worse case block level complexity is 16.2 KMAC/Pixel); 2) 16x SIMD friendly. The new proposed model is implemented on top of NNVC common software NNVC-5.0.

The AI and RA results for the new proposed model of {Y, Cb, Cr} BD-Rate gain over NNVC-5.0 tools off are as follows:

EE1-4.1: CP-decomposition with fusing and split luma chroma (16L, 16C), worst case block level complexity of 9.87 KMAC/Pixel

- SADL, fp32:  AI { -4.14%, -7.90%, 7.64%}, RA { -4.58%, -10.14%, -8.37%}

The AI and RA BD-Rate results over NNVC-5.0 anchor (NnIntraPred and Nnlf LOP tool on) are as follows:

- SADL, fp32:  AI { 0.35%, -2.59%, -2.07%}, RA { 0.48%, -3.29%, -2.14% }

# Introduction

In NNVC 5.0, the NNLF LOP has gained about 5% BD-Rate in RA at worst case block level complexity of 16.2 KMAC/Pixel compared to VTM anchor [1]. JVET-AD0157 [2] proposes several variations of the CP fused model with split luma to further reduce the NNLF LOP complexity.

EE1-4.1 focuses on testing with different settings to explore the possibility of further complexity reduction with the following aspects:

- Different combinations of model parameters

  - Number of hidden layers

  - Number of channels in luma and chroma paths

- SIMD friendly sizes: number of channels preferably multiple of 16.

# Proposed methods

Figure 1 shows NNVC 5.0 NNLF LOP (Low Operation Point) architecture: CP-decomposition of 3x3 convolution layers and fusion of adjacent 1x1 pointwise convolutions. The 3x3 convolutions of each hidden layer are decomposed into 4 layers with rank R followed by fusion of adjacent 1x1 convolution:

- 1st layer: 1x1xKxR pointwise convolution

- 2nd layer: 3x1xRxR separable convolution

- 3rd layer: 1x3xRxR separable convolution

- 4th layer: 1x1xRxK pointwise convolution

Figure 2 shows the EE1-4.1 model: split Luma, Chroma architecture combined with CP decomposition and fusing of adjacent 1x1 conv layer. Various 16x SIMD friendly channel count combinations have been tested at about 10KMAC/Pixel worst case block-level complexity. The proposed new model setting is shown in Table 1.

3x3 conv 10xM

Leaky ReLu

3x3 conv KxL

1x1 conv MxM

Leaky ReLu

3x3 CPD KxK

1x1 conv MxK

1x1 conv KxM

Leaky ReLu

3x3 CPD KxK

1x1 conv MxK

BS info

QStep

Yx4+U+V 72x72x10

NxN

1x1 conv KxM

Leaky ReLu

3x3 CPD KxK

1x1 conv MxK

Y’x4+U’+V‘

64x64x6

N’xN’

N Hidden Layers

1x1 conv KxR

3x1 Sep conv R

1x3 Sep conv R

CP decomposition

Layers approximated with CP decomposition

Separable Convolution layers

1x1 conv MxR

1x1 conv RxM

Figure 1 NNVC 5.0 LOP: CP Decomposition + fusion of 1x1 conv layers

Ky

NxN

KC

Channel Concat

LY

NxN

LC

3x3 CPD 10xM

Leaky ReLu

3x3 CPD KYxLY

1x1 conv MxM

Leaky ReLu

3x3 CPD KxK

1x1 conv MxK

Leaky ReLu

3x3 CPD KYxKY

1x1 conv MYxKY

BS info

QStep

Yx4+U+V 72x72x10

NxN

1x1 conv KYxMY

Leaky ReLu

3x3 CPD KYxKY

1x1 conv MYxKY

Y’x4+U’+V‘ 64x64x6

N’xN’

NY Hidden Layers (Luma)

NC Hidden Layers (Chroma)

1x1 conv MCxKC

1x1 conv KCxMC

3x3 CPD KCxKC

1x1 conv MCxKC

3x3 CPD KCxKC

1x1 conv KCxMC

Leaky ReLu

3x3 CPD KCxLC

Leaky ReLu

1x1 conv KYxMY

Layers approximated with CP decomposition

Separable Convolution layers

3x3 CPD KYxKY

Rank RY CP decomposition

1x1 conv KYxRY

3x1 Sep conv RY

1x1conv RYxKY

1x3 Sep conv RY

Illustration of CP decomposition in Luma layer hidden unit

Note: Adjacent 1x1 conv layers after 3x3 CPD of hidden layers are fused into single 1x1 conv layer

Figure 2 Split luma and chroma model + CP Decomposition + fusion of 1x1 conv layer (EE1-4.1)

Table 1 EE1-4.1 Split Luma Chroma (16L, 16C) setting and complexity

| Model/Exp | K | Ky | Kc | M | My | Mc | Ry | Rc | Ny | Nc | kmac |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| split luma chroma (16L, 16C) | 32 | 16 | 16 | 64 | 48 | 16 | 16 | 16 | 10 | 6 | 9.87 |

# Simulation results

Simulations are performed based on the common test conditions for neural-network-based video coding technology for AI and RA configurations [4].

Table 2-and Table 3 show the AI and RA performances of CP fused model in EE1-1.10.1 compared against NNVC-5.0 anchor with NN tool off (NnIntraPred and Nnlf LOP disabled) for SADL fp32. It is noted that the DecT is reliable and EncT is not reliable in our simulation.

Table 2 AI performance of EE1-4.1 (SADL, fp32, NNVC-5.0 tool off)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-5.0 (NN tool off) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -3.81% | -6.09% | -6.68% | 153% | 5225% |
| Class A2 | -3.84% | -8.17% | -6.17% | 147% | 4942% |
| Class B | -3.77% | -8.09% | -8.17% | 155% | 5143% |
| Class C | -4.06% | -9.06% | -8.70% | 139% | 5025% |
| Class E | -5.49% | -7.55% | -7.74% |  |  |
| Overall | -4.14% | -7.90% | -7.64% | 149% | 5087% |
| Class D | -4.57% | -8.09% | -9.01% | 138% | 3907% |

Table 3 RA performance of EE1-4.1 (SADL, fp32, NNVC-5.0 tool off)

|  | Random Access Main10 (NN tool off) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-5.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -4.79% | -7.73% | -6.55% | 134% | 2003% |
| Class A2 | -5.00% | -9.73% | -6.47% | 143% | 2485% |
| Class B | -4.24% | -10.67% | -9.48% | 127% | 2300% |
| Class C | -4.53% | -11.59% | -9.78% | 127% | 2860% |
| Class E |  |  |  | 138% | 2025% |
| Overall | -4.58% | -10.14% | -8.37% | 132% | 2340% |
| Class D | -5.67% | -11.27% | -10.99% | 120% | 2927% |

Table 4 and Table 5 show the AI and RA performances of CP fused model in EE1-1.10.1 compared against NNVC-5.0 anchor (NnIntraPred and Nnlf LOP enabled) for SADL fp32.

Table 4 AI performance of EE1-4.1 (SADL, fp32, NNVC-5.0)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc-5.0 (SADL, int16) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.61% | -2.75% | -2.06% | 101% | 54% |
| Class A2 | 0.06% | -2.79% | -0.95% | 103% | 54% |
| Class B | 0.28% | -2.36% | -2.39% | 103% | 53% |
| Class C | 0.39% | -2.15% | -1.70% | 103% | 51% |
| Class E | 0.48% | -3.21% | -3.13% | 100% | 56% |
| Overall | 0.35% | -2.59% | -2.07% | 102% | 53% |
| Class D | 0.35% | -2.72% | -2.51% | 106% | 52% |

Table 5 RA performance of EE1-4.1 (SADL, fp32, NNVC-5.0)

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc-5.0 (SADL, int16) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.38% | -4.04% | -2.06% | 68% | 49% |
| Class A2 | 0.35% | -4.38% | -2.15% | 68% | 50% |
| Class B | 0.47% | -3.18% | -2.81% | 74% | 49% |
| Class C | 0.66% | -2.05% | -1.35% | 81% | 48% |
| Class E |  |  |  |  |  |
| Overall | 0.48% | -3.29% | -2.14% | 73% | 49% |
| Class D | 0.71% | -2.79% | -2.40% | 85% | 49% |

# Training and inference information

The models are trained based on DIV2K [5] for AI and BVI-DVC [3] for RA. The raw DIV2K image data was compressed using AI configurations to create the training examples for AI. 2 models are used for Intra and 2 models are used for Inter. Training dataset generation and training method is identical to the NNVC-5.0 low complexity operation point (LOP) filterset.

More information related to the training & inference of models can be found in the Table 6 and Table 7 below.

Table 6  Training information of split luma and chroma model + CP decomposition + fusion

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Nvidia GeForce RTX3090 |
|  | Framework: | Tensorflow 2.8.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 400 for AI 40 for RA stage 1 30 for RA stage 2 |
|  | Batch size: | 16 |
|  | Training time: | 120 hours for AI 200 hours per each RA stage |
|  | Training data information: | DIV2K(AI), BVI-DVC(RA) |
|  | Training configurations for generating compressed training data (is different to VTM CTC): | QP=20,25,30,35,40,45 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 72x72 (4 luma and 2 chroma planes) |
|  | Learning rate: | AI: - 2E-4 0-300 epochs - 2E-5 300-350 epochs - 2E-6 350-400 epochs RA stage 1: - 2E-4 0-20 epochs - 1E-4 20-30 epochs - 2E-5 30-35 epochs - 2E-6 35-40 epochs RA stage 2: - 2E-4 0-10 epochs - 1E-4 10-20 epochs - 2E-5 20-25 epochs - 2E-6 25-30 epochs |
|  | Optimizer: | ADAM |
|  | Loss function: | 12-1-1 Weighted L2 |
|  | Preprocessing: | Convert 144x144 YUV420 signal to 6 72x72 blocks. Normalize to 0~1 |
|  | Other information: |  |

Table 7  Inference information of split luma and chroma model + CP decomposition + fusion

| Network Information in Inference Stage |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | NNVC-5.0 LOP |  | Split LC (16L, 16C) |
| Mandatory | HW environment: |  |  |  |
|  | GPU Type | CPU only |  |  |
|  | Framework: | libTensorflow, SADL float, SADL int16 |  |  |
|  | Number of GPUs per Task | 0 |  |  |
|  | Number of Parameters (Each Model) | 52566 |  | 32262 |
|  | Total Parameter Number | 210264 |  | 129048 |
|  | Parameter Precision (Bits) | Floating (32 bits) / integer (16 bits) |  |  |
|  | Memory Parameter (MB) | 0.402 |  | 0.246 |
|  | MAC/Pixel (Kilo) | 16.2 |  | 9.87 |
| Optional |  |  |  |  |
|  | Total Conv. Layers | 46 |  | 72 |
|  | Total FC Layers | 0 |  |  |
|  | Total Memory (MB) |  |  |  |
|  | Batch size: |  |  |  |
|  | Patch size | 72x72 (4 luma and 2 chroma planes ) |  |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |  |
|  | Peak Memory Usage (Total) |  |  |  |
|  | Peak Memory Usage (per Model) |  |  |  |
|  | Other information: |  |  |  |

# Conclusion

This contribution shows the SIMD 16x friendly split luma-chroma architecture plus CP decomposition fused model with worst case complexity of 9.87 KMAC/pixel for (16L,16C) split in EE1-4.1. For AI case, the luma gain is about 4.1% and RA case, the luma gain is 4.6% when compared with NNVC 5.0 NN tools off. Compared to NNVC-5.0 tool on anchor, for AI case, the luma loss is 0.35% and chroma gain is more than 2%, and for RA case, the luma loss is about 0.5% with chroma gain of more than 2% for 40% complexity reduction in NNLP LOP filter.

The contribution proposes to continue EE-1.4.1 in following aspects:

- improve training for stabilization and SADL int16 quantization

# References

- J. N. Shingala, S. Kadaramandalgi, A. Shyam, T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy, “EE1: Complexity reduction on neural-network loop filter”, JVET-AD0156, Antalya, April 2023

- T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy, J. N. Shingala, S. Kadaramandalgi, A. Shyam, “EE1-related: Neural-network loop filters in EE1-1.1.2 with further complexity reduction”, JVET-AD0157, Antalya, April 2023

- Di Ma, Fan Zhang, and David, R. Bull, "BVI-DVC: A Training Database for Deep Video Compression", arXiv preprint arXiv:2003.13552 (2020)

- S. Liu, A. Segall, E. Alshina, R.-L. Liao, “Common Test Conditions and evaluation procedures for neural network-based video coding technology”, JVET-AD2016, April 2023

- R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool, https://data.vision.ee.ethz.ch/cvl/DIV2K/

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).