| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0068-v1 |
| --- | --- |

| Title: | AHG9: On extensibility of purpose syntax element in NNPFC SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Seung hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com |
| Source: | LG electronics |  |  |

_____________________________

# Abstract

With the current fixed 16 bits design for signalling the nnpfc_purpose, only 10 additional purposes can be supported in the future. It is asserted that the current signalling of nnpfc_purpose may not be future-proof considering the high interest of supporting additional purposes for NNPF shown in the previous JVET meeting.

This contribution suggests that the signalling of nnpfc_purpose to have extensibility feature to avoid the situation where additional purposes need to be signalled at the end of the syntax table as extension bits as it may defeat the purpose of signalling nnpfc_purpose as early as possible in the current syntax table.

The proposed solution to give the signalling of nnpfc_purpose extensibility feature is as follows:

- Signal using a fixed number of bits (e.g., u(8)) the information about how many bits are used for signalling the purpose syntax element. This syntax element may be called nnpfc_purpose_len.

- Change the signalling of nnpfc_purpose for u(16) to u(v) in which the number of bits for this syntax element depends on the value of nnpfc_purpose_lens.

- Specify that for bitstream conforming to v3 of VSEI specification, the value of nnpfc_purpose_len shall be equal to a fixed number (e.g., 6).

# Problem description

In the current design of neural-network post-filter characteristic (NNPFC) SEI message, the signalling of purpose of the neural-network post-filter is signalled using 16 bits in which each bit corresponds to a particular purpose. There are six purposes of NNPF have been recognized and they are specified in the following table:

Table 20 – Definition of nnpfc_purpose

| bitMask | Interpretation |
| --- | --- |
| 0x01 | General visual quality improvement |
| 0x02 | Chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| 0x04 | Resolution resampling (increasing or decreasing the width or height) |
| 0x08 | Picture rate upsampling |
| 0x10 | Bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| 0x20 | Colourization |

It is important to note that the current signalling method for nnpfc_purpose syntax element is capable to support only 16 purposes, which means there are only 10 more possible purpose can be supported in the future which may not be enough considering the interest for supporting more purposes in the future. In the 30th JVET meeting, there were already contributions that suggested additional purposes of NNPF to be supported in the NNPFC SEI message. The list of contributions and the suggested purposes to be included into NNPFC SEI message are as follow:

- JVET-AD0093: Chroma downsampling, picture rate downsampling, bitdepth downsampling

- JVET-AD0094: depth generation,

- JVET-AD0154: colour range conversion

- JVET-AD0240: deinterlacing, background replacement/blurring, film grain synthesis, video stabilization, face/object redaction, visualization for computer vision task, and video retargeting.

Considering the limited capacity for the current signalling to support future NNPF purposes, it would be desirable that the signalling is extensible.

# Discussion

If the problem of extensibility of the signalling of nnpfc_purpose is not dealt with at the finalization of v3 of the specification, when more purposes need to be supported in which the current bits signalling cannot accommodate them anymore, we may be forced to signalling additional bits for this as extension that is appear at the end of the current existing syntax element.

Such design would defeat the current design in which the information about the purpose of the NNPF can be find as early as possible. Thus, we would really encourage that this extensibility issue of signalling the purpose information is discussed, solved, and included in the v3 of the specification.

# Proposal

To enable extensibility for signalling nnpfc_purpose, we propose the following changes:

- Signal using a fixed number of bits (e.g., u(8)) the information about how many bits are used for signalling the purpose syntax element. This syntax element may be called nnpfc_purpose_len.

- Change the signalling of nnpfc_purpose for u(16) to u(v) in which the number of bits for this syntax element depends on the value of nnpfc_purpose_lens.

- Specify that for bitstream conforming to v3 of VSEI specification, the value of nnpfc_purpose_len shall be equal to a fixed number (e.g., 6).

# Text implementation

The implementation of the above proposal is as follows:

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose_len | u(8) |
| nnpfc_purpose | u(v) |
| nnpfc_id | ue(v) |
| … |  |
| } |  |

nnpfc_purpose_len specifies the number of bits for signalling the syntax element nnpfc_purpose. The value of nnpfc_purpose_len shall be equal to 6, in bitstreams conforming to this edition of this document.

nnpfc_purpose indicates the purpose of the NNPF as specified in Table 20, where ( nnpfc_purpose & bitMask ) not equal to 0 indicates that the NNPF has the purpose associated with the bitMask value in Table 20. When nnpfc_purpose is greater than 0 and ( nnpfc_purpose & bitMask ) is equal to 0, the purpose associated with the bitMask value is not applicable to the NNPF. When nnpfc_pupose is equal to 0, the NNPF may be used as determined by the application.

The value of nnpfc_purpose shall be in the range of 0 to 63, inclusive, in bitstreams conforming to this edition of this document. Values of 64 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 64 to 65 535, inclusive. The length of the syntax element is equal to the value of nnpfc_purpose_len.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).