| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0071-v2 |
| --- | --- |

| Title: | AHG12 - Decoder-side Intra Mode Derivation Merge |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Saverio Blasi Ivan Zupancic Jani Lainema | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes to merge the Decoder-side Intra Mode Derivation (DIMD) information from neighbouring blocks to predict the current block. When neighbouring blocks encoded with DIMD are available, the DIMD histograms are combined to form a new DIMD Merge histogram for the current block. DIMD Merge modes and weights are computed based on this merged histogram.

It is reported that adding the proposed DIMD Merge to ECM in AI configuration provides -0.08%, 0.01% and 0.07% BD-rate impact for Y, U and V, respectively.  In RA configuration the impact is reportedly x%, y%, z%  for Y, U and V, respectively. The encoder and decoder runtimes are reportedly unchanged with respect to ECM.

# Proposed approach

## Background

ECM includes Decoder-side Intra Mode Derivation (DIMD). When using DIMD, up to five DIMD modes are derived, by analysing the directionality of the content in the surrounding of the current block. A Histogram of Gradients (HoG) is computed on a three-sample wide/high L-shaped template formed of already reconstructed samples. This is obtained using the Sobel filter, accumulating the magnitudes of all gradients at a given direction, for all the samples within the gray region in Figure 1. The directions with highest cumulative magnitude are selected as main and secondary DIMD modes. The predictors obtained with the DIMD modes are then blended to form the final DIMD prediction. Uniform or spatial blending is used, where the DIMD predictors are combined with a Planar predictor, using weights that depend on the relative magnitudes of the modes in the histogram of gradients.

Figure 1 - the DIMD template to form the HoG

## DIMD Merge

When using DIMD Merge, the DIMD information extracted from neighbouring blocks is used to compute the intra prediction for the current block. In particular, a new Merged Histogram of Gradients (MHoG) is computed for the current block based on the HoGs of neighbouring blocks. Only neighbouring blocks encoded with DIMD or with DIMD Merge are considered.

When a single DIMD or DIMD Merge neighbouring block is available, then its histogram of gradients is used to form the MHoG for the current block. If more than one DIMD or DIMD Merge neighbouring blocks are available, the corresponding histograms are combined by means of amplitude averaging to derive the MHoG.

Finally, the MHoG is used to compute intra-prediction modes and weights, as in conventional DIMD. The directional modes and their weights corresponding to the five highest amplitudes in the MHoG are selected, and the corresponding predictors are blended as in conventional DIMD.

## Integration in ECM

DIMD Merge is only available as an option if the current block has at least one neighbour that is coded with DIMD or DIMD Merge. Under these conditions, usage of DIMD Merge is then signalled with a CABAC coded CU level flag. One new CABAC context was included to support coding of the DIMD Merge flag. When it comes to signalling, DIMD Merge is considered a sub-mode of DIMD. That is, the DIMD Merge flag is only signalled if DIMDflag = 1 and if there are neighbouring CUs that are coded with DIMD or DIMD Merge.

# Experimental results

The tables below summarize the coding efficiency and runtime impact of the proposed method in ECM 9.0 using the JVET common test conditions.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.07% | -0.02% | 100% | 102% |
| Class A2 | -0.11% | -0.06% | -0.04% | 97% | 100% |
| Class B | -0.07% | 0.05% | 0.05% | 99% | 101% |
| Class C | -0.06% | -0.01% | 0.10% | 100% | 100% |
| Class E | -0.06% | -0.01% | 0.24% | 99% | 98% |
| Overall | -0.08% | 0.01% | 0.07% | 99% | 100% |
| Class D | -0.03% | -0.08% | 0.21% | 100% | 103% |
| Class F | -0.08% | 0.02% | 0.12% | 98% | 100% |
| Class TGM | -0.01% | -0.03% | 0.04% | 97% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.04% | 0.02% | -0.02% | 100% | #NUM! |
| Class C | 0.00% | -0.28% | -0.03% | 99% | #NUM! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | -0.22% | -0.01% | 99% | #NUM! |
| Class F | -0.02% | 0.06% | -0.03% | 101% | #NUM! |
| Class TGM | -0.02% | 0.01% | 0.02% | 98% | 99% |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).