| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting: Geneva, CH, 11–19 July 2023 | Document: JVET-AE0072-v1 |
| --- | --- |

| Title: | [AHG11] On residual adjustments for NNLF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhenyu Dai Yue Yu Haoping Yu Dong Wang | Tel: Email: | daizhenyu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution proposes residual adjustments for NNLF’s output based on low complexity operation point (LOP) NN-based filters in the NNVC reference software. In JVET-AD0156[1], a residual network based NNLF was proposed to predict the residual between the reconstructed image and the original image to get better coding performance. It is proposed to adjust the residuals at the frame level, which is signaled by a flag.

Compared with the NNVC-5.1rc2 default configuration (enabling NN-intra and low complexity NN-filter), it is reported that the proposed method can further improve the coding efficiency {Y, U, V, EncT, DecT} with the following results:

RA: Y -0.05%, U -0.59%, V -0.49%, EncT 100%, DecT 99%.

LDB: Y -0.20%, U -1.44%, V -0.90%, EncT 100%, DecT 99%.

AI: Y -0.03%, U -0.32%, V -0.24%, EncT 100%, DecT 98%.

Compared with the NNVC-5.1rc2 anchor, it is reported that the proposed method can further improve the coding efficiency {Y, U, V, EncT, DecT} with the following results:

RA: Y x.xx%, U x.xx %, V x.xx %, EncT xx%, DecT xx %;

LDB: Y x.xx %, U x.xx %, V x.xx %, EncT xx %, DecT xx %.

AI: Y x.xx %, U x.xx %, V x.xx %, EncT xx %, DecT xx %.

# Introduction

JVET-AD0156[1] presented a low complexity design for the neural network-based in-loop filter. The proposed system has a constrained memory size and it was adopted as the low complexity operation point (LOP) NN-based filter in the NNVC reference software.

The network structure of JVET-AD0156[1] is shown in Fig. 1. Along with the reconstructed image, additional side information is also fed into the network, such as the deblocking boundary strengths and QStep.

Fig. 1: Network architecture of JVET-AD0156

# Proposed method

This contribution proposes a two-step residual adjustment method. Firstly, a frame level residual offset can be used to adjust the output of the NNLF in the inference stage.

More specifically, the residual is adjusted by reducing the magnitude of the residual at each pixel by a small offset value. In this contribution, the offset candidates are{1, 2}. For example, if a residual adjustment is performed with an offset value of 1, all positive residual values are decremented by 1, while all negative residual values are incremented by 1. An example of residual adjustment with offset value 1 is shown in Fig. 2.

Fig. 2: An example of a residual offset adjustment

Secondly, a block level residual averaging can be used to adjust the output of the NNLF in the inference stage.

More specifically, for each CTU in the current frame, the average residual value can be derived by computing the average of the residual values for all pixels. And then, all the residual values are decremented by the average residual value so that the DC component of the residual signals is close to zero. Since the calculation can be done both in the encoder and decoder, there is no need to signal the average residual value. An example of residual averaging with an average value of 2 is shown in Fig. 3.

Fig. 3: An example of residual averaging adjustment

Two frame level flags are signalled to indicate whether the residual adjustment method is true for luma and chroma, respectively. If the corresponding flag is true, another frame level flag is signalled to indicate which offset value is used for adjusting the corresponding residual colour component(s).

# Experiment results

The proposed method is implemented on top of LOP NN-based filters in the NNVC5.1rc2 reference software. The simulations were performed by following the NNVC common test condition (CTC)[2], and the SADL int16 results are reported.

Table 1. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC5.1rc2 (NNIntra + LOP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.32% | -0.84% | 100% | 99% |
| Class A2 | -0.09% | -0.40% | -0.15% | 100% | 99% |
| Class B | -0.03% | -0.52% | -0.59% | 100% | 99% |
| Class C | -0.02% | -1.02% | -0.37% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.59% | -0.49% | 100% | 99% |
| Class D | -0.02% | -0.96% | -0.82% | 100% | 99% |
| Class F | -0.06% | -0.56% | -0.26% | 101% | 99% |
| Class H |  |  |  |  |  |

Table 2. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC5.1rc2 (NNIntra + LOP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | -1.26% | -1.15% | 99% | 98% |
| Class C | -0.24% | -1.67% | -0.89% | 100% | 100% |
| Class E | -0.21% | -1.45% | -0.48% | 101% | 98% |
| Overall | -0.20% | -1.44% | -0.90% | 100% | 99% |
| Class D | -0.08% | -0.88% | -0.70% | 101% | 100% |
| Class F | -0.17% | -1.43% | -0.48% | 101% | 98% |
| Class H |  |  |  |  |  |

Table 3. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC5.1rc2 (NNIntra + LOP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.30% | -0.26% | 99% | 98% |
| Class A2 | -0.05% | -0.16% | -0.19% | 100% | 99% |
| Class B | -0.02% | -0.41% | -0.27% | 100% | 98% |
| Class C | -0.01% | -0.37% | -0.32% | 100% | 98% |
| Class E | -0.02% | -0.27% | -0.09% | 99% | 97% |
| Overall | -0.03% | -0.32% | -0.24% | 100% | 98% |
| Class D | 0.00% | -0.44% | -0.93% | 101% | 100% |
| Class F | -0.05% | -0.37% | -0.25% | 101% | 97% |
| Class H |  |  |  |  |  |

Table 4. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC5.1rc2 (anchor) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -6.64% | -11.49% | -9.91% | 133% | 7195% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -7.59% | -10.35% | -10.46% | 133% | 7772% |
| Class F | -3.12% | -6.42% | -5.19% | 156% | 10305% |
| Class H |  |  |  |  |  |

Table 5. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC5.1rc2 (anchor) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -5.46% | -12.65% | -9.82% | 130% | 6112% |
| Class E | -5.09% | -5.56% | -5.63% | 165% | 10080% |
| Overall |  |  |  |  |  |
| Class D | -6.61% | -11.58% | -10.46% | 128% | 6293% |
| Class F |  |  |  |  |  |
| Class H |  |  |  |  |  |

Table 6. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC5.1rc2 (anchor) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -7.35% | -10.09% | -10.22% | 201% | 3638% |
| Class E | -10.31% | -9.30% | -8.93% | 205% | 4962% |
| Overall |  |  |  |  |  |
| Class D | -7.57% | -8.52% | -10.19% | 188% | 3762% |
| Class F |  |  |  |  |  |
| Class H |  |  |  |  |  |

# Conclusions

This contribution proposes a residual adjustment method on top of the low complexity operation point (LOP) NN-based filters in the NNVC reference software. Simulation results show that it can provide good performance without increasing the computation complexity. It is suggested to adopt this method into LOP NN-based filters in the NNVC reference software.

# References

- EE1-1.1: Complexity Reduction on Neural-Network Loop Filter, JVET-AD0156, [J. N. Shingala, A. Shyam, A. Suneja, S. Badya (Ittiam), T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, Sean McCarthy (Dolby)].

- Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AD2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).