| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0074-v2 |
| --- | --- |

| Title: | Non-EE2: Improvement on cross-component prediction merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hang Huang, Yue Yu, Haoping Yu, Dong Wang | Email: | huanghang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution presents improvement methods for the cross-component prediction (CCP) merge mode. The CCP merge mode is proposed to be applied to the linear-model part of the chroma fusion mode. The cross-component model parameters applied to the linear-model predictor of chroma fusion mode are inherited from the spatial adjacent, spatial non-adjacent, history-based, or default CCP models. In addition, a filter flag for local boosting is also proposed to be inherited in the CCP merge mode. Therefore, the prediction samples generated by the CCP merge mode may also be filtered with neighboring samples.

On top of ECM-9.0, the simulation results of the proposed method are reported as follows:

AI: {-0.01%, -0.34%, -0.33%, 100.9%, 99.9%}; RA:{ -0.xx%, -0.xx%, -0.xx%, xxx%, xxx%}

Introduction

Cross-component prediction (CCP) methods, including cross-component linear model (CCLM), convolutional cross-component model (CCCM), gradient linear model (GLM), and their variants have been adopted in ECM to exploit the cross-component correlation.

In the CCP merge mode, a candidate list is created, which includes CCP models collected from the previous blocks coded by CCLM, MMLM, CCCM, GLM, chroma fusion, and CCP merge modes. Then one candidate in the list will be selected as the CCP model for the current block.

In the chroma fusion method, the DM mode, DIMD mode, and the four default modes can be fused with the linear model (LM) mode as follows:

pred=w0*pred0+w1*pred1+(1≪(shift-1))≫shift

where pred0 is the predictor obtained by applying the non-LM mode, pred1 is the predictor obtained by applying the LM mode (MM-CCLM or MM-CCCM) and pred is the final predictor of the current chroma block.

In ECM-9.0, a flag is signaled to indicate whether a 3x3 local low-pass filter is applied or not to a block coded with MM-CCLM/MM-CCCM.

Proposed methods

In this contribution, the CCP merge method is proposed to be applied to the LM part in chroma fusion mode, so pred1 can be generated by using the inherited cross-component model parameters. This merge method is similar to the CCP merge mode, and the process is summarized as follows.

A CCP merge candidate list is constructed from the spatial adjacent, spatial non-adjacent, or history-based candidates. It is noted that only parameters of multi-model LM modes are added to the list. After including these candidates, the default models are further included to fill the remaining empty positions in the merge list. In order to remove redundant CCP models from the list, a pruning operation is applied. After constructing the list, the CCP models in the list are reordered depending on the SAD costs, which are obtained using the neighbouring template of the current block.

In addition, the predictor obtained by applying the CCP merge mode may be filtered based on the inherited filter flag. As shown in Fig.1, a 3×3 low-pass filter is applied to filter prediction samples generated by MM-CCLM/MM-CCCM. For a sample at a top/left boundary, the filtering window may involve neighboring reconstructed samples. For inner samples, the filtering window only involves prediction samples, which may be padded. A flag is signaled to indicate whether filtering is applied or not to a block coded with MM-CCLM/MM-CCCM.

Fig. 1 The filter on samples of MM-CCLM/MM-CCCM

A flag is signaled to indicate whether the CCP merge method is applied or not to the LM part in chroma fusion mode when the current block is coded in chroma fusion mode. If this flag is true, an index is further signaled to indicate which candidate model is used by the current block. In addition, this method is not allowed for the current chroma block coding when the current CU is coded by ISP with a single tree, or the current chroma block size is less than or equal to 16.

Experimental results

The proposed method is implemented on top of ECM-9.0 [1] and tested following the common test conditions [2]. The simulation results are summarized in Table 1.

Table 1. Simulation results of the proposed method

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.49% | -0.46% | 100.8% | 99.8% |
| Class A2 | 0.00% | -0.37% | -0.36% | 102.0% | 99.5% |
| Class B | -0.01% | -0.29% | -0.35% | 100.6% | 99.6% |
| Class C | 0.00% | -0.35% | -0.25% | 100.9% | 100.6% |
| Class E | -0.03% | -0.22% | -0.21% | 100.5% | 100.0% |
| Overall | -0.01% | -0.34% | -0.33% | 100.9% | 99.9% |
| Class D | 0.04% | -0.16% | -0.20% | 101.8% | 99.9% |
| Class F | 0.01% | -0.18% | -0.38% | 101.1% | 99.3% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.01% | -0.13% | -0.22% | 100.6% | 100.4% |
| Class C | -0.01% | -0.12% | -0.10% | 100.4% | 100.6% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.04% | -0.40% | 0.13% | 99.9% | 101.1% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Conclusion

This contribution proposes improvements to the cross-component prediction merge mode. The results show that the gain of the CCP merge mode can be further improved. It is suggested to further study this proposal in EE2.

References

- ECM9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, April, 2023.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).