| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0079-v1 |
| --- | --- |

| Title: | AHG8/AHG9: Source picture timing information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sean McCarthy Gary J. Sullivan Peng Yin | Email: | sean.mccarthy@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a source picture timing information (SPTI) SEI message for inclusion in the next version of VSEI for use with VVC, HEVC, and AVC. The proposed SPTI SEI message indicates the actual temporal distance between source pictures associated with the corresponding decoded output pictures prior to encoding, e.g., for camera-captured content, the temporal distance between source pictures is the difference between the time at which an image sensor was exposed to produce a source picture associated with the current decoded picture and the time at which the image sensor was exposed to produce the source picture associated with a previous decoded picture in output order. The proposed SPTI SEI message conveys information that supports various post-decoder processes. As examples, the proposed SPTI SEI message could facilitate the following: playback of slow-motion video at actual capture rate; playback of high-speed and time-lapse imaging for human viewing whilst providing data for scientific, industrial, and machine analysis; indicating synthesized pictures and corresponding timing information for frame rate conversion applications; playback of reverse-time video in original causal time; conveyance of data to facilitate machine analysis of actual motion and physical dynamics for applications including high-speed imaging, security, defense, traffic and collision analysis; and conveyance of data to facilitate conventional and AI-based post-decode processing such as frame rate conversion and synthesis of video using generative AI technologies, for example.

# Introduction

JVET-AC0141 [1] proposed an alternative picture timing SEI message. JVET-AD0161 [2] proposed an alternative output timing hint SEI message. For both contributions, the proposed SEI message is intended to indicate the actual motion speed of the content at capture time for the case in which the video bitstream contains slow motion scenes.

The JVET meeting notes record that it might generally be beneficial to know the timing scale factor between the actual capture timing and the output timing, e.g., if in a video sequence only one part is contained in “slow motion” modes, as often in sports events. The meeting notes also record that the proposed SEI messages are intended to influence the output time of VVC decoding and it was asked how to ensure HRD functions correctly.

The current proposed SPTI SEI message is intended to address similar but broader range of use cases as JVET-AC0141 and JVET-AD0161 whilst avoiding all HRD conformance issues.

The proposed SPTI SEI message conveys information only about when source pictures were captured or otherwise created. It does not influence timing of decoded output pictures and thus does not impact HRD conformance.

# Problem Statement

Example scenarios where source and decoded output picture timing differ

Figure 1A-F illustrate scenarios in which the output timing of decoded pictures is different from the timing with which source pictures were captured or otherwise created. A receiver or post-decode process would not be able to retrieve the original source picture timing from the timing of output pictures. However, such information could be conveyed within the same bitstream as the coded video by the proposed SPTI SEI message.

Figure 1. Example scenarios in which the output timing of decoded pictures is different from the timing of source pictures: A) conversion of source frame rate to slow motion output at the same frame rate prior to encoding; B) frame rate conversion over the same output duration prior to encoding; C) encoding of a high-speed source for output of decoded pictures at standard frame rate; D) encoding of a time-lapse source for output of decoded pictures at standard frame rate; E) encoding of sporadic or event-driven source video for output of decoded pictures at standard frame rate; and F) temporal reversal of source video prior to encoding. In panels A-B, black bars denote decoded output pictures corresponding to original source pictures. Gray bars indicate decoded output pictures corresponding to synthesized pictures.

Figure 1A illustrates a typical slow motion playback scenario. The temporal distance between decoded output pictures corresponding to original source pictures (black bars) is increased relative to the temporal distance between original source pictures. Figure 1B illustrates frame rate conversion in which the temporal distance between decoded output pictures corresponding to original source pictures is the same as the temporal distance between original source pictures. In slow motion processing and frame rate conversion, additional pictures (gray bars) are typically synthesized (often using AI methods) and inserted prior to encoding to produce smoother motion.

When decoded output pictures correspond to both original source pictures and synthesized pictures, it can be beneficial to have a means of indicating which pictures correspond to original source pictures and which to synthesized pictures, in addition to providing a means of indicating source picture timing.

Figure 1C illustrates a special case of slow-motion output for high-speed imaging. (Some production high-speed cameras currently advertise 9600 fps at 2560x1664 resolution for use in scientific, industrial, automotive, and media applications [3].) Figure 1D illustrates encoding of time-lapse source content. Figure 1E illustrates encoding of event-driven source content, for example, source video used in security applications. In high-speed imaging, time-lapse applications, and event-driven use cases, it can be beneficial that the timing of decoded output pictures corresponds to standard frame rates to facilitate human inspection whilst information contained in an SPTI SEI message could be used to facilitate scientific and machine analysis.

Figure 1F illustrates a “rewind” use case in which decoded pictures are output in reverse order relative to the corresponding source pictures. Temporal reversal of source video is used in media applications for artistic effect and storytelling, as examples.

Another example use case (not shown in Figure 1) is freeze frame in which a source picture corresponds to more than one consecutive decoded output picture. Freeze frame cannot typically be determined by analysis of the decoded output pictures because the decoded output pictures might not be bit-exact duplicates of each other as a result of coding artefacts.

Example applications

As illustrated in Figure 2, source picture timing information conveyed by the proposed SPTI SEI message can support several categories of applications:

–	Control of playback of decoded video: Examples include facilitating playback of slow-motion, high-speed, time-lapse, reverse motion, and freeze frame coded video for human viewing on commonly available consumer and professional displays. In such applications, an intermediate buffer might be required to store decoded output pictures before playback.

–	Machine analysis: Examples include determining the speed of a car; determining cardiac timing in medical imaging; tracking of athletes and balls in sports video; modelling of physics in a scene to facilitate integration in gaming; forensic analysis to detect missing or deleted source pictures in a coded video bitstream; and differentiating original source pictures from synthesized pictures in the coded bitstream.

–	Auxiliary input for post-processing: Examples include frame rate conversion; synthesis of video using generative AI technologies; and motion-aware spatial scaling.

Figure 2. Example applications facilitated by source picture timing information conveyed by the proposed SPTI SEI message, including: buffered playback of decoded output pictures with timing different from timing otherwise indicated in the coded bitstream; modification of output pictures, for example, motion-aware spatial and temporal scaling; and machine analysis of source video for scientific, industrial, and forensic applications, as examples.

# Proposal

8.xx 	Source picture timing information SEI message

8.xx.1	Source picture timing information SEI message syntax

| source_picture_timing_info( payloadSize ) { | Descriptor |
| --- | --- |
| spti_cancel_flag | u(1) |
| if( !spti_cancel_flag ) { |  |
| spti_persistence_flag | u(1) |
| spti_source_picture_timing_type | u(8) |
| spti_source_timing_equals_output_timing_flag | u(1) |
| if( !spti_source_timing_equals_output_timing_flag ) { |  |
| spti_time_scale | u(32) |
| spti_num_units_in_elemental_source_picture_interval | u(32) |
| spti_max_sublayers_minus_1 | u(3) |
| for( i = 0; i <= spti_max_sublayers_minus1; i++ ) { |  |
| spti_sublayer_source_picture_interval_scale_factor[ i ] | ue(v) |
| spti_sublayer_synthesized_picture_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

8.xx.2	Source picture timing information SEI message semantics

The source picture timing information (SPTI) SEI message indicates the temporal distance between source pictures associated with the corresponding decoded output pictures prior to encoding, e.g., for camera-captured content, the temporal distance between source pictures is the difference between the time at which an image sensor was exposed to produce a source picture associated with the current decoded picture and the time at which the image sensor was exposed to produce the source picture associated with a previous decoded picture in output order.

spti_cancel_flag equal to 1 indicates that the SPTI SEI message cancels the persistence of any previous SPTI SEI message in output order that applies to the current layer. spti_cancel_flag equal to 0 indicates that source picture timing information follows.

spti_persistence_flag specifies the persistence of the SPTI SEI message for the current layer.

spti_persistence_flag equal to 0 specifies that the SPTI SEI message applies to the current decoded picture only.

spti_persistence_flag equal to 1 specifies that the SPTI SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an SPTI SEI message is output that follows the current picture in output order.

spti_source_picture_timing_type indicates the timing relationship between source pictures and corresponding decoded output pictures as specified in Table 1, where ( spti_source_picture_timing_type & bitMask ) not equal to 0 indicates that the timing relationship has the interpretation associated with the bitMask value in Table 1. When spti_source_picture_timing_type is greater than 0 and ( spti_source_picture_timing_type & bitMask ) is equal to 0, the interpretation associated with the bitMask value is not applicable to the SPTI SEI message. When spti_source_picture_timing_type is equal to 0, the timing relationship may be specified by the application.

The value of spti_source_picture_timing_type shall be in the range of 0 to 127, inclusive, in bitstreams conforming to this edition of this document. Values of 128 to 255, inclusive, for spti_source_picture_timing_type are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore SPTI SEI messages with spti_source_picture_timing_type in the range of 128 to 255, inclusive.

Table 1 – Interpretation of spti_source_picture_timing_type

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Slow motion |
| 0x02 | Frame rate conversion |
| 0x04 | High-speed imaging |
| 0x08 | Time-lapse imaging |
| 0x10 | Temporal reversal |
| 0x20 | Still image / freeze frame |
| 0x40 | Sporadic or event-driven |

The variable temporalReversalFlag is equal to ( spti_source_picture_timing_type & 0x10 )? 1 : 0.

spti_source_timing_equals_output_timing_flag equal to 1 indicates the timing of source pictures is the same as the timing of corresponding decoded output pictures. spti_source_timing_equals_output_timing_flag equal to 0 indicates the timing of source pictures might not be the same as the timing of corresponding decoded output pictures.

When spti_source_timing_equals_output_timing_flag is equal to 1 and a picture timing SEI message is present for the current picture, source picture timing could be determined from information conveyed in the picture timing SEI message.

spti_time_scale specifies the number of time units that pass in one second. The value of spti_time_scale shall not be equal to 0. For example, a time coordinate system that measures time using a 27 MHz clock has an spti_time_scale of 27,000,000.

spti_num_units_in_elemental_source_picture_interval specifies the number of time units of a clock operating at the frequency spti_time_scale Hz that corresponds to the indicated elemental source picture interval of consecutive pictures in output order in the CLVS.

The indicated elemental source picture interval, also to be denoted by the variable ElementalSourcePictureInterval, in units of seconds, is equal to the quotient of spti_num_units_in_elemental_source_picture_interval divided by spti_time_scale. For example, to represent an elemental source picture interval equal to 0.04 seconds, spti_time_scale may be equal to 27,000,000 and spti_num_units_in_elemental_source_picture_interval may be equal to 1,080,000.

spti_max_sublayers_minus_1 plus 1 specifies the maximum number of temporal sublayers that may be present in the CLVS.

spti_sublayer_source_picture_interval_scale_factor[ i ], when present, specifies a scale factor used in determining the source picture interval of corresponding consecutive pictures in output order in the CLVS having TemporalId less than or equal to i. The value 0 may be used to indicate that the source picture corresponding to the current decoded output picture is identical to the source picture corresponding to the previous decoded output picture.

The indicated source picture interval associated with output pictures having TemporalId less than or equal to i, denoted by the variable SourcePictureInterval[ i ], in units of seconds, is derived as follows:

SourcePictureInterval[ i ] = ElementalSourcePictureInterval * spti_sublayer_source_picture_interval_scale_factor[ i ]

When picture n is a picture that is output having TemporalId less than or equal to i and is not the first picture of the bitstream that is output, the value of the variable SourcePictureTime[ n ] is derived as follows:

–	If temporalReversalFlag is equal to 0, SourcePictureTime[ n ] = SourcePictureTime[ previousPicInOutputOrder ] + SourcePictureInterval[ i ]

–	Otherwise, temporalReversalFlag is equal to 1, SourcePictureTime[ n ] = SourcePictureTime[ previousPicInOutputOrder ] - SourcePictureInterval[ i ]

where previousPicInOutputOrder is the last picture that is output having TemporalId less than or equal to i that precedes picture n in output order (if any). When the value of SourcePictureTime[ 0 ] is not provided by external means not specified in this document, the value of SourcePictureTime[ 0 ] is inferred to be 0.

spti_sublayer_synthesized_picture_flag[ i ], when present, equal to 1 indicates that decoded output pictures belonging to the ith temporal sublayer are synthesized and do not correspond to unmodified original source pictures. spti_sublayer_synthesized_picture_flag[ i ] equal to 0 provides no such indication. When not present, the value of spti_sublayer_synthesized_picture_flag[ i ] is inferred to be equal to 0.

# References

- Han Boon Teo, Jingying Gao, Chong Soon Lim, Kiyofumi Abe, “AHG9: Alternative Picture Timing SEI”, JVET-AC0141, 29th Meeting, by teleconference, 11–20 January 2023

- Han Boon Teo, Jingying Gao, Chong Soon Lim, Kiyofumi Abe, “AHG9: Alternative Output Timing Hint SEI”, JVET-AD0161, 30th Meeting, Antalya, TR, 21–28 April 2023

- Phantom/AMETEK T4040 camera main Web Page, accessed June 24, 2023, https://www.phantomhighspeed.com/products/cameras/tseries/t4040

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).