| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0080-v1 |
| --- | --- |

| Title: | AHG9: Generative Face Video SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sean McCarthy, Peng Yin, Guan-Ming Su, Anustup Kuma Choudhury, Walt Husak | Email: | {pyin, smcca}@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a generative face video (GFV) SEI message for inclusion in the next version of VSEI for use with VVC. The proposed GFV SEI message conveys information for facial features extracted from source video prior to encoding that can be applied to a previously decoded reference picture to reconstruct a face picture at the current time. The proposed GFV SEI message also specifies a neural network, for example, a generative adversarial network (GAN), that could be used to generate face pictures based on the face feature information conveyed in the GFV SEI message. As noted in previous related JVET contributions JVET-AC0088 and JVET-AD0051, the GFV SEI message is intended to enable face picture reconstruction processes to use VVC-coded pictures as reference pictures. It is also noted that extracting face features on the encoder-side can reduce receiver-side complexity. The present contribution proposes additions and refinements to previously proposed syntax and semantics to address issues noted by experts during previous JVET meetings. The proposed GFV SEI message is intended for ultra low bitrate face video compression applications such as video conferencing, live entertainment, and face animation.

# Introduction

The objective of generative face video is to create a picture that matches the original source picture (called the driving picture) using only a reference picture (called a key picture) and information about how face features changed between the driving picture and key picture. Face features are modelled as a compact description of positions of eyes, tilt of head, state of eyes (open or closed), etc. Usually, the process of animating the key picture with face features from driving pictures is performed by a neural network (called a Generator) such as a GAN.

The objective of the proposed GFV SEI message is to enable techniques of generative face video production to be combined with conventional video coding to enable very low bitrate visual communications. In this use case, the inputs to the neural network are as follows: 1) face feature information extracted from key and driving pictures prior to encoding; and 2) a previously decoded output picture corresponding to the key picture. Optionally, a previously decoded picture corresponding to the driving picture can also be input to the neural network to improve face features, such as wrinkles on a forehead that would not be anticipated to be generated by the neural network, or handle background changes, for example.

Versions of a GFV SEI message were proposed at the 29th and 30th JVET meetings (JVET-AC0088 [1] and JVET-AD0051 [2], respectively). During discussion at the 30th JVET meeting, the following comments were recorded in the meeting notes:

- It was asked what the amount of interoperability is that this SEI message would provide? More precisely, would it still have benefit if not exactly one of the mentioned end-to-end algorithms with perfectly matching encoder and decoder would be used.

- It appears necessary to provide more information what operation a decoding device should perform (if possible). For example, what is a decoder expected to do with the keypoints, and how is the texture from the base picture combined with the keypoints to produce the output?

- From the current proposal, it appears that it is expected to have an aligned pair of encoder and decoder. A question would be if this would require to signal which encoder has produced the bitstream.

- This is generally asserted to be a very interesting direction with high potential e.g. for extreme compression of facial videos, but further study appears necessary how it fits into the general concept of SEI messages which typically don’t describe a detailed decoding process.

# Proposal

The proposed GFV SEI message includes 2 aspects that are intended to address issues noted in previous JVET meetings:

- Aspect 1: Specifies a neural network, denoted as Generator( ), to address interoperability and alignment between encoder and receivers. It is proposed to re-use syntax and semantics specified in the NNPFC SEI message for this purpose.

- Aspect 2: Provides information on what a receiver is expected to do with the face feature information conveyed in the proposed GFV SEI message. Specifically, the inputs and outputs to the neural network, Generator( ), are described.

The proposed GFV SEI message also provides a 3rd aspect to improve compression efficiency for facial feature information:

- Aspect 3: Provides for representation of face feature information as 2D keypoints coded in either absolute coordinates or differentially. Differential coding could be beneficial when a large number of 2D keypoints are signalled in the proposed GFV SEI message.

The pseudocode for use of the Generator( ) is as follows:

Parse and process the GFV SEI message {
	if the current decoded output picture corresponds to a key picture {
		store the decoded 2D keypoints representing the face feature information for the key picture;
		store the current decoded output picture corresponding to the key picture;
	}
	else /*the current decoded output picture corresponds to a driving picture*/ {
		store decoded 2D keypoints representing the face feature information for the driving picture;
		optionally, store the current decoded output picture corresponding to the key picture;
		prepare input to Generator( ):
			decoded picture corresponding to key picture;
			decoded 2D keypoints for key picture;
			decoded 2D keypoints for driving picture;
			optionally, decoded picture corresponding to driving picture
		apply inputs to Generator( ) to generate an output picture that approximates the driving picture;		Store generated output picture
	}
}

8.1 	General

| Table 4 – Persistence scope of SEI messages (informative) |  |
| --- | --- |
| SEI message | Persistence scope |
| … | … |
| Generative face video | The picture associated with the SEI message |
| … | … |

8.xx 	Generative face video SEI message

8.xx.1	Generative face video SEI message syntax

| generative_face_video( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_key_pic_flag /*indicate if current decoded output picture is a key picture*/ | u(1) |
| if( gfv_key_pic_flag ) { /*specify Generator( )*/ |  |
| gfv_nn_base_flag | u(1) |
| gfv_nn_mode_idc | ue(v) |
| if( gfv_nn_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_reserved_zero_bit_a | u(1) |
| gfv_nn_tag_uri | st(v) |
| gfv_nn_uri | st(v) |
| } |  |
| } else /* current decoded output picture is a driving picture*/ |  |
| gfv_drive_pic_flag /*indicate if DrivePicture is input to Generator( )*/ | ue(v) |
| /*Face feature information is coded as 2D keypoints*/ |  |
| gfv_log2_kp_quant_factor | ue(v) |
| gfv_num_kp_minus1 | ue(v) |
| gfv_kp_pred_flag | ue(v) |
| for( i = 0; i <= gfv_num_kp_minus1; i++ ) |  |
| if(!gfv_kp_pred_flag ) { |  |
| gfv_kp_grid_pos_x[ i ] | ue(v) |
| gfv_kp_grid_pos_y[ i ] | ue(v) |
| } else { |  |
| gfv_kp_grid_pos_dx[ i ] | se(v) |
| gfv_kp_grid_pos_dy[ i ] | se(v) |
| } |  |
| gfv_kp_local_matrix_flag | u(1) |
| if( gfv_kp_local_matrix_flag ) |  |
| for( i = 0; i <= gfv_num_kp_minus1; i++ ) |  |
| for( j = 0; j < 4; j++ ) |  |
| gfv_kp_local_matrix[ i ][ j ] | ue(v) |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( gfv_key_pic_flag ) |  |
| if(gfv_nn_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| gfv_nn_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

8.xx.1	Generative face video SEI message semantics

The generative face video (GFV) SEI message indicates face feature information for source pictures prior to encoding and specifies a neural network, denoted as Generator( ), that may be used to generate novel output pictures using the indicated face feature information and previously decoded output pictures. A Generator( ) may be a generative adversarial network (GAN), for example.

Use of this SEI message requires the definition of the following variables:

- Input picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	Luma sample array keyCroppedYPic and chroma sample arrays keyCroppedCbPic and keyCroppedCrPic for a decoded output picture, denoted as KeyPicture, corresponding to a source key picture.

- Luma sample array driveCroppedYPic and chroma sample arrays driveCroppedCbPic and driveCroppedCrPic for a decoded output picture, denoted as DrivePicture, corresponding to a source driving picture.

–	Bit depth BitDepthY for the luma sample array of the input pictures.

–	Bit depth BitDepthC for the chroma sample arrays, if any, of the input pictures.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

gfv_id contains an identifying number that may be used to identify face feature information and specify a neural network that may be used as Generator( ). The value of gfv_id shall be in the range of 0 to 232–− 2, inclusive. Values of gfv_id from 256 to 511, inclusive, and from 231 to 232–− 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering a GFV SEI message with gfv_id in the range of 256 to 511, inclusive, or in the range of 231 to 232–− 2, inclusive, shall ignore the SEI message.

NOTE – Different values of gfv_id in different GFV SEI messages could be used to identify different faces when more than one face is present in an output picture, for example.

gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_reserved_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_payload_byte[ i ] specify a neural network that may be used as a Generator( ). gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_reserved_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_payload_byte[ i ] have the same syntax and semantics nnpfc_base_flag, nnpfc_mode_idc, nnpfc_reserved_zero_bit_a, nnpfc_tag_uri, nnpfc_uri, nnpfc_payload_byte[ i ], respectively.

gfv_key_pic_flag equal to 1 indicates the current decoded output picture corresponds to a key picture. gfv_key_pic_flag equal to 0 indicates the current decoded output picture does not correspond to a key picture.

gfv_drive_pic_flag, when present, equal to 1 indicates the current decoded output picture, which corresponds to a driving picture, may be input to Generator( ). gfv_drive_pic_flag equal to 0 indicates the current decoded output picture should not be input to Generator( ).

NOTE – A gfv_drive_pic_flag value of 1 could be used to indicate the current decoded output picture could be used to improve face details or handle background changes, as examples.

Face feature information is coded as 2D keypoints specified using ( x , y ) coordinate pairs in the grid [ − 1, + 1] x [ − 1, + 1 ].

gfv_log2_kp_quant_factor specifies a quantization factor qKp determined as follows: qKp = 2( gfv_log2_kp_quant_factor )

The dequantization function DeqKp( ) is specified as follows: DeqKp( x ) = Clip3( − 1, 1, ( x ÷ qKp ) – 1)

gfv_num_kp_minus1 plus 1 specifies the number of 2D keypoints specified in the GFV SEI message.

gfv_kp_pred_flag equal to 1 indicates syntax elements gfv_kp_grid_pos_dx[ i ] and gfv_kp_grid_pos_dy[ i ] are present in the GFV SEI message and the values of syntax elements gfv_kp_grid_pos_x[ i ] and gfv_kp_grid_pos_y[ i ] are derived. gfv_kp_pred_flag equal to 0 indicates syntax elements gfv_kp_grid_pos_dx[ i ] and gfv_kp_grid_pos_dy[ i ] are not present and syntax elements gfv_kp_grid_pos_x[ i ] and gfv_kp_grid_pos_y[ i ] are present in the GFV SEI message.

gfv_kp_grid_pos_x[ i ] specifies the x component of the ( x , y ) coordinate pair for the i-th 2D keypoint.

gfv_kp_grid_pos_y[ i ] specifies the y component of the ( x , y ) coordinate pair for the i-th 2D keypoint.

gfv_kp_grid_pos_dx[ i ], when present, specifies the delta-x component of the ( x, y ) coordinate pair for the i-th 2D keypoint.

gfv_kp_grid_pos_dy[ i ], when present, specifies the delta-y component of the ( x, y ) coordinate pair for the i-th 2D keypoint.

When gfv_kp_pred_flag is equal to 1, gfv_kp_grid_pos_x[ i ] and gfv_kp_grid_pos_y[ i ] are derived as follows:

if( gfv_key_pic_flag ) {
	predX = ( i > 0 ) ? gfv_kp_grid_pos_x[ i – 1 ] : 0
	predY = ( i > 0 ) ? gfv_kp_grid_pos_y[ i – 1 ] : 0
	gfv_kp_grid_pos_x[ i ] = gfv_kp_grid_pos_dx[ i ] + predX
	gfv_kp_grid_pos_y[ i ] = gfv_kp_grid_pos_dy[ i ] + predY
}
else{
	predX = KeyKpGridPosX[ i ]
	predY = KeyKpGridPosY[ i ]
	gfv_kp_grid_pos_x[ i ] = gfv_kp_grid_pos_dx[ i ] + predX
	gfv_kp_grid_pos_y[ i ] = gfv_kp_grid_pos_dy[ i ] + predY
}

The x and y components of the ( x , y ) coordinate pair for the i-th 2D keypoint for the key picture, denoted as KeyKpGridPosX[ i ] and KeyKpGridPosY[ i ], respectively, where i = 0..gfv_num_kp_minus1, are specified as follows:

if( gfv_key_pic_flag ) {
	KeyKpGridPosX[ i ] = gfv_kp_grid_pos_x[ i ]
	KeyKpGridPosY[ i ] = gfv_kp_grid_pos_y[ i ]
}

The dequantized x component and dequantized y component of the ( x , y ) coordinate pair for the i-th 2D keypoint for the current picture, denoted as deqKpGridPosX[ i ] and deqKpGridPosY[ i ], respectively, where i = 0..gfv_num_kp_minus1, are specified as follows:

deqKpGridPosX[ i ] = DeqKp( gfv_kp_grid_pos_x[ i ] )
deqKpGridPosY[ i ] = DeqKp( gfv_kp_grid_pos_y[ i ] )

gfv_kp_local_matrix_flag equal to 1 indicates that syntax element gfv_kp_local_matrix[ i ][ j ] is present in the GFV SEI message. gfv_kp_local_matrix_flag equal to 0 indicates syntax element gfv_kp_local_matrix[ i ][ j ] is not present in the GFV SEI message.

gfv_kp_local_matrix[ i ][ j ] specifies the value of j-th local matrix coefficient for the i-th 2D keypoint used to derive a feature array.

The dequantized j-th local matrix coefficient for the i-th 2D keypoint, denoted as deqLocalMat[ i ][ j ], is derived as follows:

deqLocalMat[ i ][ j ] = DeqKp( gfv_kp_local_matrix[ i ][ j ])
where i = 0..gfv_num_kp_minus1, j = 0..3

The feature array for the current picture, denoted as deqFeature[ i ][ j ], is derived as follows:

deqFeature[ i ][ 0 ] = deqKpGridPosX[ i ]
deqFeature[ i ][ 1 ] = deqKpGridPosY[ i ]
if( gfv_kp_local_matrix_flag ) {
	deqFeature[ i ][ 2 ] = deqLocalMat[ i ][ 0 ]
	deqFeature[ i ][ 3 ] = deqLocalMat[ i ][ 1 ]
	deqFeature[ i ][ 4 ] = deqLocalMat[ i ][ 2 ]
	deqFeature[ i ][ 5 ] = deqLocalMat[ i ][ 3 ]
} else {
	deqFeature[ i ][ 2 ] = 0
	deqFeature[ i ][ 3 ] = 0
	deqFeature[ i ][ 4 ] = 0
	deqFeature[ i ][ 5 ] = 0
}
where i = 0..gfv_num_kp_minus1.

The feature array for the key picture, denoted as KeyDeqFeature[ i ][ j ], is derived as follows:

if( gfv_key_picture_flag ) {
	KeyDeqFeature[ i ][ 0 ] = deqFeature[ i ][ 0 ]
	KeyDeqFeature[ i ][ 1 ] = deqFeature[ i ][ 1 ]
	KeyDeqFeature[ i ][ 2 ] = deqFeature[ i ][ 2 ]
	KeyDeqFeature[ i ][ 3 ] = deqFeature[ i ][ 3 ]
	KeyDeqFeature[ i ][ 4 ] = deqFeature[ i ][ 4 ]
	KeyDeqFeature[ i ][ 5 ] = deqFeature[ i ][ 5 ]
}
where i = 0..gfv_num_kp_minus1.

8.xx.3	Description of inputs and outputs of Generator( )

Input values to Generator( ) are real numbers and the functions InpY( ) and InpC( ) are specified as follows:

InpY( x ) = x ÷ ( ( 1  <<  BitDepthY ) – 1 )

InpC( x ) = x ÷ ( ( 1  <<  BitDepthC ) – 1 )

Output values from Generator( ) are real numbers and the functions OutY( ) and OutC( ) are specified as follows:

OutY( x ) = Clip3( 0, ( 1  <<  BitDepthY ) – 1 , x * ( ( 1  <<  BitDepthY ) – 1 )

OutC( x ) = Clip3( 0, ( 1  <<  BitDepthC ) – 1 , x * ( ( 1  <<  BitDepthC ) – 1 )

When gfv_key_pic_flag is equal to 1, the KeyPicture luma sample array KeyY is derived as follows:

KeyY[ x ][ y ] = InpY( keyCroppedYPic[ x ][ y ] ). where x = 0..CroppedWidth − 1, y = 0..CroppedHeight – 1

When gfv_key_pic_flag is equal to 1 and chromaIDC is not equal to 0, the KeyPicture chroma sample arrays KeyCb and KeyCr are derived as follows:

KeyCb[ x ][ y ] = InpC( keyCroppedCbPic[ x ][ y ] ). where x = 0..CroppedWidth / SubWidthC − 1, y = 0..CroppedHeight /SubHeightC − 1

KeyCr[ x ][ y ] = InpC( keyCroppedCrPic[ x ][ y ] ). where x = 0..CroppedWidth / SubWidthC − 1, y = 0..CroppedHeight /SubHeightC − 1

When gfv_drive_pic_flag is equal to 1, the DrivePicture luma sample array driveY is derived as follows:

driveY[ x ][ y ] = InpY( driveCroppedYPic[ x ][ y ] ). where x = 0..CroppedWidth − 1, y = 0..CroppedHeight – 1

When gfv_drive_pic_flag is equal to 1 and chromaIDC is not equal to 0, the DrivePicture chroma sample arrays driveCb and driveCr are derived as follows:

driveCb[ x ][ y ] = InpC( driveCroppedCbPic[ x ][ y ] ). where x = 0..CroppedWidth / SubWidthC − 1, y = 0..CroppedHeight /SubHeightC − 1

driveCr[ x ][ y ] = InpC( driveCroppedCrPic[ x ][ y ] ). where x = 0..CroppedWidth / SubWidthC − 1, y = 0..CroppedHeight /SubHeightC − 1

Input to Generator() are:

- When gfv_key_pic_flag is equal to 0 and gfv_drive_pic_flag is equal to 0, deqFeature, KeyDeqFeature, KeyY, KeyCb, KeyCr

When gfv_key_pic_flag is equal to 0 and gfv_drive_pic_flag is equal to 1, deqFeature, KeyDeqFeature, KeyY, KeyCb, KeyCr, driveY, driveCb, and driveCr

Output of Generator( ) are:

- When chromaIDC is equal to 0, a luma sample array genY

- When chromaIDC is not equal to 0, a luma sample array genY and chroma sample arrays genCb and genCr.

Arrays outYPic[ x ][ y ], outYPic[ x ][ y ], and outYPic[ x ][ y ] are derived as follows:

- If gfv_key_pic_flag is equal to 0, the following applies:

- outYPic[ x ][ y ] = OutY( genY[ x ][ y ] ). where x = 0..CroppedWidth − 1, y = 0..CroppedHeight – 1

- if chromaIDC is not equal to 0, the following applies:

- outCbPic[ x ][ y ] = OutC( genCb[ x ][ y ] ). where x = 0..CroppedWidth / SubWidthC − 1, y = 0..CroppedHeight /SubHeightC − 1

- outCrPic[ x ][ y ] = OutC( genCr[ x ][ y ] ). where x = 0..CroppedWidth / SubWidthC − 1, y = 0..CroppedHeight /SubHeightC − 1

- Otherwise (gfv_key_pic_flag is equal to 1), the following applies:

- outYPic[ x ][ y ] = keyCroppedYPic[ x ][ y ]

- if chromaIDC is not equal to 0, the following applies:

- outCbPic[ x ][ y ] = keyCroppedCbPic[ x ][ y ]

- outCrPic[ x ][ y ] = keyCroppedCrPic[ x ][ y ]

# Conclusion

This contribution proposes a generative face video (GFV) SEI message for inclusion in the next version of VSEI for use with VVC. The proposed GFV SEI message conveys information for facial features extracted from source video prior to encoding that can be applied to a previously decoded reference picture to reconstruct a face picture at the current time. The objective of the proposed GFV SEI message is to enable techniques of generative face video production to be combined with conventional video coding to enable very low bitrate visual communications.

# References

- B. Chen, J. Chen, et al. “AHG9: Generative Face Video SEI Message,” doc. no. JVET-AC0088, January 2023.

- B. Chen, J. Chen, et al. “AHG9: Common SEI Message of Generative Face Video,”, doc. no. JVET-AD0051, April 2023.

- Goodfellow I, Pouget-Abadie J, Mirza M, et al. “Generative adversarial nets,” Advances in neural information processing systems, vol. 27, 2014.

- Oquab M, Stock P, Haziza D, et al. Low bandwidth video-chat compression using deep generative models[C]//Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition. 2021: 2388-2397.

- Konuko G, Valenzise G, Lathuilière S. Ultra-low bitrate video conferencing using deep image animation[C]//ICASSP 2021-2021 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP). IEEE, 2021: 4210-4214.

- Feng D, Huang Y, Zhang Y, et al. A generative compression framework for low bandwidth video conference[C]//2021 IEEE International Conference on Multimedia & Expo Workshops (ICMEW). IEEE, 2021: 1-6.

- Siarohin A, Lathuilière S, Tulyakov S, et al. First order motion model for image animation[J]. Advances in Neural Information Processing Systems, 2019, 32.

- Siarohin A, Woodford O J, Ren J, et al. Motion representations for articulated animation[C]//Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition. 2021: 13653-13662.

- Wang T C, Mallya A, Liu M Y. One-shot free-view neural talking-head synthesis for video conferencing[C]//Proceedings of the IEEE/CVF conference on computer vision and pattern recognition. 2021: 10039-10049.

- Chen B, Wang Z, Li B, et al. Beyond keypoint coding: Temporal evolution inference with compact feature representation for talking face video compression[C]//2022 Data Compression Conference (DCC). IEEE, 2022: 13-22.

- Chen B, Wang Z, Li B, et al. Interactive Face Video Coding: A Generative Compression Framework[J]. arXiv preprint arXiv:2302.09919, 2023.

- Pearson D E, Robinson J A. Visual communication at very low data rates[J]. Proceedings of the IEEE, 1985, 73(4): 795-812.

- Musmann H G, Hötter M, Ostermann J. Object-oriented analysis-synthesis coding of moving images[J]. Signal processing: Image communication, 1989, 1(2): 117-138.

- Nakaya Y, Chuah Y C, Harashima H. Model-based/waveform hybrid coding for videotelephone images[C]//Acoustics, Speech, and Signal Processing, IEEE International Conference on. IEEE Computer Society, 1991: 2741, 2742, 2743, 2744-2741, 2742, 2743, 2744.

- Chung J S, Nagrani A, Zisserman A. , “Voxceleb2: Deep speaker recognition,” in Interspeech, 2018

- S. McCarthy et al, “Additional SEI messages for VSEI (Draft 4)”, doc. no. JVET-AD2006, April 2023.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).