| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0081-v5 |
| --- | --- |

| Title: | [AHG8] De-noising filter as pre-processing for machine task |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Dong-Gyu Gwak Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | chulkeun.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports the machine analysis performance of applying a de-noising filter as a pre-processing. The Gaussian blur function consisting of 3x3 and 5x5 kernel is exploited to reduce the level of noise. Experimental results show that de-noising filtering on input video can give following changes.

- SFU-HW: -14.07%, -17.39%, -18.63% for RA, LD and AI respectively (Pareto-mAP)

- TVD: -18.75%, -22.30% for RA and AI respectively (Pareto-mAP)

Introduction

At the 30th JVET meeting, the second draft for a technical report [1] on optimization of encoders and receiving systems for machine analysis of coded video content was produced. It includes pre-processing as a technology for improvement on coding efficiency for machine tasks. The pre-processing technologies described in the draft include region of interest-based methods, pre-processing methods of foreground and background, temporal subsampling, and spatial subsampling. In this contribution, de-noising filter is proposed as a pre-processing technology. There is no change on the encoder and decoder side.

Motivation

Noise which may be captured along with video can reduce encoding efficiency. If a de-noising filter is applied before encoding on the video transmission side, not only the machine analysis performance can be improved by reducing unnecessary noise, but also the burden of video transmission can be reduced by enhancing the encoding efficiency.

Furthermore, video capturing conditions (e.g. environment such as brightness, used camera, purpose, etc) can be utilized to determine the appropriate level of de-noising by applying the de-noising filter as a pre-processing rather than post-processing, which can be positive for machine analysis.

Method and results

Proposed method and results

In this contribution, a Gaussian blur filter is used as the de-noising filter. The GaussianBlur function provided by OpenCV [2] is applied to the test datasets defined in CTC [3]. Figure 1 shows the kernels used in the experiment. Experiments are conducted with the evaluation method defined by CTC [3] using the generated dataset.

Figure 1. Gaussian blur 3 x 3, 5 x 5 kernel used in the experimental

Table 1 represents the performance of the proposed method on SFU-HW dataset for object detection task. A 5 x 5 kernel is applied to Class A, B, and C, and a 3 x 3 kernel is applied to Class D. Pareto-mAP results of -14.07% for random access, -17.39% for low delay, and -18.63% for all intra are observed.

Table 2 represents performance of the proposed method on TVD video dataset for object tracking task. A 3 x 3 kernel is applied to TVD video dataset. Pareto-MOTA result of -18.75% for random access and -22.3% for all intra are observed.

Table 1. Performance of the proposed method on SFU-HW dataset for object detection task. (The ‘/’ means non-monotonic performance.)

Table 2. Performance of the proposed method on TVD video dataset for object tracking task. (The ‘/’ means non-monotonic performance.)

Figure 2 visually represents the results of encoding the original pictures and pre-processed pictures of “BQTerrace” with QP40. The bitrate of the original pictures is 12597.27 kbps and mAP is 41.79. The bitrate of pre-processed pictures with the de-noising filter is 8768.737 kbps and mAP is 43.04. Figures (b) represents the original images of regions A, while figures (c) represent the pre-processed result of regions A. Noise is observed in the original image and the noise is reduced in the pre-processed image with blurring the boundary of the object. Figures (d) visually represents the result of encoding the original pictures with QP40, while Figures (e) represents the result of encoding the pre-processed pictures with QP40. It is observed that the encoding of the preprocessed picture retains the shape of the object compared to the encoding of the original picture.

- Original input

- Original picture of region A                 (c) pre-processed picture of region A

- encoding result of the original picture      (e) encoding result of the pre-processed picture

Figure 2. Result of encoding the original pictures and pre-processed pictures (5 x 5 Gaussian blur) of “BQTerrace” with QP40

Experimental results combined with JVET-AD0122

Table 3 and 4 represent the performance of the proposed pre-processing method combined with the modified configuration for low delay and random access of JVET-AD0122[4].  Pareto-mAP results of -19.25%/-22.48% (SFU-HW/TVD for random access and -25.84% (SFU-HW) for low delay are observed. Additional performance improvements (5.18%/3.73% (SFU-HW/TVD) for random access, 8.45% (SFU-HW) for low delay) are observed through combining. (JVET-AD0122 standalone: RA -7.34%/6.96% (SFU-HW/TVD), LD -9.57%(SFU-HW))

Table 3. Performance of combined with the proposed pre-processing method and the modified configuration for low delay and random access of JVET-AD0122 (SFU-HW, object detection task)

Table 4. Performance of combined with the proposed pre-processing method and the modified configuration for low delay and random access of JVET-AD0122 (TVD video dataset, object tracking task)

Conclusion

In this contribution, de-noising filter is proposed as a pre-processing technology for optimizations for encoders and receiving systems for machine analysis of coded video content. A Gaussian blur filter is used in the experiment. For SFU-HW dataset, the Pareto-mAP results are -14.07% for random access, -17.39% for low latency and -18.63% for all intra. For the TVD dataset, the Pareto-MOTA results are -18.75% for random access and -22.30% for all intra.

Section 6 of the draft technical report [1] describes the pre-processing technologies. It is proposed to add de-noising filter as a pre-processing technology as detailed in the attached version of the technical report.

References

- J. Chen, C. Hollmann and S. Liu, " Optimization of encoders and receiving systems for machine analysis of coded video content (draft 2)," JVET-AD2030, Antalya, TR, April 2023.

- https://docs.opencv.org/4.x/d4/d86/group__imgproc__filter.html#gaabe8c836e97159a9193fb0b11ac52cf1

- S. Liu and C. Hollmann, "Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content," JVET-AD2031, Antalya, TR, April 2023.

- C. Hollmann, R. Sjöberg, J. Ström, " [AHG8] Configuration changes for random access and low delay," JVET-AD0122, Antalya, TR, April 2023.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).