| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0083-v1 |
| --- | --- |

| Title: | AHG9: Common SEI Message of Generative Face Video |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bolin Chen, Jie Chen, Yan Ye, Shiqi Wang | Email: | chenbolin.chenboli@alibaba-inc.com jiechen.cj@alibaba-inc.com yan.ye@alibaba-inc.com shiqwang@cityu.edu.hk |
| Source: | Alibaba and City University of Hong Kong |  |  |

_____________________________

- Abstract

At the 29th JVET meeting, JVET-AC0088 proposed a generative face video SEI message to allow VVC-coded pictures to be used as base pictures to drive a deep generative network to generate additional images, therefore enabling ultra low rate face video compression. At the 30th JVET meeting, JVET-AD0051 generalized the generative face video SEI message to include syntax elements that correspond to a variety of facial representations, such as 2D keypoints, 2D landmarks, 3D keypoints, facial semantics and other formats.

Compared with previous contributions, in this contribution we

- refine the syntax and semantics of the proposed SEI message

- define the interface between the decoder (which decodes the base pictures and parses the SEI message) and the generative neural network

- provide further information on available software implementations.

Introduction

Recently, deep generative models, especially generative adversarial network (GAN) [1], have shown great potentials in face video compression [2]-[9]. Following the philosophy of early model-based coding [9]-[12], the generative model based encoder can use the analysis model to economically model the input face video into a small number of transmitted symbols (e.g., semantic parameters, facial landmarks, motion features, etc.). At the decoder, a deep generative model can use the decoded symbols and a base picture to reconstruct high quality face images. Compared to the early work on model-based video coding, which had to rely on much simpler synthesis models, nowadays deep generative networks provide signifantly higher capability to infer and synthesize the reconstructed video, resulting in much better reconstruction quality. It should be mentioned that most of the work in this area use a traditional video codec such as the VVC or HEVC to compress the first picture (or a set of key pictures) of a talking face video sequence as the base picture to provide essential texture reference. Then, the subsequent face pictures can be effectively reconstructed based on such base pictures and motion parameters from the GAN models. Figure 1 illustrates the general block diagram of generative compression codec.

Figure 1 - General block diagram of generative compression codec.

In Table 1, we provide a summary of a few typical facial representations for generative face video compression algorithms. Because the face images exhibit strong statistical regularities, they can be economically characterized with 2D landmarks, 2D keypoints, region matrix, 3D keypoints, compact feature matrix or facial semantics. Such facial description strategies can lead to reduced coding bit-rate and improved coding efficiency.

Table 1 - Summary of facial representations for generative face video compression algorithms

| Facial Representation | Interpretation |
| --- | --- |
| 2D landamrks | VSBNet [4] is the representative model which can ulitize 98 groups of 2D facial landmarks R2×98 to depict the key structure information of human face, where the total number of encoding parameters for each inter frame is 196. |
| 2D keypoints+affine transformation matrix | FOMM [5] is the representative model which adopts 10 groups of learned 2D keypoints R2×10 along with their local affine transformations R2×2×10 to characterize complex motions. The total number of encoding parameters for each inter frame is 60. |
| region matrix | MRAA [6] is the representative model which extracts consistent regions of talking face to describe locations, shape, and pose, mainly represented with shift matrix R2×10, covariance matrix R2×2×10 and affine matrix R2×2×10. As such, the total number of encoding parameters for each inter frame is 100. |
| 3D keypoints | Face_vid2vid [7] is the representative model which can estimate 12-dimension head parameters (i.e., rotation matrix R3×3 and translation parameters R3×1 ) and 15 groups of learned 3D keypoint perturbations R3×15 due to facial expressions, where the total number of encoding parameters for each inter frame is 57. |
| compact feature martix | CFTE [8] is the representative model which can model the temporal evolution of faces into learned compact feature representation with the matrix R4×4, where the total number of encoding parameters for each inter frame is 16. |
| facial semantics | IFVC [9] is the representative model which adopts a collection of transmitted facial semantics to represent the face frame, including mouth parameters R6, eye parameter R1, rotation parameters R3, translation parameters R3 and location parameter R1. Totally, the number of encoding parameters for each inter frame is 14. |

It is asserted that the capability of a powerful deep generative network can enable the following use cases currently unavailable in the VVC standard:

- Ultra low rate communication: the base pictures can be sent with large time gap in between, and in between two consecutive base pictures, only a few facial parameters per time instance need to be sent, significantly reducing the coded bit rate compared to coding all pictures using VVC;

- User-specified animation/filtering: face animation and facial filters have become very popular in many mobile apps. As the encoder has the uncoded face images, it can extract more precise facial landmarks and/or keypoints, resulting in higher quality facial animation at the decoder side. This could allow the sender to have better control over the quality of animated or filtered facial pictures.

In this contribution, we propose to send the facial representation in a generative face video SEI message. This proposed SEI message was based on the proposal in JVET-AD0051, with syntax and semantics further refined. Additionally, we define the interfaces between the decoder (which produces base pictures and facial representation parameters parsed from the SEI message) and the generative netwrok. We also provide information on available open-source generative models as well as an example software implementation that links the VTM software with a generative network.

Proposal

In this section, we describe the proposed SEI message which is designed to carry facial parameters for different feature representation algorithms listed in Table 1, such as 2D keypoints, 2D landmarks, 3D keypoints, facial semantics and other fomarts.

Each picture unit (PU) may contain a proposed SEI message which carries facical parameters that can be used to generate a face picture. The decoded picture of a PU may be a base picture or a dummy picture. If the current picture is a dummy picture, the SEI message can be used to generate a face picture based on the base picture to replace the dummy picture.

Proposed SEI message scope

Table 2 - Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| … | … |
| Generative face video | Specified by the syntax of the SEI message |

Syntax of generative face video SEI message

Table 3 - generative face video SEI message syntax

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_key | u(32) |
| gfv_pic_order_cnt | u(32) |
| coordinate_present_flag | u(1) |
| if( coordinate_present_flag ) { |  |
| coordinate_precision_factor_minus1 | ue(v) |
| num_coordinates_minus1 | ue(v) |
| coordinate_z_present_flag | u(1) |
| if( coordinate_z_present_flag ) |  |
| coordinate_z_max_value_minus1 | ue(v) |
| for( i=0; i< num_coordinates_minus1; i++ ) { |  |
| coordinate_x_abs[ i ] | u(v) |
| if( coordinate_x_abs[ i ] ) |  |
| coordinate_x_sign_flag[ i ] | u(1) |
| coordinate_y_abs[ i ] | u(v) |
| if( coordinate_y_abs[ i ] ) |  |
| coordinate_y_sign_flag[ i ] | u(1) |
| if( coordinate_z_present_flag ) { |  |
| coordinate_z_abs[ i ] | u(v) |
| if( coordinate_z_abs[ i ] ) |  |
| coordinate_z_sign_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| matrix_present_flag | u(1) |
| if( matrix_present_flag ) { |  |
| matrix_element_precision_factor_minus1 | ue(v) |
| num_matrix_types_minus1 | ue(v) |
| for( i=0; i <= num_matrix_types_minus1; i++ ) { |  |
| matrix_type_idx[ i ] | u(6) |
| if( matrix_type_idx[ i ] == 0 ¦¦ matrix_type_idx[ i ] == 1) { |  |
| if( coordinate_present_flag ) |  |
| num_matrices_equal_to_num_coordinates_flag[ i ] | u(1) |
| if(!coordinate_present_flag ¦¦ !num_matrix_equal_to_num_coordinates_flag[ i ] ) |  |
| num_matrices_info[ i ] | ue(v) |
| } |  |
| else if( matrix_type_idx[ i ] == 2 ¦¦ matrix_type_idx[ i ] == 3 ¦¦ matrix_type_idx[ i ] >= 7 ) { |  |
| if( matrix_type_idx[ i ] >= 7 ) |  |
| num_matrices_minus1[ i ] | ue(v) |
| matrix_width_minus1[ i ] | ue(v) |
| matrix_height_minus1[ i ] | ue(v) |
| } |  |
| else if( matrix_type_idx[ i ] >= 4 && matrix_type_idx[ i ] <= 6 && !coordinate_present_flag ) |  |
| matrix_for_3D_space_flag[ i ] | u(1) |
| for( j=0; j< numMatrices[ i ]; j++ ) { |  |
| for( k=0; k< matrixHeight[ i ]; k++ ) { |  |
| for( l=0;l=matrixWidth[ i ]; l++ ) { |  |
| matrix_element_int[ i ][ j ][ k ][ l ] | ue(v) |
| matrix_element_dec[ i ][ j ][ k ][ l ] | u (v) |
| if( matrix_element_int[ i][ j ][ k ][ l ] ¦¦ matrix_element_dec[ i ][ j ][ k ][ l ] ) |  |
| matrix_element_sign_flag[ i ][ j ][ k ][ l ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

Semantics of generative face video SEI message

This SEI message specifies the syntax and semantics of a variety of facial representations that may be used by a deep generative network to generate pictures based on a base picture. The base picture is a picture that provides the texture information needed by the deep generative network. The base picture may be coded as a bitstream conforming to Rec. ITU-T H.264, Rec. ITU-T H.265, Rec. ITU-T H.266, etc.. Decoder may decode the base picture from the conforming bistream periodically . Based on the base picture, a number of subsequent pictures may be generated with a deep generative network using syntax elements signaled in the generative face video SEI message. The texture of the base picture and the additionally generated pictures are expected to mainly contain talking face.

Use of this SEI message requires the definition of the following variables:

- Cropped decoded output base picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

- Luma sample array CroppedYPic and chroma sample arrays CroppedCbPic and CroppedCrPic of the cropped decoded output base picture.

- A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

- The variables SubWidthC and SubHeightC derived from ChromaFormatIdc.

gfv_key indicates a key that may be used to identify the analysis network used to generate the syntax elements of the current generative face video SEI message. The value of gfv_key may be used to determine whether the analysis network at the encoder matches with the generative network at the decoder.

NOTE 1 – The generative network at the decoder may use the SEI message to generate the video pictures only if it recognizes the value of gfv_key. The value of gfv_key may be specified by the applications that rely on matching networks at the encoder and the decoder. For such applications, if the parameters in the received SEI message are extarcted with an analysis network that is not matched with the generative network at the decoder, the generative network should ignore the received SEI message.

gfv_pic_order_cnt specifies the display order count modulo 1<<31 of the picture generated with the current SEI message.

coordinate_present_flag equal to 1 indicates that the coordinate information of keypoints is present. coordinate_present_flag equal to 0 indicates coordinate information of keypoints is not present.

It is a requirement of bitstream conformance that when matrix_type_idx[ i ] for any i from 0 to num_matrix_types_minus1 is equal to 0 or 1, the value of coordinate_present_flag shall be equal to 1

coordinate_precision_factor_minus1 plus 1 indicates the length, in bits, of coordinate_x_abs[i], coordinate_y_abs[i] and coordinate_z_abs[i].

num_coordinates_minus1 plus 1 indicates the number of keypoint coordinates. The value of num_coordinates_minus1 shall be in the range of 0 to 210 -1, inclusive.

coordinate_z_present_flag equal to 1 indicates z-axis coordinate information of the keypoints is present. coordinate_z_present_flag equal to 0 indicates that the z-axis coordinate information of the keypoints is not present.

coordinate_z_max_value_minus1 plus 1 indicates the maximum absolute value of z-axis coordinates of keypoints.

coordinate_x_abs[ i ] indicates the normalized absolute value of x-axis coordinate of the i-th keypoint.

coordinate_x_sign_flag[ i ] specifies the sign of of x-axis coordinate of the i-th keypoint. When coordinate_x_sign_flag[ i ] is not present, it is inferred to be euqul to 0.

coordinate_y_abs[ i ] specifies the normalized absolute value of y-axis coordinate of i-th keypoint.

coordinate_y_sign_flag[ i ] specifies the sign of of y-axis coordinate of i-th keypoint. When coordinate_y_sign_flag[i] is not present, it is inferred to be euqul to 0.

coordinate_z_abs[ i ] specifies the normalized absolute value of z-axis coordinate of i-th keypoint.

coordinate_z_sign_flag[ i ] specifies the sign of z-axis coordinate of i-th key point. When coordinate_z_sign_flag[ i ] is not present, it is inferred to be euqul to 0.

The variable coordiateX[ i ], coordiateY[ i ] and coordiateZ[ i ] indicating the x-axis coordinate, y-axis coordinate and z-axis coordiate of the i-th keypoint respectively are derived as follows

coordiateX[ i ] = (1-2 * coordinate_x_sign_flag[ i ]) * coordinate_x_abs[ i ] * CroppedWidth1<< (coordinate_precision_factor_minus1 +1)

coordiateY[i] = (1-2 * coordinate_y_sign_flag[ i ]) * coordinate_y_abs[ i ] * CroppedHeight1<< (coordinate_precision_factor_minus1 +1)

coordiateZ[i] = (1-2 * coordinate_z_sign_flag[ i ]) * coordinate_z_abs[ i ] *(coordinate_z_max_value_minus1+1)1<< (coordinate_precision_factor_minus1 +1)

matrix_present_flag equal to 1 indicates that matrix parameters are present. matrix_present_flag equal to 0 indicates that matrix parameters are not present.

matrix_element_precision_factor_minus1 plus 1 indicates the length, in bits, of matrix_element_dec[ i ][ j ][ k ][ l ].

num_matrix_types_minus1 plus 1 indicates the number of matrix types signaled in the SEI message. The value of matrix_type_num_minus1 shall be in the range of 0 to 26 -1, inclusive.

matrix_type_idx[ i ] indicates the index of the i-th matrix type as specified in Table 4.

Table 4 - Specification of matrix_type_idx

| Value | Specification |
| --- | --- |
| 0 | Affine translation matrix with the size of 2*2 or 3*3. |
| 1 | Covariance matrix with size of 2*2 or 3*3. |
| 2 | Mouth matrix representing mouth motion. |
| 3 | Eye matrix representing the open-close status and level of eyes. |
| 4 | Head rotation paramters with the size of 2*2 or 3*3 representing the head rotation in 2D space or 3D space. |
| 5 | Head translation matrix with the size of 1*2 or 1*3 representing head translationin 2D space or 3D space. |
| 6 | Head location matrix with size of 1*2 or 1*3 representing the head location in 2D space or 3D space. |
| 7 | Compact feature matrix with the size being specified by matrix_width_minus1[i] and matrix_height_minus1[i]. |
| 8…31 | Other undefined matrix with the size being specified by matrix_width_minus1[i] and matrix_height_minus1[i]. |
| 32…63 | Reserverd |

NOTE 2 . The undefined matrxi type is used to respresnt the matrxi type rather than affine translation matrix, covariance matrix, rotation matrix, translation matrix and compact feature matrix. It is may be used by the user to extend the matrix type.

num_matrices_equal_to_num_coordinates_flag[ i ] equal to 1 indicates the number of matrices of the i-th matrix type is equal to num_coordinates_minus1 + 1. num_matrices_equal_to_num_coordinates_flag[ i ] equal to 0 indicates the number of matrices of the i-th matrix type is not equal to num_coordinates_minus1 + 1.

num_matrices_info[ i ] provides information to derive the number of the matrices of the i-th matrix type.

matrix_width_minus1[ i ] plus 1 indicates the width of the matrix of the i-th matrix type.

When matrix_width_minus1[ i ] is not present, it is inferred as follows:

––	If matrix_type_idx[ i ] is equal to 0, 1 or 4, and one of coordinate_z_present_flag and matrix_for_3D_space_flag [ i ] is present and equal to 1, matrix_width_minus1[i] is inferred to be equal to 2

––	otherwise, if matrix_type_idx[ i ] is equal to 0, 1 or 4, and one of coordinate_z_present_flag and matrix_for_3D_space_flag [ i ] is present and equal to 0, matrix_width_minus1[ i ] is inferred to be equal to 1

––	otherwise (matrix_type_idx[ i ] is equal to 5 or 6), matrix_width_minus1[ i ] is inferred to be equal to 0

matrix_height_minus1[ i ] plus 1 indicates the height of the matrix of the i-th matrix type.

When matrix_height_minus1[i] is not present, it is inferred as follows:

––	If matrix_type_idx is equal to 0, 1, 4, 5 or 6, and one of coordinate_z_present_flag and matrix_for_3D_space_flag [ i ] is present and equal to 1, matrix_height_minus1[ i ] is inferred to be equal to 2.

––	otherwise (matrix_type_idx is equal to 0, 1, 4, 5 or 6, and one of coordinate_z_present_flag and matrix_for_3D_space_flag [ i ] is 0), matrix_height _minus1[ i ] is inferred to be equal to 1.

matrix_for_3D_space_flag [ i ] equal to 1 indicates the matrix of the i-th matrix type is a maxtrix defined in three-dimension space. matrix_for_3D_space_flag [ i ] equal to 0 indicates the matrix of the i-th matrix type is a matrix defined in two-dimension space.

The variables matrixWidth[ i ] and matrixHeight[ i ] indicating the width and height of the matrix of the i-th matrix type are derived as follows

matrixWidth[ i ] = matrix_width_minus1[ i ] + 1

matrixHeight[ i ] = matrix_height_minus1[ i ] + 1

num_matrices_minus1[ i ] plus 1 indicates the number of matrices of the i-th matrix type.

The variable numMatrices[ i ] indicating the number of the matrices of the i-th matrix type is derived as follow.

if( matrix_type_idx[ i ] == 0 || matrix_type_idx[ i ] == 1 ) {

if( coordinate_present_flag )

numMatrices[ i ] = num_matrices_equal_to_num_coordinates_flag[ i ] ? num_coordinates_minus1 + 1 : ( num_matrices_idc[ i ] < num_coordinates_minus1 ? num_matrices_idc[ i ] + 1 : num_matrices_idc[ i ] + 2 )

else

numMatrices[ i ] = num_matrices_idc[ i ] + 1

}

else if( matrix_type_idx[ i ] >= 2 &&  matrix_type_idx[ i ] < 7)

numMatrices[ i ] = 1

else

numMatrices[ i ] = num_matrices_minus1[i] + 1

matrix_element_int[ i ][ j ][ k ][ l ] indicates the integer part of the value of the matrix element at position (k, l) of the j-th matrix of the i-th matrix type.

matrix_element_dec[ i ][ j ][ k ][ l ] indicates the decimal part of the value of the matrix element at position (k, l) of the j-th matrix of the i-th matrix type.

matrix_element_sign_flag[ i ][ j ][ k ][ l ] indicates the sign of the matrix element at position (k, l) of the j-th matrix of the i-th matrix type. When matrix_element_sign_flag[ i ][ j ][ k ][ l ]is not present, it is inferred to be equal to 0.

The variable matrixElementVal[ i ][ j][ k ][ l ] representing the value of the matrix element at position (k, l) of the j-th matrix of the i-th matrix type is derived as follows.

matrixElementVal[ i][ j ][ k ][ l ] = 1-2*matrix_element_sign_flag[ i ][ j ][ k ][ l ]*(matrix_element_int i  j  k  l  +matrix_element_dec[ i ][ j ][ k ][ l ]1<< (matrix_element_precision_factor_minus1 +1))

The process DeriveInputTensors( ) for deriving the input tensor inputTensorImgY, inputTensorImgCb, inputTesnsorImgCr, inputTensorKeyPoint, inputTensorMatrix is specified as follows:

Initialize inputTensorImgY, inputTensorImgCb, inputTesnsorImgCr, inputTensorKeyPoint and inputTensorMatrix with 0.

for( x = 0; x< CroppedWidth; x++ ) {

for ( y = 0; y< CroppedHeight; y++ ) {

inputTensorImgY[ x ][ y ] = CroppedYPic[ x ][ y ]

}

}

for( x=0; x< CroppedWidth/ SubWidthC; x++ ) {

for ( y=0; y< CroppedHeight/ SubHeightC; y++ ) {

inputTensorImgCb[ x][ y ] = CroppedCbPic[ x][ y ]

inputTensorImgCr[ x][ y ] = CroppedCrPic[ x ][ y ]

}

}

if( coordinate_present_flag ) {

for ( i=0; i< =num_coordinates_minus1; i++ ) {

inputTensorKeyPoint[ i ][ 0 ] = coordiateX[ i ]

inputTensorKeyPoint[ i ][ 1 ] = coordiateY[ i ]

if ( coordinate_z_present_flag )

inputTensorKeyPoint[ i ][ 2 ] = coordiateZ[ i ]

}

}

if( matrix_present_flag ) {

for ( i=0; i<=num_matrix_types_minus1; i++ ) {

for ( j=0; j< numMatrices[ i ]; j++ ) {

for( k=0; k< matrixHeight [ i ]; k++ ) {

for ( l=0;l< matrixWidth [ i ]; l++) {

inputTensorMatrix[ i ][ j ][ k ][ l ] = matrixElementVal[ i ][ j][ k][ l ]

}

}

}

}

}

The process StoreOutputTensors( ), for deriving output sample arrays OutputYPic, OutputCbPic, and OutputCrPic from the output tensor outputTensorY, outputTensorCb and outputTensorCr is specified as follows:

for(x=0; x< CroppedWidth; x++){

for(y=0; y< CroppedHeight; y++){

OutputYPic[ x ][ y ] = outputTensorY[ x ][ y ]

}

}

for(x=0; x< CroppedWidth/ SubWidthC; x++){

for(y=0; y< CroppedHeight/ SubHeightC; y++){

OutputCbPic[ x ][ y ] = outputTensorCb[ x ][ y ]

OutputCdPic[ x][ y ] = outputTensorCr[ x ][ y ]

}

The following process is used to generate a video picture, where PictureGeneration( ) is a deep generative network based process to output the picture sample values with parameters signaled in the face generation SEI message and the decoded base picture as input .

DeriveInputTensors( )

outputTensor = PictureGeneration ( inputTensorImgY, inputTensorImgCb, inputTesnsorImgCr, inputTensorKeyPoint, inputTensorMatrix)

StoreOutputTensors( )

Use of generative face video SEI message (subclause D.9.x of VVC)

For purposes of interpretation of the generative face video SEI message, the following variables are specified:

–	The variables CroppedWidth and CroppedHeight are derived for the referenced PPS as follows:

CroppedWidth = pps_pic_width_in_luma_samples –	SubWidthC * ( pps_conf_win_left_offset + pps_conf_win_right_offset )	(1)

CroppedHeight = 	pps_pic_height_in_luma_samples –	SubHeightC * ( pps_conf_win_top_offset + pps_conf_win_bottom_offset )	(2)

–	The variables CroppedYPic[ y ][ x ] and chroma sample arrays CroppedCbPic[ y ][ x ] and CroppedCrPic[ y ][ x ], when present, are set to be the 2-dimensional arrays of decoded sample values of the 0th, 1st, and 2nd component, respectively, of the cropped decoded output picture.

–	ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	BitDepthY and BitDepthC are both set equal to BitDepth.

Experimental results

Coding Results

To validate the performance of the proposed generative face video SEI message 50 talking face video sequences are selected as testing sequences from the dataset VoxCeleb[16]. Each of these testing sequences is cropped from the original sequence into 10-second long (i.e. 250 frames at 25 fps) with the resolution of 256*256. The coded color format is RGB 444.

- For VVC, the Low-Delay-Bidirectional (LDB) configuration in VTM reference software is adopted to encode these testing sequences, where the quantization parameters (QP) are set to 45, 47, 50 and 52.

- For generative face video compression scheme, its base bicture is compressed as an I picture using the VTM reference software, and other pictures are further represented with a series of facial parameters and compressed via the generative face video SEI message. In particular, the QPs of the base picture are set to 37, 42, 47 and 52.

Figure 2 provides the overall rate-distortion performance of the 50 test sequences for VVC, FOMM, MRAA, Face_vid2vide, CFTE and IFVC. As sample-based distortion measures such as PSNR are not suitable to measure the quality of the reconstructed video, in Figure 2, two well-known learning based quality metrics, LPIPS[18] and DISTS[17], are used instead. Figure 2 shows that all generative-based methods enabled by the proposed generative face video SEI messages can significantly outperform VVC, and they can operate at ultra-low bit rate range that are difficult to reach for traditional codecs.

(a) Rate-DISTS                                 (b) Rate-LPIPS

Figure 2 - Rate-distortion performance comparison with the traditional codec (VVC) and generative compression systems with SEI messages (FOMM, MRAA, Face_vid2vid, CFTE and IFVC)

Open-source implemenations

A few of the algorithms in Table 1 are available as open source implementations. Table 5 summarizes the implementation url that includes the training/test code and pretrained model.

Table 5 - Summary of facial representations for generative face video compression algorithms

| Facial Representation | Representative Method | Implementation URL |
| --- | --- | --- |
| 2D landamrks | VSBNet [4] | https://github.com/sjtu-medialab/dafc |
| 2D keypoints+affine transformation matrix | FOMM [5] | https://github.com/AliaksandrSiarohin/first-order-model |
| region matrix | MRAA [6] | https://github.com/snap-research/articulated-animation |
| 3D keypoints | Face_vid2vid [7] | https://github.com/zhanglonghao1992/One-Shot_Free-View_Neural_Talking_Head_Synthesis |
| compact feature martix | CFTE [8] | https://github.com/Berlin0610/CFTE_DCC2022 |

Conclusion

A generative face video SEI message that supports a series of facial representations is proposed in this contribution. This proposed SEI message can be used to animate the base pictures coded using VVC or HEVC, and to reconstruct high quality face video pictures at extremely low bitrate. It also allows thte encoder to send facial landmarks to the decoder for encoder-controlled face animation and face filtering.The proposed SEI is applicable to video conferencing, live entertainment, face animation/filtering and metaverse-related functionalities. It is suggested to adopt the proposed SEI message into VSEI.

References

- Goodfellow I, Pouget-Abadie J, Mirza M, et al. “Generative adversarial nets,” Advances in neural information processing systems, vol. 27, 2014.

- Oquab M, Stock P, Haziza D, et al. Low bandwidth video-chat compression using deep generative models[C]//Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition. 2021: 2388-2397.

- Konuko G, Valenzise G, Lathuilière S. Ultra-low bitrate video conferencing using deep image animation[C]//ICASSP 2021-2021 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP). IEEE, 2021: 4210-4214.

- Feng D, Huang Y, Zhang Y, et al. A generative compression framework for low bandwidth video conference[C]//2021 IEEE International Conference on Multimedia & Expo Workshops (ICMEW). IEEE, 2021: 1-6.

- Siarohin A, Lathuilière S, Tulyakov S, et al. First order motion model for image animation[J]. Advances in Neural Information Processing Systems, 2019, 32.

- Siarohin A, Woodford O J, Ren J, et al. Motion representations for articulated animation[C]//Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition. 2021: 13653-13662.

- Wang T C, Mallya A, Liu M Y. One-shot free-view neural talking-head synthesis for video conferencing[C]//Proceedings of the IEEE/CVF conference on computer vision and pattern recognition. 2021: 10039-10049.

- Chen B, Wang Z, Li B, et al. Beyond keypoint coding: Temporal evolution inference with compact feature representation for talking face video compression[C]//2022 Data Compression Conference (DCC). IEEE, 2022: 13-22.

- Chen B, Wang Z, Li B, et al. Interactive Face Video Coding: A Generative Compression Framework[J]. arXiv preprint arXiv:2302.09919, 2023.

- Pearson D E, Robinson J A. Visual communication at very low data rates[J]. Proceedings of the IEEE, 1985, 73(4): 795-812.

- Musmann H G, Hötter M, Ostermann J. Object-oriented analysis-synthesis coding of moving images[J]. Signal processing: Image communication, 1989, 1(2): 117-138.

- Nakaya Y, Chuah Y C, Harashima H. Model-based/waveform hybrid coding for videotelephone images[C]//Acoustics, Speech, and Signal Processing, IEEE International Conference on. IEEE Computer Society, 1991: 2741, 2742, 2743, 2744-2741, 2742, 2743, 2744.

- Chung J S, Nagrani A, Zisserman A. , “Voxceleb2: Deep speaker recognition,” in Interspeech, 2018

- Chen B, Chen J, et al. “AHG9: Generative Face Video SEI Message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AC0088, January 2023.

- Chen B, Chen J, et al. “AHG9: Common SEI Message for Generative Face Video,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AD0051, April 2023.

- Nagrani A, Chung J S, Zisserman A. VoxCeleb: A Large-Scale Speaker Identification Dataset[J]. Proc. Interspeech 2017, 2017: 2616-2620.

- Ding K, Ma K, Wang S, et al. Image quality assessment: Unifying structure and texture similarity[J]. IEEE transactions on pattern analysis and machine intelligence, 2020, 44(5): 2567-2581.

- Zhang R, Isola P, Efros A A, et al. The unreasonable effectiveness of deep features as a perceptual metric[C]//Proceedings of the IEEE conference on computer vision and pattern recognition. 2018: 586-595.

Patent rights declaration(s)

Alibaba and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).