| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0084 |
| --- | --- |

| Title: | EE2-2.11: Harmonization of IBC HMVP and IBC-LIC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang1, Kai Zhang2, Li Zhang2 (ByteDance Inc.) 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA Zhihuang Xie, Yue Yu, Haoping Yu Dong Wang (OPPO) | Tel: Email: | zhangna.cynthia@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com xiezhihuang@oppo.com |
| Source: | Bytedance Inc. & OPPO |  |  |

_____________________________

# Abstract

In ECM, the IBC-LIC flag is not inherited when the motion information from an IBC HMVP candidate is inherited. To harmonize IBC HMVP and IBC-LIC, it is proposed that the IBC-LIC flag can be inherited from an IBC HMVP candidate, and tested in EE2-2.11a.

On top of the ECM9.0, simulation results of EE2-2.11a are reported as below:

AI: Class F -0.20%,	-0.08%, -0.18%, 100%, 100%; Class TGM -0.05%, -0.14%, -0.07%, 100%, 99%

RA: Class F -0.28%, -0.21%, -0.43%, 100%, 100%; Class TGM -0.04%, -0.04%, -0.11%, 100%, 99%

In EE2-2.11b, the combination of EE2-2.11a and EE2-2.10 is tested. On top of the ECM9.0, simulation results of EE2.2.11b are reported as below:

AI: Overall -0.01%,	0.00%, -0.02%, 100%, 100%

Class F -0.71%, -0.58%, -0.47%, 100%, 99%; Class TGM -0.13%, -0.14%, -0.12%, 100%, 100%

RA: Overall 0.00%, -0.03%, -0.06%, 100%, 100%

Class F -0.48%, -0.62%, -0.45%, 101%, 100% ; Class TGM -0.06%, -0.04%, -0.04%, 101%, 100%

# Introduction

Intra block copy with local illumination compensation (IBC-LIC) is a coding tool which compensates the local illumination variation within a picture between the CU coded with IBC and its prediction block with a linear model.

In ECM, the inter LIC flag can be inherited from a HMVP candidate. However, the IBC-LIC flag is not inherited when the motion information from an IBC HMVP candidate is inherited.

In EE2-2.10, it is proposed to add three more modes for IBC-LIC to further improve the coding performance. It is proposed to allow IBC-LIC to use the top-only, left-only, or top and left templates for deriving the model parameters. In addition, it is proposed to extend the MMLM to IBC-LIC.

# Proposed methods

In EE2-2.11a, it is proposed that the IBC-LIC flag can be inherited from an IBC HMVP candidate to harmonize IBC HMVP and IBC-LIC similar to the inter LIC case.

In EE2-2.11b, the combination of EE2-2.11a and EE2-2.10 is tested.

# Experimental Results

The proposed method has been implemented on top of the ECM-9.0 [1]. Testing results under AI/RA configurations are summarized in Table 1 and Table 2. The simulations were performed following the common test conditions [2]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of EE2-2.11a.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.20% | -0.08% | -0.18% | 100% | 100% |
| Class TGM | -0.05% | -0.14% | -0.07% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.28% | -0.21% | -0.43% | 100% | 100% |
| Class TGM | -0.04% | -0.04% | -0.11% | 100% | 99% |

Table 2: Coding performance of EE2-2.11b

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | -0.01% | 100% | 100% |
| Class A2 | -0.02% | -0.01% | 0.02% | 100% | 100% |
| Class B | -0.04% | -0.02% | -0.09% | 100% | 100% |
| Class C | 0.00% | -0.06% | 0.03% | 101% | 100% |
| Class E | 0.02% | 0.09% | 0.00% | 100% | 100% |
| Overall | -0.01% | 0.00% | -0.02% | 100% | 100% |
| Class D | 0.02% | 0.03% | -0.08% | 100% | 100% |
| Class F | -0.71% | -0.58% | -0.47% | 100% | 99% |
| Class TGM | -0.13% | -0.14% | -0.12% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.07% | 0.01% | 100% | 100% |
| Class A2 | -0.01% | -0.07% | -0.12% | 100% | 100% |
| Class B | 0.02% | 0.12% | -0.14% | 100% | 100% |
| Class C | 0.00% | -0.16% | 0.02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.06% | 100% | 100% |
| Class D | 0.05% | -0.35% | -0.02% | 100% | 100% |
| Class F | -0.48% | -0.62% | -0.45% | 101% | 100% |
| Class TGM | -0.06% | -0.04% | -0.04% | 101% | 100% |

- Conclusions

In this contribution, to further improve the efficiency of IBC prediction, it is proposed to harmonize IBC HMVP and IBC-LIC. The proposed method can bring some coding gain with almost no coding time change. It is recommended to adopt EE2-2.11b into the next version of ECM.

# References

[1] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-9.0

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).