| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0091 |
| --- | --- |

| Title: | EE2-3.8: Combination of Test 3.3e and Test 3.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar, Yang Wang, Yuwen He, Kai Zhang, Hongbin Liu, Li Zhang Jie Chen, Ru-Ling Liao, Xinwei Li, Yan Ye | Email: | mehdi.salehifar@bytedance.com wangyanag.cs@bytedance.com jiechen.cj@alibaba-inc.com |
| Source: | Bytedance Inc., Alibaba group |  |  |

_____________________________

- Abstract

This document reports test results for EE2-3.8, which is the combination test of EE2-3.3e and EE2-3.5. In test EE2-3.3e, high-accuracy template matching (TM) is proposed. In test EE2-3.5, multi-pass DMVR is extended by introducing additional BDOF iteration. On top of the ECM-9.0, simulation results of EE2-3.8 are reported as below:

RA: {};

LDB: {-0.15%, -0.30%, -0.34%; 100%, 100%}.

Introduction

EE2-3.3e

High-accuracy template matching (TM) was proposed in JVET-AD0155 [1] and established as Test 3.3 in EE2. EE2-3.3e consists of all three aspects of Test 3.3: In aspect #1, an additional refinement is applied to TM for bi-prediction. In aspect #2, the diamond search pattern used in TM is modified from 8-point to 16-point. In aspect #3, TM for bi-prediction is enabled when DMVR condition is satisfied.

EE2-3.5

Iterative BDOF pass in multi-pass DMVR was proposed in JVET-AD0196 [2] and JVET-AD0176 [3] and established as Test 3.5 in EE2. It is proposed to extend the current multi-pass DMVR by adding another BDOF-based motion refinement pass as the 4th pass of the multi-pass DMVR (i.e., the 2nd round of BDOF-based DMVR). Also, the subblock size of the proposed round of the BDOF-based DMVR is adaptively selected depending on the area (i.e., width×height) of the coding block. For blocks smaller than 1024 pixels, the subblock size of 4×4, and 8×8 is used otherwise.

Experimental results

The proposed method is implemented on top of the ECM-9.0 [4]. Testing results of EE2-3.8 under RA and LDB configurations are summarized in Table I. The simulations are performed following the common test conditions [5]. More detailed results can be found in the attached spreadsheet.

Table I: Performance of EE2-3.8

|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.16% | -0.33% | -0.26% | 101% | 99% | -0.12% | -0.50% | -0.49% | 101% | 101% |
| Class C | -0.18% | -0.36% | -0.17% | 103% | 101% | -0.10% | -0.21% | -0.24% | 101% | 100% |
| Class E |  |  |  |  |  | -0.25% | -0.11% | -0.23% | 98% | 101% |
| Overall |  |  |  |  |  | -0.15% | -0.30% | -0.34% | 100% | 100% |
| Class D | -0.15% | -0.31% | -0.43% | 102% | 102% | -0.07% | -0.25% | 0.28% | 102% | 100% |
| Class F | -0.09% | -0.11% | -0.21% | 101% | 100% | -0.23% | -0.35% | -0.40% | 99% | 100% |

Conclusions

In this contribution, the test results of EE2-3.8, which is the combination test of EE2-3.3e and E2-3.5, are reported. It is recommended to adopt this combination method into ECM.

References

- Y. Wang, K. Zhang, and L. Zhang, “Non-EE2: High-Accuracy template matching,” Document of Joint Video Experts Team, JVET-AD0155, Antalya, TR, 21–28 April 2023.

- M. Salehifar, Y. He, K. Zhang, H. Liu, L. Zhang, “EE2-related: On Iterative BDOF and EE2-2.6 Improvement” JVET-AD0196, Antalya, TR, 21–28 April 2023.

- J. Chen, R.-L. Liao, X. Li, Y. Ye, “Non-EE2: Improvements on multi-pass DMVR” JVET-AD0176, Antalya, TR, 21–28 April 2023.

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, Antalya, TR, 21–28 April 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).