| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0103-v1 |
| --- | --- |

| Title: | Weighted Edge Enhancement Filtering for Picture Upscaling and RPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET or Output document approved by JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Claßen, Mathias Wien | Tel: Email: | tim.classen@lfb.rwth-aachen.de |
| Source: | RWTH Aachen |  |  |

_____________________________

# Abstract

This input contribution discusses Reference Picture Resampling (RPR) and picture rescaling in ECM. An adaptive filtering method for upscaled video content is proposed. It proposes an adaptive filtering method for upscaled video content, suggesting the application of the same filter for both RPR upscaling and picture upscaling. The proposed method employs a locally weighted linear filter, consisting of two components. The first component involves computing the local weighting from the upscaled video content. The second component is a linear filter that is RD-optimized and transmitted in the bitstream. The results obtained from these operations are multiplied and added to the upscaled frame or block. The reported results are based on upscaling 1.5x and 2.0x, respectively, using the RPR test conditions outlined in JVET-Q0822, which defines the 0.5sec resolution switch. These changes result in average BD-Rate savings of -7.69%/-1.44%/-1.33% for 2.0x LDP and -7.27%/-0.57%/-0.32% for 2.0x LDB in ECM8.0. The input suggests further investigation in an exploration experiment (EE).

# Introduction

The VVC compression standard introduced Reference Picture Resampling (RPR) as a tool for adaptively changing the resolution within a Coded Layer Video Sequence. Unlike previous standards, the output resolution is not restricted to the resolution at which a given frame of the video sequence is coded. To achieve this functionality, a set of filters was introduced to facilitate switching between different resolutions. Currently, there are filters for 1.5x scaling and 2.0x scaling. Recently, a proposal has been made for filters for scaling factors of 1.25x and 1.33x [1].

The filters used for resolution change in the VVC standard are currently fixed linear filters. These filters are well suited for a wide range of video content. However, the content within coded video sequences is locally correlated and depends on the specific video sequence. Furthermore, image upscaling presents an inherently non-linear problem. To account for that, this document proposes an adaptive locally weighted filter. We can make the assertion that optimizing a filter with the knowledge of errors caused by coding and the characteristics of the current frame the overall performance of the overall coding scheme can be improved.

# Current RPR implementation in ECM

ECM supports both the switching of coding resolution and, temporarily, coding at lower resolution. Therefore, it necessitates both downsampling and upsampling procedures in order to switch resolution and to reference pictures of different resolution in inter-prediction. The current reference software in ECM employs a fixed upsampling procedure using interpolation filters for both of those applications. The upsampling is done by finite impulse response filters. While the resolution of the output device remains unchanged, the output itself still need ´s to be displayed at a high resolution. This proposal focuses on the quality of the reconstructed video at the original resolution. Specifically, the proposal suggests changes to the upsampling procedure while keeping the downsampling procedure unaffected. As a result, the evaluation of these changes is most effectively conducted in the high-resolution domain.

# Problem Description

Pictures coded at lower resolution often lack high frequency information which is present in their high-resolution equivalents. Consequently, pictures which were coded at low resolution usually appear blurred after upsampling. Consequently, the prediction error is usually higher if low-resolution frames are referenced. The issue stems from the fact that the upsampling process cannot recover high-frequency information which was lost during downsampling. However, artifacts caused by Reference Picture Resampling (RPR) are most pronounced in areas that do not align with the underlying assumptions of the interpolation scheme. In the reference software, this is particularly evident in content containing high-frequency information, such as edges.

The current reference software employs a fixed set of downsampling and upsampling filters without utilizing any side information from the coded video or bitstream to enhance the quality of the upsampled picture or prediction signal. For example, the distribution of frequency components may differ depending on the coded video sequence or spatial location making the most optimal filter depend on the characteristics of the content. Current filtering methods, such as the adaptive loop filter, solve this problem by signaling a set of filters. The decision, which filter is applied to a given picture partition is signaled and may additionally be inferred from the picture content. However, in case of upsampled image content, there are different types of errors compared to in-loop filters. Typically, there is some blurring caused by down- and upsampling. For that reason, applying sharpening, especially at the edges, can be beneficial.

# Proposal

This proposal addresses the issue of the upsampling procedure in Reference Picture Resampling (RPR) and picture upsampling. The current implementation in the ECM reference software takes a frame or block as input and applies a set of multi-phase interpolation filters to interpolate subsample positions. An issue is that the problem of image upsampling is inherently ill-posed. Therefore, no filtering method can solve this problem for an arbitrary type of content. However, side information which is inferred from the coded video or encoded in the bitstream may be used to amend this problem. It is proposed to add another processing step after the upsampling; namely, an adaptive locally weighted filter. This process is depicted in Figure 1.

Figure 1: Schematic diagram of weighted adaptive filter

The underlying assumption is that the optimal strength of the filter depends on the spatial location and can be inferred from the content. Since the filter is least-squared optimized based on the applied weighting and the upsampled picture, there is an interdependency. Therefore, the overall performance of the filter depends on the initial choice of weighting function. Consider, for example, a filter which is least-squares optimized (without weighting) such that the result is added to the filtered picture to obtain the output. After this optimization, certain pixels may have an optimal, too large, too small or incorrectly signed computed offset. Now, if it were possible to compute a local weighting that reduced the offset applied by the filter at regions where the computed offset is too high or has the wrong sign and increase the offset at regions where it is too small, the error would be decreased. Additionally, since the weighting is known beforehand, a better filter could be optimized. However, it is not practically feasible to find a weighting map function which is optimal for each type of content and at every location in the picture. Nonetheless, approximating the optimal weighting with a weighting map could already lead to improved results.

In the application of RPR upscaling, the content is usually blurred, especially in high-frequency structures such as edges. Linear filters usually exhibit ringing and overshoot artifacts and the filtered edge frequently still has less steepness compared to the ground truth edge. Therefore, increasing the offset of the filter at the predicted location of the edge and decreasing it at the surrounding would improve the filter.

## Algorithmic description

An overview of the proposed method is depicted in Figure 2. The inputs are the decoded, upscaled picture or block and the side information which is encoded in an adaption parameter set in the bitstream. First, the filter parameters are decoded. This includes information regarding the weighting map function, filter shape, luma and chroma flags, and filter coefficients. Next, the weighting map function is applied to obtain the local weighting map. Then, the filter is applied to the upscaled picture or block to generate a filtered map. The filtered map is multiplied by the local weighting and added to the upsampled picture as illustrated in Figure 2.

Figure 2: Schematic diagram of weighted edge enhancement filter

## Filter optimization

This filtering scheme is designed based on the characteristics of linear high-pass filters. When sharpening a blurred edge, it is necessary to amplify the steepness of the edge, particularly at its center. Achieving this effect with a linear filter can lead to issues such as ringing, overshoot, and noise amplification. Moreover, amplification of noise is a problem as well. To mitigate these effects, a local weighting is computed such that the strength of the filter is increased at the location of edges and decreased, or set to zero, in parts of the picture with small local gradient. The linear filter is optimized by a least-squares optimization which takes the local weighting into account. The weighting function can, in general, be any operation that is applied to the upsampled picture. The optimization problem, which seeks to obtain the filter coefficients, is expressed in the equation:

E[e2(x, y)]=E[(gx, y-sx,y)2]

Here, s represents the upsampled signal and g is the ground-truth picture. The filtering function is given by the following equation.

sx,y=wx, y+mx, ym,nαmnw(x-m, y-n)

Solving this equation leads to a matrix-vector equation which can be least-squares optimized to find the optimal filter coefficients. In a next step, the filter coefficients are quantized. Then, an optimal encoding of the filter coefficients is tested in an RD-optimization procedure. It is possible to, approximately, predict the quantized filter coefficients from previously decoded filters or to exploit correlations within a filter.

The weighting map used in the current implementation approximates the length of the gradient at each spatial location. Currently, the Sobel operator is employed for this purpose, as shown in the equation:

mx, y=hsobel, x(x,y)2+hsobel, y(x,y)2

The filters have an adaptive shape following the available shapes and filter symmetries for the Adaptive Loop Filter (ALF). The shape is signaled for each frame in the adaption parameter set. Figure 3 illustrates the available filter shapes. It is worth noting that equivalent indices at different spatial locations of the filter indicate that the coefficients are equivalent.

Figure 3: Different filter shapes

## Filter Prediction and Coding

To reduce the signaling costs of the filter, a prediction and entropy coding approach is employed to reduce coding costs for the quantized filter coefficients and parameters. Four modes are evaluated in the RD-optimization.

- Filter Intra: This mode predicts all filter coefficients by encoding the difference between the current filter coefficient and the previous filter coefficient. The last filter coefficient is encoded as the mean of all filter coefficients.

- Filter Inter: In this mode, the filter coefficients of the current filter are encoded as the difference between the coefficients of the current filter and those of a previously signaled filter. The index of the previous filter is signaled.

- Copy Filter: This mode involves copying all parameters from a previously signaled filter and applying the same filter to the current frame.

- Fallback: This mode does not employ prediction except for the last filter coefficient.

The encoding of flags and mode parameters is performed by fixed-length codes, while the filter coefficients are encoded by signed exponential Golomb coding.

### Filter Intra

Figure 4 illustrated the filter intra mode for an exemplary filter shape. The first filter coefficient is directly transmitted (c0=a0). The subsequent filter coefficients ai are predicted by the previous filter coefficient (a1…ai-2). The last filter coefficient is encoded as the sum over all filter coefficients (cn-1=iai).

Figure 4: Filter intra mode

### Filter Inter

Figure 5: Filter inter mode

In this mode, the current filter is predicted from a previous filter as illustrated in Figure 5. Thereby, the difference between both filters is calculated to obtain the encoded coefficients. By calculating the rate costs for each filter, the best filter from the list of previous reference filters is selected.

### Filter Copy

Figure 6: Filter copy mode

In the filter copy mode, the best filter is selected by measuring the RD-performance of applying each of the previous filters to the current picture as illustrated in Figure 6. The filter with the best performance is chosen and applied to the current picture.

### Fallback

The fallback mode serves as an alternative when the statistical assumptions do not hold. This mode performs only the prediction of the last coefficient (cn-1=iai) and directly encodes all the other coefficients without prediction.

# Simulation Results

The proposed method has been implemented in ECM-8.0 and ECM-9.0. They were tested for the “all intra”, “random access”, “low-delay P” and “low-delay B” configurations. The low-delay configuration tests follow the procedure described in [2]. In the random access configuration, the same setup is used but, additionally, the “-SwitchPocPeriod” is set to half the frame rate of the video sequence to enable a resolution switch every 0.5 seconds.

The RD-results are provided below for easy reference. Generally, the gain is more significant for the Y-component compared to the U- and V-components. For ease of access, the RD-results are shown below:

|  | Low delay P Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT* | DecT* |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -6,96% | -1,44% | -1,20% | 118% | 128% |
| Class C | -3,93% | -1,39% | -1,92% | 106% | 114% |
| Class E | -13,93% | -1,50% | -0,73% | 112% | 170% |
| Overall | -7,69% | -1,44% | -1,33% | 112% | 132% |
| Class D | -2,82% | -0,74% | -1,44% | 119% | 115% |
| Class F | -7,45% | -4,55% | -5,68% | 109% | 146% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT* | DecT* |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -6,87% | -0,79% | -1,04% | 102% | 115% |
| Class C | -3,55% | -1,24% | -0,63% | 83% | 92% |
| Class E | -12,90% | 0,67% | 1,29% | 70% | 90% |
| Overall | -7,27% | -0,57% | -0,32% | 87% | 100% |
| Class D | -2,62% | 1,03% | -1,06% | 68% | 77% |
| Class F | -7,06% | -3,63% | -4,81% | 80% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT* | DecT* |
| Class A1 | #WERT! | #WERT! | #WERT! | #DIV/0! | #DIV/0! |
| Class A2 | #WERT! | #WERT! | #WERT! | #DIV/0! | #DIV/0! |
| Class B | -6,02% | -0,65% | -0,96% | 125% | 132% |
| Class C | -4,17% | -1,22% | -1,36% | 89% | 100% |
| Class E |  |  |  |  |  |
| Overall | #WERT! | #WERT! | #WERT! | #DIV/0! | #DIV/0! |
| Class D | -3,05% | -0,30% | -1,99% | 66% | 84% |
| Class F | -8,08% | -5,03% | -6,40% | 110% | 151% |

*runtime measurements are unreliable because of inhomogeneous cluster.

# Summary

A filtering method for the filtering of upsampled pictures, based on a weighted linear filter, is proposed. The filtering method is based on a weighted linear filter. The weighting approximates the local gradient, while the linear filter is optimized using a least-squares approach. The results of the method consistently show gains across all coding configurations and classes. Reported gains range from -2.62% to -13.93% for luma-channel bd-rate improvement. The input suggests further investigation in an exploration experiment (EE).

# References

[1] Jonatan Samuelsson-Allendes, Sachin Deshpande, “AHG12: RPR filters for scale factors below 1.5x”, April 2023

[2] Jiancong Luo, Vadim Seregin, “Reference picture resampling functionality confirmation test”, January 2020

# Patent rights declaration(s)

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).