| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0104-v1 |
| --- | --- |

| Title: | AHG10: GOP-based RPR encoder control using parallel resolution encoding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Daichi Arai Shimpei Nemoto Shunsuke Iwamura Atsuro Ichigaya | Tel: Email: | arai.d-es@nhk.or.jp nemoto.s-fy@nhk.or.jp iwamura.s-gc@nhk.or.jp ichigaya.a-go@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution aims to improve the performance of JVET-AB0080 in controlling the encoder for GOP-based Reference Picture Resampling (RPR). The GOP-based selection method utilizes parallel resolution encoding for Rate-Distortion (RD) optimization. This method performs multi-pass encoding on the downscaled pictures and then calculates the RD cost for each resolution to determine the optimal resolution in each GOP.

The BD-rate performance (Y/Cb/Cr) compared to CTC with VTM-20.2 as anchor is as follows:

RA (luma/Cb/Cr): -0.65%/1.52%/0.48%

LDB (luma/Cb/Cr): -0.06%/-0.12%/-0.33%

AI (luma/Cb/Cr): -0.21%/4.83%	/2.23%

This method is especially beneficial for 4K resolution in the RA configuration.

Introduction

Adaptive Resolution Change (ARC) is highly effective in fixed bitrate scenarios, such as broadcasting. JVET-AB0080 [1] introduces a method to determine the use of RPR based on the quantization parameter (QP) and the self-similarity of the rescaled image. While this approach offers significant advantages in terms of encoding efficiency and computational complexity, it may not always select the most suitable resolution. To address this limitation, this contribution proposes an RD optimization method that employs multi-pass encoding to explore the performance limits of ARC.

Proposal

Fig. 1 shows an overview of GOP-based RPR encoder control using parallel resolution encoding. This method initially downscales the original pictures into multiple resolutions and performs multi-pass encoding on the reduced resolutions. Based on JVET-AB0080, this method employs three reduced resolutions (1/2, 2/3 and 4/5) and adjusts the QP values by -6, -4, and -2 respectively to achieve similar bitrates as the original resolution. Subsequently, the reduced decoded pictures are upscaled to the original resolution, and the RD cost for each resolution is calculated. The RD cost for the i-th resolution is calculated as follows.

Li=cYCbCrtGOPsizewc(λc,tri,t+Xc,t-Xi,c,t22)

where wc represents the weight coefficient for each YCbCr, ri,t represents the number of bits in the t-th frame at the i-th resolution, λc,t represents the value used to encode the original picture of the c-th channel in the t-th frame, Xc,t is the original picture of the c-th channel in the t-th frame, and Xi,c,t represents the decoded picture of the c-th channel in the t-th frame at the i-th resolution. To ensure an equal evaluation of the RD cost for each resolution, the value of λc,t corresponds to the per-frame, per-channel value used to encode the original pictures. In this experiment, wc=[8,1,1] is used for the YCbCr-PSNR evaluation. Finally, this method determines the optimal resolution for each GOP-based on the RD cost.

Fig. 1 The overview of GOP-based RPR encoder control using parallel resolution encoding.

Results

The proposed method is implemented on top of VTM-20.2 [2], and the experimental results of this method are compared with the anchor results under VTM CTC [3] and the case of enabling GOP-based RPR in VTM-20.2, respectively. Decoding time are not reliable. As shown in the results, this approach provides benefits in BDR, especially for 4K resolution in RA configuration. Although there is some loss in chroma BDR, no chroma issues have been observed through visual inspection.

CTC (QP=22, 27, 32, 37)

BD-rate Over VTM-20.2

|  | Random access Main10 (QP22,27,32,37) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-20.2 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -2.24% | -0.18% | -2.90% | 118% | 137% |
| Class A2 | -1.14% | 7.42% | 5.36% | 111% | 111% |
| Class B | 0.06% | 0.20% | -0.05% | 166% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 165% | 92% |
| Class E |  |  |  |  |  |
| Overall | -0.65% | 1.52% | 0.48% | 143% | 106% |
| Class D | 0.00% | 0.00% | 0.00% | 167% | 93% |

|  | Low delay B Main10 (QP22,27,32,37) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-20.2 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | -0.30% | -0.79% | 182% | 110% |
| Class C | 0.00% | 0.00% | 0.00% | 173% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 211% | 101% |
| Overall | -0.06% | -0.12% | -0.33% | 185% | 105% |
| Class D | 0.00% | 0.00% | 0.00% | 167% | 100% |

|  | All Intra Main10 (QP22,27,32,37) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-20.2 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.77% | 2.91% | -4.19% | 116% | 120% |
| Class A2 | -2.04% | 26.32% | 17.87% | 104% | 104% |
| Class B | -0.01% | -0.14% | -0.19% | 149% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 133% | 95% |
| Class E | 0.00% | 0.00% | 0.00% | 127% | 99% |
| Overall | -0.21% | 4.83% | 2.23% | 128% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 149% | 98% |

BD-rate Over VTM-20.2 GOP-based RPR

|  | Random access Main10 (QP22,27,32,37) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-20.2 GOP-based RPR |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.89% | 3.06% | -0.55% | 137% | 122% |
| Class A2 | -1.10% | 2.31% | 1.14% | 121% | 96% |
| Class B | -0.11% | 0.01% | 0.41% | 169% | 93% |
| Class C | 0.00% | 0.00% | 0.00% | 166% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.44% | 1.08% | 0.25% | 151% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 166% | 96% |

|  | Low delay B Main10 (QP22,27,32,37) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-20.2 GOP-based RPR |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | -0.18% | -0.41% | 183% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 173% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 211% | 99% |
| Overall | -0.05% | -0.08% | -0.17% | 186% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 168% | 100% |

|  | All Intra Main10 (QP22,27,32,37) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-20.2 GOP-based RPR |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 1.20% | 3.61% | -3.31% | 111% | 113% |
| Class A2 | -1.08% | 14.50% | 11.55% | 105% | 103% |
| Class B | -0.06% | 0.03% | 0.04% | 146% | 96% |
| Class C | 0.00% | 0.00% | 0.00% | 130% | 97% |
| Class E | 0.00% | 0.00% | 0.00% | 127% | 99% |
| Overall | 0.00% | 3.02% | 1.39% | 126% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 148% | 98% |

QP=22, 27, 32, 37, 42

BD-rate Over VTM-20.2

|  | Random access Main10 (QP22,27,32,37,42) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-20.2 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -3.72% | -0.65% | -3.66% | 122% | 142% |
| Class A2 | -1.47% | 7.76% | 5.32% | 116% | 122% |
| Class B | 0.08% | -0.01% | -0.52% | 173% | 104% |
| Class C | 0.00% | 0.00% | 0.00% | 165% | 92% |
| Class E |  |  |  |  |  |
| Overall | -1.01% | 1.42% | 0.16% | 147% | 110% |
| Class D | 0.01% | -0.01% | 0.02% | 173% | 93% |

|  | Low delay B Main10 (QP22,27,32,37,42) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-20.2 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.20% | -0.78% | -1.56% | 191% | 116% |
| Class C | 0.00% | 0.00% | 0.00% | 181% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 216% | 101% |
| Overall | -0.08% | -0.33% | -0.65% | 194% | 107% |
| Class D | 0.02% | 0.06% | 0.06% | 176% | 102% |

|  | All Intra Main10 (QP22,27,32,37,42) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-20.2 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.63% | 3.22% | -5.65% | 124% | 121% |
| Class A2 | -2.44% | 27.01% | 16.57% | 109% | 107% |
| Class B | -0.10% | -0.53% | -0.68% | 149% | 104% |
| Class C | 0.00% | 0.00% | 0.00% | 140% | 95% |
| Class E | 0.00% | -0.01% | -0.01% | 126% | 100% |
| Overall | -0.54% | 4.89% | 1.63% | 131% | 104% |
| Class D | 0.01% | 0.00% | 0.01% | 156% | 99% |

BD-rate Over VTM-20.2 GOP-based RPR

|  | Random access Main10 (QP22,27,32,37,42) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-20.2 GOP-based RPR |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.43% | 3.10% | 0.32% | 149% | 117% |
| Class A2 | -0.97% | 1.74% | 0.62% | 135% | 98% |
| Class B | 0.09% | 0.16% | 0.54% | 178% | 95% |
| Class C | 0.00% | 0.00% | 0.00% | 165% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | 1.02% | 0.37% | 159% | 99% |
| Class D | 0.01% | -0.01% | 0.02% | 170% | 95% |

|  | Low delay B Main10 (QP22,27,32,37,42) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-20.2 GOP-based RPR |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.20% | -0.50% | 193% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 181% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 217% | 99% |
| Overall | 0.02% | -0.08% | -0.21% | 195% | 101% |
| Class D | 0.02% | 0.06% | 0.06% | 176% | 101% |

|  | All Intra Main10 (QP22,27,32,37,42) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-20.2 GOP-based RPR |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.28% | 5.16% | -3.66% | 115% | 109% |
| Class A2 | -1.17% | 13.36% | 10.14% | 109% | 102% |
| Class B | -0.01% | 0.07% | 0.03% | 144% | 97% |
| Class C | 0.00% | 0.00% | 0.00% | 135% | 97% |
| Class E | 0.00% | -0.01% | -0.01% | 125% | 100% |
| Overall | -0.15% | 3.10% | 1.09% | 127% | 100% |
| Class D | 0.01% | 0.00% | 0.01% | 155% | 98% |

Conclusions

The results indicate that the GOP-based RPR encoder control using parallel resolution encoding can provide additional advantages for VTM, particularly in terms of 4K resolution. This approach offers potential for further contributions, such as refining the RD cost formula to achieve even better results. Therefore, it is suggested to enable the parallel resolution encoding option in the VTM reference software.

References

[1] K. Andersson, J. Ström, R. Yu, P. Wennersten, W. Ahmad, “AHG10: GOP-based RPR encoder control”, JVET-AB0080, 2022.

[2] VTM-20.2, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-20.2.

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, 2021.

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).