| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0106 |
| --- | --- |

| Title: | AHG9: On missing value ranges for some syntax elements in the NNPFC SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chaoyi Lin1, Ye-Kui Wang2, Jizheng Xu2, Wei Jia2, Junru Li2, Yue Li2, Kai Zhang2, Li Zhang2 1Bafangcheng 9th building F14 Yuhang District, 311100, Hangzhou City, Zhejiang Province, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | linchaoyi.cy@bytedance.com yekui.wang@bytedance.com xujizheng@bytedance.com wei.jia@bytedance.com lijunru@bytedance.com yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the latest text for the neural-network post-filter characteristics (NNPFC) SEI message in JVET-AD2006, the value ranges of ue(v)-coded syntax elements nnpfc_inp_format_idc and nnpfc_out_format_idc are missing. It is proposed to specify that the value of nnpfc_inp_format_idc and nnpfc_out_format_idc shall be in the range of 0 to 7, inclusive.

# Proposal

The proposed text changes are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts.

### 8.28.2.2 Neural-network post-filter characteristics SEI message semantics

...

nnpfc_inp_format_idc indicates the method of converting a sample value of the input picture to an input value to the NNPF. The value of nnpfc_inp_format_idc shall be in the range of 0 to 7, inclusive. Values of 2 to 7, inclusive, for nnpfc_inp_format_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_inp_format_idc in the range of 2 to 7, inclusive.

When nnpfc_inp_format_idc is equal to 0, the input values to the NNPF are real numbers and the functions InpY( ) and InpC( ) are specified as follows:

InpY( x ) = x ÷ ( ( 1  <<  BitDepthY ) − 1 )		(81)

InpC( x )= x ÷ ( ( 1  <<  BitDepthC ) − 1 )		(82)

When nnpfc_inp_format_idc is equal to 1, the input values to the NNPF are unsigned integer numbers and the functions InpY( ) and InpC( ) are specified as follows:

shiftY = BitDepthY − inpTensorBitDepthY
if( inpTensorBitDepthY  >=  BitDepthY)
	InpY( x ) = x  <<  ( inpTensorBitDepthY − BitDepthY )	(83)
else
	InpY( x ) = Clip3(0, ( 1  <<  inpTensorBitDepthY ) − 1, ( x + ( 1  <<  ( shiftY − 1 ) ) )  >>  shiftY )

shiftC = BitDepthC − inpTensorBitDepthC
if( inpTensorBitDepthC  >=  BitDepthC )
	InpC( x ) = x  <<  ( inpTensorBitDepthC − BitDepthC ) 	(84)
else
	InpC( x ) = Clip3(0, ( 1  <<  inpTensorBitDepthC ) − 1, ( x + ( 1  <<  ( shiftC − 1 ) ) )  >>  shiftC )

The variable inpTensorBitDepthY is derived from the syntax element nnpfc_inp_tensor_luma_bitdepth_minus8 as specified below. The variable inpTensorBitDepthC is derived from the syntax element nnpfc_inp_tensor_chroma_bitdepth_minus8 as specified below.

Values of nnpfc_inp_format_idc greater than 1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages that contain reserved values of nnpfc_inp_format_idc.

...

nnpfc_out_format_idc equal to 0 indicates that the sample values output by the NNPF are real numbers where the value range of 0 to 1, inclusive, maps linearly to the unsigned integer value range of 0 to ( 1  <<  bitDepth ) − 1, inclusive, for any desired bit depth bitDepth for subsequent post-processing or displaying.

nnpfc_out_format_idc equal to 1 indicates that the luma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  outTensorBitDepthY ) − 1, inclusive, and the chroma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  outTensorBitDepthC ) − 1, inclusive.

The value of nnpfc_out_format_idc shall be in the range of 0 to 7, inclusive. Values of 2 to 7, inclusive, for Values of nnpfc_out_format_idc greater than 1 are reserved for future specification use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with that contain reserved values of nnpfc_out_format_idc in the range of 2 to 7, inclusive.

...

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).