| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0108-v1 |
| --- | --- |

| Title: | AHG12: DMVR with robust MV derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, R. Yu | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In JVET-AD0045, it was reported that DMVR in VVC can sometimes produce dislocated subblocks with unreliable motion vectors which leads to subjective artifacts. It is found that multi-pass DMVR in ECM can also have such a problem. It is therefore proposed to include additional criterions on top of bilateral block matching cost calculation for cases where there is a risk to have an unreliable motion vector to make sure that selected motion vectors result in subblocks that are aligned with neighbouring subblocks. The proposed method inherits low spatial activity check, subblock motion vector difference check and boundary difference checks from JVET-AD0045. When there is a risk for unreliable motion vectors, a boundary distortion metric is added to the block matching cost to bias the selection of motion vectors to produce subblocks that are aligned with above and left neighbouring subblocks.

Objective performance (BDR) versus ECM-9.0 is reported as:

For CTC at normal QP range:

RA: Y: 0,00%, U -0,03%, V -0,01%	 Enc:100%	Dec:101%

For CTC at higher QP range QP= 40, 43, 47, 50:

RA: Y -0,07%, U -0,06%, V -0,02%	 Enc: 100%	Dec:101%

It is also asserted that the proposal can improve subjective quality.

# Introduction

In JVET-AD0045 [1], an encoder only solution was suggested to fix visual artifacts from DMVR. This was then added to the VVC reference software. In ECM [2], DMVR has been further developed to multi-pass DMVR which combine DMVR refinements on both block and subblock basis, and also use a larger search range than for VVC followed with refinements by BDOF in 8x8 subblocks.

It is found that the multi-pass DMVR in ECM can also produce visual artifacts similar to those for VVC for larger block sizes such as 32x32 or greater. Since ECM is work in progress and not a standard it is better to address the issue normatively rather than trying to avoid use of DMVR from encoder side when there is a risk to get unreliable motion vectors.

# Proposal

The proposal is to add subblock boundary distortion to the subblock bilateral matching distortion to make sure that selected subblock motion vectors produce a subblock aligned with neighboring subblocks.

Firstly, spatial activity for the reference subblocks centered at the block motion vector is determined and if the spatial activity is lower than a threshold, the subblock is determined to have high risk of having unreliable motion vectors. For the subblock that risks getting unreliable motion vectors, each candidate motion vector for the subblock is compared with a neighboring subblock’s motion vectors across a subblock boundary (if there exists). If the absolute difference between the candidate motion vector and the neighboring subblock’s motion vector for one component is greater than 16, a boundary check which is based on neighboring samples is made for the corresponding subblock boundary. If the boundary check indicates that there not is a true edge along the subblock boundary, then subblock boundary distortion is determined across that subblock boundary. If the subblock boundary distortion is greater than a threshold the subblock boundary distortion is added to the block matching distortion and the motion vectors that minimize the total distortion is selected.

# Results

## Subjective impact

The proposal can improve subjective quality by a robustification of the motion vector derivation for multi-pass DMVR, see broken line in Figure 1.

Figure 1: Random access QP 43, BQTerrace, POC 296. Left: ECM-9.0, Right: Proposal

## Objective impact

The performance has been compared both for CTC at normal QP range and CTC at a higher QP range (40, 43, 47 and 50). Encoding times are not accurate.

For CTC at normal QP range there is very similar objective performance but for the higher QP range some objective benefit can be shown.

### CTC [3]

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | -0,01% | 0,05% | 102% | 101,0% |
| Class A2 | -0,04% | -0,06% | -0,08% | 104% | 101,1% |
| Class B | -0,01% | 0,01% | 0,02% | 99% | 101,0% |
| Class C | 0,04% | -0,05% | -0,06% | 98% | 101,5% |
| Class E |  |  |  |  |  |
| Overall | 0,00% | -0,03% | -0,01% | 100% | 101,2% |
| Class D | -0,03% | -0,08% | -0,02% | 98% | 100,4% |

### QP 40, 43, 47, 50

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 (QP 40,43,47,50) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,02% | -0,21% | -0,05% | 92% | 100,6% |
| Class A2 | -0,11% | -0,12% | -0,05% | 97% | 101,1% |
| Class B | -0,06% | -0,01% | 0,01% | 101% | 102,1% |
| Class C | -0,11% | 0,02% | -0,02% | 101% | 101,1% |
| Class E |  |  |  |  |  |
| Overall | -0,07% | -0,06% | -0,02% | 98% | 101,4% |
| Class D | -0,13% | 0,11% | 0,26% | 106% | 101,5% |

# Conclusion

It has been shown that the proposed method can improve robustness of the derivation of motion vectors in multi-pass DMVR in cases where it is a risk to get unreliable motion vectors. The proposed method maintains objective performance of ECM for CTC QP range and can achieve objective improvements for a higher QP range where DMVR also is used more often.

It is suggested to include the method to ECM.

# References

[1] K. Andersson, R, Yu, “AHG10: Encoder MV selections and DMVR revisited”, JVET-AD0045, Antalya Turkey, April 2023

[2] ECM-9.0, available at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-9.0

[3] M. Karczewice, Y. Yan, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).